3GPP TSG-RAN WG1 Meeting #112bis-e                                                                                 R1-2302342

E-meeting, 17-26 April, 2023

Agenda Item:	9.6.1

Source:	Huawei, HiSilicon

Title:	Discussion on potential solutions to further reduce UE complexity

Document for:	Discussion and Decision

# Introduction

In RAN1#112 [1], the following agreements have been made for UE bandwidth reduction and UE peak data rate reduction.

Peak data rate target Agreement Revise the earlier agreement by removing the square brackets like this: - The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306. The same value for X is used for DL and UL Maximum number of PRBs Agreement For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS Note: No intention to change the RAN4 RF specifications about maximum transmission PRB number Time relaxation between RAR and Msg3 Agreement For the earlier RAN1 agreement achieved in RAN1#111 as following, For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS Note: Single Value pair for X is to selected for SCSs Additional separate initial DL/UL BWP Conclusion There is no consensus to continue discussion on “whether additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is allowed to be configured by the SIB in the cell”. Simultaneous reception of two broadcast PDSCH Conclusion For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR. Peak data rate reduction Agreement For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2. - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X. Paging PDSCH Agreement Update the agreement for PDSCH paging with the clarification as follows: - From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). The scheduling of paging PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. MsgA PUSCH Agreement For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Msg4 PDSCH Working Assumption - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

In this contribution, potential solutions to further reduce UE complexity in FR1 is discussed.

# UE BB bandwidth reduction

This section will discuss on UE baseband bandwidth reduction scheme.

## Random access timeline

In last meeting [1], for UE BB channel bandwidth reduction, the following agreement was made regarding the RAR bandwidth and Msg3 timeline.

Agreement For the earlier RAN1 agreement achieved in RAN1#111 as following, For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS Note: Single Value pair for X is to selected for SCSs

Considering that the UE can only process no more than 5 MHz part of the RAR PDSCH per slot and the scheduling of RAR PDSCH is up to 20MHz, additional PDSCH processing time X is needed for UE to first buffer then process RAR PDSCH and the value of X can be set up to 3NT,1. Furthermore, because NT,1 also includes some processing time of PDCCH, the exact value of X for PDSCH processing can be 2NT,1. For 15kHz SCS, the value of X can equal to 2ms. For 30kHz SCS, the value of X can equal to 1ms.

Proposal 1: For additional PDSCH processing time, the value of X can be 2/1 ms for 15/30kHz SCS.

Similar as RAR and Msg3, in current spec the timeline between RAR and PRACH is also specified. Rel-17 RedCap and non-RedCap UEs shall be ready to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1  symbols corresponding to a PDSCH processing time as captured following in TS 38.213.

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window and LSBs of a SFN field in the DCI format 1_0, if included and applicable, are not same as corresponding LSBs of the SFN where the UE transmitted PRACH, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit a PRACH. If requested by higher layers, the UE shall be ready to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 assuming μ corresponds to the smallest SCS configuration among the SCS configurations for the PDCCH carrying the DCI format 1_0, the corresponding PDSCH when additional PDSCH DM-RS is configured, and the corresponding PRACH. For μ=0, the UE assumes N1,0=14 [6, TS 38.214]. For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines N1 assuming SCS configuration μ=0.

Similarly, when the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, additional PDSCH processing time Y is needed for UE to first buffer then process RAR PDSCH without RAR PDSCH performance loss. The value of Y should be same as that of X because of the similar RAR PDSCH processing method.

Proposal 2: For UE BB bandwidth reduction, when the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, the UE shall be ready to transmit a PRACH no later than NT,1+0.75+Y msec after the last symbol of the window, or the last symbol of the PDSCH reception.

- For additional PDSCH processing time, the value of Y should be same as that of X.

## Simultaneous reception

In last meeting[1], simultaneous reception of one unicast PDSCH and one broadcast PDSCH was further discussed and following FL5 Proposal 2.5-2c was made based on companies’ responses.

High Priority Proposal 2.5-2c: For UE BB complexity reduction, it is FFS whether some specification changes (e.g., regarding prioritization for processing and decoding) are needed for simultaneous reception of unicast and broadcast PDSCH transmissions.

In current spec, for legacy non-eRedCap UE, UE shall be able to decode a unicast PDSCH and another broadcast PDSCH that partially or fully overlap in time in non-overlapping PRBs. The RF and BB bandwidth for legacy non-eRedCap UE is same. And the bandwidth for unicast PDSCH and broadcast PDSCH is confined within the active BWP, which will not exceed legacy non-eRedCap UE’s BB bandwidth.

For Rel-18 eRedCap UE with BB bandwidth reduction, the BB bandwidth is not larger than 5MHz. If the total number of PRBs for the unicast PDSCH and broadcast PDSCH is not larger than the BB bandwidth, the UE shall be able to decode the unicast PDSCH and broadcast PDSCH that partially or fully overlap in time in non-overlapping PRBs, which is similar as legacy non-eRedCap UE.

Proposal 3: For UE BB bandwidth reduction, when the total number of PRBs for the two PDSCH (unicast and broadcast PDSCH) is not larger than 5MHz, the UE shall be able to decode a unicast PDSCH, and another broadcast PDSCH in non-overlapping PRBs.

For Rel-18 eRedCap UE with BB bandwidth reduction, it is agreed that a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. So it is possible that the total number of PRBs for the unicast PDSCH and broadcast PDSCH is larger than the BB bandwidth. In this case, the new UE behavior should be specified for Rel-18 eRedCap UE with BB bandwidth reduction.

However, unicast PDSCH need to differentiate Msg4 and unicast PDSCH other than Msg4. For unicast PDSCH other than Msg4, the scheduling bandwidth is not larger than 5MHz agreed in previous agreement from 110b-e. For Msg4, it is possible that the scheduling bandwidth is larger than 5MHz due to Msg3 collision if Msg1 early indication is not configured.

For Msg4, if the scheduling bandwidth is larger than 5MHz, the UE is not required to process or receive a Msg4 PDSCH as the limited UE BB bandwidth capability. In this case, the broadcast PDSCH should be prioritized. If the scheduling bandwidth of Msg4 PDSCH is not larger than 5MHz, the UE shall prioritize to decode the Msg4 PDSCH because the Msg4 PDSCH has HARQ-ACK timeline requirements.

Proposal 4: For UE BB bandwidth reduction, when the total number of PRBs for the two PDSCH (Msg4 and broadcast PDSCH) is larger than 5MHz,

- if the PRBs for the Msg4 PDSCH is larger than 5MHz, the UE prioritizes to decode a broadcast PDSCH.

- otherwise, the UE prioritizes to decode a Msg4 PDSCH.

For unicast PDSCH other than Msg4, the scheduling bandwidth is always not larger than 5MHz, so the UE shall prioritize to decode the unicast PDSCH other than Msg4 because the unicast PDSCH other than Msg4 has HARQ-ACK timeline requirements.

Proposal 5: For UE BB bandwidth reduction, when the total number of PRBs for the two PDSCH (normal unicast and broadcast PDSCH) is larger than 5MHz, the UE prioritizes to decode a normal unicast PDSCH.

## Separate early indication

In RAN#2 last meeting [3], the following agreement was made on need for early indication:

Discussion on need for Msg3/MsgA PUSCH early indication: Introduce Msg3/MsgA PUSCH based early indication for Rel-18 eRedCap. FFS how to implement this in the spec (e.g., new LCIDs or not). Discussion on need for Msg1 early indication: We will wait for RAN1 progress to see if there is a need for a Msg1 early indication for eRedCap.

In RAN#99 meeting [2], it is agreed that Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps. In addition, there are four notes as following:

Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

In this section, the key point is whether the separate early indication is supported by Msg1 for Rel-18 eRedCap UE.

For Rel-18 eRedCap UE capable of BW3/PR3 + PR1, a separate early indication by Msg1 is needed. As discussed in section 2.1, if the number of PRBs for RAR PDSCH exceeds 5MHz, additional PDSCH processing time is needed for the R18 eRedCap UE capable of BW3/PR3 + PR1 to first buffer then process RAR PDSCH. If the separate early indication for Rel-18 eRedCap UEs by Msg1/MsgA is not introduced, the network needs to schedule the RAR within a relaxed timeline for all the UEs, including eRedCap UE and non-eRedCap UE.

As agreed in RAN#99, for Rel-18 eRedCap UE capable of 20MHz + PR1, it will use the same initial access procedure as Rel-18 eRedCap UE capable of BW3/PR3 + PR1. Consequently, the same separate early indication by Msg1 also needed for this UE.

Proposal 6: A separate early indication by Msg1/MsgA PRACH is supported for Rel-18 eRedCap UE.

## FDRA for Msg3

For PUSCH Msg3 hopping, the frequency resource allocation field in the current UL grant of MAC RAR is fixed to 14 bits[2]. The actual bits for resource allocation are dependent on the BWP size. For eRedCap UEs in FR1, at most 13 bits are needed to make resource allocation within 20MHz for 15KHz SCS, so one bit is reserved in the 14 bits FDRA field.

Table 8.2-1: Random Access Response Grant Content field size

| RAR grant field | Number of bits |
| --- | --- |
| Frequency hopping flag | 1 |
| PUSCH frequency resource allocation | 12, for operation with shared spectrum channel access in FR1 or for FR2-2 when ChannelAccessMode2-r17 is provided 14, otherwise |
| PUSCH time resource allocation | 4 |
| MCS | 4 |
| TPC command for PUSCH | 3 |
| CSI request | 1 |
| ChannelAccess-CPext | 2, for operation with shared spectrum channel access in FR1 or for FR2-2 when ChannelAccessMode2-r17 is provided 0, otherwise |

Observation 1: For eRedCap, 13 bits is needed for flexible Msg3 frequency resource allocation.

Moreover, one or two bits in the resource allocation field is consumed to indicate the frequency offset when frequency hopping is enabled, as shown in the table below[2], where NBWPsize is the size of initial UL BWP.  So actually only 12 or 11 bits can be used for resource allocation.

Table 8.3-1: Frequency offset for second hop of PUSCH transmission with frequency hopping scheduled by RAR UL grant or of Msg3 PUSCH retransmission

| Number of PRBs in initial UL BWP | Value of NUL,hop Hopping Bits | Frequency offset for 2nd hop |
| --- | --- | --- |
| NBWPsize<50 | 0 | NBWPsize2 |
|  | 1 | NBWPsize4 |
| NBWPsize≥50 | 00 | NBWPsize2 |
|  | 01 | NBWPsize4 |
|  | 10 | -NBWPsize4 |
|  | 11 | Reserved |

Observation 2: For eRedCap, 1 bit of FDRA is reserved. When frequency hopping is enabled, only 11/12 bits of FDRA can be used for Msg3 frequency resource allocation.

In such case, the resource allocation can only be less than 20 PRBs or more than 88 PRBs. As the lengths of 88~106 cannot be allocated for eRedCap, the actual available resource allocation for Msg3’s FH is very limited. Instead, gNB has to allocate more time domain resource for eRedCap UEs, which will impose significant restriction on PUSCH frequency resource allocation, especially when the eRedCap is co-existence with Rel-17 RedCap UEs or eMBB UEs.

Thus, instead of using one or two bits from the frequency resource assignment, the unused bits can be used for frequency offset indication, to provide flexibility for Msg3 resource allocation for eRedCap UEs.

Proposal 7: For BB bandwidth reduction, maximum 13 bits in the RAR UL grant are used for Msg3 resource allocation (15KHz SCS) regardless of Msg3 FH is enabled or disabled.

## Msg4 PDSCH

In last meeting[1], the following working assumption was made.

Working Assumption For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

The working assumption discusses whether the UE needs to process the Msg4 PDSCH, under the assumption that UE needs to receive the Msg4 PDSCH. But once the UE decodes the Msg4 PDCCH, and knows the frequency resource allocation is larger than 25 PRBs, the UE does not have to receive the Msg4 PDSCH, e.g. PDCCH is on the first 3 symbols, and PDSCH is on the last 7 symbols of the slot. Not receiving PDSCH can help saving UE’s power consumption and stop the current RACH procedure in time, which is also helpful for improving the overall latency of RACH procedure. So we propose to

Proposal 8: Confirm the working assumption with minor modifications:

Working Assumption

For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

- The UE is not required to process receive a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

# UE peak data rate reduction

For UE peak data rate reduction, the following agreements were made in last meeting [1].

Agreement Revise the earlier agreement by removing the square brackets like this: - The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306. - The same value for X is used for DL and UL Agreement For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2. - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X.

When UE peak data rate reduction is supported as an add-on to UE BB bandwidth reduction, the value for X will be discussed through such below situations. Based on the PDSCH/PUSCH bandwidth reduction, the downlink peak data rate is more than 13Mbps calculated by the equation below when X equals to 4 (vLayers·Qm·f ≥ 4) and SCS is 15kHz.

To meet the downlink peak data rate of 10 Mbps, the constraint value X can be relaxed. For SCS=15 kHz, where vLayers(j)⋅Qmj⋅fj=4, Rmax = 948/1024, NPRBBWj,μ=25, μ=0，OH(J)=0.14  in the equation below to ~3. While for SCS=30 kHz, assuming  NPRBBWj,μ=12, the constraint value X should be relaxed to ~3.2. eRedCap is more likely to be deployed with existing NR network. In commercial NR network, typically 30kHz SCS is used in TDD band and 15kHz SCS is used in FDD band. To avoid deployment restriction, the relaxed value should be 3.2 to ensure that both SCS cases can meet the peak data rate target.

Proposal 9: For UE peak data rated reduction supported as an add-on to UE BB bandwidth reduction, the constraint value X can be relaxed to 3.2.

# Conclusions

Based on the analysis, we have the following observations and proposals:

Observation 1: For eRedCap, 13 bits is needed for flexible Msg3 frequency resource allocation.

Observation 2: For eRedCap, 1 bit of FDRA is reserved. When frequency hopping is enabled, only 11/12 bits of FDRA can be used for Msg3 frequency resource allocation.

Proposal 1: For additional PDSCH processing time, the value of X can be 2/1 ms for 15/30kHz SCS.

Proposal 2: For UE BB bandwidth reduction, when the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, the UE shall be ready to transmit a PRACH no later than NT,1+0.75+Y msec after the last symbol of the window, or the last symbol of the PDSCH reception.

- For additional PDSCH processing time, the value of Y should be same as that of X.

Proposal 3: For UE BB bandwidth reduction, when the total number of PRBs for the two PDSCH (unicast and broadcast PDSCH) is not larger than 5MHz, the UE shall be able to decode a unicast PDSCH, and another broadcast PDSCH in non-overlapping PRBs.

Proposal 4: For UE BB bandwidth reduction, when the total number of PRBs for the two PDSCH (Msg4 and broadcast PDSCH) is larger than 5MHz,

- if the PRBs for the Msg4 PDSCH is larger than 5MHz,

the UE prioritizes to decode a broadcast PDSCH.

- otherwise,

the UE prioritizes to decode a Msg4 PDSCH.

Proposal 5: For UE BB bandwidth reduction, when the total number of PRBs for the two PDSCH (normal unicast and broadcast PDSCH) is larger than 5MHz, the UE prioritizes to decode a normal unicast PDSCH.

Proposal 6: A separate early indication by Msg1/MsgA PRACH is supported for Rel-18 eRedCap UE.

Proposal 7: For BB bandwidth reduction, maximum 13 bits in the RAR UL grant are used for Msg3 resource allocation (15KHz SCS) regardless of Msg3 FH is enabled or disabled.

Proposal 8: Confirm the working assumption with minor modifications:

Working Assumption

For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

- The UE is not required to process receive a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Proposal 9: For UE peak data rated reduction supported as an add-on to UE BB bandwidth reduction, the constraint value X can be relaxed to 3.2.

# References

- R1-2301889, FL summary #4 on Rel-18 RedCap UE complexity reduction, Moderator (Ericsson), RAN1#112

- RP- 230778, Proposal for PR1 in eRedcap, RAN#99

- R2-2301910 Report from eRedCap breakout session Session chair (Ericsson) Report