3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2302345

E-meeting, 17-26 April, 2023

Agenda Item:	9.13.1

Source:	Huawei, HiSilicon

Title:	UE capability and RRC signaling for UAV beamforming

Document for:	Discussion and Decision

- Introduction

The work item on NR support for UAV (Uncrewed Aerial Vehicles) was approved in RAN#94e [1]. The RAN1 objective in this WI is to study UE capability signaling to indicate UAV beamforming capabilities:

Study UE capability signaling to indicate UAV beamforming capabilities and, if necessary, RRC signaling [RAN1, RAN2]:

- FR1 with directional antenna at UE side

According to the WID, UAV beamforming capabilities, i.e., FR1 with directional antenna at UE side, should be studied. In order to make RAN1’s efforts count, it is beneficial to evaluate the motivation and necessity of supporting beamforming capabilities and capability signaling.

In this contribution, the availability of existing techniques under the most common event area scenario with UL interference issue is firstly justified. After that, introducing beam characteristics/directional antenna indication is analyzed. Finally, extending FR2 beam management mechanism to FR1 under Rel-17 unified TCI framework is concluded to be not preferred.

- Target scenarios and potential issues

In RAN1#110bis-e [2], the following note is agreed within each agreement,

Agreement - Note: Whether or not to specify above parameters should depend on the identification of the target scenarios and the potential issues faced by UAV, the identification of which existing capabilities and/or mechanisms and/or frameworks can be treated as candidate for addressing above issues, and the necessity of specifying above parameters (i.e., whether the potential issues faced by UAV can already be solved with the existing capabilities and/or mechanisms and/or frameworks)

which means target scenarios, potential issues faced by UAV, whether the potential issues can already be solved with the existing techniques should be justified before further investigation.

During RAN1#110bis-e and RAN1#112, one of the most common use cases companies mentioned is to address the UL interference to non-target gNBs under dense-deployed scenario (e.g., HD video surveillance in urban hotspot, live streaming from event area) as shown in Figure 1, which is caused by the line of sight propagation condition of UAV.

Figure 1 UL interference issue caused by UAV

While after analysis, we believe the aforementioned UL interference issue can already be addressed through switching to a gNB outside the densely-deployed area and conducting digital beamforming as shown in Figure 2, where the handover procedure specified in previous NR releases can be reused. In detail, a UE can be configured with measurement configuration information by RRC. Then, UE performs measurement on both the serving gNB and neighboring gNBs, and reports measurement results, e.g., RSRP, to the serving gNB. Based on the measurement results, the serving gNB can indicate the UE to switch to a gNB outside of the dense-deployed area. After handover, the new serving gNB will indicate/choose the codebook that UE used for digital beamforming, which will mitigate the aforementioned UL interference to the utmost extent.

Moreover, in order to further enlarge the handover range as well as confine the interference, the spec-transparent directional antenna architecture with narrower beamwidth can be considered. Specifically, UAV can perform spec-transparent Rx beam sweeping and report the largest RSRP corresponding to its best Rx beam during handover, which means the beamforming gain of directional antenna is already implicitly included. After handover, the UAV could use the same beam for UL transmission.

Figure 2 Addressing UL interference issue by handover and digital beamforming

Observation 1: The UL interference issue can be alleviated by switching to a gNB outside the dense-deployed area through existing switching procedure and conducting digital beamforming. A spec-transparent directional antenna architecture can be further considered to enlarge the handover range as well as confine the interference.

Based on the analysis above, it can be observed that the most common UL interference issue can already be addressed by existing techniques, while it seems that companies have not reached consensus on other specific issues under certain scenarios that need to be solved. As a result, we should first focus on identifying target scenarios and potential issues, and the new UE capability signaling is only needed if the ineffectiveness of existing techniques can be proved.

Proposal 1: The UE capability signaling is only needed if the ineffectiveness of existing techniques can be proved under the justified target scenarios with potential issues.

- Introduction of UE capability signaling for beam characteristics

In RAN1#110bis-e and RAN1#112 [3], the following agreement on UAV UE capability signaling for beam characteristics is made:

Agreement - Study indication of beam characteristics, e.g., number of beams, beamwidth, beam center, radiated EIRP, etc. as UAV UE capability - FFS: Feasibility/benefit of indicating orientation of beams including height dependence - FFS: Necessary parameters, ranges of suitable values, and method of indication - FFS: Height-dependence on relevant parameters - FFS: Indication of beams as either ‘fixed’ or ‘adaptive’ - Note: Whether or not to specify above parameters should depend on the identification of the target scenarios and the potential issues faced by UAV, the identification of which existing capabilities and/or mechanisms and/or frameworks can be treated as candidate for addressing above issues, and the necessity of specifying above parameters (i.e., whether the potential issues faced by UAV can already be solved with the existing capabilities and/or mechanisms and/or frameworks) Agreement UE UAV beamforming for FR1 based on beam switching among fixed directional antennas is supported in Rel-18. - Note 1: new UE capabilities may not be necessary to support beam switching among fixed directional antennas - Note 2: no RAN4 specification impact is assumed - FFS: whether updating (e.g. extending to FR1) legacy UE capabilities is needed, and it is not precluded if it is needed - FFS: whether/how specification may be impacted

If the target scenarios and the potential issues are justified and ineffectiveness of existing techniques are proved, introducing a directional antenna indication enabling beam switching can be treated as a candidate direction. With aforementioned indication, gNB can perform more aggressive power control strategies considering the interference mitigation effect of directional beam.

Observation 2: If existing techniques are proven ineffective, introducing a directional antenna indication can be treated as a candidate UE capability enabling beam switching.

- Introduction of UE capability signaling for beam management

In RAN1#110bis-e and RAN1#112, the following agreements on UAV UE capability signaling for beam management parameters are made:

Agreement - Study extending application of FR2-only beam management parameters e.g., spatial relation, beam correspondence, etc. to FR1 for UAV UEs - FFS: Other parameters - FFS: Impacts to legacy beam management for FR1 - FFS: Application of beam correspondence in FDM bands - Note: Identification of relevant UAV UE capabilities does not require commitment to support a specific TCI framework, and relevant parameters may change depending on the framework supported - Note: Whether or not to specify above parameters should depend on the identification of the target scenarios and the potential issues faced by UAV, the identification of which existing capabilities and/or mechanisms and/or frameworks can be treated as candidate for addressing above issues, and the necessity of specifying above parameters (i.e., whether the potential issues faced by UAV can already be solved with the existing capabilities and/or mechanisms and/or frameworks) Agreement - Study indication of minimum beam application latency as UAV UE capability - If unifiedJointTCI-r17 is supported, suitable range of values for minBeamApplicationTime-r17 - If unifiedJointTCI-r17 is not supported, enhancements to timedurationforQCL may be considered - FFS: additional parameters, e.g., beamSwitchTiming - Note: further consideration does not require commitment to support a specific TCI framework. - Note: Whether or not to specify above parameters should depend on the identification of the target scenarios and the potential issues faced by UAV, the identification of which existing capabilities and/or mechanisms and/or frameworks can be treated as candidate for addressing above issues, and the necessity of specifying above parameters (i.e., whether the potential issues faced by UAV can already be solved with the existing capabilities and/or mechanisms and/or frameworks) Agreement If new UE UAV beamforming capabilities for FR1 with directional antennas at UE side are supported, Rel-17 unified TCI framework is considered as baseline. Agreement UE UAV beamforming for FR1 based on beam switching among fixed directional antennas is supported in Rel-18. - Note 1: new UE capabilities may not be necessary to support beam switching among fixed directional antennas - Note 2: no RAN4 specification impact is assumed - FFS: whether updating (e.g. extending to FR1) legacy UE capabilities is needed, and it is not precluded if it is needed FFS: whether/how specification may be impacted

According to the agreement, if the target scenarios and the potential issues are justified and ineffectiveness of existing techniques are proved, new UAV UE beamforming capabilities can be supported and Rel-17 unified TCI framework can be considered as baseline. Then, extending FR2 beam management mechanism to FR1 under it can be treated as a candidate direction. Note that not all parameters related to unified TCI framework are required for the potential target scenario (e.g., MPE (Maximum Permissible Exposure) related parameters), here only the essential ones are selected and presented in Table 1. Furthermore, essential beam management parameters defined in Rel-15 are also presented in Table 1. Then all the parameters for FR2 only in Table 1 should be extended to FR1.

Table 1 Summary of essential parameters

| UE capability parameters | FR1-FR2 DIFF |
| --- | --- |
| minBeamApplicationTime-r17 | FR2 only |
| unifiedJointTCI-BeamAlignDLRS-r17 | FR2 only |
| timeDurationForQCL | FR2 only |
| maxNumberRxTxBeamSwitchDL | FR2 only |
| beamSwitchTiming | FR2 only |
| beamCorrespondenceWithoutUL-BeamSweeping | FR2 only |
| uplinkBeamManagement | FR2 only |
| unifiedJointTCI-r17 | N/A |
| unifiedJointTCI-Legacy-SRS-r17 | N/A |
| unifiedJointTCI-multiMAC-CE-r17 | N/A |
| unifiedJointTCI-InterCell-r17 | N/A |
| unifiedSeparateTCI-r17 | N/A |
| unifiedSeparateTCI-InterCell-r17 | N/A |
| unifiedSeparateTCI-multiMAC-CE-r17 | N/A |

Considering the unclear benefit over introducing directional antenna indication as well as the non-negligible spec. effort, extending application of FR2-only beam management parameters to FR1 is not preferred.

Proposal 2: Considering the unclear benefit over introducing directional antenna indication as well as the non-negligible spec. effort, extending application of FR2-only beam management parameters to FR1 is not preferred.

- Conclusions

In this contribution, the availability of existing techniques under the most common event area scenario with UL interference issue is justified, and introducing beam characteristics/directional antenna indication is analyzed. Furthermore, extending FR2 beam management mechanism to FR1 under Rel-17 unified TCI framework is concluded to be not preferred. The following observations and proposal are made:

Observation 1: The UL interference issue can be alleviated by switching to a gNB outside the dense-deployed area through existing switching procedure and conducting digital beamforming. A spec-transparent directional antenna architecture can be further considered to enlarge the handover range as well as confine the interference.

Observation 2: If existing techniques are proven ineffective, introducing a directional antenna indication can be treated as a candidate UE capability enabling beam switching.

Proposal 1: The UE capability signaling is only needed if the ineffectiveness of existing techniques can be proved under the justified target scenarios with potential issues.

Proposal 2: Considering the unclear benefit over introducing directional antenna indication as well as the non-negligible spec. effort, extending application of FR2-only beam management parameters to FR1 is not preferred.

References

- RP-213600, “New WID on NR support for UAV (Uncrewed Aerial Vehicles)”, Nokia, RAN#94e, December 06 - 17, 2021.

- R1-2210312, “Summary#3 of discussion on UAV capability and RRC signaling for UAV”, Nokia, RAN WG1 #110bis-e, October 10 - 19, 2022.

- R1- 2301955, “Summary#3 of discussion on UAV capability and RRC signaling for UAV”, Nokia, RAN WG1 #112, February 27th –March 3rd, 2023.