3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2302351

e-Meeting, April 17th –26th, 2023

Agenda Item:	9.12.2

Source:	Huawei, HiSilicon

Title:	Discussion on coverage enhancement in power domain

Document for:	Discussion and Decision

- Introduction

In RAN#112 [1], some agreements were approved to study frequency domain spectrum shaping (FDSS) with spectrum extension (FDSS-SE) and tone reservation (TR) further, including performance of FDSS-SE under different filters and different extension ratios, extension methods of data and DMRS, and receiver operations, to improve coverage performance. In this contribution we show that for given spectral efficiency, the coverage performance of discrete Fourier transform spread orthogonal frequency-domain multiplexing (DFT-s-OFDM) signal can be improved by FDSS-SE and TR.

Specifically, SE provides power improvement due to reduction of the peak-to-average-power ratio (PAPR) and cubic metric (CM). However, if SE uses resource elements (REs) which otherwise could have been used for data transmission, the code rate increases and thus results in a larger required signal-to-noise ratio (SNR) to achieve desired decoding performance. This makes the optimal SE ratio for coverage enhancement gain not only vary with the FDSS filter but also vary with modulation and coding scheme (MCS) and resource block (RB) allocation. And, the DMRS and receiver operation are studied by performance evaluation. Moreover, the signaling and configurations to enable FDSS-SE and TR are also discussed.

A set of FFS points were listed in the agreements of RAN1#112, which we will address in this contribution:

- Which extensions factor(s) to consider, where extension factor (α) is given by spectrum extension size / Total allocation size.

- Impact of shaping filter on FDSS-SE performance

- How to extend data to spectrum extensions

- How to extend DMRS sequence to spectrum extensions, based on either the existing low PAPR sequence type 1, low PAPR sequence type 2 and enhanced low PAPR sequence type 1 through DFT transformation

- How extension size, RB allocation, transport block, and power allocation is determined

- Coverage enhancement by FDSS with SE

  - Performance comparison between FDSS with SE and FDSS

In RAN#112[1], the following non-transparent candidate solution for MPR/PAR reduction is listed:

- Frequency domain spectrum shaping with spectrum extension (FDSS-SE)

FDSS without spectrum extension can be a transparent scheme which is used as a benchmark. In this section, the performance of FDSS-SE where both π/2-BPSK and QPSK modulation are considered is given.

Besides, according to the working assumption in [1], several FDSS filters are mentioned for calibration:

- 3-tap, Pulse shaping filter with (-0.28 1 -0.28)

- Truncated RRC with parameters (0.5, 0.1667)

The resulting FDSS window from 3-tap filter is:

Wk=m=02fme-j2πNSCkm, 0≤k≤NSC, f=-0.28,1,-0.28            (1)

where NSC means the number of subcarriers. The resulting FDSS window form TRRC (p, q) is defined in [9]. One can see that truncated RRC with parameters (0.5, 0.1667) is sub-optimal for 1/4 extension factor. To verify net gain under other extension factor in section 2.1.4, e.g. 1/9, we also find the suboptimal filters for extension factors of 1/9 by computer search:

TRRC (0.56, 0.47) for 1/9 extension factor

These FDSS filters fulfill the existing RAN4 EVM spectral flatness requirement for Nsc=96 subcarriers[2].

    - Performance evaluation of π/2-BPSK using FDSS with SE

The differences in CM at the 1% of the CDF for π/2-BPSK are contained in Table 1 (for 4 RBs) and Table 9 (for 8 RBs) in Appendix. The baseline used here is π/2-BPSK modulated DFT-s-OFDM without FDSS. π/2-BPSK modulated symbols with FDSS-SE provides only up to 0.2 dB of CM performance gain compared with π/2-BPSK modulated symbols with FDSS. As shown in [2], compared π/2-BPSK modulated DFT-s-OFDM with FDSS w/o extension, π/2-BPSK modulated DFT-s-OFDM with FDSS w/ extension has almost no extra MPR gain. While, spectrum extension could result in severe SNR degradation when the code rate is medium or high, hence, the net gain of π/2-BPSK with FDSS-SE is small even negative compared with π/2-BPSK with FDSS. It is unnecessary to adopt SE for π/2-BPSK modulation.

Table 1: ∆CM@1e-2 of π/2-BPSK with FDSS-SE or FDSS, compared π/2-BPSK with DFT-s-OFDM without FDSS, under 4 RB allocations.

| FDSS SE ratio | 0% | 12.5% | 25% | 37.5% |
| --- | --- | --- | --- | --- |
| TRRC (0.5, 0.1667) | 1.13 | 1.24 | 1.15 | 0.46 |
| 3 tap (-0.28, 1, -0.28) | 1.40 | 1.47 | 1.38 | 0.76 |

Observation 1: FDSS with SE provides small gains for π/2-BPSK compared to π/2-BPSK with FDSS w/o SE.

Proposal 1: The π/2-BPSK using FDSS with SE is not supported.

    - Performance comparison of QPSK using FDSS with and w/o SE

To evaluate the practical transceiver performance, net gain can be defined as the sum of the SNR degradation under the given BLER requirement and the value of improved transmission power measured by MPR, for brevity. This definition of gain is given as

| G=∆MPR+∆SNR | (2) |
| --- | --- |

where G is the net gain, ∆MPR is the improvement of transmission power measured by MPR, ∆SNR is the SNR degradation under the given BLER requirement. Throughout this work, unless otherwise noted, we take the BLER = 10-1 as the default BLER requirement. For performance calibration, some working assumptions are proposed in [1]:

Table 2: simulation parameters in agreement of RAN1 112b

| No spectrum extension |  |  | With spectrum extension |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| #case | #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor |
| 1 | 8 | 0 [only QPSK] | 6 | 8 | 1 [only QPSK] | ¼ |
| 2 | 8 | 6 | 6 | 8 | 8 | ¼ |
| 3 | 40 | 2 | 30 | 40 | 3 | ¼ |
| 4 | 40 | 6 | 30 | 40 | 8 | ¼ |
| 5 | [6 | 3 | 4 | 6 | 5 | 1/3] |
| 6 | [36 | 7 | 32 | 36 | 8 | 1/9] |

Figure 1 and 2 show the net gain of case 3/4 in table under different FDSS filters, in comparison to QPSK without FDSS. Other simulation parameters can be found in table 13. One can see that the gain of inband MPR of QPSK under FDSS filters without extension is extremely close to zero, or even negative, while the gains of in-band MPR of QPSK under FDSS filters with extension are 0.8-2dB. It implies that spectrum extension is necessary for QPSK with FDSS filters to achieve improvement gain.

Figure 1: Net gain for case 3 under different FDSS filters and 1/4 extension ratio, in comparison to QPSK without FDSS

Figure 2: Net gain for case 4 under different FDSS filters and 1/4 extension ratio, in comparison to QPSK without FDSS

Observation 2: The net gains of QPSK under FDSS w/o SE are extremely close to zero, or even negative. While the net gains of QPSK under FDSS with extension are significant,0.8-2dB. FDSS should be combined with SE for QPSK to achieve significant coverage gain.

    - MRC receiver performance

Unlike the MRC receiver, the basic receiver directly discards the extended data portion. As mentioned in [1] [2], the MRC receiver has a BLER gain of about 0.4 dB compared to the basic receiver. The gain of MRC is dependent on FDSS type and extension ratio. In this section, more simulation results are shown in Table 3 (for 4 RBs), Table 11 (for 8 RBs), and Table 12 (for 32 RBs) in Appendix. The simulation parameters used in Table 3 are given in Table 10, the simulation parameters used in Table 11 and Table 12 are given in Table 13. With given filter, the gain of MRC increases with the increase of RB number, extension ratio, and code rate. A maximum of 1 dB extra power is required to achieve the same demodulation performance for basic receiver. The MRC gain comes from the improvement of demodulation performance due to the increase of power and resource element for data transmission.

Table 3: SNR degradation of basic receiver compared to MRC receiver under 4 RB allocations.

| SE ratio Code rate | TRRC (0.5,0.1667) +1/8 | TRRC (0.5,0.1667) +1/4 | TRRC (0.5,0.1667) +3/8 | 3tap (-0.28,1, -0.28) +1/8 | 3tap (-0.28,1, -0.28) +1/8 | 3tap (-0.28,1, -0.28) +3/8 |
| --- | --- | --- | --- | --- | --- | --- |
| 1/8 | -0.11 | -0.35 | -0.72 | -0.17 | -0.33 | -0.47 |
| 1/4 | -0.25 | -0.40 | -0.73 | -0.51 | -0.43 | -0.53 |
| 1/2 | -0.25 | -0.43 | -0.73 | -0.33 | -0.53 | -0.57 |

Observation 3: With given filter, the gain of MRC increases with the increase of RB number, extension ratio, and code rate, and up to 1 dB improvement of SNR degradation for the same demodulation performance is observed compared to basic receiver.

Proposal 2: MRC receiver should be supported when FDSS and SE is performed.

    - Performance of different SE ratios

In Figure 3, we plot the net gain of QPSK with different FDSS filters for different RB allocations with length of 36 RBs and different MCS. The simulation parameters are given in Table 10. One can see the result that for most edge-band RB allocations, the best extension ratio maximizing the net gain is 1/4, while for inner-band RB allocations, when MCS increases from 0 to 4 and 7, the best extension ratio changes from 1/4 to 1/9, and extension ratio 1/9 shows a significant performance improvement for MCS = 7. The gain gap of extension ratio 1/9 compared to 1/4 becoming larger with the increase of the spectral efficiency is due to the difference of SNR degradation and the stability of MPR performance, which validates the superiority of small extension ratios for medium spectral efficiency. The best extension ratio maximizing the net gain varies for different MCS and positions of RB allocation.

(a) MCS=0                  (b) MCS=4                    (c) MCS=7

Figure 3: Net gain of QPSK with different FDSS filters, SE ratios, RB allocation and MCS with length of 36 RBs

Observation 4: The best extension ratio maximizing net gain for QPSK is different at least for different MCS and positions of RB allocation:

- for inner-band RB allocations, when MCS index exceeds 4, extension ratio 1/9 shows a performance improvement of 0.5 dB at most compared to extension ratio 1/4.

Thus, extension ratios 1/4 and 1/9 should be supported to achieve the best performance for different MCS indexes and RB allocations.

Proposal 3: Two spectrum extension ratios should be supported which are 1/4 and 1/9.

  - Performance comparison of spectrum extension methods

Three extension methods have been agreed for studying:

•	Option 1: Symmetric extension

•	Option 2: Cyclic extension

•	Option 3: Cyclic shift plus symmetric extension.

In fact Option 1 and Option 2 are special cases of Option 3 with specific cyclic shift. Option 3 is proposed in order to optimize the cyclic shift as a function of the number of REs used for data, Ndata, and the number REs used for SE, Ne. We found that for QPSK selecting L= Ndata8-Ne-12, where ⋅ is a rounding operation to the closest integer, provides consistently the best PAPR and CM reduction, which is explained below.

Option 3 in its general formulation is given as

| Xsek=X[k+L mod Ndata], k∈{0,1,…Nsc-1} | (3) |
| --- | --- |

where X is a sequence of frequency domain symbols, and Xse its spectrally-extended version.

This formulation uses a general shift parameter L encompassing the other options which correspond to specific values of L:

- Option 1 is obtained by selecting L=Ndata-Ne/2. The SE becomes:

| X(se)=XNdata-Ne2,…,XNdata-1, X0,…,XNdata-1, X0, …,XNe2-1 | (4) |
| --- | --- |

- Option 2 is obtained by selecting L=0. The SE is then specifically

| X(se)=X0,…,XNdata-1, X0,…,XNe-1 | (5) |
| --- | --- |

Observation 5: Option 1 and Option 2 are special cases of Option 3 with specific cyclic shift L.

For any value of L, the original sequence of Fourier coefficient X0,…, XNdata-1 as shown in Figure 4 is always included in the in-band spectrum up to a cyclic-shift by L+Ne2 symbols; and the left-side excess-band symbols are always the repetition of symbols of the right-side in-band edge; and similarly for the right-side excess band.

For all options, which corresponds to different shift values L, the definition of SE needs to be known at the receiver. Beside this, it does not change the requirement of the receiver, except of performing the corresponding inverse cyclic shift.

Observation 6: The choice of L does not change the set of symbols in the in-band and thus does not change the receiver requirement.

Figure 4: Illustration of spectrum-extended data sequence as a function of the shift parameter L with Ndata=10 symbols and Ne=4.

In fact, any option can be equivalently be implemented as a cyclic extension or symmetric extension:

- Cyclic extension implementation of (1): cyclically shifted X[k] by L then perform cylic extension.

- Symmetric extension implementation of (1): cyclically shifted X[k] by L+Ne2 symbols and then perform symmetrical extension.

Observation 7: All options can either be implemented as a cyclic extension or symmetric extension.

While the definition of SE, i.e. the value of L, does not change the receiver requirement, it does change the PAPR of the received signal. This is because it changes the phase differences among the DFT-s-OFDM pulses. Indeed, it can be shown that with DFT-s-OFDM using FDSS-SE, the constellation symbols are multiplexed by the set of pulses gmnm=0Ndata-1 where

| gmn=hn-NfftNdatame-j2πLNdatam | (6) |
| --- | --- |

with the pulse-shaping filter hn=k=0Nsc-1Wkej2πkNfftn

Using Lemma 1 of [6] the phase difference between two neighboring pulses can be shown to be

| gm[n]gm+1[n]=-2πNdataL+Ne-12 (mod π) | (7) |
| --- | --- |

This phase difference influence thee coherent combining of pulses and thus the overall fluctuation of the signal’s envelope.

With Option 1 (L=Ndata-Ne/2) and Option 2 (L=0) , the phase differences become -2πNe-1Ndata and gm[n]gm+1[n]=-πNe-1Ndata , respectively. This means that with these options the phase difference between neighboring pulses depends of the extension size and is uncontrolled.

With Option 3 and L= Ndata8-Ne-12, we instead have gm[n]gm+1[n]≈-π4. In this case the phase difference between neighboring pulses is maintained constant. As a result, consecutive QPSK symbols will always be transmitted with an absolute phase difference of at least π4, and the signal’s envelope fluctuation would be minimized.

In Figure 5 and Figure 6, Option 1-3 are compared in term of PAPR and CM for QPSK with FDSS and SE. A total of 8 and 24 RBs for total bandwidth is considered, with different RBs for SE. As it can be seen, the performance difference between these three options can change depending on the configurations. Nevertheless, it can be remarked that Option 1 is always the worst, and Option 3 with an optimized cyclic shift parameter of L= Ndata8-Ne-12 provides consistently the best result.

Observation 8: For FDSS with SE for QPSK, Option 1 always provides the worst performance PAPR and CM, while Option 3 with an optimized shift L= Ndata8+Ne-12 always provides the best PAPR and CM reduction.

(a) PAPR                                   (b) CM

Figure 5: PAPR and CM for 8 RBs total bandwidth allocation with 0 to 3 RBs for SE.

(a) PAPR                                   (b) CM

Figure 6: PAPR and CM for 24 RBs total bandwidth allocation with 0 to 4 RBs for SE.

For practical performance comparison between option 2 and option 3, as shown in Figure 7 and 8, cyclic shift plus symmetric extension (Option 3) has about MPR gain of 0.1-0.2dB compared to symmetric extension (Option 2), which is consistent with the results of the CM.

Figure 7: MPR comparison between Option 2 and Option 3 using TRRC (0.5,0.1667) and 1/4 extension

Figure 8: MPR comparison between Option 2 and Option 3 using TRRC (0.5,0.1667) and 1/3 extension

Moreover, the Option 1, 2, and 3 all have to copy a portion of initial DFT coefficients with the same length to obtain extended DFT coefficients, except that the location of copied portion is different. Thus, the implementation complexities of Option 1, 2, and 3 are similar and do not have significant difference.

Proposal 4: Cyclic shift plus symmetric extension should be supported.

  - DMRS scheme and performance evaluation

    - DMRS scheme and signal processing description

The generation of low PAPR sequence type 1 and low PAPR sequence type 2 are specified in [3]. Specifically, when DMRS length is larger than 24, ZC sequence is used as low PAPR sequence type 1 and pi/2-BPSK modulated gold sequence is used as low PAPR sequence type 2. When DMRS length is 6/12/18/24, QPSK modulated CGS (computer generated sequence) is used as low PAPR sequence type 1, pi/2-BPSK modulated CGS is used as low PAPR sequence type 2 when DMRS length is 12/18/24 and 8PSK modulated CGS is used as low PAPR sequence type 2 when DMRS length is 6.

In RAN#112[1], the following DMRS solutions are proposed:

When the DMRS sequence length before extension of the sequence, if any, is larger than or equal to 30:

Approach A1: Type 1 DMRS is generated considering the number of PRBs in the inband and extension is performed.

In particular, for ZC sequence, x(n) is generated for length Nzc(which is the largest prime number and less than M1, where M1 is the inband+extension length), the final DMRS sequence is generated as follows:

xextn=x(n mod Nzc)                            (8)

Approach A2: Type 2 DMRS is generated considering the number of PRBs in the inband and extension is performed.

Approach B1: Type 1 DMRS is generated considering the number of PRBs in the inband + extension.

Approach B2: Type 2 DMRS is generated considering the number of PRBs in the inband + extension.

When the DMRS sequence length before extension of the sequence, if any, is less than 30:

Approach A1: The sequence is obtained by DFT transformation of an existing DMRS sequence, e.g., Type 1 DMRS sequence considering the number of PRBs in the inband and extension is performed.

Approach A2: Type 1/2 DMRS is generated considering the number of PRBs in the inband and extension is performed.

Approach B: Type 1/2 DMRS is generated considering the number of PRBs in the inband + extension.

As discussed in [2], extension method including the following two options:

Option b: A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. The sequence is then cyclically extended to span the PRBs in the extension.

Assume the inband length is M2, extended length is d=M1-M2. In option 1-c/2-c, initial DMRS sequence of length M2 is generated, then the final DMRS sequence is generated as follows:
                              xextn=x(n-d2mod M2)                          (9)

Option c: DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. DMRS extension is applied similar to data to span the PRBs in the extension.

It should be noted that option b and option c are the same except for ZC sequence in approach A1. For ZC sequence, in option b, ZC sequence x(n) of length Nzc is generated., one extended part will have a size of d2, while the other will have a size of d2+M2-Nzc.

    - Comparison of extended ZC and low PAPR sequence

The PAPR/CM of the DMRS sequence needs to be less than or equal to the PAPR/CM of the data symbol. When inband length is larger than or equal to 30, as shown in [7], PAPR of pi/2-BPSK modulated data is already less than QPSK modulated data with FDSS w/ spectrum extension. it seems that there is no need to extend Type 2 DMRS sequence. However, DFT transformation is used during the generation of Type 2 DMRS sequence, where DFT size should be a multiple of 2a* 3b* 5c, where [a, b, c] are non-negative integers. The inband+extension length may not meet the requirement of 2a* 3b* 5c, if spectrum extension is not used for Type 2 DMRS sequence, the sequence length should be truncated to be a multiple of 2a* 3b* 5c, which will affect the channel estimation accuracy. Thus, approach A2 is suitable for Type 2 DMRS sequence.

As shown in Figure 9, approach A1 with option c will break the symmetrical properties, which results in high CM value. Thus, approach A1 with option b is the only available choice for ZC sequence to meet the requirements of the PAPR/CM for Type 2 DMRS sequence.

Figure 9: CM performance of Type 1 DMRS sequence with different approaches and options

Observation 9: When inband length is larger than or equals to 30, approach A1 with option b and approach A2 with option b are available considering the requirement of PAPR/CM and DFT size limit.

In addition to PAPR/CM performance, channel estimation accuracy is also important, which affects data demodulation performance. As shown in figure 10/22/23/24/25, net gain of case 1/2/3/4/6 with approach A1 and A2 is given. Simulation parameters can be found in Table 13. Note that only option b is performed for approach A1 and A2. When FDSS w/o extension is performed, approach B1 is sufficient considering both PAPR/CM and channel estimation accuracy. One can see that data with approach A1 outperforms data with approach A2, about 1-3dB gap could be found. As shown in Table 4 and Figure 11 The performance difference is mainly caused by the channel estimation precision of the two DMRS solutions. The larger the MCS, the smaller the net gain difference. This is mainly because the SNR operating point increases with the increase of the MCS, thereby improving the channel estimation precision.

Figure 10: net gain of case 1 with different DMRS solutions

Table 4: SNR degradation[dB] of case 1 with different FDSS and DMRS solutions

|  | ∆SNR@BLER=0.1 |
| --- | --- |
| 8RB, MCS0, TRRC (0.5,0.1667) | -0.4 |
| 8RB, MCS0, 3tap (-0.28,1, -0.28) | -0.8 |
| 8RB, MCS1, TRRC (0.5,0.1667) +1/4 extension, approach A1 | -0.3 |
| 8RB, MCS1, TRRC (0.5,0.1667) +1/4 extension, approach A2 | -2.6 |
| 8RB, MCS1, 3tap (-0.28,1, -0.28) +1/4 extension, approach A1 | -0.5 |
| 8RB, MCS1, 3tap (-0.28,1, -0.28) +1/4 extension, approach A2 | -2.8 |

Figure 11: channel estimation accuracy comparison of approach A1 and A2 in case 1

Observation 10: Approach A1 with option b has better demodulation performance than approach A2 with option b, 1-3 dB gap can be observed.

In order to get a comprehensive view of all the long sequence approaches, the following table lists the advantages and disadvantages of all the approaches. It should be note that approach A1 and approach B1 use ZC sequences with constant amplitude, so their channel estimation performance is optimal. One can see that approach A1 with option b is the most appropriate.

Table 5: summary for long DMRS sequence approaches

|  | Fulfill PAPR/CM requirement | Total PRB size restricted by existing DFT size limitation | Demodulation performance loss compared to approach A1 with option b |
| --- | --- | --- | --- |
| Approach A1 with option b | YES | NO | NO |
| Approach A1 with option c | NO | NO | NO |
| Approach A2 with option b/c | YES | YES | YES, 1-3dB gap |
| Approach B1 | NO | NO | NO |
| Approach B2 | YES | NO | YES, 1-3dB gap |

Proposal 5: When inband length is larger than or equal to 30, adopt approach A1 with option b, where A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. The sequence is then cyclically extended to span the PRBs in the extension.

    - Comparison of short DMRS sequence

When inband DMRS length is less than 30, Type 1 DMRS is generated by QPSK modulated sequence, FDSS or FDSS w/ extension will destroy the low PAPR characteristic. In Figure 12, we plot the CM performance of Type 1 DMRS with FDSS and FDSS w/ extension. To estimate channel information more accurately, DMRS and data need to pass through the same FDSS. One can see that when the inband+extension length equals to 18, no matter whether a 3-tap filter or a 3-tap filter with 1/3 extension is used, the CM of Type 1 DMRS increases after FDSS or FDSS w/ extension is performed.

Figure 12: CM performance of Type 1 DMRS with FDSS and FDSS w/ extension.

When inband length is less than 24, Type 2 DMRS is generated based on 8PSK (when length equals to 6) modulated sequence and pi/2-BPSK (when length is larger than 6) modulated sequence. As shown in Figure 13, one can see both approach A2 and approach B outperforms data with FDSS w/ extension.

Figure 13: CM performance of low PAPR sequence type 2 with FDSS and FDSS w/ extension

In Figure 14, we plot the CM performance of approach A1 with FDSS and FDSS w/ extension. One can see that their CM performance using FDSS w/ extension is very close. In fact, when inband length is 6/12/18/24, the low PAPR sequence type 1 is QPSK modulated sequence in frequency domain, DFT transformation makes the enhanced low PAPR sequence type 1 undergoes the same waveform of data, which result in a similar CM performance. Compared to Type 1 DMRS, approach A1 only need DFT transformation which has minor protocol change. Besides, because DMRS sequences can be generated in advance, DFT transformation does not increase computational complexity.

Figure 14: CM performance of approach A1 with FDSS and FDSS w/ extension

Observation 11: When inband length is less than 30, approach A1 with option b, approach A2(Type 2 DMRS) with option b and approach B (Type 2 DMRS) are available considering the requirement of PAPR/CM.

Comparison of Demodulation performance of candidate sequences

In Figure 15, net gain of case 5 with approach A1 and A2(Type 2 DMRS) is plotted. Simulation parameters can be found in Table 13. When FDSS w/o extension is performed, approach B (Type 1 DMRS) is sufficient considering both PAPR/CM and channel estimation accuracy. One can see that data with approach A1 outperforms data with approach A2(Type 2 DMRS), about 0.3dB gap could be found. As shown in Figure 16, channel estimation accuracy of approach A2(Type 2 DMRS) is slightly worse than that of approach A1, which further corroborates the simulation results in Figure 15.

Figure 15: net gain of case 5 with different DMRS solutions

Figure 16: channel estimation accuracy comparison of approach A1 and A2 in case 5

Besides, some short DMRS sequences have zero elements, and special processing must be performed on the receiver side, for example, the channel estimation value on the zero element is obtained by interpolation by using the channel estimation value on the element adjacent to the zero element. In Table 6, the number of sequences containing zero elements in approach A1 and A2(Type 2 DMRS) of different lengths have been counted. It can be found that the number of zero elements in approach A2(Type 2 DMRS) is much greater than that in approach A1.

Table 6: number of sequences containing zero elements in approach A1 and approach A2

| Inband length | Approach A1 | Approach A2 |
| --- | --- | --- |
| 6 | 3 | 1 |
| 12 | 4 | 10 |
| 18 | 1 | 0 |
| 24 | 1 | 7 |

when inband+extension length is less than 30, considering spectrum extension basically does not change the flatness of the sequence in frequency domain, approach B(Type 2 DMRS) has the same demodulation performance with approach A2(Type 2 DMRS). When inband+extension length is larger than 30, pi/2-BPSK modulated gold sequences have very poor demodulation performance as mentioned before. Thus, approach A1 has better demodulation performance than approach A2(Type 2 DMRS) and approach B (Type 2 DMRS).

Observation 12: Approach A1 has better demodulation performance compared to approach A2(Type 2 DMRS) and approach B (Type 2 DMRS), 0.3 dB gain can be observed.

Comparison of correlation performance of candidate sequences

In addition to the channel estimation performance, the cross-correlation performance between sequences is also important, which determines the interference level between different cells. Assume two sequence a=(a0,a1,…,aN-1) and b=(b0,b1,…,bN-1), the cross-correlation value of these two sequences are:

C1=maxτ∈[0,1]n=0N-1an*bn*e-nτ∙2πj(n=0N-1an2)12*(n=0N-1bn2)12                   (10)

The larger the C1, the larger the interference level. Figure 17 gives the CDF of cross-correlation values of the above three approaches. One can see that cross-correlation performance of approach A1 and approach B (Type 2 DMRS) are the best.

Figure 17: Cross-correlation performance of three candidate sequences

The cross-correlation performance determines the DMRS interference level between different cells, and the auto-correlation performance determines the interference level of multi-user code division multiplexing in the same cell.  Assume a=(a0,a1,…,aN-1), b=(w0*a0,w1*a1,…,wN-1*aN-1),where wi=1,if i is even-1, else presents the orthogonal cover code, the auto-correlation value of a is calculated as follows:

C2=maxτ∈[0,0.1]n=0N-1an*bn*e-nτ∙2πj(n=0N-1an2)12*(n=0N-1bn2)12                     (11)

The larger the C2, the larger the interference level. Figure 18 gives the CDF of autocorrelation values of the above three DMRS solutions. The autocorrelation performance of approach A1 and approach A2(Type 2 DMRS) is the best among the three approaches. But one can see that there exists some sequence in approach A2(Type 2 DMRS) with poor autocorrelation performance, which could cause pilot pollution, then reduces the system throughput.

Figure 18: auto-correlation performance of three candidate sequences

Observation 13: Approach A1 has better cross-correlation and autocorrelation performance than approach A2（Type2 DMRS）and better auto-correlation performance than approach B (Type 2 DMRS)

In order to get a comprehensive view of all the short sequence approaches, the following table lists the advantages and disadvantages of all the approaches. It should be noted that demodulation and correlation performance of approach A1 is chosen as baseline as it has the optimal demodulation and correlation performance among the three approaches above. The demodulation performance and correlation performance are not compared for approaches that do not meet the PAPR/CM requirements. One can see that approach A1 with option b/c is the most appropriate.

Table 7: summary for short DMRS sequence approaches

|  | Fulfill PAPR/CM requirement | Total PRB size restricted by existing DFT size limitation | Demodulation performance loss compared to approach A1 with option b/c | Cross-correlation performance compared to approach A1 with option b/c | Auto-correlation performance compared to approach A1 with option b/c |
| --- | --- | --- | --- | --- | --- |
| Approach A1 with option b/c | YES | NO | NO | Same | Same |
| Approach A2 (Type 1 DMRS) with option b/c | NO | NO |  |  |  |
| Approach A2 (Type 2 DMRS) with option b/c | YES | NO | YES, 0.3 dB gap | Worse | Worse |
| Approach B (Type 1 DMRS) | NO | NO |  |  |  |
| Approach B (Type 2 DMRS) | YES | YES | YES, 0.3 dB gap | Similar | Worse |

Proposal 6: When inband length is less than 30, adopt approach A1 with option b/c.

  - Signaling and configurations to enable FDSS with SE

Different UEs may have different FDSS capability, including supporting FDSS or not. To enable scheduling of a UE with FDSS and SE, the information of UE capability should be reported to the gNB. Moreover, to decode the received signal, the SE ratio, the SE shift and possibly also the FDSS filter needs to be known by the gNB.

The results herein have shown that the coverage enhancement gain depends on the FDSS filter. If FDSS filters are specified, parameters such as SE ratio and SE shift could either be pre-defined or be signaled from the gNB to the UE. However, if the FDSS filter is not specified, the gNB cannot determine the best values of SE ratio or SE shift, and could therefore not guarantee that there is any coverage enhancement gain. In that case, the UE needs to signal to the gNB some preferred values of, e.g., the SE ratio or SE shift, or its FDSS filter.

By having pre-defined values of SE ratio and SE shift no signaling is required, but on the other hand, the values may depend on MCS, PRB allocation etc, resulting in plenty of combinations. For the best performance improvement, it is suitable for UEs to use different filters, SE ratios and SE shifting. Thus, it may be beneficial to let gNB to dynamically indicate the filter and its SE ratio by jointly considering UE’s FDSS capability, system resource usage, channel state information, and transmission configuration.

Observation 14: The UE needs to report the FDSS capability to gNB.

Observation 15: Additional information such as SE ratio, SE shift and FDSS filter can either be pre-defined or be dynamically signaled from the gNB to the UE. If FDSS filter is not specified, the UE may also need to signal preferred values of SE ratio and SE shift to the gNB, or its FDSS filter.

    - Resource block indication

- RB allocation indication signaling

In terms of frequency resource indication signaling, it is important to minimize the specification impact by preserving legacy operations as much as possible. Based on this motivation, there are mainly two methods to indicate the resource allocation and usage in frequency domain.

The first method is that indicate the RB allocation of non-extension spectrum Ballocation,1 and SE ratio α at the same time, as illustrated in Figure 19, where UEs can find the location of extension spectrum by pre-defined spectrum extension method and the computed extension spectrum length.

Figure 19: Illustration of first method regarding frequency resource allocation indication and usage.

The second method is that indicate the RB allocation of total spectrum after extension (non-extension spectrum and extension spectrum) Ballocation,2 and SE ratio α at the same time, as illustrated in Figure 20, where UEs can find the location of non-extension spectrum and extension spectrum by pre-defined spectrum extension method and the computed non-extension spectrum length.

Figure 20: Illustration of second method regarding frequency resource allocation indication and usage.

Both methods are based on the legacy frequency resource indication signaling and a new SE ratio indication signaling, and rely on computation results between these two signaled values.

- RB allocation usage in data signal generation

Although, there are the similar signaling cost and computing complexity between above two methods, their implementation complexity to determine the RB of non-extension spectrum and extension spectrum are different due to following specifications.

- First, the number DFT precoded symbols is required to be a multiple of 2a* 3b* 5c, where [a, b, c] are non-negative integers.

- Second, the number of RBs used in the formula of computing transport block size is restricted to be an integer.

Based on above restrictions, the number of RBs of non-extension spectrum should be an integer that can be expressed as multiple of 2a* 3b* 5c, where [a, b, c] are integers ≥ 0.

Observation 16: In the first method, the number of allocated RB should be the multiple of 2a* 3b* 5c, where [a, b, c] are integers ≥ 0. In the second method, the number of allocated RB whether could guarantee that the number of RB for non-extension spectrum is the multiple of 2a* 3b* 5c, is closely associated with the using SE ratio. Moreover, it seems no need to restrict the starting RB of the allocation.

- Rounding operation

In the first method, BS can indicate the suitable number of RB for non-extension spectrum Lallocation,1 directly but the total RB could require extra operation, such as

- UEs obtain the number of RB for total spectrum by ceiling the computed value towards the next full RB, where the computed value equals to the quotient between allocated RB and (1-α), i.e., Lallocation,11-α.

In the second method, BS can indicate the suitable number of total RB to guarantee the computed number of RB for non-extension spectrum to satisfy above restrictions. Otherwise, UEs perform following operation:

- UEs take the largest integer that can be expressed as multiple of 2a* 3b* 5c and is below the computed value as the number of RB for non-extension spectrum, where [a, b, c] are integers ≥ 0 and the computed value equals to the product between allocated RB and (1-α), i.e., (1-α)Lallocation,2.

- UEs take the integer that can be expressed as multiple of 2a* 3b* 5c and is closest to the computed value as the number of RB for non-extension spectrum, where [a, b, c] are integers ≥ 0 and the computed value equals to the product between allocated RB and (1-α), i.e., (1-α)Lallocation,2.

Comparing above two methods, one can find that the first method has a less complexity than the second method.

Observation 17: RB allocation of non-extension spectrum and spectrum extension ratio is more suitable than the second method due to less complexity.

- TBS calculation

With the number of non-extension spectrum RBs and the indicated MCS, UE can compute the transport block size directly, where the MCS is indicated from the standard MCS tables. Then, after channel coding, modulation, and DFT operation, UE can readily extend the DFT coefficients of modulated data into all REs of total spectrum after extension and further process it to generate signal for transmission.

Proposal 7: The gNB should indicate the RB allocation of non-extension spectrum, spectrum extension ratio, MCS index to UEs, based on which the UEs calculate the size of transport block.

    - Power control

In DFT-s-OFDM with FDSS and SE, the allocated RBs are divided into two parts, termed as data spectrum and extended spectrum. Considering filter shape and spectrum operation, the power density of REs of data spectrum and extended spectrum would be changed, which is quite different from that of DFT-s-OFDM. On the whole, transmission power is divided into two significantly unequal parts to be used by data spectrum and extended spectrum, whose power ratio is determined by filter coefficients and SE ratio. Specifically, the power density of REs cannot be regarded as the random variables with the same expectation since the used filter makes the probability distribution function (PDF) of power density on different REs significantly different, especially for edge and in-band REs. As a result, the average of power density of REs is different.

Observation 18: The power density of RE has been changed by FDSS-SE.

Moreover, the extension spectrum only can be used by copy data, which only provide diversity gain and power gain but lose multiplexing gain. The significant average power difference on REs and data copy restriction can affect the channel capacity, as the restrictions due to filter shaping and extended spectrum usage have changed the characteristic of channel. Consequently, relationship between throughput and transmission power is different from that without power restriction, especially for sharper filters, larger SE ratio and larger RB length. It degrades the accuracy of estimating the mapping relationship between throughput and transmission power by existing power control formula.

For example, according to the power control formula, total allocated RB number is used to compute transmission power. In this case, as illustrated in Figure 21, the computed transmission power according to target throughput is significantly different from that used in practice, especially for extension spectrum. And, it is easy to deduce that this difference becomes more significant for steeper filter and larger SE ratio. To deal with this problem, the power control formula should be adjusted according to the difference of power density of REs.

Observation 19: The power density of RE has been changed due to FDSS-SE, its impacts include

- affecting the channel capacity, i.e., the transport block size under the given BLER requirement for the same transmission power and RB allocation.

- degrading the accuracy of estimating the mapping relationship between throughput and transmission power by existing power control formula in open-loop.

- resulting in a challenge for BS to allocate RB, select MCS to make the best use of transmission power.

Figure 21: Illustration of power density of REs in the computation of power control formula and the transmission in practice.

Proposal 8: Study whether/how to enhance the power control to take into account the difference of power spectral density of the REs due to the FDSS-SE.

The power ratio between extended and data spectrum can be used to improve BS’s resource allocation. Specifically, power ratio information can tell BS how the power allocation is used by UE, and then BS can adjust the resource allocation correspondingly.

Besides, receiver operation selection that is determined by the trade-off between complexity and MRC improvement is also closely associated with power ratio as MRC improvement depends on power ratio. For example, when the power of symbols on extended spectrum is large, it is more reasonable to utilize MRC due to its performance improvement. Otherwise, it should be considered to remain symbols on data spectrum alone for low complexity. Thus, the information of power usage should be reported to BS for better performance when transparent FDSS is used.

Observation 20: The reported power ratio between extension spectrum and non-extension spectrum can help BS allocate resource to make the best use of transmission power and determine receiver operation.

- Coverage enhancement by tone reservation

As illustrated in [2], the principle of tone reservation aims to use the peak reduction tones (PRT) to construct a compensating waveform. This compensating waveform is constructed by identifying peaks to cancel in the original waveform. The compensating waveform only uses the PRTs and thus leaves the data tones unaltered. One well known algorithm to obtain the compensating waveform is by J. Tellado [10] that iteratively cancels the peaks in the original waveform.

Some important observations have been given in [2]:

- To achieve the same PAPR/CM performance, sideband tone reservation size is proportional to the length of the data subcarrier.

- The number of PRT depends on MCS

- PRT patterns of unequal numbers of PRT on two sides can optimize PAPR/CM performance and spectrum resource utilization.

To prevent signals on the PRT from affecting other users, the base station needs to notify the UE of the actually used PRT resources or the maximum available PRT resources.

Observation 21: The gNodeB should indicate the actually used PRT resources or the maximum available PRT resources to UE.

As mentioned before, the PAPR/CM of the DMRS needs to be less than or equal to the PAPR/CM of the data symbol. Hence, PRTs should be added to DMRS symbols. As shown in [2], Type 2 DMRS can meet the requirement of PAPR/CM with or without PRT, while Type 1 DMRS can only meet the requirement of PAPR/CM with PRT.

Observation 22: When low PAPR sequence type 2 is adopted, there is no need to add PRTs to DMRS symbols.

Observation 23: When low PAPR sequence type 1 or enhanced low PAPR type 1 is adopted, PRTs should be added to DMRS symbols.

In order to have more intuitive performance comparisons between TR and FDSS with SE, the coverage enhancement gain of TR is given in Table 8 and the maximum coverage enhancement gain of FDSS with SE under different code rates and extension ratios is given in Table 14. The simulation parameters are given in Table 10. The maximum coverage enhancement gain under a certain extension ratio is obtained by traversing the four FDSSs given above. Considering that the additional power consumed by PRT is only 0.1~0.2 dB, the total transmission power of TR and FDSS with SE should be aligned for a fair comparison. For any code rate and extension ratio, FDSS-SE outperforms TR with a maximum gap of 1.18 dB.

Table 8: Coverage enhancement gain[dB] of QPSK with TR under different code rates and SE ratios.

| SE ratio Code rate | 1/8 | 1/4 | 3/8 |
| --- | --- | --- | --- |
| 1/16 | 0.65 | 0.81 | 1.25 |
| 1/8 | 0.32 | 0.63 | 1.08 |
| 1/6 | 0.26 | 0.55 | 1.28 |
| 1/4 | 0.15 | 0.41 | 0.70 |
| 1/3 | 0.19 | 0.27 | 0.34 |
| 1/2 | -0.05 | -0.32 | -0.67 |
| 0.64 | -0.41 | -1.13 | N/A |

Due to the inferior performance, TR should have lower priority than FDSS with SE.

Proposal 9: Tone reservation should be deprioritized.

- Conclusions

Based on our study of coverage enhancement in this contribution, we make the following proposals.

Observation 1: FDSS with SE provides small gains for π/2-BPSK compared to π/2-BPSK with FDSS w/o SE.

Observation 2: The net gains of QPSK under FDSS w/o SE are extremely close to zero, or even negative. While the net gains of QPSK under FDSS with extension are significant,0.8-2dB. FDSS should be combined with SE for QPSK to achieve significant coverage gain.

Observation 3: With given filter, the gain of MRC increases with the increase of RB number, extension ratio, and code rate, and up to 1 dB improvement of SNR degradation for the same demodulation performance is observed compared to basic receiver.

Observation 4: The best extension ratio maximizing net gain for QPSK is different at least for different MCS and positions of RB allocation:

- for inner-band RB allocations, when MCS index exceeds 4, extension ratio 1/9 shows a performance improvement of 0.5 dB at most compared to extension ratio 1/4.

Observation 5: Option 1 and Option 2 are special cases of Option 3 with specific cyclic shift L.

Observation 6: The choice of L does not change the set of symbols in the in-band and thus does not change the receiver requirement.

Observation 7: All options can either be implemented as a cyclic extension or symmetric extension.

Observation 8: For FDSS with SE for QPSK, Option 1 always provides the worst performance PAPR and CM, while Option 3 with an optimized shift L= Ndata8+Ne-12 always provides the best PAPR and CM reduction.

Observation 9: When inband length is larger than or equals to 30, approach A1 with option b and approach A2 with option b are available considering the requirement of PAPR/CM and DFT size limit.

Observation 10: Approach A1 with option b has better demodulation performance than approach A2 with option b, 1-3 dB gap can be observed.

Observation 11: When inband length is less than 30, approach A1 with option b, approach A2(Type 2 DMRS) with option b and approach B (Type 2 DMRS) are available considering the requirement of PAPR/CM.

Observation 12: Approach A1 has better demodulation performance compared to approach A2(Type 2 DMRS) and approach B (Type 2 DMRS), 0.3 dB gain can be observed.

Observation 13: Approach A1 has better cross-correlation and autocorrelation performance than approach A2（Type2 DMRS）and better auto-correlation performance than approach B (Type 2 DMRS)

Observation 14: The UE needs to report the FDSS capability to gNB.

Observation 15: Additional information such as SE ratio, SE shift and FDSS filter can either be pre-defined or be dynamically signaled from the gNB to the UE. If FDSS filter is not specified, the UE may also need to signal preferred values of SE ratio and SE shift to the gNB, or its FDSS filter.

Observation 16: In the first method, the number of allocated RB should be the multiple of 2a* 3b* 5c, where [a, b, c] are integers ≥ 0. In the second method, the number of allocated RB whether could guarantee that the number of RB for non-extension spectrum is the multiple of 2a* 3b* 5c, is closely associated with the using SE ratio. Moreover, it seems no need to restrict the starting RB of the allocation.

Observation 17: RB allocation of non-extension spectrum and spectrum extension ratio is more suitable than the second method due to less complexity.

Observation 18: The power density of RE has been changed by FDSS-SE.

Observation 19: The power density of RE has been changed due to FDSS-SE, its impacts include

- affecting the channel capacity, i.e., the transport block size under the given BLER requirement for the same transmission power and RB allocation.

- degrading the accuracy of estimating the mapping relationship between throughput and transmission power by existing power control formula in open-loop.

- resulting in a challenge for BS to allocate RB, select MCS to make the best use of transmission power.

Observation 20: The reported power ratio between extension spectrum and non-extension spectrum can help BS allocate resource to make the best use of transmission power and determine receiver operation.

Observation 21: The gNodeB should indicate the actually used PRT resources or the maximum available PRT resources to UE.

Observation 22: When low PAPR sequence type 2 is adopted, there is no need to add PRTs to DMRS symbols.

Observation 23: When low PAPR sequence type 1 or enhanced low PAPR type 1 is adopted, PRTs should be added to DMRS symbols.

Proposal 1: The π/2-BPSK using FDSS with SE is not supported.

Proposal 2: MRC receiver should be supported when FDSS and SE is performed.

Proposal 3: Two spectrum extension ratios should be supported which are 1/4 and 1/9.

Proposal 4: Cyclic shift plus symmetric extension should be supported.

Proposal 5: When inband length is larger than or equal to 30, adopt approach A1 with option b, where A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. The sequence is then cyclically extended to span the PRBs in the extension.

Proposal 6: When inband length is less than 30, adopt approach A1.

Proposal 7: The gNB should indicate the RB allocation of non-extension spectrum, spectrum extension ratio, MCS index to UEs, based on which the UEs calculate the size of transport block.

Proposal 8: Study whether/how to enhance the power control to take into account the difference of power spectral density of the REs due to the FDSS-SE.

Proposal 9: Tone reservation should be deprioritized.

# Appendix A: Additional simulation results

Figure 22: net gain of case 2 with different DMRS solutions

Figure 23: net gain of case 3 with different DMRS solutions

Figure 24: net gain of case 4 with different DMRS solutions

Figure 25: net gain of case 6 with different DMRS solutions

Table 9: ∆CM@1e-2 of π/2-BPSK with FDSS+SE or FDSS compared π/2-BPSK with DFT-s-OFDM under 8 RB allocations.

| SE ratio FDSS | 0% | 12.5% | 25% | 37.5% |
| --- | --- | --- | --- | --- |
| TRRC1(0.5, 0.1667) | 1.1 | 1.15 | 1.05 | 0.39 |
| 3 tap (-0.28,1,0.28) | 1.29 | 1.35 | 1.25 | 0.69 |

Table 10: First set of simulation parameters.

| Channel | PUSCH, 14 symbols |
| --- | --- |
| Carrier frequency and scenario | 700MHz (Rural) |
| Channel BW | 20MHz for Rural |
| SCS | 15 kHz (700 MHz) |
| Channel model | TDL-D 30ns for Rural |
| UE speed | 3km/h |
| Waveform | According to agreements |
| Modulation | According to agreements |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 4 |
| Number of DMRS symbols | 2 |
| Number of PUSCH data symbols | 12 |
| HARQ configuration | No retransmissions |
| Number of PRBs | 4 /8/16/24/36 RB |
| MCS/code rate | Table below |
| Channel estimation | DFT based estimation |
| Receiver type | MRC |
| BLER | 10% |

Table 11: SNR degradation of basic receiver compared to MRC receiver under 8 RB allocations

| SE ratio R | TRRC (0.5,0.1667) +1/8 | TRRC (0.5,0.1667) +1/4 | TRRC (0.5,0.1667) +3/8 | 3tap (-0.28,1, -0.28) +1/8 | 3tap (-0.28,1, -0.28) +1/8 | 3tap (-0.28,1, -0.28) +3/8 |
| --- | --- | --- | --- | --- | --- | --- |
| 1/8 | -0.13 | -0.41 | -0.83 | -0.08 | -0.25 | -0.50 |
| 1/4 | -0.20 | -0.50 | -1.00 | -0.22 | -0.36 | -0.59 |
| 1/2 | -0.30 | -0.53 | -1.07 | -0.35 | -0.70 | -1.00 |

Table 12: SNR degradation of basic receiver compared to MRC receiver under 32 RB allocations

| SE ratio R | TRRC (0.5,0.1667) +1/8 | TRRC (0.5,0.1667) +1/4 | TRRC (0.5,0.1667) +3/8 | 3tap (-0.28,1, -0.28) +1/8 | 3tap (-0.28,1, -0.28) +1/8 | 3tap (-0.28,1, -0.28) +3/8 |
| --- | --- | --- | --- | --- | --- | --- |
| 1/8 | -0.47 | -0.59 | -0.86 | -0.45 | -0.55 | -0.67 |
| 1/4 | -0.48 | -0.65 | -0.81 | -0.5 | -0.59 | -0.94 |
| 1/2 | -0.59 | -0.75 | -0.92 | -0.68 | -0.8 | -0.96 |

Table 13: Second set of simulation parameters.

| Channel | PUSCH, 14 symbols |
| --- | --- |
| Carrier frequency and scenario | 4GHz (Urban) |
| Channel BW | 100MHz for Urban |
| SCS | 30 kHz (4GHz) |
| Channel model | TDL-C 300ns for FR1 Urban (4GHz) |
| UE speed | 3km/h |
| Waveform | According to agreements |
| Modulation | According to agreements |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 4 |
| Number of DMRS symbols | 2 |
| Number of PUSCH data symbols | 12 |
| HARQ configuration | No retransmissions |
| Number of PRBs | 6/8/36/40 RB |
| Channel estimation | DFT based estimation |
| Receiver type | MRC |
| BLER | 10% |

Table14: Maximum coverage enhancement gain of FDSS-SE under different code rates and SE ratios.

| SE ratio Code rate | 1/8 | 1/4 | 3/8 |
| --- | --- | --- | --- |
| 1/16 | 1.05 | 1.59 | 1.75 |
| 1/8 | 0.77 | 1.47 | 1.72 |
| 1/6 | 0.86 | 1.53 | 1.51 |
| 1/4 | 0.76 | 1.30 | 1.39 |
| 1/3 | 0.66 | 1.25 | 1.01 |
| 1/2 | 0.58 | 0.84 | -0.31 |
| 0.64 | 0.27 | 0.05 | N/A |

Appendix B: Spectrum extension operation

Figure 26: DFT-s-OFDM transmitter structure with FDSS-SE

Figure 26 illustrates the transmitter block diagram of the DFT-s-OFDM with FDSS-SE. We first define the parameters as follows.

  - Nfft is the IFFT size of the OFDM modulation.

  - Ndata is the number of modulation constellation symbol.

  - Nsc=Ne+Ndata is the total number of OFDM subcarriers.

  - Ne is the number of subcarriers used for SE, that we refer also as the size of SE.

  - W0, …,WNsc-1 are the FDSS filter coefficients.

  - L is a Fourier coefficient shift parameter.

  - N‘data is the initial length of DMRS, i.e., ZC sequence with length N‘data, before cyclical extension, N‘data is the prime closest to integer Ndata.

For the reference case not using SE, for a fair comparison the number of subcarriers available for data transmission would be Nsc.

The operations of DFT-s-OFDM with FDSS-SE are as follows.

- The coded bit stream is mapped to modulation symbol stream through the constellation map in modulation module.

- The modulation symbol stream goes through serial-to-parallel (S/P) operation and obtain modulation symbol block with length Ndata.

- The modulation symbol block with length Ndata is converted into the symbol block in frequency domain with length Ndata by discrete Fourier transform (DFT).

- The DFT symbol block with length Ndata is cyclically shifted by a factor L, and then extended to a DFT symbol block with length Nsc by the symmetric spectrum extension method. The DFT symbols after spectrum extension are circularly symmetrical where the first part of the DFT coefficients are the copy of the corresponding last DFT coefficients of the middle part and the last part of DFT coefficients are the copy of the first DFT coefficients of the middle part.

- The extended DFT symbol block with length Nsc goes through the FDSS filter and is mapped to the used subcarriers. The used subcarriers consist of the extended subcarriers and the data subcarriers.

- The mapped DFT symbol block is converted into the OFDM symbol in time domain by inverse fast Fourier transformation (IFFT) with length Nfft and add cyclic prefix (CP) for transmission.

- The operation of DMRS is similar to the operation of data from the spectrum extension module, which would be discussed below.

Overall, a DFT-s-OFDM signal with FDSS-SE can be defined for samples 0≤n≤Nfft-1 by

| sn=1NscNdatak=0Nsc-1Wk Xsekej2πNfftnk | (12) |
| --- | --- |
| Xse is the frequency domain symbols with extension and can be given by Xsek=Xk+L (mod Ndata), k∈{0,1,…Nsc-1} | (13) |

with frequency domain symbols

| Xk=1Ndatam=0Ndata-1xme-j2πNdatakm | (14) |
| --- | --- |

where (mod N) is the modulo-N operator.

In above formula, we used a general shift parameter L to encompass two SE methods available in the literature:

- L=0 which is common in older literature, see for example [3]. The SE is then specifically

| X(se)=X0,…,XNdata-1, X0,…,XNe-1 | (15) |
| --- | --- |

- L=Ndata-Ne/2 which was used in the more recent paper [4] with motivation to keep and centered the original sequence in the middle band. The SE becomes:

| X(se)=XNdata-Ne2,…,XNdata-1, X0,…,XNdata-1, X0, …,XNe2-1 | (16) |
| --- | --- |

Figure 27 illustrates the receiver block diagram of the DFT-s-OFDM with FDSS-SE. The operations of receiver to demodulate the DFT-s-OFDM with FDSS-SE are given as follows.

- The receiver removes the CP of the received OFDM symbol, obtains the samples with length Nfft, converts the samples into the symbol block with length Nfft in frequency domain by IFFT with length Nfft, deletes the symbols on the unused subcarriers, and equalizes the symbols on used subcarriers with channel estimation results by equalization module, i.e., DFT symbol block with length Nsc. The used subcarriers consist of the SE subcarriers and the data subcarriers.

- After equalization, to obtain the DFT symbol block with length Ndata, the corresponding symbols on the SE and data subcarriers are either combined by maximum ratio combining (MRC) or the symbols on the SE subcarriers are not used.

- The DFT symbol block with length Ndata is converted into the modulated symbol block in time domain with length Ndata by IDFT.

- The modulated symbol block with length Ndata is demodulated into bit stream.

Figure 27: DFT-s-OFDM receiver structure with FDSS-SE.

Based on the introduced transceiver block diagram of DFT-s-OFDM with FDSS-SE, we further explain the important module and the related conception in detail.

# References

- R1-2301872, “FL summary #4 of power domain enhancements (AI 9.14.2)”, Nokia, Nokia Shanghai Bell, 3GPP RAN1 #112, February 27 – March 3, 2023

- R1-2300090, “Discussion on coverage enhancement in power domain”, Huawei, HiSilicon, RAN1#112, February 27th –March 3rd, 2023

- TS 38.211, “Physical channels and modulation”,

- RP-213579, “New WI: Further NR coverage enhancements”, China Telecom, RAN1#94e, December 6th- 17th, 2021.

- User Equipment (UE) Radio Transmission and Reception; Part 1: Range 1 Standalone (Release 16), document TS38.101-1 V16.5.0, Sep. 2020.

- J. Kim, Y. H. Yun, C. Kim and J. H. Cho, "Minimization of PAPR for DFT-Spread OFDM With BPSK Symbols," in IEEE Transactions on Vehicular Technology, vol. 67, no. 12, pp. 11746-11758, Dec. 2018.

- P. Nasarre, T. Levanen, K. Pajukoski, A. Lehti, E. Tiirola and M. Valkama, "Enhanced Uplink Coverage for 5G NR: Frequency-Domain Spectral Shaping With Spectral Extension," in IEEE Open Journal of the Communications Society, vol. 2, pp. 1188-1204, 2021.

- J. Tellado, Multicarrier Modulation With Low PAR: Applications to DSL and Wireless, 2000

- R4-2213561, “WF on simulation assumptions for PAPR/MPR reduction”, Qualcomm, Nokia, RAN4#106, Athens, GR, Feb-Mar. 2023