3GPP TSG-RAN WG1 Meeting #112bis-e	  R1-2302358

e-Meeting, April 17 – 26, 2023

Agenda Item:	9.2.2.1

Source:	Huawei, HiSilicon

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

# Introduction

In previous RAN1 meetings, evaluation methodology of AI/ML for Channel State Information (CSI) feedback enhancement has been discussed and several agreements of the evaluation methodology have been achieved. In this contribution, we will provide further discussions on the evaluations on AI/ML for CSI feedback enhancement, including evaluation methodology and simulation results.

# Evaluation methodology

In this section, we will discuss the issues of evaluation methodology for AI/ML-based CSI feedback.

## 2.2 Specific evaluation methodology for AI/ML-based CSI prediction

### 2.2.1 Collaboration level x CSI prediction

For the FFS part of the working assumption, it is our understanding that collaboration level x AI/ML-based CSI prediction means Network is not aware of the existence of UE-side CSI prediction model and thus Network cannot perform the LCM operations, e.g., model switching/selection/updating, for the UE-side CSI prediction model. Considering Network is responsible for the performance of the entire cellular network, and takes the obligation to guarantee a robust performance, it is more beneficial to allow Network to eventually confirm the switching/updating of UE-side AI/ML models, which is the same principle as enabling/disabling UE functionalities as widely applied in legacy; on the other hand, the UE device may not be adequate to accurately decide when to perform model switching/updating due to lack of sufficient monitoring data collected by per single device.

In this regard, it can be assumed that the collaboration level x, if reported by companies, is modeled as adopting a fixed CSI prediction model at the UE for all scenarios without being updated/switched. Therefore, the simulation methodology to compare the LCM-enabled AI/ML solution with collaboration level x based CSI prediction is to assume the AI/ML models are initially trained with the dataset subject to a specific scenario while inference is performed under a different scenario; the AI/ML-based CSI prediction enabled with LCM can perform fine-tuning (e.g., under the indication of model updating by gNB) to improve the performance, while the level x based CSI prediction cannot perform model updating similar to the generalization Case 2.

Proposal 1: For the CSI prediction sub use case, if collaboration level x is reported as the benchmark, the simulation methodology to evaluate the AI/ML-based CSI prediction enabled by LCM can emulate the fine-tuning case, while the simulation methodology to evaluate collaboration level x CSI prediction can emulate generalization Case 2.

## 2.3 Specific evaluation methodology for AI/ML-based CSI compression

### 2.3.1 Evaluation methodology for monitoring accuracy

In the last meeting, it has been agreed to study the intermediate KPIs based monitoring in 9.2.2.2. To evaluate the performance impact of the monitoring accuracy using different approaches, the EVM of intermediate KPI based monitoring is provided in this section.

The calculation of intermediate KPI such as SGCS is based on the correlation between two kinds of CSI: one is the recovery CSI of the CSI reconstruction model and another is the ground-truth CSI (i.e., target CSI subject to input-CSI-NW). If the ground-truth CSI obtained for calculating the intermediate KPI is imperfect, it will impact the monitoring performance. In light of this, the monitoring can be evaluated in terms of the monitoring accuracy of the intermediate KPI under the given ground-truth CSI. For the format of the ground-truth CSI, there may be various choices, e.g., CSI in Float32 format, legacy CSI reported with Rel-16 Type II CB (e.g., PC1-PC8), higher resolution CSI reported by Rel-16 Type II CB with new parameters, etc. CSI in Float32 format can be regarded as the genie-aided value (i.e., 100% accurate) for calculating the monitoring accuracy of other formats of ground-truth CSI. For a quantized ground-truth CSI with relatively lower resolution than Float32, the calculated intermediate KPI (namely actual intermediate KPI) may bias from the intermediate KPI calculated with the genie-aided ground-truth CSI (namely target intermediate KPI); for a lower resolution of quantized ground-truth CSI, it is anticipated with a larger gap/variation between the actual intermediate KPI and the target intermediate KPI. Such statistical gap can be considered to represent the accuracy of the monitoring.

For example, if we want to assess the monitoring accuracy of ground-truth CSI format A, the SGCS between the recovery CSI and ground-truth CSI format A is calculated and denoted as SGCSA. The SGCS between the same recovery CSI and CSI in Float32 format is calculated and denoted as SGCSTarget. Then for each data sample including a ground-truth CSI and a corresponding recovery CSI, the SGCS gap between SGCSA (actual SGCS) and SGCSTarget (target SGCS) can be calculated, and thus for a test dataset with a number of samples, a number of corresponding SGCS gaps can be collected. The mean SGCS gap or the percentage of samples for which the SGCS gap is smaller than a threshold can be used to assess the monitoring accuracy of ground-truth CSI format A. Smaller SGCS gap or smaller percentage of SGCS gap exceeding the threshold means higher monitoring accuracy. In particular, for describing the percentage of the gaps, the following two options can be considered, which are also demonstrated by the figure in below:

- Option 1: the gap is an absolute value (i.e., gap > threshold is deemed as an inaccurate monitoring occasion).

- Option 2: the gap consists of a negative bias (i.e., SGCSA<SGCSTarget with the gap < threshold, which is deemed as a false alarm occasion) and a positive bias (i.e., SGCSA>SGCSTarget with the gap > threshold, which is deemed as a misdetection occasion).

The simulation results in terms of the metrics described above are shown in Section 3.9.

Figure 1 Description of the calculation of monitoring accuracy

Proposal 2: To evaluate the monitoring accuracy of the intermediate KPI under a given ground-truth CSI format (e.g., quantized ground-truth CSI), the methodology can be considered in terms of the statistical gap between the actual intermediate KPI and the target intermediate KPI for a group of test data samples, where the actual intermediate KPI is calculated with the recovery CSI and the given ground-truth CSI format, and the target intermediate KPI is calculated with the same recovery CSI and the ground-truth CSI of Float32.

Proposal 3: For the monitoring accuracy of the intermediate KPI under a given ground-truth CSI format (e.g., quantized ground-truth CSI) by using the statistical gap for a group of test data samples, the following metrics can be considered to describe the monitoring accuracy:

- Metric 1: The mean gap over the group of test data samples

- Metric 2: The percentage of the samples for which the gap in terms of an absolute value is smaller than a threshold

- Metric 3: The percentage of the samples for which the gap in terms of a positive bias lower than a positive threshold, or in terms of a negative bias higher than a negative threshold, where bias is denoted as actual intermediate KPI - target intermediate KPI

  - Metric 3-1: if the positive bias is higher than the positive threshold, such inaccuracy case is deemed as misdetection.

  - Metric 3-2: if the negative bias is lower than the negative threshold, such inaccuracy case is deemed as false alarm.

### 2.3.2 Evaluation methodology for training types

In the previous meetings, some agreements about training type related EVM had been achieved. In this section, EVM related issues for training Type 2 and Type 3 are further discussed.

Remaining issue for training Type 2

The following proposal on Type 2 had been discussed in the RAN1#111 meeting on the training method for multi-vendors.

| Proposal 3.5.1: For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), besides the evaluation of Case 2 (one NW part model and M>1 separate UE part models) and Case 3 (one UE part model and N>1 separate NW part models), N>1 NW part models to M>1 UE part models are additionally considered as an evaluation case (i.e., Case 4), and companies to report - The training method/order, including the preference and details to achieve - Case A: Simultaneously train all N Network part models and M UE part models involving all vendors - Case B: Sequentially train N Network part models and M UE part models in a pairwise manner - Others - The generation of loss function, e.g., whether loss function is jointly generated over N>1 NW part models, or separately generated for N>1 NW part models. |  |
| --- | --- |
| (a) Case A | (b) Case B |

Figure 2 Implementation options of Type 2 for multi-vendors scenario

For Case A, shown as Figure 2(a), it is very difficult to align the timeline of product development over all involved Network vendors and UE vendors to perform training at the same time. In addition, model parameter updating for each UE vendor and Network vendor will be impacted by all other involved Network vendors and UE vendors. Thus, if AI/ML models with poor performance (e.g., small model or lame model backbone) are adopted by some vendors, it will become the bottleneck and may negatively impact the performance of all other Network-UE pairs.

For Case B, shown as Figure 2(b), after the training of one Network-UE pair, the model at one side may need to be frozen when training the next pair to ensure the model still work for the previous pair with another vendor. E.g., after the training of first pair between UE 1 and Network 1, AI/ML model of UE 1 need to be frozen when training the next pair between UE 1 and Network 2. Therefore, the training order will impact the performance of each Network-UE pair. Intuitively, the latter Network-UE pair will achieve worse performance than the former Network-UE pair, thus the order of training will bring gap of performance and harm the fairness among vendors. In addition, as shown in Figure 2 (b), after training of the first 3 Network-UE pairs, AI/ML models of all involved Network and UE are already frozen, thus it may not be even feasible for the last Network-UE pair between UE 2 and Network 1 in step 4 (with dashed arrow) to train their two-sided model.

In addition, it is not necessary, nor practical for multiple Network vendors or multiple UE vendors to jointly generate loss function during Type 2 training stage; in contrast, it is more practical to generate loss function separately for N>1 Network part models.

Proposal 4: For the evaluation of Type 2 (Joint training of the two-sided model at Network side and UE side, respectively), analyze the feasibility for the following two cases of N>1 NW part models to M>1 UE part models (i.e., Case 4), and companies to report the training method/order, including the preference and details to achieve

- Case A: Simultaneously train all N Network part models and M UE part models involving all vendors

- Case B: Sequentially train N Network part models and M UE part models in a pairwise manner

- Note: The loss function should be separately generated for N>1 NW part models.

Observation 1: For Type 2 (Joint training of the two-sided model at Network side and UE side, respectively), challenges are observed for both of the two training methods/orders of Case A (Simultaneously train all N Network part models and M UE part models involving all vendors) and Case B (Sequentially train N Network part models and M UE part models in a pairwise manner)

- For Case A, there may be challenges on aligning the training timelines of all involved Network vendors and UE vendors.

- For Case B, fairness may be harmed for later trained Network-UE pairs where only one side may be able to update parameters during the joint training.

Remaining issue for training Type 3

In the last meeting [1], an issue was discussed about the boundary between Type 2 and Type 3.

Upd Question 3.3.1: For the evaluation of training types of CSI compression, which of Type 2 and Type 3 or a new type should it belong to, if - Step A: first side is trained, and after the training for the first side is finished - Step B: the second side is trained afterwards, with forward propagation and backward propagation (e.g., gradients) exchanged across the two sides, while the first side is frozen

According to the definition of Type 2 and Type 3 training, the key difference between them is whether the UE and Network are training jointly and separately. In our understanding, joint training means UE and Network are involved in each FP/BP loop, while separate training means UE or Network can finish all FP/BP loops independently and the FP/BP is processed within per side without being exchanged to the other side. On the other hand, for the definition of Type 3, an individual Step 2 (information exchange) is in between Step 1 (training phase for Network/UE side) and Step 3 (training phase for UE/Network side), i.e., the information exchange is out of the training phase for either side.

Conclusion For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training (NW-first training): - Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly - Step2: After NW side training is finished, NW side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part - Step3: UE side trains the UE side CSI generation part based on the received set of information - Other Type 3 NW-first training approaches are not precluded and reported by companies Conclusion For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with UE side training (UE-first training): - Step1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly - Step2: After UE side training is finished, UE side shares NW side with a set of information (e.g., dataset) that is used by the NW side to be able to train the CSI reconstruction part - Step3: NW side trains the NW side CSI reconstruction part based on the received set of information - Other Type 3 UE-first training approaches are not precluded and reported by companies

For the case one side is trained first, and the second side is trained afterwards, with forward propagation and backward propagation (e.g., gradients) exchanged across the two sides, it is obvious that FP/BP exchange will happen in each FP/BP loop during the training phase of the second side. Although the side trained first will be frozen when the second side is trained, the first side still operates with its AI/ML part for each loop for the training of the second side. Therefore, this case should belong to Type 2 training.

Proposal 5: The boundary between Type 2 and Type 3 is whether the information exchange is required in each FP/BP loop or not:

- Type 2 training requires information exchange in each FP/BP loop.

- Type 3 training doesn’t require information exchange in each FP/BP loop.

Observation 2: If one side is trained first, and the second side is trained afterwards, with forward propagation and backward propagation (e.g., gradients) exchanged across the two sides, it belongs to Type 2 training.

In addition, how to perform such “frozen-and-training” method for the case of N>1 Networks and M>1 UEs was also discussed as given in below.

Question 3.3.2: For the new behavior raised in Question 3.3.1, whether/how such sequential training behavior can support the training of N>1 NWs and M>1 UEs?

In our view, it is feasible for the case of 1 Network to M>1 UEs or the case of 1 UE to N>1 Networks, but as shown in our Figure 2(b), for the case of N>1 Networks and M>1 UEs, the training between UE 2 and Network 1 will become a deadlock since both have been frozen under a previous training procedure.

An argument raised in the last meeting for justifying the feasibility is that UE 2 can train a different CSI generation model in step 4 in Figure 2(b). However, if we assume UE can train multiple models to pair with different Network vendors, it is more straightforward to adopt the Type 3 NW first training, i.e., as the newly coming UE anyway needs to train new model(s) to match the legacy Network vendors, it can train the new model(s) with the dataset(s) delivered by Network vendor(s); in that way, Type 3 NW first training will also achieve the extendibility without impacting the pairing of legacy Network/UE vendors as analyzed in our companion contribution [2].

### 2.3.3 Remaining issues of templates for collecting evaluation results

In the last meeting, the templates for collecting evaluation results for different training cases and generalization/scalability verification have been agreed. But there are still some FFS parts for the templates. In this section, the remaining issues for the templates are discussed, including the ranges of CSI feedback overhead for the eventual KPI, CSI overhead reduction and baseline of performance comparison for different training cases.

CSI feedback payload size range of eventual KPI

In the last meeting, the following agreement for CSI feedback payload calculation and working assumption for CSI feedback payload range X/Y/Z in template have been achieved, and as highlighted in the working assumption for the template, there is still a note to revisit the CSI feedback overhead for the eventual KPI, since it is desired to align the range of overall CSI feedback overhead for the cross check of the eventual UPT over companies.

Agreement The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. - For AI/ML based solutions: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank. - For legacy Type II CB: Option 2b is mandatorily reported by companies, while Option 2a can be optionally reported up to companies if partial NZC report is assumed for the legacy Type II CB - Option 2a: The above-mentioned “CSI feedback overhead” is calculated as each CSI reported payload with a given rank - Option 2b: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank Working Assumption For the initial template for AI/ML-based CSI compression without generalization/scalability verification achieved in the working assumption in the RAN1#111 meeting, X, Y and Z are determined as: - X is <=80bits - Y is 100bits-140bits - Z is >=230bits Working assumption The following initial template is considered to replace the template achieved in the working assumption in the RAN1#111 meeting, for companies to report the evaluation results of AI/ML-based CSI compression of 1-on-1 joint training without generalization/scalability verification - To be collected before 112bis-e meeting - FFS the description and results for generalization/scalability may need a separate table - Note: the values of CSI feedback overhead for the mean UPT and 5% UPT may need to be revisited in the 112bis-e meeting - FFS: training related overhead - FFS: how to capture CSI overhead reduction to the template - Note: It is to be captured to the template after a way is found on how to derive the CSI overhead reduction. …

According to the agreement, the CSI feedback payload size of the eventual KPI should be calculated as the weighted average of CSI payload per rank.

For the case max rank = 1, the CSI feedback payload range of eventual KPI should directly use X, Y and Z. That is to say, for rank 1, the same range as the per layer basis CSI feedback payload can be applied.

For the case max rank = 2, the rank distribution {rank 1, rank 2} for different schemes and CSI payload sizes varies from {70%, 30%} to {56%, 44%} in our evaluation for different settings, and the CSI payload size under rank 2 is configured as doubled over rank 1, so that the weighted rank is in range from 0.7+0.3*2=1.3 to 0.56+0.44*2=1.44. A middle scale factor of 1.4 is adopted, so that the CSI feedback payload ranges for eventual KPI are suggested to be:

- Low overhead: <= 112 bits (i.e., 80bits*1.4),

- Medium overhead: 140bits – 196 bits (100bits*1.4-140bits*1.4)

- High overhead: >= 322 bits (i.e., 230bits*1.4).

For the case max rank = 3 and 4, since the CSI payload size for rank=3 and rank=4 is similar as the CSI payload size for rank=2, the CSI feedback payload range of eventual KPI should be the same as the range for rank = 2.

Proposal 6: For the template of AI/ML-based CSI compression of 1-on-1 joint training, the CSI feedback overhead range of eventual KPI is suggested as:

- Low overhead: <=β* 80 bits.

- Medium overhead: β* (100bits – 140 bits).

- High overhead: >=β* 230 bits.

- Note: β=1 for max rank = 1, andβ=1.4 for max rank = 2/3/4.

CSI overhead reduction

Another FFS issue about the template is how to calculate the CSI overhead reduction. In particular, CSI overhead reduction is calculated by drawing a horizontal line across the y axis (which is regarded as a reference KPI) and compare the CSI feedback overheads corresponding to its two cross points with the two curves. However, the issue is that for different UPT results (or equivalently different CSI payload sizes), the CSI overhead reduction results may be quite different, e.g., as shown in the figure in below, the CSI overhead reduction would be larger in case of a higher UPT value (or a larger CSI feedback overhead). Thus it is better to report CSI overhead reduction per CSI feedback overhead ranges basis, which is similar to the eventual KPI as in the previous proposal. That is to say, the eventual KPIs of the benchmark curve corresponding to range “Low/Medium/High” CSI feedback overhead can be used as the reference KPIs to calculate the CSI overhead reduction, respectively.

Figure 3 Example of CSI overhead reduction in different CSI feedback overhead range

Proposal 7: For the template of AI/ML-based CSI compression of 1-on-1 joint training, 3 values of CSI overhead reduction can be reported corresponding to 3 CSI feedback overhead ranges to be determined in the template: “Low overhead”, “Medium overhead”, and “High overhead”, respectively.

- For a given CSI feedback overhead range, it is calculated as the gap, between the CSI feedback overhead of the benchmark which should within the given CSI feedback overhead range, and the CSI feedback overhead of the AI/ML solution corresponding to the same eventual KPI.

  - E.g., for a given CSI feedback overhead range, a horizontal line is drawn on the figure which will have two cross points with two curves of benchmark and AI/ML, respectively, from which the CSI overhead reduction is derived. The cross point with the curve of benchmark should fall into the given CSI feedback overhead range.

Benchmark of performance comparison for separate training

In the last meeting, the following agreement was achieved to use three separate templates to collect different training types/cases for AI/ML-based CSI compression without generalization/scalability.

Agreement For how to separate the templates for different training types/cases for AI/ML-based CSI compression without generalization/scalability verification, the following is considered: - The determined template in the RAN1#111 working assumption is entitled with “1-on-1 joint training” - A second separate template is introduced to capture the evaluation results for “multi-vendor joint training” - Note: this table captures the results for the joint training cases of 1 NW part model to M>1 UE part models, N>1 NW part models to 1 UE part model, or N>1 NW part models to M>1 UE part models. An example is multi-vendor Type 2 training. - A third separate template is introduced to capture the evaluation results for “separate training” - FFS: additional KPIs for each template, e.g., overhead, latency, etc.

For separate training, in particular, it was agreed that a new case of 1-on-1 training with joint training is determined as the benchmark for performance comparison. This is to evaluate the potential performance loss of the separate training behavior.

Agreement For the evaluation of training Type 3 under CSI compression, besides the 3 cases considered for multi-vendors, add one new Case (1-on-1 training with joint training) as benchmark/upper bound for performance comparison. - FFS the relationship between the pair(s) of models for Type 3 and the pair(s) of models for new Case

For how to determine the pair of Network part model/UE part model for the new case, the following proposal was discussed.

Proposal 3.5.1: For the evaluation of training Type 3 under CSI compression, for the new case (1-on-1 joint training) benchmark/upper bound for performance comparison, the structures for the pair of NW part model/UE part model for the new case are the same with the Type 3 case to be compared.

The intention of the proposal is to decouple the impact of the restricted model pairing from the separate training behavior, which are two factors resulting in the eventual performance loss of Type 3. The restricted model pairing lies in the non-cooperative model structure development or misaligned target performance metric between the Network vendor and the UE vendor, and would cause suboptimal end-to-end performance in contrast with the ideal model pairing. Such restriction applies to all of Type 1/2/3 in practical networks: for Type 1, there may be software/hardware compatibility issue at the UE device, so that the CSI generation part model may not be freely designed by the Network vendors; for Type 2 and Type 3, the model structure is deemed as proprietary and unknown to the other side, so the unmatched pair of models may occur.

Thus, it does not make a strong sense to compare the performance between restricted model pairing with ideal model pairing, since the performance highly depends on the learning capabilities of the paired models; e.g., the paired models of TF#1 + CNN#A (which can be deemed as restricted pairing) may outperform CNN#1 + CNN#A (which can be deemed as ideal pairing but both with weak learning capabilities), but it cannot be used to justify the performance benefit of restricted pairing. As another example for separate training, the paired models of TF#1 + CNN#A with separate training may be significantly inferior to TF#1 + TF#A with joint training, but this loss is largely due to model pairing rather than dataset sharing.

Based on the above analysis, when determining the new case of benchmark, the impact of performance difference due to the learning capability of the model itself should be decoupled from the performance loss due to the separate training manner (i.e., dataset sharing). On one hand, the new case of benchmark should take into account the 1-on-1 joint training with the same pair of Network part model and UE part model to the Type 3 case - this is to mitigate the factor where a lame Network or UE part model jeopardizes the end-to-end performance. On the other hand, the new case of benchmark should also take into account the 1-on-1 joint training with the same pair of Network part CSI reconstruction model and virtual Network part CSI generation model for NW first training, or UE part CSI generation model and virtual UE part CSI reconstruction model for UE first training - this is to mitigate the factor where a lame virtual Network part CSI generation model or virtual UE part CSI reconstruction model jeopardizes the end-to-end performance.

In our understanding, the learning capability of the UE part model(s) and Network part model(s) can be easily aligned by aligning the target performance metric between the Network side and the UE side.

Proposal 8: For the performance comparison of training Type 3, the learning capability difference of model itself should not be attributed to the potential performance loss due to the specific training behaviors (e.g., dataset sharing).

Proposal 9: For the evaluation of training Type 3 under CSI compression, the new case (1-on-1 joint training) benchmark/upper bound for performance comparison should separately take into account the following aspects:

- Aspect 1: The same pair of Network part model and UE part model to the Type 3 case being compared.

- Aspect 2: The same pair of Network part CSI reconstruction model and virtual Network part CSI generation model for NW first training Type 3 case being compared, or UE part CSI generation model and virtual UE part CSI reconstruction model for UE first training Type 3 case being compared.

Calculation of FLOPs/memory storage for different rank>1 options

In the last meeting, the following agreement was achieved to let companies report the specific option adopted for AI/ML model settings when rank>1. However, there is a remaining issue on how to calculate the FLOPs/memory storage of each of the report options for rank>1 case, where there could be multiple models for different layers or ranks, or one common model needs to perform inference multiple times for different layers for a single rank. Aligning the understanding of this metric may be helpful for the comparison of the prices among the rank>1 options (which may have different spec impacts).

Agreement For the evaluation of the AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers. - Option 1-1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference, any specific model operates on multi-layers jointly. - FFS on the reported complexity and storage - FFS: input/output type - Option 1-2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference, the model operates on multi-layers jointly. - FFS: input/output type - Option 2 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference. - FFS on the reported complexity and storage - Note: input/output type is Precoding matrix - Companies to report the setting is - Option 2-1: layer specific and rank common (different models applied for different layers; for a specific layer, the same model is applied for all rank values), or - Option 2-2: layer specific and rank specific (different models applied for different layers; for a specific layer, different models are applied for different rank values) - Option 3 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference. - FFS on the reported complexity and storage - Note: input/output type is Precoding matrix - Companies to report whether the setting is - Option 3-1: layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference), or - Option 3-2: layer common and rank specific (different models applied for different rank values; for a specific rank, the same model is applied for all layers) - Other options not precluded.

In our understanding, for the report of FLOPs, for a single CSI report with a specific rank, if there are one layer common model or multiple layer specific models to perform separately inference for different layers, then the FLOPs would be a summation of FLOPs over each layer of this rank. Actually, how to calculate the FLOPs may be related with the assumption of how to obtain the RI, e.g., if the RI is determined by using the target CSI without using AI/ML model, then the above rule (i.e., per rank FLOPs) is applicable; on the other hand, if the RI is determined by exhaustively searching all rank values as a post AI/ML operation, then the summation of FLOPs for inference over all rank values and all layers for per rank needs to be added on top of the per rank FLOPs. It is our understanding that it is more simple and realistic to calculate RI based on the target CSI, so per rank report of FLOPs is preferred.

On top of that, considering the rank adaptation, there could be two alternatives for calculating the overall FLOPs:

- Alt.1: Report the FLOPs as a weighted average with per rank FLOPs and rank distribution. This is with a similar rule as the calculation of the CSI feedback overhead, and can reflect the average complexity for a long term scale.

- Alt.2: Report the maximum FLOPs over all ranks. This can reflect the peak complexity for the worst case when maximum configured rank is reported by UE.

As per our flavor, Alt.2 is simpler and can be considered for the report of FLOPs.

On the other hand, for the calculation of the memory storage/number of parameters, as the storage depends on the total number of models and irrelevant to the number models used for per CSI measurement/report, the principle to calculate the storage should be a single value in terms of summation of memory storage/number of parameters over all models potentially used for any layer/rank.

Take Option 2-2 (layer specific and rank specific) as an example, for a specific CSI report with rank = k, k models will be running, i.e., model for layer 1, model for layer 2,…, model for layer k. Thus, the complexity of this single CSI report is the sum of the FLOPs of these k models. Considering the configuration with max rank=K, the overall FLOPs is derived as the sum of K models as addressed by Alt.2. For the storage analysis, the number of models need to be stored is 1+2+…+K= (1+K)K/2. Following the similar logic, the number of models need to run for each CSI report, complexity and the number of models need to store for all rank>1 options are listed in Table 1.

Table 1 Complexity and storage of different options with max rank=K

| Rank>1 options | Number of needed models for per CSI report | Overall FLOPs | Number of models for storage |
| --- | --- | --- | --- |
| Option 1-1 (rank specific) | 1 | Max FLOPs over K rank specific models | K |
| Option 1-2 (rank common) | 1 | FLOPs of the rank common model | 1 |
| Option 2-1: layer specific and rank common | K | Sum of the FLOPs of K models (for the rank=K) | K |
| Option 2-2: layer specific and rank specific | K | Max of the FLOPs over K ranks, 1,…k, each with a sum of FLOPs over k models | (1+K)K/2 |
| Option 3-1: layer common and rank common | 1 | K * FLOPs of the common model | 1 |
| Option 3-2: layer common and rank specific | 1 | Max of the FLOPs over K ranks, 1,…k, each with k * FLOPs of the layer common model | K |

It should be noted that, although the values of overall FLOPs may be similar over layer common options (Option 2-1/2-2) and layer specific options (Option 3-1/3-2), layer specific options need to run different models separately for each layer as each of the layer specific models are subject to different structures/parameters. This may cause challenge for the gNB/UE and may also lead to additional processing delay. As a comparison, layer common options only need to run a single model in one processing where the multiple layers can be processed as one batch by making use of the GPU processing, which largely alleviates the complexity.

Proposal 10: For the rank >1 options under AI/ML-based CSI compression, the complexity of FLOPs is reported as the maximum FLOPs over all ranks each includes the summation of FLOPs for inference per layer if applicable, e.g.,

- Option 1-1 (rank specific): Max FLOPs over K rank specific models.

- Option 1-2 (rank common): FLOPs of the rank common model.

- Option 2-1 (layer specific and rank common): Sum of the FLOPs of K models (for the rank=K).

- Option 2-2 (layer specific and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with a sum of k models.

- Option 3-1 (layer common and rank common): K * FLOPs of the common model.

- Option 3-2 (layer common and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with k * FLOPs of the layer common model.

Observation 3: For a given rank, layer common options only need to run a single model in one processing where the multiple layers can be processed as one batch by making use of the GPU, which largely alleviates the complexity as opposed to the layer specific models.

Proposal 11: For the rank >1 options under AI/ML-based CSI compression, the storage of memory storage/number of parameters is reported as the summation of memory storage/number of parameters over all models potentially used for any layer/rank, e.g.,

- Option 1-1 (rank specific)/Option 3-2 (layer common and rank specific): Sum of memory storage/number of parameters over all rank specific models.

- Option 1-2 (rank common): A single memory storage/number of parameters for the rank common model.

- Option 2-1 (layer specific and rank common): Sum of memory storage/number of parameters over all layer specific models.

- Option 2-2 (layer specific and rank specific): Sum of memory storage/number of parameters for the specific models over all ranks and all layers in per rank.

- Option 3-1 (layer common and rank common): A single memory storage/number of parameters for the common model.

# Evaluations for CSI compression

In this section, evaluations for spatial-frequency domain CSI compression will be discussed, including AI/ML model description, evaluation methodology for different training types and evaluation results.

## 3.1 AI/ML model description

### 3.1.1 CSI compression with current CSI as input

The CSI generation part including an encoder and a quantizer are deployed at the UE side for CSI compression, while the CSI reconstruction part including a decoder and a de-quantizer are deployed at the Network side for CSI recovery. The quantizer is used to quantize the output of the encoder which is a floating-point vector to fit the bit width for CSI feedback, while the de-quantizer is used to recover the floating-point vector as the input to the decoder. The AI/ML-based CSI feedback considering both spatial and frequency domain channel correlation is named as AI/ML-based spatial-frequency compression (AI-SF), which is depicted in Figure 4 (a). In our simulation, Transformer shown as Figure 4 (b) is used as the backbone of both encoder and decoder unless state otherwise.

(a)

(b)

Figure 4  The structure of AI/ML-based CSI compression with current CSI only as input

- Encoder: The encoder takes the original eigenvectors as the input, and outputs the compressed CSI with smaller size than the original eigenvectors. Specifically, the input of the encoder includes eigenvectors for N subbands, which are formulated as V=V1…VN, where Vn denotes the eigenvector for the n-th subband. Then, the encoder can use multiple Transformer layers to process the eigenvector matrix V, and obtains the compressed CSI as a floating-point vector as a result. The compressed CSI can be formulated as VEn=fencoder(V), where fencoder(∙) represents the function of the encoder. The SVD decomposition is applied as the pre-processing prior to the encoder to derive the original eigenvectors.

- Quantizer: The quantizer at the UE side maps the compressed CSI of a floating-point vector to a quantized bit sequence to fit the bit width for CSI feedback. Various methods of quantization may be adopted, such as scalar quantization, vector quantization (quantizing a vector utilizing its probability density functions), and etc. The quantized CSI feedback can be formulated as VQ=fquantizer(VEn), where fquantizer(∙) represents the function of the quantizer. In our simulation, vector quantization is used. To avoid the huge dimension of the quantization dictionary, we divide the quantization dictionary into several sub-dictionaries and divide the compressed CSI into several parts. Each part of compressed CSI is quantized by a sub-dictionary.

- De-quantizer: The de-quantizer recovers the compressed CSI from the feedback CSI bit sequence and sends it as the input to the decoder. The de-quantized CSI can be formulated as VDe=fdequantizer(VQ) where fdequantizer(∙)represents the function of the de-quantizer.

- Decoder：The decoder recovers the eigenvectors. Specifically, the decoder can use multiple Transformer layers for CSI reconstruction, which is in alignment with the structure of the encoder. The recovered eigenvectors can be formulated as V=fdecoder(VDe), where fdecoder(∙) represents the function of the decoder.

### 3.1.2 CSI compression with additional past CSI as input

The AI/ML-based spatial-frequency domain CSI compression can also learn the temporal domain correlation of channels on top of spatial-frequency domain compression, namely AI-SFT, which is depicted in Figure 5. In our simulation, LSTM is chained on top of a Transformer backbone for both the CSI generation part and the CSI reconstruction part to take the past/historical CSI into account of the AI/ML model input.

As shown in Figure 5, the AI/ML model can store past/historical information from previous slots and use this information to compress/recover the CSI of the current slot. The past/historical information from previous slots can be regarded as accumulated CSI information and thus the CSI feedback payload for the current slot can be regarded as delta CSI information on top of the accumulated CSI information. Therefore, compared to AI-SF, the overhead of the CSI feedback under AI-SFT can be further reduced to achieve the same CSI feedback accuracy due to the stored accumulated CSI information. Note that, for each slot, only the eigenvectors of the current slot are the input to the AI/ML model.

Figure 5 The procedure of AI/ML-based CSI compression with additional past CSI as input

- Encoder: Similar to AI-SF, the input of the encoder includes eigenvectors for N subbands. Different from AI-SF, the encoder for AI-SFT can store and utilize the accumulated CSI information at encoder for further CSI compression due to the LSTM layers. Specifically, the compressed CSI can be formulated as VEn=fencoder(Vt|Wt-1), where fencoder(∙) represents the function of the encoder. Wt-1 represents the accumulated CSI information at encoder of time t-1 (t = 1,2,3,…), which are already stored by the encoder fencoder(∙).

- Quantizer: The quantizer at the UE side maps the compressed CSI of a floating-point vector to a quantized bit sequence. Scalar quantization, vector quantization, etc., can be adopted. In our simulation, vector quantization is used. The quantized CSI feedback can be formulated as  VQ=fquantizer(VEN).

- De-Quantizer: The de-quantizer recovers the compressed CSI from the feedback CSI bit sequence and sends it as the input to the decoder. The de-quantized CSI can be formulated as  VDe=fdequantizer( VQ).

- Decoder: The decoder recovers the eigenvectors for N subbands. Different from AI-SF, the decoder for AI-SFT can store and utilize accumulated CSI information at decoder for CSI reconstruction, where the accumulated CSI information is mostly synchronized with the encoder part. By considering a long observation window, occasionally missing CSI feedbacks (e.g., due to UCI missing) would not impact the whole performance seriously, although the performance of the nearest occasion will be inferior to AI-SF. Specifically, the recovered eigenvectors can be formulated as V=fdecoder(VDe|Wt-1), where fdecoder(∙) represents the function of the decoder. Wt-1 denotes the accumulated CSI information at decoder of time t-1, which are stored by the decoder fdecoder(∙). Note that, for each slot, only the de-quantized CSI of the current slot (VDe) is needed for compression.

## 3.2 Evaluation results for 1-on-1 joint training

This section provides the evaluation results of the CSI compression at spatial-frequency domain based on joint training with ideal model pairing and ideal quantization of the dataset. In the simulation of this section, 800K training samples are used and the SGCS is caculated in the system level simulation with the same dataset as the throughput evaluation. Both rank=1 with fixed rank and rank=2 with rank adaptation are considered, where for rank=2, Option 3-1 (layer common and rank common) in Section 2.3.3 is applied for inference of each layer separately to derive the compressed CSI per layer.

### 3.2.1 CSI compression with current CSI as input

As mentioned previously, weighted average CSI payload of each rank by rank distribution are used as the final CSI payload for rank>2 cases. For AI-SF, separate AI/ML models with CSI payload size 60, 120, 168 and 240 bits are evaluated. Layer 1 and layer 2 use the same AI/ML model for per CSI payload size and thus the maximum CSI payloads for rank 2 are 120, 240, 336 and 480 bits. The rank distribution {rank 1, rank 2} for different schemes varies from {70%, 30%} to {56%, 44%}. Therefore, the weighted average CSI payloads for Rel-16 Type II codebook are 76, 114, 144, 224, 303 and 380 bits, while the weighted average CSI payloads for AI-SF are 80, 163, 234 and 341 bits.

Figure 6 illustrates the comparison of SGCS between AI-SF and Rel-16 Type II codebook under the rank=1 case and the rank=2 case. It can be seen that, AI-SF outperforms Rel-16 Type II codebook in terms of SGCS for each rank, indicating higher accuracy of CSI recovery by AI-SF. In addition, the accuracy of the 1st rank outperforms the accuracy of the 2nd rank for both AI-SF and Rel-16 Type II since the eigenvectors of the 1st rank is sparser than the 2nd rank, so the SGCS performance is higher. As the AI-SF can better learn the characteristics of the eigenvectors especially for the less sparse enviroment, i.e. the 2nd layer, the AI-SF can achieve more performance gain over the Rel-16 Type II codebook on the 2nd layer than on the 1st layer. For example, with 80 bits CSI payload, AI-SF can provide 0.06 SGCS gain over Rel-16 Type II codebook on the 1st layer and 0.08 SGCS gain on the 2nd layer.

| Rank 1 | Rank 2 |
| --- | --- |

Figure 6  SGCS between AI/ML-based output CSI and the target CSI

The system level simulation results of the average throughput for AI-SF using full buffer traffic are illustrated in Figure 7. It is illustrated that, with the same overhead of CSI feedback, AI-SF has a performance gain of 7%-10.73% over Rel-16 Type II codebook in terms of the throughput under rank=1, while it has a performance gain of 8.9%-14.8% over Rel-16 Type II codebook under rank=2.

| Rank 1 | Rank 2 |
| --- | --- |

Figure 7  Throughput gain over Rel-16 Type II codebook for full buffer traffic

Observation 4: With the same overhead of CSI feedback and in terms of cell average throughput, AI/ML-based CSI compression with spatial-frequency domain CSI as input can outperform Rel-16 Type II codebook with

- 7%-10.3% gain on rank 1 under full buffer traffic.

- 8.9%-14.8% gain on rank 2 under full buffer traffic.

Observation 5: For the rank=2 case, the 1st layer achieves higher SGCS than that of the 2nd layer as the eigenvectors of the 2nd layer are more sparse, and AI/ML-based CSI compression with spatial-frequency domain CSI as input can achieve more gains over the Rel-16 Type II on the 2nd layer than on the 1st layer.

The system level simulation results of mean UPT and 5% UPT using FTP traffic are illustrated in Figure 8 and Figure 9, respectively. It is illustrated that, AI-SF can achieve higher gain for higher RU compared to low RU; this may because the network benefits more from the accurate CSI for MU pairing under heavy traffic load. In addition, AI-SF can achieve higher gain for 5% UPT compared to average UPT; this may because the UE with lower SINR can benefit more on the improvement of the accurate CSI and the accurate DL precoding accordingly, as opposed to the cell center UEs which already have high MCS with limited margin to further improvement. In terms of the mean UPT, AI-SF has a performance gain of 4.8%-8.8% over Rel-16 Type II codebook under rank=1 and RU=80%, while it has a performance gain of 7.5%-14.5% over Rel-16 Type II codebook under rank=2 and RU=80%. In terms of the 5% UPT, AI-SF has a performance gain of 7.1%-16.3% over Rel-16 Type II codebook under rank=1 and RU=80%, while it has a performance gain of 9.1%-23.7% over Rel-16 Type II codebook under rank=2 and RU=80%.

On the other hand, Figure 10 shows that, for achieving the same throughput, AI-SF requires less feedback overhead, with an overhead reduction of about 42% in high range and 46% in medium range for rank = 1, and 48% in high range and 43% in medium range for rank = 2.

Figure 8 Mean UPT gain over Rel-16 Type II codebook for FTP traffic (left: rank=1; right: rank=2)

Figure 9 5% UPT gain over Rel-16 Type II codebook for FTP traffic (left: rank=1; right: rank=2)

Figure 10 Overhead reduction over Rel-16 codebook for FTP traffic (left: rank=1; right: rank=2)

Observation 6: With the same overhead of CSI feedback and in terms of mean UPT, AI/ML-based CSI compression with spatial-frequency domain CSI as input can outperform Rel-16 Type II codebook with

- 2.2%-2.5% gain on rank 1 under FTP traffic and 50% RU.

- 3.2%-7.8% gain on rank 1 under FTP traffic and 70% RU.

- 4.8%-8.8% gain on rank 1 under FTP traffic and 80% RU.

- 5.4%-10.1% gain on rank 2 under FTP traffic and 50% RU.

- 7.2%-14.4% gain on rank 2 under FTP traffic and 70% RU.

- 7.5%-14.5% gain on rank 2 under FTP traffic and 80% RU.

Observation 7: With the same overhead of CSI feedback and in terms of 5% UPT, AI/ML-based CSI compression with spatial-frequency domain CSI as input can outperform Rel-16 Type II codebook with

- 2.2%-7% gain on rank 1 under FTP traffic and 50% RU.

- 6.4%-14.6% gain on rank 1 under FTP traffic and 70% RU.

- 7.1%-16.3% gain on rank 1 under FTP traffic and 80% RU.

- 4.3%-13.3% gain on rank 2 under FTP traffic and 50% RU.

- 8.1%-18% gain on rank 2 under FTP traffic and 70% RU.

- 9.1%-23.7% gain on rank 2 under FTP traffic and 80% RU.

Observation 8: AI/ML-based CSI compression with spatial-frequency domain CSI as input can achieve more obvious gains over the Rel-16 Type II codebook on high RU than low RU.

Observation 9: AI/ML-based CSI compression with spatial-frequency domain CSI as input can achieve more obvious gains over the Rel-16 Type II codebook for cell edge UPT than average UPT.

Observation 10: For achieving the same throughput, AI/ML-based CSI compression with spatial-frequency domain CSI as input requires less feedback overhead than Rel-16 Type II codebook, with an overhead reduction of

- 42% in high overhead range and 46% in medium overhead range for rank = 1.

- 48% in high overhead range and 43% in medium overhead range for rank = 2.

### 3.2.2 CSI compression with additional past CSI as input

The following part provides the evaluation results of the AI-SFT compression scheme, where rank=1 and rank=2 with rank adaptation is considered. Figure 11 illustrates the SGCS of AI-SFT with feedback overhead of 60bits, 120bits and 240bits. It can be seen that AI-SFT has improved SGCS over Rel-16 Type II codebook. Figure 12 illustrates that AI-SFT can provide 18.3%-25.4% gain for rank=1 and 23.3%-30.2% gain for rank=2 over Rel-16 Type II codebook in terms of throughput under full buffer traffic. Figure 13 illustrates that AI-SFT can provide 7.7%-14.9% gain for rank=1 and 16.6%-28.6% gain for rank=2 over Rel-16 Type II codebook in terms of mean UPT under FTP traffic. Figure 14 illustrates that AI-SFT can provide 10%-28.4% gain for rank=1 and 17%-39.2% gain for rank=2 over Rel-16 Type II codebook in terms of 5% UPT under FTP traffic.

As shown in Figure 15, the UPT corresponding to the ideal CSI is provided as the upper bound and the performances of Rel-16 Type II codebook, AI-SF, and AI-SFT are provided as the ratio to the ideal CSI. It can be seen that for rank=2, compared to the mean UPT corresponding to the ideal CSI and under the payload size of 80/163/341 bits, respectively: Rel-16 Type II codebook has 37%/25%/19% margin; AI/ML-based CSI compression with spatial-frequency domain CSI as input has 28%/19%/10% margin; AI/ML-based CSI compression with additional past CSI as input has 19%/13%/5% margin.

On the other hand, Figure 15 also shows that, for achieving the same throughput, AI-SFT requires lesser feedback overhead than both Rel-16 Type II codebook and AI-SF, with an overhead reduction of about 73% in high range for rank = 1, and 70% in high range for rank = 2.

| Rank 1 | Rank 2 |
| --- | --- |

Figure 11  SGCS between AI/ML-based output CSI and the target CSI

| Rank 1 | Rank 2 |
| --- | --- |

Figure 12  Throughput gain over Rel-16 Type II codebook for full buffer traffic

| Rank 1 | Rank 2 |
| --- | --- |

Figure 13 Mean UPT gain over Rel-16 Type II codebook for FTP traffic at 80% RU

| Rank 1 | Rank 2 |
| --- | --- |

Figure 14 5% UPT gain over Rel-16 Type II codebook for FTP traffic at 80% RU

| Rank 1 | Rank 2 |
| --- | --- |

Figure 15 UPT performance compared with ideal CSI for FTP traffic at 80% RU

Observation 11: With the same overhead of CSI feedback and in terms of cell average throughput, AI/ML-based CSI compression with additional past CSI as input can outperform Rel-16 Type II codebook with

- 18.3%-25.4% gain on rank 1 under full buffer traffic.

- 23.3%-30.2% gain on rank 2 under full buffer traffic.

Observation 12: With the same overhead of CSI feedback and in terms of mean UPT, AI/ML-based CSI compression on spatial-frequency domain with additional past CSI as input can outperform Rel-16 Type II codebook with

- 7.7%-14.9% gain on rank 1 under FTP traffic and 80% RU.

- 16.6%-28.6% gain on rank 2 under FTP traffic and 80% RU.

Observation 13: With the same overhead of CSI feedback and in terms of 5% UPT, AI/ML-based CSI compression on spatial-frequency domain with additional past CSI as input can outperform Rel-16 Type II codebook with

- 10%-28.4% gain on rank 1 under FTP traffic and 80% RU.

- 17%-39.2% gain on rank 2 under FTP traffic and 80% RU.

Observation 14: With the same overhead of CSI feedback and in terms of mean UPT for rank 2, compared to the mean UPT corresponding to the ideal CSI,

- Rel-16 Type II codebook has 37%/25%/19% margin under the payload size of 80/163/341 bits, respectively.

- AI/ML-based CSI compression with spatial-frequency domain CSI as input has 28%/19%/10% margin under the payload size of 80/163/341 bits, respectively.

- AI/ML-based CSI compression with additional past CSI as input has 19%/13%/5% margin under the payload size of 80/163/341 bits, respectively.

Observation 15: For achieving the same throughput, AI/ML-based CSI compression on spatial-frequency domain with additional past CSI as input requires less feedback overhead than Rel-16 Type II codebook, with an overhead reduction of

- 73% in high overhead range for rank = 1.

- 70% in high overhead range for rank = 2.

## 3.3 Evaluation results for multi-vendor joint training

This section provides the evaluation results of training Type 1/2 with multi-vendor cases for CSI compression at spatial-frequency domain. The detailed training procedure is elaborated in our companion contribution [2]. For the training order, simultaneously train all N Network part models and M UE part models involving all vendors is adopted in this section.

Support of one Network part model to M>1 UE part models

In this simulation, one Network part model to M=3 different UE part models are considered and the payload size of CSI feedback is 240 bits. The AI/ML models of UE 1, UE 2 and Network 1 are assumed to have the same backbone (Transformer backbone) but different structures (different number of layers), while the model of UE 3 has a different backbone (CNN backbone). Case 2 of Figure 16 shows the evaluation case of training Type 2 and all of the 3 UE-Network pairs (UE 1~3 and Network 1) are simultaneously trained. As analyzed in Section 2.3.3, 3 different UE-Network pairs with restricted model pairing (same model structure as Case 2 for each UE-Network pair) are used as baseline for performance comparison, shown as Case 1.

Figure 16 Simulation cases of training Type 2 with one Network part model to M>1 UE part models

Table 2 SGCS of training Type 2 with one Network part model to M>1 UE part models

|  | Case 1 | Case 2 |
| --- | --- | --- |
| UE1-NW1 | 0.918 | 0.904 (-1.4%) |
| UE2-NW1 | 0.913 | 0.902 (-1.1%) |
| UE3-NW1 | 0.857 | 0.792 (-6.5%) |

Table 2 shows the evaluation results of the two cases. For the UE-Network pairs with the same model backbone but different structures (UE1-NW1 and UE2-NW1), the SGCS loss comparing to the 1 UE to 1 Network joint training is 1.1% and 1.4%, respectively, while for the UE-Network pair with different model backbones (UE3-NW1), the SGCS loss comparing to the 1 UE to 1 Network joint training is 6.5%.

Observation 16: For AI/ML-based CSI compression, compared to the joint training of one Network part model to one UE part model, the SGCS loss of training Type 2 is 1.1%~6.5% for one Network part model to M=3 UE part models with different backbones or different structures.

Support of N>1 Network part model to M>1 UE part models

In this simulation, N=2 Network part models to M=2 different UE part models are considered. The AI/ML models of UE 1, UE 2, Network 1 and Network 2 are assumed to have the same backbone (Transformer backbone) but different structures (different number of layers). Case 2 of Figure 17 shows the evaluation case of training Type 2 and all of the 4 models (UE 1, UE 2, Network 1 and Network 2) are jointly trained using simultaneous training method, i.e., Case A as discussed in Section 2.3.2. Similarly, 4 combinatorial UE-Network pairs with restricted model pairing (same model structure as Case 2 for each UE-Network pair) are used as baselines for performance comparison, shown as Case 1.

Figure 17 Simulation cases of training Type 2 with N>1 Network part models to M>1 UE part models

Table 3 SGCS of training Type 2 with N>1 Network part models to M>1 UE part models

|  | 60 bits | 120 bits | 240 bits |
| --- | --- | --- | --- |
| Case 1 UE1-NW1 | 0.767 | 0.861 | 0.918 |
| Case 1 UE2-NW1 | 0.759 | 0.844 | 0.913 |
| Case 1 UE1-NW2 | 0.764 | 0.852 | 0.918 |
| Case 1 UE2-NW2 | 0.741 | 0.849 | 0.910 |
| Case 2 UE1-NW1 | 0.741 (-2.6%) | 0.841 (-2%) | 0.907 (-1.1%) |
| Case 2 UE2-NW1 | 0.74 (-1.9%) | 0.838 (-0.6%) | 0.904 (-0.9%) |
| Case 2 UE1-NW2 | 0.741 (-2.3%) | 0.838 (-1.4%) | 0.905 (-1.3%) |
| Case 2 UE2-NW2 | 0.74 (-0.1%) | 0.835 (-1.4%) | 0.903 (-0.7%) |

Table 3 shows the evaluation results of the two cases. For the UE-Network pair with aligned model backbone, the SGCS loss comparing to the 1 UE to 1 Network joint training is 0.1%~2.6%.

Observation 17: For AI/ML-based CSI compression, compared to the joint training of one Network part model to one UE part model, the SGCS loss of training Type 2 is 0.1%~2.6% for N=2 Network part models to M=2 UE part models with different structures.

## 3.4 Evaluation results for separate training

This section provides the evaluation results of training Type 3 for CSI compression at spatial-frequency domain. The detailed procedure of separate training scheme is elaborated in our companion contribution [2]. In the simulations of this section, 300K training samples and 15K testing samples are used.

### 3.4.1 NW first separate training

Support of one Network part model to M>1 UE part models

One common CSI reconstruction part to multiple CSI generation parts of different UEs is natural supported for NW first training, since the dataset generated by one Network side CSI generation part can be delivered to multiple UE sides to train multiple CSI generation parts independently. From evaluation perspective, there is nothing different between the cases of one CSI reconstruction part to one CSI generation part and one common CSI reconstruction part to multiple CSI generation parts.

In the following we simulate the case of one Network part model to M=4 different UE part models as shown in Case 2~5 (UE A~UE D) and Case 7~10 (UE A’~UE D’) described in the following tables, where the UE part models in Case 2/7 adopt the same structure with the Network side CSI generation part, and the UE part models in Case 3~5/8~10 adopt a different backbone/structure from the Network side CSI generation part. Case 1 and Case 6 are assumed as 1 Network vs 1 UE joint training with ideal/restricted model pairing as a reference.

The UEs use the same dataset as the Network to train the UE side CSI generation part for Case 2~4 and Case 7~9, which means Network share both the input (original CSI) and output (CSI feedback) of the Network side CSI generation part. For Case 6 and Case 10, the UE only uses its own original CSI as inputs (which is a subset of the input CSI for Network side collected from multiple UEs) and the corresponding CSI feedback shared by Network as labels to train the UE side CSI generation part, i.e., Network shares only the output of the Network side CSI generation part to the UE.

Figure 18 SGCS of separated training with Transformer as backbone of Network part

Table 4 Case description of separated training with Transformer as backbone of Network part

| Case | Training type | Dataset used at UE | CSI generation part at UE | CSI generation/reconstruction part at Network |
| --- | --- | --- | --- | --- |
| 1 | Joint training | 300K | Transformer | Transformer |
| 2 (UE A) | Separate training | Same as Case 1 | Transformer, same structure as Case 1 | Transformer |
| 3 (UE B) | Separate training | Same as Case 1 | Transformer, transformer layers are 30% less than Case 1 | Transformer |
| 4 (UE C) | Separate training | Same as Case 1 | Transformer, attention heads are 50% less than Case 1 | Transformer |
| 5 (UE D) | Separate training | 100K (subset of Case 1) | Transformer, same structure as Case 1 | Transformer |

Figure 19 SGCS of separated training with CNN as backbone of Network part

Table 5 Case description of separated training with CNN as backbone of Network part

| Case | Training type | Dataset used at UE | CSI generation part at UE | CSI generation/reconstruction part at Network |
| --- | --- | --- | --- | --- |
| 6 | Joint training | 300K | CNN | CNN |
| 7 (UE A’) | Separate training | Same as Case 6 | CNN, same structure as Case 1 | CNN |
| 8 (UE B’) | Separate training | Same as Case 6 | CNN, CNN layers are 6% less than Case 1 and CNN channels are 6% less than Case 1 | CNN |
| 9 (UE C’) | Separate training | Same as Case 6 | Transformer | CNN |
| 10 (UE D’) | Separate training | 100K (subset of Case 6) | CNN, same structure as Case 1 | CNN |

Based on the results of Case 1~5 as shown in Figure 18, it can be found that the SGCS margin between separate training and joint training is less than 0.5% when the UE-side CSI generation part and the Network side CSI generation part have a same backbone but same or different structures.

Based on the results of Case 6~10 as shown in Figure 19, it can be found that the SGCS margin between separate training and joint training is less than 0.5% even when the UE-side CSI generation part and the Network side CSI generation part have different backbones but the UE-side CSI generation part has similar or stronger learning capability than the Network side CSI generation part.

Based on the results of Case 1 and Case 5 in Figure 18, or Case 6 and Case 10 in Figure 19, it can be found that the SGCS margin between separate training and joint training is less than 0.5% even when the dataset of UE side is only a subset of the dataset of the Network side.

Comparing Case 1 with Case 6, Transformer has better performance than CNN which is because the learning capability of Transformer is more excellent than CNN, as analyzed in Section 2.3.3. For the same reason, Transformer-based CSI generation part at UE side trained by the dataset generated by CNN-based CSI generation part at Network side (i.e., Case 9) can achieve even slightly better performance than the joint training baseline of both CNN models (i.e., Case 6), but not the other way around.

In general, if the backbone of UE-side CSI generation part has similar or stronger learning capability than that of virtual Network side CSI generation part, the performance margin between the separate training and the joint training is <0.5%, which is minor gap. Although for the separate training, it may not always be the case that the UE-side CSI generation part outperforms the virtual Network side CSI generation part, the learning capabilities between Network side model and UE side model may easily aligned to some extent by, e.g., sharing the target performance metric of the models in together with the shared dataset.

Observation 18: For AI/ML-based CSI compression, there is only minor margin (<0.5%) between the SGCS of the NW first separate training and the SGCS of the joint training if the backbone of UE-side CSI generation part has similar or stronger learning capability than that of Network side CSI generation part.

- This observation applies regardless when the input dataset for the UE side is only a subset of or equal to the input dataset for the Network side.

Observation 19: For separate training, good performance can be achieved by pairing with comparable learning capabilities between Network side model and UE side model, which may be easily aligned by, e.g., sharing the target performance metric of the models in together with the shared dataset.

Support of one UE part model and N>1 separate Network part models

The evaluation cases for N>1 CSI reconstruction parts to one common CSI generation part at UE are given in the following table, and the corresponding results are shown in Figure 20. In this evaluation, UE uses a mixture of N=2 datasets generated by two different Networks to train a common CSI generation part.

For Case 1 and Case 2, joint training is applied to UE and Network with the same backbone, while the Network part model in Case 1 and the Network part model in Case 2 are subject to different Transformer structures, denoted by Transformer A and Transformer B; the paired UE part models are denoted as Transformer A’ and Transformer B’, respectively. For Case 3 and Case 4, one UE part model matches with two Network part models with Transformer A and Transformer B, respectively, and the common CSI generation part at UE is also Transformer-based (denoted as Transformer C) but with different from neither Transformer A’ nor Transformer B’.

For Case 5 and Case 6, the CSI generation parts and CSI reconstruction parts of the two Networks are MLP-based and CNN-based, respectively. For case 7 and 8, the common CSI generation part at UE is Transformer-based, matching with the two Network part models in Case 5 and 7. Since the data distributions of the outputs over different CSI generation parts are different even under same input dataset, to better match with the two different CSI reconstruction parts, adaptation layers are applied for the common CSI generation part at UE for different Networks, which similar to the design for supporting scalability over payload sizes as described in Section 3.6.

Table 6 Case description of N=2 Network part models to one common UE part model

| Case | Training type | CSI generation part at UE | CSI reconstruction part at Network |
| --- | --- | --- | --- |
| 1 | Joint training, one CSI reconstruction part to one CSI generation part | Transformer A’ | Transformer A |
| 2 | Joint training, one CSI reconstruction part to one CSI generation part | Transformer B’, transformer layers are 30% less than Case 1 | Transformer B |
| 3 | Separate training, 2 reconstruction parts of different Networks to one common CSI generation part | Transformer C, attention heads are 50% less than Case 1 | Transformer A, same as Case 1 |
| 4 |  |  | Transformer B, same as Case 2 |
| 5 | Joint training, one CSI reconstruction part to one CSI generation part | MLP | MLP |
| 6 | Joint training, one CSI reconstruction part to one CSI generation part | CNN | CNN |
| 7 | Separate training, 2 reconstruction parts of different Networks to one common CSI generation part | Transformer | MLP, same as Case 5 |
| 8 |  |  | CNN, same as Case 6 |

(a) Same backbone, different structures

(b) Different backbones

Figure 20 SGCS for N=2 Network part models to one common UE part model

Based on the results, it can be found that the SGCS margin between separate training for multiple CSI reconstruction parts to one common CSI generation part and joint training is less than 0.5%.

Observation 20: For AI/ML-based CSI compression, there is only minor margin (<0.5%) between the SGCS of the NW first separate training and the SGCS of the joint training when the separate training considers N>1 Network part models to one common UE part model with a backbone with similar or stronger learning capability than any of the N Network part models.

### 3.4.1 UE first separate training

Support of one UE part model to N>1 Network part models

We simulate the cases of one UE part model to N=4 different Network part models as shown in Case 2~5 (Network A~ Network D) and Case 7~10 (Network A’~ Network D’) described in the following tables, where the Network part models in Case 2/7 adopt the same structure with the UE side CSI reconstruction part, and the Network part models in Case 3~5/8~10 adopt a different backbone/structure from the UE side CSI reconstruction part. Case 1 and Case 6 are assumed as 1 Network vs 1 UE joint training with ideal/restricted model pairing as a reference.

The Networks use the same dataset as the UE to train the Network side CSI reconstruction part for Case 2~4 and Case 7~9, which means UE shares both the input (CSI feedback) and labels (original CSI) of the UE side CSI reconstruction part. For Case 6 and Case 10, the Network only uses its own CSI as labels (which is a subset of the dataset for UE side collected from multiple Networks) and the corresponding CSI feedback shared by UE as inputs to train the Network side CSI reconstruction part.

Based on the results of Case 1 and Case 5 in Figure 21, or Case 6 and Case 10 in Figure 22, it can be found that the SGCS margin between separate training and joint training is less than 1% even when the dataset of UE side is only a subset of the dataset of the Network side.

Figure 21 SGCS of separated training with Transformer as backbone of UE part

Table 7 Case description of separated training with Transformer as backbone of UE part

| Case | Training type | Dataset used at UE | CSI generation/reconstruction part at UE | CSI reconstruction part at Network |
| --- | --- | --- | --- | --- |
| 1 | Joint training | 300K | Transformer | Transformer |
| 2 (Network A) | Separate training | Same as Case 1 | Transformer | Transformer, same structure as Case 1 |
| 3 (Network B) | Separate training | Same as Case 1 | Transformer | Transformer, transformer layers are 25% less than Case 1 |
| 4 (Network C) | Separate training | Same as Case 1 | Transformer | Transformer, attention heads are 50% less than Case 1 |
| 5 (Network D) | Separate training | 100K (subset of Case 1) | Transformer | Transformer, same structure as Case 1 |

Figure 22 SGCS of separated training with CNN as backbone of UE part

Table 8 Case description of separated training with CNN as backbone of UE part

| Case | Training type | Dataset used at UE | CSI generation/reconstruction part at UE | CSI reconstruction part at Network |
| --- | --- | --- | --- | --- |
| 6 | Joint training | 300K | CNN | CNN |
| 7 (Network A’) | Separate training | Same as Case 1 | CNN | CNN, same structure as Case 6 |
| 8 (Network B’) | Separate training | Same as Case 1 | CNN | CNN, CNN layers are 6% less than Case 6 and CNN channels are 6% less than Case 6 |
| 9 (Network C’) | Separate training | Same as Case 1 | CNN | Transformer |
| 10 (Network D’) | Separate training | 100K (subset of Case 6) | CNN | CNN, same structure as Case 6 |

Observation 21: For AI/ML-based CSI compression, there is minor margin (<1%) between the SGCS of the UE first separate training and the SGCS of the joint training even if the backbone of Network-side CSI reconstruction part has similar or stronger learning capability than that of UE side CSI reconstruction part.

- This observation applies regardless when the dataset for the Network side is only a subset of or equal to the dataset for the UE side

## 3.5 Evaluation results for generalization performance

This section provides the evaluation results of the generalization over scenarios for CSI compression with spatial-frequency domain CSI as input. The CSI feedback payload size is 240 bits in this section.

Generalization over channel models

Table 9 shows the generalization performances on various channel models, and the generalization is verified from the perspective of the dataset composition (Case 1/2/3) and fine-tuning. For Case 1 and Case 2, the training dataset size for UMa, UMi and InH cases is 300K for each. For Case 3, the mixed dataset contains sub-datasets of UMa, UMi and InH, each of which is of 100K samples. For fine-tuning, the initial AI/ML model is trained based on an InH dataset of 300K samples, and the fine-tuning is performed based on an UMa dataset of 25K samples. The size of the testing dataset is 60K samples for each of UMa/UMi/InH for Case 1/2/3/fine-tuning.

The results show that the characteristics of UMa and UMi are similar, and the AI/ML model trained by UMa/UMi dataset can be used for each other with generalized performance; the AI/ML model trained by UMa/UMi dataset can also be applied for testing at InH scenario with minor loss. On the other hand, the AI/ML model trained by InH dataset provides good performance for the InH scenario but poor performance for the UMa/UMi scenario (i.e., Case 2), since the channel characteristics under InH are less diverse than UMa/UMi, so that the characteristic of the UMa/UMi are not well learnt for the AI/ML model trained by InH. The AI/ML model trained by the mixed dataset (i.e., Case 3) shows moderate performance on each of the UMa/UMi/InH testing dataset, but compared with the overfitting dataset (i.e., Case 1), there is still a gap.

The AI/ML model trained by the dataset of InH shows poor performance if it is directly applied for the testing dataset of UMa. However, after fine-tuned by a dataset of UMa channel, the performance can be improved obviously. This demonstrates the benefit of fine-tuning.

Table 9 Generalization performances on channel models

| Testing | Training, channel model |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | UMa | UMi | InH | Mixed | InH, fine-tuned with UMa |
| UMa | 0.916 (Case 1) | 0.911 (Case 2) | 0.855 (Case 2) | 0.909 (Case 3) | 0.889 (fine-tuning) |
| UMi | 0.91 (Case 2) | 0.909 (Case 1) | 0.859 (Case 2) | 0.895 (Case 3) | \ |
| InH | 0.962 (Case 2) | 0.96 (Case 2) | 0.968 (Case 1) | 0.965 (Case 3) | \ |

Observation 22: For an AI/ML model of CSI compression trained under Scenario#A (e.g., InH) dataset but applied to Scenario#B (e.g., UMa/UMi) for inference (i.e., generalization Case 2), its performance may be degraded compared to inference under Scenario#A (i.e., generalization Case 1), but mixing the dataset over the three scenarios for AI/ML model training (i.e., generalization Case 3) is helpful to improve the generalization.

Observation 23: For an AI/ML model of CSI compression trained under Scenario#A (e.g., InH) dataset but applied to Scenario#B (e.g., UMa) for inference, its performance can be improved by applying fine-tuning using a relatively small dataset from Scenario#B.

Generalization over indoor/outdoor UEs

Table 10 shows the generalization performances on various indoor/outdoor UE distributions. The size of each training dataset is 300K, and the size of the testing dataset is 60K for each of indoor/outdoor. The results show that AI/ML models trained by any indoor/outdoor UE distribution performs similarly on outdoor testing dataset. On the other hand, with the decrease of the ratio of indoor UEs (i.e., O2I channel samples) in the training dataset, the performance on indoor testing dataset (wherein all UEs are subject to indoor UEs) becomes worse. This is because the characteristics of O2I channels are more diverse for learning due to penetration, scattering, etc., than outdoor only.

Table 10 Generalization performances on indoor/outdoor UE distribution

| Testing | Training, Indoor/outdoor ratio |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 10:0 | 8:2 | 5:5 | 2:8 | 0:10 |
| Indoor (O2I) | 0.916 (Case 1) | 0.914 (Case 3) | 0.914 (Case 3) | 0.909 (Case 3) | 0.892 (Case 2) |
| Outdoor | 0.949 (Case 2) | 0.948 (Case 3) | 0.949 (Case 3) | 0.949 (Case 3) | 0.948 (Case 1) |

Observation 24: For generalization of AI/ML-based CSI compression over indoor/outdoor UE distribution ratios,

- AI/ML model trained by any indoor/outdoor UE distribution ratio shows similar performance on the outdoor testing dataset.

- With the decrease of the indoor channel ratio for the training dataset, the trained AI/ML model brings decreased performance on the indoor testing dataset.

Generalization over TxRU mappings

Table 11 shows the generalization performances on various TxRU mappings. The size of each training dataset is 300K, and the size of the testing dataset is 60K for each of TxRU mapping scenario. The results show that AI/ML models trained by TxRU mapping scenario (8,8,2,1,1,2,8) can provide good performance on both (8,8,2,1,1,2,8) and (2,8,2,1,1,2,8). On the other hand, AI/ML models trained by TxRU mapping scenario (2,8,2,1,1,2,8) can provide excellent performance on same scenario but relatively poorer performance on (8,8,2,1,1,2,8). This is because the more antenna elements leads to more complicated channel characteristics. The AI/ML model trained by the mixed dataset shows moderate performance on each of the TxRU mapping scenarios.

Table 11 Generalization performances on TxRU mapping

| Testing | Training, TxRU mapping |  |  |
| --- | --- | --- | --- |
|  | (8,8,2,1,1,2,8) | (2,8,2,1,1,2,8) | Mixed |
| (8,8,2,1,1,2,8) | 0.916 (Case 1) | 0.89 (Case 2) | 0.913 (Case 3) |
| (2,8,2,1,1,2,8) | 0.914 (Case 2) | 0.922 (Case 1) | 0.92 (Case 3) |

Observation 25: For generalization of AI/ML-based CSI compression over various TxRU mapping methods including (8,8,2,1,1,2,8) and (2,8,2,1,1,2,8), the AI/ML model trained by the mixed dataset shows moderate performance on each of the TxRU mapping methods.

## 3.6 Evaluation results for scalability performance

Scalability over different payload sizes

Figure 23 shows the method to achieve scalability over different payloads by adaptation layer adopted in this evaluation. A pair of scalable CSI generation part and scalable CSI reconstruction part is evaluated, where the CSI generation part includes a common part and 4 output adaptation layers corresponding to 4 different payload sizes (Y1/Y2/Y3/Y4=60/120/168/240 bits), and the CSI reconstruction part includes one common part and 4 input adaptation layers paired with the 4 output adaptation layers, respectively. Adaptation layers in the AI/ML models are used to adjust the dimension of CSI generation output and CSI reconstruction input, where different adaptation layers correspond to separate output/input dimensions to be consistent with the CSI payload sizes. In the simulations of this section, 300K training samples and 15K testing samples are used.

Figure 23 Diagram of scalability over different payloads by adaptation layer

Table 12 Scalability performance over different payloads

|  | 60 bits | 120 bits | 168 bits | 240bit |
| --- | --- | --- | --- | --- |
| Payload-specific models (generalization Case 1) | 0.767 | 0.861 | 0.887 | 0.918 |
| Unified model for 4 different payloads (generalization Case 3) | 0.757 (-1%) | 0.855 (-0.6%) | 0.884 (-0.3%) | 0.908 (-1%) |

Table 12 shows the scalability performance over different payloads. The results show that the SGCS margin between payload-specific models and a unified model for 4 different payloads is less than 1%.

Observation 26: For scalability of AI/ML-based CSI compression over various CSI payload sizes, the SGCS margin between the payload-specific models and the unified scalable model supporting 4 different CSI payload sizes is less than 1%.

- Adaptation layers in the AI/ML model are used to achieve the scalability.

Scalability over different antenna port numbers

Figure 24 shows the method to achieve scalability over different antenna port numbers, where a pair of scalable CSI generation part and scalable CSI reconstruction part is considered and truncation/padding is adopted to achieve the scalability in this evaluation. The input dimension of the CSI generation part is set as up to the maximum antenna port number (32), and zeroes are padded to match the input dimension in case of 16 ports input. Symmetrically, the output dimension of CSI reconstruction part is set as up to the maximum antenna port number (32) and the output is truncated from the tail to generate the output of 16 ports.

Figure 24 Diagram of scalability over different antenna port numbers by truncation/padding

Table 13 Scalability performance over different antenna port numbers

| Training | Test | 60 bits | 120 bits | 168 bits | 240bit |
| --- | --- | --- | --- | --- | --- |
| Port-specific models (generalization Case 1) | 16 ports | 0.831 | 0.894 | 0.92 | 0.944 |
|  | 32 ports | 0.767 | 0.861 | 0.887 | 0.918 |
| Unified model for 2 antenna port numbers (generalization Case 3) | 16 port | 0.827 (-0.4%) | 0.892 (-0.2%) | 0.919 (-0.1%) | 0.942 (-0.2%) |
|  | 32 port | 0.766 (-0.1%) | 0.852 (-0.9%) | 0.884 (-0.3%) | 0.913 (-0.5%) |

Table 13 shows the scalability performance over different antenna port numbers. The results show that the SGCS margin between port-specific models and a unified model for 2 different antenna port numbers is less than 0.9%.

Observation 27: For scalability of AI/ML-based CSI compression over various antenna port numbers, the SGCS margin between the port-specific models and the unified scalable model supporting 2 different antenna port numbers is less than 0.9%.

- Pre/post processing of padding/truncation is used to achieve the scalability.

## 3.7 Evaluation results for quantized ground-truth CSI

In this section, evaluation results for the two quantization methods (scalar quantization and quantization by Rel-16 Type II-like codebook generation method with new parameters) for the ground-truth CSI are provided.

Table 14 SGCS for quantized channel information as training dataset

| Quantization method | Size per input/Total overhead, bytes | SGCS |  |  |
| --- | --- | --- | --- | --- |
|  |  | 60 bits | 120 bits | 240 bits |
| Float32 | 3.3K/992M | 0.767 | 0.861 | 0.918 |
| Float16 | 1.67K/499M (-50%) | 0.764 (-0.3%) | 0.855 (-0.6%) | 0.916 (-0.2%) |
| 8 bits scalar quantization | 0.83K/250M (-75%) | 0.762 (-0.5%) | 0.852 (-0.9%) | 0.911 (-0.7%) |
| Rel-16 Type II CB with new parameters: L=10, p=0.9, beta=0.31, amplitude: 4 bits, phase: 6 bits | 127/40M (-96%) | 0.768 (+0.1%) | 0.854 (-0.7%) | 0.917 (-0.1%) |
| PC8 of Rel-16 Type II CB | 41/12M (-97.5%) | 0.76 (-0.7%) | 0.832 (-2.9%) | 0.887 (-3.1%) |

The results show that compared with the training dataset quantized by Float32 format, the training dataset quantized by Rel-16 Type II CB with new parameters can provide only minor performance margin (<0.7%) but reduces 96% overhead for training dataset delivery.

Observation 28: For AI/ML-based CSI compression, compared with the training dataset with Float32 format, other high resolution ground-truth CSI quantization methods with lower overhead show minor SGCS loss with remarkable overhead reduction.

- Training dataset quantized by 8 bits scalar quantization has <0.9% SGCS loss but reduces 75% overhead of training dataset delivery.

- Training dataset quantized by Rel-16 Type II CB with new parameters has <0.7% SGCS loss but reduces 96% overhead of training dataset delivery.

Observation 29: For AI/ML-based CSI compression, compared with the training dataset with Float32 format, training dataset quantized by a legacy quantization resolution of PC8 of Rel-16 Type II CB causes significant SGCS loss of around 3% when the CSI payload size is medium and large.

## 3.8 Evaluation results for quantization aware/non-aware training

This section provides the evaluation results for different quantization methods of CSI feedback, including quantization aware training and quantization non-aware training.

For quantization aware training, the codebook of vector quantization is trained jointly with the CSI generation part and the CSI reconstruction part (i.e., Case 2-2). The parameters of the codebook of vector quantization is updated in each training epoch.

For quantization non-aware training, the CSI generation part and the CSI reconstruction part are trained by using the FP of floating vectors without CSI feedback quantization. And the dictionary/codebook for the vector quantization is generated by applying the clustering algorithm to the outputs of the well-trained CSI generation part.

Table 15 SGCS for quantization aware training and quantization non-aware training

|  | 60 bits | 120 bits | 240bit |
| --- | --- | --- | --- |
| Quantization aware training (Case 2-2) | 0.767 | 0.861 | 0.918 |
| Quantization non-aware training (Case 1) | 0.69 (-7.7%) | 0.738 (-12.3%) | 0.829 (-8.9%) |

Observation 30: For AI/ML-based CSI compression, quantization non-aware training shows significant performance loss compared to quantization aware training.

## 3.9 Evaluation results for monitoring accuracy

Based on the discussions in Section. 2.3.1, this section provides the evaluation results for the monitoring accuracy using intermediate KPI. Three formats of ground-truth CSI are considered:

- PC8 of Rel-16 Type II CB, which adopts the legacy CB resolution and is regarded as a lower bound baseline. The CSI payload of the PC8 setting is 328 bits.

- High resolution ground-truth CSI#1 using Rel-16 Type II CB with new parameters: {L=10, p=0.9, beta=0.31, amplitude quantization: 4 bits, phase quantization: 6 bits}. The CSI payload of high resolution ground-truth CSI#1 is 1014 bits.

- High resolution ground-truth CSI#2 using Rel-16 Type II CB with new parameters: {L=12, p=0.8, beta=0.5, amplitude quantization: 4 bits, phase quantization: 6 bits}. The CSI payload of high resolution ground-truth CSI#2 is 1610 bits.

For the CSI payload size of inference, both a low CSI payload size of 60 bits and a high CSI payload size of 240 bits are evaluated, and rank=1 is assumed.

The PDF and CDF of absolute SGCS gap are shown in Figure 25 and Figure 26, respectively, for PC8 of Rel-16 Type II CB, high resolution ground-truth CSI#1, and high resolution ground-truth CSI#2. From the figures, it can be intuitively observed from the distributions of the SGCS gaps that PC8 of Rel-16 Type II CB has larger variation (i.e., higher ratio of large SGCS gaps) compared to the high resolution ground-truth CSI formats.

To demonstrate the statistical characteristic of the SGCS gap over different ground-truth CSI formats, Metric 2 (i.e., monitoring accuracy) and Metric 3 (i.e., false alarm/misdetection) are also captured in Table 16 and Table 17, respectively. Three threshold values are considered: 0.01, 0.02, and 0.05; considering the resulting SGCS of inference ranges from 0.7 to 0.9 for layer 1 (take 0.8 for average) and ranges from 0.6 to 0.8 for layer 2 (take 0.7 for average), the threshold of +/- 0.01, +/- 0.02, +/- 0.05 contributes a bias ratio of 2.5%, 5%, and 12.5% for layer 1, and 2.9%, 5.7%, and 14.3% for layer 2.

From the tables, it can be seen that the performance in terms of the monitoring accuracy is significantly deteriorated when PC8 is adopted for monitoring: for the 60 bits CSI payload case, only 21%/42%/81% monitoring accuracy can be achieved for threshold=0.01/0.02/0.05, respectively; when the CSI payload increases to 240 bits, the situation is further aggravated to 8%/21%/67%. When a higher resolution ground-truth CSI format with 1014 bits resolution is applied for monitoring, however, the monitoring accuracy can be largely improved to 56%/82%/99% for 60 bits CSI payload and 63%/89%/100% for 240 bits CSI payload. When a 1610 bits resolution ground-truth format is applied for monitoring, the monitoring accuracy can be further improved to 63%/88%/100% for 60 bits CSI payload and 75%/97%/100% for 240 bits CSI payload.

Similar trends can also be observed for false alarm and misdetection, where the false alarm ratio or misdetection ratio are largely decreased when a higher resolution ground-truth CSI format is adopted.

| 60 bits | 240 bits |
| --- | --- |

Figure 25 PDF of SGCS gap between CSI in Float32 format and other ground-truth CSI formats

| 60 bits | 240 bits |
| --- | --- |

Figure 26 CDF of absolute SGCS gap between CSI in Float32 format and other ground-truth CSI formats

Table 16 False alarm and misdetection of various ground-truth CSI formats

| CSI payload size |  | False alarm |  |  | Misdetection |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Threshold of SGCS gap (Threshold_neg/ Threshold_pos for False alarm/Misdetection) | 0.01 | 0.02 | 0.05 | 0.01 | 0.02 | 0.05 |
| 60 bits | PC8 | 41% | 27% | 4% | 38% | 31% | 15% |
|  | 1014 bits | 14% | 4% | 0% | 30% | 14% | 1% |
|  | 1610 bits | 15% | 4% | 0% | 22% | 8% | 0% |
| 240 bits | PC8 | 86% | 75% | 32% | 6% | 4% | 1% |
|  | 1014 bits | 31% | 9% | 0% | 6% | 2% | 0% |
|  | 1610 bits | 19% | 2% | 0% | 6% | 1% | 0% |

Table 17 Monitoring accuracy of various ground-truth CSI formats

| CSI payload size |  | Accuracy |  |  |
| --- | --- | --- | --- | --- |
|  | Threshold of absolute SGCS gap | 0.01 | 0.02 | 0.05 |
| 60 bits | PC8 | 21% | 42% | 81% |
|  | 1014 bits | 56% | 82% | 99% |
|  | 1610 bits | 63% | 88% | 100% |
| 240 bits | PC8 | 8% | 21% | 67% |
|  | 1014 bits | 63% | 89% | 100% |
|  | 1610 bits | 75% | 97% | 100% |

Observation 31: For AI/ML-based CSI compression, the monitoring performance is heavily deteriorated if the Rel-16 Type II CB with legacy resolution (e.g., PC8) is adopted. Introducing higher resolution ground-truth CSI format of Rel-16 Type II CB with new parameters can significantly improve the monitoring performance in terms of monitoring accuracy, false alarm, and misdetection. E.g., for threshold of absolute SGCS gap = 0.01/0.02/0.05, respectively:

- When PC8 is adopted for monitoring, only 21%/42%/81% monitoring accuracy is achieved for the 60 bits CSI payload, and 8%/21%/67% monitoring accuracy is achieved for the 240 bits CSI payload.

- When 1014 bits high resolution ground-truth CSI is adopted for monitoring, the monitoring accuracy can be improved to 56%/82%/99% for 60 bits CSI payload and 63%/89%/100% for 240 bits CSI payload.

- When 1610 bits high resolution ground-truth CSI is adopted for monitoring, the monitoring accuracy can be improved to 63%/88%/100% for 60 bits CSI payload and 75%/97%/100% for 240 bits CSI payload.

Proposal 12: For AI/ML-based CSI compression, consider the UE report of high resolution ground-truth CSI using Rel-16 Type II CB with new parameters for the monitoring of intermediate KPI at the Network side.

- Example 1: New parameters of Rel-16 Type II CB are {L=10, p=0.9, beta=0.31, amplitude quantization: 4 bits, phase quantization: 6 bits}.

- Example 2: New parameters of Rel-16 Type II CB are {L=12, p=0.8, beta=0.5, amplitude quantization: 4 bits, phase quantization: 6 bits}.

# Evaluations for CSI prediction

In this section, evaluations for CSI prediction will be discussed, including AI/ML model description and evaluation results.

## 4.1 AI/ML model description

The AI/ML-based CSI prediction is used to predict future CSI based on historic CSI. As shown in Figure 27, the input of the CSI predictor includes k historic channel matrixes which are obtained from the k historic CSI-RS, respectively. The output of the CSI predictor is the predicted channel matrixes at moment of the nearest future CSI-RS. In our simulation, k is set to 4 and a fully-connected network is used. Therefore, the observation window is the latest k=4 observation instances with 5 slots distance to each other, while the prediction window is 1 future slot. The frequency domain size of input/output channel matrix is 1RB in this simulation, after which the subband CSIs to be reported are calculated based on the predicted CSI per RB.

Figure 27 The structure of AI/ML-based CSI prediction

## 4.2 Performance comparison with benchmarks

This section provides the evaluation results of CSI prediction. In this simulation, the interval of CSI-RS is 5ms and the UE speed is 60km/h. Two baselines are evaluated for comparison, wherein one is sample-and-hold scheme (without CSI prediction) and another is non-AI/ML-based CSI prediction. Auto-regression (AR) method is adopted for non-AI/ML-based CSI prediction. The input/output type of CSI prediction is channel matrix, and SGCS is considered as the performance metric. SGCS is calculated based on the eigenvectors of the first layer decomposed from the predicted channel matrix, and the formula of SGCS calculation is same as the CSI compression sub use case.

Table 18 shows the SGCS performances of the AI/ML-based CSI prediction and the two benchmarks. For AI/ML-based CSI prediction and non-AI/ML-based CSI prediction, SGCS is calculated based on the eigenvectors corresponding to the output of the AI/ML model (i.e., predicted CSI for the target future slot) and the corresponding ground-truth eigenvectors of the same target future slot. For sample-and-hold scheme, SGCS is calculated with the latest non-predicted CSI and the corresponding ground-truth eigenvectors of the target future slot. From the preliminary results, it can be observed that the CSI prediction provides better SGCS performance than both of the two baselines.

Table 18 SGCS performance of AI/ML-based CSI prediction and no prediction

|  | Without CSI prediction Benchmark 1 | Non-AI/ML-based CSI prediction Benchmark 2 | AI/ML-based CSI prediction |
| --- | --- | --- | --- |
| SGCS | 0.734 | 0.829 | 0.861 |

| Mean UPT | 5% UPT |
| --- | --- |

Figure 28 UPT gain over baseline (left: mean UPT; right: 5% UPT)

Figure 28 shows the UPT performance of the AI/ML-based CSI prediction and two benchmarks, where rank 1 is considered in this evaluation. AI/ML-based CSI prediction can outperform nearest historical CSI with 2.5%-4% gain in terms of mean UPT and with 9%-17% gain in terms of 5% UPT. With the same configuration of the observation window, AI/ML-based CSI prediction can outperform non-AI/ML prediction with 2%-3% gain in terms of mean UPT and with 6%-10% gain in terms of 5% UPT.

Observation 32: For AI/ML-based CSI prediction,

- It outperforms nearest historical CSI with 2.5%-4% gain in terms of mean UPT and with 9%-17% gain in terms of 5% UPT.

- With the same configuration of the observation window, it outperforms non-AI/ML prediction with 2%-3% gain in terms of mean UPT and with 6%-10% gain in terms of 5% UPT.

## 4.3 Generalization verification over various UE speeds

Table 19 shows the generalization performances over different UE speeds for CSI prediction. Generalization Case 1 assumes the same UE speed (30km/h or 60km/h) for both training and testing; generalization Case 2 assumes different UE speeds for both training and testing (30km/h for training and 60km/h for testing, or vice versa); generalization Case 3 assumes mixed dataset composed of 30km/h and 60km/h for training and either 30km/h or 60km/h is adopted for testing.

The results show that AI/ML models can provide good performance if the UE speed of the training data is the same as that of the testing data, while the performance becomes worse if the UE speed of the training data is different from that of the testing data, especially for the case where training is performed with low speed but testing is performed with high speed. In addition, the AI/ML model trained by the mixed dataset shows moderate performance on each scenarios of different UE speeds.

Table 19 Generalization performance over different UE speeds

| Testing | Training, UE speed |  |  |
| --- | --- | --- | --- |
|  | 30 km/h | 60 km/h | mixed |
| 30 km/h | 0.979 (Case 1) | 0.924 (Case 2) | 0.97 (Case 3) |
| 60 km/h | 0.591 (Case 2) | 0.861 (Case 1) | 0.849 (Case 3) |

Observation 33: For generalization of AI/ML-based CSI prediction over UE speeds of 30km/h and 60km/h,

- AI/ML model trained by low UE speed has poor performance when tested on high UE speed.

- AI/ML model trained by mixed dataset shows moderate performance when tested on each of the UE speeds.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: For the CSI prediction sub use case, if collaboration level x is reported as the benchmark, the simulation methodology to evaluate the AI/ML-based CSI prediction enabled by LCM can emulate the fine-tuning case, while the simulation methodology to evaluate collaboration level x CSI prediction can emulate generalization Case 2.

Proposal 2: To evaluate the monitoring accuracy of the intermediate KPI under a given ground-truth CSI format (e.g., quantized ground-truth CSI), the methodology can be considered in terms of the statistical gap between the actual intermediate KPI and the target intermediate KPI for a group of test data samples, where the actual intermediate KPI is calculated with the recovery CSI and the given ground-truth CSI format, and the target intermediate KPI is calculated with the same recovery CSI and the ground-truth CSI of Float32.

Proposal 3: For the monitoring accuracy of the intermediate KPI under a given ground-truth CSI format (e.g., quantized ground-truth CSI) by using the statistical gap for a group of test data samples, the following metrics can be considered to describe the monitoring accuracy:

- Metric 1: The mean gap over the group of test data samples

- Metric 2: The percentage of the samples for which the gap in terms of an absolute value is smaller than a threshold

- Metric 3: The percentage of the samples for which the gap in terms of a positive bias lower than a positive threshold, or in terms of a negative bias higher than a negative threshold, where bias is denoted as actual intermediate KPI - target intermediate KPI

  - Metric 3-1: if the positive bias is higher than the positive threshold, such inaccuracy case is deemed as misdetection.

  - Metric 3-2: if the negative bias is lower than the negative threshold, such inaccuracy case is deemed as false alarm.

Proposal 4: For the evaluation of Type 2 (Joint training of the two-sided model at Network side and UE side, respectively), analyze the feasibility for the following two cases of N>1 NW part models to M>1 UE part models (i.e., Case 4), and companies to report the training method/order, including the preference and details to achieve

- Case A: Simultaneously train all N Network part models and M UE part models involving all vendors

- Case B: Sequentially train N Network part models and M UE part models in a pairwise manner

- Note: The loss function should be separately generated for N>1 NW part models.

Proposal 5: The boundary between Type 2 and Type 3 is whether the information exchange is required in each FP/BP loop or not:

- Type 2 training requires information exchange in each FP/BP loop.

- Type 3 training doesn’t require information exchange in each FP/BP loop.

Proposal 6: For the template of AI/ML-based CSI compression of 1-on-1 joint training, the CSI feedback overhead range of eventual KPI is suggested as:

- Low overhead: <=β* 80 bits.

- Medium overhead: β* (100bits – 140 bits).

- High overhead: >=β* 230 bits.

- Note: β=1 for max rank = 1, andβ=1.4 for max rank = 2/3/4.

Proposal 7: For the template of AI/ML-based CSI compression of 1-on-1 joint training, 3 values of CSI overhead reduction can be reported corresponding to 3 CSI feedback overhead ranges to be determined in the template: “Low overhead”, “Medium overhead”, and “High overhead”, respectively.

- For a given CSI feedback overhead range, it is calculated as the gap, between the CSI feedback overhead of the benchmark which should within the given CSI feedback overhead range, and the CSI feedback overhead of the AI/ML solution corresponding to the same eventual KPI.

  - E.g., for a given CSI feedback overhead range, a horizontal line is drawn on the figure which will have two cross points with two curves of benchmark and AI/ML, respectively, from which the CSI overhead reduction is derived. The cross point with the curve of benchmark should fall into the given CSI feedback overhead range.

Proposal 8: For the performance comparison of training Type 3, the learning capability difference of model itself should not be attributed to the potential performance loss due to the specific training behaviors (e.g., dataset sharing).

Proposal 9: For the evaluation of training Type 3 under CSI compression, the new case (1-on-1 joint training) benchmark/upper bound for performance comparison should separately take into account the following aspects:

- Aspect 1: The same pair of Network part model and UE part model to the Type 3 case being compared.

- Aspect 2: The same pair of Network part CSI reconstruction model and virtual Network part CSI generation model for NW first training Type 3 case being compared, or UE part CSI generation model and virtual UE part CSI reconstruction model for UE first training Type 3 case being compared.

Proposal 10: For the rank >1 options under AI/ML-based CSI compression, the complexity of FLOPs is reported as the maximum FLOPs over all ranks each includes the summation of FLOPs for inference per layer if applicable, e.g.,

- Option 1-1 (rank specific): Max FLOPs over K rank specific models.

- Option 1-2 (rank common): FLOPs of the rank common model.

- Option 2-1 (layer specific and rank common): Sum of the FLOPs of K models (for the rank=K).

- Option 2-2 (layer specific and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with a sum of k models.

- Option 3-1 (layer common and rank common): K * FLOPs of the common model.

- Option 3-2 (layer common and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with k * FLOPs of the layer common model.

Proposal 11: For the rank >1 options under AI/ML-based CSI compression, the storage of memory storage/number of parameters is reported as the summation of memory storage/number of parameters over all models potentially used for any layer/rank, e.g.,

- Option 1-1 (rank specific)/Option 3-2 (layer common and rank specific): summation of memory storage/number of parameters over all rank specific models.

- Option 1-2 (rank common): a single memory storage/number of parameters for the rank common model.

- Option 2-1 (layer specific and rank common): summation of memory storage/number of parameters over all layer specific models.

- Option 2-2 (layer specific and rank specific): summation of memory storage/number of parameters for the specific models over all ranks and all layers in per rank.

- Option 3-1 (layer common and rank common): a single memory storage/number of parameters for the common model.

Proposal 12: For AI/ML-based CSI compression, consider the UE report of high resolution ground-truth CSI using Rel-16 Type II CB with new parameters for the monitoring of intermediate KPI at the Network side.

- Example 1: New parameters of Rel-16 Type II CB are {L=10, p=0.9, beta=0.31, amplitude quantization: 4 bits, phase quantization: 6 bits}.

- Example 2: New parameters of Rel-16 Type II CB are {L=12, p=0.8, beta=0.5, amplitude quantization: 4 bits, phase quantization: 6 bits}.

Observation 1: For Type 2 (Joint training of the two-sided model at Network side and UE side, respectively), challenges are observed for both of the two training methods/orders of Case A (Simultaneously train all N Network part models and M UE part models involving all vendors) and Case B (Sequentially train N Network part models and M UE part models in a pairwise manner)

- For Case A, there may be challenges on aligning the training timelines of all involved Network vendors and UE vendors.

- For Case B, fairness may be harmed for later trained Network-UE pairs where only one side may be able to update parameters during the joint training.

Observation 2: If one side is trained first, and the second side is trained afterwards, with forward propagation and backward propagation (e.g., gradients) exchanged across the two sides, it belongs to Type 2 training.

Observation 3: For a given rank, layer common options only need to run a single model in one processing where the multiple layers can be processed as a batch by making use of the GPU, which largely alleviates the complexity as opposed to the layer specific models.

Observation 4: With the same overhead of CSI feedback and in terms of cell average throughput, AI/ML-based CSI compression with spatial-frequency domain CSI as input can outperform Rel-16 Type II codebook with

- 7%-10.3% gain on rank 1 under full buffer traffic.

- 8.9%-14.8% gain on rank 2 under full buffer traffic.

Observation 5: For the rank=2 case, the 1st layer achieves higher SGCS than that of the 2nd layer as the eigenvectors of the 2nd layer are more sparse, and AI/ML-based CSI compression with spatial-frequency domain CSI as input can achieve more gains over the Rel-16 Type II on the 2nd layer than on the 1st layer.

Observation 6: With the same overhead of CSI feedback and in terms of mean UPT, AI/ML-based CSI compression with spatial-frequency domain CSI as input can outperform Rel-16 Type II codebook with

- 2.2%-2.5% gain on rank 1 under FTP traffic and 50% RU.

- 3.2%-7.8% gain on rank 1 under FTP traffic and 70% RU.

- 4.8%-8.8% gain on rank 1 under FTP traffic and 80% RU.

- 5.4%-10.1% gain on rank 2 under FTP traffic and 50% RU.

- 7.2%-14.4% gain on rank 2 under FTP traffic and 70% RU.

- 7.5%-14.5% gain on rank 2 under FTP traffic and 80% RU.

Observation 7: With the same overhead of CSI feedback and in terms of 5% UPT, AI/ML-based CSI compression with spatial-frequency domain CSI as input can outperform Rel-16 Type II codebook with

- 2.2%-7% gain on rank 1 under FTP traffic and 50% RU.

- 6.4%-14.6% gain on rank 1 under FTP traffic and 70% RU.

- 7.1%-16.3% gain on rank 1 under FTP traffic and 80% RU.

- 4.3%-13.3% gain on rank 2 under FTP traffic and 50% RU.

- 8.1%-18% gain on rank 2 under FTP traffic and 70% RU.

- 9.1%-23.7% gain on rank 2 under FTP traffic and 80% RU.

Observation 8: AI/ML-based CSI compression with spatial-frequency domain CSI as input can achieve more obvious gains over the Rel-16 Type II codebook on high RU than low RU.

Observation 9: AI/ML-based CSI compression with spatial-frequency domain CSI as input can achieve more obvious gains over the Rel-16 Type II codebook for cell edge UPT than average UPT.

Observation 10: For achieving the same throughput, AI/ML-based CSI compression with spatial-frequency domain CSI as input requires less feedback overhead than Rel-16 Type II codebook, with an overhead reduction of

- 42% in high overhead range and 46% in medium overhead range for rank = 1.

- 48% in high overhead range and 43% in medium overhead range for rank = 2.

Observation 11: With the same overhead of CSI feedback and in terms of cell average throughput, AI/ML-based CSI compression with additional past CSI as input can outperform Rel-16 Type II codebook with

- 18.3%-25.4% gain on rank 1 under full buffer traffic.

- 23.3%-30.2% gain on rank 2 under full buffer traffic.

Observation 12: With the same overhead of CSI feedback and in terms of mean UPT, AI/ML-based CSI compression on spatial-frequency domain with additional past CSI as input can outperform Rel-16 Type II codebook with

- 7.7%-14.9% gain on rank 1 under FTP traffic and 80% RU.

- 16.6%-28.6% gain on rank 2 under FTP traffic and 80% RU.

Observation 13: With the same overhead of CSI feedback and in terms of 5% UPT, AI/ML-based CSI compression on spatial-frequency domain with additional past CSI as input can outperform Rel-16 Type II codebook with

- 10%-28.4% gain on rank 1 under FTP traffic and 80% RU.

- 17%-39.2% gain on rank 2 under FTP traffic and 80% RU.

Observation 143: With the same overhead of CSI feedback and in terms of mean UPT for rank 2, compared to the mean UPT corresponding to the ideal CSI,

- Rel-16 Type II codebook has 37%/25%/19% margin under the payload size of 80/163/341 bits, respectively.

- AI/ML-based CSI compression with spatial-frequency domain CSI as input has 28%/19%/10% margin under the payload size of 80/163/341 bits, respectively.

- AI/ML-based CSI compression with additional past CSI as input has 19%/13%/5% margin under the payload size of 80/163/341 bits, respectively.

Observation 15: For achieving the same throughput, AI/ML-based CSI compression on spatial-frequency domain with additional past CSI as input requires less feedback overhead than Rel-16 Type II codebook, with an overhead reduction of

- 73% in high overhead range for rank = 1.

- 70% in high overhead range for rank = 2.

Observation 16: For AI/ML-based CSI compression, compared to the joint training of one Network part model to one UE part model, the SGCS loss of training Type 2 is 1.1%~6.5% for one Network part model to M=3 UE part models with different backbones or different structures.

Observation 17: For AI/ML-based CSI compression, compared to the joint training of one Network part model to one UE part model, the SGCS loss of training Type 2 is 0.1%~2.6% for N=2 Network part models to M=2 UE part models with different structures.

Observation 18: For AI/ML-based CSI compression, there is only minor margin (<0.5%) between the SGCS of the NW first separate training and the SGCS of the joint training if the backbone of UE-side CSI generation part has similar or stronger learning capability than that of Network side CSI generation part.

- This observation applies regardless when the input dataset for the UE side is only a subset of or equal to the input dataset for the Network side.

Observation 19: For separate training, good performance can be achieved by pairing with comparable learning capabilities between Network side model and UE side model, which may be easily aligned by, e.g., sharing the target performance metric of the models in together with the shared dataset.

Observation 20: For AI/ML-based CSI compression, there is only minor margin (<0.5%) between the SGCS of the NW first separate training and the SGCS of the joint training when the separate training considers N>1 Network part models to one common UE part model with a backbone with similar or stronger learning capability than any of the N Network part models.

Observation 21: For AI/ML-based CSI compression, there is minor margin (<1%) between the SGCS of the UE first separate training and the SGCS of the joint training even if the backbone of Network-side CSI reconstruction part has similar or stronger learning capability than that of UE side CSI reconstruction part.

- This observation applies regardless when the dataset for the Network side is only a subset of or equal to the dataset for the UE side

Observation 22: For an AI/ML model of CSI compression trained under Scenario#A (e.g., InH) dataset but applied to Scenario#B (e.g., UMa/UMi) for inference (i.e., generalization Case 2), its performance may be degraded compared to inference under Scenario#A (i.e., generalization Case 1), but mixing the dataset over the three scenarios for AI/ML model training (i.e., generalization Case 3) is helpful to improve the generalization.

Observation 23: For an AI/ML model of CSI compression trained under Scenario#A (e.g., InH) dataset but applied to Scenario#B (e.g., UMa) for inference, its performance can be improved by applying fine-tuning using a relatively small dataset from Scenario#B.

Observation 24: For generalization of AI/ML-based CSI compression over indoor/outdoor UE distribution ratios,

- AI/ML model trained by any indoor/outdoor UE distribution ratio shows similar performance on the outdoor testing dataset.

- With the decrease of the indoor channel ratio for the training dataset, the trained AI/ML model brings decreased performance on the indoor testing dataset.

Observation 25: For generalization of AI/ML-based CSI compression over various TxRU mapping methods including (8,8,2,1,1,2,8) and (2,8,2,1,1,2,8), the AI/ML model trained by the mixed dataset shows moderate performance on each of the TxRU mapping methods.

Observation 26: For scalability of AI/ML-based CSI compression over various CSI payload sizes, the SGCS margin between the payload-specific models and the unified scalable model supporting 4 different CSI payload sizes is less than 1%.

- Adaptation layers in the AI/ML model are used to achieve the scalability.

Observation 27: For scalability of AI/ML-based CSI compression over various antenna port numbers, the SGCS margin between the port-specific models and the unified scalable model supporting 2 different antenna port numbers is less than 0.9%.

- Pre/post processing of padding/truncation is used to achieve the scalability.

Observation 28: For AI/ML-based CSI compression, compared with the training dataset with Float32 format, other high resolution ground-truth CSI quantization methods with lower overhead show minor SGCS loss with remarkable overhead reduction.

- Training dataset quantized by 8 bits scalar quantization has <0.9% SGCS loss but reduces 75% overhead of training dataset delivery.

- Training dataset quantized by Rel-16 Type II CB with new parameters has <0.7% SGCS loss but reduces 96% overhead of training dataset delivery.

Observation 29: For AI/ML-based CSI compression, compared with the training dataset with Float32 format, training dataset quantized by a legacy quantization resolution of PC8 of Rel-16 Type II CB causes significant SGCS loss of around 3% when the CSI payload size is medium and large.

Observation 30: For AI/ML-based CSI compression, quantization non-aware training shows significant performance loss compared to quantization aware training.

Observation 31: For AI/ML-based CSI compression, the monitoring performance is heavily deteriorated if the Rel-16 Type II CB with legacy resolution (e.g., PC8) is adopted. Introducing higher resolution ground-truth CSI format of Rel-16 Type II CB with new parameters can significantly improve the monitoring performance in terms of monitoring accuracy, false alarm, and misdetection. E.g., for threshold of absolute SGCS gap = 0.01/0.02/0.05, respectively:

- When PC8 is adopted for monitoring, only 21%/42%/81% monitoring accuracy is achieved for the 60 bits CSI payload, and 8%/21%/67% monitoring accuracy is achieved for the 240 bits CSI payload.

- When 1014 bits high resolution ground-truth CSI is adopted for monitoring, the monitoring accuracy can be improved to 56%/82%/99% for 60 bits CSI payload and 63%/89%/100% for 240 bits CSI payload.

- When 1610 bits high resolution ground-truth CSI is adopted for monitoring, the monitoring accuracy can be improved to 63%/88%/100% for 60 bits CSI payload and 75%/97%/100% for 240 bits CSI payload.

Observation 32: For AI/ML-based CSI prediction,

- It outperforms nearest historical CSI with 2.5%-4% gain in terms of mean UPT and with 9%-17% gain in terms of 5% UPT.

- With the same configuration of the observation window, it outperforms non-AI/ML prediction with 2%-3% gain in terms of mean UPT and with 6%-10% gain in terms of 5% UPT.

Observation 33: For generalization of AI/ML-based CSI prediction over UE speeds of 30km/h and 60km/h,

- AI/ML model trained by low UE speed has poor performance when tested on high UE speed.

- AI/ML model trained by mixed dataset shows moderate performance when tested on each of the UE speeds.

# References

- R1-2301940, “Summary#5 for CSI evaluation of [112-R18-AI/ML]”, RAN1#112, Athens, Greece, Moderator (Huawei), February 27 – March 3, 2023.

- R1-2302359, “Discussion on AI/ML for CSI feedback enhancement”, RAN1#112bis-e, e-Meeting, April 17 – 26, 2023.