3GPP TSG-RAN WG1 Meeting #112bis-e	  R1-2302359

e-Meeting, April 17 – 26, 2023

Agenda Item:	9.2.2.2

Source:	Huawei, HiSilicon

Title:	Discussion on AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

# Introduction

In previous RAN1 meetings, sub use cases and potential specification impact for CSI feedback enhancement have been discussed and several agreements/conclusions have been achieved. This contribution will further discuss the potential specification impact for CSI feedback enhancement.

# Sub use case

In the RAN1#111 meeting, it has been agreed that the potential spec impact discussion on the sub use case of CSI prediction is deferred to the RAN1#112b-e meeting. In our evaluations, it has been justified that the CSI prediction can outperform both the sample-and-hold benchmark as well as a non-AI/ML CSI prediction algorithm. Therefore, it is suggested that the potential spec impact of CSI prediction is to be studied after RAN1#112b-e meeting.

Agreement Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Note: Continue evaluation discussion in 9.2.2.1. Note: RAN1 defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer further till the end of R18 AI/ML SI. Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases.

Proposal 1: Start the study of the potential spec impact of CSI prediction after RAN1#112b-e meeting.

# Potential specification impact for CSI compression

## Data collection

In the last meeting [1][2], the following agreement had been achieved about data collection. In this section, further discussions are provided on enhancement of CSI-RS configuration, Network side data collection, UE side data collection and delivery of the dataset.

Agreement - In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least - Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection - In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least: - Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection

### Enhancement of CSI-RS configuration for Network/UE side data collection

Since the AI/ML operation is data-driven, the quality of the dataset can significantly contribute to the performance of the AI/ML model in principle. Therefore, enhanced CSI-RS may be considered specifically for the data collection procedure to generate the dataset with more accurate ground-truth CSI as samples. For example, by setting a higher power to the CSI-RS or allocating more REs in time/frequency domain to the CSI-RS for data collection so that UE can achieve more accurate DL measured channel as the ground-truth CSI labels.

However, the above enhancement of CSI-RS for each AI/ML inference occasion will cause additional overhead or decreased power of other channels; in addition, such enhancement can be also applied to the legacy CSI compression, so that the improved performance cannot fully attribute to the introduction of AI/ML. Therefore, the enhancement of CSI-RS is not desired to the model inference stage. For the model training, on the other hand, such enhancement is meaningful to improve the quality of the ground-truth CSI labels without the sacrifice of overhead/power unfairness issue, since the training data collection is triggered in a quite infrequent manner. By using the CSI measured from normal CSI-RS as the model input and using the CSI measured from the enhanced CSI-RS as the labels to generate the training samples, the model will be trained with a super resolution effect, where the inference with an input CSI measured from normal CSI-RS (i.e., low resolution) can output an recovery CSI imitating that measured from enhanced CSI-RS (high resolution). That is to say, separate CSI-RS resources/CSI reports are applied for generating ground-truth CSI labels and model inputs, respectively, during the training procedure. From the UE perspective, it may need to be aware of the relationship between the two CSI-RS resources to achieve the pairing of the input and the label.

Such super resolution is similar with the concept of using the measurement of a sparse beam set as AI/ML model input (i.e., Set B) to infer the best beam subject to a dense beam set (i.e., Set A) in the beam management sub use case.

Figure 1 Separate CSI-RS resources during training data collection

Proposal 2: For the enhancement of CSI-RS configurations for Network/UE side data collection under CSI compression, separate CSI-RS resources/CSI reports can be adopted for generating ground-truth CSI labels (e.g., measured with higher power/density CSI-RS) and model inputs (e.g., measured with lower power/density CSI-RS) to support the super resolution of using low resolution input to infer high resolution output.

### Network side data collection

Latency requirement for data collection

For model training, the latency requirement are generally non real time and thus both L1 signaling and L3 signaling can be supported.

For model monitoring, on the other hand, real time reporting is necessary to enable the gNB to identify the reason of the performance fluctuation/deterioration as soon as possible. E.g., when the throughput of the UE running a specific UE part/UE side model suddenly degrades, gNB has to fast identify what is the reason leading to this degradation – whether it is due to the failure of the AI/ML model(s), or due to the gNB strategies of scheduling, MU pairing, resource assignment, carrier assignment, etc. Then gNB can trigger the monitor of the intermediate KPI afterwards. If it is due to the failure of the UE part/UE side model, gNB can disable it immediately; otherwise gNB can preclude the factor of AI/ML and may need to adjust the gNB strategies of scheduling, etc. Such monitoring window is event-triggered with an on-demand manner, and does not need to be always-on in a period manner, i.e., the UE report can be switched off after the monitoring window expires.

Observation 1: It is necessary to support real time UE report of the monitoring results to gNB to enable fast identification of network performance fluctuation/degradation and AI/ML model failure.

- E.g., in case of performance degradation, event-triggered monitoring window can be activated so that gNB can efficiently collect data and thereby quickly identify whether the degradation is due to the AI/ML model failure.

To enable such on-demand fast identification, L1 report is a more suitable choice than L3/MDT report which may have much longer latency. If the report of the intermediate KPIs is via L3/MDT signaling, gNB needs to wait for 120ms-30min (L3/MDT report interval as captured in RAN2 assumptions) before obtaining the accuracy of the AI/ML model, so the degradation persists before the reception of this RRC report; if the report of the intermediate KPIs is via UCI, on the other hand, gNB can obtain the AI/ML accuracy in TTI level latency, and deactivate the AI/ML model immediately if the model fails. Note that decreasing the interval of L3/MDT report may face the risk of increased probability of radio link failure (RLF), thus the interval of L3/MDT report is trade-off between latency and RLF by the gNB, where the typical value may be configured as much larger than 120ms.

An example is shown in Figure 2, where the needed monitoring data is assumed as 10 data samples of target CSI to identify the AI/ML model failure. The L3/MDT measurement report interval is assumed as 1s (considering a trade-off), while the L1 report is 5ms period (CSI reporting period). The activated monitoring window is therefore 50ms for L1 report, while 1s for L3/MDT report. This means for L3/MDT report, the gNB can do nothing on the failed AI/ML model in the remaining 950ms. Moreover, if the PUSCH carrying the L3/MDT signaling is missed, this latency may be even larger. Therefore, UE reporting based on L1 signaling (ms level latency) may be a better flavor under this case. On the other hand, as the L1 monitoring window will expire after 10 data samples have been collected, the overhead of L1 signaling is comparable with L3/MDT signaling.

Figure 2 Latency comparison of L1 signaling based and L3 signaling based monitoring

Observation 2: For Network side monitoring based on intermediate KPI, the reporting of the ground-truth CSI and the associated CSI report by the UE via L1 signaling has comparable overhead with L3 signaling but with much less latency which can enable fast identification of AI/ML model failure.

Proposal 3: For the container of Network side data collection under CSI compression,

- Both of L1 signaling and RRC signaling can be supported for model training.

- At least L1 signaling should be supported for model monitoring to enable fast identification of AI/ML model failure.

Data sample format

For Network side data collection by UE reports, the ground-truth CSI can be reported per sample, or reported in a batch, e.g., UE can report several hundreds of ground-truth CSI samples in a batch after a relative longer time of measurement over CSI-RS. Based on the analysis of our companion contribution [3], the examples of the overhead per sample include 127 Bytes with Rel-16 Type II-like CB, 833 Bytes with 8 bits scalar quantization. As a comparison, the maximal PMI payload of legacy Rel-16 Type II CB is around 80 Bytes, thus the overhead is still acceptable to be carried on UCI, e.g., UCI on PUSCH, if the ground-truth CSI is in format of codebook-based quantization. On the other hand, if the ground-truth CSI is reported in the format of scalar quantization or in a batch, such overhead can hardly be accommodated by UCI, and RRC signaling may be more suitable.

Considering L1 signaling is more applicable to monitoring data collection, while L1 and/or L3 signaling are applicable to training data collection as analyzed above, both scalar quantization (more suitable on L3 signaling) and codebook-based quantization (applicable to both L1 signaling and L3 signaling) can be supported for the model training purpose, while codebook-based quantization can be supported for the model monitoring purpose.

Proposal 4: For data sample format of Network side data collection under CSI compression:

- Both of scalar quantization and codebook-based quantization can be supported for model training.

- Codebook-based quantization should be supported for model monitoring.

Signaling for triggering the data collection

To reuse the channel measurement of legacy as much as possible, the collection of the ground-truth CSI data can be triggered based on the configuration of CSI-RS resources/report. E.g., for UE side data collection, the CSI-RS resources can be configured without reporting, while for Network side data collection, the CSI-RS resources can be configured with UE reporting (e.g., L1 or L3/MDT reporting). On the signaling of triggering the data collection window, both event-triggered signaling and periodic signaling can be studied.

In addition, as a difference with legacy, the number of ranks and the index(es) of layer(s) for the report of ground-truth CSI can be designated by the gNB rather than autonomously calculated and reported by UE as in the legacy CSI feedback in UCI; this will benefit gNB to collect the data of the wanted layers, e.g., the higher layers which may be rarely reported by UE at relatively medium/low SINR region.

Proposal 5: For the Network side data collection of ground-truth CSI, the number of ranks and the index(es) of layer(s) for the report of ground-truth CSI can be designated by the gNB rather than autonomously calculated and reported by UE.

### UE side data collection

In the last meeting, it has been captured to study the assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. (namely data categorization ID). For the data categorization ID for assisting UE side data collection, in our understanding, the applicable cases may need to be further clarified.

- If the data categorization ID is kind of antenna layout/TxRU mapping information, it is our understanding that UE side can train a generalized model to adapt various TxRU mapping patterns, for which the evaluation results have been provided with good performance in our companion contribution [3].

- If the data categorization ID is intended for identifying scenario/area/zone information, it may not be so necessary also, since the UE can autonomously sense such scenarios without being notified by gNB, e.g., it can obtain the geographic position with its own positioning functionality to identify UMa/UMi, or obtain the UE speed calculated from Doppler shift.

- In addition, as the UE vendor may have a different data categorization principle from the Network vendor, how harmonize the understanding of the indicated data categorization ID between the Network vendor and the UE vendor needs to be clarified. E.g., for Cell#A and Cell#B, what would the UE vendor do, if the UE identifies the two cells are subject to different scenarios as per its own sensing, while gNB indicates them with the same data categorization ID, or the other way around (i.e., UE identifies them with the same scenario, while gNB indicates with different data categorization IDs)? Similarly, the understandings of the indicated data categorization ID between different Network vendors may also be different.

If data categorization ID is really necessary after justification, as mentioned in the sub-bullet of the agreement, it needs to preserve the proprietary. In our understanding, any explicit interpretation on the physical meaning of the data categorization ID would disclose the specific gNB implementation (e.g., TxRU mapping) or network deployment strategy (e.g., deployment scenario). One possible way to preserve the Network/MNO proprietary is to consider the data categorization ID in terms of an implicit/virtualized ID without notifying the physical meaning. The physical meaning of such ID and the granularity on how to categorize the scenarios (e.g., the potential combination of different aspects of deployment scenarios, TxRU mapping patterns, UE speeds, etc.) is up to Network implementation and transparent to the UE side.

Based on this assumption, from the UE perspective, the feasible way to make use of such data categorization ID is to split the collected data samples into separately datasets associated with different data categorization IDs and train separate models accordingly. This may lead to tremendous number of AI/ML models at the UE side, if such models are trained per MNO, per Network vendor, and per data categorization ID of the specific MNO/Network vendor.

Observation 3: The applicable cases for the categorization ID for assisting UE side data collection may need to be further clarified with respect to the following points:

- Generalized model can be trained at UE side over scenarios/antenna layouts.

- UE can autonomously sense the scenario without being notified by gNB.

- The categorization or granularity of the scenarios identified by Network may not match the categorization principle of the UE side.

Proposal 6: The data categorization ID, if justified, should be determined by Network side in forms of virtualized ID without specifying the physical meaning.

- The physical meaning of such ID and the granularity of such ID is up to Network implementation without being indicated to the UE side.

## Dataset delivery

### General aspects

As will be analyzed in Section 3.3, some training types such as Type 2/3 need the Network side and the UE side to train the CSI reconstruction part and the CSI generation part, respectively, based on aligned dataset, so that the signaling and the procedure for dataset delivery need to be studied as part of the training types. In order to align the understanding of the delivered dataset between the Network side and the UE side, the following aspects should be considered for dataset delivery:

- Dataset ID, which is used to differentiate the models to be trained at the opposite side.

- Size of the dataset, e.g., how many data samples are contained in the delivered dataset.

- Format of data sample, e.g., the high resolution quantization method (codebook based or scalar based), the dimension of the data samples, etc.

- Type(s) of data sample, e.g., the type of the target CSI (e.g., channel matrix or eigenvectors), and the type of the CSI report (e.g., before or after quantization) in case of Type 3.

- Quantization/de-quantization related information, etc.

Proposal 7: In CSI compression using two-sided model with training collaboration Type 3, further study potential specification impact of dataset delivery over air-interface on the following aspects:

- Training dataset and/or other information delivery from UE side to Network side for UE first training.

- Training dataset and/or other information delivery from Network side to UE side for NW first training.

- The specification impact includes dataset ID, the size of the dataset, format of data sample, type(s) of the data sample, quantization/de-quantization related information etc.

### Overhead analysis

As per the discussions of previous meetings, one concern of dataset delivery over the air-interface is the enormous size of the training dataset may lead to issue of power consumption and air-interface overhead on the UE device.

However, it should be clarified that the overall dataset does not necessarily be sent from a single gNB to a single UE. On the other hand, the original dataset can be split into massive pieces of subsets each with limited number of data samples. Considering the model training at UE side is usually performed at a non-3GPP entity belonging to the UE vendor, each UE may only need to receive one subset of the original dataset and the non-3GPP entity can recombine all subsets received and uploaded by the massive UEs to recover the original dataset, which is then used for model training. Each subset of the original dataset is associated with a common unique dataset ID to facilitate the dataset recombination.

On the frequency of such dataset delivery, as the data distribution would not vary drastically, Network does not need to frequently update the model, but rather in a per week/month frequency. If a generalized model is trained based on this overall dataset, the period of updating the model is even longer. Therefore, the average air-interface overhead of dataset delivery in a per gNB or per UE basis is negligible.

Moreover, some quantization or compression methods can be adopted to largely reduce the overhead of the ground-truth CSI, as has been widely evaluated in 9.2.2.1. An example of the overhead analysis is provided in Table 1, where two quantization methods are provided, including 1) Scalar quantization, e.g., Float32, Float16, 8bit scalar quantization, and 2) an enhanced Rel-16 Type II CB with new parameters to achieve higher resolution. The total overheads of datasets for training an AI/ML model from scratch with Transformer backbone as adopted in [3] are provided for both quantization methods. It is shown that by using enhanced Rel-16 Type II CB with new parameters, it can reduce the overhead of dataset delivery down to 40MB (which is 96% overhead reduction).

Table 1 Overhead analysis for ground-truth CSI

| Compression method | Overhead per sample | Total overhead, 300k samples | Average overhead per hour |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | dataset delivery frequency = 1 month | dataset delivery frequency = 1 week | dataset delivery frequency = 1 day |
| Float32 | 3.3 KB | 992 MB | 1.4 MB | 5.9 MB | 41 MB |
| Float16 | 1.67 KB | 499 MB | 0.69 MB | 2.97 MB | 21 MB |
| 8bit scalar quantization | 832 B | 250 MB | 0.35 MB | 1.49 MB | 10.4 MB |
| Rel-16 Type II CB with new parameters: L=10, p=0.9, beta=0.31, amplitude: 4 bits, phase: 6 bits | 127 B | 40 MB | 56 KB | 238 KB | 1.7 MB |

E.g., as shown in Figure 3, for NW first training, the Network side can split the overall dataset into K*N subsets, each of which is delivered from a gNB to a UE (i.e., the overall dataset can be delivered by K gNBs each of which sends N subsets to N UEs, respectively). Thus, the size of the dataset can be thinned to K*N times for per UE basis. Assuming K*N=10000 UEs are used to share this dataset delivery, and the size of the overall dataset is 40 MB by using Rel-16 Type II-like quantization method as given in Table 1, the overhead for per UE is only 4KB, which is comparable with RRC signaling and is therefore negligible with respect to the per month/per week/per day level frequency of dataset delivery.

Figure 3 Dataset delivery over massive UEs

Proposal 8: For the dataset delivery of CSI compression over air-interface, the following approaches can be considered to largely alleviate the overhead/power consumption of per UE:

- Quantization on the ground-truth CSI with high resolution quantization format, e.g., R16 Type II-like method with new parameters.

- Network splits the overall dataset into massive subsets each with limited number of data samples (e.g., with comparable overhead as RRC signaling). The subsets can be separately sent to numerous UEs, and all subsets are associated with a common dataset ID for UE side combination.

### Model pairing with dataset delivery

It has been discussed in 9.2.1 on the functionality/model identification framework. If the dataset delivery is enabled, the gNB and UE can achieve the model pairing based on an online manner without introducing offline model registration, which we identify with non-trivial offline co-engineering and SA impact as mentioned in our companion contribution [4].

As shown in Figure 4, for NW first separate training, the gNB(s) can deliver the dataset (or its massive subsets) to UEs over the air-interface, which is associated with a unique dataset ID. This dataset ID can also be used as the data categorization ID to categorize the data samples from different scenarios/antenna layouts. For the UE side, it receives (and recombine) the dataset delivered from the gNB(s), and trains the specific UE part model accordingly. After the UE part model is trained and deployed at the UE device, the UE device can pair the UE part model with the NW part model by using the dataset ID. E.g., UE can report the supported dataset ID(s) as part of UE capability to gNB, and the gNB can activate a specific UE part model by indicating the dataset ID that can be paired with the NW part model running at the gNB.

Figure 4 Pairing of the Network part model and UE part model based on dataset delivery and dataset ID

Proposal 9: For the dataset delivery of CSI compression over air-interface, the dataset ID associated with the delivered dataset can be used to achieve the pairing of the Network part model and the UE part model.

- E.g., for NW first separate training, UEs receive the dataset associated with a dataset ID to perform the training; after the UE part model is trained, UE and gNB will use the dataset ID to achieve the pairing.

## Training collaboration types

In the last meeting [1], the following conclusion had been achieved for analyzing training collaboration types.

Conclusion In CSI compression using two-sided model use case, further discuss the pros/cons of different offline training collaboration types including at least the following aspects: - Whether model can be kept proprietary - Requirements on privacy-sensitive dataset sharing - Flexibility to support cell/site/scenario/configuration specific model - gNB/device specific optimization – i.e., whether hardware-specific optimization of the model is possible, e.g. compilation for the specific hardware - Model update flexibility after deployment - feasibility of allowing UE side and NW side to develop/update models separately - Model performance based on evaluation in 9.2.2.1 - Whether gNB can maintain/store a single/unified model - Whether UE device can maintain/store a single/unified model - Extendability: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use - Whether training data distribution can be matched to the device that will use the model for inference - Whether device capability can be considered for model development - Other aspects are not precluded - Note: training data collection and dataset/model delivery will be discussed separately

The pros and cons of each training collaboration type are analyzed separately in below.

### Training collaboration Type 1

For Type 1, the two-sided AI/ML model is trained at one side, which could be at Network side or at UE side. After model training is completed, Network delivers the trained CSI generation part to UE, as shown in Figure 5(a), or UE delivers the trained CSI reconstruction part to Network, as shown in Figure 5(b).

| (a) Joint training of the two-sided model at Network side | (b) Joint training of the two-sided model at UE side |
| --- | --- |

Figure 5 Joint training of the two-sided model at a single side/entity

Joint training at Network side

Joint training at the Network side has several advantages:

- Flexibility on scenario-specific model: Network can support cell/site/scenario/configuration specific model flexibility. Network can also dynamically update UE’s model after deployment when the wireless scenario changes, e.g., UE handover.

- Extendibility: For the case of 1 Network to M>=1 UEs, when a new UE with specific requirement join in a Network, the Network can train a new UE part model for the new UE by freezing the Network part model in use.

- Burden on model maintenance/storage at UE side: Since the UE device only needs to store one CSI generation part corresponding to the specific cell being camped, the burden on model maintenance/storage at UE side is not critical.

- Training data distribution: Since the Network may use the dataset mixed from multiple UEs served by the Network, the data distribution of a specific UE can be represented by the distribution of the mixed training dataset which will conduct a generalized model.

- Note that theoretically it may achieve the optimal network performance since CSI generation part and CSI reconstruction part are designed and trained jointly at Network with ideal model pairing. However, due to the practical restrictions as described in below, such theoretical performance may hardly be achieved.

Joint training at the Network side also faces some challenges/downsides:

- Software/hardware compatibility: The algorithm design of AI/ML model is coupled with the hardware (e.g., chipset) and the software platforms (e.g., runtime environment), so that an unseen delivered AI/ML model arbitrarily developed by the Network vendor may not run successfully at the UE side. In particular, the CSI generation part model structure developed without involving the corresponding UE vendor may suffer low operating efficiency, long operating latency, high power consumption, or even failure of running at the UE modem. To summarize, the UE may face the compatibility issue for the model structure developed by Network without interoperation with the UE vendor. To alleviate the compatibility issue as aforementioned, it would incur the following restrictions/issues.

  - Engineering isolation: The supported model structure(s) of the CSI generation part need to be aligned between the Network vendor and the UE vendor, e.g., in an offline manner, as analysed in our companion contribution [3]. This would lead to offline co-engineering, i.e., the engineering isolation is crippled to large extent.

  - Device specific optimization: Due to the interoperation, the device specific optimization at the UE side may also be restricted by the preference of the Network side.

  - Burden on model maintenance/storage at Network side: Network, in particular gNB, can train and maintain a unified CSI reconstruction part over multiple UEs. However, as different UE vendors would probably support different structures of the CSI generation part, the Network vendor may need to maintain/store numerous CSI generation parts from different UE vendors and different UE versions of per UE vendor (though only a single CSI reconstruction part is maintained/stored at the gNB). Considering there are UEs from multiple UE vendors/UE versions in the same cell, this maintenance/storage burden is imposed on the gNB.

  - Performance: Due to the co-engineering restriction, the Network vendor may not freely develop the CSI generation part to achieve best match with the CSI reconstruction part at gNB (i.e., restricted model pairing), so that the performance may become suboptimal. Moreover, as the CSI reconstruction part may need to be jointly trained with multiple CSI generation parts from different UE vendors, the performance would be further impacted, which is similar to the Type 2 training between 1 Network part model to M UE part models as evaluated in our companion contribution [3].

- UE capability on compiling: Current UE modem may not have the capability of compiling AI/ML models. If the AI/ML model subject to the open format is transferred/delivered to UE, UE may need to upload the AI/ML model to a non-3GPP entity for compiling which consumes additional overhead over air-interface resources as analysed in our companion contribution [4].

- Model proprietary: The implementation of AI/ML models are usually proprietary. Whether or how to keep the proprietary of AI/ML models when Network side model is transferred/delivered to the UE needs to be further studied.

- Overhead: The air-interface overhead is needed for model transfer/delivery regardless it is transferred/delivered via CP, UP, or 3GPP transparent. For CSI generation part with large size, in particular, the overhead could become a critical issue if the model is updated.

Joint training at UE side

The merits of joint training at UE side are listed as below:

- Burden on model maintenance/storage at UE side: UE side can maintain a unified CSI generation part over multiple Network vendors which reduces the storage burden of UE side.

- Training data distribution: The UE side can train UE vendor/type/version specific models to match different data distributions due to different UE implementations. However, it may fail to match to other UE venders/types/versions. As a result, gNB has to incorporate various models from different UE venders/types/versions, which imposes burden of model maintenance/storage as analysed in below.

The cons of joint training at UE side include follows:

First, joint training at UE side faces the similar challenges/issues as the joint training at Network side, including: software/hardware compatibility issue (which further incurs engineering isolation issue, device specific optimization issue, suboptimal performance, etc.), overhead, and model proprietary issue. For the same reason, joint training at UE side also may hardly achieve engineering isolation, device specific optimization may also be restricted, and the performance may hardly reach optimum.

Second, in contrast with joint training at Network side, there are a couple of specific downsides for joint training at UE side:

- Flexibility on scenario-specific model: For the joint training at Network side, Network vendor can flexibly perform cell/scenario specific model training based on specific network planning and site types, thus it is more realistic for Network to train AI/ML models that best match the cell environment. As a comparison, for the joint training at UE side, dataset collected by UE vendors may not match the specific cell environment of the Network vendor/MNO, so that the model would be suboptimal.

- Model updating flexibility: The model update for the joint training at Network side (which can train the model at the gNB with an on-demand manner) is much easier than model update at the UE side which cannot train the model at the UE device due to the limitation of the UE capability.

- Burden on model inference/storage/running at Network side: gNB needs to inference/store/run multiple CSI reconstruction parts delivered from different UE vendors/UE versions at the same time which increases the burden of computing and storage on the Network side. As CSI reconstruction part is generally with larger size to CSI generations part, the burden of storage at gNB is heavier than joint training at Network side. In addition, as the gNB has to run different AI/ML decoders corresponding to different UEs, the complexity of joint training at UE side is higher than the other way around (where only one decoder performs inference over all UEs as one batch).

Observation 4: For CSI compression with two-sided model, training Type 1 may suffer software/hardware compatibility issue, and the following restrictions/issues may need to be considered to relieve the compatibility issue:

- Network may have to interoperate with various UE vendors/UE versions to dedicatedly train the CSI generation part for UE, which harms the engineering isolation.

- Network, in particular gNB, may have to maintain/store multiple CSI generation parts trained for different UE vendors/UE versions.

- Network vendor may not freely develop the CSI generation part for UE, which may restrict the pairing with the CSI reconstruction part and thereby result in sub-optimal performance.

Observation 5: For training Type 1 of CSI compression, compared with joint training at Network side, performing joint training at UE side and delivering the model to the Network incurs extra challenges for Network due to the following reasons:

- Inconvenience of training cell/scenario specific models.

- Inflexible model update.

- Burden of inference/storing/running multiple Network part models at gNB delivered from different UE vendors/UE versions.

Based on the observation above, it is then proposed to deprioritize the mode of joint training at UE side and delivering the model to the Network.

Proposal 10: For training Type 1 of CSI compression, deprioritize the mode of joint training at UE side and delivering the model to the Network.

### Training collaboration Type 2

Type 2 can be defined as: a process to train the CSI generation part at UE side and CSI reconstruction at Network side in one forward propagation (FP) & backward propagation (BP) loop across the Network and the UE. In this type, both Network and UE are involved in model training while no AI/ML model is transferred over air-interface.

Figure 6 Joint training of the two-sided model at Network side and UE side, respectively

In particular, the model structure of CSI generation part and CSI reconstruction is designed separately by UE vendor and Network vendor, respectively, as shown in Figure 6, and the model of one vendor is unaware of by the vendor at the opposite side. By defining the BP and the FP interaction procedure and under a common dataset, the parameters of CSI generation part and CSI reconstruction can be trained jointly through iterative FP/BP loops. As an interaction approach, the FP information (e.g., the compressed CSI) and the BP information (e.g., the gradients information) during training process can be exchanged.

The pros of training Type 2 are list as below:

- Model proprietary: Since the UE part model the Network part model are designed and trained by UE vendor and Network vendor, respectively, model proprietary can be kept and device specific optimization is also allowed.

- Burden on model maintenance/storage at UE side: Since the UE device only needs to store one CSI generation part corresponding to the specific cell being camped, the burden on model maintenance/storage at UE side is not critical.

- Burden on model maintenance/storage at Network side: Network can maintain a unified model over multiple UEs with the sacrifice of performance loss to some extent, as described and evaluated in our companion contribution [3].

- Training data distribution: For training Type 2, the training dataset should be aligned between Network and UE. More likely, each UE sides may use their own dataset and thus the Network have to train its Network part model by the mixed dataset from all UE sides. Therefore, the training data distribution can be matched to both UE and Network side.

However, training Type 2 relies on complex design to support real-time interaction of FP/BP iterations between Network and UE which introduces huge challenges. E.g., it is challenging to align the joint training timeline over multi-Network vendors and multi-UE vendors as different vendors would have separate time plans of model/product development. Therefore, the cons of training Type 2 include follows:

- Engineering isolation: Due to the real-time interaction of FP/BP iterations between Network and UE, engineering isolation is seriously breached. UE vendor and Network vendor can’t accomplish model training/updating independently. Thus, model update will be not flexible after deployment since cooperation between UE vendor and Network vendor is needed.

- Flexibility on scenario-specific model: Due to the offline joint development, cell/site/scenario/ configuration specific model is also not flexible to support.

- Model updating flexibility: Due to the offline joint development, model update may be not flexible after deployment.

In the last meeting, it has been discussed on the extendibility of Type 2. It is therefore separately analyzed in below.

- Extendibility: For the case of 1 Network to M>=1 UEs, when a new UE joins, the UE can jointly train its UE part model with Network while the Network can freeze its Network part model in use. Symmetrically, for the case of 1 UE to N>=1 Networks, when a new Network joins, the UE can also freeze its UE part model in use to jointly train the Network part model.

  - However, for the case of N>1 Network to M>1 UEs, it is hard to ensure the new UE can train a single UE part model compatible to the N Networks since all of the N Networks have frozen their Network part models. Therefore, the new UE has to adopt multiple UE part models to pair with different Networks. To clarity, for training Type 3, it is also extendible when the UE is assumed to adopt multiple UE part models to pair with different Networks.

  - In addition, the performance of both ways may be not the optimal since only one part of the two-sided models can be updated.

Observation 6: For training Type 2 of CSI compression and model updating, it relies on complex design to support real-time interaction of FP/BP iterations between Network side and UE side, which causes strong challenges to engineering isolation especially for the case of multi-Network vendors to multi-UE vendors.

### Training collaboration Type 3

The procedure of how to proceed Type 3 are clarified in below.

NW first training

The procedure of Type 3 for NW first training is illustrated in Figure 7, including following steps:

- Step 1, Network trains a two-sided AI/ML model, which includes a CSI generation part and a CSI reconstruction part with dataset#1 of original CSI, Vin. Note that the Network side CSI generation part is used only for training but will not be deployed for inference.

- Step 2, Network side shares the dataset#2 to UE side. The dataset#2 contains both input (Vin) and output (CSI feedback, Vq) of the Network side CSI generation part.

- Step 3, UE trains a UE side CSI generation part using the dataset#2, with the training input as Vin and the loss function generated as MSEVq, Vq', where Vq' is the output of UE side CSI generation part. The output of the Network side CSI generation part, Vq, is regarded as labels for the UE side CSI generation part.

- Step 4, Network side CSI reconstruction part at Step 1 and UE side CSI generation part in Step 3 can be separately deployed for joint inference.

Once the training at UE is finished, the output of the UE side CSI generation part will be close to the output of the Network side CSI generation part used in Step 1 under the same input for them, so the Network side CSI reconstruction part can recognize the output of the UE side CSI generation part and accurately recover the target CSI accordingly. For Type 3, the UE side CSI generation part is designed and trained by the UE with UE side FP/BP iterations, and the Network side CSI reconstruction part is designed and trained by Network with Network side FP/BP iterations. The design of the Network side CSI reconstruction part and the UE side CSI generation part can therefore be kept proprietary.

Figure 7 Procedure of separate training for CSI compression

Separate training facilitates the training of two-sided model by introducing only dataset sharing between Network side and UE side. The advantages of separating training with dataset sharing is as follows:

- Software/hardware compatibility: Model training at Network and UE are performed separately, thus the hardware/software compatibility issue can be avoided. Consequently:

  - Engineering isolation: The engineering isolation in terms of the CSI generation part alignment can be better ensured since the Network part model and the UE part model are individually developed and trained.

  - Device specific optimization: The development of the CSI generation part and the CSI reconstruction part is performed by the UE vendor and the Network vendor, respectively, thus the gNB/device specific optimization is allowed.

  - Burden on model maintenance/storage at Network side: The burden on model maintenance/storage at the gNB is relieved, as the Network can maintain a unified CSI reconstruction part over multiple UE.

  - Burden on model maintenance/storage at UE side: The UE side can also maintain a unified CSI generation part to match multiple Network vendors using dataset mixing; even if the generalized model compatible to multiple Network vendors is not achieved, it would not cause burden on the UE device, since the UE device only needs to store one CSI generation part corresponding to the specific cell being camped.

- Model proprietary: Model proprietary can be guaranteed as model disclosure or joint development between Network vendor and UE vendor on model structure may not be needed.

- Flexibility on scenario-specific model: Network can flexibly generate the cell/site/scenario/ configuration specific model and deliver cell/site/scenario/configuration specific dataset to UE for training.

- Model updating flexibility: For the model updating, if the dataset sharing is performed offline, it is not flexible to update model after deployment. However, if the dataset sharing is performed over the air-interface, as the required dataset size of model updating/fine-tuning would be much less than that of model training, the consumed overhead would be even smaller than that of training. Therefore, the flexibility of model updating can be improved to some extent. Compared with Type 1 joint training at Network side though, it may be less flexible for model updating, since for Type 1, the delivered model can be directly used, or implemented after compiling; while for Type 3, UE side still needs to perform training in prior. But compared with Type 2 or Type 3 with offline dataset delivery, it is more flexible due to less offline co-engineering.

- Extendibility: When a new UE join in a Network, it can use the mixed dataset delivered by N Networks train its single UE part model which is compatible with the N Network part models in use. Even if the data distribution of the new UE is different from the original dataset, the Network can also use the new dataset joint to train a new virtual UE part model by freezing the Network part model in use. The Network then generate a new dataset corresponding to the input/output of the new virtual UE part model and then send it to the new UE.

- Training data distribution: Since the Network may use the dataset mixed from multiple UEs served by the Network, the data distribution of a specific UE can be represented by the distribution of the mixed training dataset which will conduct a generalized model.

The potential issues faced by NW first separate training are listed and analyzed as below:

- Performance: The Type 3 may face the issue of suboptimal performance compared with joint training with ideal model pairing. In our companion contribution [3], however, the evaluation results have shown that there is only minor margin (<0.5%) between the performance of the separate training and the performance of the joint training even when the UE side CSI generation part has a different structure with the Network side CSI generation part. On the other hand, compared with Type 1/2 where the multi-vendor training may be inevitable (e.g., 1 Network to M>1 UEs, N>1 Networks to 1 UE, or N>1 Networks to M>1 UEs) and causes further performance loss, NW first Type 3 can naturally support 1 Network to M>1 UEs.

- Overhead: For the dataset sharing over air-interface, it would introduce additional overhead of dataset delivery. However, as analysed previously, the model updating for a generalized model could be quite infrequent, and the overhead of training dataset can be reduced significantly by using some quantization methods. These would largely alleviate the overhead issue.

- Privacy-sensitive dataset sharing: As a clarification, the shared dataset is constituted by the CSI-related data which is irrelevant with the user privacy (e.g., user position, etc.) at least from the evaluated solutions so far. The dataset sharing/delivery can be performed under the contract agreement with the MNOs/UE vendors to mitigate the data ownership problem.

UE first training

There is an alternative mode for Type 3 which is symmetric to the above mode, i.e., UE first training: UE trains a two-sided AI/ML model and shares the dataset#2’ including the input and output of the UE side CSI reconstruction part, then Network trains a Network side CSI reconstruction part based on dataset#2’. It has similar pros with NW first training in terms of compatibility, model proprietary, and gNB/device specific optimization, extendibility, while the aspects of performance, overhead, and privacy-sensitive dataset sharing are similar as NW first training.

On top of that, UE first training faces the following specific issues:

- Flexibility on scenario-specific model: The dataset collected by UE side may not match the channel characteristics at the Network, regarding the Network vendor may want to perform cell/scenario specific model trainings while the dataset provided by UE vendors may not involve that categorization. Thus, cell/site/scenario/configuration specific model is not flexible to support.

- Model updating flexibility: Model update may be not flexible after deployment for the UE first training, since the training entity of the UE side would be the non-3GPP entity rather than the UE device due to the limitation of the UE capability, while for NW first training for comparison, the gNB can perform the update of the Network part model flexibly.

- Burden on model inference/storage at Network side: For the NW first training, if the generalized model pairing to multiple Network vendors is not achieved, it would not bring burden to UE device as it only stores one AI/ML model corresponding to the cell being camped. However, for the UE first training, the gNB may need to inference/store multiple Network part models to separately pair with UE part models subject to different UE vendors/UE versions.

Observation 7: For training Type 3 of CSI compression,

- The shared dataset is constituted by the CSI-related data which may be irrelevant with the user privacy (e.g., user position, etc.).

- The dataset sharing/delivery can be performed under the contract agreement between the Network vendors/MNOs/UE vendors to mitigate the data ownership problem.

Observation 8: For training Type 3 of CSI compression, compared with NW first training, performing UE first training incurs extra challenges for Network due to the following reasons:

- Inconvenience of training cell/scenario specific models.

- Inflexible model update.

- Burden of maintaining/storing multiple Network part models at gNB to pair with multiple UE vendors/UE versions.

### Comparison of the training collaboration types

Based on the above analysis, the pros and cons of aforementioned 3 training types (only offline based Type 2 is considered) are summarized in following Table 2.

Table 2 Brief comparison of the training types for two-sided model

|  | Type 1 |  | Type 2 | Type 3 |  |
| --- | --- | --- | --- | --- | --- |
|  | NW-sided | UE-sided |  | NW first | UE first |
| Whether model can be kept proprietary | No | No | Yes | Yes | Yes |
| Whether require privacy-sensitive dataset sharing | No | No | No | No | No |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | No | No | Yes | No |
| Whether gNB/device specific optimization is allowed | Restricted | Restricted | Yes | Yes | Yes |
| Overhead | Model | Model | N/A | Dataset | Dataset |
| Model update flexibility after deployment | Flexible | Not flexible | Not flexible | Semi-flexible | Not flexible |
| Engineering isolation (feasibility of allowing UE side and NW side to develop/update models separately) | Non-isolable | Non-isolable | Strongly non-isolable | Isolable | Isolable |
| Model performance | Suboptimal | Suboptimal | Suboptimal | Suboptimal | Suboptimal |
| Whether gNB can maintain/store a single/unified model | No | No | Yes | Yes | Restricted |
| Whether UE device can maintain/store a single/unified model | Yes | Yes | Yes | Yes | Yes |
| Extendibility | Support | Restricted | Support | Support | Support |
| Whether training data distribution can match the inference device | Restricted | Yes | Restricted | Restricted | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Compatibility issue exists | Compatibility issue exists | Free of compatibility issue | Free of compatibility issue | Free of compatibility issue |

## Model inference

### CQI determination

In the last RAN1 meeting, the following agreement about CQI determination had been achieved [1].

Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

For AI/ML-based CSI compression, except for Type 1 joint training at UE side, UE doesn’t have the CSI reconstruction part and thus doesn’t know the output channel matrix/eigenvector at Network. Therefore, Option 1 (CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation) should be a starting point. A straightforward way is that the UE adopts the original eigenvectors for CQI calculation which is different from what will be recovered by Network. Such misalignment between the original eigenvectors and recovered eigenvectors will lead to misalignment of the CQI between Network and UE, and the CQI calculated by UE would be overestimated. To report a more accurate CQI, a simple way is that UE compensates the CQI calculated with the original eigenvectors; as the UE may not have information of the recovery CSI, the CQI compensation can be derived based on some assistance of Network indication, e.g., Network to indicate a previous output-CSI-UE to UE for calculating the compensation value.

Proposal 11: For CQI determination of CSI compression, consider Option 1 (CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation) as a starting point.

### CSI configuration and report

In the last RAN1 meeting, the following agreement about CSI configuration and report had been achieved [1].

Agreement In CSI compression using two-sided model use case, further study the following aspects for CSI configuration and report: - NW configuration to determine CSI payload size, e.g., possible CSI payload size, possible rank restriction and/or other related configuration. - How UE determines/reports the actual CSI payload size and/or other CSI related information within constraints configured by the network.

In legacy CSI feedback framework, the maximum rank number, the codebook type and the codebook parameter combinations (including the number of frequency/spatial domain basis and the maximum number of non-zero coefficients), can be configured so that the gNB has the flexibility on the maximum CSI feedback payload size in terms of rank number and CSI feedback granularity per layer. In addition, the UE can autonomously determine the RI and the number of non-zero coefficients which are fed back to the gNB so that the UE also has the flexibility of determining the CSI feedback payload. The AI/ML-based solutions should also be ensured with the flexibility of configuring/determining the CSI payload size by both gNB and UE so that it is not inferior to the legacy CSI feedback mechanism.

For Network configuration to determine CSI payload size, as a straightforward solution, gNB can configure a set of candidate CSI payload sizes for each layer separately, where the number of the candidate CSI payload sizes depend on the scalability of the AI/ML model. Then the UE can determine the RI based on its own measurement, and select the actual CSI payload size for each layer from the configured set of CSI payload sizes subject to the selected RI. After that, the UE needs to report the selected RI and the actual CSI payload size for each layer subject to the reported RI.

For how to report the actual CSI payload size, a simple way is to report the index of the actual CSI payload size for each layer among the configured set of CSI payload sizes. However, as the number of the actual CSI payload size for per layer is dependent on the RI, and the overall size of the latent space is dependent on the actual CSI payload size, how to map the reports of RI, index of actual CSI payload size per layer and the content of latent space subject to the actual CSI payload size, under the current framework of two parts CSI, needs to be further studied. E.g., if the UE reports the index of actual CSI payload size per layer in CSI part 1 by always assuming the maximum RI, there will be redundant overhead in case the selected RI < maximum RI. Thus, how to map the CSI report on the two parts CSI by alleviating the dependency (e.g., mitigate the dependency between RI and actual CSI payload size per layer, or mitigate the dependency between actual CSI payload size per layer and the overall size of the latent space) needs further study.

Proposal 12: For Network configuration to determine CSI payload size, gNB can configure a set of candidate CSI payload sizes for each layer separately.

Proposal 13: For UE determination/reporting of the actual CSI payload size, UE reports the selected RI and the index of the actual CSI payload size (among the set of candidate CSI payload sizes) for each layer subject to the selected RI.

- FFS how to map the CSI report on the two parts CSI to avoid redundant feedback.

### Quantization

In the last RAN1 meeting, the following proposal about quantization had been discussed.

## Proposal 3-3-6: In CSI compression using two-sided model use case, further study at least the following potential specification impact on quantization alignment including: - For vector quantization scheme, the format and size of the VQ codebook, the distance metric (or quantization rule), the segmentation approach, and configuration of VQ codebook. - For scaler quantization scheme including uniform and non-uniform quantization

For scalar quantization, the dictionary can be understood as a mapping from floating values to certain quantization levels, where the quantization levels can be uniform or non-uniform.

For vector quantization, on the other hand, it is usually trained together with the AI/ML models and it is hard to specify a fixed dictionary. A realistic way for vector quantization is to indicate its dictionary (including format and size) to the other side, e.g., for training Type 1, the training entity/side can send the dictionary to the other side in together with the model; for training Type 3, the entity/side performing the first step training can send the dictionary to the opposite side which performs the second step training, while the other side only trains the model but keeps the dictionary unchanged. In addition, considering the varying channel status, it is also possible that the AI/ML model is unchanged but the dictionary is updated and indicated to the other side to adapt to the varying channel.

Moreover, in order to avoid the extreme size of the vector quantization dictionary which would cause high complexity on quantization, the segmentation of the vector quantization dictionary is also necessary, so that the output of the encoder can be segmented to multiple vectors each with relatively small size to perform the quantization.

In addition, whether/how to achieve derivable (approximated) quantization and gradient passing is implementation of model training as per our understanding, so the motivation to discuss its spec impact on model inference is unclear. The distance metric (quantization rule) of vector quantization is also related to the training of the AI/ML model and can also up to implementation.

As a summary, the potential spec impact of the dictionary alignment can be studied, e.g., the format of the scalar/vector quantization dictionary and the method to configure/report/update the dictionary.

Proposal 14: In CSI compression using two-sided model, further study potential specification impact on the vector quantization and scalar quantization.

- For vector quantization,

  - The format/size of the vector quantization dictionary.

  - Segmentation of the encoder output.

  - Configuration/reporting/updating of the quantization dictionary.

- For scalar quantization,

  - Uniform and non-uniform quantization.

  - The configuration of the quantization granularity.

### Legacy CSI reporting principles

In the last RAN1 meeting, the following agreement about legacy CSI reporting principles had been achieved [1].

Agreement In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least: - The priority rule regarding CSI collision handling and CSI omission - Codebook subset restriction - CSI processing Unit

In legacy CSI reporting framework, the priority rules among different CSI report is based on periodicity, content, cell ID, report configuration ID, etc. If AI/ML-based CSI feedback is specified, the priority rule may need to differ whether it is AI/ML-based CSI report or legacy codebook-based CSI report. Moreover, for AI/ML-based CSI feedback, some new contents should be considered for the priority rule, e.g., ground-truth CSI for model training, for model monitoring, and CSI report for model inference. In addition, for the priority rules within single AI/ML-based CSI report for model inference, since the content is a bit sequence without any physical meaning, how to specify the priority rules of this bit sequence needs further study.

For codebook subset restriction, the legacy method is to restrict the amplitude of the group of spatial bases indicated by the Network. For AI/ML-based CSI feedback, since the CSI report doesn’t include the explicit components of spatial bases and amplitude, how to achieve codebook subset restriction needs further study.

For CSI processing Unit, the legacy determination method is based on the reportQuantity and number of CSI-RS resources in the CSI-RS resource set; e.g., different OCPU values are specified for the quantity types of ‘none’, RSRP/SINR, and CQI/PMI/RI. For AI/ML-based CSI feedback, as different UE part models may have different complexity or equivalently required CPU values, it may hardly specify a fixed value of CPU for the AI/ML-based CSI compression report. Therefore, the UE can report and calculate the needed OCPU values for per UE part model basis. For those UE part models without sufficient CPU, the corresponding CSI reports could be dropped or fall back to legacy reporting.

Proposal 15: For the CSI priority rules and CSI processing Unit, on top of the legacy CSI reporting principles, following AI/ML specific aspects may need to be further considered:

- The priority rules for different LCM procedures of training data collection, inference, and monitoring data collection; and the priority rule within the latent space of per CSI report.

- The required CPU value of CSI calculation for per AI/ML model basis.

## Model monitoring

### Monitoring metrics

In the RAN1#110bis meeting, the following agreement related with monitoring had been achieved on the monitoring metrics.

Agreement In CSI compression using two-sided model use case, further study at least the following options for performance monitoring metrics/methods: - Intermediate KPIs as monitoring metrics (e.g., SGCS) - Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK). - Legacy CSI based monitoring: schemes using additional legacy CSI reporting - Other monitoring solutions, at least including the following option: - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection

Input/output data drift

For input or output data based monitoring, since they do not reflect the end-to-end KPIs of AI/ML model but only monitors the distribution of the input/output data, it could not be used to directly identify whether the AI/ML model works well or not. E.g., the AI/ML model may fail due to unmatched Network part model and UE part model even when the distribution of the input/output data is not changed. In addition, as per our knowledge, there is little evaluation on the effect of input/output data in 9.2.2.1. Having that in mind, how the AI/ML performance is reflected by the input/output data distribution, what metrics can be adopted for evaluating the feature of monitored data (e.g., how to quantize the bias between training set and monitor set), and how to generate the distribution of data (e.g., the distribution of SGCS/NMSE for monitored samples?) should be evaluated at 9.2.2.1 before further discussing their spec impacts at 9.2.2.2.

In theory, the distribution of input data would impact the performance of AI/ML models, which can be used as an assistance information (but not standalone) for model switching. If distribution of monitored input data is very different from the distribution of training data, it means unseen data is taken for inference, which may result in degraded inference performance. If the distribution of input data is to be further studied, as a starting point, the probability distribution function (PDF) or centroids of input samples may be used to represent the distribution of monitored input data and training data. The difference of PDF or centroids between input data and training data could be used to represent the data drift or out-of-distribution. For example, for a single input sample, the distance (e.g., Euclidean distance, Cosine similarity) between this sample and the training data centroids could be used for out-of-distribution detection. As another example, for a group of input samples, the divergence between the input data PDF and the training data PDF could be used to calculate data drift.

For the distribution of output data, in contrast, it is rather a result of AI/ML models than the reason of what impacts the AI/ML model, e.g., for biased input data, the output may still have similar distribution as unbiased input data after the AI/ML inference (as the AI/ML has not learnt the feature of the biased/unseen input data), thus the failure of the AI/ML model may hardly be reflected by the output drift.

Proposal 16: The input or output data based monitoring should be evaluated at 9.2.2.1 before being further discussed at 9.2.2.2, including: what metrics can be adopted for evaluating the distribution, how to generate the distribution of data, how accurate the data drift reflects the AI/ML model performance.

Observation 9: If monitoring of input data drift is to be further studied, the data drift or out-of-distribution can be reflected by probability distribution function (PDF) or centroids between monitored input data and training data.

Observation 10: Motivation for output data drift is not clear, since the failure of AI/ML model may not be reflected by the output drift.

Eventual KPI

For the eventual KPI, in particular, a proposal has been discussed in the RAN1#111 meeting.

## Proposal 3-4-2: In CSI compression using two-sided model use case, if eventual KPI is adopted as monitoring metric, further study methods to remove the impacts of other factors other than model performance.

For the Network side monitoring, how to remove the impacts of other factors other than the model performance is up to the Network implementation, e.g., gNB can schedule two PDSCHs under the same MU pairing/resource assignment/MCS mechanism but one applied with the precoder from AI/ML-based CSI compression while the other with the precoder from non-AI/ML-based CSI compression (e.g., Type II CB) for performance comparison. For the UE side monitoring, on the other hand, it may be needed to study some spec impact on whether/how to indicate UE with the two scheduled PDSCHs with different CSI compression schemes, by assuming that the UE side may also need to perform the performance monitoring in terms of eventual throughput. E.g., as the UE is not aware of the how the precoders are generated for the PDSCH transmission which is gNB implementation, it cannot distinguish the PDSCH corresponding to AI/ML-based CSI and the PDSCH corresponding to non-AI/ML-based CSI under the legacy framework. The details are elaborated in Section 3.5.3.

Observation 11: In CSI compression, if eventual KPI is adopted as monitoring metric, the potential spec impact for methods of removing the impacts of other factors other than model performance

- is up to the Network implementation for the Network side monitoring mode.

- can be studied for the UE side monitoring mode.

Power consumption KPI

In general, high performance model is with large model size and imposes high power consumption on the UE device. A balance between model performance and UE power consumption is required in commercialization. In model monitoring, besides the performance KPIs that have between considered for the comparison between AI/ML-based and legacy non-AI/ML methods, it could also consider the report of power consumption from UE to gNB. For example, to indicate whether the power consumption of the undergoing AI/ML model is higher than the legacy method, or to indicate the gap of power consumption between AI/ML method and legacy method. If comparable network performances are monitored between AI/ML and non-AI/ML but the AI/ML is operated with the sacrifice of much higher power consumption than non-AI/ML, gNB may consider triggering the model switching or fallback.

Proposal 17: To assist the model monitoring by taking into account the aspect of power consumption, it can be considered to introduce the metric report of power consumption from UE to gNB, e.g., whether the power consumption of the undergoing AI/ML model is higher (and how much higher if so) than the legacy non-AI/ML method.

### Monitoring modes

In the last RAN1 meeting, the following agreement related with monitoring had been achieved on the monitoring modes, where the Network side monitoring mode and UE side monitoring mode are to be studied as illustrated in Figure 8.

| Agreement In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency. |  |
| --- | --- |
| - Network side monitoring | - UE side monitoring |

Figure 8 Procedure of intermediate KPIs based monitoring

Network side monitoring

For how to report the target CSI (or ground-truth CSI) and the associated CSI reporting, there may be the following three options.

- Option 1: UE reports target CSI via RRC signaling and reports CSI report via UCI.

- Option 2: UE reports both ground-truth CSI and CSI report via UCI.

- Option 3: UE reports both ground-truth CSI and CSI report via RRC, wherein the CSI report is duplicated to the CSI report in the inference.

As analyzed in Section 3.1.2, Option 1 and Option 3 will result in a long latency of the measurement report, which can hardly support the fast identification of network performance fluctuation/degradation at the gNB side. In addition, Option 3 may cause additional UL overhead by reporting a duplicate of CSI report. Therefore, reporting ground-truth CSI and CSI report via UCI should be studied with higher priority for Network side monitoring.

Proposal 18: For Network side monitoring based on intermediate KPI, study the reporting of the target CSI and the associated CSI report by the UE via UCI with higher priority.

UE side monitoring

For UE-side monitoring, considering the L1 signaling, i.e., DCI, is too small to carry the recovery CSI, gNB can use RRC signaling to indicate the recovery CSI to UE. In addition, for each recovery CSI, gNB needs to indicate the association to the corresponding CSI report fed back at the inference stage, e.g., in terms of slot index of the CSI report, so that UE can perform the comparison.

In addition, as demonstrated by Step 4 in Figure 8 (b), whether the UE needs to feedback the calculated intermediate KPI can be further studied, as the gNB can trigger Network side monitoring with much smaller overhead and latency.

Proposal 19: For UE side monitoring based on intermediate KPI, study the indication of the recovery CSI with RRC signaling.

- The association to the corresponding CSI report fed back at the inference stage can be indicated in together.

For the Network side monitoring, it collects the measurements or reports from UE, while how to judge the performance of the current AI/ML model to ensure robust network performance is up to Network implementation. For the UE side monitoring, however, it may have a different strategy of activating/deactivating/selecting/updating the model from the Network, e.g., UE side may have a different SGCS threshold of activating an AI/ML model than Network. To avoid misaligned metrics between Network and UE and the unneeded reporting, Network can configure the threshold criterion to UE, so that UE can perform the monitoring based on the threshold criterion. Further details on the threshold criterion can be explored on the following aspects:

- Usage of the threshold criterion. For example, UE can perform conditional report of the monitoring metrics, where UE reports the monitored information when conditions of the threshold are satisfied, i.e., conditional report of the monitoring metrics. As another example, UE can make the conditional monitoring decisions such as deactivation, switching, etc., when conditions of the threshold are satisfied.

- Types of the threshold criterion. E.g., the threshold can be considered in forms of eventual KPI (e.g., ACK/NACK ratio, throughput, RSRP, etc.) and/or intermediate KPI (e.g., SGCS, NMSE, etc.).

Proposal 20: For UE side performance monitoring, Network may configure a threshold criterion to facilitate UE to perform model monitoring from the following aspects:

- Usage of the threshold criterion, e.g., UE to perform conditional report of monitoring metrics, or to make the conditional monitoring decisions such as deactivation, switching, etc., based on the threshold.

- Types of the threshold criterion, e.g., eventual KPI (e.g., ACK/NACK ratio, throughput, RSRP, etc.) and/or intermediate KPI (e.g., SGCS, NMSE, etc.).

### Co-existence

In the RAN1#110bis meeting, the following agreement related with monitoring had been achieved on the co-existence of AI/ML-based CSI feedback and non-AI/ML-based CSI feedback.

Agreement In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode.

For performance monitoring, the legacy CSI feedback mechanism can be used as a baseline to compare/judge whether the ongoing AI/ML model performs well, and if the AI/ML performance is inferior to the legacy, the model deactivation/switch would be triggered as the monitoring decision. Therefore, co-existence and dynamic switch between AI/ML-based CSI feedback and legacy CSI feedback should be considered. In order to mitigate the fluctuation of other impact factors (e.g., varying channel status, varying scheduling/pairing mechanism, etc.), Network can configure or indicate UE to switch the two modes for real time performance comparison in a monitoring time window. E.g., gNB can configure a time pattern where the AI/ML-based CSI measurement/report and legacy CSI measurement/report operate in different time durations of the pattern; alternatively, the gNB can indicate UE with differentiated measurement resources for the two CSI feedback modes.

In addition, as mentioned in Section 3.5.1, for UE side monitoring based on co-existence between AI/ML-based CSI feedback and legacy CSI feedback, UE is not aware of the mechanism (AI/ML or non-AI/ML) for the scheduled PDSCH by nature. To enable UE to make a pair-wise comparison between AI/ML and non-AI/ML in terms of eventual KPI, Network can indicate which precoding type/CSI feedback mechanism is applied to the PDSCH transmission in the monitoring time window, e.g., PDSCH transmission precoded using the AI/ML-based CSI feedback or using the non-AI/ML-based CSI feedback .

Proposal 21: For the co-existence between AI/ML-based CSI feedback and legacy CSI feedback, further study:

- Configuration/indication of AI/ML-based measurement/report and legacy CSI measurement/report, e.g., configuring separate time durations of different CSI feedback mechanisms, indicating differentiated measurement resources, etc.

- Configuration/indication of the precoding type applied to the PDSCH transmission for UE side performance monitoring, e.g., PDSCH precoded by using AI/ML-based CSI feedback or non-AI/ML-based CSI feedback.

## UE capability

AI/ML solution is a brand-new feature for NR system and not all the UE may support AI/ML-based CSI feedback. Therefore, UE capability for supporting AI/ML-based CSI feedback should be studied. AI/ML-based CSI feedback involves many sub-features, such as data collection, dataset delivery, training type, model switching, model updating, model monitoring and CSI report timeline. Each of the above procedure may require a new UE capability. For example, different UEs may have different capabilities to support data collection/dataset delivery due to different storages; different UEs may or may not support the capacity for model training/updating/monitoring. For the inference, different UEs may have different inference latency which may result in different the CSI report timelines for AI/ML-based CSI feedback.

Proposal 22: Study the potential specification impact for UE capability, including the following as a starting point: data collection, dataset delivery, training, model switching, model updating, monitoring, and CSI report timeline.

# Conclusions

In this contribution, we discussed the potential specification impact and the considerations for CSI compression. Based on the discussions, we have the following observations and proposals.

Observation 1: It is necessary to support real time UE report of the monitoring results to gNB to enable fast identification of network performance fluctuation/degradation and AI/ML model failure.

- E.g., in case of performance degradation, event-triggered monitoring window can be activated so that gNB can efficiently collect data and thereby quickly identify whether the degradation is due to the AI/ML model failure.

Observation 2: For Network side monitoring based on intermediate KPI, the reporting of the ground-truth CSI and the associated CSI report by the UE via L1 signaling has comparable overhead with L3 signaling but with much less latency which can enable fast identification of AI/ML model failure.

Observation 3: The applicable cases for the categorization ID for assisting UE side data collection may need to be further clarified with respect to the following points:

- Generalized model can be trained at UE side over scenarios/antenna layouts.

- UE can autonomously sense the scenario without being notified by gNB.

- The categorization or granularity of the scenarios identified by Network may not match the categorization principle of the UE side.

Observation 4: For CSI compression with two-sided model, training Type 1 may suffer software/hardware compatibility issue, and the following restrictions/issues may need to be considered to relieve the compatibility issue:

- Network may have to interoperate with various UE vendors/UE versions to dedicatedly train the CSI generation part for UE, which harms the engineering isolation.

- Network, in particular gNB, may have to maintain/store multiple CSI generation parts trained for different UE vendors/UE versions.

- Network vendor may not freely develop the CSI generation part for UE, which may restrict the pairing with the CSI reconstruction part and thereby result in sub-optimal performance.

Observation 5: For training Type 1 of CSI compression, compared with joint training at Network side, performing joint training at UE side and delivering the model to the Network incurs extra challenges for Network due to the following reasons:

- Inconvenience of training cell/scenario specific models.

- Inflexible model update.

- Burden of inference/storing/running multiple Network part models at gNB delivered from different UE vendors/UE versions.

Observation 6: For training Type 2 of CSI compression and model updating, it relies on complex design to support real-time interaction of FP/BP iterations between Network side and UE side, which causes strong challenges to engineering isolation especially for the case of multi-Network vendors to multi-UE vendors.

Observation 7: For training Type 3 of CSI compression,

- The shared dataset is constituted by the CSI-related data which may be irrelevant with the user privacy (e.g., user position, etc.).

- The dataset sharing/delivery can be performed under the contract agreement between the Network vendors/MNOs/UE vendors to mitigate the data ownership problem.

Observation 8: For training Type 3 of CSI compression, compared with NW first training, performing UE first training incurs extra challenges for Network due to the following reasons:

- Inconvenience of training cell/scenario specific models.

- Inflexible model update.

- Burden of maintaining/storing multiple Network part models at gNB to pair with multiple UE vendors/UE versions.

Observation 9: If monitoring of input data drift is to be further studied, the data drift or out-of-distribution can be reflected by probability distribution function (PDF) or centroids between monitored input data and training data.

Observation 10: Motivation for output data drift is not clear, since the failure of AI/ML model may not be reflected by the output drift.

Observation 11: In CSI compression, if eventual KPI is adopted as monitoring metric, the potential spec impact for methods of removing the impacts of other factors other than model performance

- is up to the Network implementation for the Network side monitoring mode.

- can be studied for the UE side monitoring mode.

Proposal 1: Start the study of the potential spec impact of CSI prediction after RAN1#112b-e meeting.

Proposal 2: For the enhancement of CSI-RS configurations for Network/UE side data collection under CSI compression, separate CSI-RS resources/CSI reports can be adopted for generating ground-truth CSI labels (e.g., measured with higher power/density CSI-RS) and model inputs (e.g., measured with lower power/density CSI-RS) to support the super resolution of using low resolution input to infer high resolution output.

Proposal 3: For the container of Network side data collection under CSI compression,

- Both of L1 signaling and RRC signaling can be supported for model training.

- At least L1 signaling should be supported for model monitoring to enable fast identification of AI/ML model failure.

Proposal 4: For data sample format of Network side data collection under CSI compression:

- Both of scalar quantization and codebook-based quantization can be supported for model training.

- Codebook-based quantization should be supported for model monitoring.

Proposal 5: For the Network side data collection of ground-truth CSI, the number of ranks and the index(es) of layer(s) for the report of ground-truth CSI can be designated by the gNB rather than autonomously calculated and reported by UE.

Proposal 6: The data categorization ID, if justified, should be determined by Network side in forms of virtualized ID without specifying the physical meaning.

- The physical meaning of such ID and the granularity of such ID is up to Network implementation without being indicated to the UE side.

Proposal 7: In CSI compression using two-sided model with training collaboration Type 3, further study potential specification impact of dataset delivery over air-interface on the following aspects:

- Training dataset and/or other information delivery from UE side to Network side for UE first training.

- Training dataset and/or other information delivery from Network side to UE side for NW first training.

- The specification impact includes dataset ID, the size of the dataset, format of data sample, type(s) of the data sample, quantization/de-quantization related information etc.

Proposal 8: For the dataset delivery of CSI compression over air-interface, the following approaches can be considered to largely alleviate the overhead/power consumption of per UE:

- Quantization on the ground-truth CSI with high resolution quantization format, e.g., R16 Type II-like method with new parameters.

- Network splits the overall dataset into massive subsets each with limited number of data samples (e.g., with comparable overhead as RRC signaling). The subsets can be separately sent to numerous UEs, and all subsets are associated with a common dataset ID for UE side combination.

Proposal 9: For the dataset delivery of CSI compression over air-interface, the dataset ID associated with the delivered dataset can be used to achieve the pairing of the Network part model and the UE part model.

- E.g., for NW first separate training, UEs receive the dataset associated with a dataset ID to perform the training; after the UE part model is trained, UE and gNB will use the dataset ID to achieve the pairing.

Proposal 10: For training Type 1 of CSI compression, deprioritize the mode of joint training at UE side and delivering the model to the Network.

Proposal 11: For CQI determination of CSI compression, consider Option 1 (CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation) as a starting point.

Proposal 12: For Network configuration to determine CSI payload size, gNB can configure a set of candidate CSI payload sizes for each layer separately.

Proposal 13: For UE determination/reporting of the actual CSI payload size, UE reports the selected RI and the index of the actual CSI payload size (among the set of candidate CSI payload sizes) for each layer subject to the selected RI.

- FFS how to map the CSI report on the two parts CSI to avoid redundant feedback.

Proposal 14: In CSI compression using two-sided model, further study potential specification impact on the vector quantization and scalar quantization.

- For vector quantization,

  - The format/size of the vector quantization dictionary.

  - Segmentation of the encoder output.

  - Configuration/reporting/updating of the quantization dictionary.

- For scalar quantization,

  - Uniform and non-uniform quantization.

  - The configuration of the quantization granularity.

Proposal 15: For the CSI priority rules and CSI processing Unit, on top of the legacy CSI reporting principles, following AI/ML specific aspects may need to be further considered:

- The priority rules for different LCM procedures of training data collection, inference, and monitoring data collection; and the priority rule within the latent space of per CSI report.

- The required CPU value of CSI calculation for per AI/ML model basis.

Proposal 16: The input or output data based monitoring should be evaluated at 9.2.2.1 before being further discussed at 9.2.2.2, including: what metrics can be adopted for evaluating the distribution, how to generate the distribution of data, how accurate the data drift reflects the AI/ML model performance.

Proposal 17: To assist the model monitoring by taking into account the aspect of power consumption, it can be considered to introduce the metric report of power consumption from UE to gNB, e.g., whether the power consumption of the undergoing AI/ML model is higher (and how much higher if so) than the legacy non-AI/ML method.

Proposal 18: For Network side monitoring based on intermediate KPI, study the reporting of the target CSI and the associated CSI report by the UE via UCI with higher priority.

Proposal 19: For UE side monitoring based on intermediate KPI, study the indication of the recovery CSI with RRC signaling.

- The association to the corresponding CSI report fed back at the inference stage can be indicated in together.

Proposal 20: For UE side performance monitoring, Network may configure a threshold criterion to facilitate UE to perform model monitoring from the following aspects:

- Usage of the threshold criterion, e.g., UE to perform conditional report of monitoring metrics, or to make the conditional monitoring decisions such as deactivation, switching, etc., based on the threshold.

- Types of the threshold criterion, e.g., eventual KPI (e.g., ACK/NACK ratio, throughput, RSRP, etc.) and/or intermediate KPI (e.g., SGCS, NMSE, etc.).

Proposal 21: For the co-existence between AI/ML-based CSI feedback and legacy CSI feedback, further study:

- Configuration/indication of AI/ML-based measurement/report and legacy CSI measurement/report, e.g., configuring separate time durations of different CSI feedback mechanisms, indicating differentiated measurement resources, etc.

- Configuration/indication of the precoding type applied to the PDSCH transmission for UE side performance monitoring, e.g., PDSCH precoded by using AI/ML-based CSI feedback or non-AI/ML-based CSI feedback.

Proposal 22: Study the potential specification impact for UE capability, including the following as a starting point: data collection, dataset delivery, training, model switching, model updating, monitoring, and CSI report timeline.

# References

RAN1 Chairman’s notes, RAN1#111, November 14-18, 2022.

R1-2301913, “Summary #5 on other aspects of AI/ML for CSI enhancement”, RAN1#112, Moderator (Apple), February 27 – March 3, 2023

R1-2302358, “Evaluation on AI/ML for CSI feedback enhancement”, RAN1#112bis-e, e-Meeting, April 17 – 26, 2023.

R1-2302357, “Discussion on general aspects of AI/ML framework”, RAN1#112bis-e, e-Meeting, April 17 – 26, 2023.