3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2302368

E-meeting, 17-26 April, 2023

Agenda Item:	9.10.1

Source:	Huawei, HiSilicon

Title:	L1 enhancements for inter-cell beam management

Document for:	Discussion and Decision

# Introduction

In RAN1 #112 meeting, the candidate scenarios and solutions on further mobility enhancement have been identified for further study.

In this paper, we share our views on L1 enhancements for L1/L2 triggered mobility, including L1 measurement and reporting, and beam indication. The rest of this paper are structured as follows. Our considerations on L1 measurements and reporting for LTM are provided in section 2 and section 3 respectively. Potential issues regarding beam indication and cell switch command are discussed in section 4. The RAN1 impacts from subsequent LTM are discussed in section 5. In section 6, some other aspects on UE’s behavior after receiving cell switch command and LTM failure issue are discussed. Conclusions are made in section 7.

# L1 Measurements for LTM

## L1 measurement on CSI-RS

CSI-RS based L1 measurement for LTM was discussed in previous meetings. It helps gNB and UE to align the transmitting and receiving beam more accurately and efficiently than SSB based measurement. Considering the limited number of SSB in a cell, SSB is usually transmitted by Base Station with a relative wider beam. A refined beam between UE and BS will be determined by beam training with CSI-RS for BM for data transmission. If the beam refinement in target cell were performed after cell switch, the throughput of UE may drop during the cell switch and beam training, due to lack of beamforming gain. Such gain is critical especially for the UE at the cell edge that throughput is limited by SNR. Moreover, the time/frequency resources of CSI-RS for BM are more flexible.

In current releases, CSI-RS for tracking (TRS) provides better time/frequency synchronization performance than SSB due to its wider bandwidth. To support high modulation and coding scheme (MCS), TRS is required as a reference RS for QCL Type-A in a TCI state. If DL synchronization for candidate cell(s) is only based on SSB before cell switch command, only low MCS can be used in the target cell after cell switch. In Rel-18 LTM, to reduce the interruption for UE to recover high throughput communication (first data transmission), performing TRS tracking for candidate cell before cell switch command should be supported. Furthermore, it is agreed in RAN1#111 that beam indication (TCI) can be carried in cell switch command complying with Rel-17 unified TCI framework. To activate TCI state(s) in the target cell before cell switch command, TRS measurement is also necessary, as it must be used as reference RS for QCL Type-A in a TCI state.

CSI-RS is used in the serving cell for CSI measurement. GNB determines the resource allocation, link adaption and MIMO precoding based on UE’s feedback. In L3 mobility procedure, after UE arriving at new serving cell, BS can only transmit signal to UE with default precoder. It means efficient MIMO operation cannot be used before accurate CSI is measured and reported. It takes long time for UE to measure and report the CSI on the refined beam in the target cell. To reduce the throughput degradation during cell switch, CSI measurement on CSI-RS of candidate cell(s) before cell switch can be supported. UE can feedback measured CSI right after cell switch to target cell and high rank/MCS data transmission can be recovered shortly.

Despite the benefit of CSI-RS based L1 measurement for LTM, it requires additional UE complexity. For example, it may be challenging for UE to tracking extra CSI-RS apart from serving cell’s CSI-RS and those CSI-RS resources may by transmitted out of serving cell’s RBs. Hence, the L1 measurement on CSI-RS may be optional supported by UEs with additional capability.

Proposal 1: In LTM, L1 measurement on candidate cell’s CSI-RS could be supported with UE capability report, including CSI-RS for BM, tracking and CSI.

## Remaining issue on SSB based intra-frequency L1 measurement

RAN4 replied RAN1’s questions on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility in [1]. Regarding the scenario that receiving time difference (RTD) between the SSBs of serving cell and neighbor cell on the same carrier is larger than CP length of the corresponding SCS (also called asynchronous measurement scenario in short), RAN4 agreed to support the scenario and may introduce additional UE capability depending on further discussion. Similar as L3 intra frequency measurement, UE could indicate the capability whether measurement gap is required for L1 measurement. In asynchronous measurement scenario, a measurement gap may be needed for UE do not report the capability of L1 intra frequency measurement without gap. UE may stall the transmission/reception in serving cell and adjust its Rx window to finish the measurement on candidate cell(s). GNB should restrict the scheduling to the UE in the serving cell during the measurement gap.

According to the UE capabilities and different measurement scenario, network should determine whether the measurement gap is required for inter cell L1 measurement. For example, in synchronous scenario where RTD is within CP, no measurement gap is required no matter whether UE has the capability perform intra frequency L1 measurement without gap. On the other side, even for the UEs who are capable of intra frequency L1 measurement without gap, depends on RAN4, there may have an upper limit of DL timing of arrival difference for a UE to support. Thus, network should be informed of the RTD at UE side before configuring the L1 intra frequency measurement. A straightforward solution is to support UE to report an event that RTD exceed CP or cannot be handled without gap by its capability.

Proposal 2: A UE capability of L1 intra-frequency measurement without gap can be introduced for LTM. And L1 measurement gap should be configured for the UE in asynchronous measurement scenario if UE do not indicate the capability.

Proposal 3: Support UE to report the RTD exceed CP or L1 measurement cannot be handled without gap by its capability.

# L1 measurement reporting

## Additional report quantity and container

In previous meeting, L1-RSRP based on SSB has been agreed as L1 report quantity in LTM. Whether to support L1-RSRP on CSI-RS and L1-SINR are FFS. We support to specify L1-RSRP on CSI-RS due to the reasons discussed in section 2.2.

SINR can reflect the communication quality more accurately because the intra/inter-cell interference are considered as well as the received signal strength. In current release, the L3 measurement on SS-SINR and CSI-SINR are used in legacy HO. UE can measure the signal power and interference/noise power on secondary synchronization signals (SSS) of SSB or CSI-RS for candidate cell. In Rel-16, L1-SINR was also specified for beam measurement in the serving cell. In addition to CMR (CSI-RS or SSB) for UE to measure of signal power, IMR is also configured for UE to measure the interference and noise power. The IMR can be either CSI-IM or NZP CSI-RS for interference or both. If L1-SINR is introduced as report quantity for LTM in Rel-18, UE should be configured with IMR for candidate cells. As IMR is a kind of CSI-RS, CSI-RS based measurement for candidate cells is prerequisite for L1-SINR no matter the CMR is SSB or CSI-RS.

Proposal 4: Support to specify L1-RSRP on CSI-RS and L1-SINR on SSB/CSI-RS for candidate cells as optional capabilities.

It was agreed to support at least L1 report with UCI semi-persistently or aperiodically on PUSCH in RAN1#111. There is an FFS on whether to support periodic and semi-persistent PUCCH due to the concern from single company on the flexibility of PUCCH resource to adapt to potential different UCI payload sizes from the report configuration in LTM scenario. For L1 report, UE usually strictly follows gNB’s configuration, which is relatively stable compared with L3 report. Moreover, if UE is configured with PUCCH format 2/3, the number of RB can also be scaled according to the UCI payload size which provide flexibility to some extent. Thus, periodic and semi-persistent L1 report on PUCCH are supported in Rel-17 ICBM, which has similar usage scenario as LTM. We believe L1 report on PUCCH can also be applied in Rel-18 LTM.

Proposal 5: For gNB scheduled L1 measurement report for Rel-18 LTM, semi-persistent report and periodic report on PUCCH are supported.

## Reporting overhead reduction

In the legacy design of L1-RSRP/SINR measurement report, the measurement resources, on which the measurement results to be reported corresponding to a CSI-ReportConfig at same report instance, are configured in a resource set, e.g. NZP-CSI-RS-ResourceSet or CSI-SSB-ResourceSet. Among multiple resources in a resource set, UE can report the results for at most 4 resources in a report instance. In Rel-17 ICBM, the resources in a resource set can associate with either the serving cell PCI or a PCI different from the serving cell PCI.

However, such mechanism may have issues for L1/L2 mobility where the network expects to know the measurement results of candidate cells to determine the target cell. For example, it is possible that the 4 beams in the report are all from the serving cell, which is out of control from network. To solve the issue, based on the configuration, measurement resources in a resource set can be divided into several groups and UE reports N beams for each group in a single reporting instance. The CSI resources associated with same PCI can be grouped together. Following figure shows an example to compare the legacy and proposed solution for reporting.

Figure 1: An example of legacy reporting and enhanced beam group based reporting.

Proposal 6: To ensure the L1-RSRP/SINR report for candidate cells, the measurement resources in a resource set could be divided into multiple groups according to its associated PCI and UE reports for the best N resources in each group at a single reporting instance.

## On Event-triggered reporting

Due to the long periodicity of L3 measurement and large amount of L3 measurement results to be reported, event triggered reporting is supported. By such mechanism, UE is not required to report all results after every measurement, but only report them when certain event (condition) is satisfied. If L1 beam RSRP reporting is conveyed by PUCCH as a periodic manner. The reporting resource needs to be reserved by the Network anyway even if the reporting is not triggered. For the SP/AP reporting, Network can also control the reporting on candidate cell RS based on L1 results of serving cell, i.e. only when the L1 measurement results is not good then Network will trigger the report for candidate cell RS. With the L3 measurement results, L1 based measurement and reporting could be confined in very limited number of candidate cells. The conditions to report L1 results are satisfied in most cases if similar event as L3 measurement report are introduced in L1. Therefore, the benefits of overhead reduction to introduce event triggered scheme in L1 report is marginal.

Observation 1: The benefit to introduce event triggered L1 reporting is marginal for L1/2 mobility, especially when L3 measurement is involved in preparation phase.

On the other side, specifying event triggered L1 report will take significant standard effort considering the long list of FFS identified in RAN1#111. For example, if UE is configured simultaneously with the Network triggered report and UE event triggered report, the report overhead may not be saved.  Network does not know when the event is satisfied and UE is going to report L1 measurement results. It is possible that Network may trigger a measurement and report on the same CSI resource (set) as the event triggered report which is already sent by UE.

As the P/SP/AP L1 reporting with potential enhancement can already meet the requirement in LTM and the event triggered report may not reduce the overhead, the work on event-triggered L1 report should be with low priority.

Observation 2: When both UE event triggered and network triggered reporting are simultaneously configured, and the triggered report have the same or overlapped content, the report overhead cannot be reduced.

Proposal 7: The work on the event-triggered L1 report in RAN1 should be with low priority.

# Cell switch command and beam indication

## Content of cell switch command

During the discussion of RAN1#111 and RAN1#112, several candidate contents to be carried in cell switch command were proposed by companies. First, for the cell identity or cell group identity, it is necessary for UE to recognize the target cell (group) for mobility. However, it is not necessary to directly include it into the command. RAN2 assumes the triggering MAC CE for L1/2 mobility contains at least a candidate configuration index. The configuration index may map to a CellGroupConfig, which includes configurations for serving cell which contains a PCI. On the other hand, it may have security issue to send PCI of serving cell for a UE in plaintext. It may expose UE’s geographical location, which may cause privacy leakage.

Proposal 8: Configuration index can be used to indicate the target cell(s) in the cell switch command.

BWP ID was also mentioned by some companies in their contributions. In L3 handover, BWP ID in the target cell is provided by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id in RRC configuration. Similar design can be reused for LTM, such as including BWP ID in the CellGroupConfig of the candidate cells. Additionally, the initial BWP could also be assumed as the active BWP after cell switch if firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id is not provided in CellGroupConfig.

If beam indication (TCI state) is provided cell switch command and the TCI state is associated with a BWP different from that provided by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id, the BWP ID may be indicated in the cell switch command.

Proposal 9: The active BWP ID for the target cell can be provided by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id in CellGroupConfig for target cell, similar as in L3 handover. If the TCI state indicated in cell switch command is associated with a BWP different from those provided by CellGroupConfig, the active BWP ID associating with the indicated TCI state can be additionally carried in cell switch command.

## Beam indication/activation

In RAN1#111 meeting, it is agreed to design beam indication in cell switch command based on Rel-17 unified TCI framework. It is further clarified in RAN1#112 that the beam indication is carried in the MAC CE for LTM triggering.

Agreement - The agreement on scenario 2 (Beam indication together with cell switch command) at RAN1#111 is further clarified as the following: - Beam indication for the target cell(s) is conveyed in the MAC CE used for LTM triggering for scenario 2

Regarding the beam indication, two options were proposed by companies, i.e. RS index and TCI state ID. The TCI state in Rel-17 unified TCI framework includes several information that a RS index cannot provide. Firstly, TCI state can provide both QCL Type-A RS for fine time/frequency tracking and QCL Type-D RS for spatial domain reference which is essential for FR2 operation. However, an RS index is not clear or cannot provide both time/frequency and spatial domain reference. Secondly, Rel-17 TCI state includes power control related information, e.g. P0, alpha and PL-RS. With the RS index only, UE cannot determine the PL-RS and power control parameters for UL transmission after cell switch. Last but not least, the CC ID and BWP ID associated with the RS for reference also provides more flexibility for Network especially for the CA enabled operation. In comparison, the RS index cannot provide such flexibility hence it is not preferred.

Proposal 10: For the beam indication in the MAC CE used for LTM triggering, TCI state ID is preferred.

In Rel-17 unified TCI state framework, there two TCI state modes called joint and separate. A UE can be provided with TCI state lists for both modes at the same time, i.e. dl-OrJoint-TCIStateList and TCI-UL-State. And gNB use a flag called unifiedTCI-StateType-r17 configured per cell to determine UE’s TCI mode. For the beam indication in LTM triggered MAC CE, it is necessary for UE to be aware of the TCI state mode of corresponding beam indication. For example, the unifiedTCI-StateType-r17 can be configured associated to candidate cells before cell switch. Detailed signalling design is up to RAN2.

Proposal 11: Per candidate cell TCI state mode should be provided to UE to determine the type of TCI state indication in the cell switch command.

Another issue related to TCI state activation/indication is how to configure TCI state list of candidate cells. Usually the TCI state list is configured per cell. However according to the design of Rel-17 beam management, if simultaneousU-TCI-UpdateList were configured then the TCI state list can refer to the reference Cell. This design can be inherited in Rel-18 LTM. A common TCI state pool can only be shared by Cells that configured with simultaneously TCI state updating. For other cases, per Cell TCI state list should be provided to UE before cell switch.

Proposal 12: For a set of CCs (candidate Cells) configured with simultaneous TCI state update, a common TCI state list can be provided in the reference Cell, otherwise, per cell TCI state list should be provided to UE before cell switch.

In RAN2#119e, LTM in CA scenarios are supported according to the following working assumption, where there are more than on target serving cells present.

R2 assumption: Rel-18 L1/L2 mobility includes both non-CA (PCell only) and CA scenarios (PCell and SCell). This includes the following cases - a) the target PCell/target SCell(s) is not a current serving cell (CA CA scenario with PCell change) - b) FFS the target PCell is a current SCell - c) FFS the target SCell is the current PCell.

If both serving cell and target cells support Rel-17 unified TCI framework, the indicated TCI state should be applied to a set of CCs, which are configured for simultaneous TCI update. The beam indication overhead in cell switch command can be saved and SCells can be fast activated when more than one target cells share the same beam indication.

Proposal 13: The beam indication in the cell switch command can be applied to a set of CCs, if simultaneous TCI state updating is configured for more than one target candidate cell.

In addition, there is also scenarios that multiple CC cannot share the same beam indication, or there are more than one set of CCs. For such cases, multiple beam indication can be provided in the cell switch command and each beam indication can be applied to a CC or a set of CCs which share the beam indication.

Proposal 14: One or more than one beam indication for the target cell(s) can be conveyed in the MAC CE used for LTM, each beam indication can be applied to a CC or a set of CCs which share the beam indication.

Besides the beam indication together with cell switch command, the beam indication for candidate cell before the cell switch command is already supported for some scenarios. For example, when the candidate cell is one of SCell or the candidate cell is served as a TRP (e.g. Rel-17 ICBM and Rel-18 mTA mTRP). In these cases, there is data transmission/reception between candidate cell and UE even before the cell switch occurs, thus the beam indication is necessary.

Proposal 15: Support beam indication before cell switch command at least for the case when the candidate cell is one of SCell or the candidate cell is served as a TRP (e.g. Rel-17 ICBM and Rel-18 mTA mTRP).

According to the definition of the intra-cell TCI state activation delay in TS 38.133, the activation delay of TCI state depends on whether the TCI is known or not. For the TCI state activation in LTM procedures, the factors to impact the length of delay may be similar. If the TCI state indicated in the cell switch command can only be activated after the cell switch, a long activation delay is required. It means that a longer interruption between UE stop transmission/reception in serving cell and recover the data transmission in new serving cell. To fast recover the high data rate between UE and target cell, the TCI state of candidate cells should be activated in prior to the cell switch.

Proposal 16: Support TCI activation before cell switch command to reduce the interruption in LTM.

In RAN1#112 meeting, the behaviour of DL synchronization and TCI activation with candidate cell before cell switch command was discussed and several alternatives were listed by FL as following:

- Companies are encouraged to study the following aspects related to the DL synchronization and TCI state activation when Rel-17 unified TCI is used for LTM beam indication: - Timing to perform DL synchronization - Alt.1 Two-step DL synchronization procedure - UE maintains DL synchronization (to find frame boundary and for TA management) with SSB after L1 measurement and then - gNB activates TCI state(s), and then the UE starts DL synchronization (for PDSCH/PDCCH reception) with the QCL source of the TCI states - Alt.2-1 One-step DL synchronization procedure - UE maintains DL synchronization with SSB after L1 measurement - Alt.2-2 One-step DL synchronization procedure - gNB activates TCI state(s), and then UE starts DL synchronization with the QCL source of the TCI states - Necessity for DL synchronization for TA: whether and how DL synchronized is performed before TA - Applicability of CSI-RS (if agreed) in addition to SSB - RAN1 spec impact (UE capability, configuration, activation etc) - Timing of TCI state activation, i.e. whether TCI state activation is performed before TCI state indication or together with TCI state indication.

Alt.1 is the legacy procedure in which UE first acquires DL synchronization with SSB, and then further acquires finer DL synchronization through TCI state activation procedure. According to existing TCI frameworks, only TCI states having TRS as QCL Type-A source RS can be used for PDCCH/PDSCH reception.

For the Alt.2-1, UE can only acquire a rough time synchronization based on SSB, due to the limited bandwidth. Such DL synchronization cannot support a high order MCS transmission as we discussed in previous section.

Regarding the Alt.2-2, we think it is not a complete procedure. According to TS 38.133 section 3.6.7, a DL TCI chain consists of an SSB, and one or more CSI-RS resources. In other words, UE has to measure/track the SSB that belongs to this DL TCI chain if a TCI state is going to be activated.

Considering the DL synchronization and TCI state activation are two separate procedures according to RAN4’s specification and the CSI-RS measurement may optional be supported by UE based on capability reporting, both DL synchronization based SSB and TCI state activation are required before TCI state is indicated to UE in cell switch command.

Proposal 17: both DL synchronization based SSB and TCI state activation are required before TCI state is indicated to UE in cell switch command (Alt 1).

# Discussion on subsequent LTM

RAN2 is considering “Subsequent” LTM:

Use the term “Subsequent” LTM for the case when cell switch between L1/L2 mobility candidates is done without RRC reconfiguration in between.

In this scenario, UE may be indicated a subsequent cell switch command in the target cell shortly after finish a cell switch from the original serving cell. The target cell for subsequent cell switch can be one of the original candidate cells or the original serving cell. From RAN2’s point of view, the high layer configurations for the candidate cells provided by original serving cell can be kept after first cell switch to save signalling overhead and reduce reconfiguration delay. To support subsequent LTM, in addition to the high layer configuration for candidate cell, some other L1/L2 configurations for LTM, such as TA and L1 measurement, should be kept.

- TA

As discussed in AI9.12.2, UE/NW may acquire the TA in advance before cell switch command to reduce the latency. The mechanism is still beneficial in subsequent LTM. Different from the initial LTM, the TA values of some candidate cells and the original serving cell may be already available and valid. These TA values can be kept after cell switch to reduce the latency for the subsequent cell switch. If the TA is maintained by network (UE is not required to receive RAR), whether to keep the TA values acquired when UE is in the original serving cell can be up to network’s implementation. If the TA of candidate cell(s) is maintained by UE (received from RAR), UE should be indicated whether TA values of candidate cells configured in original serving cell should be released or kept.

- L1 measurement results

In legacy handover, after UE successfully access the target cell, the L1 measurement and report is required in the new serving cell for the high data rate transmission, especially in FR2 where finer beam alignment is required. As the candidate cells configured in original serving cell and even the original serving cell itself can be the candidate cells in subsequent LTM, the L1 measurement results acquired when UE is still associated with original serving cell can be kept for subsequent serving cell DU to make the decision. Furthermore, the measurement results of original candidate cell which is current serving cell are also useful for beam refinement after cell switch.

Proposal 18: To achieve the low latency of potential subsequent LTM, at least TA and L1 measurement results can be kept for original serving cell and part of candidate cells configured when UE is associated with original serving cell.

# Other issues on LTM

## Beam application time for LTM

For the intra-cell beam switch, RAN4 has specified TCI state switching delay for different cases. The cases include MAC-CE based TCI switching and DCI based TCI switching. The switching delay may also depend on whether the TCI state is known to UE refer to the definition in TS 38.133. Usually, if the indicated TCI state is unknown, it takes longer time for UE to activate the TCI state.

As the TCI state indication may be included in the cell switch command, the delay for UE to apply this TCI state under new serving cell needs to be considered. In addition to the factors in intra cell beam switch, several LTM specific factors should be considered.

1) TCI state activation before or after cell switch command. If the TCI state for target cell indicated in the cell switch command could be activated in advance, then the application time of TCI state can be shortened.

2) Inter or intra frequency. As the inter-frequency mobility is one of the target scenarios, when UE is indicated a TCI state in the cell switch command and perform inter-frequency cell switch, more time is required to finish the cell switch as well as TCI state switch.

3) Type of the target cell. If the target PCell is current SCell as considered by RAN2, the beam application time could be relatively shorter when the SCell is activated and TCI state is activated as well.

Proposal 19: The beam application time needs to be considered in LTM when TCI state is included in cell switch command.

## UE arrival indication

In Rel-15 L3 based handover as well as Rel-16 CHO, the random access is performed after UE is switched to the target cell. The RACH not only helps UE to acquire UL timing advance and UL grant, it is also treated as a UE arrival indication. In other words, target cell only schedules the UE after random access process finished. In the LTE RACH-less handover, UE can be configured with pre-allocated UL grant in RRC reconfiguration carried by handover command. UE can use the UL resource to transmit message to inform the target cell of its arrival. If the pre-allocated UL grant is not provided, UE can also monitor the PDCCH and wait for dynamic scheduling. For the LTM, as the timing advance of candidate cells may be acquired before cell switch and UE may not be required to send PRACH after receiving cell switch command, the UE arrival indication mechanism specified for the LTE RACH-less handover can be referred.

- UL based mechanism

UE could be configured with UL resource in the candidate cell before cell switch command is issued. After RRC reconfiguration is complete, UE can transmit an UL signal to the new serving cell. The UE specific UL resources could be dedicated PUCCH, or configured grant PUSCH resource. No matter which kind of UL resource adopted, target cell DU need to provide and reserve the resource for the UE and coordination among cells/DUs is needed.

- DL based mechanism

After UE applying cell switch command, according to pre-configuration, UE may start to monitor a search space associated with CORESET in the target cell. In the network, source cell DU may inform the target cell that UE is migrating in advance. And target cell can start to transmit dynamic scheduling indication on a specific CORESET.

Proposal 20: UE could be configured with PUSCH/PUCCH resource in the target cell before cell switch command is issued. FFS: beam application, timing, power control etc.

## LTM failure issue

In legacy L3 handover, a timer called T304 is initiated when HO command is received. If UE cannot complete RACH procedure in target cell before T304 expires, UE initiate RRC re-establishment with source cell. Although the duration of this timer is configurable, such failure detection and recovery process is not based on L1 measurement and may take long time. Moreover, DL sync and RACH with target cell may not happen after receiving cell switch command in LTE. Hence the handover failure due to RACH failure may not happen in LTM. It was also agreed that beam indication together with cell switch command. Since the target cell and target beam is determined by serving DU based on UE’s L1 measurement results, the delay between cell switch decision and UE apply the target cell configuration could be large. It is possible that the indicated TCI state determined by serving DU in the cell switch command is blocked or failed.

Observation 3: Different from L3 handover, the LTM failure may not results from RACH failure during the cell switch procedure.

To fast detect the HO failure in cell switch command, a BFD/RLM-like procedure could be considered. For example, the periodic CSI-RS associated to the indicated TCI state or a set of preconfigured RSs for radio link monitoring in target cell can be used to detect the DL failure. Furthermore, whether the success of UL transmission on the preconfigured resource (as discussed in section 6.2) can be used to determine the validity of TA indicated in cell switch command. Once the LTM failure is detected, UE can directly fall back to source cell and recover the beam or radio link with CFRA procedure instead of waiting for expiration of T304 and performing RRC re-establishment.

Proposal 21: UE can additionally determine the LTM failure according to L1 measurement of DL RS and success of UL transmission on preconfigured resource in the target cell. UE can recover the link with source cell when LTM failure is detected without waiting for expiration of T304.

# Summary and conclusion

In this contribution, we observe and propose the following:

Observation 1: The benefit to introduce event triggered L1 reporting is marginal for L1/2 mobility, especially when L3 measurement is involved in preparation phase.

Observation 2: When both UE event triggered and network triggered reporting are simultaneously configured, and the triggered report have the same or overlapped content, the report overhead cannot be reduced.

Observation 3: Different from L3 handover, the LTM failure may not results from RACH failure during the cell switch procedure.

Proposal 1: In LTM, L1 measurement on candidate cell’s CSI-RS could be supported with UE capability report, including CSI-RS for BM, tracking and CSI.

Proposal 2: A UE capability of L1 intra-frequency measurement without gap can be introduced for LTM. And L1 measurement gap should be configured for the UE in asynchronous measurement scenario if UE do not indicate the capability.

Proposal 3: Support UE to report the RTD exceed CP or L1 measurement cannot be handled without gap by its capability.

Proposal 4: Support to specify L1-RSRP on CSI-RS and L1-SINR on SSB/CSI-RS for candidate cells as optional capabilities.

Proposal 5: For gNB scheduled L1 measurement report for Rel-18 LTM, semi-persistent report and periodic report on PUCCH are supported.

Proposal 6: To ensure the L1-RSRP/SINR report for candidate cells, the measurement resources in a resource set could be divided into multiple groups according to its associated PCI and UE reports for the best N resources in each group at a single reporting instance.

Proposal 7: The work on the event-triggered L1 report in RAN1 should be with low priority.

Proposal 8: Configuration index can be used to indicate the target cell(s) in the cell switch command.

Proposal 9: The active BWP ID for the target cell can be provided by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id in CellGroupConfig for target cell, similar as in L3 handover. If the TCI state indicated in cell switch command is associated with a BWP different from those provided by CellGroupConfig, the active BWP ID associating with the indicated TCI state can be additionally carried in cell switch command.

Proposal 10: For the beam indication in the MAC CE used for LTM triggering, TCI state ID is preferred.

Proposal 11: Per candidate cell TCI state mode should be provided to UE to determine the type of TCI state indication in the cell switch command.

Proposal 12: For a set of CCs (candidate Cells) configured with simultaneous TCI state update, a common TCI state list can be provided in the reference Cell, otherwise, per cell TCI state list should be provided to UE before cell switch.

Proposal 13: The beam indication in the cell switch command can be applied to a set of CCs, if simultaneous TCI state updating is configured for more than one target candidate cell.

Proposal 14: One or more than one beam indication for the target cell(s) can be conveyed in the MAC CE used for LTM, each beam indication can be applied to a CC or a set of CCs which share the beam indication.

Proposal 15: Support beam indication before cell switch command at least for the case when the candidate cell is one of SCell or the candidate cell is served as a TRP (e.g. Rel-17 ICBM and Rel-18 mTA mTRP).

Proposal 16: Support TCI activation before cell switch command to reduce the interruption in LTM.

Proposal 17: both DL synchronization based SSB and TCI state activation are required before TCI state is indicated to UE in cell switch command (Alt 1).

Proposal 18: To achieve the low latency of potential subsequent LTM, at least TA and L1 measurement results can be kept for original serving cell and part of candidate cells configured when UE is associated with original serving cell.

Proposal 19: The beam application time needs to be considered in LTM when TCI state is included in cell switch command.

Proposal 20: UE could be configured with PUSCH/PUCCH resource in the target cell before cell switch command is issued. FFS: beam application, timing, power control etc.

Proposal 21: UE can additionally determine the LTM failure according to L1 measurement of DL RS and success of UL transmission on preconfigured resource in the target cell. UE can recover the link with source cell when LTM failure is detected without waiting for expiration of T304.

# References

[1]   R4-2303308, Reply LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility, RAN4, Athens, GR, February 27th – March 03rd, 2023