3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2302375

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.1.4.1

Source:	Huawei, HiSilicon

Title:	Discussion on UL precoding indication for multi-panel transmission

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide our views on RAN1#112 meeting agreements and discuss some of our considerations for sDCI/mDCI based STxMP transmission of PUSCH/PUCCH.

# Discussion

## DFT-s-OFDM support for STxMP

It was discussed in the last meeting [1] whether DFT-s-OFDM waveform is supported for STxMP. In current specifications, DFT-s-OFDM waveform is supported for UL transmission where the number of transmission layers is limited to 1. DFT-s-OFDM is usually used in coverage limited scenarios such as the coverage edge of a gNB which is also a target use case of STxMP. Therefore, it is well-justified to support DFT-s-OFDM for STxMP.

Three transmission schemes are supported for STxMP, i.e., SDM, SFN and mDCI based STxMP. In SFN transmission, the same layer(s) of a PUSCH are transmitted by the two panels in an SFN manner. Similar to the legacy DFT-s-OFDM based transmission, if DFT-s-OFDM is configured for SFN, the number of PUSCH layers should be limited to 1. In mDCI based STxMP, two PUSCHs are transmitted from the two panels, respectively. If DFT-s-OFDM is configured for mDCI based STxMP, the number of layers for each PUSCH should be limited 1. As such, the legacy rank 1 restriction of each DFT-s-OFDM based PUSCH transmission would still be satisfied.

In RAN1 112, some companies mentioned that the spec impact of supporting DFT-s-OFDM for SFN and mDCI based STxMP should be clarified. To our understanding, the support for DFT-s-OFDM for SFN and mDCI based STxMP does not require any spec impact. To clarify, to support DFT-s-OFDM, the restriction of a single layer transmission per PUSCH is already imposed in the current spec. In the current spec, when DFT-s-OFDM is configured (transform precoding is enabled), the corresponding TPMI tables only contain precoders with one layer. Similarly, the two TPMIs indicated for SFN PUSCH or mDCI based STxMP PUSCH would only have one layer and the same TPMI tables as in the current spec can be reused.

Observation 1: The support for a single-layer DFT-s-OFDM SFN PUSCH or a single-layer DFT-s-OFDM per PUSCH for mDCI based STxMP does not have any specification impact. In particular, current TPMI tables applicable to DFT-s-OFDM can be reused for DFT-s-OFDM based SFN and mDCI based STxMP transmission.

Above discussion shows that, in fact, a single-layer DFT-s-OFDM SFN PUSCH or a single-layer DFT-s-OFDM per PUSCH for mDCI based STxMP is already supported in the specifications. However, this is apparently not a consensus among all companies. Therefore, RAN1 still needs to have the common understanding that single-layer DFT-s-OFDM is supported for SFN and mDCI based STxMP. Otherwise, gNB may not configure a single-layer DFT-s-OFDM SFN PUSCH or a single-layer DFT-s-OFDM per PUSCH for mDCI based STxMP, as it does not know whether the UE vendor would support it. Therefore, we suggest the following proposal.

Proposal 1: Support DFT-s-OFDM for SFN and mDCI based STxMP.

- For SFN, the number of layers is limited to 1;

- For mDCI based STxMP, the number of layers for each PUSCH is limited to 1;

Note: No spec impact is expected to support DFT-s-OFDM for SFN and mDCI based STxMP. Current TPMI tables for DFT-s-OFDM can be reused to support above two schemes.

For SDM transmission, the number of PUSCH layers is at least 2 and, unlike in the case of SFN or mDCI-based STxMP, it is not possible to restrict the number of layers of each PUSCH to 1. Therefore, whether to support DFT-s-OFDM for SDM may need more discussion. In the current specifications, more than 1 layer is not supported when DFT-s-OFDM is configured. As discussed, with multiple layers, the single carrier property of each layer may be distorted due to the precoding. This leads to a PAPR increase and, consequently, the loss of the benefit of using DFT-s-OFDM. For example, as shown in Fig. 1, when a fully coherent codebook is adopted for the two layers, there will be cross mapping in precoding, i.e., both layers are mapped to the same antenna ports. This would disrupt the single-carrier property of each layer and may lead to the PAPR increase of the signal. However, in SDM PUSCH with the layer combination {1+1}, the precoding of each panel is independent and each layer is only mapped to the antenna port(s) of one panel and, hence, the single carrier property of each layer is preserved. Therefore, it is possible to support DFT-s-OFDM for the SDM transmission while maintaining its low PAPR property. However, the number of layers transmitted from each panel should be limited to 1.

Figure 1 Procedure of data processing with DFT-s-OFDM

Observation 2: In legacy UL transmission, transmitting multiple layers with DFT-s-OFDM may disrupt the single carrier property of DFT-s-OFDM and lead to an increase in PAPR. However, in SDM transmission with the layer combination {1+1}, the single carrier property of DFT-s-OFDM is maintained and, hence, the benefit of DFT-s-OFDM can be fully preserved.

Similar to the case of DFT-s-OFDM for SFN and mDCI based STxMP, the support for DFT-s-OFDM for SDM does not involve any spec impact as the same TPMI tables for DFT-s-OFDM in the current spec can be reused for the single-layer SDM transmission from each panel.

Proposal 2: Support DFT-s-OFDM for SDM transmission in which only one layer per panel is transmitted.

Note: No spec impact is expected to support DFT-s-OFDM for SDM based STxMP. Current TPMI tables for DFT-s-OFDM can be reused to support above scheme.

## SDM based STxMP of PUSCH

### Maximum number of layers for sTRP and SDM based STxMP

Working Assumption (RAN1 112) For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following: - For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1) - For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1) - FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same. - Note: This corresponds to the case that digital ports are shared between the panels - Note: RAN1 supports both implementations that digital ports are shared or separate among panels

In the last meeting [2], a working assumption was achieved on the maximum number of layers for each panel for sTRP and SDM based STxMP schemes. This working assumption is based on the underlying common understanding that only symmetric panels are supported in Rel-18 STxMP. Considering the limited remaining TUs in Rel-18 and a substantial specification impact required to support asymmetric panels, we prefer to confirm the working assumption.

Proposal 3: Confirm the working assumption in RAN1 112 regarding the maximal number of layers of sTRP and SDM schemes.

According to the WA, one remaining issue is whether/how to enable the case that the total number of used PUSCH antenna ports for the sTRP and SDM is the same which corresponds to the architecture in which digital ports are shared between the panels. This will be further discussed in the next section.

### TPMI indication sTRP and SDM based STxMP

In this section, we focus on TPMI indication for CB based sTRP and SDM scheme. SRI indication for NCB based sTRP and SDM scheme follows a similar principle.

#### Reporting available SRS ports for SDM transmission

In the last meeting [2], it was agreed that RAN1 supports both architectures with shared and non-shared digital ports between two panels. Figure 2 shows a non-shared digital port architecture wherein four digital ports are separately deployed for each panel. In this example, all four digital ports of the selected panel are available for the sTRP transmission. Similarly, all four digital ports of each of the two panels are available for SDM transmission amounting to the total of 8 available digital ports over the UE. In turn, Figure 3 shows a shared digital port architecture wherein the total of 4 ports are shared between the two panels. Similar to the non-shared digital port architecture, all 4 digital ports of the selected panel are available for the sTRP transmission. However, since the 4 digital ports should be shared between the two transmitting panels for SDM, only a subset of these digital ports is usable for each panel transmission. For instance, in Figure 3, port 0 and 1 are used for panel #1 transmission and port 2 and 3 are used for panel #2 transmission. Since, in general, each of the used digital ports is measured with an SRS port, above observation translates into another observation which is of more interest to RAN1 and, that is, in architectures with shared digital ports, the available SRS ports per panel for SDM transmission is a subset of the available SRS ports for that panel for sTRP transmission.

Observation 3: In architectures with shared digital ports, in general, the available SRS ports per panel for SDM transmission is a subset of the available SRS ports for that panel for sTRP transmission.

Figure 2. Non-shared digital ports between two panels

Figure 3. Shared digital ports between two panels

Since in architectures with shared digital ports, in general, only a subset of SRS ports per panel are available for SDM transmission, UE should report the available SRS ports per panel for the SDM transmission to the gNB. gNB would require this information to estimate UL channels based on the SRS ports that are actually available for the SDM transmission and, consequently, to indicate a suitable TPMI for each panel transmission. To further explain this, let us again consider the UE in Figure 3 for which two SRS resource sets with 4 ports per SRS resource is configured. Once UE transmits the configured 4 port SRS resources from the two panels, gNB would be able to estimate the UL channels corresponding to all four SRS ports of each of the two panels. However, without knowing which subset of SRS ports at each panel is actually available for the SDM transmission, gNB would be unable to determine suitable TPMIs for the SDM transmission from each panel. In particular, in Figure 3, the SDM TPMI corresponding to panel#1 should be based on the channel measurements on SRS port 1 and SRS port 2 of the SRS resource set transmitted from the panel 1 while the SDM TPMI corresponding to panel 2 should be based on the channel measurements on SRS port 3 and SRS port 4 of the SRS resource set transmitted from panel 2. Note also that, reporting the available subset of SRS ports per panel for SDM transmission would also implicitly signal the maximum allowed length of the SDM precoder. For instance, in Figure 3, if UE does not report that only SRS port 1 and SRS port2 from the first panel and SRS port 3 and SRS port 4 from the second panel are available for the SDM transmission, and, hence, the maximum permissible length of each of the two SDM precoders should be 2, gNB would assume that, similar to the sTRP transmission, all 4 SRS ports of each of the two panels are available and may indicate two TPMIs each with four SRS ports for the SDM transmission. Note that above UE report on the available SRS ports per panel for SDM transmission is only necessary for architectures with shared digital ports. If UE does not provide this report, gNB may assume that all ports of each panel are available for the SDM transmission.

Observation 4: For the SDM transmission from UEs with shared digital ports architecture, to indicate a suitable TPMI pairs, gNB would require to estimate UL channels based on the subset of SRS ports that are actually available for the SDM transmission at each panel.

Proposal 4: For the SDM transmission, UE optionally reports the supported subset of SRS ports of each panel.

- If UE does not report the supported subset of SRS ports, gNB assumes that all SRS ports are available for the SDM transmission.

#### TPMI table selection for SDM transmission

For the sTRP transmission, UE selects the precoder by mapping the TPMI field onto the TPMI index of the selected TPMI table. The TPMI table is selected based on multiple parameters including the number of ports of the indicated SRS resource for the PUSCH transmission. For TPMI indication of each panel in SDM transmission, if digital ports are not shared between panels (i.e., for each panel, the supported subset of SRS ports for SDM transmission is not reported), the above mechanism can be reused as all ports of the indicated SRS resource are used for the PUSCH transmission. However, if digital ports are shared between panels (i.e., for each panel, the supported subset of SRS ports for SDM transmission is reported), the above mechanism is not suitable as the SRS ports used for the SDM transmission from each panel is a subset of the ports of the indicated SRS resource for that panel. In such a case, the TPMI table for each panel should be selected based on the number of reported SRS ports for the SDM transmission from that panel.

Proposal 5: For TPMI indication in SDM transmission, the TPMI table for each panel is selected based on the number of SRS ports used for the SDM transmission from the panel, that is,

- The number of SRS ports in the reported subset of SRS ports for the SDM transmission from the panel, if such report is provided by the UE;

- The number of configured SRS ports for the panel, otherwise.

#### TPMI field design

To support dynamic switching between sTRP and SDM schemes, it is necessary to align the DCI size for sTRP and SDM transmission. For sTRP transmission, one TPMI is indicated while, for SDM transmission, two TPMIs are indicated. Therefore, it should be ensured that, including possible zero padded/reserved bits, the total number of bits of the two TPMI fields for the SDM transmission is equal to the number of bits of the single TPMI field for the sTRP transmission.

A simple solution to guarantee such bit-length alignment is as follows: The size of the first TPMI field is max{LsTRP, LSDM,1} where LsTRP is the number of bits needed for the TPMI indication in the sTRP transmission and LSDM,1 is the number of bits needed for the TPMI indication corresponding to the first panel in the SDM transmission. In turn, the size of the second TPMI field is LSDM,2 where LSDM,2 is the number of bits needed for the TPMI indication corresponding to the second panel in the SDM transmission. For the sTRP transmission, the second TPMI field is reserved. Above design is simple, but it increases the DCI overhead. For example, when 6 bits are needed for the TPMI indication in the sTRP transmission and 4 bits are need for the TPMI indication for each panel in the SDM transmission, above design consumes 10 DCI bits while, for sTRP and SDM TPMI indication, the total number of 6 and 4+4=8 bits are required, respectively. Such waste of DCI bits is due to the facts that, regardless of the actual transmission scheme, the first TPMI field has the maximum length between the bit length required for the sTRP and the SDM TMPI indication and, further, all bits of the second TPMI field are unused in the case of sTRP transmission.

Above waste of DCI bits can be avoided if the bits in the two TPMI fields for the SDM transmission are concatenated to indicate the TPMI for the sTRP transmission. In this more economic design, the size of each SDM TPMI field LSDM,1 or LSDM,2 (without considering possible reserved bits) equals to the actual number of required bits for the TPMI indication for each panel, which, as discussed before, can be derived based partly on the number of the available SRS ports for the SDM transmission (all ports or a reported subset of ports of the configured SRS resources; see section 2.2.2.2). In turn, the size of TPMI for sTRP transmission LsTRP (without considering possible reserved/zero padded bits) equals to the actual number of required bits for the TPMI indication for sTRP.

There are two possible cases regarding the lengths of the two SDM TPMI fields relative to the length of the sTRP TPMI field:

- Case 1: The total number of required bits for the two SDM TPMI indications is not smaller than the number of bits required for the sTRP TPMI indication, that is, LSDM,1 + LSDM,2 >= LsTRP (See the left subfigure of Figure 4)

- Case 2: The total number of required bits for the two SDM TPMI indications is smaller than the number of bits required for the sTRP TPMI indication, that is, LSDM,1 + LSDM,2 < LsTRP (See the left subfigure of Figure 5)

In the first case, LsTRP bits of the LSDM,1 + LSDM,2 bits are used to indicate the TPMI for sTRP transmission. Other LSDM,1 + LSDM,2 - LsTRP bits are ignored by the UE or padded with zero as shown in the right subfigure of Figure 4.

In the second case, in order to indicate sTRP TPMI, additional LsTRP – (LSDM,1 + LSDM,2) bits are required. To accommodate this, a reserved field with LsTRP – (LSDM,1 + LSDM,2) bits is included in the DCI. The two TPMI fields and the reserved field are concatenated together to indicate the LsTRP bits of sTRP TPMI as shown in the right subfigure of Figure 5. The presence and the length of the reserved field can be configured by gNB or derived by UE based on LsTRP, LSDM,1, and LSDM,2: The reserved field is present in DCI only when LSDM,1 + LSDM,2 is smaller than LsTRP and its length is LsTRP – (LSDM,1 + LSDM,2).

Figure 4 Case 1: LSDM,1 + LSDM,2 >= LsTRP

Figure 5 Case 1: LSDM,1 + LSDM,2 < LsTRP

Proposal 6: The lengths of each of the two TPMI fields for SDM transmission are derived based on the maximum number of layers and the number of SRS ports used for the SDM transmission from the corresponding panel.

For sTRP TPMI indication, the two TPMI fields are concatenated as one TPMI field as follows to reduce the DCI payload:

- If the total number of bits of the two TPMI fields LSDM,1 + LSDM,2 is not smaller than the LsTRP bits needed for the sTRP TPMI indication, the first LSDM,1 + LSDM,2 - LsTRP bits of the bit string that carries the two TPMI fields is ignored by the UE (reserved or zero padded);

- If the total number of bits of the two TPMI fields LSDM,1 + LSDM,2 is smaller than the LsTRP bits needed for the sTRP TPMI indication, a reserved field with LsTRP – (LSDM,1 + LSDM,2) bits is further concatenated with the two TPMI fields.

### PTRS-DMRS association

Agreement (RAN1 112) When max 2 PTRS ports are configured for SDM scheme of single-DCI based STxMP PUSCH: - Actual number of PTRS ports in SDM is 2 and 2-bit “PTRS-DMRS association” DCI field is used to indicate the PTRS-DMRS association for the DMRS ports associated with two TMPI/SRI fields. - The MSB indicates the association between PTRS port 0 and the DMRS port(s) associated with the first TPMI/SRI field. - The LSB indicates the association between PTRS port 1 and the DMRS port(s) associated with the second TPMI/SRI field. - Regarding the “ptrs-PortIndex” configured to SRS resource for NCB PUSCH of SDM scheme, the UE ignores the configuration of “ptrs-PortIndex” per SRS resource. FFS: Whether additional RRC configuration is needed for the max number of PTRS ports for SDM transmission

For SDM, it was agreed that the actual number of PTRS ports is 2 and the 2-bit “PTRS-DMRS association” DCI field is used to indicate the PTRS-DMRS association for the DMRS ports associated with the two TMPI/SRI fields. Therefore, regarding the FFS of the above agreement, additional RRC configuration for the max number of PTRS ports is not needed for SDM transmission.

Observation 5: Additional RRC configuration for the max number of PTRS ports is not needed for SDM transmission since the actual number of PTRS ports for SDM is 2.

### Codebook capability reporting

In current specification, UE reports the supported codebook type by UE capability parameter pusch-TransCoherence. Currently, UE only reports one codebook capability that is applicable to the transmitting panel. In STxMP where two panels are used for the transmission, each panel may have different capability for supporting of UL codebook type. For example, one panel may support fullCoherent/partialCoherent/nonCoherent codebook, while the other panel may only support partialCoherent/nonCoherent codebook. In such a case, the legacy mechanism on reporting a single UE level codebook capability is sub-optimal as the more conservative choice among the two possibilities would have to be reported. For instance, in the above example, if the legacy mechanism is used, ‘partialCoherent’ must be reported for the UE. In our view, in order for the gNB to obtain the codebook capability of each panel, panel specific codebook capability should be reported for STxMP.

Observation 6: Legacy UE level codebook capability reporting is not applicable to STxMP as the two panels may have different codebook capabilities.

Proposal 7: Panel specific codebook capability reporting should be supported for STxMP.

### UCI multiplexing with STxMP of PUSCH

NR supports UCI multiplexing with PUSCH since Rel-15. When UCI and UL data are present at the same time, they can be multiplexed in the same PUSCH resource. UCI includes essential UL information. Therefore, its coverage is important and usually uses the QPSK modulation with a low code rate. On the other hand, PUSCH may require a better spectrum efficiency and a higher MCS order may be configured. To ensure the coverage when UCI is multiplexed into PUSCH resources, specifications allow the gNB to control the code rate as well as the proportion of REs for the multiplexed UCI. The UCI-OnPUSCH IE can be RRC configured for PUSCH and it includes betaOffsets and scaling parameters where betaOffsets is used to control the offset of the UCI bit rate with respect to the data bit rate and the scaling is used to specify the upper limit of the proportion of REs that can be used by the UCI to all REs that are allocated for PUSCH. For the STxMP scenario, UCI may also be multiplexed with such STxMP PUSCH. Further study is needed on how to configure or indicate the UCI multiplexing on the STxMP PUSCH; in particular, whether or not panel-specific betaOffsets and scaling are required.

Proposal 8: To adapt to different pathloss/channel conditions of the two panels, for UCI multiplexing on STxMP PUSCH, study whether or not to support panel specific PUCCH bit rate and/or PUCCH REs ratio configuration.

## SRS configuration

Agreement (RAN1 110b-e) For SDM scheme of single-DCI based STxMP PUSCH - Configure two SRS resource sets for CB or NCB . - FFS: These two SRS resource sets can have different number of SRS resources for codebook -based or non-codebook based. - For codebook -based PUSCH , DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set. - For non-codebook based PUSCH and codebook -based PUSCH , DCI indicates two SRI fields and each field indicates SRS resource(s) for each SRS resource set separately. FFS : For codebook -based PUSCH , the two SRS resources indicated by the two SRI fields can have different number of SRS ports Agreement (RAN1 111) For the SFN scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets for CB or NCB. - FFS: Number of SRS resources of SRS resource set, and number of SRS ports of SRS resource - The DCI indicates two SRI fields and TPMI fields for SFN transmission, - On the indication of number of layers for CB and NCB PUSCH: - Alt1: Similar to rel-17 mTRP TDM scheme, the number of layers is indicated by the first SRI field (for NCB PUSCH) or the first TPMI field (for CB PUSCH);

In RAN1#112, it was agreed as a WA for SDM to support only one configured maximal number of layers that is applied to both SRS resource sets. Note that the number of layers of a panel is upper bounded by the maximal number of SRS ports of the panel and this WA was reached due to the underlying understanding that only the symmetric panel assumption is supported in Rel-18 where the number of antenna ports of both panels is the same. Given this context, we think the same legacy CB/NCB SRS configuration restrictions with respect to the number of SRS ports and the number of SRS resources per set should be applied for CB/NCB SRS configuration for both STxMP SDM and STxMP SFN transmission schemes.

Proposal 9: For both CB and NCB based STxMP SFN and STxMP SDM transmission schemes, the number of SRS resources in the two SRS resource sets should be the same.

- For CB based transmission, except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', support up to 2 SRS resources in a resource set.

- For NCB based transmission, support up to 4 single-port SRS resources in a resource set.

Proposal 10: For CB based STxMP SFN and STxMP SDM transmission schemes, support SRS resources with up to 4 ports.

- Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2’, the number of ports of the configured SRS resources within a set and across the two sets should be the same.

## mDCI based STxMP of PUSCH

### DMRS configuration

Agreement (RAN1 110b-e) Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18. - Two independent PUSCHs associated with different TRPs can be transmitted by a UE simultaneously in same active BWP. - The total number of layers of these two PUSCHs is up to 4. - FFS: whether the number of layers of each of these two PUSCHs is up to 2.

For mDCI based STxMP PUSCH+PUSCH, it was agreed in RAN1#110b that the total number layers of the two scheduled PUSCHs is 4. One remaining issue is whether to restrict the number of layers of each PUSCHs to be up to 2. We think such restriction is reasonable as it is also adopted for the sDCI based SDM PUSCH and mDCI based PDSCH transmission.

Proposal 11: For mDCI based STxMP PUSCH+PUSCH, the number of layers of each PUSCH is up to 2 layers.

To simplify the UE implementation in Rel-16 mDCI based PDSCH transmission, the UE does not expect that different DM-RS configuration with respect to the actual number of front-loaded DM-RS symbol(s), the actual number of additional DM-RS symbol(s), the actual DM-RS symbol location, and DM-RS configuration type, are configured for the two scheduled PDSCHs. Similarly, to simplify the implementation of Rel-18 mDCI based STxMP PUSCH+PUSCH, such restrictions should also be adopted.

Proposal 12: For the two scheduled PUSCH in mDCI based STxMP PUSCH+PUSCH, support the same DM-RS configuration with respect to the following parameters:

- the actual number of front-loaded DM-RS symbol(s),

- the actual number of additional DM-RS symbol(s),

- the actual DM-RS symbol location,

- DM-RS configuration type.

### DG-PUSCH and CG-PUSCH

Agreement Multi-DCI based STxMP PUSCH+PUSCH transmission at least supports the following PUSCH combinations: - DG-PUSCH + DG-PUSCH - CG-PUSCH + DG-PUSCH Agreement Support CG PUSCH + CG PUSCH in multi-DCI based STxMP PUSCH+PUSCH transmission.

PUSCH-TRP association should be discussed in 9.1.1.1 AI as it is related to the beam determination in unified TCI framework for mTRP. More details on our views can be found in our companion paper [3]. In the legacy design, some rules/restrictions were introduced to avoid overlapping CG-PUSCH and DG-PUSCH resources. However, for a UE with the STxMP capability, these rules should be updated. For example, if UE is able to perform STxMP and the pair of TCI states (one for CG and one for DG) can be used for simultaneous transmission, then both PUSCH transmissions could be performed. Otherwise, one of the PUSCH transmissions, e.g. CG-PUSCH resource in the current period, may be dropped.

Proposal 13: For mDCI based STxMP CG-PUSCH+DG-PUSCH, unless TCI states for CG-PUSCH and DG-PUSCH can be simultaneously used for transmission, one of the PUSCH transmissions should be dropped.

In addition, for mDCI based STxMP CG-PUSCH+DG-PUSCH, dropping rules due to UE Tx power limitation should be considered.  For example, if an ongoing PUSCH transmission, e.g., DG-PUSCH, has already used most of the maximum transmission power Pcmax, the other PUSCH, e.g. CG-PUSCH, that is overlapped with the ongoing PUSCH should be dropped.

Proposal 14: For mDCI based STxMP CG-PUSCH+DG-PUSCH, dropping rules due to UE Tx power limitation should be considered.

# Conclusions

In this contribution, we have the following observations and proposals

Proposals:

Proposal 1: Support DFT-s-OFDM for SFN and mDCI based STxMP.

- For SFN, the number of layers is limited to 1;

- For mDCI based STxMP, the number of layers for each PUSCH is limited to 1;

Note: No spec impact is expected to support DFT-s-OFDM for SFN and mDCI based STxMP. Current TPMI tables for DFT-s-OFDM can be reused to support above two schemes.

Proposal 2: Support DFT-s-OFDM for SDM transmission in which only one layer per panel is transmitted.

Note: No spec impact is expected to support DFT-s-OFDM for SDM based STxMP. Current TPMI tables for DFT-s-OFDM can be reused to support above scheme.

Proposal 3: Confirm the working assumption in RAN1 112 regarding the maximal number of layers of sTRP and SDM schemes.

Proposal 4: For the SDM transmission, UE optionally reports the supported subset of SRS ports of each panel.

- If UE does not report the supported subset of SRS ports, gNB assumes that all SRS ports are available for the SDM transmission.

Proposal 5: For TPMI indication in SDM transmission, the TPMI table for each panel is selected based on the number of SRS ports used for the SDM transmission from the panel, that is,

- The number of SRS ports in the reported subset of SRS ports for the SDM transmission from the panel, if such report is provided by the UE;

- The number of configured SRS ports for the panel, otherwise.

Proposal 6: The lengths of each of the two TPMI fields for SDM transmission are derived based on the maximum number of layers and the number of SRS ports used for the SDM transmission from the corresponding panel.

For sTRP TPMI indication, the two TPMI fields are concatenated as one TPMI field as follows to reduce the DCI payload:

- If the total number of bits of the two TPMI fields LSDM,1 + LSDM,2 is not smaller than the LsTRP bits needed for the sTRP TPMI indication, the first LSDM,1 + LSDM,2 - LsTRP bits of the bit string that carries the two TPMI fields is ignored by the UE (reserved or zero padded);

- If the total number of bits of the two TPMI fields LSDM,1 + LSDM,2 is smaller than the LsTRP bits needed for the sTRP TPMI indication, a reserved field with LsTRP – (LSDM,1 + LSDM,2) bits is further concatenated with the two TPMI fields.

Proposal 7: Panel specific codebook capability reporting should be supported for STxMP.

Proposal 8: To adapt to different pathloss/channel conditions of the two panels, for UCI multiplexing on STxMP PUSCH, study whether or not to support panel specific PUCCH bit rate and/or PUCCH REs ratio configuration.

Proposal 9: For both CB and NCB based STxMP SFN and STxMP SDM transmission schemes, the number of SRS resources in the two SRS resource sets should be the same.

- For CB based transmission, except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', support up to 2 SRS resources in a resource set.

- For NCB based transmission, support up to 4 single-port SRS resources in a resource set.

Proposal 10: For CB based STxMP SFN and STxMP SDM transmission schemes, support SRS resources with up to 4 ports.

- Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2’, the number of ports of the configured SRS resources within a set and across the two sets should be the same.

Proposal 11: For mDCI based STxMP PUSCH+PUSCH, the number of layers of each PUSCH is up to 2 layers.

Proposal 12: For the two scheduled PUSCH in mDCI based STxMP PUSCH+PUSCH, support the same DM-RS configuration with respect to the following parameters:

- the actual number of front-loaded DM-RS symbol(s),

- the actual number of additional DM-RS symbol(s),

- the actual DM-RS symbol location,

- DM-RS configuration type.

Proposal 13: For mDCI based STxMP CG-PUSCH+DG-PUSCH, unless TCI states for CG-PUSCH and DG-PUSCH can be simultaneously used for transmission, one of the PUSCH transmissions should be dropped.

Proposal 14: For mDCI based STxMP CG-PUSCH+DG-PUSCH, dropping rules due to UE Tx power limitation should be considered.

Observations:

Observation 1: The support for a single-layer DFT-s-OFDM SFN PUSCH or a single-layer DFT-s-OFDM per PUSCH for mDCI based STxMP does not have any specification impact. In particular, current TPMI tables applicable to DFT-s-OFDM can be reused for DFT-s-OFDM based SFN and mDCI based STxMP transmission.

Observation 2: In legacy UL transmission, transmitting multiple layers with DFT-s-OFDM may disrupt the single carrier property of DFT-s-OFDM and lead to an increase in PAPR. However, in SDM transmission with the layer combination {1+1}, the single carrier property of DFT-s-OFDM is maintained and, hence, the benefit of DFT-s-OFDM can be fully preserved.

Observation 3: In architectures with shared digital ports, in general, the available SRS ports per panel for SDM transmission is a subset of the available SRS ports for that panel for sTRP transmission.

Observation 4: For the SDM transmission from UEs with shared digital ports architecture, to indicate a suitable TPMI pairs, gNB would require to estimate UL channels based on the subset of SRS ports that are actually available for the SDM transmission at each panel.

Observation 5: Additional RRC configuration for the max number of PTRS ports is not needed for SDM transmission since the actual number of PTRS ports for SDM is 2.

Observation 6: Legacy UE level codebook capability reporting is not applicable to STxMP as the two panels may have different codebook capabilities.

# References

[1] R1-2301821, Summary #3 on Rel-18 STxMP, RAN1#112, February 27-March 3, 2023.

[2] Draft Minutes report RAN1#112, Chairman, February 27-March 3, 2023.

[3] R1-2302370, “Discussion on unified TCI framework extension for multi-TRP ”, RAN1#112bis-e, April 17–26, 2023.