3GPP TSG-RAN WG1 Meeting #112bis-e	 R1-2302376

E-meeting, April 17 – April 26, 2023

Agenda Item:	9.1.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission

Document for:	Discussion and Decision

- Introduction

For 8TX UL operation, the agreement achieved in last meeting can be found in [1].

In this contribution, we discuss SRI and TPMI enhancements to enable 8TX UL transmission.

- Dual CW transmission with up to 8 layers

Working Assumption For uplink transmission with rank>4, support dual CW transmission. Agreement To support dual CW PUSCH transmission for rank>4 by an 8TX UE, for MCS indication, support - Alt.2: A second MCS field (5 bits) is indicated for the second codeword Agreement To support dual CW PUSCH transmission for rank>4 by an 8TX UE, a second set of NDI (1 bit) and RV (2 bits) fields are indicated. - FFS: Details on how to signal Agreement To support dual CW PUSCH transmission for rank>4 by an 8TX UE, reuse DL PDSCH scrambling mechanism to initialize the scrambling sequence generator for codeword q∈{0,1}, cinit=nRNTI∙215+q∙214+nID where nRNTI, and nID are defined similar to the legacy single CW PUSCH transmission. Agreement To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on one of the CWs, down-select from, - Alt1: First CW - Alt2: The CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW)

In RAN1#110bis-e meeting [2], dual CW transmission for larger than 4 layers was agreed as a WA. The spectrum efficiency (SE) performance of 2 CWs and 1CW for UL 8TX with rank>4 is evaluated by link-level simulation, and the results are illustrated in Figure 1, where non-Codebook based PUSCH is assumed that the SRS precoders are derived by singular value decomposition (SVD) of channel matrix. The CW-layer mapping of Rel-15 PDSCH is reused for PUSCH with 8TX. The detailed simulation assumptions can be found in Appendix A.

(a) Rank 5                              (b) Rank 6

(c) Rank 7                              (d) Rank 8

Figure 1 LLS performance of 2CWs vs 1CW for 8TX

It can be observed from the above figures that dual CWs perform better than single CW. Hence, we have the following proposal.

Proposal 1: Confirm the WA that supporting dual CW for uplink transmission with rank>4.

In RAN#112 meeting, some essential enhancements are agreed to enable dual CW transmission, and there are three remaining issues to be discussed.

First, considering CBGTI field, the DL schemes can also be reused. The CBGTI field is of length NTB∙N  bits, where NTB is the number of CWs. If NTB=2, the CBGTI field bits are mapped such that the first set of N  bits starting from the MSB corresponds to the first TB while the second set of N  bits corresponds to the second TB, if scheduled. The first M bits of each set of N  bits in the CBGTI field have an in-order one-to-one mapping with the M CBGs of the TB, with the MSB mapped to CBG#0.

Second, for enabling/disabling the second CW, we propose to reuse DL scheme to save effort on discussion. If PUSCH transmission with up to 8 layers is enabled, the second MCS, NDI and RV field always exist regardless of the rank. One of the two transport blocks is disabled if IMCS = 26 and if rvid = 1 for the corresponding transport block. If both transport blocks are enabled, transport block 1 and 2 are mapped to CW 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first CW.

Finally, to support UCI multiplexing on PUSCH with rank>4 by an 8TX UE, it’s agreed to down-select from the following options:

- Alt1: First CW

- Alt2: The CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW)

We prefer Alt2 to save the number of resource elements for UCI multiplexing. Hence, we have the following proposal.

Proposal 2: For dual CW PUSCH transmission with up to 8 layers by an 8TX UE

- support to reuse Rel-15 NR DL scheme to indicate CBGTI

- support to reuse Rel-15 NR DL enabling/disabling scheme

- support to always multiplex UCI on the CW with the highest MCS.

- SRI/SRS enhancement for enabling 8TX UL transmission

This section discusses the enhancement of SRI for NCB PUSCH.

  - NCB based UL transmission

Agreement For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, Alt1 is supported, that is - Alt1: A single SRS resource set configured with up to 8 single-port SRS resources - FFS: Configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively. Proposal 6.2: For NCB-based 8TX PUSCH transmission with NSRS>4, where NSRS is the number of configured single-port SRS resources in a resource set, - All SRS port combinations are supported - For SRI indication, down-select from, - Option 1: Use an NSRS bit length bitmap - Option 2: Use a legacy-based solution - Consideration of Lmax for SRI indication For NSRS≤4, Rel-15 SRI indication is reused

For NCB based UL transmission, SRI is used to indicate one or more SRS resources from NSRS SRS resources to UE, where UE uses the same precoder of selected SRS resource to transmit PUSCH. For 4TX UE, the UE can only be configured with up to 4 single-port SRS resource for an SRS resource set. For 8TX UE, the number of candidate precoders of 8TX UE is up to 8. In RAN#110bis-e meeting, it was agreed to support a single SRS resource set configured with up to 8 single-port SRS resources for SRS configuration. In addition, configuration of up to two, or four SRS resource sets is FFS. However, we fail to see the benefits to use multiple SRS resource sets. Moreover, there will be several issues if multiple SRS resource sets are used. For example, considering two SRS resource sets each with up to 4 SRS resources, if NSRS=6, there can be multiple possibilities in configuration across two SRS resource sets, such as 3+3, 2+4, or 4+2, which needs to be further discussed. Hence, we think it is not reasonable to support multiple SRS resource sets.

Proposal 3: For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, configuration of up to two, or four SRS resource sets is not supported.

In legacy, k (1≤k≤NSRS) SRS resources can be selected from NSRS SRS resource, thus the SRI overhead is  bits, where Lmax is given by the maximum number of layers. For 4TX UE, is up to 4, and the maximum SRI overhead is 4 bits. For 8TX UE, it is agreed in RAN#112 meeting that all SRS port combinations are supported with NSRS>4. For SRI indication, there are two options to be selected. Option 1 is to use a NSRS bit length bitmap, and option 2 is to use a legacy-based solution. For option 2, the overhead of SRI is  bits, which is up to NSRS bits and related to RRC parameter Lmax. For example, if NSRS=8, the overhead of SRI is 6bits when Lmax=2 and 3bits when Lmax=1. On the contrary, for option 1, the overhead of SRI is always 8 bits no matter what Lmax. To save SRI overhead, we have the following proposal.

Proposal 4: For NCB-based 8TX PUSCH transmission with NSRS>4, a legacy-based SRI indication solution is supported.

To further reduce SRI overhead, the reserved NDI field corresponding to disabled transport block with rank<=4 can be considered. For NCB PUSCH with up to 8 layers, if Rel-15 NR DL enabling/disabling scheme is reused, the disabling/enabling of a TB is indicated to UE through MCS fields and RV fields. Because the 1-bit NDI field corresponding to disabled transport block is not used, it can be considered to use it to reduce DCI overhead.

Proposal 5: Consider to use the reserved NDI field of the disabled transport block for rank<=4 to reduce DCI overhead.

- UL 8TX codebook

In this section, we discuss the codebook design for fully coherent UEs and partially/non-coherent UEs.

  - Fully coherent codebook

Agreement For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook, the following pairs of (N1, N2) values are supported, - (N1, N2) = (4, 1) - (N1, N2) = (2, 2)` A pair of (N1, N2) can be configured with subject to UE capability. Agreement Fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook - Precoding matrices generated according to (O1, O2) = (1, 1) is supported - Further study additional support of precoding matrices generated according to (O1, O2) where O1>1 or O2>1 - Subject to UE capability - FFS: Different O1, O2 values for different ranks Agreement For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), - Study whether/how to support (O1, O2) = (2,1), (2,2) - whether for all rank, or rank 1-2, or rank 3-8 - applicability of different (O1, O2) values per agreed (N1, N2) - companies are encouraged to submit simulation results

    - Larger oversampling ratios (O1>1, O2>1)

In RAN1#111 meeting [3], it is agreed to use Rel-15 DL Type I single-panel codebook as the starting point for design of the fully coherent UL 8TX codebook. In RAN1#112 meeting, (O1, O2) = (1, 1) is supported to generate fully coherent precoding matrices, and additional support of larger oversampling ratios are FFS. System-level simulation is performed to compare the performance with different oversampling ratios. In the simulation, the UE antenna layouts agreed in RAN1#109-e meeting [4] are used, as shown in Table 1. In RAN1#110bis-e meeting, fully-coherent precoders with Ng=1 is agreed. Hence, we just consider the Case 1 with Ng=1 in Table 1.

Table 1 The exemplary cases of UE antenna layout

| Case | Ng | (N1, N2, P) per group | Antenna Layout |
| --- | --- | --- | --- |
| 1 | 1 | (2, 2, 2) (4, 1, 2) |  |
| 2 | 2 | (2, 1, 2) |  |
| 3 | 4 | (1, 1, 2) |  |

For (N1, N2) = (2, 2) and (N1, N2) = (4, 1), we respectively choose (O1, O2) = (2, 2) and (O1, O2) = (2, 1). The detailed simulation assumptions can be found in Appendix B, where the antenna setup and port layouts at TRP is (12,4,2,1,1,4,4).

Figure 2 and 3 show the performance of (O1, O2) = (2, 2) for antenna layout 1-a and (O1, O2) = (2, 1) for antenna layout 1-b, respectively, where rank adaptation with up to 4 layers and 8 layers are considered. It can be seen that larger oversampling ratios in all considered cases have very marginal throughput gain than (O1, O2) = (1, 1). Considering larger oversampling ratios require more indication overhead, we prefer to only support (O1, O2) = (1, 1).

Proposal 6：For fully coherent precoder by an 8TX UE, don’t support larger O1, O2 values.

(a) maxRank=4                                    (b) maxRank=8

Figure 2 Throughput comparison between (O1, O2) = (1, 1) and (O1, O2) = (2, 2) for antenna layout 1-a

(a) maxRank=4                                    (b) maxRank=8

Figure 3 Throughput comparison between (O1, O2) = (1, 1) and (O1, O2) = (2, 1) for antenna layout 1-b

    - Phase alignment error between antennas

The Rel-15 DL type I single-panel codebook is optimized for highly correlated dual-polarized antennas, which is based on DFT matrix. It is agreed in RAN1 #109-e that 8 TX antenna layout follows the regular structure of ULA or UPA. For ULA structure, Rel-15 DL type I single-panel codebook requires that there is a linear phase ramp across the 4 antenna ports in one polarization. For UPA structure, Rel-15 DL type I single-panel codebook requires that the phase difference between the former 2 antenna ports is equal to that between the latter 2 antenna ports. However, as discussed in RAN1#110bis-e meeting, such requirement is typically unsatisfied in practice, where each of the 4 antenna ports in one polarization has a random phase offset. According to the simulation results reported by most companies in RAN1#111 meeting, such random phase offset leads to large performance loss when using Rel-15 DL type I single-panel codebook. Hence, Rel-15 DL type I single-panel codebook needs to be enhancement to compensate for the impairment of random phase offset.

A straightforward method to solve this issue is phase pre-compensation. Specifically, for an 8TX codeword w=[w1,w2,w3,w4,φw1,φw2,φw3,φw4]Tselected from Rel-15 DL type I single-panel codebook, we can introduce three phase pre-compensation coefficients σ1, σ2 and σ3 to constitute a new codeword as

w'=[w1,σ1w2,σ2w3,σ3w4,φw1,φσ1w2,φσ2w3,φσ3w4]T

where σ1, σ2 and σ3 are respectively used to pre-compensate the phase offset between 2nd, 3rd, 4th antenna ports and 1st antenna port. Typically,

σ1=exp(jθ1),   θ1=0:2πN1:(N1-1)2πN1

σ2=exp(jθ2),   θ2=0:2πN2:(N2-1)2πN2

σ3=exp(jθ3),   θ1=0:2πN3:(N3-1)2πN3

where N1, N2 and N3 are used to control the phase resolution.

In following evaluation, we denote the UL 8TX fully coherent codebook based on Rel-15 DL type I single-panel codebook as “DL type I”, and that based on enhanced Rel-15 DL type I single-panel codebook as “DL type I with phase error compensation”.

System-level simulation is performed to compare the performance of two candidate codebooks for UL 8TX. For “DL type I” and “DL type I with phase error compensation”, we adopt O1=1，O2=1, codebookMode=1 for antenna layout 1-a and 1-b. For “DL type I with phase error compensation”, the number of phase pre-compensation coefficients are N1=N2=N3=4. The detailed simulation assumptions can be found in Appendix B, where the antenna setup and port layouts at TRP is (12,4,2,1,1,4,4). To explicitly analyze the effect of phase alignment error, we consider each UE is rank-one.

Figure 4 and 5 show the throughput comparison between “DL type I” and “DL type I with phase error compensation” with random phase offset for antenna layout 1-a and 1-b, respectively. Here, the phase offset values of each antenna port is assumed uniformly distributed over [-μ,μ], where μ takes 0, 45, 90, 135 and 180 degrees. In Figure 4 and 5, we take “DL type I” with φ=0 as the baseline. It can be observed that in all considered cases, the throughput of two codebooks decrease with the increase of φ. However, “DL type I with phase error compensation” achieves larger throughput than “DL type I”, especially for large φ. For “DL type I” codebook, there is 9.28%~15.44% throughput loss with μ=135 and 12.09%~18.44% throughput loss with μ=180. With the help of phase pre-compensation, the throughput loss of “DL type I with phase error compensation” codebook with μ=180 reduces to 1.81%~4.75%. Table 2 summarizes the throughput gains of “DL type I with phase error compensation” codebook than “DL type I” codebook.

(a) P0 = -80 dBm                                    (b) P0 = -50 dBm

Figure 4 Throughput comparison between “DL type I” and “DL type I with phase error compensation” with random phase offset for antenna layout 1-a

(a) P0 = -80 dBm                                   (b) P0 = -50 dBm

Figure 5 Throughput comparison between “DL type I” and “DL type I with phase error compensation” with random phase offset for antenna layout 1-b

Table 2 Throughput gains of “DL type I with phase error compensation” codebook than “DL type I” codebook

| Phase error | μ=0 | μ=45 | μ=90 | μ=135 | μ=180 |
| --- | --- | --- | --- | --- | --- |
| 1-a with P0 = -80 dBm | 102.60% | 103.99% | 108.05% | 111.50% | 112.51% |
| 1-a with P0 = -50 dBm | 103.63% | 105.20% | 111.50% | 118.99% | 119.73% |
| 1-b with P0 = -80 dBm | 102.21% | 103.17% | 106.48% | 108.43% | 108.84% |
| 1-b with P0 = -50 dBm | 102.97% | 103.92% | 108.35% | 110.62% | 111.78% |

Observation 1: For fully coherent 8TX codewords based on “DL type I” codebook, there will be large throughput loss due to phase alignment error, such as 9.28%~15.44% throughput loss with μ=135 and 12.09%~18.44% throughput loss with μ=180.

Observation 2: For fully coherent 8TX codewords, the throughput gains of “DL type I with phase error compensation” than that of “DL type I” are 2.21%~3.63% for μ=0, 3.17%~5.20% for μ=45, 6.48%~11.50% for μ=90, 8.43%~18.99% for μ=135 and 8.84%~19.73% for μ=180.

Since fully coherent 8TX codewords based on DL type I codebook has large throughput loss with phase alignment error, and such throughput loss can be compensated by using enhanced DL type I codebook with phase pre-compensation. Hence, we have the following proposal.

Proposal 7：For fully coherent precoder by an 8TX UE, support to enhance DL Type I SP codebook to mitigate the impact of phase alignment error.

  - Partially/non-coherent codebook

Agreement For non-coherent uplink precoding by an 8TX UE, following precoders are supported for 1 layer transmission. 10000000,01000000,00100000,00010000,00001000,00000100,00000010,00000001 with the scaling factor of 122. Proposal 3.1b: For partially coherent uplink precoding by an 8TX UE codebook, - When Ng=2 - Precoding design is based on Rel-15 UL 4TX codebook, - Full-coherent precoders are used - FFS whether partial-coherent precoders are needed - When Ng=4, down-select from, - Alt1: - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Alt2: - Precoding design is based on Rel-15 UL 4TX codebook, - Partial-coherent precoders are used

For partially/non-coherent precoders, it is agreed in RAN1#110bis-e meeting to support NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point.

    - Partially coherent precoders with Ng=2

In RAN1#112 meeting, it is agreed to design partial coherent precoders with Ng=2 based on Rel-15 UL 4TX codebook. A typical precoding structure can be shown as follows:

- For rank = 1

  - A04×18×1 or  04×1A8×1 where A is a rank-1 fully coherent precoding matrix taken from Rel-15 UL 4TX codebook

- For rank = 2, 3, 4

  - A04×rank8×rank or 04×rankA8×rankor A104×rank(A2)04×rank(A1)A28×rank , where A, A1 and A2 are fully coherent precoding matrices taken from Rel-15 UL 4TX codebook, and rank= rank(A) or rank(A1) + rank(A2) is one of 2, 3 or 4

- For rank >4,

  - A104×rank(A2)04×rank(A1)A28×rank, where A1 and A2 are fully coherent precoding matrices taken from Rel-15 UL 4TX codebook, the rank=rank(A1) + rank(A2) is one of 5, 6, 7 or 8,

where only fully coherent Rel-15 UL 4TX precoders are considered to save overhead.

Furthermore, the layer splitting schemes for each rank needs to be discussed. The following FL proposal agreed in RAN1#112 meeting list all possible layer splitting schemes for each rank.

Proposal 3.3: For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, - Following rank and layer splitting cases are supported Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 1; (1,0), (0,1). 8; (4,4). - Select from the following cases based on the performance and overall DCI overhead Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (2,1), (1,2). 4; (4,0), (0,4). 4; (2,2), (3,1), (1,3). 5; (4,1), (1,4), (2,3), (3,2). 6; (4,2), (2,4), (3,3). 7; (4,3), (3,4). Note: Above is not relevant to how precoders are indicated.

To accommodate various UE layouts and dynamically varied channel environments, we prefer to support all possible layer splitting schemes for each rank. From the perspective of indication, we prefer a simple scheme with a TPMI field composing of two 5-bit sub-fields, where each sub-field is to indicate one precoder. Such scheme can support all possible layer splitting schemes for each rank.

Proposal 8：For partially coherent precoder with Ng=2 by an 8TX UE, support all possible layer splitting schemes for each rank.

    - Partially coherent precoders with Ng=4

For partially coherent precoders with Ng=4, we can also adopt the above precoding structures for Ng=2 with slight change to save spec effort, where A, A1 and A2 are partially coherent precoding matrices taken from Rel-15 UL 4TX codebook.

A second precoding structure based on Rel-15 UL 2TX codebook was also discussed, shown as below

A102×rankA202×rankA1A202×rankA302×rankA402×rankA302×rankA402×rankA102×rankA202×rankA102×rankA2A302×rankA402×rankA3A4

where rank=rank(A1) + rank(A2) + rank(A3) + rank(A4) is one of 1~ 8, rank(A1), rank(A2), rank(A3) and rank(A4) can be one of 0~2, and A1, A2, A3, and A4 are empty matrices or fully coherent precoding matrices taken from Rel-15 UL 2TX codebook.

Table 3 lists the number of codewords for partially coherent precoders with Ng=4 based on “UL 4TX” and “UL 2TX”. It can be observed that the number of codewords of “UL 2TX” is almost 6 times than that of “UL 4TX”, which leads to 12-bit indication overhead. Intuitively, it is not necessary to use so much overhead for partially-coherent precoders with Ng=4, where only two antenna ports can be performed with coherent transmission.

Table 3 Number of codewords

| Rank | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Total number of codewords |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| “UL 4TX” | 16 | 80 | 132 | 100 | 64 | 36 | 8 | 4 | 440 |
| “UL 2TX” | 16 | 104 | 352 | 664 | 704 | 416 | 128 | 16 | 2400 |

Observation 3: For partial coherent 8TX precoders with Ng=4, the codebook based on Rel-15 UL 2TX requires 3 more indication bits than that based on Rel-15 UL 4TX.

System-level simulation is performed to compare the performance of two candidate precoding structures for partially-coherent precoders with Ng=4. Here, we consider the Case 3 with Ng=4 in Table 1. Within each group, antenna elements are spaced by 0.5λ. Both the horizontal and vertical spacings (dG-H, dG-V) between the centers of adjacent antenna groups are 2λ. The detailed simulation assumptions can be found in Appendix B, where the antenna setup and port layouts at TRP is (12,4,2,1,1,4,4). It can be seen from Figure 6 that “UL 2TX” has very marginal throughput gain than “UL 4TX”.

(a) antenna layout 3-a                          (b) antenna layout 3-b

Figure 6 Throughput comparison between “UL 4TX” and “UL 2TX” for Ng=4

It can be observed that:

Observation 4: For partial coherent 8TX precoders with Ng=4, the codebook based on Rel-15 UL 2TX has very marginal throughput gain than that based on Rel-15 UL 4TX, about 0.51%~1.69%.

Considering both performance and overhead, we have the following proposal:

Proposal 9：For partially coherent precoder with Ng=4 by an 8TX UE, support to design precoder based on Rel-15 UL 4TX codebook.

    - Non-coherent precodes

For non-coherent precoders, we can also adopt the above precoding structures for Ng=2 and Ng=4 with slight change to save spec effort, where A, A1 and A2 are non-coherent precoding matrices taken from Rel-15 UL 4TX codebook.

Based on the above analysis, we can design a general precoding structure for partially coherent precoders with Ng=2/Ng=4 and non-coherent precoders, and we have the following proposal.

Proposal 10：For partially/non-coherent precoder by an 8TX UE, support the following precoding structures

- For rank = 1

  - A04×18×1 or  04×1A8×1

- For rank = 2, 3, 4

  - A04×rank8×rank or 04×rankA8×rankor A104×rank(A2)04×rank(A1)A28×rank

- For rank >4,

  - A104×rank(A2)04×rank(A1)A28×rank

where A, A1 and A2 are fully coherent or partially coherent or non-coherent precoding matrices taken from Rel-15 UL 4TX codebook for partial coherent precoders with Ng=2 or partial coherent precoders Ng=4 or non-coherent precoders.

- UL precoder indication enhancement

In this section, we discuss the precoder indication.

  - Discussion on field size of DCI precoder indication field

In the existing NR specification, TPMI and TRI are jointly encoded to indicate the selected codeword and rank to UE, which is indicated by the “Precoding information and number of layers” field in DCI. For 4TX UE in Rel-15, the UL transmission layer per UE is up to 4. When the UL transmission layer for 8TX UE is increased to 8, the indication of TPMI and TRI will require a larger field than the legacy. In the existing NR specification, the maximum overhead of “Precoding information and number of layers” field is 6 bits. If UL 8TX codebook is designed based on Rel-15 UL 4TX codebook and reuse the legacy TPMI and TRI indication rule, the maximum size of “Precoding information and number of layers” field may reach up to 12 bits. If UL 8TX codebook is designed based on Rel-15 DL type I codebook with antenna selection vector and reuse the legacy PMI indication rule, the maximum indication overhead may exceed 12 bits.

Observation 5: To support 8TX CB based UL transmission, the field size of TPMI indication field is increased by at least twice than legacy.

Although the indication overhead can be reduced by reducing the number of codewords, such as lower oversampling ratios for DL type 1 and fixed rank for UL 4TX, the performance will be degraded.

  - Discussion on high-resolution UL precoder

To reduce the DCI payload, the existing Rel-15 UL 2TX/4TX codebook and Rel-15 DL Type I codebook, as well as the above-mentioned UL 8TX codebook have limited number of codewords, resulting in a low-resolution precoder. 8TX introduces more spatial degree of freedom compared to 2TX and 4TX, which can improve the UL throughput. To maximize the throughput improvement introduced by 8TX, high-resolution precoder should be considered.

Figure 7 and 8 show UL system-level simulation results for 8TX UE with up to 8 layers. The detailed simulation assumptions can be found in Appendix B, where the antenna setup and port layout at UE is (2,2,2,1,1,2,2). Network layout is 1*3 cell, 10 UE per cell. Here we consider DL type 1 single-panel codebook with as a basis with O1=4，O2=4 and codebookMode=1. The label of EigenVector denotes that the precoder of each UE is from the eigenvectors of SVD decomposition of the channel matrix, which represents the upper bound of high-resolution precoder. It can be observed that “EigenVector” precoder can obtain 20~33% throughput gain compared with codebooks of “DL type I”.

(a) 64QAM                                           (b) 256QAM

Figure 7 Throughput for DL type I and EigenVector precoder at 32R

(a) 64QAM                                           (b) 256QAM

Figure 8 Throughput for DL type I and EigenVector precoder at 64R

Observation 6: High-resolution precoder such as eigenvector precoder can obtain 20~33% throughput gain compared with that based on DL type I for UL 8TX.

  - Beamformed CSI-RS based UL precoder indication

As we have discussed above, the field size of DCI indication field for indicating 8TX codebook is increased by at least twice than legacy, that is at least 12 bits, which is a large overhead. In addition, high-resolution precoder will lead to unacceptable indication overhead with the legacy DCI indication method. Hence, it is essential to consider new UL precoder indication method to resolve the above issue.

Beamformed CSI-RS can resolve the issues of large DCI overhead, where CSI-RS is used to indicate the selected precoder by TRP to UE, instead of traditional DCI. Specifically, TRP transmits a beamformed CSI-RS with precoder WDL. Then after the TRP-UE channel H, UE can receive the CSI-RS as a vector PUL with dimension nRxx1 , where nRx is the number of receive antenna ports at UE. Then the received vector can be used as a UL precoder for UE.

PUL=HWDL

By choosing the precoder WDL of CSI-RS, the selected precoder PUL can be indicated to UE with high precision. Because the CSI-RS overhead is not related to the number of codewords, we can design a high-resolution codebook with large codebook size. Even TRP can directly indicate a high-resolution precoder (such as “EigenVector” precoder) to UE without codebook. Note that the considered beamformed CSI-RS based UL precoder indication requires reciprocity between UL and DL channels, which can be used for TDD networks.

To indicate the precoder of one layer, the above beamformed CSI-RS based UL precoder indication requires 1 CSI-RS port. For 8TX UL transmission with up to 8 layers, 8 CSI-RS ports are required, which occupies 8 resource elements. If we use the 8 resource elements to transmit DCI in PDCCH with QPSK modulation, the number of bits to be transmitted is typically 8*2*0.2=3.2, where 0.2 is a typical value of code rate in PDCCH. Compared to the DCI indication overhead in the above-mentioned UL 8TX codebook, the overhead of beamformed CSI-RS based UL precoder indication is greatly reduced.

With the beamformed CSI-RS, the overhead can be reduced and the resolution of indication can be increased. Therefore, we have the following proposal.

Proposal 11: The beamformed CSI-RS should be considered to indicate UL precoders to UE.

- Conclusions

The contribution discusses the SRI, TPMI (including codebook) enhancement for enabling 8TX UL transmission, based on which the following observations and proposals are made.

Observation 1: For fully coherent 8TX codewords based on “DL type I” codebook, there will be large throughput loss with phase alignment error, such as 9.28%~15.44% throughput loss with μ=135 and 12.09%~18.44% throughput loss with μ=180.

Observation 2: For fully coherent 8TX codewords, the throughput gains of “DL type I with phase error compensation” than that of “DL type I” are 2.21%~3.63% for μ=0, 3.17%~5.20% for μ=45, 6.48%~11.50% for μ=90, 8.43%~18.99% for μ=135 and 8.84%~19.73% for μ=180.

Observation 3: For partial coherent 8TX precoders with Ng=4, the codebook based on Rel-15 UL 2TX requires 3 more indication bits than that based on Rel-15 UL 4TX.

Observation 4: For partial coherent 8TX precoders with Ng=4, the codebook based on Rel-15 UL 2TX has very marginal throughput gain than that based on Rel-15 UL 4TX, about 0.51%~1.69%.

Observation 5: To support 8TX CB based UL transmission, the field size of TPMI indication field is increased by at least twice than legacy.

Observation 6: High-resolution precoder such as eigenvector precoder can obtain 20~33% throughput gain compared with that based on DL type I for UL 8TX.

Proposal 1: Confirm the WA that supporting dual CW for uplink transmission with rank>4.

Proposal 2: For dual CW PUSCH transmission with up to 8 layers by an 8TX UE

- support to reuse Rel-15 NR DL scheme to indicate CBGTI

- support to reuse Rel-15 NR DL enabling/disabling scheme

- support to always multiplex UCI on the CW with the highest MCS.

Proposal 3: For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, configuration of up to two, or four SRS resource sets is not supported.

Proposal 4: For NCB-based 8TX PUSCH transmission with NSRS>4, a legacy-based SRI indication solution is supported.

Proposal 5: Consider to use the reserved NDI field of the disabled transport block for rank<=4 to reduce DCI overhead.

Proposal 6：For fully coherent precoder by an 8TX UE, don’t support larger O1, O2 values.

Proposal 7：For fully coherent precoder by an 8TX UE, support to enhance DL Type I SP codebook to mitigate the impact of phase alignment error.

Proposal 8：For partially coherent precoder with Ng=2 by an 8TX UE, support all possible layer splitting schemes for each rank.

Proposal 9：For partially coherent precoder with Ng=4 by an 8TX UE, support to design precoder based on Rel-15 UL 4TX codebook.

Proposal 10：For partially/non-coherent precoder by an 8TX UE, support the following precoding structures

- For rank = 1

  - A04×18×1 or  04×1A8×1

- For rank = 2, 3, 4

  - A04×rank8×rank or 04×rankA8×rankor A104×rank(A2)04×rank(A1)A28×rank

- For rank >4,

  - A104×rank(A2)04×rank(A1)A28×rank

where A, A1 and A2 are fully coherent or partially coherent or non-coherent precoding matrices taken from Rel-15 UL 4TX codebook for partial coherent precoders with Ng=2 or partial coherent precoders Ng=4 or non-coherent precoders.

Proposal 11: The beamformed CSI-RS should be considered to indicate UL precoders to UE.

# References

- Chairman’s Notes RAN1#112, February 27th – March 3rd, 2023.

- Chairman’s Notes RAN1#110bis-e, October 10th – 19th, 2022.

- Chairman’s Notes RAN1#111, November 14th – 18th August 22th – 26th, 2022.

- Chairman’s Notes RAN1#109-e, May 9th – 20th, 2022.

# Appendix A

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5GHz |
| Waveform | CP-OFDM |
| Bandwidth | 20MHz |
| SCS | 30kHz |
| BS antenna configuration | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 |
| UE antenna configuration | (1,4,2,1,1,1,4) with (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-B with 100ns delay spread |
| UE speed | 3Km/h |
| Modulation | Adaptive MCS with up to 64QAM |
| Layer mapping | Fixed Rank Rank 5: 1st CW@RANK1-2, 2nd CW@RANK3-5 Rank 6: 1st CW@RANK1-3, 2nd CW@RANK4-6 Rank 7: 1st CW@RANK1-3, 2nd CW@RANK4-7 Rank 8: 1st CW@RANK1-4, 2nd CW@RANK4-8 |
| DMRS configuration | Type 1 double symbol DMRS |

# Appendix B

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Carrier Frequency | 3.5G |
| Channel Model | According to the TR 38.901 |
| Scenario | Uma with 500 m ISD |
| Antenna setup and port layouts at TRP | (12,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (12,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 |
| Antenna setup and port layouts at UE | (2,2,2,1,1,2,2) , (1,2,2,2,1,1,2) , (1,1,2,2,2,1,1) (1,4,2,1,1,1,4) , (1,2,2,1,2,1,2) , (1,1,2,1,4,1,1) (𝑑H, 𝑑V) = (0.5, 0.5)𝜆 (𝑑G-H, 𝑑G-V) = (2, 2)𝜆 |
| TRP antenna height | 30m |
| TRP receiver noise figure | 5dB |
| UE Tx power | 23 dBm |
| UE antenna height | 3m |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS |
| Modulation | up to 256QAM |
| Bandwidth | 20 MHz |
| MIMO scheme | SU-MIMO |
| Rank | Rank adaptation with up to 8 layers |
| Traffic model | Full buffer |
| UE distribution | 100% outdoor (3km/h) |
| BS receiver | MMSE-IRC |
| Network Layout | 1*3 cell, 30 UE per cell |
| Precoder granularity | Wideband |
| Scheduling granularity | 4RB |
| Power control | Open loop; alpha = 0.8; P0 = -50, -80 dBm |