3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2302378

E-meeting, 17-26 April, 2023

Agenda Item:	9.5.1.2

Source:	Huawei, HiSilicon

Title:	Discussion on SL positioning measurement and reporting

Document for:	Discussion and Decision

# Introduction

In RAN1#112 [1], we have discussed the measurements and reporting for SL RTT-type, SL-AoA, SL-TDOA positioning methods. In this paper, we further provide our views on SL positioning measurement and reporting.

# RTT-type methods using SL

## Measurements

Rx-Tx time difference has been adopted in the Uu interface for supporting multi-RTT positioning. It is natural to support Rx-Tx time difference for single-sided RTT. Moreover, double-sided RTT can be implemented based on single-sided RTT, which can also use the Rx-Tx time difference measurement.

In RAN1#112 [1], definition of SL-PRS based Rx-Tx time difference measurement has been discussed and we have the following agreement.

- Agreement For definition of SL-PRS based Rx-Tx measurement, downselect one of the following alternatives in RAN1# 112b to minimize the impact of UE reference timing offset and mobility - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt2: SL-PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu.

We think that Alt1 and Alt3 are the same in most cases. For the cases when they are different, which could be caused by synchronization change, etc. Alt.1 actually requires UE to track the timing change between SL-PRS Rx and SL-PRS Tx, which is not friendly to UE implementation. Besides, Alt1 may increase the positioning latency since the Rx-Tx measurement is always reported after an actual SL-PRS transmission. Therefore, we prefer Alt3.

In Uu positioning, no performance requirement was defined when there is TA change, we do not think any performance requirement would apply as well for the synchronization change. Hence, there is no need to redefine the Rx-Tx measurement using Alt1.

Proposal 1: For definition of SL-PRS based Rx-Tx measurement, support

Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu.

## Single-sided RTT and double-sided RTT

In RAN1#112 [1], double sided RTT without sidelink PRS transmission order restriction has been supported.

Agreement For SL-PRS based Rx-Tx measurement for double sided RTT, consider sidelink PRS transmission without order restriction between multiple rounds of PRS transmission of involved UEs. - FFS on how to differentiate different PRS transmissions for sidelink PRS Rx-Tx measurement and report - FFS on impact of Scheme 2 resource allocation when the different orders in double sided RTT is considered and whether and how to minimize number of different orders - Aspects related to scheme 2 resource allocation are to be discussed in agenda 9.5.1.3

For the first FFS point, we think that the measurement of Rx – Tx time difference should anyway be based on the Rx timing of SL-PRS with no connection to the SL-PRS Tx timing, i.e., following the definition of UE Rx – Tx time difference defined in Uu as discussed in section 2.1 of the paper. The timestamp of the measurement should be sufficient to identify the SL-PRS resources used to generate the corresponding measurement.

For the second FFS point, since the transmission of SL-PRS in RTT-type methods using SL is very flexible for both SS-RTT and DS-RTT, the reception UE may not have any knowledge of positioning methods and the overall transmission orders. An inter-UE SL-PRS transmission request can be used to indicate whether future backward SL-PRS transmission is desired. We noticed that this has already been covered by the following discussion in resource allocation.

Agreement In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE

- Proposal 2: With regards to flexible transmission orders of SL-PRS for RTT-type sidelink positioning

The time stamp in the measurement report with the existing UE Rx – Tx time difference measurement is sufficient to differentiate different SL-PRS transmissions.

An inter-UE SL-PRS transmission request, which can be based on either higher layer signaling or lower layer signaling, should be introduced.

## Report-free RTT

The positioning latency and signaling overhead can be reduced if Rx-Tx time difference is not reported in SL-RTT. To support such report-free RTT, the reply time should be known in advance. To achieve this goal, the transmission time of SL-PRS can be adjusted based on the actual time of reception of another SL-PRS, so that the reply time can be a fixed value. For example, in Figure 1, UE1 transmits SL-PRS1 to UE2 at time t1, UE2 would transmit SL-PRS2 to UE1 at time t2 according SL-PRS resource allocation procedure. When UE2 obtains the exact time of reception t3 after measuring SL-PRS1, UE2 can adjust the actual transmission time of SL-PRS2 to t4 = t2+ (t3-t1). Hence, the reply time of UE2 is t4-t3 = t2-t1. Since t1 and t2 are known at UE1, UE1 can calculate the time of flight without measurement report from UE2. The timing adjustment is t3-t1, which is usually 10~100ns level and much shorter than cyclic prefix. Hence, the introduced interference caused by the timing adjustment is negligible.

t3-t1

t3

UE1

UE2

t1

t4

t5

t2

t3-t1

Figure 1: Example of report free RTT method

Proposal 3: Support report-free RTT based on adjusting the transmission time of SL-PRS.

# SL-TDOA

## General procedures

In RAN1#112 [1], we have discussed synchronization problem for SL-TDOA, and proposed different alternatives to mitigate impact of synchronization errors

FL proposal 3.2.3-v1: Study whether and how to mitigate impact of synchronization errors between anchor UEs for TDoA measurement, considering the following alternatives: - Alt1: indication/reporting of e.g., synchronization source type, synchronization error or quality of synchronization information - Alt2: SL-PRS transmission between anchor UEs - Alt3: (pre-)configuration and capability report of the clock source’s accuracy requirement/threshold. If a UE does not satisfys the requirement, it does not participate in the positioning. - Others are not precluded

Overall, there are two ways to solve the synchronization problem, considered in Alt1 and Alt2, respectively.

One way is to check whether a UE’s synchronization performance satisfy the positioning requirement when choosing a UE to participate in SL-TDOA positioning. The key to achieve this function is to acquire the assistant information of the UE’s synchronization performance. In sidelink communication, multiple synchronization source types can be used for different UEs such as: GNSS, gNB, UE (UE synchronized to GNSS, UE synchronized to gNB, standalone UE with its own local timing). Different synchronization sources will have different transmit timing, and even for the same type synchronization source, like gNB, different PLMN will have different timing setting. For sidelink positioning, the same timing is required. When a UE report the synchronization source type information to the target UE/LMF, the target UE/LMF can assess the range of the synchronization error and decide whether this UE can participate in the positioning.

- Observation 1: Indication/reporting of synchronization source type helps to choose proper anchor UEs to participate in SL-TDOA positioning.

Another way to solve the synchronization problem is to reduce the influence of synchronization error. Since there could be some cases that there are not sufficient UEs whose synchronization performance satisfy the positioning requirement, the target UE/LMF can relax the synchronization requirement and introduce SL-PRS transmission between the anchor UEs to reduce the influence of synchronization error. For example, in Figure 2, both the target UE P and anchor UE B measure the SL-PRS transmitted from anchor UE A. Meanwhile, both the target UE P and anchor UE A measure the SL-PRS transmitted from anchor UE B. The whole procedure can be seen as a DL-TDOA-like method combined with an additional RTT method between anchor UEs. By introducing the SL Rx-Tx time difference measurement from the anchor UEs (t1,t2 in Figure 2), synchronization problems between the anchor UEs are solved. Since the above procedure only requires additional RTT between anchor UEs, which can reuse the normal SL-RTT positioning procedure, there is no additional RAN1 spec impact so far.

- Observation 2: SL-PRS transmission between anchor UEs, which may reuse the SL-RTT positioning procedure, helps to reduce the influence of synchronization error.

t3

Anchor UE A

Target UE P

t1

t2

Anchor UE B

Figure 2 TDOA combined with RTT

Based on the above analyze, we think the Alt1 and Alt2 proposed in last meeting can be supported to solve the synchronization problem.

Proposal 4: For alternatives to mitigate impact of synchronization errors, support Alt1 and Alt2.

For Alt2, send an LS to RAN2 to inform them that SL positioning measurement procedure can also be applicable between anchor UEs.

## Measurements

Reference signal time difference (RSTD) measurement is specified for DL-TDOA, which requires a reference Transmission Point (TP), while Relative Time of Arrival (RTOA) is specified for UL-TDOA, which requires a reference time determined from SFN initialization time. We think that SL-RTOA can be used for both DL-TDOA like and UL-TDOA like SL-TDOA positioning methods, but RSTD is not suited for both since anchor UEs cannot perform differential TOA with respect to another anchor UE. Hence, we support at least SL-RTOA measurement for SL-TDOA positioning.

- Proposal 5: Support at least SL-RTOA measurement for SL-TDOA positioning.

In RAN1#112 [1], SL-RTOA measurement has been discussed and we have the following agreement.

Agreement SL-PRS based RTOA TSL-RTOA is defined as the beginning time of SL subframe #i containing SL-PRS received from a UE, relative to the RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where - T0 is the nominal beginning time of SFN 0 or DFN0. - FFS on how to select between SFN 0 or DFN 0 for determination of T0. - FFS: the source for the reference timing - tSL-PRS = (10nf + nsf) x 10-3tSRS=10nf+nsf×10-3, where nf and nsfnsf are the SFN or DFN and the subframe number of the SL-PRS, respectively - FFS on how to select between SFN or DFN

- For UL-like SL-TDOA, the most typical cases would be that all anchor UEs are either synchronized to GNSS (using DFN) or synchronized to gNB (using SFN), and we do not think there should be a deployment with mixture of synchronization sources. Within a single UE, it should simply use its own configuration, and thus we do not think that the selection between SFN0 and DFN0 will ever happen. From the measurement reporting perspective, indication of whether the RTOA is based on SFN or DFN should be sufficient.

- Proposal 6: Support the indication of whether SFN or DFN is used along with the RTOA measurement reporting.

# SL-AoA

## Measurements

In RAN1#112 [1], SL-AoA measurements have been discussed and we have the following agreement.

Agreement Support both GCS and LCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement. - FFS on the applicable scenario/service for AoA/ZoA relative to LCS without translation of the LCS to GCS

With regards to AoA/ZoA in LCS, it is worth noting that the relative AoA/ZoA is not reciprocal between a pair of UEs, as shown in Figure 3 due to different orientations, unlike that for AoA in GCS where

θZOAA→B=π-θZOAB→A
θAOAA→B=π+θAOAB→A

We think that there should be use cases that the AoA measurement only in LCS is of interest in the ranging service. For example, when a cellphone user wants to find the location of another device, it is sufficient if the direction is defined relative to the reference cellphone, while having the direction expressed in the global way (e.g. defined with respect to geographical north) actually make it more complicated by further requiring the cellphone to use its own orientation. Hence, AoA/ZoA relative to LCS without translation of the LCS to GCS should be supported.

LCS/relative direction (ϕ')

GCS/absolute direction (ϕ)

Target UE

Anchor UE

SL-PRS

LCS/relative direction

Figure 3 Relative direction for two UEs

Proposal 7: Support AoA/ZoA relative to LCS without translation of the LCS to GCS.

# Multi-panel based SL positioning

With positioning of a UE using radio, the positioning is actually of the UE’s antenna reference point (ARP). However there also exists distributed antenna system (DAS) deployment for vehicular UEs, e.g., UEs with multiple panels, where the antenna or panel displacement may not be neglected for positioning.

For TDOA and RTT based positioning, each distributed antenna provides an ARP for transmission and/or reception of SL-PRS to better locate a target UE’s antenna.

For AoA or AoD positioning, each distributed group of antennas or panel provides an angle estimate towards a target UE’s antenna.

There was discussion in RAN1#112 on support ARP specific measurement reporting. There was concerns on overcomplicating the UE location information transferred over the Uu/PC5 interface (e.g. each UE may be represented by multiple ARP locations in 3GPP). Introducing ARP specific measurement reporting in Rel-18 should only target distinction of transmission/reception ARPs, without actually revealing the location information of different ARPs. In other words, having an ARP ID without actual ARP location reporting (similar to the concept of TEG ID without actual group delay value) associated with transmission/measurement could be considered.

- For RTT-based positioning, the Rx-Tx time difference measurements at both sides need to be associated with the same pair of ARPs to make the measurement meaningful. For some cases, in the Rx-Tx time difference measurement, the Rx antenna/panel used for a SL-PRS reception may not be the same as the Tx antenna/panel used for a SL-PRS transmission as shown in Figure 4. For RTT-based positioning, it would be beneficial to know whether for a Rx-Tx time difference, the Rx antenna/panel and the Tx antenna/panel are the same, i.e., to avoid an erroneous positioning estimation.

UE 2

UE 1

#1

#1

#2

SL PRS 1

SL PRS 2

Figure 4: Transmission and reception of SL-PRS with different panels

- Proposal 8: Support per ARP/panel based measurement/transmission in sidelink positioning, where an indication is used to identify whether

Two measurements are from the same ARP

Two transmissions are from the same ARP

Rx time and Tx time are from the same ARP for UE Rx – Tx time difference

# Joint Uu and SL positioning

For joint Uu and SL positioning, we prefer not to specify any new measurements. In most useful cases, Uu and SL are in different bands, the RSTD between Uu DL and SL will anyway be measurement in different occasions, and synchronization error/group delay difference between Uu DL and SL may also be present.

- Proposal 9: Not to specify any new measurements for joint Uu and SL positioning.

# Reporting of SL positioning measurements

- Regarding the positioning relevant information, such as capability, assistance data, and measurement results, it should be managed as a regular data service following the existing PDCP/RLC/MAC assembly procedure and multiplexed with other communication data defined in PC5 interface.

- Proposal 10: SL positioning measurement shall be treated as regular communication data, and be transparent to physical layer and transmitted over the PC5 communication resources.

- In addition to the measurement, the measurement report should contain identification information, timestamp, quality metric, etc. The identification information mainly composes:

Identification information for UE.

  - For reporting to UE, if the report is between the two UEs which perform SL PRS transmission and measurement, the identification information for UE may not be needed. If the measurement is reported to a third-party UE (e.g., the server UE), the ID of the UE transmitting SL-PRS may be needed. However, it is not clear whether a mechanism such as LCS correlation ID can be used for this case.

  - For reporting to LMF, a core network (CN) UE ID recognizable by LMF may be required to assist the LMF to identify the transmitting UE.

  - We do not think the UE ID receiving the SL-PRS is needed (a.k.a. destination ID), since the measurement report is only expected from the UE receiving the SL-PRS and performing the measurement.

Identification information for SL-PRS.  If multiple SL-PRS resources are configured for a UE, the SL-PRS resource ID should be reported to distinguish different SL-PRS resource.

- The content of measurement report is summarized in Table 1.

Table 1 Content of measurement report

|  | reporting to LMF | reporting to UE |
| --- | --- | --- |
| SL-PRS based Rx-Tx measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based RSTD measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based RSRP measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based RSRPP measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based RTOA measurement | CN UE ID transmitting SL-PRS SL PRS resource ID SFN or DFN indication Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID SFN or DFN indication Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator GCS/LCS indication and translation | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator GCS/LCS indication and translation |

Proposal 11: No need to provide ID for the UE receiving SL-PRS (destination ID) to either LMF or UE.

Proposal 12: Adopt the following table for the content of measurement report.

|  | reporting to LMF | reporting to UE |
| --- | --- | --- |
| SL-PRS based Rx-Tx measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based RSTD measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based RSRP measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based RSRPP measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based RTOA measurement | CN UE ID transmitting SL-PRS SL PRS resource ID SFN or DFN indication Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID SFN or DFN indication Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator GCS/LCS indication and translation | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator GCS/LCS indication and translation |

- Note the related IDs contained in the measurement report targeting to LMF or to UE as summarized in Table 1 is based on the assumption that the measurement request can be triggered by a UE A to perform SL positioning measurement based on SL-PRS transmitted from UE C or LMF can request UE B to perform SL positioning measurement based on SL-PRS transmitted from UE C. However, such assumption needs to be confirmed by RAN2/SA2 via LS.

- Proposal 13: RAN1 should send an LS to RAN2 regarding the potential difference in the UE ID in the measurement report to LMF or to the UE.

- Proposal 14: RAN1 should send an LS to RAN2/SA2 regarding the following questions:

Whether UE A can request UE B to perform SL positioning measurement based on SL-PRS transmitted from UE C

Whether LMF can request UE B to perform SL positioning measurement based on SL-PRS transmitted from UE C

# Conclusion

In this contribution, we provided our views on SL positioning measurement and reporting. Based on the discussion, we have the following observations and proposals.

- Observations:

- Observation 1: Indication/reporting of synchronization source type helps to choose proper anchor UEs to participate in SL-TDOA positioning.

- Observation 2: SL-PRS transmission between anchor UEs, which may reuse the SL-RTT positioning procedure, helps to reduce the influence of synchronization error.

- Proposals:

Proposal 1: For definition of SL-PRS based Rx-Tx measurement, support Alt3.

- Proposal 2: With regards to flexible transmission orders of SL-PRS for RTT-type sidelink positioning

The time stamp in the measurement report with the existing UE Rx – Tx time difference measurement is sufficient to differentiate different SL-PRS transmissions.

An inter-UE SL-PRS transmission request, which can be based on either higher layer signaling or lower layer signaling, should be introduced.

- Proposal 3: Support report-free RTT based on adjusting the transmission time of SL-PRS.

Proposal 4: For alternatives to mitigate impact of synchronization errors, support Alt1 and Alt2.

For Alt2, send an LS to RAN2 to inform them that SL positioning measurement procedure can also be applicable between anchor UEs.

- Proposal 5: Support at least SL-RTOA measurement for SL-TDOA positioning.

- Proposal 6: Support the indication of whether SFN or DFN is used along with the RTOA measurement reporting.

Proposal 7: Support AoA/ZoA relative to LCS without translation of the LCS to GCS.

- Proposal 8: Support per ARP/panel based measurement/transmission in sidelink positioning, where an indication is used to identify whether

Two measurements are from the same ARP

Two transmissions are from the same ARP

Rx time and Tx time are from the same ARP for UE Rx – Tx time difference

- Proposal 9: Not to specify any new measurements for joint Uu and SL positioning.

- Proposal 10: SL positioning measurement shall be treated as regular communication data, and be transparent to physical layer and transmitted over the PC5 communication resources.

Proposal 11: No need to provide ID for the UE receiving SL-PRS (destination ID) to either LMF or UE.

Proposal 12: Adopt the following table for the content of measurement report.

|  | reporting to LMF | reporting to UE |
| --- | --- | --- |
| SL-PRS based Rx-Tx measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based RSTD measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based RSRP measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based RSRPP measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based RTOA measurement | CN UE ID transmitting SL-PRS SL PRS resource ID SFN or DFN indication Timestamp Quality metric LoS/NLoS indicator | PC5 UE ID transmitting SL-PRS SL PRS resource ID SFN or DFN indication Timestamp Quality metric LoS/NLoS indicator |
| SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement | CN UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator GCS/LCS indication and translation | PC5 UE ID transmitting SL-PRS SL PRS resource ID Timestamp Quality metric LoS/NLoS indicator GCS/LCS indication and translation |

- Proposal 13: RAN1 should send an LS to RAN2 regarding the potential difference in the UE ID in the measurement report to LMF or to the UE.

- Proposal 14: RAN1 should send an LS to RAN2/SA2 regarding the following questions:

Whether UE A can request UE B to perform SL positioning measurement based on SL-PRS transmitted from UE C

Whether LMF can request UE B to perform SL positioning measurement based on SL-PRS transmitted from UE C

# Reference

- Chair’s Notes RAN1#112.