3GPP TSG-RAN WG1 Meeting #112bis-e	 R1-2302386

E-Meeting, 17 – 26 April, 2023

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on UL Tx switching across 3 or 4 bands in Rel-18

Document for:	Discussion and Decision

Introduction

In RAN4#106 LS, the following question was raised [1].

In addition, RAN4 would like to ask RAN1 one question: From RAN1 perspective, is it possible that the two Tx chains are switched concurrently between two different band pairs and with overlapping switching period? Two examples are given below: - Example #1: In the case of 3-band Tx switching, the switching is performed from 1T+1T on band A and B to 2T on band C. - Example #2: In the case of 4-band Tx switching, the switching is performed from 1T+1T on band A and B to 1T+1T on band C and D.

In this contribution, the question is discussed.

Discussion on the determination of UL Tx switching

In Rel-16, the determination of UL Tx switching was discussed. According to sub-clause 6.1.6 of TS 38.214, one UL Tx switching can be triggered by two UL transmissions overlapping in time as long as both the scheduling DCIs arrive early than both the cut-off time T0-Toffset of the two UL transmissions where the T0 and Toffset are the starting time and UE processing time of a UL transmission, respectively. For example, as shown in Figure 1, if after 2-port UL transmission on band B a UE receives DCI #1 & DCI #2 that are scheduling two 1-port UL transmissions on band A and band B with starting time T0 and T1, respectively, the UE performs only one UL Tx switching for both the UL transmissions. It is noteworthy that the UL interruption caused by the UL Tx switching must be prior to T0, otherwise it would interrupt the UL transmission on Band A according to Rel-16 RAN4 agreement. If the two scheduled UL transmissions are not overlapping in time, then one UL transmission is the preceding transmission of the other UL transmission, resulting in the second UL Tx switching, as shown in Figure 2. Therefore, it is straightforward that the Rel-16 mechanism on the determination of UL Tx switching should be reused in Rel-18 to avoid unnecessary UL interruption on scheduled transmissions.

Figure 1 UL transmissions on band A and band B are overlapping in time domain

Figure 2 UL transmission on band A and band B are separate in time domain

Observation 1: In Rel-16, when a UE receives two DCI’s before T0-Toffset, only one UL Tx switching is triggered for both the UL transmission scheduled by two DCIs if the UL transmissions on the two bands are at least partially overlapped in time domain, otherwise two UL Tx switching are performed.

Furthermore, it has been supported in Rel-17 that the two Tx chains are switched simultaneously for one triggered UL Tx switching, e.g. an UL Tx switching from 2-port transmission on Band A to 2-port transmission on Band B. In Rel-18, the UE should still be capable of it.

Observation 2: In Rel-17, it has been supported that the two Tx chains are switched simultaneously for one triggered UL Tx switching.

Figure 3 One UL Tx switching from band A & B to band C & D and its switching gap

In LS R1-2300029/R4-2220548, it is agreed that the effective switching time is the maximum of all potential switching periods for an UL Tx switching between band A&B to band C&D. In our understanding, the maximum operation in the RAN4 agreement can only be agreed on the basis that two Tx chains can be switched concurrently. For example, as shown in Figure 3 where X1, X2, X3, X4 are switching period of switching band pairs {band A, band C}, {band A, band D}, {band B, band C}, {band B, band D} respectively, the two Tx chains are switched concurrently from 1T+1T on band A and B to 1T+1T on band C and D if the transmissions on band C and D are at least partially overlapped in time domain. Therefore, if two Tx chains are switched for a triggered UL Tx switching, they have to be able to be switched concurrently. Otherwise, it is not in line with the RAN4 agreement on the effective switching time.

LS R1-2300029/ R4-2220548 Issue 2: Ambiguity issue when two Tx chains are switched between two different band pairs For Rel-18 UL Tx switching among 4 bands, when switching from 1T+1T on band A and B to 1T+1T on band C and D is performed, and it is not clear whether UE performs Tx switching {from band A to C + B to D} or {from band A to D + B to C}, RAN4 agreed that: - As baseline UE assumption, no need to resolve the ambiguity issue of the switching pattern for each Tx chain and determine the switching gap based on the worst case by default, i.e., neither of the two Tx chains is expected to be used for transmission during the maximum of the four switching periods, i.e., max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}. Note: Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C are the switching periods reported by the UE for band pair A&C, B&D,A&D and B&C, respectively.

For the two examples provided in R1-2302266, if the scheduled gap between transmissions on band A&B and transmissions on band C(&D) equals to the effective switching time determined by R1-2300029/R4-2220548, then all UL transmissions should not be interrupted. Therefore, in this case, the two Tx chains must be switched simultaneously for the triggered UL Tx switching regardless of any difference between the starting time of transmissions on band C and band D.

Proposal: Confirm that Rel-16 and Rel-17 mechanism of Tx switching can be directly reused in Rel-18 UL Tx switching among 3 or 4 bands.

- For dual UL, two UL transmissions that overlap in time triggers only one UL Tx switching.

- If the two Tx chains are switched for one triggered UL Tx switching, the UE should be capable to switch both Tx chain simultaneously.

Conclusion

According to the above discussions, we have the following observations and proposals:

Observation 1: In Rel-16, when a UE receives two DCI’s before T0-Toffset, only one UL Tx switching is triggered for both the UL transmission scheduled by two DCIs if the UL transmissions on the two bands are at least partially overlapped in time domain, otherwise two UL Tx switching are performed.

Observation 2: In Rel-17, it has been supported that the two Tx chains are switched simultaneously for one triggered UL Tx switching.

Proposal: Confirm that Rel-16 and Rel-17 mechanism of Tx switching can be directly reused in Rel-18 UL Tx switching among 3 or 4 bands.

- For dual UL, two UL transmissions that overlap in time triggers only one UL Tx switching.

- If the two Tx chains are switched for one triggered UL Tx switching, the UE should be capable to switch both Tx chain simultaneously.

- References

R4-2303507, “LS on Rel-18 Multi-carrier enhancement for NR”, TSG RAN4 Meeting #106, February 27 – March 3, 2023.