3GPP TSG RAN WG1 #112bis-e			R1-2302389

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.7.1

Source:	Panasonic

Title:	Spatial and power domain adaptation for network energy saving

Document for:	Discussion/Decision

# Introduction

In RAN1 #112 meeting, the agreements [1] achieved for spatial and power domain adaptation are listed in the appendix. And some evaluation results to show the performance and the utilized simulation assumption are also attached in the appendix.

In this contribution, we further share our views on the signal and procedure design for spatial and power domain adaptation.

# Discussion

## On spatial/antenna domain adaptation

According to the current specification, RRC signaling configures both CSI resource and CSI report in the IE CSI-MeasConfig and CSI-ReportConfig.

In the IE CSI-ReportConfig, not only the reference resource (including CSI-RS and potentially SSB) is configurable, but also the report quantities and used resource are configured with all the required details. Then, to further investigate more details on the previous meeting agreements, our views are provided in below.

  - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern.

As introduced above, current IE CSI-ReportConfig may well support all the required flexibility on the CSI reference resource configuration, which corresponds to the wanted spatial adaptation pattern. Network may configure multiple CSI-ReportConfig to achieve all the candidate spatial adaptation patterns. Therefore, the specification impact of this option is little to RRC. Then the main specification impact may fall in L1 signaling design to trigger/adapt one or multiple of the CSI-ReportConfig so that UE reports the CSI demanded by the network.

This option also aligns with the A1-1) in the below agreement, where our interpretation of “spatial adaptation pattern” is the CSI-RS resource configuration within IE CSI-ReportConfig.

Agreement For spatial element adaptation, further study the following - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern - FFS: Details on how the association is done - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns - FFS: Details on how the association is done - FFS: Details on the definition of “spatial adaptation patterns”

Observation 1: Using the option combination of A2-1) + A1-1) may have little specification impact to RRC to support CSI report and CSI-RS configuration for spatial pattern adaptation, as shown in Figure.1 below. On the other hand, the flexibility and supported number of active/total spatial adaptation patterns will be restricted by the maxNrofCSI-ReportConfigurations = 48.

Figure.1 An illustrating example of RRC configuration supporting multiple spatial adaptation patterns using A2-1) + A1-1)

  - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern.

In our understanding, the multiple sub-configurations correspond to multiple CSI-RS resource setting within the IE CSI-ReportConfig. Thus, this means the single CSI-RS resource configuration in the current IE CSI-ReportConfig shall be extended to multiple ones to support multiple spatial adaptation pattern. This option may be combined with either A1-1) or A1-2), depending on the detailed signaling structure design on how the CSI-RS resource configuration is extended. In addition, the CSI report corresponding to each CSI report sub-configuration may be separately or jointly reported by one CSI, depending on detailed signaling design. To us, this option has later impact to RRC specification, including both CSI-RS resource configuration and CSI report.

Observation 2: Option A2-2) has larger RRC specification impact for supporting CSI report and CSI-RS configuration for spatial pattern adaptation. A1-1) and A1-2) are both possibly used, depending on the detailed signaling structure design, as shown by Figure.2 and 3 below. On the other hand, more flexibility and higher number of active/total spatial adaptation patterns may be supported, which can be scaled from the maxNrofCSI-ReportConfigurations = 48. This also means higher UE complexity and required capability.

Figure.2 An illustrating example of RRC configuration supporting multiple spatial adaptation patterns using A2-2) + A1-1)

Figure.3 An illustrating example of RRC configuration supporting multiple spatial adaptation patterns using A2-2) + A1-2)

Therefore, starting from the pros/cons explained in Observation 1 and 2, we think it is important to check both realistic flexibilities to support for this feature from network side and the UE side complexity and capability, before concluding on which alternative.

Proposal 1: It is important to check both network side regarding realistic flexibilities to support for this feature and the UE side complexity and capability, before concluding on which alternative to adopt.

In our current understanding, when network needs UE to report CSI adapting to a new set of spatial adaptation patterns, no matter which alternative of the CSI report/resource configuration structure is adopted, network may trigger UE using L1 signaling to report CSI according to one or multiple of the spatial adaptation patterns. It is suggested to further study the enhancement of the L1 signaling, considering:

- Enhancement based on aperiodic CSI report procedure,

- Enhancement based on semi-persistent CSI report procedure,

- Enhancement based on adaptation of periodic CSI report procedure.

Proposal 2: Further study below L1 signaling enhancement:

- Enhancement based on aperiodic CSI report procedure,

- Enhancement based on semi-persistent CSI report procedure,

- Enhancement based on adaptation of periodic CSI report procedure.

## On power domain adaptation

- The power domain adaptation may lead to power change of either PDSCH or CSI-RS. On the other hand, the antenna domain adaptation may also lead to the power change of CSI-RS (when antenna virtualization is used for digital beamforming). Thus, the potential adapted power gaps due to above reasons can be:

- Power Offset #1 (PF#1):The power offset between CSI-RS and PDSCH. In the current specification, the PF#1 is RRC configured.

- Power Offset #2 (PF#2):The power offset between CSI-RS and SSS. In the current specification, it is also RRC configured.

- To calculate usable and proper CSI, especially CQI, the adaptation of both PF#1 and PF#1 needs to be informed to UE to calibrate the estimated CSI.

- Proposal 3: For efficient and effective power domain adaptation, the power offset between CSI-RS and PDSCH and the power offset between CSI-RS and SSS should be adapted with enhanced L1/L2 signaling for CSI report.

- From the above analysis, CSI-RS power adaptation can be a consequence of both spatial and power domain adaptation. On the other hand, as can be seen in the evaluation results, spatial and power domain adaptation can be jointly applied. RAN1 can consider supporting the spatial and power domain adaptation with unified framework.

- Proposal 4: Enhancement of L1/L2 signal can be designed with unified structure to support dynamic spatial element and power domain adaptation.

# Conclusion

- Based on the discussion, the following proposals are highlighted:

Observation 1: Using the option combination of A2-1) + A1-1) may have little specification impact to RRC to support CSI report and CSI-RS configuration for spatial pattern adaptation, as shown in Figure.1 below. On the other hand, the flexibility and supported number of active/total spatial adaptation patterns will be restricted by the maxNrofCSI-ReportConfigurations = 48.

Figure.1 An illustrating example of RRC configuration supporting multiple spatial adaptation patterns using A2-1) + A1-1)

Observation 2: Option A2-2) has larger RRC specification impact for supporting CSI report and CSI-RS configuration for spatial pattern adaptation. A1-1) and A1-2) are both possibly used, depending on the detailed signaling structure design, as shown by Figure.2 and 3 below. On the other hand, more flexibility and higher number of active/total spatial adaptation patterns may be supported, which can be scaled from the maxNrofCSI-ReportConfigurations = 48. This also means higher UE complexity and required capability.

Figure.2 An illustrating example of RRC configuration supporting multiple spatial adaptation patterns using A2-2) + A1-1)

Figure.3 An illustrating example of RRC configuration supporting multiple spatial adaptation patterns using A2-2) + A1-2)

Proposal 1: It is important to check both network side regarding realistic flexibilities to support for this feature and the UE side complexity and capability, before concluding on which alternative to adopt.

Proposal 2: Further study below L1 signaling enhancement:

- Enhancement based on aperiodic CSI report procedure,

- Enhancement based on semi-persistent CSI report procedure,

- Enhancement based on adaptation of periodic CSI report procedure.

- Proposal 3: For efficient and effective power domain adaptation, the power offset between CSI-RS and PDSCH and the power offset between CSI-RS and SSS should be adapted with enhanced L1/L2 signaling for CSI report.

- Proposal 4: Enhancement of L1/L2 signal can be designed with unified structure to support dynamic spatial element and power domain adaptation.

# Reference

[1] RAN1 Chairman’s notes of 3GPP RAN1 #112 meeting

[2] 3GPP TR 38.864 V18.0.0, Study on network energy savings for NR

# Appendix

Agreements from RAN1#112 meeting:

Agreement

For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases

- Type 1: all antenna elements associated to a logical antenna port is disabled/enabled

- Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

Agreement

For spatial element adaptation, further study the following

  - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern

    - FFS: Details on how the association is done

  - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns

    - FFS: Details on how the association is done

  - FFS: Details on the definition of “spatial adaptation patterns”

Agreement

For spatial element adaptation, further study the following

  - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern

  - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern

    - FFS: Details of sub-configuration

Agreement

For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g.

  - FFS: gNB indicates to UE which CSI(s) the UE shall report

  - FFS: the UE selects which CSI(s) are reported

  - FFS: multiple CSI(s) are reported in a joint CSI report

  - FFS: Overhead reduction for multiple CSI(s)

Note: UE complexity needs to be taken into account.

Agreement

For adaptation of power offset values between PDSCH and CSI-RS, further study the following

  - Where/how to configure multiple power offset values

    - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception

    - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS

    - Whether other UE report content can be included

Agreement

For spatial and power domain adaptation, solution(s) based on adaptation within an active BWP is considered as baseline

Agreement

Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that

  - Whether there is a need for transition time per adaptation (for UE)

  - Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

Evaluations results of spatial and power domain joint adaptation

- Table.1 shows evaluation results of spatial and power domain joint adaptation. The simulation assumptions are in Table.2, 3 and 4, which is based on [2].

Table.1 Evaluation of the network energy saving gain of power and spatial domain adaptation

| Simulated cases of power and antenna domain adaptation | load | Energy saving gain [%] |  | UPT loss [%] |  | Latency increase [%] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | category 1 | category 2 | Average | 50% | Average | 50% |
| Tx power 0 dB offset, Tx/Rx 8*2 | 8.13 | 0.00 | 0.00 | 0.00 | 0.00 | 0,0 | 0.00 |
| Tx power 0 dB offset, Tx/Rx 4*2 | 10.35 | 5.37 | 6.16 | 16.79 | 20.62 | 22,4 | 7.34 |
| Tx power 0 dB offset, Tx/Rx 2*2 | 13.61 | 12.36 | 13.44 | 31.41 | 34.01 | 45,5 | 20.73 |
| Tx power -3 dB offset, Tx/Rx 8*2 | 9.95 | 3.13 | 3.43 | 7.10 | 9.94 | 14,8 | 0.67 |
| Tx power -3 dB offset, Tx/Rx 4*2 | 12.51 | 8.78 | 9.74 | 24.06 | 27.60 | 34,7 | 14.20 |
| Tx power -3 dB offset, Tx/Rx 2*2 | 16.15 | 17.15 | 18.27 | 38.25 | 40.02 | 63,9 | 33.85 |
| Tx power -6 dB offset, Tx/Rx 8*2 | 11.76 | 5.65 | 6.05 | 16.53 | 22.57 | 26,9 | 6.93 |
| Tx power -6 dB offset, Tx/Rx 4*2 | 15.33 | 12.92 | 13.77 | 31.47 | 33.68 | 47,5 | 20.32 |
| Tx power -6 dB offset, Tx/Rx 2*2 | 19.2 | 22.59 | 23.59 | 45.22 | 46.20 | 75,3 | 40.58 |

Table.2 Basic SLS simulation assumption

| Set 2 FR1 |  |
| --- | --- |
| Duplex | FDD |
| System BW | 20 MHz |
| SCS | 15 kHz |
| Number of TRP | 1 |
| Total number of DL TX RUs | 8, 4, 2 |
| Total DL power level | 43, 46, 49 dBm |
| Total number of UL Rx RUs | 2 |

Table.3 Other more detailed parameters of simulation assumption

| Parameters based on Set 2 with some change |  |
| --- | --- |
| Channel model | 3D-Uma as in TR 38.901 (low-loss O2I penetration model) |
| Percentage of high loss and low loss building type | 100% low loss |
| Device deployment | 80% indoor, 20% outdoor |
| Inter-site distance | 500m |
| Network Topology | 7*3 Sector |
| Carrier Frequency | 2.1GHz |
| Multiple access | OFDMA |
| Duplexing | FDD |
| Numerology | 15KHz, 14 OFDM symbol slot |
| Guard band ratio on simulation bandwidth | FDD: 6.4% (104RB for 15kHz SCS and 20 MHz BW) |
| UT attachment | Based on RSRP |
| Wrapping around method | Geographical distance based wrapping |
| Traffic model | Follow previous RAN1 agreements |
| BS antenna height | 25 m |
| BS noise figure | 5 dB |
| BS antenna element gain | 14 dBi |
| UE noise figure | 9 dB |
| UE antenna element gain | 0 dBi |
| UE antenna height | Outdoor UEs: 1.5 m; Indoor Uts: 1.5m or consider floor height |
| Modulation | Up to 64 QAM |
| Transmission scheme | SU-MIMO |
| SU dimension | For 4Rx: Up to 4 layers |
| DL CSI measurement | Non-precoded CSI-RS based |
| DL codebook | Type I codebook |
| CSI feedback | Delay: 3slot |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Channel estimation | Ideal |
| HARQ scheme | Ideal |
| Max HARQ retransmission | 3 |
| Target BLER | 10% of first transmission |

Table.4 Traffic model used in the simulation

| Traffic type | FTP |
| --- | --- |
| Model | FTP model 3 |
| Packet size | 0.5 Mbytes |
| Mean inter-arrival time | 200 ms |