3GPP TSG RAN WG1 #112bis-e			R1-2302390

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.7.2

Source:	Panasonic

Title:	Cell DTX/DRX enhancement for network energy saving

Document for:	Discussion/Decision

# Introduction

In RAN1 #112 meeting, the agreements listed in the appendix were achieved [1].

In this contribution, we further share our views on the potentially impacted channels/signals and the more detailed Cell DTX/DRX enhancement.

# Discussion

## Cell DTX/DRX mechanism

In principle, from the base station side, cell DTX/DRX enhancement aims to save energy by time domain adaptation. In this sense, the reduction of the gNB activity can be discussed separately for DL transmission (Cell DTX) and UL reception (Cell DRX), with the following points:

- DL transmission of SSB should not be impacted according to the WID

- DL transmission of SIBs should also not be impacted, as the WID states the impact to IDLE/INACTIVE UEs should be avoided.

- Paging should not be impacted due to the same reason as above bullet.

- As the enhancement of RRC_CONNECTED mode is the focus, cell DTX enhancement should at least support UE specific PDCCH adaptation.

- As of UL reception of PRACH, SR PUCCH, periodic SRS, CG PUSCH, periodic PUCCH with CSI, a cell DRX may support all or some of them in order to maintain system stability and service quality. When UE is configured with legacy C-DRX, it is not allowed to transmit periodic SRS and CSI during OFF period for better UE power saving and system can also make use of these resource for other purpose. However, for the case of cell DRX, it is not always necessarily defined this way, as the purpose is for gNB energy saving rather than UE power saving although UE power saving gain should not be neglected. Furthermore, the gNB UL reception energy consumption is even closer to the micro sleep than DL transmission according to the gNB power consumption model [2].

Proposal 1: Cell DTX design can be more prioritized than Cell DRX, as gNB DL transmission dominates the total energy consumption. The exact UE behaviour impacted by the Cell DRX should be carefully justified for better tradeoff among network energy saving, UE power saving and system stability.

As suggested by the previous meeting agreement, our views are as following, regarding UE behaviour of not to transmit or receive during non-active periods of cell DTX/DRX:

  - DL

    - Periodic/Semi-persistent CSI-RS (including TRS). To maintain beam tracking and link monitoring, keeping TRS ON is beneficial for both CONNECTED and IDLE/INACTIVE UEs. On the other hand, solely relying on SSB may also work but with longer latency and larger UE power consumption. Therefore,

      - Depending on tradeoff between network and UE side energy consumption, TRS configured for beam and radio link monitoring and UE mobility may be kept ON during Cell DTX. To keep network side flexibility, whether ON/OFF can be configurable in the Cell DTX configuration.

      - As of periodic/semi-persistent CSI-RS for CSI measurement/report, we think they can be assumed OFF. Because the purpose to assist the PDSCH scheduling during Cell DTX is invalid.

- Proposal 2: UE is not expected to receive periodic/semi-persistent CSI-RS for CSI measurement/report during non-active periods of cell DTX. However, for TRS configured for beam and radio link monitoring and UE mobility, the availability can be at least configurable.

    - PRS. According to the IE NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r17, the periodicity of DL PRS can be flexibly configured and as large as 1024 radio frames. Therefore, in our view the question is whether it is needed to configure a periodicity of PRS shorter than that of Cell DTX.

      - If yes, as in Cell DTX non-active period, the SSB, SIB and paging transmission are all still ON, DL PRS may be configured properly by implementation to be sufficiently close or even aligned with these cell/group-common signals so network energy saving can be still not compromised.

      - If no, network can also configure DL PRS properly to align with the active duration of the Cell DTX so that the network energy saving is kept without having to transmit it during non-active period. Therefore,

- Proposal 3: For Cell DTX/DRX, UE behaviour of receiving PRS does not require specification change and can be up to gNB implementation of configuration.

    - PDCCH scrambled with UE specific RNTI. In our view, it is not required to monitor in general.

    - PDCCH in Type-3 CSS. So far, no particular reason is observed to monitor this during Cell DTX.

- Proposal 4: For Cell DTX/DRX, UE is not expected to receive PDCCH scrambled with UE specific RNTI and PDCCH in Type-3 CSS.

    - SPS-PDSCH. It is reasonable to follow handling of that in C-DRX as starting point.

- Proposal 5: For Cell DTX/DRX, UE behaviour of receiving SPS-PDSCH may follow handling of that in C-DRX as starting point.

  - UL

    - SR. Our current thinking is this behaviour can be kept as same with C-DRX, which can be a starting point. If totally disabling SR, the service quality nay be compromised. Although UE may be allowed to use random access procedure, this leads to much longer latency and system overheand.

- Proposal 6: For Cell DTX/DRX, UE behaviour relevant to SR can be same with that of C-DRX as a starting point.

    - Periodic/Semi-persistent CSI report. Similar reason with that of CSI-RS for CSI report, we think it is not necessary to let UE continue the periodic/semi-persistent CSI report during non-active period of Cell DTX/DRX.

    - Periodic/Semi-persistent SRS. Similar as above, we think it is not necessary to transmit during non-active period of Cell DTX/DRX.

- Proposal 7: For Cell DTX/DRX, UE is not expected to transmit periodic/semi-persistent CSI report and periodic/semi-persistent SRS during non-active period.

    - CG-PUSCH. Similar with the discussion for SPS-PDSCH, it is reasonable to start from the UE behaviour of C-DRX.

- Proposal 8: For Cell DTX/DRX, UE behaviour of transmitting CG-PUSCH may follow handling of that in C-DRX as starting point.

Although the WID restricts the enhancement of Cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, it is always beneficial to design the Cell DTX/DRX patterns/parameters which may align with I-DRX. This provides more opportunities for gNB to switch into low activity mode, considering gNB serves both CONNECTED and IDLE/INACTIVE mode UEs and the WID note that IDLE/INACTIVE mode UEs should not be impacted.

Proposal 9: For better network energy saving gain, the cell DTX/DRX patterns/parameters can be considered to be aligned with I-DRX.

When UE C-DRX is not configured, gNB may adapt the cell DTX/DRX parameters in the configuration for flexible control on tradeoff between energy consumption and service quality, depending on proper traffic prediction, e.g. lower load may only require relatively longer cell DTX/DRX cycle and shorter active time. Vice versus. Therefore, multiple cell DTX/DRX configurations can be considered. The switching between different configurations may need enhancement of L2/L1 signaling.

Proposal 10: Multiple cell DTX/DRX configurations should be considered for better energy saving adaptation. The switching between configurations needs possible L2/L1 signaling enhancement.

## Cell DTX/DRX and UE DRX alignment

When UE C-DRX is configured, for network energy saving, ideally, the DRX of all the RRC CONNECTED UEs could be aligned as much as possible by network implementation. Then the maximum gNB sleep opportunity by cell DTX/DRX can be achieved. However, depending on the UE and traffic load, frequency resource, channel condition and MIMO schemes, this may lead to resource congestion in the aligned DRX active time. This may lead to system performance degradation.

Observation 1: When UE DRX is configured, strict alignment of all UE DRX configurations and cell DTX/DRX may potentially lead to resource congestion.

Therefore, in practice, the alignment between UE C-DRX and cell DTX/DRX can be achieved by proper configurations for the UEs in a semi-synchronized or quasi-synchronized way such that some “sleep” slots can be created in system level, while UEs can still finish the DL/UL traffic in less crowed slots, as shown in Figure.1. This allows more resilience for network to arrange and aggregate the UE traffic scheduling in as less slots as possible, but in the meanwhile ensure the tradeoff between the network energy saving, UE power saving and system performance. Therefore, to flexibly adapt the traffic and align the UE DRX and cell DTX/DRX, multiple UE C-DRX configurations with different set of parameters can be considered. In addition, the switching between configurations needs possible L1/L2 signaling enhancement.

Proposal 11: Multiple UE DRX configurations can be considered for more flexible adaption to achieve alignment with cell DTX/DRX. The switching between configurations needs possible L1/L2 signaling enhancement.

Figure.1 An example of gNB implementation to arrange the C-DRX configurations of UEs and align with cell DTX/DRX to create sleep slots

- If cell DTX/DRX is supported by high layer configuration and UE is configured with C-DRX, the coexistence should be studied to ensure the mutual understanding of the network and UE. Based on the network instruction, UE behaviour may be adapted by either applying one or more configurations at the same time.

- Proposal 12: Interaction of Cell DTX/DRX and UE C-DRX needs to be clarified if both are supported and configured.

# Conclusion

- Based on the discussion, the following proposals are highlighted:

Proposal 1: Cell DTX design can be more prioritized than Cell DRX, as gNB DL transmission dominates the total energy consumption. The exact UE behaviour impacted by the Cell DRX should be carefully justified for better tradeoff among network energy saving, UE power saving and system stability.

- Proposal 2: UE is not expected to receive periodic/semi-persistent CSI-RS for CSI measurement/report during non-active periods of cell DTX. However, for TRS configured for beam and radio link monitoring and UE mobility, the availability can be at least configurable.

- Proposal 3: For Cell DTX/DRX, UE behaviour of receiving PRS does not require specification change and can be up to gNB implementation of configuration.

- Proposal 4: For Cell DTX/DRX, UE is not expected to receive PDCCH scrambled with UE specific RNTI and PDCCH in Type-3 CSS.

- Proposal 5: For Cell DTX/DRX, UE behaviour of receiving SPS-PDSCH may follow handling of that in C-DRX as starting point.

- Proposal 6: For Cell DTX/DRX, UE behaviour relevant to SR can be same with that of C-DRX as a starting point.

- Proposal 7: For Cell DTX/DRX, UE is not expected to transmit periodic/semi-persistent CSI report and periodic/semi-persistent SRS during non-active period.

- Proposal 8: For Cell DTX/DRX, UE behaviour of transmitting CG-PUSCH may follow handling of that in C-DRX as starting point.

Proposal 9: For better network energy saving gain, the cell DTX/DRX patterns/parameters can be considered to be aligned with I-DRX.

Proposal 10: Multiple cell DTX/DRX configurations should be considered for better energy saving adaptation. The switching between configurations needs possible L2/L1 signaling enhancement.

Observation 1: When UE DRX is configured, strict alignment of all UE DRX configurations and cell DTX/DRX may potentially lead to resource congestion.

Proposal 11: Multiple UE DRX configurations can be considered for more flexible adaption to achieve alignment with cell DTX/DRX. The switching between configurations needs possible L1/L2 signaling enhancement.

# Reference

[1] RAN1 Chairman’s notes of 3GPP RAN1 #112 meeting

[2] 3GPP TR 38.864 V18.0.0, Study on network energy savings for NR

# Appendix

Agreements from RAN1 #112 meeting

Agreement

- RAN1 continues discussion on the at least following physical layer related aspects of cell DTX/DRX aspects

  - physical layer signals/channels and procedures expected to be impacted during non-active periods of cell DTX/DRX

    - consider impact to at least KPIs from the SI when physical layers/signals/channels are impacted by cell DTX/DRX

- Further discussions on other aspects are not precluded

Agreement

At least the following candidate signals/channels for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, are considered from RAN1 perspective for further discussion. The exact set of signals/channels that the UE may be expected to not transmit or receive is FFS.

- DL

  - Periodic/Semi-persistent CSI-RS (including TRS)

  - PRS

  - PDCCH scrambled with UE specific RNTI

  - PDCCH in Type-3 CSS

  - SPS-PDSCH

- UL

  - SR

  - Periodic/Semi-persistent CSI report

  - Periodic/Semi-persistent SRS

  - CG-PUSCH

Other signals/channels are not precluded