3GPP TSG RAN WG1 Meeting #112bis-e	R1-2302393

E-meeting, April 17th – 26th , 2023

Agenda item:		9.7.1

Source:	Nokia, Nokia Shanghai Bell

Title:		Techniques in Spatial and Power Domains

Document for:		Discussion and Decision

# Introduction

Based on the WI description in RP-223540, the following objective has been agreed on NW energy saving techniques in spatial and power domains:

- Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

RAN1#112 was the first meeting where the above Rel-18 objectives were discussed.

In this contribution, we continue the discussions on various aspects from the above objective for each of the spatial domain technique and power domain technique, considering the discussions and agreements made in RAN1#112 for which the related FL summaries can be found in R1-2301964, R1-2301965, and R1-2301966. In our companion contribution, we discuss the enhancements on cell DTX/DRX mechanism [1].

# Techniques in Spatial domain

When considering antenna-based adaptation, based on gNB implementation, there could be different ways of mapping between (logical) antenna ports and TxRUs (transceiver units), where a TxRU is associated with an antenna ‘subarray’ that consists of a set of physical antenna elements. In that regard, in RAN1#112 the following two adaptation Types, from an antenna port perspective, were agreed to be considered in the discussions (see corresponding illustration in Figure 1):

Agreement For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases - Type 1: all antenna elements associated to a logical antenna port is disabled/enabled - Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

Figure 1: Example illustrating spatial adaptation from an antenna port perspective.

Considering the Type 1 and Type 2 above, it can be observed that different spatial (adaptation) patterns may have:

- different sets of antenna ports

- same set of antenna port but different set of active antenna elements.

Observation 1: Given Type 1 and Type 2 adaptation, which relate to the mapping between antenna ports and TxRUs, it can be observed that different spatial patterns may have:

- different sets of antenna ports, or

- same set of antenna ports but different set of active antenna elements.

The above observation would need to be considered when discussing the various open aspects on spatial adaptation such as the potential enhancements on CSI report configuration as well as the spatial adaptation indication.

## Enhancements on CSI measurement and reporting to enable spatial adaption

Spatial adaptation essentially consists in the network switching, in a dynamic or semi-dynamic manner, from one spatial pattern to another, to potentially achieve network energy saving gains. An example of spatial (muting) patterns can be found in Figure 2.

Figure 2: Example illustrating some spatial (muting) patterns; dotted/grey boxes correspond to muted spatial elements.

In the following section, we discuss various aspects related to CSI measurement and reporting, in order to enable spatial adaption for the purpose of achieving network energy savings.

### CSI reporting configuration to enable spatial adaptation

In order to evaluate the performance of each spatial/antenna (adaptation) pattern, it would be reasonable that the gNB asks UEs to evaluate different spatial patterns and provide related feedback/report. The gNB would then account for the reports from different UEs in order to decide which spatial pattern(s) to apply or whether to update a current applicable spatial pattern.

Based on legacy procedures, a UE can already be configured with multiple CSI report configurations and triggered to provide reports/measurements for these configurations, where each report configuration (along with a CSI resource setting configuration) could correspond to a spatial/antenna (muting) pattern.

In other words, existing specifications somewhat allow the evaluation of various spatial patterns. However, this would clearly require large number of CSI report configurations, considering that we would typically have several spatial patterns to evaluate. And it should be noted that there are currently limitations on such numbers e.g., per BWP and CC, such as (see TS 38.331 and TS 38.306):

- Maximum number of periodic CSI report configurations/settings per BWP for CSI report, for which the candidate values are {1, 2, 3, 4}.

- Maximum number of aperiodic CSI report configurations/settings per BWP for CSI report, for which the candidate values are {1, 2, 3, 4}.

- Maximum number of CSI report(s) for which the UE can measure and process reference signals simultaneously in a CC, for which the candidate values are {from 1 to 8}; note that the CSI report comprises periodic, semi-persistent and aperiodic CSI and any latency classes and codebook types; also, the CSI report includes the beam report and CSI report.

In addition, the following Note in the WI objective mandates considering legacy UE CSI related capabilities:

“Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements”

The above Note doesn’t explicitly provide all the details on which capabilities to exactly consider, it would thus be good to clarify which ones are exactly covered by this Note. We thus propose:

Proposal 1: Clarify the exact UE CSI/CSI-RS capabilities covered in the following Note (captured in the WI description):

- “Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements.”

- Without further clarifications on this Note, all the legacy capabilities related to total number of CSI reports and requirements would need to be assumed.

#### Resource configuration

In RAN1#112, the following related agreement was made:

Agreement: For spatial element adaptation, further study the following - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern - FFS: Details of sub-configuration

If the legacy capabilities regarding supported numbers of CSI report configurations are kept, then to evaluate multiple spatial patterns there would be a need for multiple CSI report configurations, each of which e.g., corresponding to a spatial pattern. However, if the number of spatial patterns is large, this would result in the need for a large number of CSI report configurations, but this would then violate the legacy capabilities – especially that some CSI report configurations would be needed for other purposes than spatial pattern evaluation (such as for ‘normal’ CSI acquisition). At the same time, this doesn’t necessarily mean that more than one CSI report configuration cannot be used for the purpose of evaluating different spatial patterns.

Observation 2: If the number of spatial patterns is large, this would result in the need for a large number of CSI report configurations, but this then violates the legacy capabilities – especially that some CSI report configurations would be needed for other purposes than spatial pattern evaluation.

Thus, if a large number of spatial patterns is assumed, which would be a reasonable assumption to get full benefits from the spatial adaptation operation, a same CSI report configuration could be needed to evaluate multiple spatial patterns, meaning that a CSI report configuration would potentially need to contain or be associated with multiple (spatial) (sub-)configurations for different spatial patterns. And an option along the lines of Alt.2-2 in the above agreement should be adopted in this case. On the other hand, if only a few spatial patterns are considered, then Alt.2-1 would be enough and then potentially no enhancements would be needed on the legacy operation.

Observation 3: If only a few spatial patterns are considered, A2-1 (i.e., independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern) might be enough and then no enhancements would be needed on the legacy operation.

Proposal 2: If a large number of spatial patterns is considered, an option along the lines of A2-2 could be considered where one CSI report configuration is used to evaluate multiple spatial adaptation patterns.

Based on the above, and assuming that a somewhat large number of spatial patterns is considered, enhancements on the existing CSI report configuration structure would then be needed. In RAN1#112, there had been some discussions on how the CSI report configuration should be enhanced, where the following agreement listing several alternatives was made:

Agreement For spatial element adaptation, further study the following - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern - FFS: Details on how the association is done - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns - FFS: Details on how the association is done - FFS: Details on the definition of “spatial adaptation patterns”

First, it’s worth recalling that in the existing specifications:

- In each CSI-RS resource set there could be up to 64 NZP CSI-RS resources, and for each NZP-CSI-RS resource there could be up to 32 antenna ports. All CSI-RS resources within one set are configured with same nrofPorts and density, except for the NZP CSI-RS resources used for interference measurement.

- A resource setting (logically) groups/selects the resources for channel measurement and interference measurement that are intended to be used for a certain purpose.

- A CSI report configuration/setting can be linked to more than one resource setting.

It’s also worth recalling, as observed earlier, that two different spatial patterns may have:

- different sets of antenna ports, or

- same set of antenna port but different set of active antenna elements.

Based on the alternatives listed in the above agreement, for the association and thus evaluation of spatial patterns, different resource levels/granularities can be considered:

- CSI-RS resource level, or

- CSI-RS resource set level, or

- Resource setting level

In our view, some important factors in deciding which alternative to opt for are specification efforts required, DL overhead, flexibility, as well as applicability to both Type 1 and Type 2 adaptation. In the following, we discuss the above alternatives from those perspectives. We specifically consider the alternatives A1-1 and A1-2 listed in the above agreement and discuss different options and sub-options extracted from those alternatives.

Option 1: CSI-RS resource level: Two variants can be listed here:

- Option 1-1: one CSI-RS resource is associated to one spatial pattern.

- Option 1-2: one CSI-RS resource is associated to / used to evaluate multiple spatial patterns.

Under Option 1, the existing constraint/limitation (mentioned above) that a CSI report is only possible considering same nrofPorts for the CSI-RS resources of a CSI-RS resource set associated with the CSI report configuration would need to be lifted.

In addition, considering specifically Option 1-2, using one CSI-RS resource transmission, or more generally, one CSI-RS resource to evaluate multiple spatial patterns could work in general. For example, gNB would transmit a 32-port CSI-RS resource and ask the UE to evaluate different spatial patterns corresponding to different antenna port subsets, such as patterns corresponding to 16 ports and 8 ports etc. If such an option is adopted/leveraged to cover at least most of the cases, large DL overhead could be potentially saved as multiple spatial patterns can be evaluated using a single CSI-RS resource; this in turn allows more network energy saving gains.

One case where Option 1-2 wouldn’t work if we rely on one CSI-RS resource occasion/instance is when considering Type 2 adaptation where the different spatial patterns to evaluate have the same set/subset of antenna ports but have difference in terms of active, or alternatively muted, antenna elements; this may then call for the use of Option 1-1 for such cases. Nevertheless, if considered, such cases could still be avoided if two such spatial patterns are evaluated using different CSI-RS resource occasions in time and if the UE is aware that the two spatial patterns have difference in terms of active, or alternatively muted, antenna elements.

Furthermore, Option 1-2 seems to be primarily aimed at e.g., FR1 and/or considering non-precoded CSI-RS. Whether this option would work for all cases and scenarios of interest could be discussed. For instance, it should be discussed whether this option would work just fine for beamformed CSI-RS, or even whether it should be supported at all in this case.

Clearly, with Option 1-2, the UE would need to be signalled by the gNB at least the subsets of antenna ports that it needs to evaluate considering one CSI-RS resource. It should be also discussed if other parameters/configurations need to be signalled/updated to the UE under this option.

Observation 4: Compared to Option 1-1 (where different spatial patterns are evaluated using different CSI-RS resources), with Option 1-2, where one CSI-RS resource is used to evaluate multiple spatial patterns, DL overhead could be potentially saved; this in turn allows more network energy saving gains.

Observation 5: One case where Option 1-2 (where one CSI-RS resource is used to evaluate multiple spatial patterns) may not work is when considering Type 2 adaptation where the different spatial patterns to evaluate have the same set/subset of antenna ports but have difference in terms of active, or alternatively muted, antenna elements. Instead, such case could be feasible e.g., with Option 1-1 where different spatial patterns are evaluated using different CSI-RS resources.

Observation 6: Option 1-2 (where one CSI-RS resource is used to evaluate multiple spatial patterns) seems to be primarily aimed at FR1 and considering non-precoded CSI-RS. Whether this option would work or for all cases and scenarios of interest can be discussed.

Observation 7: With Option 1-2 (where one CSI-RS resource is used to evaluate multiple spatial patterns), the UE would need to be signalled by the gNB at least the subsets of antenna ports that the UE needs to evaluate considering one CSI-RS resource.

Option 2: CSI-RS resource set level: Two variants can be listed here:

- Option 2-1: one CSI-RS resource set is associated to one spatial pattern.

- Option 2-2: one CSI-RS resource set is associated to /used to evaluate multiple spatial patterns.

At least under Option 2-1, several CSI-RS resource sets associated to a same CSI report configuration would be needed, and this may result in the need to lift the existing constraint/limitation on the maximum number of CSI-RS resource sets that can be associated to a CSI report configuration. Also, an indication would be needed under this option in order to signal the UE which CSI-RS resource set(s), or alternatively which spatial pattern(s), to consider for evaluation at a certain time.

On the other hand, under Option 2-2, an indication of which spatial patterns to consider for evaluation at a certain time would be needed. In addition, for this option, the existing limitation that CSI report is only possible considering same nrofPorts for the CSI-RS resources of a CSI-RS resource set associated with a CSI report configuration would need to be lifted.

Note that Option 2-2 is in a way similar to Option 1-2 as a CSI-RS resource set could e.g., comprise one CSI-RS resource with large number of antenna ports (such as 32). Also, Option 2-2 could cover Option 1-1 as different CSI-RS resources in a set could be corresponding to different spatial patterns.

Observation 8: Under Option 2-1, where one CSI-RS resource set is associated to one spatial pattern, several CSI-RS resource sets associated to a same CSI report configuration would be needed, and this may result in the need to lift the existing constraint/limitation on the maximum number of CSI-RS resource sets (e.g., for channel measurement) that could be associated to a CSI report configuration.

Observation 9: Under Option 2-2, where one CSI-RS resource set is associated to / used to evaluate multiple spatial patterns, the existing limitation that CSI report is only possible considering same nrofPorts for the CSI-RS resources of a CSI-RS resource set associated with a CSI report configuration would need to be lifted.

Observation 10: Option 2-2 (where one CSI-RS resource set is associated / used to evaluate multiple spatial patterns) could cover Option 1-2 (where one CSI-RS resource is associated / used to evaluate multiple spatial patterns. In addition, Option 2-2 could also cover Option 1-1 (where one CSI-RS resource set is associated to one spatial pattern).

Option 3: resource setting level: Two variants can be listed here:

- Option 3-1: one resource setting is associated to one spatial pattern.

- Option 3-2: one resource setting is associated to /used to evaluate multiple spatial patterns.

Option 3 is similar to Option 2, so in a way similar observations as for Option 2 can be made here. But Option 3 has the advantage of covering resources for both channel and interference measurements. Also, depending on how they are exactly designed, Option 3-1 and Option 3-2 could achieve same/similar outcome. Comparing between Option 3-1 and Option 3-2, Option 3-2 would consume less resource settings compared to Option 3-1.

It’s worth recalling that, in legacy, typically only a single CSI-RS resource set within a resource setting is used for measurement for a certain CSI report, and in fact, periodic/semi-persistent resource settings can only contain a single resource set; one exception is the case with groupBasedBeamReporting-r17 where the number of CSI-RS resource sets configured is 2. However, for aperiodic resource settings multiple resource sets can be defined, and which set is to be used for a certain aperiodic measurement and report is indicated in the aperiodic trigger state.

Thus, for periodic/semi-persistent resource settings, Option 3-1 would require that multiple resource settings are associated to a CSI report configuration.

Observation 11: Option 3, where resource setting level is considered, is similar to Option 2 (where a CSI-RS resource set level is considered), so in a way similar observations as for Option 2 can be made here. For instance, Option 3-2 (where one resource setting is associated to /used to evaluate multiple spatial patterns) could cover Options 1-2 and Option 1-1.

Observation 12: Option 3, where resource setting level is considered, has the advantage of covering resources for both channel and interference measurements.

Observation 13: Option 3-2 (one resource setting is associated to /used to evaluate multiple spatial patterns) would consume less resource settings compared to Option 3-1 (one resource setting is associated to one spatial pattern).

Observation 14: Under Option 3-1, where one resource setting is associated to one spatial pattern, for periodic/semi-persistent resource settings it’s required that multiple resource settings are associated to a CSI report configuration.

* * * * * * * * * * *

Looking at the above discussions, Options 3-1 and 2-1 (and 1-1) would potentially require higher DL overhead due to the need for more CSI-RS transmissions, and would thus consume more energy, compared specifically to Option 1-2, as multiple CSI-RS resources /resource sets would be needed to evaluate multiple patterns under these former options. On the other side, specs effort is required for all the options, with some options requiring less or more effort as can be seen from the above discussions. Also, all the options could be flexible enough depending on how they are exactly designed. Further, recall that Option 3 has the advantage (compared to other options) of covering resources for both channel and interference measurements.

Therefore, considering DL overhead (which also impacts energy saving) as one criterium, we propose to focus on Option 1-2 (along with Option 1-1 if seen needed), Option 2-2 and Option 3-2 in the upcoming RAN1 discussions. One could also consider the combinations of these options if determined to be needed.

Proposal 3: To enable CSI assistance information for spatial adaptation, further consider Option 1-2 and Option 2-2:

- Option 1-2: one CSI-RS resource is associated to / used to evaluate multiple spatial patterns.

- Option 2-2: one CSI-RS resource set is associated to / used to evaluate multiple spatial patterns.

- FFS: whether these options can be used for all cases and scenarios of interest, and if not, which other option(s) should be adopted.

- FFS: how to indicate the UE with the antenna port subsets which represents different spatial patterns for evaluation/reporting under these options.

  - FFS: whether other information should be indicated/updated to the UE.

Proposal 4: To enable CSI assistance information for spatial adaptation, further consider Option 3-2:

- Option 3-2: one resource setting is associated to /used to evaluate multiple spatial patterns.

- FFS: whether this option can be used for all cases and scenarios of interest.

On whether there is a need to introduce new CSI-RS resource patterns:

At least one company [4] mentioned about the nested structure of CSI-RS resource patterns and whether there is a need to introduce additional patterns for that purpose. In our view, additional CSI-RS patterns are not needed and would anyhow potentially lead to large specs effort. Instead, we should focus on ways that leverage existing CSI-RS patterns to enable the spatial adaptation.

Observation 15: Additional CSI-RS resource patterns do not seem needed and would anyhow potentially lead to large specs effort. Instead, we should focus on ways that leverage existing CSI-RS patterns to enable the spatial adaptation.

That is another reason to consider an option along the lines of Option 1-2/Option 2-2/Option 3-2 at least as a basis, meaning that to consider a CSI-RS resource(s) associated with a large set of antenna ports, such as 32-port CSI-RS resource, and indicate the UE which one or more port subsets to consider for evaluation and/or report in order to assist the spatial adaptation procedure at the gNB. An illustration of a 32-port CSI-RS resource considering row #17 from Table 7.4.1.5.3-1 in TS 38.211 is provided below. The gNB can then signal the UE with different antenna port subsets for evaluation and reporting. For instance, the gNB can signal the UE to evaluate and report CSI corresponding to AP 0-15 or AP 16-31 for 16-port spatial patterns, and to AP 0-7 or AP 8-15 etc. for 8-port spatial patterns.

Figure 3: 32-port CSI-RS resource according to row #17 (cdm4-FD2-TD4) from Table 7.4.1.5.3-1 in TS 38.211.

Proposal 5: Do not introduce additional CSI-RS resource patterns for the purpose of spatial adaptation.

#### Spatial (pattern) configuration

In addition to (active) spatial elements (e.g., antenna ports, TxRUs), a spatial (muting) pattern may also correspond to other configurations/elements, such that dedicated configurations/elements are corresponding to a spatial (muting) pattern; the set of all these configurations/elements could be seen and termed as ‘spatial configuration’. For instance, one such configuration/element may be the ‘codebook’ configuration (including codebook subset restriction). Recall that, among other parameters, the codebook configuration comprises and, for a given number of antenna ports, corresponds to the following parameters:

- N1, N2, which respectively refer to the antenna port layout in vertical and horizontal dimensions, and O1, O2, which respectively refer to the oversampling ratios in vertical and horizontal dimensions. See the table below (copied from TS 38.214) for the supported configurations of N1, N2 and O1, O2, considering different number of CSI-RS antenna ports.

Table 5.2.2.2.1-2: Supported configurations of and

| Number of CSI-RS antenna ports, |  |  |
| --- | --- | --- |
| 4 | (2,1) | (4,1) |  |
| 8 | (2,2) | (4,4) |  |
|  | (4,1) | (4,1) |  |
| 12 | (3,2) | (4,4) |  |
|  | (6,1) | (4,1) |  |
| 16 | (4,2) | (4,4) |  |
|  | (8,1) | (4,1) |  |
| 24 | (4,3) | (4,4) |  |
|  | (6,2) | (4,4) |  |
|  | (12,1) | (4,1) |  |
| 32 | (4,4) | (4,4) |  |
|  | (8,2) | (4,4) |  |
|  | (16,1) | (4,1) |  |

When CSI feedback, including PMI, needs to be derived for a certain spatial pattern, the UE would need to have a correct assumption regarding the corresponding spatial configuration (such as codebook configuration). It should thus be discussed how to enable the UE to determine the spatial configuration corresponding to a spatial pattern. Note that this aspect would potentially depend on the option to adopt considering the above-described options on CSI-RS resource level, CSI-RS resource set level, and resource setting level. It’s also worth noting that we shouldn’t aim at modifying or introducing new combinations of N1, N2, O1, O2.

Observation 16: When CSI feedback, including PMI, needs to be derived for a certain spatial pattern, in general the UE would need to have a correct assumption regarding the corresponding spatial configuration, such as codebook configuration including codebook subset restriction.

Proposal 6: For evaluating a spatial pattern, discuss how to enable the UE to determine the spatial configuration, such as codebook configuration (including codebook subset restriction), corresponding to a spatial pattern.

#### Frequency domain related configuration

Based on legacy, the CSI reporting band for the CSI report is defined as a subset of sub-bands of the BWP. The UE shall only consider these sub-bands when determining the CSI. Considering the options discussed above, when the number of spatial elements (such as antenna ports or TxRUs) is reduced, the beam pattern from lower (resp., larger) number of spatial elements may be wider (resp., narrower) due to low (resp., high) spatial resolution. A wider beam may increase the number of multipaths, which then results in higher delay spread; a tighter beam may decrease the number of multipaths, which then results in lower delay spread. High delay spread is reflected as frequency selectivity in channel.

In other words, the channels with different spatial patterns may have different frequency selectivity characteristics. It would thus be important to discuss this observation when revisiting the CSI report configuration design from frequency domain configuration perspective, and specifically from sub-band configuration perspective.

Observation 17: When the number of spatial elements (such as antenna ports or TxRUs) is reduced, the beam pattern from lower (resp., larger) number of spatial elements may be wider (resp. narrower) due to low (resp., high) spatial resolution. A wider beam may increase the number of multipaths, which then results in higher delay spread; a tighter beam may decrease the number of multipaths which then results in lower delay spread. High delay spread is reflected as frequency selectivity in channel.

Proposal 7: Discuss the implications of different spatial patterns potentially having different channel characteristics in terms of frequency selectivity on the design of CSI report configuration, specifically from sub-band configuration perspective.

### CSI report content and overhead, and related UE burden

#### CSI report content and overhead

First, in RAN1#112, the following high-level agreement was made regarding CSI report to enable spatial pattern adaptation.

Agreement: For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g. - FFS: gNB indicates to UE which CSI(s) the UE shall report - FFS: the UE selects which CSI(s) are reported - FFS: multiple CSI(s) are reported in a joint CSI report - FFS: Overhead reduction for multiple CSI(s) Note: UE complexity needs to be taken into account.

Here, we provide discussion on the various aspects related to the FFS points of the above agreement.

On what the UE is expected to report to assist the gNB in selecting a suitable spatial pattern:

- Option A - UE selects one or more reports corresponding to an indicated set of candidate spatial patterns: In our view it, would be reasonable to allow the gNB to configure/ trigger the UE to select and report one (or more) spatial pattern from a set of candidate spatial patterns the UE is configured or indicated with. In addition to the UE selecting and reporting best/preferrable pattern(s), gNB would ask the UE to also report CSI measurements, such as PMI/RI, for the selected pattern(s). Generally speaking, gNB should be able to configure the UE to report more than one pattern, e.g., 2 out of X patterns, and one of the reported patterns will be selected by the gNB e.g., according to such reports from multiple UEs.

- Option B - UE provides reports for all the spatial patterns in an indicated set of candidate patterns: Another approach would be that the UE is (always) required to report measurements corresponding to all the spatial patterns in the set. This approach provides additional knowledge for the gNB; however, it results in high UL control overhead (especially when the number of spatial patterns is large) without necessarily bringing noticeable additional benefits compared with the approach consisting in selecting and reporting one or a couple of (best/preferrable) spatial pattern(s).

Observation 18: Relying on the approach where the UE is (always) required to provide CSI report(s) corresponding to all or several of configured spatial patterns would result in large UL control overhead, especially when the number of spatial patterns is large.

As discussed above, asking the UE to select and report one or a couple of spatial patterns, along with their corresponding measurements, can guarantee somewhat low UL control overhead. We now discuss some aspects related to how such selection could be performed. One aspect is if there is a need to differentiate between the cases where the set of candidate patterns the UE is selecting from have same number of active antenna elements or not. Specifically:

- Case 1: When the spatial patterns in the set of candidate patterns have same number of (active) spatial/antenna elements. In this case, there is practically no difference in power/energy saving level (from network Tx perspective) between the different spatial patterns. One way for pattern selection would be to follow similar logic as for CRI (CSI-RS resource indicator) down-selection in Rel-15, by basically letting the UE select one preferrable pattern based on UE implementation.

  - Other than UE implementation, it’s also possible to let the gNB configure criteria for the selection, such as ones related to rank, RSRP or CQI, etc.

- Case 2: When the patterns in the set of patterns have different number of (active) spatial/antenna elements. In this case, different spatial patterns may have different power/energy saving levels. Hence, UE would need to report a pattern(s) with best power/energy saving while fulfilling one or more criteria related with performance constraint, such as minimum rank.

The above approach, considering Case 1 and Case 2, would then allow the gNB to be informed about one (or more) preferrable/best spatial patterns from UE perspective without the need to get feedback for all the set of spatial patterns that the UE has been instructed to select from. At the same time, such an approach may require more specs effort.

Observation 19: It may not be necessary for the UE to provide CSI report corresponding to all candidate spatial patterns, and it could be sufficient to have UE reporting based on a selection of one or a couple of spatial patterns. This would then allow keeping the UL control overhead low. At the same time, such an approach may require more specs effort.

Observation 20: For spatial pattern selection at the UE, there may be a need to differentiate between the case where the set of candidate patterns (the UE is selecting from) have same and the case where they have different number of (active) spatial/antenna elements.

Proposal 8: To minimize the CSI feedback overhead, discuss on the CSI report content, i.e., whether/how this report would be based on a subset selection from multiple spatial patterns by the UE.

Proposal 9: Discuss whether there is a need to differentiate the CSI report content depending on whether the spatial patterns have same or different number of spatial/antenna elements.

From another perspective, the size of CSI feedback information bits may be different according to the spatial pattern. Considering e.g., Option 1-2 (described earlier) with 32-port CSI-RS resource, the size of CSI feedback information may be different according to the corresponding spatial patterns. When the spatial pattern is changed, the number of ports P and the CSI codebook parameters (N1, N2, O1, O2) may change. The size of PMI reporting is a function of these parameters, and when the UE reports PMI with/for different spatial patterns, the size of PMI reporting is different. Furthermore, the CSI reporting size depends on the corresponding sub-band configuration.

Given that different spatial patterns would potentially have different channel characteristics in terms of frequency selectivity (as discussed earlier), different sub-band configurations for different spatial patterns seem necessary. This would then allow to reduce the CSI feedback overhead. This observation is valid mainly for Option B (described above) but may also work for Option A depending on how the CSI report content is exactly defined under this latter option.

Proposal 10: Discuss CSI report feedback size reduction considering sub-band configuration adaptation to each spatial pattern.

#### Burden at the UE

Even when the CSI report would rely on a selection of one or a couple of spatial patterns, the UE burden due to evaluating multiple (indicated) candidate spatial patterns would be high. It should thus be discussed whether there could be ways to reduce the burden at the UE for searching for/ selecting suitable/preferred/best pattern(s). Otherwise, the UE would need to evaluate all candidate patterns, by e.g., calculating their PMI/RI, in order to be able to select a pattern(s). However, this would increase the UE burden and power consumption, especially when the number of patterns to evaluate and select from is large. In this regard, it should be discussed whether it’s necessary to evaluate all the patterns before making selection of best suitable/preferred/best pattern by the UE. This observation is valid for at least Case 2 described earlier.

Not mandating the UE to evaluate all the candidate spatial patterns would clearly decrease the burden at the UE. To this end, one way would be to consider a ‘search approach’ that allows the UE to skip the evaluation of some spatial patterns based on some rules. For instance, some sort of ‘hierarchical’ search approach could be considered where the UE may e.g.: start with the patterns with highest number of antenna/spatial elements that satisfy one or more of the above-mentioned (performance) criteria and selects at least one of these patterns, and discard from evaluation/measurement/report patterns that are not partially or fully subsets of the selected at least one pattern.

Observation 21: Evaluations/measurements of all (indicated) candidate spatial patterns increases the UE burden and power consumption.

Proposal 11: In the case where the UE is required to provide CSI report based on multiple (indicated) candidate spatial patterns, to reduce the burden at the UE, discuss whether/how the UE is allowed to skip evaluating some of the candidate spatial patterns based on some rules.

From another perspective, evaluating multiple spatial patterns and providing a corresponding CSI report(s) using one UL (PUCCH/PUSCH) reporting occasion would increase the consumption of CSI processing units (CPU) at the UE, given that a CPU is only released after the CSI report is transmitted. And thus, the required number of additional CPUs would potentially need to scale up with the number of spatial patterns to evaluate before the CSI report transmissions. Hence, considering CSI report(s) through different UL reporting occasions would allow to reduce the need to consume many CSI processing units at a time in this case. However, for this purpose, it should be discussed how to configure the CSI reports in time, considering different reporting types (semi-persistent, periodic, aperiodic).

Observation 22: Evaluating multiple spatial patterns and providing a corresponding CSI report(s) using a single UL (PUCCH/PUSCH) reporting occasion would increase the consumption of parallel CSI processing units required at the UE.

Proposal 12: Discuss how to configure CSI measurements and reports for different spatial patterns in time, considering different reporting types (semi-persistent, periodic, aperiodic).

### Impact on CSI derivation/computation

The discussion here essentially assumes that spatial adaptation can impact at least some of the CSI-RSs (other than the ones used for spatial patterns evaluation). This would be reasonable to support, otherwise less network energy savings will be achieved and the gNB wouldn’t be able to e.g., multiplex CSI-RS and PDSCH in the frequency domain at least in some cases.

Observation 23: If spatial adaptation is not allowed to impact at least some of the CSI-RSs (other than the ones used for spatial patterns evaluation), less network energy savings will be achieved and the gNB wouldn’t be able to e.g., multiplex CSI-RS and PDSCH in the frequency domain (at least in some cases).

The impact of spatial adaptation on CSI computation/measurements should be discussed, and this includes channel measurements and interference measurements. Specifically, when a new spatial pattern becomes applicable within a period of CSI measurements, it should be clarified how the CSI computation should be performed if this calculation is based on CSI-RS resources impacted by switching to a new spatial pattern, i.e., impacted by the applicability of a new spatial pattern.

Based on legacy procedures (TS 38.214), in the CSI Report configuration/setting, it is possible to configure measurement restrictions in the time-domain, through timeRestrictionForChannelMeasurements and timeRestrictionForInterferenceMeasurements, for channel and interference resources respectively. If measurement restriction is configured, the UE is only allowed to use the latest occurrence of the CSI-RS/IM for channel/interference measurement into account when deriving the CSI; basically, the UE is restricted from temporally averaging the measurement of the resources in this case. Otherwise, if this measurement restriction is not configured, the UE would typically perform averaging in order to improve the channel/interference estimation performance. In this case, the gNB doesn’t have full knowledge of which resources/occurrences the UE has used for the CSI computation; see Figure 3 below.

Figure 3: Illustration of legacy CSI derivation based on multiple reference signals measurement resources/occurrences/samples.

Observation 24: Spatial adaptation may impact CSI computation/derivation, and this includes channel measurements and interference measurements.

Proposal 13: Discuss how the CSI computation/derivation operation is impacted due to switching to a new spatial pattern, considering channel and interference measurements.

## Enhancements on beam management

### Impact on TCI state indication/update due to spatial adaptation

Different spatial patterns may have different characteristics e.g., in terms of beam pattern. Specifically, as explained earlier, when the number of spatial elements is reduced, the beam pattern from lower number of spatial elements may be wider due to low spatial resolution. It can thus be observed that spatial adaptation may impact applicable TCI states.

Based on legacy procedures, either Rel-15 TCI framework or Rel-17 unified TCI framework, essentially up to 8 TCI states are activated in MAC CE and 1 (DL) TCI state could be indicated in DCI. This up to 8 active TCI states are essentially used to represent different beam directions, or equivalently, quasi-colocation information (such as QCL type A and type D). However, given that there is potentially a relationship between an applicable spatial pattern and applicable TCI states, there may be need to discuss whether a frequent update of active TCI states would be required to follow the spatial pattern change/adaptation; if it turned out to be necessary, such update would then incur latency and overhead and thus may not preferrable.

Observation 25: Spatial pattern adaptation may impact and require updating at least active TCI states.

Proposal 14: Discuss whether the existing TCI state indication procedures should be enhanced when considering spatial pattern adaptation.

### Beam failure related procedures

Another aspect that would require discussions is how the spatial adaptation would impact the beam failure related procedures. Specifically, such impact may be in terms of beam failure detection and/or beam recovery. Otherwise, a restriction could be added that RSs (reference signals) used for beam failure detection and recovery should not be impacted by spatial adaptation.

Although it may result in reducing the network energy saving opportunities, such a restriction could be used at least as fallback option for the related RAN1 discussions – in case the impact of spatial adaptation on legacy beam failure related procedures is seen to be somewhat big.

Proposal 15: Discuss how/whether spatial adaption impacts beam failure detection and beam recovery procedures.

## Signalling aspects

### Spatial pattern change indication need and content

In the previous sections, we discussed some of the enablers for spatial adaptation. One additional important enabler is on how/whether the spatial pattern change/adaptation should be signalled to a UE. It’s worth noting that the discussions in RAN1#112 touched upon this aspect, as can be noticed from the following related agreement:

Agreement: Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that - Whether there is a need for transition time per adaptation (for UE) - Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

Considering the previous discussions on different Types of spatial adaptation (namely, Type 1 and Type 2, which are discussed from a single antenna port perspective), we recall that two different spatial patterns may differ in at least one of the following spatial elements:

- subset/set of (active) antenna ports

- subset/set of active (or muted) antenna elements or TxRUs for one or more antenna ports

What to signal to the UE for spatial adaptation would depend on:

- whether to only consider signalling to enable spatial patterns evaluation and related CSI reporting, or

- whether spatial pattern change/update for PDSCH/CSI-RS would also need to be informed to the UE. In general, depending e.g., on how a spatial pattern change would impact several operations such as TCI state operation, it seems beneficial to signal the UE such information so that it adapts its reception for PDSCH (and CSI-RS) accordingly.

In the following, we provide some additional observations regarding the spatial adaptation signalling.

- In case spatial adaptation could impact CSI-RS in general, signalling a new spatial pattern may consist in signalling an update of parameters for (active) CSI-RS configurations, such as in terms of subset of antenna ports. Alternatively, the signalling of a new spatial pattern may consist in providing corresponding (active) antenna port subset without necessarily binding it to (active) CSI-RS configurations, and the UE would then determine how/whether this new spatial pattern, and thus new subset of antenna ports, impacts these CSI-RS configurations – such as in terms of active (and muted) antenna ports.

- Similar study should be conducted when spatial adaptation consists of switching from one spatial pattern to a new/different spatial pattern and these patterns have different number of active TxRUs corresponding to one or more antenna ports that are common between the two patterns – including the case where the two patterns have the same antenna port subset. When an antenna port is ‘common’ between the previous spatial pattern and the new spatial pattern, the number of (active) TxRUs or antenna elements corresponding to this port may or may not change between the two patterns.

  - It’s worth noting that such an antenna port can be for NZP CSI-RS resource(s) used for channel measurement or for interference measurement. Whether the number of TxRUs of a common port has changed or not could be leveraged in measurement averaging and computation from multiple RS resources/occurrences.

Observation 26: It seems beneficial to signal the UE(s) the spatial pattern change so that it adapts its reception of PDSCH/CSI-RS accordingly.

Observation 27: When an antenna port is ‘common’, or still active, between a previous spatial pattern and a new applicable spatial pattern, the number of (active) antenna elements (or TxRUs) corresponding to this port may or may not change between the two spatial patterns.

Proposal 16: Support signalling of spatial pattern change to the UE.

- Discuss signalling content of spatial adaptation, considering that different spatial patterns may differ in at least one of the following spatial elements:

- Set of antenna ports,

- Set/number of active (or muted) antenna elements or TxRUs for one or more antenna ports.

### Signalling means for spatial pattern change indication

In addition to the signalling content, signalling means/ways to inform a UE about applicable spatial pattern(s) or about spatial pattern change needs to be studied.

We foresee the following options on how to signal/configure the UE the spatial adaptation/change:

- Option 1: Use DCI, including group common DCI if seen beneficial, to indicate the UE(s) spatial pattern change/adaptation.

  - Here, it should be discussed whether/how existing operation(s) could be leveraged in such a way to also carry the indication of spatial pattern change. For instance, one could leverage existing DCI-based activation of ZP-CSI-RS for indicating the change of spatial pattern.

- Option 2: Use MAC CE to indicate the UE spatial pattern change/adaptation.

- Option 3: Rely on semi-static or even semi-dynamic configuration and operation, i.e., via RRC or MAC CE, for switching between various spatial patterns over different period of times, i.e., spatial partitions in time. And use dynamic signaling, via DCI or MAC CE, to update such configuration as needed.

Option 3 has the advantage of reduced signaling overhead as, with this option, there is no need to provide indication each time the gNB is switching from one spatial pattern to another, and a frequent update of the configuration of spatial partitions (in time) is not expected. However, Option 3 provides occasions (in time) for a spatial pattern even when the gNB doesn’t necessarily want/prefer to use that pattern. On the other side, Option 1 would provide a faster spatial adaptation compared to Option 2 (and Option 3, if the spatial pattern to switch to is not part of the configuration/ spatial partitions), whereas Option 2 would introduce non-negligible transient periods (e.g., due to the acknowledgment for the PDSCH carrying the MAC CE). On the other hand, Option 1 is more prone to errors (i.e., missing DCIs) compared to Option 2. Depending on whether signaling for an existing operation(s) could be leveraged, such as for ZP-CSI-RS related operation, Option 1 may also have reduced signaling overhead.

Overall, it seems that Options 1 and 2 would require less specifications effort than Option 3.

Proposal 17: Discuss signalling ways for spatial adaptation, considering the following options as a baseline:

- Option 1: Use DCI, including group common DCI if seen beneficial, to indicate the UE(s) a spatial pattern change/adaptation.

  - This option could include leveraging signaling for existing operation(s) if feasible/possible.

- Option 2: Use MAC CE to indicate the UE(s) a spatial pattern change/adaptation.

- Option 3: Use semi-static or even semi-dynamic configuration and operation, i.e., via RRC or MAC CE, for switching between various spatial patterns over different period of times, i.e., spatial partitions in time. And use dynamic signaling, via DCI or MAC CE, to update such configuration.

## Transition time for spatial pattern adaptation

In RAN1#112, one transition time aspect was captured in the following agreement:

Agreement: Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that - Whether there is a need for transition time per adaptation (for UE) - Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

Figure 4: Illustration of transition time for one example of spatial pattern adaptation.

Regarding the transition time for spatial pattern change, there are two main cases to consider:

- Case 1 – CSI-RS transmissions to enable spatial adaptation: depending on which option(s) among the various options discussed earlier in this contribution, there may be a need for a transition time between two (consecutive) CSI-RS resources transmitted using different spatial patterns – such as in the case of Options 1-1 and 2-1 (as different CSI-RS resources correspond to different spatial patterns).

- Case 2 – spatial adaptation for PDSCH: when the gNB decides to adapt the spatial pattern for PDSCH transmissions, there would be a need for a transition time to switch from one spatial pattern to another.

One aspect to discuss is the assumption to make regarding the impact on gNB transmissions during the transition time. In our view, there would be no transmissions possible during this time, at least considering the spatial elements being impacted by the adaptation.

Another important aspect, be it for Case 1 or Case 2 above, is whether the gNB could accommodate the transition time for spatial adaptation based on gNB implementation. This could be feasible potentially depending on the range/values to assume for the required transition time. The transition time range would depend on the sleep state the gNB is transitioning from/into, and it might also depend on whether analog or hybrid beamforming is considered etc.

If a somewhat large transition time is required, it should be discussed whether specifications would need to accommodate for the transition time so that its impact on the transmission (and reception) of signals/channels, and related procedures, could be alleviated. In general, even for such cases, the gNB should be able to reduce such impact as much as possible still based on implementation. Also, introducing and specifying a transition time would potentially require noticeable specifications efforts. Further, for CSI-RS to enable spatial adaptation, an option along the lines of Option 1-2 would allow to overcome the need for a transition time, at least in some cases.

Observation 28: There would be no transmissions of (DL) signal/channels possible during a transition time for spatial adaptation, at least considering the spatial elements being impacted by the adaptation.

Observation 29: The transition time range for spatial adaptation would depend on the sleep state the gNB is transitioning from/into, and it might also depend on whether analog or hybrid beamforming is considered.

Observation 30: Even for the cases where a somewhat large transition time is required for spatial adaptation, the gNB should be able to reduce the impact of such transition time as much as possible still based on implementation. Also, introducing and specifying a transition time would potentially require noticeable specifications efforts.

Proposal 18: The impact of transition time for spatial adaptation should be accounted for when deciding on the option(s) (regarding resource configuration discussed above) to adopt to enable the evaluation of multiple spatial patterns.

Proposal 19: Discuss the need to specify transition time for spatial adaptation taking into account the different (sleep) states the gNB could be transitioning from/into.

# Techniques in Power domain

## Enhancements on CSI-related procedures

In legacy NR, the transmit power of SSB is semi-statically configured via ss-PBCH-BlockPower in SIB1 and the transmit power offset (powercontrolOffsetSS) is semi-statically configured to indicate the transmission power of CSI-RS relative to the transmit power of SSB. Based on the received power of CSI-RS, the UE derives the CSI feedback for PDSCH transmission, which accounts also for the semi-statically configured power offset (powerControlOffset) between PDSCH and CSI-RS transmissions. Note also that the power offset configurations for PDSCH and CSI-RS are BWP-specific, and each NZP-CSI-RS resource is configured with one power offset setting between CSI-RS and PDSCH. In the RAN1#112 meeting enhancements to the adaptation of power offset values between PDSCH and CSI-RS were discussed, and the following was agreed:

Agreement: For adaptation of power offset values between PDSCH and CSI-RS, further study the following - Where/how to configure multiple power offset values - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS - Whether other UE report content can be included

Regarding the question where (or whether)/how to configure multiple power offset values, as pointed out above, each NZP-CSI-RS resource is currently configured with one power offset setting between CSI-RS and PDSCH. According to the current specification if CSI reports based on multiple hypothetical power offset values are needed to determine how much PDSCH power can be reduced, multiple NZP-CSI-RS resources with different powerControlOffset value needs to be configured. The number of NZP-CSI-RS resources could then be large. To our view, the NZP-CSI-RS resource configuration should be enhanced so that the configuration of multiple powerControlOffset values within a single NZP-CSI-RS resource is supported.

The CSI report based on NZP-CSI-RS configuration that includes multiple powerControlOffset values could be relatively large. On the other hand, from the overhead point of view single report is more efficient than multiple reports if CSI reports based on all the offset values are anyway needed. One way to reduce the size of the CSI reports is to dynamically indicate to UE one or more power offset values to be used for CSI measurement and reporting. But because gNB may not know upfront which power control offsets to use, it may need to try multiple values and spend additional energy for L1/L2 signalling. Dynamic indication would also mean bigger changes to the specifications. Therefore, in our view the benefits of such dynamic indication for CSI measurement/reporting are unclear, and specification of the method is not worth the effort.

Instead of gNB asking UE to measure and report CSI based on different offset values, UE could determine and indicate to gNB, if and how much PDSCH power could be reduced without adverse effects to bit rate and BLER. UE could indicate for example, how much PDSCH power can be reduced and still maintain the rank and/or MCS that is achievable with the power offset value included in the NZP-CSI-RS configuration. Instead of or in addition to CSI reports based on multiple powerControlOffset values, the CSI report could be enhanced by adding information about how much power can be reduced to the report.

Proposal 20: Configuration of multiple powerControlOffset values within a single NZP-CSI-RS resource is supported.

Proposal 21: CSI report based on NZP-CSI-RS resource containing multiple power offset values is specified

- FFS how the size of the report is reduced

Proposal 22: CSI reporting is enhanced by adding information about how much PDSCH power can be reduced and still maintain the same rank and/or MCS that is achievable with the powerControlOffset value included in the NZP-CSI-RS configuration.

## Indication of PDSCH power change

In the RAN1#112 meeting it was agreed to further discuss on the need of a dynamic indication of the PDSCH power change to the UE for assisting its PDSCH reception (see the yellow highlights):

Agreement: Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that - Whether there is a need for transition time per adaptation (for UE) - Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

Agreement: For adaptation of power offset values between PDSCH and CSI-RS, further study the following - Where/how to configure multiple power offset values - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS - Whether other UE report content can be included

As discussed above in section 2.3, indication of spatial pattern change would be beneficial. In this section, we discuss about the PDSCH power change indication.

Currently the UE employs already means to cope with relatively large changes in the received power level e.g. because of fading. However, the dynamic PDSCH power adaptations in Rel.18 may introduce fast and abrupt power changes, which the UE may not be able to cope with if the UE is not aware of them. This is explained for example in ​[3]. In fact, fast and abrupt power changes may result in the ADC to operate in sub-optimal regimes (e.g., UE’s AGC setting may not be optimal, and saturation may occur if a sudden increase of transmit power is applied). Therefore, when the network applies dynamic changes of the power of the PDSCH transmissions, the UE’s receiver might benefit from receiving some information about the power change in order to optimize its processing.

Observation 31: The UE might benefit from receiving some information about the PDSCH power change in order to optimize its processing related to PDSCH reception.

The network could in principle send a transmission power change indication every time the power level is changed. However, such an indication would increase the signalling overhead and, in turn, the energy consumption, which is undesired. Therefore, it may be beneficial to minimize the provided indications from the network to the UE only to those cases where it is beneficial/needed by the UE. The cases where the indication is beneficial/necessary for the UE may include cases when the power change is large (high power change range) and/or too fast (high power change rate). Such indications would allow the network to optimize power adaptations flexibly for increased energy saving, while enabling the UE to optimize its processing properly and cope with the network adaptations.

Observation 32: It is beneficial to minimize (signalling and energy consumption of) the PDSCH power change indications by providing the indication only in those cases where it is needed for the UE processing, such as if the power change rate is high and/or the power change is large.

As the need and benefit from such indications may differ for different UEs, it may be considered whether the UE could indicate when it benefits from the indication (e.g., by indicating its need, or by indicating a power change rate or range which the UE can/cannot cope with).

Proposal 23: Define PDSCH transmission change indication limited to cases where it is beneficial for the UE.

- FFS Discuss in which cases the indication is beneficial to the UE (e.g., if power change rate is high and/or power change is large)

- FFS Discuss whether the UE should provide information related to when it benefits from the indication (e.g., by indicating its need, or by indicating a power change rate or range which the UE can/cannot cope with).

# Conclusions

In this contribution, we have the following observations and proposals:

On the techniques in spatial domain:

Observation 1: Given Type 1 and Type 2 adaptation, which relate to the mapping between antenna ports and TxRUs, it can be observed that different spatial patterns may have:

- different sets of antenna ports, or

- same set of antenna ports but different set of active antenna elements.

Proposal 1: Clarify the exact UE CSI/CSI-RS capabilities covered in the following Note (captured in the WI description):

- “Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements.”

- Without further clarifications on this Note, all the legacy capabilities related to total number of CSI reports and requirements would need to be assumed.

Observation 2: If the number of spatial patterns is large, this would result in the need for a large number of CSI report configurations, but this then violates the legacy capabilities – especially that some CSI report configurations would be needed for other purposes than spatial pattern evaluation.

Observation 3: If only a few spatial patterns are considered, A2-1 (i.e., independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern) might be enough and then no enhancements would be needed on the legacy operation.

Proposal 2: If a large number of spatial patterns is considered, an option along the lines of A2-2 could be considered where one CSI report configuration is used to evaluate multiple spatial adaptation patterns.

Observation 4: Compared to Option 1-1 (where different spatial patterns are evaluated using different CSI-RS resources), with Option 1-2, where one CSI-RS resource is used to evaluate multiple spatial patterns, DL overhead could be potentially saved; this in turn allows more network energy saving gains.

Observation 5: One case where Option 1-2 (where one CSI-RS resource is used to evaluate multiple spatial patterns) may not work is when considering Type 2 adaptation where the different spatial patterns to evaluate have the same set/subset of antenna ports but have difference in terms of active, or alternatively muted, antenna elements. Instead, such case could be feasible e.g., with Option 1-1 where different spatial patterns are evaluated using different CSI-RS resources.

Observation 6: Option 1-2 (where one CSI-RS resource is used to evaluate multiple spatial patterns) seems to be primarily aimed at FR1 and considering non-precoded CSI-RS. Whether this option would work or for all cases and scenarios of interest can be discussed.

Observation 7: With Option 1-2 (where one CSI-RS resource is used to evaluate multiple spatial patterns), the UE would need to be signalled by the gNB at least the subsets of antenna ports that the UE needs to evaluate considering one CSI-RS resource.

Observation 8: Under Option 2-1, where one CSI-RS resource set is associated to one spatial pattern, several CSI-RS resource sets associated to a same CSI report configuration would be needed, and this may result in the need to lift the existing constraint/limitation on the maximum number of CSI-RS resource sets (e.g., for channel measurement) that could be associated to a CSI report configuration.

Observation 9: Under Option 2-2, where one CSI-RS resource set is associated to / used to evaluate multiple spatial patterns, the existing limitation that CSI report is only possible considering same nrofPorts for the CSI-RS resources of a CSI-RS resource set associated with a CSI report configuration would need to be lifted.

Observation 10: Option 2-2 (where one CSI-RS resource set is associated / used to evaluate multiple spatial patterns) could cover Option 1-2 (where one CSI-RS resource is associated / used to evaluate multiple spatial patterns. In addition, Option 2-2 could also cover Option 1-1 (where one CSI-RS resource set is associated to one spatial pattern).

Observation 11: Option 3, where resource setting level is considered, is similar to Option 2 (where a CSI-RS resource set level is considered), so in a way similar observations as for Option 2 can be made here. For instance, Option 3-2 (where one resource setting is associated to /used to evaluate multiple spatial patterns) could cover Options 1-2 and Option 1-1.

Observation 12: Option 3, where resource setting level is considered, has the advantage of covering resources for both channel and interference measurements.

Observation 13: Option 3-2 (one resource setting is associated to /used to evaluate multiple spatial patterns) would consume less resource settings compared to Option 3-1 (one resource setting is associated to one spatial pattern).

Observation 14: Under Option 3-1, where one resource setting is associated to one spatial pattern, for periodic/semi-persistent resource settings it’s required that multiple resource settings are associated to a CSI report configuration.

Proposal 3: To enable CSI assistance information for spatial adaptation, further consider Option 1-2 and Option 2-2:

- Option 1-2: one CSI-RS resource is associated to / used to evaluate multiple spatial patterns.

- Option 2-2: one CSI-RS resource set is associated to / used to evaluate multiple spatial patterns.

- FFS: whether these options can be used for all cases and scenarios of interest, and if not, which other option(s) should be adopted.

- FFS: how to indicate the UE with the antenna port subsets which represents different spatial patterns for evaluation/reporting under these options.

  - FFS: whether other information should be indicated/updated to the UE.

Proposal 4: To enable CSI assistance information for spatial adaptation, further consider Option 3-2:

- Option 3-2: one resource setting is associated to /used to evaluate multiple spatial patterns.

- FFS: whether this option can be used for all cases and scenarios of interest.

Observation 15: Additional CSI-RS resource patterns do not seem needed and would anyhow potentially lead to large specs effort. Instead, we should focus on ways that leverage existing CSI-RS patterns to enable the spatial adaptation.

Proposal 5: Do not introduce additional CSI-RS resource patterns for the purpose of spatial adaptation.

Observation 16: When CSI feedback, including PMI, needs to be derived for a certain spatial pattern, in general the UE would need to have a correct assumption regarding the corresponding spatial configuration, such as codebook configuration including codebook subset restriction.

Proposal 6: For evaluating a spatial pattern, discuss how to enable the UE to determine the spatial configuration, such as codebook configuration (including codebook subset restriction), corresponding to a spatial pattern.

Observation 17: When the number of spatial elements (such as antenna ports or TxRUs) is reduced, the beam pattern from lower (resp., larger) number of spatial elements may be wider (resp. narrower) due to low (resp., high) spatial resolution. A wider beam may increase the number of multipaths, which then results in higher delay spread; a tighter beam may decrease the number of multipaths which then results in lower delay spread. High delay spread is reflected as frequency selectivity in channel.

Proposal 7: Discuss the implications of different spatial patterns potentially having different channel characteristics in terms of frequency selectivity on the design of CSI report configuration, specifically from sub-band configuration perspective.

Observation 18: Relying on the approach where the UE is (always) required to provide CSI report(s) corresponding to all or several of configured spatial patterns would result in large UL control overhead, especially when the number of spatial patterns is large.

Observation 19: It may not be necessary for the UE to provide CSI report corresponding to all candidate spatial patterns, and it could be sufficient to have UE reporting based on a selection of one or a couple of spatial patterns. This would then allow keeping the UL control overhead low. At the same time, such an approach may require more specs effort.

Observation 20: For spatial pattern selection at the UE, there may be a need to differentiate between the case where the set of candidate patterns (the UE is selecting from) have same and the case where they have different number of (active) spatial/antenna elements.

Proposal 8: To minimize the CSI feedback overhead, discuss on the CSI report content, i.e., whether/how this report would be based on a subset selection from multiple spatial patterns by the UE.

Proposal 9: Discuss whether there is a need to differentiate the CSI report content depending on whether the spatial patterns have same or different number of spatial/antenna elements.

Proposal 10: Discuss CSI report feedback size reduction considering sub-band configuration adaptation to each spatial pattern.

Observation 21: Evaluations/measurements of all (indicated) candidate spatial patterns increases the UE burden and power consumption.

Proposal 11: In the case where the UE is required to provide CSI report based on multiple (indicated) candidate spatial patterns, to reduce the burden at the UE, discuss whether/how the UE is allowed to skip evaluating some of the candidate spatial patterns based on some rules.

Observation 22: Evaluating multiple spatial patterns and providing a corresponding CSI report(s) using a single UL (PUCCH/PUSCH) reporting occasion would increase the consumption of parallel CSI processing units required at the UE.

Proposal 12: Discuss how to configure CSI measurements and reports for different spatial patterns in time, considering different reporting types (semi-persistent, periodic, aperiodic).

Observation 23: If spatial adaptation is not allowed to impact at least some of the CSI-RSs (other than the ones used for spatial patterns evaluation), less network energy savings will be achieved and the gNB wouldn’t be able to e.g., multiplex CSI-RS and PDSCH in the frequency domain (at least in some cases).

Observation 24: Spatial adaptation may impact CSI computation/derivation, and this includes channel measurements and interference measurements.

Proposal 13: Discuss how the CSI computation/derivation operation is impacted due to switching to a new spatial pattern, considering channel and interference measurements.

Observation 25: Spatial pattern adaptation may impact and require updating at least active TCI states.

Proposal 14: Discuss whether the existing TCI state indication procedures should be enhanced when considering spatial pattern adaptation.

Proposal 15: Discuss how/whether spatial adaption impacts beam failure detection and beam recovery procedures.

Observation 26: It seems beneficial to signal the UE(s) the spatial pattern change so that it adapts its reception of PDSCH/CSI-RS accordingly.

Observation 27: When an antenna port is ‘common’, or still active, between a previous spatial pattern and a new applicable spatial pattern, the number of (active) antenna elements (or TxRUs) corresponding to this port may or may not change between the two spatial patterns.

Proposal 16: Support signalling of spatial pattern change to the UE.

- Discuss signalling content of spatial adaptation, considering that different spatial patterns may differ in at least one of the following spatial elements:

- Set of antenna ports,

- Set/number of active (or muted) antenna elements or TxRUs for one or more antenna ports.

Proposal 17: Discuss signalling ways for spatial adaptation, considering the following options as a baseline:

- Option 1: Use DCI, including group common DCI if seen beneficial, to indicate the UE(s) a spatial pattern change/adaptation.

  - This option could include leveraging signaling for existing operation(s) if feasible/possible.

- Option 2: Use MAC CE to indicate the UE(s) a spatial pattern change/adaptation.

- Option 3: Use semi-static or even semi-dynamic configuration and operation, i.e., via RRC or MAC CE, for switching between various spatial patterns over different period of times, i.e., spatial partitions in time. And use dynamic signaling, via DCI or MAC CE, to update such configuration.

Observation 28: There would be no transmissions of (DL) signal/channels possible during a transition time for spatial adaptation, at least considering the spatial elements being impacted by the adaptation.

Observation 29: The transition time range for spatial adaptation would depend on the sleep state the gNB is transitioning from/into, and it might also depend on whether analog or hybrid beamforming is considered.

Observation 30: Even for the cases where a somewhat large transition time is required for spatial adaptation, the gNB should be able to reduce the impact of such transition time as much as possible still based on implementation. Also, introducing and specifying a transition time would potentially require noticeable specifications efforts.

Proposal 18: The impact of transition time for spatial adaptation should be accounted for when deciding on the option(s) (regarding resource configuration discussed above) to adopt to enable the evaluation of multiple spatial patterns.

Proposal 19: Discuss the need to specify transition time for spatial adaptation taking into account the different (sleep) states the gNB could be transitioning from/into.

On the techniques in power domain:

Proposal 20: Configuration of multiple powerControlOffset values within a single NZP-CSI-RS resource is supported.

Proposal 21: CSI report based on NZP-CSI-RS resource containing multiple power offset values is specified

- FFS how the size of the report is reduced

Proposal 22: CSI reporting is enhanced by adding information about how much PDSCH power can be reduced and still maintain the same rank and/or MCS that is achievable with the powerControlOffset value included in the NZP-CSI-RS configuration.

Observation 31: The UE might benefit from receiving some information about the PDSCH power change in order to optimize its processing related to PDSCH reception.

Observation 32: It is beneficial to minimize (signalling and energy consumption of) the PDSCH power change indications by providing the indication only in those cases where it is needed for the UE processing, such as if the power change rate is high and/or the power change is large.

Proposal 23: Define PDSCH transmission change indication limited to cases where it is beneficial for the UE.

- FFS Discuss in which cases the indication is beneficial to the UE (e.g., if power change rate is high and/or power change is large)

- FFS Discuss whether the UE should provide information related to when it benefits from the indication (e.g., by indicating its need, or by indicating a power change rate or range which the UE can/cannot cope with).

# References

- R1-2302394, “Enhancements on cell DTX DRX mechanism”, Nokia, Nokia Shanghai Bell, RAN1#112bis-e, April 17th – 26th, 2023.

- R1-2211098, “Network Energy Saving Techniques”, Nokia, Nokia Shanghai Bell, RAN1#111, Nov. 14th-18th, 2022.

- R1-2301425, “Techniques in spatial and power domains”, Qualcomm, RAN1#112, Feb. 27th- Mar. 3rd, 2023.