3GPP TSG RAN WG1 Meeting #112bis-e	R1-2302394

E-meeting, April 17th – 26th , 2023

Agenda item:		9.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:		Enhancements on cell DTX/DRX Mechanism

Document for:		Discussion and Decision

# Introduction

Based on the WI description in RP-223540, the following objective has been agreed for network energy saving with enhancements on cell DTX/DRX:

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

Based on the previous RAN1#112 meeting, the following agreements were made.

Agreement: - RAN1 continues discussion on the at least following physical layer related aspects of cell DTX/DRX aspects - physical layer signals/channels and procedure expected to be impacted during non-active periods of cell DTX/DRX - consider impact to at least KPIs from the SI when physical layers/signals/channels are impacted by cell DTX/DRX - Further discussions on other aspects are not precluded Agreement: At least the following candidate signals/channels for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, are considered from RAN1 perspective for further discussion. The exact set of signals/channels that the UE may be expected to not transmit or receive is FFS.​ - DL - Periodic/Semi-persistent CSI-RS (including TRS) - PRS - PDCCH scrambled with UE specific RNTI - PDCCH in Type-3 CSS - SPS-PDSCH - UL - SR - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - CG-PUSCH Other signals/channels are not precluded

Based on the previous RAN2 #121 meeting, the following agreements were made.

Agreement: - There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs - Rel-18 NES capable CONNECTED UE(s) can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX (i.e., same behavior for cell DTX and cell DRX). No further enhancements for CBRA and CFRA will be pursued. - Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell. FFS whether we have DTX UE specific inactivity timer. FFS on configuration signaling and stage 3. - Confirm study item agreement that we can have separate DTX and DRX configuration. We will focus on designing DTX/DRX for at least single configuration. FFS whether multiple configurations of cell DTX or DRX will be supported.

In this contribution, we will discuss the issues related to the enhancement on cell DTX/DRX. And in our companion contribution, we will discuss the issues on techniques in spatial and power domains [1].

# Enhancements on cell DTX/DRX Mechanism

According to RAN2 discussions, a cell DTX/DRX pattern can be defined by transitions between active periods (during which NW can perform transmissions and receptions) and non-active periods (during which the transmissions/receptions are partially or completely turned off for data traffic and/or reference signal).

## Cell DTX/DRX configuration, activation and deactivation

Based on RAN2 #121 meeting outcome, the pattern configuration for cell DTX/DRX is commonly applied for all UEs in a cell, where the cell DTX and cell DRX pattern can be separately configured. And RAN2 will further discuss on whether there can have DTX UE specific inactivity timer and the support of multiple configurations of cell DTX or DRX.

Moreover, RAN2 has discussed the following open issues as captured in RAN2 [POST121][312][NES] DTX/DRX - Configuration/ activation/ deactivation and alignment (Huawei) without consensus yet:

- Methods of configuring Cell DTX/DRX (not including joint/separate configuration and single/multiple configuration, as they have already been discussed and progressed online)

- Methods of activating and deactivating of Cell DTX/DRX

- Alignment between Cell DTX/DRX and UE C-DRX

Proposal 1: Wait for RAN2 agreements on Cell DTX/DRX configuration/ activation/ deactivation.

Observation 1: The Alignment between Cell DTX/DRX and UE C-DRX has no RAN1 impact, and it ups to RAN2 discussions and decision.

## Analysis of cell DTX/DRX impact on UE’s behaviour

According to the WID, there are design restrictions with the following two notes:

- Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

And based on RAN2 #121 meeting outcome, there will be no impact to the RACH, paging, and SIB1 transmissions and corresponding procedures, where the RRC_IDLE/INACTIVE UEs may rely on. Moreover, it has been agreed that, for the Rel18 NES capable RRC_CONNECTED UE(s), it can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX, and no further enhancement for CBRA and CFRA.

And based on RAN1#112 meeting outcome, a list of candidate signals/channel(s) for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, is considered for further discussion. But the exact set of signals/channels has not been agreed yet. In the following, we will first discuss the potential impacts if the candidate set of signals/channel(s) are not transmitted during the cell DTX/DRX non-active period.

### Analysis of cell DTX impact

Based on the list of candidate DL physical channels and signals as agreed in RAN1#112 meeting, we further extend the list of candidates as follows by separating the CSI-RSs according to their different purposes. We will use the following list as a baseline for more detailed discussion.

- Periodic/Semi- persistent CSI-RS for L1/L3-RSRP

- Periodic/Semi- persistent CSI-RS for Radio Link Management (RLM)/beam Failure detection (BFD)

- Periodic/Semi-persistent CSI-RS for tracking (TRS)

- PRS

- PDCCH scrambled with UE specific RNTI

- PDCCH in Type-3 CSS

- SPS-PDSCH

Note that, for aperiodic CSI-RS transmission, it is expected that the network scheduling could dynamically avoid the aperiodic CSI-RS transmissions during cell DTX non-active period, and only periodic/semi-persistent CSI-RS transmission are to be discussed. Similarly for DG-PDSCH, the NW could avoid dynamic PDSCH assignments during cell DTX non-active period and only SPS-PDSCH transmission is to be discussed.

Proposal 2: We propose the following extended list of DL channels/signals to be used as a baseline for more detailed discussion.

- Periodic/Semi-persistent CSI-RS for L1/L3-RSRP

- Periodic/Semi-persistent CSI-RS for Radio Link Management (RLM)/beam Failure detection (BFD)

- Periodic/Semi-persistent CSI-RS for tracking (TRS)

- PRS

- PDCCH scrambled with UE specific RNTI

- PDCCH in Type-3 CSS

- SPS-PDSCH

In Table 1, we provided our views on the impact of cell DTX to RRC_CONNECTED UEs if given DL channel/signal is not transmitted during the non-active period.

Table 1: Cell DTX impact to RRC_CONNECTED UEs

| Signal | NW behaviour during non-active period | Impact to RRC_CONNECTED UEs |
| --- | --- | --- |
| Periodic/Semi- persistent CSI-RS for L1/L3-RSRP | If corresponding CSI-RS Tx is dropped | - If UE’s CDRX on-duration period falls in the cell DTX non-active period, the measurements samples can’t be taken, reporting can’t be done, and this may lead to beam/radio link failure. |
| Periodic/Semi- persistent CSI-RS for RLM/BFD | If corresponding CSI-RS Tx is dropped | - If UE’s CDRX on-duration period falls in the cell DTX non-active period, the measurements samples can’t be taken, and this may impact RLM/BFD procedures. |
| Periodic/Semi- persistent CSI for tracking (TRS) | If corresponding Tx is dropped | - If the UE’s CDRX on-duration period falls in the cell DTX non-active period, the measurements samples can’t be taken and the UE would not be able to perform T/F tracking based on TRS which may impact PDCCH/PDSCH reception performance, if allowed. |
| PRS | If corresponding Tx is dropped | - If the UE’s CDRX on-duration period falls in the cell DTX non-active period, the measurements samples can’t be taken, and this may impact positioning procedure. |
| PDCCH scrambled with UE specific RNTI | Under email discussion in RAN2 with no agreements yet | As per RAN2 [POST121][311][NES] DTX/DRX - gNB and UE behaviours]: For dynamic new transmissions: - Proposal 5a: UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. (20/24) - RAN2 Proposal 5b: The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time (21/24) For dynamic retransmissions: - Proposal 6a: RAN2 to discuss the following options for UE behaviour for PDCCH monitoring for dynamic retransmissions during cell DTX non-active period: - UE doesn’t monitor PDCCH for dynamic grants/assignments for dynamic retransmissions during Cell DTX non-active, even if the UE is in C-DRX Active time. (13/23) - UE monitor PDCCH for dynamic grants/assignments for retransmissions during the UE’s C-DRX Active time per legacy behaviour, even during the Cell DTX non-active period. (7/23) - Option 1 for retransmission of dynamically scheduled TBs, FFS for retransmission of CG or SPS (2/23) - Proposal 6b: RAN2 to discuss the following options for the understanding for the gNB scheduling behaviour for dynamic retransmissions during Cell DTX non-active period: - gNB does not schedule UE-specific dynamic grants/assignments for retransmissions during cell DTX non-active periods, even if the UE is in C-DRX Active Time (13/23) - gNB can schedule UE-specific dynamic grants/assignments for retransmissions during cell DTX non-active periods, but not outside of the UE’s C-DRX Active time. (6/23) - Option 1 for retransmission of dynamic scheduling, FFS for retransmission of CG or SPS (2/23). |
| PDCCH in Type-3 CSS | If corresponding Tx is dropped | - The UE will not receive indication for the following procedures, e.g., pre-emption in DL, for slot format on the given slot, power control on PUCCH/PUSCH and SRS trigger and power control, cancellation in the uplink. |
| SPS-PDSCH | Under email discussion in RAN2 with no agreements yet | As per RAN2 [POST121][311][NES] DTX/DRX - gNB and UE behaviours]: - Proposal 1: As baseline, UE drops monitoring SPS occasions during Cell DTX non-active period. gNB is assumed to be not transmitting PDSCH on such SPS occasions during the Cell DTX non-active period. (20/24) FFS: whether it is possible to configure an exception to this (e.g. per SPS or cell DTX configuration) such that the UE monitors SPS occasions during Cell DTX non-active period to support low latency traffic. |

Proposal 3: Discuss whether all types of periodic/semi-persistent CSI-RS and PRS transmission shall be dropped during cell DTX non-active period.

- FFS on the potential solutions to overcome the impact from the dropped transmissions due to cell DTX non-active period.

Proposal 4: Wait for RAN2 agreements on the gNB/UE behaviour for PDCCH and SPS-PDSCH (re)-transmission during the Cell DTX non-active period.

- FFS on potential RAN1 impacts.

Apart from the impact analysis of cell DTX to RRC_CONNECTED UEs, the potential impact to RRC_INACTIVE/IDLE UEs should be avoided as per the WID. Thus, this is left to NW implementation.

Proposal 5: Confirm that RRC_INACTIVE/IDLE UEs should not be impacted by cell DTX. It is up to NW implementation to avoid any potential impact(s).

### Analysis of cell DRX impact

Based on the list of candidate UL physical channels and signals as agreed in RAN1#112 meeting, we further extend the list of candidates as follows by adding HARQ feedback for SPS/DG-PDSCH. We will use the following list as a baseline for more detailed discussion.

- SR

- Periodic/Semi-persistent CSI report

- Periodic/Semi-persistent SRS

- HARQ feedback for SPS/DG-PDSCH

- CG-PUSCH

Note that, it is expected that by implementation the network scheduling could dynamically avoid the aperiodic CSI-RS reports scheduling during cell DRX non-active period, and only periodic/semi-persistent CSI-RS reports are to be discussed. Similarly for DG-PUSCH, the NW could avoid PUSCH dynamic grants during the cell DRX non-active period, and only CG-PUSCH reception is discussed.

Proposal 6: We propose the following list of UL channels/signals to be used as a baseline for more detailed discussion.

- SR

- Periodic/Semi-persistent CSI report

- Periodic/Semi-persistent SRS

- HARQ feedback for SPS/DG-PDSCH

- CG-PUSCH

We analyse in Table 2 the cell DRX impact to RRC_CONNECTED UEs if given UL channel/signal is not received during the non-active period.

Table 2: Cell DRX impact to RRC_CONNECTED UEs

| Signal | NW behaviour during non-active period | Impact to RRC_CONNECTED UEs |
| --- | --- | --- |
| SR | Under email discussion in RAN2 with no agreements yet | As per RAN2 [POST121][311][NES] DTX/DRX - gNB and UE behaviours]): - Proposal 3: As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period (17/24). FFS: whether it is possible to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to support low latency traffic. - Proposal 4: If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH. (21/24). |
| Periodic/Semi-persistent CSI report | If corresponding Rx is omitted | - If the UE’s CDRX on-duration period falls in the cell DRX non-active period, the UEs are not able to send CSI reports which will be discarded. |
| Periodic/Semi-persistent SRS | If corresponding Rx is omitted | - If the UE’s CDRX on-duration period falls in the cell DRX non-active period, the UEs will not send SRS. |
| HARQ feedback for SPS/DG-PDSCH | Depends on RAN2 discussions on DG/SPS-PDSCH, if something sent from NW and received by UE, then the corresponding ACK/NACK could be responded or deferred. | - Unnecessary PDSCH retransmission if no acknowledgement from the UE due to cell DRX non-active period. |
| CG-PUSCH | Under email discussion in RAN2 with no agreements yet | As per RAN2 [POST121][311] [NES] DTX/DRX - gNB and UE behaviours]): - Proposal 2: As baseline, UE does not transmit on CG occasions overlapping with Cell DRX non-active periods. (20/24) FFS: whether it is possible to configure an exception to this (e.g. per CG or Cell DRX configuration) such that the UE can transmit on CG occasions overlapping with Cell DRX non-active periods to support low latency traffic. |

Proposal 7: Discuss whether periodic/semi-persistent CSI CSI-RS-reports and SRS receptions shall be omitted during cell DRX non-active period.

  - FFS on the potential solutions to overcome the impact from the omitted occasions due to cell DRX non-active period.

Proposal 8: Wait for RAN2 agreements on the gNB/UE behaviour for SR and DG/CG-PUSCH reception during the cell DRX non-active period.

  - FFS on potential RAN1 impacts.

Proposal 9: Wait for RAN2 progress on HARQ feedback for DG/SPS-PDSCH reception during the cell DTX non-active period.

Apart from the impact analysis of cell DRX to RRC_CONNECTED UEs, the potential impact to RRC_INACTIVE/IDLE UEs should be avoided as per the WID. Thus, This is left to NW implementation.

Proposal 10: Confirm that RRC_INACTIVE/IDLE UEs should not be impacted by cell DRX. It is up to NW implementation to avoid any potential impact(s).

## Joint consideration for cell DTX/DRX with legacy operations

In this section, we discuss potential interactions of cell DTX/DRX with the following legacy operations:

- HARQ-ACK codebook generation

- PUCCH deferral (including PUCCH repetition deferral and Rel-17 SPS HARQ-ACK deferral)

### HARQ-ACK codebook generation

First, it’s worth recalling that in legacy Type 1 HARQ-ACK codebook generation, the UE should omit PDSCH allocations that would overlap with a set of symbols indicated as uplink in the TDD configuration. Similarly, considering now cell DTX non-active periods, the impact of such periods on HARQ-ACK feedback content and reporting would need to be considered. Specifically, if e.g., an SPS PDSCH is dropped due to overlap with a cell DTX non -active period, it should be discussed how this dropping would impact the SPS HARQ-ACK and more generally the generation/construction of HARQ-ACK codebook (such as Type 1 codebook).

Proposal 11: Discuss the impact of cell DTX non-active periods on existing HARQ-ACK codebook generation (at least considering Type 1 HARQ-ACK codebook).

### PUCCH deferral

SPS HARQ-ACK deferral, as described in clause 9.2.5.3 of TS 38.213, was introduced in Rel-17. As shorter SPS PDSCH periodicities (down to 1 slot) are supported, there could be cases that the PUCCH carrying HARQ-ACK of SPS HARQ may not be transmitted for NR TDD operation as there may be collisions with for example NR SSB, semi-static downlink symbols, etc.

Further, deferral is in general supported since Rel-15 for PUCCH repetition operation, where a repetition of a PUCCH transmission is only performed in valid UL symbols/slots, i.e., symbols that are not SSB, semi-static downlink symbols, etc.

Observation 2: PUCCH deferral operations in legacy consist of (i) deferral for PUCCH repetition operation (from Rel-15), and (ii) SPS HARQ-ACK deferral (from Rel-17).

Considering now the presence of cell DRX non-active periods, the interaction of cell DRX with the above existing PUCCH deferral operations would need to be discussed.

Proposal 12: Discuss the interaction of cell DRX (non-active periods) with the existing PUCCH deferral operations, i.e., PUCCH repetition deferral and Rel-17 SPS HARQ-ACK deferral.

## Joint consideration of cell DTX/DRX mechanism with techniques in spatial and power domains

In RAN1, another objective in the WID to be discussed and specified with Rel-18 NW ES is the techniques in spatial and power domains.

Practically, when the NW is going to deeper and deeper sleep mode, more hardware components can be switched off for energy saving purpose. Hence, the number of PAs can be reduced when network transits to a deeper sleep mode, which implies lower transition energy. For instance, for a cell DTX/DRX pattern that is mapped to micro sleep mode, the 32-port with multiple panels/TRPs can be applied in the cell DTX/DRX active periods, and for another cell DTX/DRX pattern that is mapped to leverage a deeper sleep mode, the number of antenna ports or panels or TRPs can be reduced during the cell DTX/DRX active periods by muting transceiver chains with the associated PAs, e.g. to 8-port with single panel/TRP for better energy efficiency. Thus, the configurations of cell DTX/DRX can be considered jointly with adaptation of spatial and power domain techniques.

Proposal 13: Enhancement on cell DTX/DRX mechanism can be jointly considered with adaptation of spatial and power domain techniques.

# Conclusions

In this contribution, we have the following observations and proposals:

Proposal 1: Wait for RAN2 agreements on Cell DTX/DRX configuration/ activation/ deactivation.

Observation 1: The Alignment between Cell DTX/DRX and UE C-DRX has no RAN1 impact, and it ups to RAN2 discussions and decision.

Proposal 2: We propose the following extended list of DL channels/signals to be used as a baseline for more detailed discussion.

- Periodic/Semi- persistent CSI-RS for L1/L3-RSRP

- Periodic/Semi- persistent CSI-RS for Radio Link Management (RLM)/beam Failure detection (BFD)

- Periodic/Semi-persistent CSI-RS for tracking (TRS)

- PRS

- PDCCH scrambled with UE specific RNTI

- PDCCH in Type-3 CSS

- SPS-PDSCH

Proposal 3: Discuss whether all types of periodic/semi-persistent CSI-RS and PRS transmission shall be dropped during cell DTX non-active period.

- FFS on the potential solutions to overcome the impact from the dropped transmissions due to cell DTX non-active period.

Proposal 4: Wait for RAN2 agreements on the gNB/UE behaviour for PDCCH and SPS-PDSCH (re)-transmission during the Cell DTX non-active period.

- FFS on potential RAN1 impacts.

Proposal 5: Confirm that RRC_INACTIVE/IDLE UEs should not be impacted by cell DTX. It is up to NW implementation to avoid any potential impact(s).

Proposal 6: We propose the following list of UL channels/signals to be used as a baseline for more detailed discussion.

- SR

- Periodic/Semi-persistent CSI report

- Periodic/Semi-persistent SRS

- HARQ feedback for SPS/DG-PDSCH

- CG-PUSCH

Proposal 7: Discuss whether periodic/semi-persistent CSI CSI-RS-reports and SRS receptions shall be omitted during cell DRX non-active period.

  - FFS on the potential solutions to overcome the impact from the omitted occasions due to cell DRX non-active period.

Proposal 8: Wait for RAN2 agreements on the gNB/UE behaviour for SR and DG/CG-PUSCH reception during the cell DRX non-active period.

  - FFS on potential RAN1 impacts.

Proposal 9: Wait for RAN2 progress on HARQ feedback for DG/SPS-PDSCH reception during the cell DTX non-active period.

Proposal 10: Confirm that RRC_INACTIVE/IDLE UEs should not be impacted by cell DRX. It is up to NW implementation to avoid any potential impact(s).

Proposal 11: Discuss the impact of cell DTX non-active periods on existing HARQ-ACK codebook generation (at least considering Type 1 HARQ-ACK codebook).

Observation 2: PUCCH deferral operations in legacy consist of (i) deferral for PUCCH repetition operation (from Rel-15), and (ii) SPS HARQ-ACK deferral (from Rel-17).

Proposal 12: Discuss the interaction of cell DRX (non-active periods) with the existing PUCCH deferral operations, i.e., PUCCH repetition deferral and Rel-17 SPS HARQ-ACK deferral.

Proposal 13: Enhancement on cell DTX/DRX mechanism can be jointly considered with adaptation of spatial and power domain techniques.

# References

- R1-2302393, “Techniques in Spatial and Power Domains”, Nokia, Nokia Shanghai Bell, RAN1#112bis-e, April 17th-26th, 2023.