3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2302398

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.8

Source:	Ericsson

Title:	PDCCH monitoring resumption after UL NACK

Document for:	Discussion, Decision

# 1	Introduction

In RAN plenary 98-e, the Rel-18 WI on eXtended Reality (XR) was agreed [1] and was further revised in RAN#99 [2], with the following objectives:

Specify the enhancements related to power saving: - DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2). Specify the enhancements related to capacity: - Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2); - Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2); - Discard operation of PDU Sets for DL and UL (RAN2, RAN3); Specify the enhancements for XR Awareness: - Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); - Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2); - Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); - Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

Note that in addition to these objectives, it was agreed that two additional power saving enhancements can be discussed in working groups:

- PDCCH monitoring resume if UE transmits NACK after PDCCH skipping starts (RAN1, see e.g. [3]); and

- CG without retransmissions for uplink XR traffic (RAN2, see e.g. [4]).

With respect to the first power saving enhancement scheme above based on PDCCH monitoring resumption, a draft CR is proposed to identify the specification impact to support this feature [5]. In this contribution, we discuss additional details regarding the draft CR in [5]

# 2	Discussion

From our perspective, it is beneficial to support the proposed enhancement as described in [5].

## Background:

During the discussion among companies for drafting the CR in [5], we raised an issue that led to a discussion on whether/how the draft CR should explicitly reflect the interaction with MAC specification TS 38.321 for specification of this enhancement.

When draft CR was originally discussed, the following TP was proposed:

- Proposed TP1:

When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR before the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE shall monitor PDCCH regardless of PDCCH skipping indication on all serving cells of the corresponding Cell Group when the SR is pending [11, TS 38.321]. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group. When the UE is configured with [PdcchMornitoringResumptionAfterNack] and the UE transmits a PUCCH or a PUSCH providing a NACK value, after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH or PUSCH transmission in the serving cell. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a SpCell, when contention resolution is successful [11, TS 38.321], the UE resumes PDCCH monitoring on the SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a serving cell, when a pending SR is cancelled [11, TS 38.321], the UE resumes PDCCH monitoring in all serving cells of the corresponding Cell Group. If UE transmits a RACH due to positive SR, the UE shall not skip PDCCH monitoring on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell.

We proposed the following updates to TP1:

When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR before the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE shall monitor PDCCH regardless of PDCCH skipping indication on all serving cells of the corresponding Cell Group when the SR is pending [11, TS 38.321]. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group. When the UE is configured with [PdcchMornitoringResumptionAfterNack] and the UE transmits a PUCCH or a PUSCH providing a NACK value, after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting from the beginning of a first slot that is after a last symbol of the PUCCH or PUSCH transmission in the serving cell when the PDCCH monitoring conditions according to [11, TS 38.321] are satisfied. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a SpCell, when contention resolution is successful [11, TS 38.321], the UE resumes PDCCH monitoring on the SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a serving cell, when a pending SR is cancelled [11, TS 38.321], the UE resumes PDCCH monitoring in all serving cells of the corresponding Cell Group. If UE transmits a RACH due to positive SR, the UE shall not skip PDCCH monitoring on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell.

The discussion led to consider the following TP in [5]. However, it was discussed whether the updates in TP2 as compared to TP1 is needed.

- Proposed TP2 (as in [5]):

When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field. If the UE transmits a PUCCH providing a positive SR before the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE shall monitor PDCCH regardless of PDCCH skipping indication on all serving cells of the corresponding Cell Group when the SR is pending [11, TS 38.321]. If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group. When the UE is configured with [PdcchMornitoringResumptionAfterNack] and the UE transmits a PUCCH or a PUSCH providing a NACK value, after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring, starting from the beginning of a first slot that is after a last symbol of the PUCCH or PUSCH transmission in the serving cell, and additionally according to the procedures described in [11, TS 38.321] if DRX is configured. During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a SpCell, when contention resolution is successful [11, TS 38.321], the UE resumes PDCCH monitoring on the SpCell. After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a serving cell, when a pending SR is cancelled [11, TS 38.321], the UE resumes PDCCH monitoring in all serving cells of the corresponding Cell Group. If UE transmits a RACH due to positive SR, the UE shall not skip PDCCH monitoring on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running. If the DRX group of the serving cell is configured and enters outside Active Time, the UE terminates PDCCH skipping for the serving cell.

## Justifications:

When the UE has received a PDDCH that indicates PDCCH skipping for a skipping period, the UE skips PDCCH monitoring. If the UE transmits NACK in a PUCCH/PUSCH, the desired behaviour is “not to skip PDCCH monitoring” and “resume PDCCH monitoring”.

There are two cases:

- Outside Active Time: Based on the existing specifications, the PDCCH skipping is terminated after transmission of NACK by PUSCH/PUCCH that occurs outside the Active Time. Hence, the current specification already supports the desired behaviour. See example in Figure 1.

- Inside active Time: Based on the existing specifications, if transmission of NACK by PUSCH/PUCCH occurs inside the Active Time while drx-onDurationTimer is still running, the UE continues to skip PDCCH monitoring for the skipping perdio until the drx-onDurationTimer expires after transmission of PUCCH/PUSCH carrying NACK. See example in Figure 2. Therefore, a CR is needed to reflect the desired behaviour as shown in Figure 3.

Figure 1: PDCCH skipping within blue area. Termination of PDCCH skipping after blue area outside Active Time after transmission of NACK.

Figure 2: PDCCH skipping within blue area. Transmission of NACK ends inside the Active Time and PDCCH is skipped within the skipping period (blue area) while drx-onDurationTimer is running.

Figure 3: PDCCH skipping within blue area. Transmission of NACK ends inside the Active Time and PDCCH is skipped in yellow area within the skipping period (blue and yellow area) while drx-onDurationTimer is running.

Please note that based on the recent skipping indication by PDCCH, the UE would skip monitoring PDCCH for the skipping period while drx-onDurationTimer is running. Therefore, the TP should define a point in time in this interval where the UE can start monitoring PDCCH. In other words, the start of the yellow area in Figure 3 should be specified to realize the desired behaviour.

Both TP1 and TP2 intend to determine this point as the following:

- TP1: at the beginning of the slot after PUSCH/PUCCH carrying NACK

  - starting point is A for the example shown in Figure 4

- TP2: from the beginning of the slot after PUSCH/PUCCH carrying NACK when conditions in MAC procedures are satisfied

  - starting point is B for the example shown in Figure 5

The motivation for TP2 is as the following:

The enhancement intends to resume PDCCH monitoring because of NACK such that potential rescheduling of the corresponding TB is not delayed because of skipping period. However, due to this NACK, the UE is not expected to be scheduled until drx-HARQ-RTT-TimerDL is expired.

If the timer expires before point A, there is no difference between TP1 and TP2. However, for the scenarios shown in Figure 4/Figure 5, TP1 may cause confusion at the UE:

- Following TP1, UE should start monitoring PDCCH at point A.

- Following MAC procedures, UE should expect PDCCH scheduling retransmission after drx-HARQ-RTT-TimerDL is expired.

TP2 intends to avoid the conflict. Moreover, the power saving properties during drx-HARQ-RTT-TimerDL are maintained.

Figure 4: PDCCH skipping within blue area. Transmission of NACK ends inside the Active Time and PDCCH is skipped in yellow area within the skipping period (blue and yellow area) while drx-onDurationTimer is running. Yellow area starts at point A according to TP1.

Figure 5: PDCCH skipping within blue area. Transmission of NACK ends inside the Active Time and PDCCH is skipped in yellow area within the skipping period (blue and yellow area) while drx-onDurationTimer is running. Yellow area starts at point B according to TP2.

Therefore, we propose:

Endorse TP2 (or the draft CR in [5]).

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Endorse TP2 (or the draft CR in [5]).

# References

RP-223502		New WID on XR Enhancements for NR; Nokia, Qualcomm

RP-230786		Updated WID on XR Enhancements for NR; Nokia, Qualcomm

RP-230649		XR PDCCH skipping enhancement - monitoring resumption after NACK; MediaTek Inc., Apple, Ericsson, Google, Meta, Qualcomm, vivo

RP-230456		Updates of XR WID for retransmission-less CG; Huawei, HiSilicon, Apple, Google, InterDigital, Lenovo, Meta, Nokia, Nokia Shanghai Bell, Qualcomm

R1-2302500		Draft CR on PDCCH monitoring resumption after NACK; vivo, MediaTek, Google, Qualcomm, Panasonic, Meta, Ericsson, China Unicom, Huawei, HiSilicon, Xiaomi, Apple, China Telecom