3GPP TSG RAN WG1 #112bis-e										     	 	        R1-2302401

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.9.2

Source:	Thales

Title:	Discussion on network verified UE location in NR NTN

Document for:	Discussion and decision

# Introduction

As part of Release 18, a new work item is proposed to define enhancements for NG-RAN based Non-Terrestrial Networks [1]. Among other objective, this work item aims to:

Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied. Note 1: Enhancements assume reuse of the RAT dependent positioning framework Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity) Note 4 : Multiple satellite in view by the UE may be considered if time allows Note 5 : The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved Note 6 : The enhancements should take into account the mirror-image ambiguity Note 7 : Network verified UE location is an optional UE feature

The agreements and conclusion made at RAN1#112 can be found in section 8.1.

In this contribution we discuss potential necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view.

# Principle of multi-RTT positioning with single satellite in view

The principle of the multi-RTT computation performed with only one satellite in the 5G NTN context is illustrated in Figure 1. The RTT measurements (corresponding to the round trip time on the service link) can be repeated in different instants, and from different positions on the satellite orbit. On ground, every measured RTT corresponds to a circle ( or a ring by considering the RTT measurement error). The subsequent measurement of RTT will describe a different circle of different diameter (depending if distance increase or decrease), and with a different center. In this way the intersection of all the circles will point at the UE location (assumed not moving).

Figure 1 multi-RTT based positioning with single satellite

To perform RTT measurements, satellite movement between TX and RX (the uplink and down link one-way delays are different) should be taken into account. To this aim, every measured RTT should correspond to an ellipse (and not circle) on the ground : For each measured RTT the possible UE location is traced by a point moving in  the space so that the sum of its distances from two anchor points (the foci/ the two focal points) is constant = measured RTT. The two anchor points are the satellite positions at TX and RX respectively.

Different measurements of RTT will describe different ellipses with different eccentricity. The intersection of these ellipses with the ground will determine the resulting UE position uncertainty area.

# Rx-Tx time difference measurements in NTN

The UE Rx – Tx time difference is a report from the UE about the time difference between the start of a downlink subframe and the start of the corresponding uplink subframe. It is defined in clause 5.1.30 of TS 38.215 [7] as follows:

Table 1 Definition of UE Rx – Tx time difference [TS 38.215]

| Definition | The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [TS 38.305], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. Multiple DL PRS or CSI-RS for tracking resources, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the TP. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

For enhancing UE Rx-Tx time difference in NTN, the following options where down-selected in RAN1#112:

Option 1: The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX

Where:

  - UE Rx-Tx time difference is defined with respect to the Rx and Tx subframe timing associated with the TRP.

For a Transmission Point

  - TUE-RX is the UE received timing of downlink subframe #i from this Transmission Point (TP), defined by the first detected path in time.

  - TUE-TX is the UE transmit timing of the uplink subframe corresponding to subframe #i received from the TP

  - One or multiple DL RS for positioning, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the TP.

FFS: For a Transmission Point different from the serving cell (e.g. a DL-PRS-only TP)

Option 2:

- For RTT measurement in NTN, support UE report that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS.

- FFS: details of report and the definition of UE Rx-Tx time difference

Option 3:

The legacy R17 definition of UE Rx-Tx time difference is adopted for NTN with an offset that is determined based on one of the following options:

- Option 3-1: This offset is reported as the nearest integer value in the unit of milliseconds by rounding the time difference of transmit timing of uplink subframe #i and receive timing of downlink subframe#i

- Option 3-2: UE report the index of the subframe j that is closest in time to the subframe #i received from the TP and LMF can derive the offset

- Option 3-3: TA report which corresponds to the time difference of received timing of downlink subframe #i and transmit timing of uplink subframe#i rounding up to slot granularity.

Option 4:

- UE Rx – Tx time difference TUE-RX – TUE-TX  can be directly derived from timing advance TTA

  - FFS: the granularity and the reporting range of TA.

  - Note: This implies that the existing framework for Multi-RTT positioning report can be used without need to specify a new TA report.

Note: The impact of UE autonomous adjustment of TA (when applied) should be taken into account

In essence, with option 1 the measurement is restricted to the serving cell and corresponds to the timing advance. To determine TUE-RX, DL RS for positioning is used to determine the start of the DL subframe #i. However, the TUE-TX is determined by the UE as the transmit timing of the uplink subframe corresponding to subframe #i received from the TP. Which means that the determination of TUE-TX is subject to potential timing error i.e. Te_NTN  which could be up to 29*64*Tc or 944ns in FR1 with 15kHz SCS. This will clearly affect the positioning accuracy and thereby this option might not be appropriate for the measurement of  UE Rx – Tx time difference in NTN. Similarly, the UE Rx – Tx time difference determination based on option 3 and option4 is also affected by  Te_NTN.

The UE Rx – Tx time difference determined based on option 1, option 3 and Option 4 (refer to RAN1#112 agreement) may be affected by the timing error i.e. Te_NTN  which could be up to 29*64*Tc or 944ns in FR1 with 15kHz SCS. De facto, with these options the positioning accuracy/UE location verification accuracy can be degraded.

From our perspective, option 2 should be adopted for enhancing UE Rx-Tx time difference in NTN. By considering this option, the UE Rx – Tx time difference in NTN can be defined as TUE-RX – TUE-TX

Where:

  - UE Rx-Tx time difference is defined with respect to the Rx and Tx subframe timing associated with the TRP.

  - TUE-RX is the UE received timing of downlink subframe #i from this Transmission Point (TP), defined by the first detected path in time.

    - One or multiple DL RS for positioning, as instructed by higher layers, can be used to determine the start of downlink subframe #i.

  - TUE-TX is the UE transmit timing of the uplink subframe #j

    - SRS transmission can be used to determine the start of the uplink subframe #j

Based on the above discussion, we made the following proposals:

Proposal 1:

For RTT measurement in NTN, support UE report that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS.

Proposal 2:

The UE Rx – Tx time difference in NTN is defined as TUE-RX – TUE-TX

Where:

- UE Rx-Tx time difference is defined with respect to the Rx and Tx subframe timing associated with the Transmission Point (TRP)

- TUE-RX is the UE received timing of downlink subframe #i from this TRP, defined by the first detected path in time.

  - One or multiple DL PRS, as instructed by higher layers, can be used to determine the start of downlink subframe #i.

- TUE-TX is the UE transmit timing of uplink subframe #j containing SRS associated with UE

The principle UE Rx-Tx time difference measurement as per Proposal 1 is illustrated in Figure 2.

Figure 2 -	UE Rx-Tx time difference measurement

The reporting range for the absolute UE Rx-Tx time difference measurement is defined in clause 10.1.25 of  TS 38.133. This range is from -985024 × Tc to 985024×Tc with the resolution step of 2k×Tc,

Tc is equal to 0,509ns as defined in TS 38.211. And k = 0, 1, 2, 3, 4, or 5 depending on PRS and SRS resource configuration in FR1 and FR2 and the timing reporting granularity factor configured by LMF via LPP.

The maximum resolution step of is given by k=5, that is, 32×Tc =16ns.

The same reporting granularity of UE Rx-Tx time difference as defined for TN  can be also used for the network verified UE location based on Multi-RTT method as the maximum additional error on RTT measurement would be equal to 16ns.

The time at which the SRS should be transmitted in response to DL PRS may need to take into account the scheduling constraints. But it should be limited by the reporting range of the absolute UE Rx-Tx time difference measurement. We propose the reporting range in the order of RTT in NTN.

The following table provides the reporting ranges for the absolute UE Rx-Tx time difference measurement that should be supported to suit different NTN orbits.

Table 2 The Reporting range of UE Rx-Tx time difference in NTN

|  | GEO | LEO 600km | 1200km |
| --- | --- | --- | --- |
| Max RTD (ms) | 541,46 | 25,77 | 41,77 |
| Reporting range of UE Rx-Tx time | From -531886051 to +531886051 | From -25314342 to +25314342 | From -41031434 to +41031434 |

We propose to support the reporting ranges for the absolute UE Rx-Tx time difference measurement that can be used to accommodate the RTD in GEO.

Proposal 3: For multi-RTT-based positioning in NTN, the reporting range for the absolute UE Rx-Tx time difference measurement is from -531886051 to +531886051.

- Note: The same reporting granularities of UE Rx-Tx time difference as defined in clause 10.1.25 of  TS 38.133 are reused in NTN.

The gNB Rx – Tx time difference is defined in clause 5.2.3 of  TS 38.215 as follows:

Table 3 Definition gNB Rx – Tx time difference in TN [TS 38.215]

| Definition | The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the Transmission and Reception Point (TRP) [TS 38.305] received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. Multiple SRS resources can be used to determine the start of one subframe containing SRS. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104: the Tx Transceiver Array Boundary connector. |
| --- | --- |

For the enhancement of gNB Rx-Tx time difference in NTN, the following options were down-selected in RAN1#112:

Option 1:

- The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX

Where:

For a Transmission Point

  - TgNB-RX is the Transmission and Reception Point (TRP) received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time.

  - TgNB-TX is the TRP transmit timing of the downlink subframe corresponding to uplink subframe #i received from the UE

  - Multiple SRS resources can be used to determine the start of one subframe containing SRS.

FFS: For a Transmission Point different from the serving cell (e.g. a DL-PRS-only TP)

Option 2:

- For RTT measurement in NTN, support gNB report of gNB Rx-Tx time as defined in 38.215 with the following change:

  - Only the SRS resource starting within a subframe can be used to determine the start of the subframe.

Option 3:

- Keep the current gNB Rx-Tx definition, and report an offset which can covers the time duration corresponds to kmac if needed.

Option 4:

- For RTT measurement in NTN, support gNB report that indicates the time difference between the transmit time of a DL RS for positioning and the arrival time of an SRS.

FFS: details of report.

Note: The impact of UE autonomous adjustment of TA (when applied) should be taken into account

We support option 4 as it will allow to derive the RTT as illustrated in Figure 2.

Proposal 4:

For RTT measurement in NTN, support gNB report that indicates the time difference between the transmit time of a DL RS for positioning and the arrival time of an SRS.

In existing specification, unlike the SRS, the PRS is not associated to a specific UE. Therefore, To be able to calculate the RTT based on the time difference between the transmit time of a DL PRS and the arrival time of an SRS as reported by the gNB and the time difference between the arrival time of a DL PRS and the transmit time of an SRS as reported by the UE, the DL PRS used for the measurement of these time difference should be uniquely identified by both UE and gNB.

The reporting range of gNB Rx-Tx time difference, is defined in clause 13.7 of TS 38.133. It ranges from -985024Tc to +985024Tc. The reporting resolution is uniform across the reporting range and is defined as Tc*32 [8]. The same reporting granularity can be also used for the network verified UE location based on Multi-RTT method, as the additional error on RTT measurement would be equal to 16ns. However, the reporting range of gNB Rx-Tx time difference should be adapted to NTN to suit different orbits including GEO.

The same reporting granularity of gNB Rx-Tx time difference as defined for TN  can be also used for the network verified UE location based on Multi-RTT method as the additional error on RTT measurement would be equal to 16ns.

The following table provides the reporting ranges for the gNB Rx-Tx time difference measurement that should be supported to suit different NTN orbits.

Table 4 Reporting range of gNB Rx-Tx time difference in NTN

|  | GEO | LEO 600km | 1200km |
| --- | --- | --- | --- |
| Max RTD (ms) | 541,46 | 25,77 | 41,77 |
| Reporting range of gNB Rx-Tx time | From -531886051 to +531886051 | From -25314342 to +25314342 | From -41031434 to +41031434 |

Based on the above, we propose the following:

Proposal 5:

For multi-RTT-based positioning in NTN, the reporting range for the gNB Rx-Tx time difference measurement is from -531886051 to +531886051.

Note: The same reporting granularities of gNB Rx-Tx time difference as defined in clause 13.7 of  TS 38.133 are reused in NTN.

Furthermore, the reference point for TgNB-RX  and TgNB-TX used for the gNB Rx – Tx time difference measurement should be further discussed in RAN1. The following options were down-selected in RAN1#112:

-	Option 1: onboard the satellite

-	Option 2: the uplink time synchronization reference point

-	Option 3: on the gNB

In essence, all the three options above can work. However, as the satellite should be used as anchor point for positioning, having the satellite the reference point on the satellite would avoid transmitting more assistance information to the LMF such as common TA and kmac. We then have the following proposal:

Proposal 6:

For FR1, the reference point for TgNB-RX  and TgNB-TX used for the gNB Rx – Tx time difference measurement in NTN shall be:

For TgNB-RX :

- for SAN type 1-O: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna)

- for SAN type1-H: the Rx Transceiver Array Boundary connector.

For TgNB-TX:

- for SAN type 1-O: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna),

- for SAN type 1-H: the Tx Transceiver Array Boundary connector

FFS: for FR2

# Impact of the geometry on the accuracy of multi-RTT positioning method

The geometry of the receiver-transmitter is affecting position precision in any ranging system. How observation geometry relating the UE and positioning anchor points (TRP) affects the network verified UE location was discussed in [9]. For example, for a UE located on the orbital plane it is clear from Figure 1 if all RTT measurements are performed when the satellite is on left to plane v, the intersection of the three spheres will result  in very large/infinite position uncertainty areas.

The position (3D) dilution of precision (PDOP) values obtained through simulations [9] are excessively large/poor for the UE located nearby the orbital plane. In several contributions submitted to RAN1#111, it was reported that the geometry of UE location relative the satellite orbit impacts the positioning performance. In [10] for example it was also observed via simulation that the error in position estimation depends largely on the UE position relative to the satellite points on the ground which are below the orbit or close to it have very poor resolution in the position.

Figure 3 The impact of geometry of the receiver-transmitter on position precision

The geometry relating the UE and positioning anchor points (TRP) affects the network verified UE location based on Multi-RTT method.

Based on the above observation, we propose to consider the position dilution of precision (PDOP) [5] as a performance metric for the implementation of the multi-RTT based positioning method for the following scenarios: a single satellite in view and multiple satellite in view. With a single satellite in view, the arrangement of the satellite positions on the orbit (satellite geometry and how satellite positioning/anchoring points for positioning are spread) affects the accuracy of the positioning. The PDOP is a measure of the quality of derived positions based on the geometry of the virtual TRPs (corresponding to the satellite positions at which the Rx-Tx time difference measurements are performed) used as anchor points to compute the positioning. It is a fundamental metric to be considered when collecting data/RTT measurement for UE location verification: The higher the DOP, the greater the possible error in the accuracy of UE position.

With the above in mind, we propose that the satellite positions (vTRP) at which the Rx-Tx time difference measurements will be performed are determined based on PDOP calculation at network side. Because the PDOP values are only functions of UE and future satellite positions (vTRPs), it may be used by the network (e.g. LMF and/or gNB) to ahead of time for a set of future satellite positions (to be used as TRPs) in view from a specified location (e.g. UE location and/or a reference point at the beam/cell; cell center for example).

Proposal 7:

For multi-RTT based positioning in NTN, the satellite positions (vTRP) at which the Rx-Tx time difference measurements will be performed are determined based on PDOP calculated at network side (e.g at LMF or gNB).

The Multi-RTT positioning performance is assessed by the network (e.g LMF or gNB or both) from a geometric perspective by using the PDOP to determine the vTRP positions and the appropriate time windows used for Rx-Tx time difference measurements during the satellite fly-by. The basic principle vTRP determination is as follows:

Once the UE is connected, the AMF triggers the location services request toward the LMF, then:

- LMF calculates the PDOP before returning the UE location verification results to the AMF.

- PDOP is calculated based on the UE reported GNSS location and/or a reference point within the cell/beam and future satellite positions (vTRPs) on the satellite orbit in case of single satellites that are appropriate for making accurate decisions.

- By considering RTT measurement error of 200ns (resulting in a ranging error of 60m), and the target accuracy for position verification of 10km (as documented in clause « recommendations » of the 3GPP TR 38.882), the resulting position  Error = 10 000 m  =  PDOP x 60m. Thereby, the maximum acceptable PDOP value appropriate for making accurate decisions is equal to 166.

- Pre-requisites: LMF should have the constellations  (or serving satellite) ephemeris data

- If a good geometry (relating the UE and vTRPs) can be found. That is, suitable satellite anchor points for positioning that provide lower DOP values (maximizing the positioning accuracy) can be determined, the LMF will trigger the UE location verification procedures by initiating multi-RTT methods and providing the satellites anchor points for positioning to the NGRAN/UE.

LMF performs the UE location verification if the calculated DOP represents a level that marks the minimum appropriate for making accurate decision (sufficient for UE location verification). Otherwise, if the calculated DOP is high (> 166), the UE verification cannot be performed with sufficient accuracy. Thereby, the LMF will not perform the UE location verification and simply inform the AMF that the DOP is out of range. LMF returns the result to the AMF, the AMF can then take the necessary action.

For multi-RTT-based positioning in NTN, the maximum acceptable PDOP value appropriate for making accurate decisions is equal to 166

# Assistance data for Multi-RTT positioning with single Satellite

## Information to be transferred from the UE to LMF

As specified in clause 8.10.2.2 of TS 38.305, apart from the UE Rx – Tx time difference, the assistance data that may be signaled from UE to the LMF is listed in Table 5

Table 5 Information to be transferred from the UE to LMF [TS 38.305]

Information

PCI, GCI, and PRS ID, ARFCN, PRS resource ID, PRS resource set ID for each measurement

DL-PRS-RSRP measurement

UE Rx-Tx time difference measurement

Time stamp of the measurement

Quality for each measurement

TA offset used by UE

UE Rx TEG IDs, UE Tx TEG IDs, and UE RxTx TEG IDs associated with UE Rx-Tx time difference measurements

LOS/NLOS information for UE measurements

DL-PRS-RSRPP measurement

The association of UE Tx TEG ID and SRS

As discussed in section 3, the anchor point (that is TRP position) for positioning in NTN should be located at the satellite. Thereby, for the RTT-based positioning it is enough to estimate only the distance between the satellite and the UE. This can be determined based on the estimated RTT on the service link. However, if the uplink time synchronization reference point is the reference point for TgNB-RX  and TgNB-TX used for the gNB Rx – Tx time difference measurement, the UE should report also NTA, common that is applied by the UE to compensate the two-way transmission delay between the uplink time reference point and the satellite.

Proposal 8:

For multi-RTT based positioning in NTN, if the reference point for definition of gNB Rx – Tx time difference measurement is onboard the satellite, the UE should include the calculated NTA,adjcommon to the measurement results that need be transferred from UE to the LMF,

Upon the receipt of LPP Request Location Information/Multi-RTT measurements from the LMF, the UE performs DL PRS measurements at different (consecutive) vTRP. Although the LMF may has already assisted the UE by indicating the vTRP positions or the time intervals at which the PRS should be measured based on vTRP determination using the GDOP, it is reasonable that the UE includes within the measurement result the position of the satellite at DL-PRS measurements for Multi-RTT.

Figure 4 Information to be transferred from the UE to LMF in NTN

Proposal 9:

For multi-RTT based positioning in NTN, the UE includes the position of the satellite when DL-PRS measurements are performed to the measurement results that need be transferred from UE to the LMF.

## Information to be transferred from the gNB to LMF

As specified in clause 8.10.2.3 of TS 38.305 the assistance data that may be transferred from gNB to the LMF is listed in the following Table.

Table 6 Information to be transferred from the gNB to LMF [TS 38.305]

Information

PCI, GCI, ARFCN and TRP IDs of the TRPs served by the gNB

Timing information of TRPs served by the gNB

DL-PRS configuration of the TRPs served by the gNB

SSB information of the TRPs (the time/frequency occupancy of SSBs)

Spatial direction information of the DL-PRS Resources of the TRPs served by the gNB

Geographical coordinates information of the DL-PRS Resources of the TRPs served by the gNB

TRP type

On-demand DL-PRS information

TRP Tx TEG association information

Because the reference point for TgNB-RX  and TgNB-TX used for the gNB Rx – Tx time difference measurement is located at the satellite payload (refer to section 4) which may be different from the reference point for uplink synchronization, it is necessary for the gNB to indicate to the LMF the value of the value of  𝑘mac used by gNB and the value of TACommon at the time of gNB Rx – Tx time difference measurement is performed.

Further, as discussed in section 5 the gNB may need to provide the LMF with the ephemeris data in PVT state vector format or Keplerian format and associated epoch time.

Proposal 10:

For multi-RTT based positioning in NTN, if the reference point for definition of gNB Rx – Tx time difference measurement is onboard the satellite, the following assistance data may be transferred from gNB to the LMF:

    - The value of the value of  𝑘mac used by gNB

    - The value of TACommon when the gNB Rx – Tx time difference measurement is performed

Upon the receipt of NRPPa Measurement Request from the LMF, the gNB performs UL SRS measurements at different (consecutive) vTRP. Although the LMF may has already assisted the gNB by indicating the vTRP positions or the time intervals at which the SRS should be measured based on vTRP determination using the GDOP, it is reasonable that the gNB includes within the measurement report the position of the satellite at UL-SRS measurements for Multi-RTT.

Proposal 11:

For multi-RTT based positioning in NTN, the gNB includes the position of the satellite when UL-SRS measurements are performed to the assistance data that may be transferred from gNB to the LMF.

As proposed in section 5, for RTT-based positioning in NTN with single satellite in view, based on GDOP calculation, the LMF determines the appropriate geometry and vTRP positions with the suitable time to perform Multi-RTT measurements. To this aim, in the response to NRPPa TRP INFORMATION REQUEST message the gNB indicate as part of TRP configuration information the serving satellite ID, cell/beam reference point. The LMF may have access (e.g. via O&M) to satellite ephemeris data, otherwise, it may trigger NRPPa Ephemeris INFORMATION REQUEST towards the gNB to retrieve ephemeris data in PVT state vector format or Keplerian format along with the associated epoch time.

Figure 5 Information to be transferred from the gNB to LMF in NTN

Proposal 12:

For multi-RTT based positioning in NTN, the gNB may provide the LMF with assistance data including:

    - Satellite ID

    - Cell/beam reference point

    - The ephemeris data in PVT state vector format or Keplerian format along with the associated epoch time.

## Information to be transferred from the LMF to UE

The information that may be transferred from the LMF to the UE are listed in clause 8.10.2.1 of TS 38.305 and recopied in Table 7.

Table 7 Assistance data that may be transferred from LMF to the UE [TS 38.305]

Information

Physical cell IDs (PCIs), global cell IDs (GCIs), and PRS IDs, ARFCNs of candidate NR TRPs for measurement

Timing relative to the serving (reference) TRP of candidate NR TRPs

DL-PRS configuration of candidate NR TRPs

SSB information of the TRPs (the time/frequency occupancy of SSBs)

PRS-only TP indication

On-Demand DL-PRS-Configurations

Validity Area of the Assistance Data

For RTT-based positioning in NTN, to perform the necessary DL-PRS measurements, the LMF may need to assist the UE by indicating the vTRP positions or the time intervals at which the PRS should be measured based on vTRP determination using the GDOP as discussed in section 5. This assistance data can be transferred to the UE using LPP Provide Assistance Data message as shown in Figure below:

Figure 6 Information to be transferred from the LMF to UE in NTN

Proposal 13:

For multi-RTT based positioning in NTN, the LMF indicates to the UE the vTRP positions or the time intervals at which the PRS should be measured.

## Information to be transferred from the LMF to gNB

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.

Table 8 Requested positioning activation/deactivation information that may be transferred from LMF to gNB [TS 38.305] .

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation Time

Aperiodic UL-SRS - Aperiodic SRS Resource Trigger List - Activation Time

UL-SRS: - Release all

For RTT-based positioning in NTN  with single satellite in view, the LMF provides the time and time period for aperiodic SRS activation based on the vTRP determined using the GDOP.

Figure 7 Information to be transferred from the LMF to gNB in NTN

Proposal 14:

For multi-RTT based positioning in NTN, the LMF indicates to the UE the vTRP positions or the time intervals at which the aperiodic SRS should be activated.

# Resolving the mirror-position ambiguity

As discussed in previous RAN1 meetings, for UE location verification in case of single satellite, multi-RTT method cannot distinguish between the mirror positions on either side of the orbital plane and other input(s) is (are) required to resolve this ambiguity. In RAN1#112-bis, to resolve this issue, 6 options were down-selected (refer to section 8.1).

Knowing the serving cell and by using spatial relation info such as DL-PRS Resource Set ID, it will be possible to distinguish between these mirror positions. except for the case where the serving cell is crossed by the orbital plane and other inputs such reported neighbor cells for example cannot help resolving the ambiguity. In such scenario, it might be beneficial to consider other enhancement. In this regards, we propose to support Doppler reporting in NTN, at least for a handheld UEs. As shown in Figure 4 𝑣 ⃗rad (radial velocity) could be determined by the LMF based on reported Doppler. a VSAT UE may simply report its beam pointing in respect to satellite beam line of sight. Reusing existing ECID method (e.g. combine UE neighbor measurements to solve the ambiguity between mirror positions) could be also considered.

Figure 8 Doppler reporting in NTN

The determination of the Angle of Arrival can be also used to the mirror-position ambiguity. In NR TN. the UE position can be estimated based on the Angle of Arrival (azimuth and/or elevation) of uplink radio signals (i.e. SRS) taken at different TRPs. along with other configuration information and by using assistance data received from the positioning server [2]. Measurement results that may be transferred from gNBs to the LMF using NRPPa are given by the following Table:

Table 9 Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

UL Angle of Arrival (azimuth and/or elevation)

Multiple UL Angle of Arrival (azimuth and/or elevation)

SRS Resource Type

UL-SRS-RSRP

UL-SRS-RSRPP

Time stamp of the measurement

Quality for each measurement

Beam information for each measurement

LoS/NLoS information for each measurement

ARP ID of the measurement

In NR NTN. the Angle of Arrival (AoA) or Direction of Arrival (DoA) of the uplink poisoning signal (i.e. SRS) can be determined based on the AoA measurements from satellite Multi-Panel 2-D array. Different techniques for angle-based positioning can be used to estimate UE location depending on satellite antenna architecture and whether digital. analog or hybrid beamforming are used.

Different techniques for angle-based positioning can be used to estimate UE location depending on satellite antenna architecture and whether digital. analog or hybrid beamforming are used.

UL-AoA can be determined from the difference of phase observed between adjacent antenna elements (AE). The basic principle for angle estimation is as follows: For each pair of AE. AoA is a plane perpendicular to the line joining the AE. with a theta angle. The intersection of AoA planes from a group of AE is theoretically a line. The intersection of this line with Earth surface is the location of the UE.

Figure 9 Difference of phase observed between adjacent antenna elements

The difference of phase between AE1 and AE2 : ∆φAE

∆φAE= 2.π.d. sinθλ

With λ= cF and θ is the AOA of SRS

Generally. the DFT beam method or MUltiple SIgnal Classification (MUSIC) are well known algorithms that can be used to estimate AoA.

The result of the UL-AoA based positioning is a point on Earth. with a certain angular accuracy.  Different defects may affect the angle estimation such as satellite beam pointing error. phase noise and defects due to all transformations (or operations) applied on the signals. from AE on board to the receiving base station on the ground.

The result of the UL-AoA based positioning is a point on Earth. with a certain angular accuracy. Different defects may affect the angle estimation such as satellite beam pointing error. phase noise and defects due to all transformations (or operations) applied on the signals. from AE on board to the receiving base station on the ground.

The residual error on ∆φAE will define the UE location accuracy performance. The angle to distance conversion (° to km) depends on the satellite altitude and the location of the UE within the satellite coverage i.e. UE elevation angle. The required angular accuracy to ensure a location accuracy of 10km is given by the following table for different orbits and for a UE located at the nadir and at 30° elevation.

Table 10 Required angular accuracy for location accuracy of 10km

| Required angular accuracy for location accuracy of 10km |  |  |  |
| --- | --- | --- | --- |
|  | 600Km | 1200km | GEO |
| At nadir | 0.96° | 0.48° | 0.016° |
| At 30° elevation | 0.28° | 0.15° | 0.008° |

The main advantage of UL-AoA positioning method is the low latency. indeed position fixes can be produced almost instantaneously after making the measurements. Other advantages are: it can be used with only one satellite in visibility and its applicability in NGSO and GSO based NTN deployment.

The main advantage of UL-AoA positioning method is the low latency and its applicability for the GEO based NTN deployment

However. UL-AoA in NTN presents some major challenges:

- This technique maybe affected by several defects. Looking at the required angular accuracy shown in Table 5. it might be particularly challenging in practice to maintain the satellite beam pointing error under acceptable angular accuracy with a reasonable cost on the system design.

- Other aspects to be considered is on the uplink coverage as the UE with a limited power budget needs to transmit SRS so that it is detectable within the geoloc cells/beams which are larger beam with unfavorable channel conditions.

From our perspective, to resolve the mirror-position ambiguity issue, multiple solutions can be adopted and used to when relevant. We therefore have the following proposal:

Proposal 15:

The following solution are used to resolve the mirror positions ambiguity for multi-RTT positioning in NTN:

- Reuse existing ECID method (e.g. combine UE neighbor measurements to solve the ambiguity between mirror positions), with potential enhancements

- NR NTN UE should report the Doppler calculated on the service link

- a VSAT UE should report its beam pointing in respect to satellite beam line of sight

- Support and potentially enhance the optional Rel-17 UL-AoA measurements defined for multi-RTT positioning

# Conclusion

In this contribution. we made the following observations and proposals:

The UE Rx – Tx time difference determined based on option 1, option 3 and Option 4 (refer to RAN1#112 agreement) may be affected by the timing error i.e. Te_NTN  which could be up to 29*64*Tc or 944ns in FR1 with 15kHz SCS. De facto, with these options the positioning accuracy/UE location verification accuracy can be degraded.

The same reporting granularity of UE Rx-Tx time difference as defined for TN  can be also used for the network verified UE location based on Multi-RTT method as the maximum additional error on RTT measurement would be equal to 16ns.

The same reporting granularity of gNB Rx-Tx time difference as defined for TN  can be also used for the network verified UE location based on Multi-RTT method as the additional error on RTT measurement would be equal to 16ns.

The geometry relating the UE and positioning anchor points (TRP) affects the network verified UE location based on Multi-RTT method.

For multi-RTT-based positioning in NTN, the maximum acceptable PDOP value appropriate for making accurate decisions is equal to 166

Different techniques for angle-based positioning can be used to estimate UE location depending on satellite antenna architecture and whether digital. analog or hybrid beamforming are used.

The result of the UL-AoA based positioning is a point on Earth. with a certain angular accuracy. Different defects may affect the angle estimation such as satellite beam pointing error. phase noise and defects due to all transformations (or operations) applied on the signals. from AE on board to the receiving base station on the ground.

The main advantage of UL-AoA positioning method is the low latency and its applicability for the GEO based NTN deployment

Proposal 1:

For RTT measurement in NTN, support UE report that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS.

Proposal 2:

The UE Rx – Tx time difference in NTN is defined as TUE-RX – TUE-TX

Where:

- UE Rx-Tx time difference is defined with respect to the Rx and Tx subframe timing associated with the Transmission Point (TRP)

- TUE-RX is the UE received timing of downlink subframe #i from this TRP, defined by the first detected path in time.

  - One or multiple DL PRS, as instructed by higher layers, can be used to determine the start of downlink subframe #i.

- TUE-TX is the UE transmit timing of uplink subframe #j containing SRS associated with UE

Proposal 3:

For multi-RTT-based positioning in NTN, the reporting range for the absolute UE Rx-Tx time difference measurement is from -531886051 to +531886051.

- Note: The same reporting granularities of UE Rx-Tx time difference as defined in clause 10.1.25 of  TS 38.133 are reused in NTN.

Proposal 4:

For RTT measurement in NTN, support gNB report that indicates the time difference between the transmit time of a DL RS for positioning and the arrival time of an SRS.

Proposal 5:

For multi-RTT-based positioning in NTN, the reporting range for the gNB Rx-Tx time difference measurement is from -531886051 to +531886051.

Note: The same reporting granularities of gNB Rx-Tx time difference as defined in clause 13.7 of  TS 38.133 are reused in NTN.

Proposal 6:

For FR1, the reference point for TgNB-RX  and TgNB-TX used for the gNB Rx – Tx time difference measurement in NTN shall be:

For TgNB-RX :

- for SAN type 1-O: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna)

- for SAN type1-H: the Rx Transceiver Array Boundary connector.

For TgNB-TX:

- for SAN type 1-O: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna),

- for SAN type 1-H: the Tx Transceiver Array Boundary connector

Proposal 7:

For multi-RTT based positioning in NTN, the satellite positions (vTRP) at which the Rx-Tx time difference measurements will be performed are determined based on PDOP calculated at network side (e.g at LMF or gNB).

Proposal 8:

For multi-RTT based positioning in NTN, if the reference point for definition of gNB Rx – Tx time difference measurement is onboard the satellite, the UE should include the calculated NTA,adjcommon to the measurement results that need be transferred from UE to the LMF,

Proposal 9:

For multi-RTT based positioning in NTN, the UE includes the position of the satellite when DL-PRS measurements are performed to the measurement results that need be transferred from UE to the LMF.

Proposal 10:

For multi-RTT based positioning in NTN, if the reference point for definition of gNB Rx – Tx time difference measurement is onboard the satellite, the following assistance data may be transferred from gNB to the LMF:

    - The value of the value of  𝑘mac used by gNB

    - The value of TACommon when the gNB Rx – Tx time difference measurement is performed

Proposal 11:

For multi-RTT based positioning in NTN, the gNB includes the position of the satellite when UL-SRS measurements are performed to the assistance data that may be transferred from gNB to the LMF.

Proposal 12:

For multi-RTT based positioning in NTN, the gNB may provide the LMF with assistance data including:

    - Satellite ID

    - Cell/beam reference point

    - The ephemeris data in PVT state vector format or Keplerian format along with the associated epoch time.

Proposal 13:

For multi-RTT based positioning in NTN, the LMF indicates to the UE the vTRP positions or the time intervals at which the PRS should be measured.

Proposal 14:

For multi-RTT based positioning in NTN, the LMF indicates to the UE the vTRP positions or the time intervals at which the aperiodic SRS should be activated.

Proposal 15:

The following solution are used to resolve the mirror positions ambiguity for multi-RTT positioning in NTN:

- Reuse existing ECID method (e.g. combine UE neighbor measurements to solve the ambiguity between mirror positions), with potential enhancements

- NR NTN UE should report the Doppler calculated on the service link

- a VSAT UE should report its beam pointing in respect to satellite beam line of sight

- Support and potentially enhance the optional Rel-17 UL-AoA measurements defined for multi-RTT positioning

# Appendix

## RAN1#112 agreements

The following agreements and conclusion were made at RAN1#112:

Agreement

Existing DL/UL reference signals for positioning are used for supporting Network verified UE location in NTN.

FFS: Whether some enhancements on these reference signals are needed for NTN

Agreement

In NTN, for the position of the reference point for definition of gNB Rx – Tx time difference measurement, consider the following options:

- Option 1: Onboard the satellite

- Option 2: The uplink time synchronization reference point

- Option 3: on the gNB

Agreement

Select one (or more) of the following options for enhancing UE Rx-Tx time difference in NTN

Option 1: The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX

Where:

  - UE Rx-Tx time difference is defined with respect to the Rx and Tx subframe timing associated with the TRP.

For a Transmission Point

  - TUE-RX is the UE received timing of downlink subframe #i from this Transmission Point (TP), defined by the first detected path in time.

  - TUE-TX is the UE transmit timing of the uplink subframe corresponding to subframe #i received from the TP

  - One or multiple DL RS for positioning, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the TP.

FFS: For a Transmission Point different from the serving cell (e.g. a DL-PRS-only TP)

Option 2:

- For RTT measurement in NTN, support UE report that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS.

- FFS: details of report and the definition of UE Rx-Tx time difference

Option 3:

The legacy R17 definition of UE Rx-Tx time difference is adopted for NTN with an offset that is determined based on one of the following options:

- Option 3-1: This offset is reported as the nearest integer value in the unit of milliseconds by rounding the time difference of transmit timing of uplink subframe #i and receive timing of downlink subframe#i

- Option 3-2: UE report the index of the subframe j that is closest in time to the subframe #i received from the TP and LMF can derive the offset

- Option 3-3: TA report which corresponds to the time difference of received timing of downlink subframe #i and transmit timing of uplink subframe#i rounding up to slot granularity.

Option 4:

- UE Rx – Tx time difference TUE-RX – TUE-TX  can be directly derived from timing advance TTA

  - FFS: the granularity and the reporting range of TA.

  - Note: This implies that the existing framework for Multi-RTT positioning report can be used without need to specify a new TA report.

Note: The impact of UE autonomous adjustment of TA (when applied) should be taken into account

Agreement

Select one (or more) of the following options for the enhancement of gNB Rx-Tx time difference in NTN

Option 1:

- The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX

Where:

For a Transmission Point

  - TgNB-RX is the Transmission and Reception Point (TRP) received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time.

  - TgNB-TX is the TRP transmit timing of the downlink subframe corresponding to uplink subframe #i received from the UE

  - Multiple SRS resources can be used to determine the start of one subframe containing SRS.

FFS: For a Transmission Point different from the serving cell (e.g. a DL-PRS-only TP)

Option 2:

- For RTT measurement in NTN, support gNB report of gNB Rx-Tx time as defined in 38.215 with the following change:

  - Only the SRS resource starting within a subframe can be used to determine the start of the subframe.

Option 3:

- Keep the current gNB Rx-Tx definition, and report an offset which can covers the time duration corresponds to kmac if needed.

Option 4:

- For RTT measurement in NTN, support gNB report that indicates the time difference between the transmit time of a DL RS for positioning and the arrival time of an SRS.

FFS: details of report.

Note: The impact of UE autonomous adjustment of TA (when applied) should be taken into account

Agreement

Study the following options to resolve the mirror positions ambiguity for multi-RTT positioning:

- Option 1: gNB or LMF implementation to solve the mirror error issue.

  - FFS: whether there is spec impact

- Option 2: Reuse existing ECID method (e.g. combine UE neighbor measurements to solve the ambiguity between mirror positions), with potential enhancements

- Option 3: NR NTN UE should report the Doppler calculated on the service link

- Option 4: a VSAT UE should report its beam pointing in respect to satellite beam line of sight

- Option 5: Reporting of cell coverage information (e.g. cell footprint and reference point, or antenna pattern) to the LMF

- Option 6: Support and potentially enhance the optional Rel-17 UL-AoA measurements defined for multi-RTT positioningOther solutions are not precluded

Conclusion

Geometry relating the UE and the TRPs (satellites) affects positioning accuracy for network verified UE location based on Multi-RTT.

## Outcomes of RAN1 study on network verified UE location

During the study carried out in RAN1 on the potential solutions for the network verified UE location, the following the conclusions were made:

Conclusion:

For network verification of UE location in NR NTN with single satellite in view with multi-RTT positioning:

- From RAN1 perspective, if the UE’s Rx-Tx time difference measurements report can be assumed to be trusted, multi-RTT positioning method using Rx-Tx time difference measurements can meet the accuracy requirement of less than 10km with 90% confidence, in case of:

  - At least LEO600 based deployment

  - Earth fixed cells

  - Earth moving cell at least if UE dwell time within the cell is enough to perform at least two RTT measurements

- Note: the required over-the-air latency reported in evaluations ranged from less than 10s up to 180s

Conclusion

For network verification of UE location in NR NTN with single satellite in view with DL-TDOA positioning: From RAN1 perspective, if the UE’s RSTD measurements report can be assumed to be trusted, DL-TDOA positioning method can meet the accuracy requirement of less than 10km with 90% confidence, in case of:

  - At least LEO600 based deployment

  - Earth fixed cells

  - Earth moving cell at least if UE dwell time within the cell is enough to perform at least two RSTD measurements

Note 1: the above is based on evaluation results that didn’t account for UE Clock drift

Note 2: the required over-the-air latency reported in evaluations ranged from less than 20s up to 180s

Note 3: The requirements of Network verification of UE location may not be met if realistic assumption on UE clock drift is considered.

Conclusion

For network verification of UE location in NR NTN based on multi-RTT using UE RX-TX time difference report, if the UE reports needed to perform multi-RTT can be assumed to be trusted, existing multi-RTT framework may be reused with potential enhancements to adapt it to NTN context. This may include, but not limited to:

- If justified: NTN-specific definition of UE RX-TX time difference, including as an example, potential modifications to UE Rx – Tx time difference to enable network verification of UE location without introducing any additional measurements at the UE (with respect to Rel-17 NTN)

  - The following is not precluded: the UE Rx – Tx time difference is defined as TUE-RX – TUE-TX, where TUE-RX – TUE-TX is directly derived from the timing advance TTA applied by the UE at a given subframe.

  - Above does not imply that the relevant work is prioritized.

- Other assistance data (e.g. ephemeris) to be transferred from gNB to the LMF.

- If justified: Other assistance data (e.g. to resolve ambiguity on mirror position issue) to be transferred from UE to LMF

- If justified: Adaptations enabling Rx-TX measurements for Multi-RTT involving multiple cells within the same satellite

For network verification of UE location in NR NTN based on DL-TDOA positioning, if the UE reports needed to perform DL-TDOA positioning can be assumed to be trusted, existing DL-TDOA positioning framework may be reused with potential enhancements to adapt it to NTN context.

# References

- RP-230809 (revision of RP-223534) Revised WID: NR NTN (Non-Terrestrial Networks) enhancements Thales

- 3GPP TS 38.305. « NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 15)  »

- 3GPP TS 38.855. “Study on NR positioning support”

- TS 38.455. NG-RAN; NR Positioning Protocol A (NRPPa)

- Richard B. Langley. "Dilution of Precision" GPS World. May 1999

- https://en.wikipedia.org/wiki/Dilution_of_precision_(navigation)

- 3GPP TS 38.215. Physical layer measurements (Release 17)

- 3GPP TS 38.133. R; Requirements for support of radio resource management (Release 17)

- R1-2210948. Discussion on network verified UE location in NR NTN. Thales. November 2022

- R1-2212137. Network verified UE location for NR NTN. Qualcomm Incorporated. November 2022