3GPP TSG RAN WG1 #112-bis-e                                                           			 R1-2302408

E meeting, April 17 – April 26, 2023

Agenda Item:	9.3.3

Source:	TCL

Title:	Potential Enhancement on dynamic/flexible TDD

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN1 # 112 meeting, several issues related to gNB-to-gNB CLI mitigation, UE-to-UE CLI mitigation, and spatial domain coordination were agreed [2]. In this contribution, we provide our views on the remaining issues concerning gNB-to-gNB CLI measurement, coordinated scheduling for time/frequency resources, and spatial domain coordination.

# gNB-to-gNB co-channel CLI measurement

In RAN1#112 meeting, an agreement was made related gNB to gNB co-channel CLI measurement as given below[3].

Agreement For the study of gNB-to-gNB co-channel interference measurement, it is assumed that both CD-SSB and NCD-SSB can be used for gNB-to-gNB CLI measurement.

Furthermore, there was a discussion about how to use CD-SSB or NCD-SSB to perform gNB-to-gNB CLI measurement, but no consensus was reached. In our view, CD-SSB/NCD-SSB of the current specification are defined from a UE perspective and may not be applicable for gNB-to-gNB CLI measurement. Therefore, an indirect method should be used instead.  In addition, since the victim gNB performs the CLI measurement, it may be necessary for it to know the aggressor gNB to ensure accurate measurements. One simple solution is for the aggressor gNB to exchange information on downlink-muted resources with the victim gNB. However, in cases where there are multiple aggressor gNBs with different muted resources, it might be challenging for the victim gNB to identify the aggressor gNBs. Hence, a detailed mechanism for gNB-to-gNB CLI measurement based on CD-SSB or NCD-SSB should be further studied.

Observation 1: CD-SBB or NCD-SSB are UE specific and may not be directly applicable to measure the gNB-to-gNB co-channel CLI.

Observation 2: To measure gNB-to-gNB CLI, it may be necessary for the victim gNB to identify the aggressor gNB.

Proposal 1: The procedure of gNB-to-gNB co-channel CLI measurement based on the CD-SSB and NCD-SSB can be further studied.

# Coordinated Scheduling for time/frequency resources between gNBs

The discussion of Coordinated scheduling for time/frequency resources between the gNBs were started in RAN1#110bis-e meeting [1], and the details of coordinated scheduling, such as the relevant information exchange, and the scheduling adaptation solutions were discussed in RAN1#111. Furthermore, an agreement was made related to the information exchange among gNBs, which can assist coordinated scheduling in RAN1#112 meeting.  In this section, we provide our views on the discussed and the remaining issues of coordinated scheduling for time/frequency resources between gNBs.

Observations 3: For coordinated scheduling of time frequency resources between the gNBs, it may necessary to consider the relevant information exchange between the aggressor and victim gNBs.

## Relevant information exchange during SBFD operation

The basic requirements of coordinated scheduling for time/frequency resources among gNBs are, to exchange the relevant information between the gNB (such as the time frequency location of SBFD symbols, the time location of dynamic TDD symbols) in order to let the gNB to perform scheduling adaptation and reduce the co-channel CLI.  In 3GPP RAN1#112 meeting [3], an agreement related to relevant information exchange with focus on SBFD related information exchange was agreed as given below.

Agreement Study the benefit of knowledge among gNBs of configurations such as - SBFD time/frequency configuration

In our view, exchanging SBFD time/frequency configuration information may assist gNBs in adapting their scheduling in the time and frequency domains to avoid or reduce gNB-to-gNB CLI. Furthermore, the prior knowledge of the SBFD time/frequency configuration among gNBs may help them to adapt the DL and UL subbands, such as the subband pattern, RBs assigned to DL and UL subbands, and SBFD slots/symbols. Consequently, this adaptation will lead neighboring gNBs to avoid or reduce the effects of gNB-to-gNB co-channel CLI. However, in addition to the SBFD time/frequency configuration, the DL/UL subbands pattern can be considered for exchange among gNBs.

Observation 4: The knowledge among gNBs about the SBFD time/frequency configuration may assist the gNBs to perform scheduling adaptation and mitigate the gNB-to-gNB co-channel CLI.

Observation 5: In addition to the SBFD time/frequency configuration, the exchanging of DL/UL subbands pattern may assist the gNBs for CLI mitigation.

Proposal 2: To assist in mitigating gNB-to-gNB CLI during SBFD operation, consider exchanging the subbands pattern among gNBs.

## Relevant information exchange during SBFD and dynamic TDD operation

In some scenarios, the neighbor gNBs may not perform the SBFD operation during the same time slots, therefore the coordinated scheduling and its information exchange may consider the SBFD operation and dynamic TDD operation. In addition, the relevant information exchange may be different from one scenario to another scenario according to the implementation of SBFD operations and legacy dynamic TDD operation. For instance, if both the aggressor and victim gNBs perform dynamic TDD operation at the same time slots, they may need to exchange the dynamic TDD slots format only. On the other hand, if the aggressor gNB performs dynamic TDD operation and the victim gNB performs SBFD operation at the same time slots, they may need to exchange the dynamic TDD slots format, the SBFD time/frequency configuration, and the subbands pattern.

Observation 6: An aggressor gNB performing dynamic TDD operation may exchange slot format with its victim gNBs, and an aggressor gNB performing SBFD operation may exchange SBFD slot format as well as the starting and numbers of RBs assigned for each DL and UL sub-bands.

Proposal 3: During the simultaneous existence of SBFD and dynamic TDD operations among gNBs, consider at-least the following information exchange among gNBs:

- TDD slot format

- SBFD time/frequency configuration

- Subbands pattern

### 3.2.1 Scheduling Adaptation for simultaneous SBFD and dynamic TDD operations

Based on the information exchange between the aggressor and victim gNBs, the gNBs can perform scheduling adaptation to reduce or avoid the gNB-to-gNB and UE-to-UE CLI. For instance, an aggressor gNB which performs dynamic TDD operation can mute those RBs which corresponds to the RBs of the opposite direction sub-band at a victim gNB, to mitigate the co-channel gNB-to-gNB and UE-to-UE CLI as shown in Figure 1. Furthermore, the gNB, which performs SBFD operation, can assign more sub-bands resources to the same transmission direction of the gNB, which performs dynamic TDD operation, in order to minimize the number of muting RBs at the aggressor gNB.

Figure 1 Scheduling adaptation of dynamic TDD (DL) at gNB1 and SBFD operation at gNB2

In addition, the coordinated scheduling for time frequency resources shall not be restricted to muting or blanking the DL and UL resources only. Several other coordinated scheduling based solutions can be adopted to mitigate CLI. For instance, the time window based solutions can be used by the neighbor gNBs  to reduce the effect of CLI as shown in Figure 2, where the aggressor and victim gNBs can assign a time domain windows of X slots/symbols to the dynamic TDD operation and SBFD operation respectively.

Figure 2 Scheduling adaptation of same direction dynamic TDD (DL) and SBFD operation at gNB1 and gNB2

Observation 7: In coordinated scheduling for time frequency resources between gNBs, muting the DL RBs or blanking the UL RBs can reduce the effect of gNB to gNB co-channel CLI.

Observation 8: In simultaneous implementation of dynamic TDD and SBFD, operation at a gNB and its neighbor gNB the following scheduling adaptation, techniques can reduce or avoid the gNB to gNB co-channel CLI.

- Each gNB can assign a time window to the dynamic TDD operation and a time window to the SBFD operation.

- Allocating the same numbers of slots or symbols in the time windows assigned to the dynamic TDD or SBFD operation across the neighbor gNBs.

Proposal 4: For coordinated scheduling of time frequency resources between gNBs for gNB to gNB co-channel CLI handling, consider at least the following.

- RB based UL and DL Resource muting to support CLI mitigation in dynamic TDD and SBFD operation.

- Time domain window based solution to handle CLI in both dynamic TDD and SBFD operation.

# Spatial Domain coordination

In RAN1#112 meeting, the following agreements reached regarding the spatial domain coordination [2].

Agreement For spatial domain enhancement of gNB-to-gNB co-channel CLI handling, DL Tx beam information of the gNB can be exchanged between gNBs. Reference signal resource ID (e.g., NZP-CSI-RS resource ID, SSB index) can be used as beam information exchange between gNBs. Agreement For spatial domain enhancement of gNB-to-gNB CLI handling, study the benefit and the procedure of the information exchange of at least the preferred/non-preferred DL beams of the aggressor gNBs, based on the beam information exchanged between gNBs

For spatial domain enhancement, the preferred/non-preferred DL beams of the aggressor gNBs, can be exchange among the gNBs to facilitate the gNB-to-gNB CLI mitigation. Since the aggressor gNB create the interference, therefore some knowledge of the victim gNB’s preferred/non preferred UL beam may also be necessary to consider. The victim gNB may share its preferred UL beam, which is assigned to the UL UE. In this way, the aggressor gNB can adjust its DL Tx beam to assist the gNB to gNB CLI mitigation. Furthermore, in addition to the DL Tx beam information based on the RS resource ID (e.g., NZP-CSI-RS resource ID, SSB index), the beams ID, and TCI state may also provide the necessary information about the beams which is used by the neighbor gNB for DL or UL transmission.

Observation 9: the information exchange of at least the preferred/non-preferred DL beams of the aggressor gNBs can help the victim gNB to identify which beam can create CLI.

Proposal 5: Consider the information exchange of the preferred/restricted DL and UL beams of the aggressor and victim gNBs with each other, based on the beam ID and TCI state.

# Conclusion

In this contribution, we have discussed gNB-to-gNB CLI measurement, coordinated scheduling for time/frequency resources, relevant information exchange during SBFD operation and simultaneous dynamic TDD and SBFD operation, and spatial domain coordination. Based on our discussion, we have made the following observations and proposals.

Observation 1: CD-SBB or NCD-SSB are UE specific and may not be directly applicable to measure the gNB-to-gNB co-channel CLI.

Observation 2: To measure gNB-to-gNB CLI, it may be necessary for the victim gNB to identify the aggressor gNB.

Observations 3: For coordinated scheduling of time frequency resources between the gNBs, it may necessary to consider the relevant information exchange between the aggressor and victim gNBs.

Observation 4: The knowledge among gNBs about the SBFD time/frequency configuration may assist the gNBs to perform scheduling adaptation and mitigate the gNB-to-gNB co-channel CLI.

Observation 5: In addition to the SBFD time/frequency configuration, the exchanging of DL/UL subbands pattern may assist the gNBs for CLI mitigation.

Observation 6: An aggressor gNB performing dynamic TDD operation may exchange slot format with its victim gNBs, and an aggressor gNB performing SBFD operation may exchange SBFD slot format as well as the starting and numbers of RBs assigned for each DL and UL sub-bands.

Observation 7: In coordinated scheduling for time frequency resources between gNBs, muting the DL RBs or blanking the UL RBs can reduce the effect of gNB to gNB co-channel CLI.

Observation 8: In simultaneous implementation of dynamic TDD and SBFD, operation at a gNB and its neighbor gNB the following scheduling adaptation, techniques can reduce or avoid the gNB to gNB co-channel CLI.

- Each gNB can assign a time window to the dynamic TDD operation and a time window to the SBFD operation.

- Allocating the same numbers of slots or symbols in the time windows assigned to the dynamic TDD or SBFD operation across the neighbor gNBs.

Observation 9: the information exchange of at least the preferred/non-preferred DL beams of the aggressor gNBs can help the victim gNB to identify which beam can create CLI.

Proposal 1: The procedure of gNB-to-gNB co-channel CLI measurement based on the CD-SSB and NCD-SSB can be further studied.

Proposal 2: To assist in mitigating gNB-to-gNB CLI during SBFD operation, consider exchanging the subbands pattern among gNBs.

Proposal 3: During the simultaneous existence of SBFD and dynamic TDD operations among gNBs, consider at-least the following information exchange among gNBs:

- TDD slot format

- SBFD time/frequency configuration

- Subbands pattern

Proposal 4: For coordinated scheduling of time frequency resources between gNBs for gNB to gNB co-channel CLI handling, consider at least the following.

- RB based UL and DL Resource muting to support CLI mitigation in dynamic TDD and SBFD operation.

- Time domain window based solution to handle CLI in both dynamic TDD and SBFD operation.

Proposal 5: Consider the information exchange of the preferred/restricted DL and UL beams of the aggressor and victim gNBs with each other, based on the beam ID and TCI state.

# References

Chairman notes of 3GPP TSG RAN WG1 #110-bis (Toulouse, France

Draft Report of 3GPP TSG RAN WG1 #111 (Toulouse, France, 14th – 18th November 2022)

Chairman notes of 3GPP TSG RAN WG1 #112 ( Athens, Greece, Feb 27th – March 3rd 2023)

R1-2301978, Summary of potential enhancement on dynamic/flexible TDD (Moderator, LG Electronics)