3GPP TSG RAN WG1 Meeting # 112-bis-e 				                              R1-2302409

E-meeting, April 17th – April 26th, 2023

Agenda Item:	9.13.3

Source:	TCL

Title:	L1 signal design and procedure for low power WUS

Document for:	Discussion and Decision

# Introduction

In 3GPP RAN1#112 meeting [3], several issues related to the LP-WUS signal waveform, bandwidth and of LP-WUS, content of LP-WUS, synchronization of LP-WUR, and LP-WUR based RRM measurement were discussed. In this contribution, we provide our views on the remaining issues related to the LP-WUR synchronization, the LP-WUS monitoring procedure, the bandwidth of LP-WUS, and the LP-WUR based RRM measurement.

# Synchronization of LP-WUR

In RAN1#112 meeting, an agreement related to the synchronization of  LP-WUR was agreed as given below [3].

Agreement Study synchronization signal used by LP-WUR, if needed, based on - Option 1: aperiodic signal transmitted as part of LP-WUS - FFS: Whether the signal can additionally be transmitted separately from LP-WUS - Option 2: periodic signal transmitted separately from LP-WUS - Option 3: Option1 + Option2

In our view, option 1 involves transmitting an aperiodic signal as part of the LP-WUS, which may be suitable for UE-specific or UEs group-specific based LP-WUR synchronization. However, this signal design may include UE ID/UE group ID payload, increasing the complexity and potentially adding overhead in case multiple UEs need to synchronize. In addition, synchronization of LP-WUR through option 1 may also vary slightly between UEs.

Option 2 involves transmitting a periodic signal separately from the LP-WUS. This approach allows all LP-WURs of UEs to receive the same synchronization signals, reducing the overhead of these signals. The periodic signal transmitted separately from LP-WUS can be broadcasted (just like SSB), and carries the essential synchronization and acquisition information of the cell. All UEs within the coverage area can receive and decode this periodic synchronization signal to synchronize their LP-WUR the network, establish LP-WUR-based communication with the cell, and subsequently receive and decode the LP-WUS.

Observation 1: Aperiodic signal transmitted as part of LP-WUS may contains UE ID/UE group ID payload which could increase complexity and overhead if multiple UEs need to synchronize, and the synchronization may vary slightly between UEs.

Observation 2:  Periodic signal transmitted separately from LP-WUS reduces the overhead of synchronization signals and allows all the UE’s LP-WURs to receive the same synchronization signals.

Proposal 1: Support both options for the LP-WUR synchronization and prefer option 2:

- Option 1: Aperiodic signal transmitted as part of LP-WUS

- Option 2: Periodic signal transmitted separately from LP-WUS

# LP-WUS/LP-WUR monitoring procedure

In RAN1#112 meeting, an agreement related to the monitoring behavior of LP-WUR was agreed as given below [3].

Agreement Study further pros and cons of the following monitoring behaviors of LP-WUR - Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states - Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time

In this section, we provide our view on the duty cycle based monitoring behavior of LP-WUR, and continuous monitoring behavior of LP-WUR.

## Duty cycle based monitoring of LP-WUR

The duty cycle-based monitoring behavior of LP-WUR relies on switching the LP-WUR ON for a specific duration and OFF for specific duration, resulting in reduced power consumption. However, duty cycle based monitoring may encounter the following two issues;

- If LP-WUS is transmitted periodically, there can be a mismatch between the LP-WUR ON duration and the LP-WUS periodicity.

- If LP-WUS is non periodic and configured for a specific occasion, there can be a mismatch between the LP-WUR ON duration and the configured LP-WUS.

These issues can be solved by deriving the ON/OFF period of the LP-WUR from DRX and eDRx configuration of the current specification. For instance, if the UE's DRX ON duration is 1280 ms, the LP-WUR duty cycle-based monitoring can be ON for 1280 ms. In addition, the periodicity of LP-SS and LP-WUS can also be defined according to the configured DRx cycle to avoid these mismatches.

Observation 3: Duty cycle based monitoring of LP-WUR for LP-WUS detection may encounter a mismatch between the LP-WUR ON duration and the periodic LP-WUS or configured LP-WUS in a specific occasion.

Proposal 2: For Duty cycle based LP-WUS monitoring, the LP-WUR ON/OFF duration can be associated with the current DRX and e-DRX durations.

## Continuous monitoring of LP-WUR

Continuous monitoring of LP-WUR for LP-WUS detection can result in unnecessary power consumption. This is because the LP-WUR may decode all periodic or configured LP-WUS, even if they do not contain that UE MR Trigger information. Similarly, continuous monitoring of LP-WUS for a -configured LP-WUS in a specific occasion can consume power when no LP-WUS is transmitted. To reduce power consumption, an optimized LP-WUR monitoring behavior is required. For instance, the monitoring can be activated for a duration when the UE is triggered by the network to wake up and disabled during the time from the first trigger until the UE is again triggered by the network to wake up.as explained below.

## 3.2.1 Activation/De-activation of LP-WUS monitoring

In RAN1#112 meeting,  a proposal regarding the activation and de-activation of LP-WUS monitoring was discussed and summarized by the FL summary as given below [4]. However, no consensus was achieved.

Proposal-14: Study further the following LP-WUS monitoring activation and deactivation procedures - Option 1: LP-WUS is activated/deactivated by gNB - e.g. SIB or Dedicated RRC - Option 2: LP-WUS activated/deactivated implicitly based on criteria - Option 3: LP-WUS activated by UE autonomously - FFS need for informing gNB

In our view, the signaling mentioned in option 1 (SIB or dedicated RRC) may be feasible for de-activating the LP-WUS monitoring but not feasible for activating the LP-WUS monitoring. This is because before LP-WUS monitoring is activated, the UE's MR is in ultra-deep sleep and cannot receive SIB or dedicated RRC signals. In the same way, option 2 may not be feasible for the activation of LP-WUS monitoring, since the activation requires a trigger from gNB. However, for de-activation, option 2 can be used based on the MR ON status, which can be used implicitly to de-activate the LP-WUS monitoring. In other words, when the MR is ON the UE can de-activate the LP-WUS monitoring. For option 3, UE may not be able to activate the LP-WUS monitoring autonomously, since the UE do not know when the gNB will trigger the LP-WUR of UE to wake up the MR.

If a periodic LP-SS is agreed for synchronization of the LP-WUR with the network, it can carry an extra payload in the form of a bitmap to activate or deactivate the LP-WUS monitoring. For instance, the payload in the LP-SS may contain a bitmap where each bit corresponds to the activation or deactivation of a UE or group of UEs based on their ID. As a result, the LP-WUR only monitors the LP-SS periodically and detects the LP-WUS on a specific occasion when the monitoring is activated by the network through the LP-SS. This behavior avoids continuous monitoring for LP-WUS detection and unnecessary decoding of each LP-WUS, as shown in figure 1.

Figure 1 LP-WUR Avoids unnecessary LP-WUS detection

Observation 4: The monitoring behavior of LP-WUR for LP-WUS detection greatly impact the power consumption of LP-WUR, and the activation/de-activation of LP-WUS monitoring can avoid continues monitoring, and unnecessary decoding of each LP-WUS.

Observation 5: The following options may not be feasible for LP-WUS activation.

- Before the LP-WUS activation the UE's MR is in ultra-deep sleep and cannot receive SIB or dedicated RRC signals.

- LP-WUS activated implicitly based on criteria is not feasible, since the activation of LP-WUS monitoring may requires a trigger from gNB

- UE may not be able to activate the LP-WUS monitoring autonomously, since the UE do not know when the gNB will trigger the LP-WUR of UE to wake up the MR.

Observation 6:  The following options may be feasible for LP-WUS de-activation

- SIB or dedicated RRC

- LP-WUS monitoring de-activated based on criteria or autonomously such as the UE can use the MR ON status to de-activate the LP-WUS

Proposal 3: For activation/de-activation of LP-WUS monitoring, study an activation/de-activation trigger from gNB, such as a payload in the form of bitmap carries in the LP-SS.

Proposal 4: For De-activation of LP-WUS monitoring at least the following options can be studied

- SIB or dedicated RRC

- Implicit or UE autonomous De-activation based on the MR ON status

## Procedures for MR upon wake-up from ultra-deep sleep

In RAN1#112 meeting, a proposal regarding the procedure of MR after wake up from ultra-deep sleep was summarized by the FL summary as given below [4]. However, no consensus was achieved in the previous meeting.

Proposal-13: Study further pros and cons of the following procedures of MR wake-up from ultra-deep sleep - perform PO monitoring, and afterwards follow legacy procedures - perform PEI monitoring, and afterwards follow legacy procedures - transmit PRACH for initial access, and follow legacy procedures

In Rel-17, when a UE is triggered by the network and wake up from deep sleep, it performs PEI monitoring before a PO that is transmitted to inform the UE about the incoming paging.  Since PEI is detected by the MR of UE, in Rel-18, LP-WUS may be transmitted before PEI to trigger the MR to wake up and be ready to receive the PEI. In this case, the MR of UE is triggered two times; once by LP-WUS and once by PEI, resulting in high power consumption compared to the Rel-17 PEI based paging as shown in Figure 3. Based on this observation, there may not be a significant power saving for UE by using LP-WUS and LP-WUR in idle/inactive state.  Therefore, LP-WUS may be used as an alternative option to replace PEI. In other words, LP-WUS can be used as a paging indication in idle/inactive states and can replace PEI.

Regarding the procedure related to PRACH after the MR wake up from ultra-deep sleep, it may be consider in scenarios. For instance, when a UE is in ultra-deep sleep and it move from one location to another location, it may lose connection and synchronization with the serving cell. In such cases when the MR wakes up from ultra-deep sleep, the MR may need to initiate the PRACH by performing a random access procedure to establish or re-establish its connection re-establish and synchronization with a cell.

Figure 3 LP-WUS in idle/inactive state

Observation 7: In idle/inactive state the MR of UE maybe trigger by two times, one time by LP-WUS to wake up and one time by PEI to indicate paging, which may increase the UE power consumption.

Observation 8: In idle/inactive state the MR trigger of a UE via the LP-WUS can be consider as an indication for a UE to monitor the upcoming PO, which can replace the Rel-17 PEI.

Proposal 5: Consider the LP-WUS as an alternative option for paging early indication.

Proposal 6: The following procedures can be considered after the MR of UE wake-up from ultra-deep sleep

- Perform PO monitoring, and afterwards follow legacy procedures

- Transmit PRACH for initial access, and follow legacy procedures

## LP-WUS Indication

Due to the simple architecture and low sensitivity of the LP-WUR, the coverage of LP-WUS may be degraded as compared to the legacy NR channel. In order to ensure the reliability of the LP-WUS an indication from the MR of UE can be used to let the gNB know that the LP-WUS has successfully received and decoded by the LP-WUR of the UE. This indication can be Implicit, where the gNB implicitly derive from the first ACK message (such as HARQ-ACK) received from the MR of UE after the UE receives data/signaling as shown in Figure 4a, or it can be an explicit indication, where the MR of UE sends an ACK message to the gNB and let the gNB know that the LP-WUS has successfully received as shown in Figure 4b. The implicit indication can reduce the false rate of ACK message to the gNB. However, the physical resources (time/frequency resources) may be wasted in case the gNB transmit the data/signaling after LP-WUS without knowing the MR on/off status.  The explicit indication of LP-WUS reception will enhance the resource utilization, however it may increase the rate of false ACK message to the gNB in case the LP-WUR of a UE decode a noise signal and consider it as an LP-WUS.

Figure 4a Implicit Indication of LP-WUS reception        Figure 4b Explicit Indication of LP-WUS reception

Observation 9: The implicit or explicit indication for the LP-WUS reception from the MR of a UE may enhance the LP-WUS reliability.

Proposal 7: Study an indication method for LP-WUS reception to ensure the successful detection of LP-WUS and improve its reliability.

# BW Configuration for LP-WUS

Regarding the BW size of LP-WUS, the following proposal was summarized by the FL summary in RAN1#112 meeting [4], but no agreement was achieved.

Proposal-6: To reduce complexity of implementation, number of supported BW sizes should be limited to maximum [2 or 3] BW sizes. - from set of agreed BW sizes, different size can be configured for Idle/Inactive and Connected mode. - guard bands are included in the BW size - FFS: sizes - FFS: whether sizes are defined in multiples of PRBs or SCs - FFS: whether set of BW sizes is defined per SCS or is applicable to all SCS

In our view, the sizes of BWP for LP-WUS can be defined in terms of frequency units such as 5 MHz or 20 MHz and the configuration of the BW size for LP-WUS can be considered in terms of PRBs, which is more feasible to configure the BW of LP-WUS using the existing signaling such as through RRC configuration or through SIBx.

Furthermore, in RAN1#112 meeting, regarding the placement of LP-WUS within a carrier, several companies proposed different options for the placement of LP-WUS in carrier/BWP as summarized in FL summary [4].

Proposal-7: LP-WUS can be configured in-band, i.e. within usable PRBs of a carrier/BWP - Option 1: placement within carrier/BWP is flexible. - Option 2: placement within carrier BWP is restricted to the middle of carrier - Option 3: placement within carrier BWP is restricted to at the edge(s) of a carrier

In our view, option 1 is the more feasible option since it is more flexible to place the LP-WUS at the lower edge or upper edge of the carrier BWP or according to empty PRBs in the carrier which do not affect the legacy NR channel. For option 2, it restricts the LP-WUS to the middle of a carrier BWP which may create holes in the middle of the carrier's bandwidth for the legacy NR channel and waste the frequency resources. Option 3 is also a feasible option since it can avoid the holes between the carrier's bandwidth for the legacy NR channel.

Furthermore, in case periodic LP-SS is used for synchronization of LP-WUR, and the LP-SS and LP-WUS  are configured in different BWP,  then the LP-WUR has to perform RF retuning and BWP switching for synchronization before LP-WUS detection as shown in Figure 5. This behavior may increase the LP-WUR power consumption.  In order to solve this issue, a dedicated downlink BWP which may contains both the LP-SS and LP-WUS with the maximum bandwidth less than or equal to the required bandwidth of the LP-WUS, can be configured to the UE. Since the dedicated downlink BWP contains the LP-SS to synchronize the LP-WUR with the network and LP-WUS to trigger the MR to receive data/signaling, therefore the LP-WUR may avoid the RF retuning for LP-SS.

Figure 5 LP-WUR RF retuning for LP-SS and LP-WUS

Observation 10: The flexible bandwidth configuration of LP-WUS in the carrier BWP, or restricting the LP-WUS to the lower edge or upper edge of the carrier BWP, may avoid holes between the carrier's bandwidth for the legacy NR channel.

Observation 11: Restricting the placement of LP-WUS to the middle of carrier BWP, creates holes in the middle of the carrier bandwidth for NR legacy channel resulting in frequency resource wasting.

Observation 12: The transmission of LP-SS and LP-WUS in different BWP, may let the LP-WUR to perform RF retuning for LP-SS, and increase its power consumption.

Proposal 8: Consider at least the following two options for the placement of LP-WUS in the carrier bandwidth.

- Placement within carrier/BWP is flexible.

- Placement within carrier BWP is restricted to at the edge(s) of a carrier

Proposal 9: Study a dedicated BWP for the placement of LP-WUS and LP-SS, with the maximum bandwidth less than or equal to the required bandwidth of the LP-WUS.

# RRM measurement of LP-WUR

In RAN1#111 meeting, LP-WUR based RRM measurement was discussed to support mobility in idle/inactive mode and the following agreement was achieved [2].

Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighboring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE)

In RAN1#112 meeting, LP-WUR based RRM measurement was discussed to support mobility in idle/inactive mode and the following agreement was achieved [2].

Agreement Study potential measurement metric used for RRM measurements performed by LP-WUR. - examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal - companies to report assumption of signal used for measurements

In our view, LP-WUR based RRM measurement may consider more RRM relaxation on the top of the existing relaxed RRM measurement of TR 38.380, to keep the low power consumption feature of the LP-WUR. In our view, the following rules of RRM measurement relaxation can be further studied.

Measurement only:   Since the LP-WUR architecture may not support transmission, therefore RRM activities of the LP-WUR can be limited to the RRM measurement only. However, when RRM reporting is required, the LP-WUR can trigger the MR to perform RRM reporting to the gNB. In order to further reduce the RRM reporting, a range of RSRP/RSRQ with a maximum and minimum threshold values can be defined.  When the values of RSRP/RSRQ is beyond the range of the threshold, the MR of UE can perform the RRM reporting.

RRM relaxation based on the UEs group:  This relaxation may be more feasible for stationary UEs such as industrial wireless sensors, where several stationary UEs can be grouped together based on their geographical area, and a relaxed RRM measurement period can be defined. A single UE in the group can perform RRM measurement in one RRM measurement period, which is considered for all other UEs in the group. Similarly, a next UE can perform RRM measurement in a next RRM measurement period and so on. In this way, the RRM measurement of each UE can be relaxed in time domain.

RRM measurement based on the LP-WUS: Since the LP-WUS for a UE can be configured in a specific time duration when it is required. Therefore RRM measurement based on the LP-WUS itself can further relax the RRM measurement of the LP-WUR.

Observation 13: LP-WUR based dedicated RRM measurement relaxation may reduce the LP-WUR power consumption.

Proposal 10: Study at least the following RRM measurement relaxation for the LP-WUR based RRM measurement.

- RRM measurement without reporting

- RRM measurement based on the UEs group

- RRM measurement based on the LP-WUS

# Conclusion

In this contribution we discussed, the LP-WUS monitoring procedure, the bandwidth of LP-WUS, and the LP-WUR based RRM measurement, and made the following observations and proposals.

Observation 1: Aperiodic signal transmitted as part of LP-WUS may contains UE ID/UE group ID payload which could increase complexity and overhead if multiple UEs need to synchronize, and the synchronization may vary slightly between UEs.

Observation 2:  Periodic signal transmitted separately from LP-WUS reduces the overhead of synchronization signals and allows all the UE’s LP-WURs to receive the same synchronization signals.

Observation 3: Duty cycle based monitoring of LP-WUR for LP-WUS detection may encounter a mismatch between the LP-WUR ON duration and the periodic LP-WUS or configured LP-WUS in a specific occasion.

Observation 4: The monitoring behavior of LP-WUR for LP-WUS detection greatly impact the power consumption of LP-WUR, and the activation/de-activation of LP-WUS monitoring can avoid continues monitoring, and unnecessary decoding of each LP-WUS.

Observation 5: The following options may not be feasible for LP-WUS activation.

- Before the LP-WUS activation the UE's MR is in ultra-deep sleep and cannot receive SIB or dedicated RRC signals.

- LP-WUS activated implicitly based on criteria is not feasible, since the activation of LP-WUS monitoring may requires a trigger from gNB

- UE may not be able to activate the LP-WUS monitoring autonomously, since the UE do not know when the gNB will trigger the LP-WUR of UE to wake up the MR.

Observation 6:  The following options may be feasible for LP-WUS de-activation

- SIB or dedicated RRC

- LP-WUS monitoring de-activated based on criteria or autonomously such as the UE can use the MR ON status to de-activate the LP-WUS

Observation 7: In idle/inactive state the MR of UE maybe trigger by two times, one time by LP-WUS to wake up and one time by PEI to indicate paging, which may increase the UE power consumption.

Observation 8: In idle/inactive state the MR trigger of a UE via the LP-WUS can be consider as an indication for a UE to monitor the upcoming PO, which can replace the Rel-17 PEI.

Observation 9: The implicit or explicit indication for the LP-WUS reception from the MR of a UE may enhance the LP-WUS reliability.

Observation 10: The flexible bandwidth configuration of LP-WUS in the carrier BWP, or restricting the LP-WUS to the lower edge or upper edge of the carrier BWP, may avoid holes between the carrier's bandwidth for the legacy NR channel.

Observation 11: Restricting the placement of LP-WUS to the middle of carrier BWP, creates holes in the middle of the carrier bandwidth for NR legacy channel resulting in frequency resource wasting.

Observation 12: The transmission of LP-SS and LP-WUS in different BWP, may let the LP-WUR to perform RF retuning for LP-SS, and increase its power consumption.

Observation 13: LP-WUR based dedicated RRM measurement relaxation may reduce the LP-WUR power consumption.

Proposal 1: Support both options for the LP-WUR synchronization and prefer option 2:

- Option 1: Aperiodic signal transmitted as part of LP-WUS

- Option 2: Periodic signal transmitted separately from LP-WUS

Proposal 2: For Duty cycle based LP-WUS monitoring, the LP-WUR ON/OFF duration can be associated with the current DRX and e-DRX durations.

Proposal 3: For activation/de-activation of LP-WUS monitoring, study an activation/de-activation trigger from gNB, such as a payload in the form of bitmap carries in the LP-SS.

Proposal 4: For De-activation of LP-WUS monitoring at least the following options can be studied

- SIB or dedicated RRC

- Implicit or UE autonomous De-activation based on the MR ON status

Proposal 5: Consider the LP-WUS as an alternative option for paging early indication.

Proposal 6: The following procedures can be considered after the MR of UE wake-up from ultra-deep sleep

- Perform PO monitoring, and afterwards follow legacy procedures

- Transmit PRACH for initial access, and follow legacy procedures

Proposal 7: Study an indication method for LP-WUS reception to ensure the successful detection of LP-WUS and improve its reliability.

Proposal 8: Consider at least the following two options for the placement of LP-WUS in the carrier bandwidth.

- Placement within carrier/BWP is flexible.

- Placement within carrier BWP is restricted to at the edge(s) of a carrier

Proposal 9: Study a dedicated BWP for the placement of LP-WUS and LP-SS, with the maximum bandwidth less than or equal to the required bandwidth of the LP-WUS.

Proposal 10: Study at least the following RRM measurement relaxation for the LP-WUR based RRM measurement.

- RRM measurement without reporting

- RRM measurement based on the UEs group

- RRM measurement based on the LP-WUS

# References

RP-213645, “Study on low-power wake-up Signal and Receiver for NR”, 3GPP TSG RAN#94-e, Dec 6th – 17th, 2021.

Draft Report of 3GPP TSG RAN WG1 #111 v0.2.0(Toulouse, France, 14th – 18th November 2022)

Chairman notes of 3GPP TSG RAN WG1 #112 v0.2.0(Athens, Greece, Feb 27th – March 3rd 2023)

R1-2302213, Final summary of discussion on L1 signal design and procedure for LP-WUS, (Moderator Nordic) 3GPP TSG RAN WG1#112 meeting , (Athens, Greece Feb 27th – March 3rd 2023)