3GPP TSG-RAN WG1 Meeting #112bis-e	Tdoc R1-2302411

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.1.1

Source:	Ericsson

Title:	Unified TCI framework extension for multi-TRP

Document for:	Discussion

# 1	Introduction

During RAN#94e, a new WID for Rel-18 MIMO evolution for DL and UL was agreed. [1].

In this contribution, we discuss how the unified TCI framework introduced in Rel-17 can be extended to fulfill these three objectives.

# 2	Discussion

Objective 2 in the Rel-18 WID states that the Rel-17 TCI framework should be extended to support indication of multiple DL and UL TCI states. The focus should be on multi-TRP, but the objective is not limited to multi-TRP: we should support unified solution that works also for single-TRP, unless it leads to complications.

On high level, we want to support:

single-DCI multi-TRP, where single-TRP operation is a special case.

multi-DCI multi-TRP

HST-SFN

STxMP: (Simultaneous multi-panel transmission)

Preferably, the solutions for these scenarios should be similar.

## 2.1	General framework for unified TCI extension

In the Rel-17 TCI framework, the network indicates either one joint DL/UL TCI state, or one DL TCI state and one UL TCI state. A channel/signal is then RRC configured to follow (or not to follow) the indicated TCI state(s). This configuration possibility exists for most channels, except for PDSCH and PUSCH, which always follow the indicated TCI state.

The underlying thought is that the indicated TCI state(s) provides one or two pipes, and a channel/signal can then be fed into this pipe. The TCI state update is separated from the individual channels, and the TCI state is signalled to the UE, rather than for a particular channel, which was the paradigm in Rel-15.

## 2.2	Supporting PDSCH-CJT

In RAN1#111, the following was agreed:

Agreement

On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT):

- Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA

- Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state

- Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB

Basically, Alt1 corresponds to ‘sfnSchemeA’, whereas Alt2 corresponds to ‘sfnSchemeB’. For Alt2, it is assumed that the network has performed frequency pre-compensation (for the second TCI state) at a TRP associate to the second TCI state, so that the {Doppler shift, Doppler spread} of the QCL source and the corresponding DMRS are not the same. Atl.3 is applicable when time delay pre-compensation is performed at a TRP associated to the second TCI state and thus, the {average delay, delay spread} of the QCL source RS and the corresponding DMRS are not the same.

In practice, all three alternatives are relevant, and should be supported:

For a UE that supports two indicated joint/DL states for PDSCH-CJT, support all three alternatives.

The network provides the UE with a configuration on which QCL assumption to apply.

## 2.3	TCI state update and activation

In RAN1#112, the following was agreed:

Agreement

On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings:

- If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2

- If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2

- If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2

- FFS: Whether and how to use the codepoint "11" of the [TCI selection field]

If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2.

- Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold.

If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold

- FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2

FFS: Detail of the capability of two default beams for S-DCI based MTRP

FFS: The threshold value

The remaining issue for this agreement is related to the behaviour for a UE in FR2 that does not support two default beams. Here we propose to reuse the configuration option in Proposal 4 in section 2.4. Since the UE does not support two default beams, the RRC parameter cannot be configured to the value {‘both’}:

For a UE in FR2 that does not support two default beams, if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold, the indicated joint/DL TCI state the UE applies is configured in RRC.

The agreement also includes FFSs on UE capabilities, both on default beams and thresholds. To simplify the handling of UE capabilities, it would make sense to define new capabilities, i.e., not to reuse the Rel-15 and Rel-16 capabilities:

Define new capabilities for the threshold, and support for two default beams, i.e., do not reuse the Rel-15 capability timeDurationForQcl, or the Rel-16 capability defaultQCL-TwoTCI-r16.

Note that the two new capabilities will be quite similar to those defined in Rel-15 and Rel-16.

In RAN1#112, the following was agreed:

Agreement

On unified TCI framework extension for S-DCI based MTRP, down-select at least one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]:

- Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception

- Alt2: The UE shall apply the first one of two indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception

- Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception

- Alt3A: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception

- Alt4: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field of the most recently applied beam indication DCI

Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold (if the threshold is needed)

Default indicated TCI states for PDSCH should be defined for all multi-TRP scenarios where there are two indicated DL/joint TCI states:

NC-JT with and without multi-TRP PDCCH repetition

multi-TRP PDSCH repetition

multi-TRP PDCCH repetition with single-TRP transmission for PDSCH

HST-SFN PDCCH

multi-TRP PUCCH/PUSCH repetition

STxMP

Alt1 provides the most flexible configuration and is aligned with the method agreed for PUCCH and PDCCH. Also, it is aligned with the behaviour agreed for the TCI selection field, where the values {00,01,10} are used to indicated {first, second, both}. Note that this is conditioned on that the UE supports two default beams in FR2.

Alt2 would not enable multi-TRP operation without the TCI selection field – only single-TRP operation would be possible. It is thus not operational.

Alt3 would work fine for NC-JT and multi-TRP PDSCH repetition, where the indicated TCI states are always applied to PDSCH reception. However, it would not work for single-TRP PDSCH transmission with multi-TRP PDCCH repetition, HST-SFN PDCCH, multi-TRP PUCCH/PUSCH repetition or STxMP because the UE would be indicated with two TCI states, and the UE could not apply both to the PDSCH reception. This is illustrated in Figure 1 for PDCCH repetition and HST-SFN PDCCH.

Figure 1: Illustration of PDCCH repetition and HST-SFN transmission. The default indicated TCI state for PDSCH is undetermined for these two cases.

Alt4 has similar disadvantages of Alt3: it is not possible to operate a single-TRP PDSCH with two indicated TCI states.

Based on this, we prefer Alt1, and thus propose to reuse the RRC signalling scheme that was already agreed for PUCCH:

Introduce an RRC parameter with the value {first, second, both} to determine the default indicated TCI states for PDSCH reception.

One reason for using RRC configuration, and not fixed default TCI state(s) for single TRP operation with two indicated unified TCI states is that it will limit the flexibility. For example, assume that we want to perform single TRP operation for PDSCH from one TRP (i.e., the first indicated unified TCI state) and single TRP operation for PUSCH to a second TRP (i.e., the second indicated unified TCI state), and multi-TRP operation for PDCCH and PUCCH. This could be a common scenario, since for example we might have a HetNet scenario, where we want to perform DL transmission from a high-power (macro) TRP, and UL reception at the low-power (pico) TRP, and we want to have high reliability multi-TRP transmission and reception schemes for the two control channels PDCCH and PUCCH. In case we have default fixed TCI state for PDSCH, this operation would not be possible. However, in case we have RRC configuration, where we can indicate which indicated unified TCI state the UE should apply for single-TRP PDSCH reception, this would still be possible.

For a PDSCH scheduled by DCI format 1_0, we still propose to rely on the RRC configuration in Proposal 4. However, for DCI format 1_0, only single-layer reception is possible, and the UE cannot receive PDSCH from both TRPs. Based on this, we instead propose

For reception of PDSCH scheduled by DCI format 1_0, the UE follows the RRC parameter in Proposal 4. If the RRC parameter indicates ‘both’, the UE follows the first indicated TCI state.

There is also a need to map indicated TCI states to DMRS ports. We propose to reuse the rule from Rel-16:

For NC-JT, the first indicated TCI state would be associated with the first DMRS CDM group, and the second TCI state would be associated with the second DMRS CDM group.

One of the remaining issues for the multi-TRP extension of unified TCI is if the TCI field in DCI should be increased. The motivation for such an increase is that the flexibility is not large enough to allow DCI switching in case of multi-TRP.

However, we do not see that any reasonable increase in the number of TCI codepoints would be sufficient: in fact, it is questionable if 8 is large enough for single-TRP. The main mode of operation for TCI state switching is and will be MAC CE. Therefore, we prefer not to complicate the design and propose

Do not increase the number of TCI codepoints.

## 2.4	How to map the indicated TCI state(s) to target channel(s)/signal(s)

In section 2.3, we described how the UE could be provided with multiple TCI states. In this section, we describe how to map each channel/signal into one of the indicated TCI states, i.e., into one of the pipes.

RAN1 made the following agreements:

Agreement (RAN1#110)

On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following:

- Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported)

Agreement (RAN1#111)

On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group

- Note: Detail of the RRC configuration is left to RAN2 design

Hence, RAN1 has agreed to introduce pointers that determine how to map PDCCH and PUCCH to the indicated TCI states. We propose to apply this solution to other channels as well:

For each channel/signal, introduce a pointer that indicates which of the indicated TCI states the UE would use for the corresponding channel/signal.

In Proposal 4, we proposed to apply this principle for the default indicated TCI states for PDSCH.

In RAN1#112, RAN1 made the following agreement on the association for PUSCH:

Agreement

On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG):

- If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s)

- If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s)

- If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator:

  - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214)

  - FFS: SDM and SFN based PUSCH Tx schemes

FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission

The first FFS in the agreement mentions SDM and SFN-based PUSCH Tx schemes. Here we note that SDM and SFN-based Tx schemes would be separately configured using RRC, and there is no problem to reuse the SRS resource indicator also for these schemes. Hence, we propose

Reuse the “SRS resource set indicator” field to inform which indicated TCI states to apply for SDM-based and SFN-based STxMP PUSCH transmission.

The proposed mapping is indicated in Table 1 both for SDM and SFN-based Tx schemes.

Table 1: Possible TCI state indication with the existing SRS resource set indicator field in DCI for STxMP. Note that the SRS resource set field indicates both the TCI state and the SRS resource set.

| SRS resource set Indicator | 1st SRI field | 2nd SRI field |
| --- | --- | --- |
| 0 | 1st SRS resource set, 1st TCI |  |
| 1 | 2nd SRS resource set, 2nd TCI |  |
| 2 | 1st SRS resource set, 1st TCI | 2nd SRS resource set, 2nd TCI |
| 3 | Not applicable | Not applicable |

We also note there is an unused DCI codepoint that could have been used to dynamically switch between SDM and SFN:

There is an unused DCI code point in Table 1 that could have been used to dynamically switch between SDM and SFN.

Since the indicated TCI state is signalled directly, and not via an SRS resource set, we realize that Proposal 9 is not applicable to CG Type 1. Here we propose to introduce an indicator in configured grant RRC configuration which of the indicated TCI states would be used:

Introduce an indicator in the configured grant configuration stating which of the indicated TCI states should be applied.

Note that both indicated TCI states would be applicable for STxMP SFN – so the indicator would be {first, second, both}.

In addition, we need to define which indicated TCI state is used when PUSCH is scheduled by DCI format 0_0 for single-DCI based multi-TRP. Since the order of the TCI states is anyway determined by the network, the first TCI state could be used:

For a PUSCH scheduled by DCI format 0_0 in single-DCI based multi-TRP, the UE uses the first indicated TCI state.

In RAN1#111, RAN1 made the following agreement on the association between indicated TCI states and PUCCH resources:

Agreement

On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group

- Note: Detail of the RRC configuration is left to RAN2 design

In RAN1#112, RAN1 formulated options for the corresponding multi-DCI based multi-TRP functionality:

Agreement

On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission:

- Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission

- Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

- Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted.

  - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint

- Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted.

Note: Either Opt1 or Opt2 must be supported

We note that in Rel-16 multi-TRP multi-DCI, the spatial relation of the PUCCH is not associated with the CORESETPoolIndex: the spatial relation is configured explicitly in the PUCCH resource, just as for single-DCI.

The spatial relation of a PUCCH transmission is separately configured for Rel-16 multi-DCI multi-TRP.

We do not see why we should deviate from this principle in Rel-18. Hence, we prefer Opt2 and propose

On unified TCI framework extension for multi-DCI based multi-TRP, use RRC configuration to inform that the UE shall apply the first or the second indicated joint/UL TCI state to a PUCCH resource/group.

This would align the signalling for single-DCI and multi-DCI based multi-TRP. The first indicated TCI state is the TCI state associated to CORESETPoolIndex 0, and the second TCI state is the TCI state associated to CORESETPoolIndex 1.

Finally, we need to consider the reference signals: CSI-RS and SRS. For the Rel-17 TCI framework, the UE would use the indicated TCI state for aperiodic CSI-RS transmissions unless TCI states are configured in the CSI-AssociatedReportConfigInfo. For CSI-RS, a similar approach can be used in Rel-18, and it would be appropriate to introduce the pointer in the CSI-AssociatedReportConfigInfo:

RRC configuration is used to define the mapping/association between a configured or indicated joint/DL TCI state and an CSI-AssociatedReportConfigInfo.

For SRS, we would again apply Proposal 8, but the question is on what level. Here we note that the spatial relations in Rel-15 are defined on SRS resource level. However, in Rel-17, the field followUnifiedTCI-StateSRS was defined on SRS resource set level. Hence, different paradigms were used for the two cases.

There are advantages of defining the TCI state on SRS resource set level. First, all SRS resources will use the same power control parameters. Second, the configuration is leaner.

If the UE supports 2 SRS resource sets, one indicated TCI state can be associated with each SRS resource set, and it is possible to sound the two TRPs. However, if the UE supports only one SRS resource set, this is not possible. Then, the TCI state must be defined per SRS resource, like the spatial relation in Rel-15.

We note that for most multi-TRP scenarios, the UE supports two SRS resource sets. The only exceptions are the Rel-16 multi-DCI multi-TRP NC-JT and the Rel-17 single-DCI multi-TRP PUCCH repetition. Most notably, the support of STxMP will require the support for two SRS resource sets, both for single-DCI and multi-DCI. Therefore, we propose

All UEs that support the Rel-18 TCI framework support two SRS resource sets.

RRC configuration is used to define the mapping/association between an indicated joint/UL TCI state and an SRS resource set.

The proposed associations are summarized in Table 2 for single-DCI and in Table 3 for multi-DCI.

| Channel | Solution |
| --- | --- |
| PDCCH | {first, second, both, none} configured by RRC for each CORESET (agreed) |
| PDSCH | Explicit in DCI (agreed) + {first, second, both} by RRC (proposed for default) first for PDSCH scheduled by DCI format 1_0 (proposed) |
| PUCCH | {first, second, both} configured in PUCCH resource/group (agreed) |
| PUSCH | Explicit in DCI (agreed) + {first, second, both} by RRC (proposed for CG) first for PUSCH scheduled by DCI format 0_0 (proposed) |
| SRS | {first, second} configured in each SRS resource set (proposed) |
| CSI-RS | {first, second} configured in CSI-AssociatedReportConfigInfo (proposed) |

Table 2: Summary of RRC configured association between channels/signals and indicated TCI states for single-DCI multi-TRP.

| Channel | Solution |
| --- | --- |
| PDCCH | Determined by followUnifiedTCI-State-r17 (agreed) |
| PDSCH | Follow scheduling PDCCH (agreed) |
| PUCCH | {first, second} configured in PUCCH resource/group (proposed) |
| PUSCH | Follow scheduling PDCCH (agreed) {first, second} (proposed for CG Type 1) |
| SRS | {first, second} configured in each SRS resource set (proposed) |
| CSI-RS | {first, second} configured in CSI-AssociatedReportConfigInfo (proposed) |
| Note: - first indicated TCI state: the indicated TCI state associated to CORESET pool index =0 - second indicated TCI state: the indicated TCI state associated to CORESET pool index =1 |  |

Table 3: Summary of RRC configured association between channels/signals and indicated TCI states for multi-DCI multi-TRP.

## 2.5	UL PC for UL MTRP

In RAN1#109-e, RAN1 made the following agreement:

Agreement

On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha for PUSCH , and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion.

- FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported

- FFS: UL PC enhancement for CB and non-CB SRS in above case

FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH

The agreement contains three FFSs. The first FFS mentions other Rel-18 MTRP scheme(s) with STxMP. In our understanding, the PC framework introduced in Rel-17 can be used as is also for STxMP. The discussion on PCmax may have an impact on maximum transmit power per panel and the power headroom report (PHR), but the agreement is related to the power control parameters. Furthermore, we do not see why any enhancement for CB and non-CB SRS would be needed.

The third FFS discusses the default behaviour when one or both indicated joint, or UL TCI state(s) is not associated with an UL PC parameter setting. The Rel-17 framework does provide the possibility to operate without associating PC parameters with the TCI state, in which case, the field ul-powerControl-r17 is absent from the UL or joint TCI state. When the field ul-powerControl-r17 is absent from the TCI state configuration, the UE applies the PC parameters defined in BWP-UplinkDedicated.

This would mean that if there are no PC parameters associated with any TCI state, then the UE would rely on the PC parameters defined in BWP-UplinkDedicated for both TRPs. To us, reusing these parameters for both TRPs seems like a reasonable default, and we do not see any need to optimize the default (!), especially considering how efficiently RAN2 has implemented this in RRC: to associate PC parameters with a TCI state, the only thing that needs to be included in the TCI state is an ID. Hence, we propose:

Do not introduce any new behaviour when one or both indicated joint, or UL TCI state(s) is not associated with an UL PC parameter setting.

## 2.6	BFR

In RAN1#110e-bis, the following agreement was reached on BFR.

Agreement

On unified TCI framework extension, study the following enhancements for TRP-specific BFR:

- Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP

- Enhancement to beam update after NW response to TRP-specific BFR request

The topic of whether TRP specific BFR for single-DCI multi-TRP should be supported was extensively discussed in Rel-17, including the possibility of supporting implicit per TRP BFD-RS.  However, implicit BFD-RS determination for single-DCI multi-TRP was not supported in Rel-17 in the end, although implicit BFD-RS determination for multi-DCI multi-TRP was.

Beam failure should be declared once the UE cannot reliably demodulate PDCCH. The RAN4 requirements also indicate that: the tests have been designed around the performance of a hypothetical PDCCH. The implicit rules for determination of the BFD RS have also been designed under this assumption: the BFD RS has been determined as the RS that serves as a QCL source for PDCCH:

Beam failure is declared when the UE cannot reliably demodulate the PDCCH.

We note that the same rule applies for RLM.

For the extension of the unified TCI state framework to multi-TRP operation, it has been agreed that up to two TCI states can be indicated, and that is RRC configured which TCI states the UE applies. Since this indication informs the UE of how the PDCCH is received by the UE, it would make sense to follow that indication also for determining the BFD RSs:

For single-DCI multi-TRP, the BFD RSs are determined by the indicated DL TCI states in combination with the RRC configured indication {first, second, both, none}.

Note that we maintain the principle that the UE monitors RSs that are related to the PDCCH reception. For example, for single-DCI NC-JT, the UE would monitor RSs that are used to receive PDCCH, while RSs that are only used to receive PDSCH would not be monitored.

We do not see any need to enhance the beam update after network response to TRP-specific BFR request.

# Conclusion

In the previous sections we made the following observations:

Observation 1	There is an unused DCI code point in Table 1 that could have been used to dynamically switch between SDM and SFN.

Observation 2	The spatial relation of a PUCCH transmission is separately configured for Rel-16 multi-DCI multi-TRP.

Observation 3	Beam failure is declared when the UE cannot reliably demodulate the PDCCH.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For a UE that supports two indicated joint/DL states for PDSCH-CJT, support all three alternatives.

Proposal 2	For a UE in FR2 that does not support two default beams, if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold, the indicated joint/DL TCI state the UE applies is configured in RRC.

Proposal 3	Define new capabilities for the threshold, and support for two default beams, i.e., do not reuse the Rel-15 capability timeDurationForQcl, or the Rel-16 capability defaultQCL-TwoTCI-r16.

Proposal 4	Introduce an RRC parameter with the value {first, second, both} to determine the default indicated TCI states for PDSCH reception.

Proposal 5	For reception of PDSCH scheduled by DCI format 1_0, the UE follows the RRC parameter in Proposal 4. If the RRC parameter indicates ‘both’, the UE follows the first indicated TCI state.

Proposal 6	For NC-JT, the first indicated TCI state would be associated with the first DMRS CDM group, and the second TCI state would be associated with the second DMRS CDM group.

Proposal 7	Do not increase the number of TCI codepoints.

Proposal 8	For each channel/signal, introduce a pointer that indicates which of the indicated TCI states the UE would use for the corresponding channel/signal.

Proposal 9	Reuse the “SRS resource set indicator” field to inform which indicated TCI states to apply for SDM-based and SFN-based STxMP PUSCH transmission.

Proposal 10	Introduce an indicator in the configured grant configuration stating which of the indicated TCI states should be applied.

Proposal 11	For a PUSCH scheduled by DCI format 0_0 in single-DCI based multi-TRP, the UE uses the first indicated TCI state.

Proposal 12	On unified TCI framework extension for multi-DCI based multi-TRP, use RRC configuration to inform that the UE shall apply the first or the second indicated joint/UL TCI state to a PUCCH resource/group.

Proposal 13	RRC configuration is used to define the mapping/association between a configured or indicated joint/DL TCI state and an CSI-AssociatedReportConfigInfo.

Proposal 14	All UEs that support the Rel-18 TCI framework support two SRS resource sets.

Proposal 15	RRC configuration is used to define the mapping/association between an indicated joint/UL TCI state and an SRS resource set.

Proposal 16	Do not introduce any new behaviour when one or both indicated joint, or UL TCI state(s) is not associated with an UL PC parameter setting.

Proposal 17	For single-DCI multi-TRP, the BFD RSs are determined by the indicated DL TCI states in combination with the RRC configured indication {first, second, both, none}.

# References

RP-213598, Revised WID: MIMO evolution for downlink and uplink, Samsung, RAN#94-e, Dec 2021

3GPP TS 38.331 V17.1.0, NR; Radio Resource Control (RRC) protocol specification, RAN#96, June 2022

3GPP TS 38.321 V17.1.0, NR; Medium Access Control (MAC) protocol, RAN#96, June 2022