3GPP TSG-RAN WG1 #112bis-e	R1-2302415

e-Meeting, April 17th – April 26th, 2022

Agenda Item:	9.17.15

Source:	Ericsson

Title:	UE features for BWP without restriction

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we discuss the UE features for BWP without restriction.

# 2	Discussion

In BWPs without cell-defining (CD) SSB, the UE can be configured to perform beam management (BM), radio link monitoring (RLM) and beam failure detection (BFD) based on CSI-RS. In [1], a new WI is described to introduce additional ways for the UE to perform BM/RLM/BFD in a BWP without CD-SSB.

In this contribution, we propose UE features for this functionality.

According to the discussion in RAN, RAN4 and RAN1, the following potential solutions were identified in [2]:

- Option A) Perform BM/RLM/BFD based on CSI-RS within active BWP

- Option B) Perform BM/RLM/BFD based on SSB outside active BWP

  - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD

    - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions

    - Option B-1-2) Using larger BW covering SSB outside active BWP with interruptions

  - Option B-2) BM/RLM/BFD on SSB outside BWP within measurement gaps

    - Option B-2-2) Dedicated MG or NCSG for RLM/BFD/BM measurements

- Option C) NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD

RAN plenary agreed to support three additional ways to support BWP without restriction:

Proposal: Noting that Option A is already “Mandatory with Capability signalling”, support the following optional UE features in Rel-18:

- Option B-1-1 without early implementability; and

- Option C; and

- Option B-1-2 with the conditions that:

  - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and

  - UE shall support option (C) NCD-SSB (subject to IoDT availability); and

  - The interruption related requirements will be decided and specified in RAN4

We propose to combine B-1-1 and B-1-2 into one FG, with different values signalled. Our proposal is captured in Table 1.

Capture the UE capabilities for BWP without restriction provided in Table 1.

Table 1: Proposed FGs for BWP with restriction.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53. BWP_wor | 53-1 | BM/RLM/BFD performed on CD-SSB without MG | Support of BM/RLM/BFD performed on CD-SSB without measurement gap | 6-1a | Yes |  | BM/RLM/BFD performed on CD-SSB outside active BWP is not supported | Per band | NA | NA | NA | B-1-1, B-1-2 Candidate values: {‘with interrupt’ ,’without interrupt’} | Optional |
| 53. BWP_wor | 53-2 | BM/RLM/BFD performed on NCD-SSB inside active BWP | Support of BM/RLM/BFD performed on NCD-SSB | 6-1 | Yes |  | BM/RLM/BFD performed on NCD-SSB is not supported | Per band | NA | NA | NA | C | Optional |

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Capture the UE capabilities for BWP without restriction provided in Table 1.

# References

RP-230805, New WID: Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, RAN#99, Rotterdam, March 2023

RP-230766, Summary of offline discussion about BWP without restriction, Vodafone, RAN#99, Rotterdam, March 2023