3GPP TSG RAN WG1 #112bis-e	    R1-2302416

e-Meeting, April 17th – April 26th, 2023

Source:	ZTE

Title:	Enhancements on unified TCI framework extension for multi-TRP

Agenda Item:	9.1.1.1

Document for:	Discussion and Decision

- Introduction

In RAN#94, a new Rel-18 WID on MIMO evaluation for DL and UL was approved as in [1]. In this contribution, we elaborate our views on Rel-18 extension for indication of multiple DL/UL TCI states, simultaneous multi-panel UL transmission, and UL power control for enabling UL multi-panel transmission.

- Enhancement for unified TCI state framework

  - Framework on unified TCI operation for mTRP

Based on the R17 unified TCI framework, QCL assumption and power control parameters can be obtained through the corresponding TCI state(s). To achieve a unified solution for associating a given TCI state with target channels as well as corresponding RS(s), the association in S-DCI and M-DCI based MTRP(s) needs to be discussed separately. The association between TCI states and target channels is shown in Figure 1.

- M-DCI: Some target channels/RSs with semi-static configuration and TCI states can be associated according to the RRC/MAC-CE configuration (RRC/MAC-CE level association), such as CORESET/PDCCH, PUCCH, and P/SP CSI-RS or SRS. And some dynamically triggered/scheduled channels/RSs and TCI states can be associated according to CORESET/PDCCH scheduling (DCI-level association), such as scheduled PDSCH/PUSCH, aperiodic CSI-RS/SRS.

- S-DCI: As with M-DCI, CORESET/PDCCH, PUCCH, and P/SP CSI-RS or SRS can still be associated with TCI states according to RRC/MAC-CE level association. For aperiodic CSI-RS/SRS and scheduled PUSCH, they can obtain the association according to the CSI-RS/SRS set index indicated by DCI. However, for PDSCH, there is no clear association indication. We will further discuss this issue in Section 2.3.

Figure 1 The association between TCI states and target channels/RSs

Then, it is recommended to configure the RRC signaling to inform the mapping between a configured or indicated joint/DL TCI state and a configurable ID representing TRP, e.g., RRC parameter coresetPoolIndex in M-DCI, rather than a fixed ID such as first/second indicated TCI state(s). Then the configurable ID representing TRP can be further associated with a DL/UL channel/RS.

- In M-DCI, it is straightforward that coresetPoolIndex can be used as the configurable ID representing TRP.

- In S-DCI, a new RRC configurable ID, such as CORESET group ID, can be introduced for representing TRP.

Proposal 1: Regarding the unified TCI framework for MTRP, a configurable ID can be introduced for achieving the association between TCI state(s) and DL/UL channel/RSs.

- For M-DCI, coresetPoolIndex can be used as the configurable parameter.

- For S-DCI, a new RRC configurable ID, e.g., CORESET group ID, can be considered.

After that, we prefer to reuse Rel-16 legacy mechanisms of distinguishing S-DCI and M-DCI based mTRP modes from sTRP modes, considering that the Rel-16/17 mechanisms for mTRP operation can still be workable with good backward compatibility:

- S-DCI based mTRP operation is enabled if more than one TCI state is activated by MAC-CE;

- M-DCI based mTRP operation is enabled, if coresetPoolIndex is configured.

Proposal 2: Regarding the unified TCI framework for MTRP, reuse the legacy mechanisms to distinguish S-DCI and M-DCI based mTRP schemes from sTRP as follows:

- For S-DCI, activate more than one DL/joint-TCI state for at least one TCI codepoint in MAC-CE,

- For M-DCI, configure coresetPoolIndex per CORESET.

    - Essential issues for the extension of unified TCI framework

#### Issue-1	P/SP-CSI-RS and P/SP-SRS

For P/SP-CSI-RS, those CSI-RS(s) are almost cell-specific (shared for UEs in the NW), and that means that the corresponding time-domain pattern may be determined semi-statically by gNB (e.g., Tx beam #1, #2, etc, corresponds to time instance #n+1, #n+2, etc., respectively). If supporting dynamic TCI state update for P/SP-CSI-RS, we may have to support UE-specific P/SP-CSI-RS, which introduces significant RS overhead from NW perspective.

Similarly, transmission occasion for P/SP-SRS (regardless of CB, NCB, AS, and BM) should correspond to a cell-specific gNB Rx spatial filter(s) (that is, there is a semi-static gNB Rx spatial filtering pattern), and then, if updating the spatial filter dynamically for P/SP-SRS, we also need to consider whether/how to provide the corresponding time domain offset information for accommodating the gNB implementation.

Observation 1: Extension schemes for supporting dynamic TCI update for P/SP-CSI-RS/SRS on the unified TCI framework may break the semi-static gNB Tx/Rx spatial filter pattern in time-domain for accommodating cell-specific P/SP-RS architecture with low RS overhead (from NW perspective).

Therefore, in our views, the corresponding unified TCI extension for P/SP-CSI-RS and P/SP-SRS should be deprioritized, and if extending unified TCI state indication for P/SP-CSI-RS/SRS, their corresponding time-domain offset should be updated simultaneously based on the indicated TCI state accordingly. That is, the Tx/Rx transmission occasion and corresponding spatial filter for P/SP-RS should be updated simultaneously.

Proposal 3: De-prioritize extension schemes on the unified TCI framework for P/SP-CSI-RS and P/SP-SRS

- If extending unified TCI state indication for P/SP-CSI-RS and P/SP-SRS, their corresponding time-domain offset should be updated simultaneously based on the indicated TCI state.

#### Issue-2	Timeline misalignment for PUSCH

In RAN1#112, we have the following agreement, in which timeline misalignment issue for PUSCH is highlighted by the following FFS.

Agreement On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission

Specifically, based on legacy framework, the beam applying to PUSCH and SRS transmission is based on beam application time (e.g., beam state is applied starting from the first slot that is at least beamAppTime symbols after the last symbol of the PUCCH or the PUSCH carrying the HARQ-ACK corresponding to the DCI carrying beam codepoint update). But, after the first slot but before the first SRS transmission for CB or NCB, the mapping between PUSCH and SRS antenna ports is still based on the previous/most recent SRS transmission, i.e., based on the previous beam. Using the same antenna port means that the PUSCH and SRS should experience the similar small-scale channel property, and, definitely, the Tx beam should be the same.

Antenna port definition in TS 38.211 An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed. Timeline for determining SRS port for PUSCH in TS 38.211 The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI.

For sake of presentation, we have the following example as shown in Figure 2. If going with the legacy mechanism of unified TCI framework, the Tx beam/spatial filter corresponds to the TCI state 1, but, while considering the assumption that PUSCH antenna port should be the same as SRS ports in the most recent SRS transmission before PUSCH, the PUSCH should use the TCI state 0.

Observation 2: Considering that sharing the same antenna port definitely means using the same Tx spatial filter, there may be two different timelines for updating spatial filter of a PUSCH transmission:

- Timeline-1: Spatial filter is updated after beam application time (BAT) of the indicated joint/UL-TCI state update;

- Timeline-2: Spatial filter is determined according to the most recent SRS transmission before its scheduling PDCCH.

Figure 2 Misalignment between using the same ports as most recent SRS and effective TCI state for PUSCH

Either way, from gNB and UE implementation perspective, a clear conclusion for handling the case that the indicated TCI state and the corresponding SRS have different spatial filters is definitely needed. Then, the following should be considered as a starting point.

Proposal 4: In the case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission, for the PUSCH transmission, down-select one of the following options:

- Option-1: The UE uses the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission, i.e., the port association between the PUSCH and the SRS is ignored in such case.

- Option-2: The UE uses the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission, i.e., the indicated joint/UL-TCI state for PUSCH is ignored in such case.

#### Issue-3	PDSCH/CSI-RS with offset < a threshold

In RAN1#112, we have the following agreement for PDSCH reception, but the case of a UE only supporting a single default beam in S-DCI based operation is FFS.

Agreement On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value

In our views, by default, the UE should support two default DL Tx beams for both M-DCI and S-DCI based mTRP operation (that means that this issue is also relevant to M-DCI based mTRP operation besides for S-DCI). In such case, once the corresponding two indicated joint/DL TCI states are applied, the UE should use the corresponding two Rx spatial filters for buffering DL data.

- It should be noticed that, having above functionality, it does NOT mean that we need to further consider default beam as in Rel-15/16 (due to latency of DCI decoding). For SDCI-based PDSCH transmission, the UE should buffer received signals based on both indicated TCI state(s), and then when decoding DCI, the UE can further determine how to demodulate the received signals accordingly. It means that the DCI further indicates which indicated and effective TCI state(s) at the time instance of the PDSCH transmission should be used, rather than selecting the currently indicated TCI state(s) in the DCI.

Based on the above assumption, we do not need to further discuss the corresponding complicate rules for PDSCH/CSI-RS with offset < a threshold, i.e., timeDurationForQCL.

However, for a UE only supports a single default beam but supports M-DCI/S-DCI based mTRP operation, further study which indicated TCI state(s) can be used for PDSCH/AP-CSI-RS with scheduling/triggering offset < a threshold.

| 16-2a-6 | Default QCL enhancement for multi-DCI based multi-TRP | Support of default QCL assumption per CORESETPoolIndex | 16-2a and 16-2c |
| --- | --- | --- | --- |
| 16-2b-0 | Two default beams for single-DCI based multi-TRP | Support of default QCL assumption with two TCI states | 16-2c |

Therefore, the corresponding default behavior should be specified, and, as a basic principle, we should try to align with the legacy behavior of Rel-17 unified TCI framework (for facilitating the NW implementation for serving different types of UEs).

Proposal 5: For a UE not supporting two default beam(s) in M-DCI/S-DCI based operation, if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2, default behavior(s) (e.g., the first TCI state in S-DCI, and TCI state corresponding to CORESET with lowest coresetPoolId in M-DCI) should be specified.

- Note: reuse the legacy threshold, i.e., timeDurationForQCL.

    - M-DCI

In RAN1#111, the TCI update for M-DCI MTRP operation was discussed and then the following agreement was reached.

Agreement On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported

Then, in order to have a unified solution for M-DCI based mTRP schemes, we prefer to use coresetPoolIndex as an anchor for achieving TRP-specific TCI state indication, besides for PDCCH/CORESET and PDSCH. Specifically, we have the following suggestions:

- For PUCCH, coresetPoolIndex can be configured per PUCCH resource group. To be more specific, considering that PUCCH may not be initiated with any PDCCH/CORESET, P/SP-PUCCH may piggyback to AP-PUCCH, vice versa. Therefore, we prefer to simplify the solution as in S-DCI, that is, for PUCCH (regardless of dynamically indicated or not), coresetPoolIndex can be configured per PUCCH resource group;

- For CSI-RS, we have the following views:

  - For P/SP-CSI-RS, if extended, coresetPoolIndex can be configured per CSI-RS resource, but, as mentioned in Section 2.2.1, the corresponding time-domain offset should be updated simultaneously via the indicated TCI state.

  - For AP-CSI-RS, coresetPoolIndex is determined according to the PDCCH/CORESET triggering the AP-CSI-RS. It should be noticed that if AP-CSI-RS still reuses the above same scheme as P/SP-CSI-RS, we may have to reduce the candidate AP-CSI-RS resource set(s)/triggering state(s) per TRP and lose NW triggering flexibility.

- For SRS, we have the following views:

  - For P/SP-SRS (for CB, NCB, AS, and BM), if extended, coresetPoolIndex can be configured per SRS resource set, but, as mentioned in Section 2.2.1, the corresponding time-domain offset should be updated simultaneously via the indicated TCI state.

  - For AP-SRS (for CB, NCB, AS, and BM), coresetPoolIndex is determined according to the PDCCH/CORESET triggering AP-SRS, in order to save SRS resource candidate(s)/triggering state(s) per TRP and guarantee NW triggering flexibility.

Proposal 6: On unified TCI framework extension for M-DCI MTRP, the following recommendation is provided for opening issues:

| M-DCI based MTRP operation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel/signal | Conclusion | TCI selection scheme |  |  |
|  |  | Scheduling/triggering offset < a threshold | Scheduling/triggering offset >= a threshold |
| PDSCH | No for the case of < a threshold | Recommendation: - If FR1 or supporting two default beams, UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value corresponding to scheduling PDCCH; - If FR2 and supporting only one default beam, UE shall apply the indicated joint/DL TCI state specific to a CORESET (e.g., with a lowest ID). | Already agreed: According to coresetPoolIndex value corresponding to scheduling PDCCH |
| AP CSI-RS for CSI/BM | No |  | Recommendation: UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value corresponding to PDCCH/CORESET triggering the AP-CSI-RS. |
| Type1 CG-PUSCH | No | Recommendation: A coresetPoolIndex value can be provided per Type1 CG-PUSCH config, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding Type1 CG-PUSCH |  |  |
| PUCCH | No | Recommendation: Opt1 - A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission |  |  |
| SRS for CB/NCB/AS and AP SRS for BM | No | Recommendation: - P/SP-SRS should be deprioritized (as mentioned in Essential issue-1) - For AP-SRS, UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value corresponding to PDCCH/CORESET triggering the AP-SRS. |  |  |

    - S-DCI

In RAN1#111, the TCI update for S-DCI based MTRP operation was discussed and then the following agreement was reached.

Agreement On unified TCI framework extension for S-DCI based MTRP, down-select at least one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]: - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt2: The UE shall apply the first one of two indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt3A: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception - Alt4: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field of the most recently applied beam indication DCI Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold (if the threshold is needed)

Regarding PDSCH, for S-DCI based mTRP operation, once the corresponding two indicated joint/DL TCI states are applied, the UE should use the corresponding two Rx spatial filters for buffering DL data. In such case, we have the following suggestion:

- For PDSCH scheduled by DCI format 1_1/2 but without TCI selection field: all indicated joint/DL TCI state(s) are applied

- For PDSCH scheduled by DCI format 1_0 and PUSCH scheduled by DCI format 0_0: the following behavior should be based on legacy as much as possible, considering co-existence between Rel-18 and before.

  - For PDSCH scheduled by DCI format 1_0: A joint/DL TCI state associated with scheduling CORESET/PDCCH is applied;

  - For PUSCH scheduled by DCI format 0_0: A joint/UL TCI state associated with PUCCH resource with the lowest ID is applied.

Regarding CSI-RS, for P/SP-CSI-RS, if extended, a configurable ID can be configured per CSI-RS resource (set), but, as mentioned in Section 2.2.1, the corresponding time-domain offset should be updated simultaneously via the indicated TCI state. Then, for AP-CSI-RS, similarly, in order to save AP-CSI-RS resource candidate(s)/triggering state(s) per TRP and guarantee NW triggering flexibility, a configurable ID can be configured per CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState.

- But, it should be noticed that for enabling the above function, we do NOT need to provide any additional conditions of “not providing QCL/TCI state in CSI-AssociatedReportConfigInfo”, due to the fact that being different from default beam as in Rel-16, we herein directly provide the corresponding association assumption.

Regarding SRS, for P/SP-SRS (for CB, NCB, AS, and BM), a configurable ID can be configured per SRS resource set, and, similarly, additionally time-domain offset should be provided. For AP-SRS, a configurable ID can be configured per SRS triggering state.

Proposal 7: On unified TCI framework extension for S-DCI MTRP, use RRC configuration of mapping between configured or indicated TCI state and configurable ID (rather than a fixed ID by a rule), which can be further associated with a DL/UL channel/RS. Then, the following recommendation is provided for opening issues.

| S-DCI based MTRP operation |  |  |  |
| --- | --- | --- | --- |
| Channel/signal | Conclusion | TCI selection scheme |  |
|  |  | Scheduling/triggering offset < a threshold in FR2 | FR1, or Scheduling/triggering offset >= a threshold in FR2 |
| PDSCH scheduled/activated by DCI format 1_1/1_2 if the [TCI selection field] is present in DCI format 1_1/1_2 | No for < a threshold | Recommendation: - If supporting two default beam(s), UE shall apply the indicated joint/DL TCI state according to [TCI selection field] in DCI format 1_1/1_2; - If supporting only one default beam, UE shall apply the first indicated joint/DL TCI state. | Already agreed: According to [TCI selection field] in DCI format 1_1/1_2 Recommendation: Support to indicate the updated order of TCI state(s) by codepoint of ‘11’ |
| PDSCH scheduled/activated by DCI format 1_1/1_2 if the [TCI selection field] is not present in DCI format 1_1/1_2 | No |  | Recommendation: Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception |
| PDSCH scheduled/activated by DCI format 1_0 (including DG and SPS) | No | Recommendation: UE shall apply the first indicated joint/DL TCI state. | Recommendation: UE shall apply the indicated joint/DL TCI state corresponding to the scheduled PDCCH/CORESET |
| AP CSI-RS for CSI/BM | No | Recommendation: - If supporting two default beam(s), UE shall apply indicated joint/DL TCI state according to configurable ID in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState - If supporting only one default beam, UE shall apply the first indicated joint/DL TCI state. | Recommendation: - If supporting two default beam(s), UE shall apply the indicated joint/DL TCI state according to RRC configuration in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState |
| PUSCH scheduled/activated by DCI format 0_0 (including DG and Type2 CG) | No | Recommendation: UE shall apply a joint/UL TCI state associated with PUCCH resource with lowest ID is applied. |  |
| Type1 CG-PUSCH | No | Recommendation: ONLY specify: its power control(s) are determined according to joint/UL state(s) associated with the indicated SRS resource set(s), in order to avoid timeline misalignment of indicated TCI and SRS transmission. |  |
| SRS for CB/NCB/AS and AP SRS for BM | No | Recommendation: - P/SP-SRS should be deprioritized (as mentioned in Essential issue-1) - For AP-SRS, UE shall apply the indicated joint/UL TCI state specific to configurable ID that is configured per SRS triggering state |  |

    - CJT and SFN

In Rel-18 MIMO, up to 4 TRPs have been agreed to be supported in CJT operation, and then, for unified TCI extension for MTRP operation, we may need to consider whether more than one TCI indication for CJT should be supported or not. For sake of presentation, one diagram for CJT architecture, in which each of DMRS ports/layer is served by all TRP links, is shown in Figure 3.

Figure 3 Diagram for CJT architecture: each of DMRS ports/layer is served by all TRP links

Technically, compared with SFN, due to support for a higher RANK transmission, since each DMRS port/layer is served by all TRP links in CJT, T/F-sync consistency between CSI-RS for CSI and PDSCH should be guaranteed. Therefore, pre-compensating delay and/or frequency offset may be considered.

- ‘Doppler shift’ offset across TRPs may introduce serious inter-layer interference, which can be hardly compensated by UE-side Wiener filter;

- ‘Average delay’ offset across TRPs may introduce serious frequency-selective fading (e.g., RE-level) or even inter-symbol interference.

Above implies that even having more than one TCI state(s) but under the gNB pre-compensation for T/F in-sync, the UE may only use a portion of QCL assumption in other TCI state(s) except for the first TCI state. In RAN1#111, TCI state(s) applied to CJT operation was discussed and then the following agreements were reached.

Agreement On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework Agreement On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state - Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB

Based on the agreed EVM for CJT transmission in 9.1.2.1 CSI, we have the following evaluation results for comparing the above three alternatives based on SLS. It can be observed that, while having two individual TCI states, Alt2 and Alt3 can obtain significant performance gains over Alt1, i.e., via gNB side pre-compensation for TRP-specific Doppler shift and TRP-specific delay offset, respectively.

Figure 4 UPT comparison in second TCI state: (a) Alt 2, i.e., with Doppler compensation vs Alt 1 w.r.t. QCL-TypeA, i.e., without any compensation (in a case of 0.025ppm per TRP); (b) Alt 2, i.e., only w.r.t. QCL-TypeB with delay compensation vs Alt 1 w.r.t. QCL-TypeA, i.e., without any compensation (for inter-site scenario)

Proposal 8: On unified TCI framework extension, for PDSCH-CJT applying both indicated joint TCI states, both Alt2 and Alt3 should be supported for pre-compensating delay and/or frequency offset.

- Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state.

- Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB.

Then, besides for S-DCI based mTRP operation, we may further consider the combination cases, like CJT-PDSCH + mDCI-based PUSCH, or CJT+CJT for M-DCI MTRP. In such case, we may need to consider how to group TCI state(s) for CJT or other mTRP operation.

  - Signaling design on unified TCI operation for mTRP

In order to achieve flexible association between target channels and TCI states for MTRP operation, the following framework should be considered.

- RRC pool configuration: To apply unified TCI for MTRP, a common RRC pool can be configured for both TRPs.

- MAC-CE activation:

  - For S-DCI-based mTRP operation, one TCI state codepoint can be activated with one or more joint TCI states or DL/UL-TCI state pairs, each of which is associated with a configurable ID;

  - For M-DCI based mTRP operation, one TCI state codepoint can be activated with only one joint TCI state or DL/UL-TCI state pair, which is associated with coresetPoolIndex.

- DCI indication: Joint or DL/UL TCI states in the indicated TCI state codepoint can apply to all channels/RSs associated with respective TRPs accordingly.

Proposal 9: Regarding unified TCI framework for MTRP, in MAC level,

- For S-DCI based mTRP operation, one TCI state codepoint can be activated with one or more joint TCI states or DL/UL-TCI state pairs, each of which is associated with configurable ID, e.g., CORESET group ID.

- For M-DCI based mTRP operation, one TCI state codepoint can be activated with only one joint TCI state or DL/UL-TCI state pair, and coresetPoolIndex can be additionally activated for activated TCI state(s) in the MAC-CE (analogous to legacy MAC-CE for M-DCI PDSCH TCI activation).

Then, for S-DCI based mTRP, a new indicator field for dynamic switching for sTRP and mTRP based PDSCH transmission has been supported in RAN1#112 as follows.

Agreement On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field]

For dynamic switch mode configuration for scheduling PDSCH reception, it can be configured by RRC explicitly according to a new parameter, such as DynamicSwitch-r18. That is, once this parameter is enabled, the corresponding new indicator field is present.

- When the dynamic switch mode is enabled, gNB needs to further indicate one or more TCI states for scheduled PDSCH from multiple indicated TCI states. In order to indicate TCI states more flexibly, we consider introducing TCI state selection function in DCI. For example, a new field (e.g., dynamic switch indicator) similar to “SRS resource set indicator” in DCI can be considered.

- For instance, the definition for each codepoint is shown in Table 1. Besides for updating TRP-order, we can further consider that CJT/SFN mode can be enabled by a codepoint, i.e., once the corresponding codepoint is enabled, the UE shall apply both the 1st and the 2nd indicated joint/DL TCI states to all of DMRS ports in the scheduled/activated PDSCH reception.

Table 1 Dynamic TRP indication to scheduled PDSCH

| Codepoint | Usage |  |
| --- | --- | --- |
| 00 | Only use TCI state associated with TRP1 |  |
| 01 | Only use TCI state associated with TRP2 |  |
| 10 | Usage-1: Mapping order of {first TCI state, second TCI state} | Usage-2: Indicating a NCJT/TD-repetition PDSCH transmission using both TCI states |
| 11 | Usage-1: Mapping order of {first TCI state, second TCI state} | Usage-2: Indicating a CJT/SFN PDSCH transmission using both TCI states |

Proposal 10: On unified TCI framework extension for S-DCI based mTRP operation, the codepoint ‘10’ and ‘11’ in [TCI selection field] can indicate one of the following information for scheduled PDSCH transmission.

- Information-1: Mapping order if both TCI states are applied, i.e., first and second TCI state are applied by order in NCJT/TD-repetition, or second and first TCI states are applied by order in NCJT/TD-repetition

- Information-2: Transmission mode indication if both TCI states are applied, i.e., to which transmission mode of NCJT/TD-repetition and CJT/SFN, both TCI state(s) are applied.

- TRP-specific BFR

In RAN1#110bis-e meeting, the TRP-specific BFR procedure was discussed and the following agreement was reached.

Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request

In Rel-17, we have an enhancement for MAC-CE based BFD-RS update. However, due to a lack of explicit association between DL/UL channel/RS and TRP in S-DCI, unfortunately we have NOT supported the implicit BFD-RS determination. Based on the mapping between the indicated joint/DL TCI state and the configurable ID representing TRP as mentioned above, the implicit BFD-RS determination can be supported straightforwardly. In particular, if CORESET(s) is configured to apply both two indicated joint/DL TCI states for PDCCH-SFN, it should be clarified that the BFD-RS is determined from which one of the indicated joint/DL TCI states.

Additionally, in Rel-17, only beam update of PDCCH/CORESET in M-DCI based MTRP is supported. Then, after having the association between TRP and DL/UL channel/RS, the corresponding update for all DL/UL channels/RSs should be supported. Specifically, in the unified TCI framework, we have a clear framework for association between TRP and DL/UL channel/RS (e.g., a configured ID, e.g., CORESET group ID, proposed for S-DCI based MTRP and coresetPoolIndex for M-DCI based MTRP). Straightforwardly, once a given TRP (i.e., a BFD-RS set) fails, QCL assumption/spatial filter corresponding to DL/UL channel/RS associated with the same configured ID/coresetPoolIndex as the failed BFD-RS set should be updated according to the new identified RS (i.e., q_new).

Proposal 11: Support the enhancement on implicit BFD-RS determination for S-DCI based MTRP as well as beam update after NW response to TRP-specific BFR request for S-DCI and M-DCI based MTRP.

- Once a given TRP (i.e., a BFD-RS set) fails, QCL assumption/spatial filter corresponding to DL/UL channel/RS associated with the same configured ID (e.g., CORESET group ID)/coresetPoolIndex as the failed BFD-RS set should be updated according to the new identified RS (i.e., q_new).

- UL power control for UL multi-panel transmission

In RAN1#109-e and RAN1#112 meetings, the uplink power control in case of UL multi-panel transmission was discussed and the following agreements were reached.

Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 is endorsed in R1-2205639. Agreement On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS

In Rel-18, in order to support the enhancement of simultaneous multi-panel UL transmission, UL power control for simultaneous uplink transmissions from multiple panels should be further considered. Basically, the transmission power of an uplink channel/signal is determined based on Tx power calculation and equally power scaling. For Tx power calculation, the maximum output power or the power limitation (Pc,max) is an essential factor, where two assumptions of power limitation for STxMP were initially discussed as above.

- Introduction of per-panel power limitation and per-UE power limitation

  - Note that the feasibility of per-panel power limitation and per-UE power limitation has been confirmed by RAN4 [2]. As far as we are concerned, both kinds of power limitation can be defined depending on the UE implementation. Specifically, the per-panel power limitation can be seen as a basis requirement from UE implementation/RF-architecture. Additionally, considering the cross interference between panels, the introduction of per-panel power limitation would also be beneficial from the interference management perspective. The per-UE power limitation shall definitely be applicable at all the time due to the FCC regulation and total power consumption of the UE side.

- Relationship between the two power limitation assumptions

  - It should be straightforward that the per-panel power limitation is always not greater than the per-UE power limitation imposed by the UE power class. However, whether the sum over all panels of the per-panel power limitation can be greater than the per-UE power limitation still needs to be further studied.

Then, regarding the relationship of two power limitation assumption, we have the following analysis:

- Case 1: the sum over all panels of the per-panel power limitation equals to the per-UE power limitation

  - In such case, the total calculated transmission power of signals/channels from multiple panels (e.g., being based on gNB configuration) cannot be larger than the power limitation for a given power class, and additional power allocation operation is not required, as illustrated in Figure 5. However, as the UE does not always use both panels for transmission, imposing such strong power limitation would result in a waste of transmission power, which is not conducive to unleash the full potential of uplink transmission.

Figure 5 UL power control for STxMP in the Case-1, where the sum over all panels of the per-panel power limitation equals to the per-UE power limitation

- Case 2: the sum over all panels of the per-panel power limitation is greater than the per-UE power limitation

  - It should be noticed that the sum of the maximum power for E-UTRA radio access and NR radio access can be set to be larger than the configured maximum transmission power for EN-DC/NE-DC operation. From UL transmission perspective, allowing that the sum over all panels of per-panel power limitation for STxMP can be greater than the existing power limitation for a given power class is beneficial for maximizing UL Tx transmission power.

  - Specifically, as shown in Figure 6, Case-2 can be divided into some detailed categories as follows:

    - Case 2a: The calculated transmission power of one panel exceeds the panel-specific power limitation, but the total calculated transmission power does not exceed the power limitation for a power class;

    - Case 2b: The calculated transmission power of one panel exceeds the panel-specific power limitation, and the total calculated transmission power exceeds the power limitation for a power class.

    - Case 2c: The calculated transmission power for each panel does not exceed the panel-specific power limitation, but the total calculated transmission power exceeds the power limitation for a power class.

    - Case 2d: The calculated transmission power for each panel does not exceed the panel-specific power limitation, and the total calculated transmission power does not exceed the power limitation for a power class.

Figure 6 UL power control for STxMP in the Case-2, where the sum over all panels of the per-panel power limitation is greater than the per-UE power limitation

Observation 3: On UE power limitation for STxMP UL transmission, both of the following two assumptions should be supported for different purposes.

- Assumption 1: per-panel power limitation for STxMP UL transmission (as a requirement from UE RF architecture);

- Assumption 2: per-UE power limitation over all UE panels that used for STxMP transmission (as a requirement from FCC requirement and UE total power saving).

It has been agreed in RAN1#112 meeting that the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS and the PL-RS included in the indicated joint/UL TCI state. Therefore, calculating the transmission power for each panel separately based on the corresponding parameters is a straightforward solution. The detailed calculating formula can be quite similar to the legacy, besides for further clarifying the above two limitation. Furthermore, as with Cases 2b and 2c shown in the above figure, the total calculated transmission power of multiple panels may exceed the pre-defined power limitation. Therefore, we need to study transmission power reductions and associated power allocation mechanisms in case of exceeding the power limitation for STxMP UL transmission.

Proposal 12: Regarding UL power control for UL multi-panel transmission,

- Transmission power is calculated per panel based on the open loop parameters (i.e. P0 and alpha), closed loop and pathloss RS determined for the corresponding panel.

- FFS: whether and how to scale or allocate the total transmission power in case of exceeding the power limitation for STxMP UL transmission.

- Conclusion

In this contribution, we provide our initial views on Rel-18 unified TCI framework. Observations/proposals are listed as follows.

Observation 1: Extension schemes for supporting dynamic TCI update for P/SP-CSI-RS/SRS on the unified TCI framework may break the semi-static gNB Tx/Rx spatial filter pattern in time-domain for accommodating cell-specific P/SP-RS architecture with low RS overhead (from NW perspective).

Observation 3: On UE power limitation for STxMP UL transmission, both of the following two assumptions should be supported for different purposes.

- Assumption 1: per-panel power limitation for STxMP UL transmission (as a requirement from UE RF architecture);

- Assumption 2: per-UE power limitation over all UE panels that used for STxMP transmission (as a requirement from FCC requirement and UE total power saving).

Proposal 1: Regarding the unified TCI framework for MTRP, a configurable ID can be introduced for achieving the association between TCI state(s) and DL/UL channel/RSs.

- For M-DCI, coresetPoolIndex can be used as the configurable parameter.

- For S-DCI, a new RRC configurable ID, e.g., CORESET group ID, can be considered.

Proposal 2: Regarding the unified TCI framework for MTRP, reuse the legacy mechanisms to distinguish S-DCI and M-DCI based mTRP schemes from sTRP as follows:

- For S-DCI, activate more than one DL/joint-TCI state for at least one TCI codepoint in MAC-CE,

- For M-DCI, configure coresetPoolIndex per CORESET.

Proposal 3: De-prioritize extension schemes on the unified TCI framework for P/SP-CSI-RS and P/SP-SRS

- If extending unified TCI state indication for P/SP-CSI-RS and P/SP-SRS, their corresponding time-domain offset should be updated simultaneously based on the indicated TCI state.

Proposal 4: In the case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission, for the PUSCH transmission, down-select one of the following options:

- Option-1: The UE uses the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission, i.e., the port association between the PUSCH and the SRS is ignored in such case.

- Option-2: The UE uses the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission, i.e., the indicated joint/UL-TCI state for PUSCH is ignored in such case.

Proposal 5: For a UE not supporting two default beam(s) in M-DCI/S-DCI based operation, if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2, default behavior(s) (e.g., the first TCI state in S-DCI, and TCI state corresponding to CORESET with lowest coresetPoolId in M-DCI) should be specified.

- Note: reuse the legacy threshold, i.e., timeDurationForQCL.

Proposal 6: On unified TCI framework extension for M-DCI MTRP, the following recommendation is provided for opening issues:

| M-DCI based MTRP operation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel/signal | Conclusion | TCI selection scheme |  |  |
|  |  | Scheduling/triggering offset < a threshold | Scheduling/triggering offset >= a threshold |
| PDSCH | No for the case of < a threshold | Recommendation: - If FR1 or supporting two default beams, UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value corresponding to scheduling PDCCH; - If FR2 and supporting only one default beam, UE shall apply the indicated joint/DL TCI state specific to a CORESET (e.g., with a lowest ID). | Already agreed: According to coresetPoolIndex value corresponding to scheduling PDCCH |
| AP CSI-RS for CSI/BM | No |  | Recommendation: UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value corresponding to PDCCH/CORESET triggering the AP-CSI-RS. |
| Type1 CG-PUSCH | No | Recommendation: A coresetPoolIndex value can be provided per Type1 CG-PUSCH config, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding Type1 CG-PUSCH |  |  |
| PUCCH | No | Recommendation: Opt1 - A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission |  |  |
| SRS for CB/NCB/AS and AP SRS for BM | No | Recommendation: - P/SP-SRS should be deprioritized (as mentioned in Essential issue-1) - For AP-SRS, UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value corresponding to PDCCH/CORESET triggering the AP-SRS. |  |  |

Proposal 7: On unified TCI framework extension for S-DCI MTRP, use RRC configuration of mapping between configured or indicated TCI state and configurable ID (rather than a fixed ID by a rule), which can be further associated with a DL/UL channel/RS. Then, the following recommendation is provided for opening issues.

| S-DCI based MTRP operation |  |  |  |
| --- | --- | --- | --- |
| Channel/signal | Conclusion | TCI selection scheme |  |
|  |  | Scheduling/triggering offset < a threshold in FR2 | FR1, or Scheduling/triggering offset >= a threshold in FR2 |
| PDSCH scheduled/activated by DCI format 1_1/1_2 if the [TCI selection field] is present in DCI format 1_1/1_2 | No for < a threshold | Recommendation: - If supporting two default beam(s), UE shall apply the indicated joint/DL TCI state according to [TCI selection field] in DCI format 1_1/1_2; - If supporting only one default beam, UE shall apply the first indicated joint/DL TCI state. | Already agreed: According to [TCI selection field] in DCI format 1_1/1_2 Recommendation: Support to indicate the updated order of TCI state(s) by codepoint of ‘11’ |
| PDSCH scheduled/activated by DCI format 1_1/1_2 if the [TCI selection field] is not present in DCI format 1_1/1_2 | No |  | Recommendation: Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception |
| PDSCH scheduled/activated by DCI format 1_0 (including DG and SPS) | No | Recommendation: UE shall apply the first indicated joint/DL TCI state. | Recommendation: UE shall apply the indicated joint/DL TCI state corresponding to the scheduled PDCCH/CORESET |
| AP CSI-RS for CSI/BM | No | Recommendation: - If supporting two default beam(s), UE shall apply indicated joint/DL TCI state according to configurable ID in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState - If supporting only one default beam, UE shall apply the first indicated joint/DL TCI state. | Recommendation: - If supporting two default beam(s), UE shall apply the indicated joint/DL TCI state according to RRC configuration in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState |
| PUSCH scheduled/activated by DCI format 0_0 (including DG and Type2 CG) | No | Recommendation: UE shall apply a joint/UL TCI state associated with PUCCH resource with lowest ID is applied. |  |
| Type1 CG-PUSCH | No | Recommendation: ONLY specify: its power control(s) are determined according to joint/UL state(s) associated with the indicated SRS resource set(s), in order to avoid timeline misalignment of indicated TCI and SRS transmission. |  |
| SRS for CB/NCB/AS and AP SRS for BM | No | Recommendation: - P/SP-SRS should be deprioritized (as mentioned in Essential issue-1) - For AP-SRS, UE shall apply the indicated joint/UL TCI state specific to configurable ID that is configured per SRS triggering state |  |

Proposal 8: On unified TCI framework extension, for PDSCH-CJT applying both indicated joint TCI states, both Alt2 and Alt3 should be supported for pre-compensating delay and/or frequency offset.

- Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state.

- Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB.

Proposal 9: Regarding unified TCI framework for MTRP, in MAC level,

- For S-DCI based mTRP operation, one TCI state codepoint can be activated with one or more joint TCI states or DL/UL-TCI state pairs, each of which is associated with configurable ID, e.g., CORESET group ID.

- For M-DCI based mTRP operation, one TCI state codepoint can be activated with only one joint TCI state or DL/UL-TCI state pair, and coresetPoolIndex can be additionally activated for activated TCI state(s) in the MAC-CE (analogous to legacy MAC-CE for M-DCI PDSCH TCI activation).

Proposal 10: On unified TCI framework extension for S-DCI based mTRP operation, the codepoint ‘10’ and ‘11’ in [TCI selection field] can indicate one of the following information for scheduled PDSCH transmission.

- Information-1: Mapping order if both TCI states are applied, i.e., first and second TCI state are applied by order in NCJT/TD-repetition, or second and first TCI states are applied by order in NCJT/TD-repetition

- Information-2: Transmission mode indication if both TCI states are applied, i.e., to which transmission mode of NCJT/TD-repetition and CJT/SFN, both TCI state(s) are applied.

Proposal 11: Support the enhancement on implicit BFD-RS determination for S-DCI based MTRP as well as beam update after NW response to TRP-specific BFR request for S-DCI and M-DCI based MTRP.

- Once a given TRP (i.e., a BFD-RS set) fails, QCL assumption/spatial filter corresponding to DL/UL channel/RS associated with the same configured ID (e.g., CORESET group ID)/coresetPoolIndex as the failed BFD-RS set should be updated according to the new identified RS (i.e., q_new).

Proposal 12: Regarding UL power control for UL multi-panel transmission,

- Transmission power is calculated per panel based on the open loop parameters (i.e. P0 and alpha), closed loop and pathloss RS determined for the corresponding panel.

- FFS: whether and how to scale or allocate the total transmission power in case of exceeding the power limitation for STxMP UL transmission.

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator)

- R4-2303494, Reply LS on UE power limitation for STxMP in FR2, 3GPP TSG RAN4#106, February 2023.