3GPP TSG RAN WG1 Meeting #112bis-e	R1-2302417

e-Meeting, April 17th – 26th, 2023

Source:	ZTE

Title:	TA enhancement for multi-DCI

Agenda Item:	9.1.1.2

Document for:	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID on MIMO evolution for both DL and UL was reached as in [1]. As identified below, TA enhancement for multi-DCI based MTRP operation is one of the objectives according to the WID.

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation. - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, we concentrate on some potential methods to facilitate timing advance enhancements for multi-DCI based multi-TRP operation.

- Discussion

  - Association between TAG and UL channels/signals

In RAN1#112 meeting, the following agreement on the association between TAG and UL channels/signals has been endorsed [2].

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following: Associate TAG to TCI-state - Associate TAG ID with UL/joint TCI state - For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized - A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs FFS: on how to handle association when Rel-15/16 spatial relation framework is used for - PUCCH - DG/CG Type 1/Type 2 PUSCH - AP/SP/P SRS

For the agreed baseline of association between TAGs and UL channels/signals, a leftover issue is that whether the association is targeting activated UL/joint TCI states only or all UL/joint TCI states associated with one CORESET Pool. Not similar as the working assumption, the baseline does not require to change association between a TCI state and a TAG flexibly or frequently. When two TAGs are configured for a serving cell, TCI states associated with CORESETPoolIndex 0 are associated with one TAG while TCI states associated with CORESETPoolIndex 1 are associated with another TAG, and such association should be constant for either activated TCI states or inactivated TCI states and it should be configured by RRC signaling.

Proposal 1: For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support that RRC-configured UL/joint TCI states of UL channels/signals associated to one CORESET Pool Index correspond to one TAG.

Regarding the working assumption, it assumes that TCI states associated with one CORESET pool/ TRP can be divided into two groups and each of the groups is associated with a TAG. However, TCI states associated with two TRPs applying the same timing advance value should be an unrepresentative case in fact, especially for inter-cell multi-TRP operation.

For most application scenarios in inter-cell multi-TRP operation, it is considerable challenging to guarantee the same TAG of TCI states towards different TRPs in terms of NW scheduling due to it requires uplink transmissions associated with two TRPs going through the similar propagation delay, and it is worth noting the non-ideal backhaul assumption of MDCI MTRP operation. Furthermore, TCI states associated with a TAG might needs to be updated frequently with the mobility of UE in order to satisfy the uplink timing requirements for reliable reception at gNB side.

Since TAG configuration is up to gNB side, and timing advance is to ensure reliable reception and decoding at gNB side, it is unclear which UE capability can guarantee the same requirements of timing advance values for TCI states of different TRPs. In contrast, whether TCI states of different TRPs can be associated with the same or different TAG should be up to gNB schedule rather than a UE capability.

In addition, whether to support the working assumption depends on its practical and reasonable use cases. Given that UL/joint TCI states with two TAGs can be associated with one CORESET pool, two potential use cases of this working assumption can be figured out so far: 1) Two TAs for sDCI MTRP operation, 2) Two TAs for MDCI MTRP operation performed by DPS as in Rel-15. Technically, both of these two use cases are out of scope as per the following agreement endorsed in RAN1#109-e meeting with regards to the statement of MDCI MTRP operation in Rel-18.

Agreement Two TA enhancement for uplink multi-DCI based multi-TRP operation are applicable to at least: - TDM based multi-DCI uplink transmission - simultaneous multi-DCI uplink transmission (if simultaneous uplink multi-DCI uplink transmission is supported in Agenda 9.1.4.1) - Note: Whether two TA enhancement is applicable to other schemes is a separate discussion, which is not in the scope of AI 9.1.1.2.

Proposal 2: For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, do NOT support to confirm the Working Assumption that “A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs”.

For the case that Rel-15/16 spatial relation framework is used for UL signals/channels, it is natural that the similar association as that for UL/joint TCI state framework can be reused, i.e., to associate TAG ID with the spatial relations of UL signals/channels associated to one CORESETPoolIndex correspond to one TAG.

Proposal 3: For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation in Rel-15/16 spatial relation framework, support that the spatial relations of UL channels/signals associated to one CORESET Pool Index correspond to one TAG.

Given that unified TCI state framework can be applied in FR1 as agreed in AI 9.1.1.1, the remaining issue is whether/how to fulfill two TAs enhancement for MDCI MTRP operation in case of spatial relation framework in FR1. Notably, spatial relation framework is fundamentally used in almost everywhere in the current specification to distinguish the discrepancy of UL signals/channels in FR1 and FR2. Enabling spatial relation in FR1 for two TAs enhancement will lead to terrible specification efforts. Consequently, it is proper to preclude this enhancement in case of spatial relations enabled in FR1.

Proposal 4: Do NOT support two TAs enhancement for multi-DCI based multi-TRP operation in FR1 when Rel-15/16 spatial relation framework is used.

  - Cross-TRP PDCCH order

In RAN1#111 and RAN1#112 meeting, the following agreements related to cross-TRP PDCCH order have been endorsed.

Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI. - FFS: for intra-cell Multi-DCI - FFS: whether there are any restrictions needed - FFS: if cross TRP RACH triggering is an optional feature Agreement Confirm the following working assumption: For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

For inter-cell Multi-DCI Multi-TRP, one leftover issue is that cross-TRP PDCCH order might lead to latency due to the transfer of the PDCCH order between two cells. However, such delay does not exist for intra-cell Multi-DCI Multi-TRP, hence it is reasonable to support cross-TRP PDCCH order for both intra-cell and inter-cell Multi-TRP. Furthermore, in case the link of a TRP to UE is not reliable enough, PDCCH order might be missed by UE for several times, and the corresponding RACH procedure will be triggered with high latency. In such case, if cross-TRP PDCCH order sent by another TRP can be allowed to trigger random access procedures of the TRP, PRACH procedures can be more efficient and reliable.

Proposal 5: For multi-DCI based Multi-TRP operation with two TAs enhancement, support the case that a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP for intra-cell Multi-DCI MTRP.

According to the confirmed working assumption that one additional PRACH configuration is supported for each configured additional PCI in inter-cell Multi-TRP, the random access preambles, RACH occasions, and the mapping of SSB to ROs configured for two TRPs can be individual. A TRP related indication should be introduced in PDCCH order to indicate the PRACH configuration to be used for the corresponding random access procedure.

  - If PDCCH order from serving cell or activated additional PCI can trigger random access procedure towards an inactivated additional PCI, the additional PCI field should be introduced in PDCCH order. However, for two TAs enhancement, there is no strong requirements of acquiring TA values of an inactivated additional PCI and it is unnecessary to trigger random access procedure towards an inactivated additional PCI.

  - If PDCCH order from serving cell or activated additional PCI can only trigger random access procedure towards serving cell or the activated additional PCI, CORESETPoolIndex should be introduced in PDCCH order due to different CORESETPoolIndex are associated between serving cell and additional PCI.

Proposal 6: For multi-DCI based inter-cell Multi-TRP operation with two TAs enhancement, support to introduce an indication of CORESETPoolIndex in PDCCH order to indicate which PRACH configuration is to be used in the triggered RACH procedure.

In RAN1#112 meeting, the following conclusion has been reached [2].

Conclusion For inter-cell multi-DCI based multi-TRP operation with two TA enhancement, there is no consensus to introduce additional type 1 CSS configuration per additional PCI.

According to the conclusion above, in response to a random access procedure towards either serving cell or non-serving cell, RAR can only be received from serving cell. Potential scenarios can be illustrated in Figure 1, where case (a) and (c) are for PDCCH order sent by one TRP triggers RACH procedure towards the same TRP, case (b), (d), (e) and (f) are for PDCCH order sent by one TRP triggers RACH procedure towards a different TRP. For cases above except case (a), issues related to the determination of TAC, PL-RS for PRACH power control and QCL properties of PDCCH/PDSCH associated with RA-RNTI should be considered.

(a)                                                       (b)

(c)                                                     (d)

(e)                                                     (f)

Figure 1: Scenarios of self/cross-TRP RACH triggering: (a) serving cell triggers RACH towards serving cell, (b) serving cell triggers RACH towards activate additional PCI, (c) activate additional PCI triggers RACH towards activate additional PCI, (d) activate additional PCI triggers RACH towards serving cell, (e) serving cell triggers RACH towards inactivate additional PCI, and (f) activate additional PCI triggers RACH towards inactivate additional PCI

Signaling of TAC

According to TS 38.321, Absolute Timing Advance Command is only for PTAG and is associated with the MSGB. Considerable specification efforts will be required if Absolute Timing Advance Command is used for contention free random access procedure triggered by PDCCH order regardless of the TAG and the type of random access procedure. RAR should still be used to signal TAC, and so the structure of RAR can reuse the legacy as analyzed in section 2.3 even in the case of cross-TRP RACH triggering.

As discussed in Rel-17, it is assumed that inter-cell MTRP operation is in intra-DU scenario. According to TS 38.801, there is only one single lower layer (PHY/MAC/RLC) of each gNB-DU, and MAC CE signaling is conveyed from serving cell only.

Based on the assumption above, RAR in response to the random access procedure towards the non-serving cell should be transacted from non-serving cell to serving cell, and then UE receives RAR from serving cell according to the configured Type-1 PDCCH CSS.

Proposal 7: For multi-DCI based multi-TRP operation with two TAs enhancement, support to receive RAR from serving cell based on the configured Type-1 PDCCH CSS in response to a random access procedure towards either serving cell or non-serving cell.

PL-RS for PRACH power control

In legacy, for a PRACH transmission in response to a PDCCH order triggered CFRA procedure, pathloss calculation is depending on the DL RS that the DM-RS of the PDCCH order is quasi-collocated with.

For cross-TRP PDCCH order, reception of a PDCCH order and transmission of the corresponding PRACH can be associated with different TRPs, then the legacy rule cannot work anymore. A general and unified solution is to calculate pathloss depending on the DL-RS used for PRACH transmission, or the DL-RS indicated by the PDCCH order in other words. For either PDCCH order from serving cell triggers random access procedure towards non-serving cell, or PDCCH order from non-serving cell triggers random access procedure towards serving cell, the solution above can work well to determine a corresponding PL-RS.

Proposal 8: For the case of a PDCCH order sent by one TRP triggers RACH procedure towards a different TRP, pathloss of PRACH transmission is calculated based on the SSB that associated with the PDCCH order.

QCL properties

For PDCCH scrambled by RA-RNTI/MSGB-RNTI or PDSCH containing RAR message, the corresponding DM-RS QCL properties should be considered.

  - For PDCCH scrambled by RA-RNTI/MSGB-RNTI

    - In legacy, when UE attempts to detect DCI format 1_0 scrambled by RA-RNTI in response to a PRACH transmission initiated by PDCCH order triggered CFRA for SpCell, UE assumes that PDCCH order and the PDCCH format 1_0 have same DM-RS port QCL properties. When UE attempts to detect DCI format 1_0 scrambled by RA-RNTI in response to a PRACH transmission initiated by PDCCH order triggered CFRA for SCell, DM-RS port quasi co-location properties of the CORESET associated with the Type1-PDCCH CSS set are assumed for receiving the PDCCH that includes the DCI format 1_0.

    - Since RAR can be transmitted from serving cell only, and PDCCH order can be transmitted from either serving cell or non-serving cell, the legacy rule that PDCCH order and the PDCCH scheduling RAR have same DM-RS port QCL properties is invalid. Reusing the general QCL assumption rule specified for normal cases rather than that for SpCell should be considered, i.e., DM-RS port quasi co-location properties of the CORESET associated with the Type1-PDCCH CSS set are assumed for receiving the PDCCH that includes the DCI format 1_0, where the DCI format 1_0 is in response to a random access procedure initiated by a cross-TRP PDCCH order. The QCL assumption rule should be generally applied regardless of whether the CFRA procedure is triggered for SpCell or SCell.

  - For PDSCH scheduled with RA-RNTI/MSGB-RNTI

    - In legacy, for PDSCH scheduled with RA-RNTI or MSGB-RNTI, DM-RS port of the PDSCH is quasi co-located with the SSB/CSI-RS used for RACH association and transmission. For PRACH transmission towards an additional PCI, the SSB/CSI-RS used for RACH association and transmission is associated with the additional PCI, and the PDSCH scheduled with RA-RNTI/MSGB-RNTI is transmitted by the serving cell, the legacy QCL relationship is invalid.

    - Therefore, the QCL relationship between SSB used for PRACH transmission and PDSCH containing RAR should be decoupled. A potential method in such case is that DM-RS port of the PDSCH containing RAR and that of the PDCCH scheduling the PDSCH are quasi co-located with the same DL-RS.

    - Furthermore, the legacy rule that DM-RS port of the PDCCH order and that of the PDSCH scheduled with RA-RNTI in response to PDCCH order triggered CFRA for SpCell are QCLed with the same SSB/CSI-RS should be excluded for the case that random access procedure is triggered by PDCCH order received from non-serving cell or random access procedure is triggered towards non-serving cell by PDCCH order received from serving cell.

Proposal 9: For multi-DCI based multi-TRP operation with two TAs enhancement, in response to a random access procedure triggered by PDCCH order received from non-serving cell or a random access procedure triggered towards non-serving cell, the following should be supported:

- DM-RS port quasi co-location properties of the CORESET associated with the Type1-PDCCH CSS set are assumed for receiving the PDCCH that includes the DCI format 1_0 scrambled by RA-RNTI/MSGB-RNTI.

- DM-RS port of the PDSCH scheduled with RA-RNTI/MSGB-RNTI and DM-RS port of the PDCCH scheduling the PDSCH are quasi co-located with the same DL-RS.

Since PDCCH ordered RACH triggering depends on network implementation, it is up to gNB to decide whether the transmitted PDCCH order is to trigger RACH procedure towards the same TRP or a different TRP. Meanwhile, supporting cross-TRP RACH triggering will not introduce additional UE complexity except extra considerations on QCL properties and PL-RS determination analyzed above. Therefore, cross-TRP RACH triggering should be a basic feature, unless UE capability of supporting cross-TRP QCL properties determination is needed.

Observation 1: Cross-TRP RACH triggering should be a basic feature rather than UE optional.

  - Association of random access procedure and TRP

In RAN1#110b-e meeting, the following agreements on random access procedure related enhancements were endorsed [4].

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support at least one of the following alternatives (down selection to be done in RAN1#111): - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 4: divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 5: divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP - Alt 6: TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order - Alt 7: Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order Note: If Alt 1 or Alt 2 is downselected, then it does not preclude indication of two TAG IDs (if supported) Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support enhancements related to indicating TAG ID via absolute TA command: - FFS: whether the indication is implicit or explicit - Detailed indication schemes are FFS - This does not preclude indication of two TAG IDs (if supported) Note: This applies at least to MSGB in case of C-RNTI

Association between TAG and RACH procedure in intra-cell MTRP

For intra-cell Multi-TRP operation, PRACH configuration can be shared by both TRPs. Consequently, the generated preambles, configured PRACH occasions, and the mapping of SSBs to ROs will be commonly used between two TRPs. Based on that, the above seven alternatives for the association between RACH procedure and TAG in intra-cell multi-DCI based Multi-TRP operation are considered as follows.

To support cross-TRP RACH triggering in inter-cell Multi-TRP operation and strive for a flexible and feasible solution in intra-cell multi-TRP operation, Alt 3 should be considered foremost.

  - For PRACH transmission, PRACH occasions and the mapping of SSB-RO are specified to transmit PRACH using a proper beam. In some ways, TRP can be regarded as different beam directions from gNB’s perspective, hence it is natural to identify TRP based on the beam directions and to divide SSBs into two groups to associate TAG with RACH procedures in intra-cell multi-TRP operation. Meanwhile, supporting Alt 4 can benefit from not introducing additional fields in PDCCH order or RAR.

    - If SSBs from different TRPs can be mapped in the same RO, preambles can be divided into two groups accordingly as in Alt 5.

    - If SSBs from different TRPs are mapped in different ROs, PRACH occasions available for PRACH transmissions of each TRP will be divided into two groups accordingly as in Alt 4.

    - According to different cases of SSB grouping and different mapping of SSB-RO, the cases above can exist simultaneously.

Besides, we have the following analysis for other alternatives.

  - Alt 4 is to divide RACH occasions into two groups will bring additional restrictions to mapping of SSB to RO. Accordingly, either additional mapping rule will be needed or a restrictive configuration of RACH resources will be needed to guarantee the reasonable RACH occasion grouping.

  - Alt 5 will result in resource wasting in some cases. For instance, when SSBs from only one TRP are mapped in a RO, only one group of preambles associated with the TRP can be used for PRACH transmission, and another group of preambles will be left and unused.

  - Both Alt 1 and Alt 2 require to introduce additional fields in the legacy signaling. When a TAG ID is indicated in PDCCH order to identify the TRP/TAG for which the random access procedure is triggered, 3 bits will be needed for the indication field. However, a one-bit field to identify TRP, e.g., a flag or CORESETPoolIndex, is better for overhead reduction. Moreover, it should be noted that the modification of fields in RAR is not expected by RAN2.

  - Both Alt 6 and Alt 7 cannot work in case that PDCCH order triggers RACH procedure towards different TRP, hence they should be considered only if cross-TRP RACH triggering is not supported in intra-cell Multi-TRP operation.

Proposal 10: For multi-DCI based intra-cell Multi-TRP operation with two TAs enhancement, support to divide SSBs into two groups where each group is corresponding to each TRP (Alt 3).

Association between TAG and RACH procedure with 2-step Type

According to TS 38.321, Absolute Timing Advance Command is used for PTAG only and is associated with the MSGB. For two TAs enhancement, Absolute Timing Advance Command should be used for TAGs containing the SPCell for two TRPs, and TRP related identification should be introduced implicitly or explicitly in Absolute Timing Advance Command to associate the TAC with PTAG of the corresponding TRP.

Similar as association between TRP and RAR, if PDCCH order can indicate the TRP identity to UE, MSGB receptions for two TRPs can be differentiated based on the respective MSGB-RNTI, and additional indication fields in absolute TA command are not needed. If PDCCH order can not indicate the TRP identity to UE, either introducing a TRP related identification field in Absolute Timing Advance Command or dividing RACH resources into two groups should be supported additionally.

In order to have a unified solution for both 4-step and 2-step random access procedure, association between a absolute TA command and a TRP should be based on the corresponding MSGB-RNTI.

Proposal 11: For multi-DCI based Multi-TRP operation with two TAs enhancement, support to associate Absolute Timing Advance Command with TAG based on the corresponding MSGB-RNTI for 2-step RACH.

  - Overlapping handling

In RAN1#112 meeting, the following agreement was endorsed for the overlapping handling between UL transmissions with two TAs [2]. Besides, one conclusion was reached in RAN1#110b-e meeting to clarify the assumption of the overlapping with regards to MDCI MTRP operation [4].

Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, down-select at least one of the following in RAN1#112bis-e: - Alt 1: Introducing a time gap X between two UL transmissions associated with two different TA values - E.g., X symbols in the slot(s) corresponding to the two UL transmission remain unused - FFS: How X is determined - Alt 2: Reduce the overlapping duration of one of the two UL transmissions - Alt 3: Scheduling restriction is applied such that the UE does not expect the two UL transmissions to overlap - Other alternatives are not precluded TBD: how to capture the downselected alternative(s) in the specifications in case specification impact is deemed needed. Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs. - Note: This doesn’t prevent the network from applying scheduling restrictions even if the TRPs have no knowledge of the overlapping region

To address the overlapping issue, three alternatives were raised in the last meeting. Subsequently, the related analyses are provided as follows:

  - For Alt 1, it is feasible to guarantee the UL transmissions with two TAGs have no overlapping for both UE side and gNB side, especially in the case that gNB has no awareness of the overlapping region due to the non-ideal backhaul assumption of MDCI MTRP operation. Regarding the value of time gap X, it deemed depends on the setting of UL transmission, i.e., numerology and frequency range. Besides, the maximum receive timing difference (MRTD) decided by RAN4 when two DL reference timings are assumed should be taken into account.

    - Assuming that values of MTTD/MRTD defined for inter-band CA can be applied for the requirements of maximum timing difference for Multi-DCI Multi-TRP operation, MTTD for FR1 and FR2 can be 34.6 µs and 8.5 µs respectively. The candidate value X of time gap can be given in Table 1 by assuming the maximum time gap equals to the value of MTTD.

Table 1: Time gap between two UL transmissions associated with two different TA values

| SCS/kHz | Lsymbol/µs | Maximum Time gap/µs | Maximum Time gap/#. of symbols |
| --- | --- | --- | --- |
| 15 | 71.35 | 34.6 | 1 |
| 30 | 35.68 | 34.6 | 1 |
| 60 | 17.84 | 34.6(FR1), 8.5(FR2) | 1(FR2), 2(FR1) |
| 120 | 8.92 | 8.5 | 1 |

  - For Alt 2, it is somehow similar to the legacy rule of two adjacent slots overlapping handling, where the overlapping part of the latter slot is reduced accordingly. Even though this way can be used to address this issue, it will negatively impact the performance of UL transmission, no matter of the former one or latter one due to the reduction itself.

  - For Alt 3, it can be seen as a variant of Alt 1 in some sense, the minor difference can be that it strictly requires gNB only to avoid this overlapping under MDCI MTRP operation. Based on the conclusion above, this way will not be workable anymore in case that the gNB has no knowledge of the overlapping region between two TRPs.

In light of the above elaboration, Alt 1 should be supported to address this issue.

Proposal 12: Regarding overlapping handling for two uplink transmissions associated with two TAs, support to introduce a time gap in which UE does not expect to transmit any uplink signals/channels if STxMP UL transmission is not supported (Alt 1).

- Candidate values of the time gap includes at least 1 and 2, and other values can be FFS.

- Note: The time gap is from the last symbol of the former uplink transmission associated with one TAG to the first symbol of the latter uplink transmission associated with another TAG.

  - Other Issues

### Issue 1: Signaling of TAC

In RAN1#109-e meeting, the following agreement on the derivation of two TAs has been endorsed [5].

Agreement Enhancement on two TAs for UL multi-DCI for multi-TRP operation is supported in Rel-18. Note 1: whether (1) the network signals two TACs or (2) the network signals one TAC and the UE deriving the second TA can be further studied. Note 2: evaluations can be considered on as-needed basis.

Basically, since the indication of TAC is determined by the network based on the measurement of uplink signals from the UE currently, the amount of timing advance adjustment is determined by the network and then the UE can directly apply the received TAC. Owing to the transmission timing of uplink signals is determined using the downlink reception timing as reference, the determination of timing advance by the network can take both the uplink and downlink propagation delay into account.

Alternatively, if the determination of timing advance is up to UE and also based on the difference of downlink reception timing, timing advance value might be twice of the difference of downlink reception timing assuming the same values of downlink and uplink propagation delay. The determination above might be inappropriate when the values of downlink and uplink propagation delay are different (e.g., corresponding to different physical path in a separate TCI indication). Hence the determination of timing advance decided by the network side should be more reliable than that by the UE side.

Furthermore, if TA value for the second TRP is determined at UE side, extra mechanism of UE reporting in terms of the second TA value is needed, otherwise the gNB cannot know the starting point of the uplink frames for the corresponding TRP or the expected reception timing of the uplink signals/channels.

Observation 2: Timing advance adjustment value should be determined by the signaling from network rather than downlink reception timing measurement by UE.

Indicating individual TACs for two TAGs within a serving cell is a straightforward solution and the legacy timing advance indication framework can be reused smoothly. In this way, UE behaviour of adjusting uplink transmission timing for two TRPs are basically the same, i.e., UE applies a received TAC to a TRP according to the association between TRP and TAG. Once receiving a TAC associated with a TRP, UE adjusts the current timing advance value for the TRP to the new timing advance value based on the corresponding TAC. In such case, TAC indication is entirely individual for both TRPs, hence it can work well even in case of the non-ideal back-haul assumption of Multi-DCI Multi-TRP operation.

Proposal 13: Regarding determination of timing advance values of two TAGs within a serving cell, support that the network signals two TACs, where each TAC is corresponding to a TAG.

### Issue 2: Number of TAGs

In RAN1#110 meeting, the following agreement on supporting two TAGs within a serving cell has been endorsed [6].

Agreement For multi-DCI based multi-TRP operation with two TAs, support configuring two TAGs belonging to a serving cell.

In the current specification, up to four TAGs can be configured, hence it is natural to double the maximum number of TAGs to be 8 when two TAGs can be configured.

Proposal 14: For multi-DCI based multi-TRP operation with two TAs, up to 8 TAGs can be configured.

The index of TAG including the primary serving cell is specified to be 0 in legacy, and the corresponding TAG is regarded as the PTAG. When two TAGs are configured within the primary serving cell, the definition of PTAG in this case and the number of PTAGs (e.g., 1 or 2) should be clarified. Consequently, the follows can be taken into consideration:

  - If only one PTAG is supported, the TAG associated with the primary serving cell and CORESETPoolIndex = 0 is the PTAG.

  - If two PTAGs are supported, both of the two TAGs associated with the primary serving cell are the PTAGs.

Proposal 15: For multi-DCI based multi-TRP operation with two TAGs, RAN1 shall strive to determine the number of PTAGs that can be supported.

### Issue 3: TAT expiry

Time alignment timer (TAT) is configured in TAG-Config per TAG in the current specification and is related with the determination that whether uplink timing is synchronized or not. On the one hand, it has been agreed that two TAGs can be configured within a serving cell and each TAG corresponds to one TRP, hence configuring TRP-specific TAT in the corresponding TAG-Config should be supported. On the other hand, TAC signaling associated with different TAGs can be transmitted individually, timing of starting/restarting the TAT for each TAG will be asynchronous accordingly, hence it is not needed to run one timer for two TAGs.

Proposal 16: For multi-DCI based multi-TRP operation with two TAs enhancement, support to configure TAT per TRP for TAGs within a serving cell in the perspective TAG-Config as legacy.

If timing advance command is separately indicated for each TRP, the reception timing of TAC for each TRP may be different, and it will be inevitable that TAT of one TRP expires while that of another TRP does not expire. Hence, UE’s behaviour in such case should be further considered. Subsequently, it can be that only to cancel the uplink transmissions associated with TRP of which the TAT expires, or fallback to single TA operation, i.e., both TRPs share the same timing advance.

Furthermore, in legacy, when TAT of PTAG expires, all TAGs are regarded as expired. By comparison, only the corresponding STAG is regarded as expired when TAT of a STAG expires. If two PTAGs can be supported, further UE behaviour in case of TAT associated with one out of two PTAGs expires needs to be clarified.

Proposal 17: For multi-DCI based multi-TRP operation with two TAs enhancement, RAN1 shall study UE behaviour in case of TAT expiry if TAT is configured per TAG as legacy.

- I.e., TAT associated with one out of two PTAGs expires.

- Conclusion

In this contribution, we discuss the potential candidate solutions and alternatives to support two TAs for multi-DCI multi-TRP operation with the following observations and proposals.

Observation 1: Cross-TRP RACH triggering should be a basic feature rather than UE optional.

Observation 2: Timing advance adjustment value should be determined by the signaling from network rather than downlink reception timing measurement by UE.

Proposal 1: For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support that RRC-configured UL/joint TCI states of UL channels/signals associated to one CORESET Pool Index correspond to one TAG.

Proposal 2: For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, do NOT support to confirm the Working Assumption that “A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs”.

Proposal 3: For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation in Rel-15/16 spatial relation framework, support that the spatial relations of UL channels/signals associated to one CORESET Pool Index correspond to one TAG.

Proposal 4: Do NOT support two TAs enhancement for multi-DCI based multi-TRP operation in FR1 when Rel-15/16 spatial relation framework is used.

Proposal 5: For multi-DCI based Multi-TRP operation with two TAs enhancement, support the case that a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP for intra-cell Multi-DCI MTRP.

Proposal 6: For multi-DCI based inter-cell Multi-TRP operation with two TAs enhancement, support to introduce an indication of CORESETPoolIndex in PDCCH order to indicate which PRACH configuration is to be used in the triggered RACH procedure.

Proposal 7: For multi-DCI based multi-TRP operation with two TAs enhancement, support to receive RAR from serving cell based on the configured Type-1 PDCCH CSS in response to a random access procedure towards either serving cell or non-serving cell.

Proposal 8: For the case of a PDCCH order sent by one TRP triggers RACH procedure towards a different TRP, pathloss of PRACH transmission is calculated based on the SSB that associated with the PDCCH order.

Proposal 9: For multi-DCI based multi-TRP operation with two TAs enhancement, in response to a random access procedure triggered by PDCCH order received from non-serving cell or a random access procedure triggered towards non-serving cell, the following should be supported:

- DM-RS port quasi co-location properties of the CORESET associated with the Type1-PDCCH CSS set are assumed for receiving the PDCCH that includes the DCI format 1_0 scrambled by RA-RNTI/MSGB-RNTI.

- DM-RS port of the PDSCH scheduled with RA-RNTI/MSGB-RNTI and DM-RS port of the PDCCH scheduling the PDSCH are quasi co-located with the same DL-RS.

Proposal 10: For multi-DCI based intra-cell Multi-TRP operation with two TAs enhancement, support to divide SSBs into two groups where each group is corresponding to each TRP (Alt 3).

Proposal 11: For multi-DCI based Multi-TRP operation with two TAs enhancement, support to associate Absolute Timing Advance Command with TAG based on the corresponding MSGB-RNTI for 2-step RACH.

Proposal 12: Regarding overlapping handling for two uplink transmissions associated with two TAs, support to introduce a time gap in which UE does not expect to transmit any uplink signals/channels if STxMP UL transmission is not supported (Alt 1).

- Candidate values of the time gap includes at least 1 and 2, and other values can be FFS.

- Note: The time gap is from the last symbol of the former uplink transmission associated with one TAG to the first symbol of the latter uplink transmission associated with another TAG.

Proposal 13: Regarding determination of timing advance values of two TAGs within a serving cell, support that the network signals two TACs, where each TAC is corresponding to a TAG.

Proposal 14: For multi-DCI based multi-TRP operation with two TAs, up to 8 TAGs can be configured.

Proposal 15: For multi-DCI based multi-TRP operation with two TAGs, RAN1 shall strive to determine the number of PTAGs that can be supported.

Proposal 16: For multi-DCI based multi-TRP operation with two TAs enhancement, support to configure TAT per TRP for TAGs within a serving cell in the perspective TAG-Config as legacy.

Proposal 17: For multi-DCI based multi-TRP operation with two TAs enhancement, RAN1 shall study UE behaviour in case of TAT expiry if TAT is configured per TAG as legacy.

- I.e., TAT associated with one out of two PTAGs expires.

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator)

- Chair’s Notes RAN1#112 v20

- Chair’s Notes RAN1#111 v15

- Chair’s Notes RAN1#110bis-e v16

- Chair’s Notes RAN1#109-e v19

- Chair’s Notes RAN1#110 v21