3GPP TSG RAN WG1 #112bis-e                                            R1- 2302426

e-Meeting, April 17th –April 26th, 2023

Source:	New H3C

Title:	Discussion for subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for:	Discussion and Decision

- Introduction

In the RAN1 #112 meeting, subband non-overlapping full duplex in Rel.18 evolution of NR duplex operation was discussed. Some agreements were made as below [1].

Agreement

For dynamic SBFD,

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are not allowed

    - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are allowed

Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency.

For each option, additional conditions may apply to determine whether the option is applicable.

Agreement

Study whether or not a slot can consist of both SBFD and non-SBFD symbols including

  - Benefits

  - Use cases

  - Scheduling flexibility

  - Implementation complexity

  - Compatibility with legacy TDD DL/UL configuration

Agreement

For inter-UE inter-subband CLI measurement, study at least the following methods:

  - Method#1: victim UE measures RSSI within DL subband

    - FFS: Whether SINR can be measured

  - Method#2: victim UE measures RSRP of aggressor UE within UL subband

  - Method#3: victim UE measures RSSI within UL subband

  - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP.

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols)

  - Study the following options for SBFD-aware UEs:

    - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

    - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

  - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following:

    - PDSCH/PUSCH/PUCCH repetitions

    - SPS PDSCH/CG PUSCH

    - TBoMS

    - Multi-PUSCH/PDSCH scheduled by a single DCI

    - Periodic/semi-persistent SRS/CSI-RS/PUCCH

    - PDCCH

Agreement

For SBFD-aware UEs, study the at least following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands. For an RBG that overlaps the subband boundary,

  - Option 1:

    - Part of the DL RBG inside the DL subband can be used

    - Part of the UL RBG inside the UL subband can be used

  - Option 2:

    - Part of the DL RBG inside the DL subband cannot be used

    - Part of the UL RBG inside the UL subband cannot be used

FFS: The part of the RBG outside.

Agreement

For SBFD-aware UEs, study at least the following issues for PDSCH:

  - PRG(s) with size of 2 and 4 that overlaps with subband boundary

  - Wideband precoder in case of non-contiguous DL subbands

Agreement:

Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options:

  - Option 1: Two contiguous CSI-RS resources that are linked

  - Option 2: One CSI-RS resource

    - Option 2-1: Non-contiguous CSI-RS resource allocation

    - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

Agreement:

For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols):

  - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols

  - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

Agreement:

Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots:

  - Whether/how to have separate resources

  - Whether/how to have separate FH parameters

  - Whether/how to have separate UL power control parameters

  - Whether/how to have separate beam/spatial relation

In this contribution, we further provide our considerations on subband non-overlapping full duplex.

- Time and frequency location of SBFD

This contribution introduce two kinds of subband resource allocation including semi-static resource allocation and dynamic resource allocation.

  - Time location of SBFD

Based on the conclusion in RAN1#111[1], we have the following agreement on flexible symbol.

Agreement For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs, Option 1: • UL transmissions within UL subband are allowed in the symbol • UL transmissions outside UL subband are not allowed in the symbol • Frequency locations of DL subband(s) are known to the SBFD aware UE • DL receptions within DL subband(s) are allowed in the symbol • FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol Option 2: • UL transmissions within UL subband are allowed in the symbol • The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same o FFS: SBFD aware UE behaviours o FFS: Whether or not signalling of guardband(s) is needed • FFS: Whether or not the symbol can be converted to a DL-only symbol • Frequency locations of DL subband(s) are known to the SBFD aware UE • DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously

Regarding above two options, we prefer option 1 because SBFD operation in a symbol configured as flexible is the same as that in a symbol configured as DL. Compared with Option 2, Option 1 can simplify SBFD design and is good for alignment on SBFD symbol among gNBs and reduces the interference between gNBs. In addition, it isn’t necessary to consider the new definition on guardband or flexible subband and SBFD aware UE behaviours are clear.

Proposal 1：For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, option1 is used for SBFD aware UEs.

Using the flexible symbol as the SBFD symbol can reuse the current mechanism of the frame structure configuration, so it reduce the complexity of the SBFD configuration. However, too much flexible symbols in the frame structure will introduce the scheduling complexity for SBFD. Alternatively, a number of dedicated symbols can be configured as the SBFD symbols in legacy TDD frame structure by using TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated or SFI, where a fixed number of slots/symbols can be configured as SBFD symbol. As shown in Figure 1, symbols #10-#13 in slot1 and symbols #0-#4 in slot0 are configured as SBFD symbol.

. Figure 1 Time location of UL and DL SBFD.

Since the gNB works in the FD mode, the UE still works in the HD mode, so the GP shall be reserved in the UL-DL switching. However, too many GP symbols will reduce the resource efficiency. Therefore, the dedicated SBFD symbol should be configured consecutively in the frame structure for saving GP symbols. The start symbol of the UL SBFD is assumed to be the first symbol of the frame structure period, then the length of the UL SBFD time domain resource can be configured explicitly. The length of UL SBFD time domain resource can be configured by using number of symbol, or by using number of slot together with number of symbol. For example, the, the UL SBFD in Fig.1 is configured as 5 symbols or 0 slot plus 5 symbols.

On the other hands, the start symbol of the DL SBFD is assumed to be the last symbol of the frame structure period, then the length of the DL SBFD time domain resource can be configured by using number of symbol, or by using number of slot together with number of symbol. For example, the, the DL SBFD in Fig.1 is configured as 4 symbols or 0 slot plus 4 symbols.

For one UE, the transmission can be scheduled before the reception in the configured SBFD symbols in order to avoid additional GP configurations. For the UE, the SBFD capability UE will know such configuration, and prepare to received or transmit in this SBFD symbols. In this scheme, once the dedicated SBFD symbols/slots are configured, the SBFD mode will be activated.

Proposal 2：Support the configurations of a number of dedicated symbols as SBFD symbols, the dedicated SBFD symbols should be consecutive and start from the first symbol in a period of the frame structure.

  - Frequency location of SBFD

2.2.1 Frequency indicator of SBFD

In terms of the below agreement of RAN1 #110bis-e meeting, frequency resource can be explicitly indicated. In our view, guard band is necessary to utilize for filtering or handling resourcing outside the UL-SB. Frequency resource of guard band/DL subband need be explicitly indicated. Compared with guard band, frequency resource indication of DL suband is more efficiency and the number of indicated bits of DL suband is less in some cases as shown in Figure 2.

Agreement For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined.

Figure 2 Frequency resource allocation of DL/ UL subband and Guard band

Proposal 3: For semi-static configuration of subband frequency locations for SBFD operation explicit indication of frequency location of DL subband that a set of contiguous RBs for DL transmission excluding RBs of UL subband is required.

2.2.2 Dynamic SBFD resource allocation

In terms of the below agreement of RAN1 #112 meeting, we have below agreement.

Agreement For dynamic SBFD, - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are not allowed - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are allowed Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency. For each option, additional conditions may apply to determine whether the option is applicable.

For SBFD-aware UEs, it is necessary to allow DL receptions outside semi-statically configured DL subband(s) in a symbol configured as DL in TDD-UL-DL-ConfigCommon in order to improve scheduling flexibility in terms of dynamic adjustment of DL traffic.

For SBFD-aware UEs, it is necessary to allow DL receptions outside semi-statically configured DL subband(s) in a symbol configured as flexible in TDD-UL-DL-ConfigCommon in order to improve scheduling flexibility in terms of dynamic adjustment of DL/UL traffic.

Proposal 4: For dynamic SBFD and SBFD-aware UEs, support allowing DL receptions outside semi-statically configured DL subband(s) in a symbol configured as DL in TDD-UL-DL-ConfigCommon.

Proposal 5: For dynamic SBFD and SBFD-aware UEs, support allowing DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside the semi-statically configured UL subbands in a symbol configured as flexible in TDD-UL-DL-ConfigCommon.

- Impact and potential enhancement on SBFD

The impact and potential enhancement on SBFD will be discussed in this section.

3.1 PRACH for SBFD

When semi-static SBFD time-frequency resource is configured by broadcast signaling such as SIB1, if RO for Type-1 random access procedure or MsgA RO and MsgA PUSCH for Type-2 random access procedure as shown in Figure 3 and Figure 4 are allowed to be configured in SBFD resource, for UE supporting SBFD operation, UL access delay is reduced due to the increased random access opportunities. So this benefit is obvious.

Figure 3. RO for Type-1 random access procedure in SBFD resource

Figure 4. MsgA RO and MsgA PUSCH for Type-2 random access procedure in SBFD resource

Proposal 6: RO for Type-1 random access procedure is supported to be configured in semi-static SBFD resource.

Proposal 7: MsgA RO and MsgA PUSCH for Type-2 random access procedure is supported to be configured in semi-static SBFD resource.

3.2 SSB on SBFD

When semi-static SBFD time-frequency resource is configured by broadcast signaling such as SIB1, there will be confliction between the SBFD resource and the legacy channels or signals. In the legacy operation: For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

In the SBFD mode, the above specification should hold true, that is the SSB should not be configured in the pure uplink symbol. However, there will be the case that the SSB is configured overlapping with the UL SBFD symbols (Full or partially overlapping). In this case, the SSB should have higher priority than UL transmission in the UL SBFD. The UE is not expected to transmit any UL in the resource that overlapping with any SSB.

On the other hand, there will also the case that the SSB is configured in the DL SBFD symbols although this SBFD symbol is in the legacy uplink symbol. In this case, the SSB should be configured inside the SBFD sub-band, as shown inFigure 5. In other words, the UE does not expect any SSB PRB or symbol configured outside SBFD sub-band.

Figure 5 SSB on SBFD symbols

Proposal 8: The SSB overlapping with SBFD sub-band should carefully studied. The SSB can be configured inside the SBFD sub-band in the legacy UL symbols. The SSB may be configured overlapping with UL SBFD sub-band in legacy Flexible or DL symbols, in this case the SSB has priority.

3.3 CORESET on SBFD

For CORESET, there are several kinds of CORESET, such as CORESET0, other common CORESET, and UE-specific CORESET. Those CORESET are semi-statically configured, and may overlapped with the semi-static UL SBFD time-frequency resource in DL or Flexible symbol. The CORESET on SBFD is shown in Figure 6, where the CORESET is overlapping with UL SBFD.

Figure 6. CORESET on SBFD

The CORESET should not be configured in the pure uplink symbols, but can be configure in the DL SBFD symbol which can be legacy DL, UL or Flexible symbols. In legacy UL symbol with SBFD configured, only CORESET0 can be configured inside the SBFD sub-band, but cannot be configured overlapping with any symbol of the RACH occasion (RO) and Ngap before valid RO. For other CORESET, they cannot be configured inside the SBFD sub-band in the legacy UL symbols.

Proposal 9: The CORESET overlapping with SBFD sub-band should carefully studied. In legacy UL symbol with SBFD configured, only CORESET0 can be configured inside the SBFD sub-band, but has lower priority than RO.

In the legacy DL symbols with UL SBFD configured, the CORESET can be configured overlapping with UL SBFD, and the CORESET has priority. In the legacy Flexible symbols with SBFD configured, the CORESET can be configured overlapping with SBFD, the common CORESET has priority. In case there is any symbol of the UE-specific CORESET overlapping with the symbols of RO and Ngap symbols before valid RO, then UE-specific CORESET cannot be used for PDCCH transmission.

Proposal 10: In legacy F symbol with SBFD configured, when CORESET overkaps with RO, common CORESET has priority, then RO, then UE-specific CORESET. In legacy DL symbol with SBFD configured, CORESET has priority.

As shown in Figure 7, when frequency allocation of CORESET for PDCCH reception overlaps with UL subband/ ULsubband+Guardband for SBFD operation in a symbol configured as DL in TDD-UL-DL-ConfigCommon, gNB is capable of configuring special/specific/dedicated CORESETs including two independent CORESETs or one CORESET for SBFD without overlapping with UL subband/ ULsubband+Guardband for SBFD operation. So that SBFD aware UE can receive PDCCH better in a symbol configured as DL with SBFD operation in TDD-UL-DL-ConfigCommon.

Proposal 11: In a symbol configured as DL with SBFD operation in TDD-UL-DL-ConfigCommon, specific CORESETs including two independent CORESETs or one CORESET without overlapping with UL subband/ ULsubband+Guardband are configured for PDCCH reception.

Figure 7. CORESET on SBFD

3.4 PUSCH on SBFD

3.4.1 PUSCH transmission in connection mode

When semi-static SBFD time-frequency resource is configured for UL transmission by broadcast/RRC dedicated signaling, gNB could dynamically/semi-static schedule UE to transmit PUSCH w/o repetition in semi-static SBFD time-frequency resource in connection mode as shown in Figure 8and Figure 9.

Figure 8. Dynamically scheduling UE to transmit PUSCH in semi-static SBFD

Figure 9. Semi-static scheduling UE to transmit PUSCH w/o repetition in semi-static SBFD

Proposal 12: UE is dynamically scheduled to transmit PUSCH w/o repetition in semi-static SBFD time-frequency resource in connection mode.

gNB could dynamically allocation SBFD and schedule UE to transmit PUSCH w/o repetition in dynamic allocated SBFD time-frequency resource in connection mode as shown in Figure 10.

Figure 10. Dynamically scheduling UE to transmit PUSCH w/o repetition in dynamic allocated SBFD

Proposal 13: UE is dynamically scheduled to transmit PUSCH w/o repetition in dynamic allocated SBFD time-frequency resource in connection mode.

3.4.2 TBoMS transmission

In order to reduce overhead of signaling, gNB uses one unified configuration to implement TBoMS transmission in SBFD symbols and non-SBFD symbols as shown in Figure 11. Certainly we can also support separated configuration on TBoMS transmission resource in SBFD symbols and non-SBFD symbols in order to avoid increasing complexity of UE implementation on rate matching because partial time and frequency resource of TBoMS is outside time and frequency resource of UL subband of SBFD configuration.

Figure 11. Scheduling UE to transmit TBoMS

Proposal 14: UE is semi-statically or dynamically scheduled to transmit TBoMS in SBFD symbols and non-SBFD symbols by one unified configuration or separated configuration.

Joint channel estimation (JCE) is important feature to improve uplink receive performance. If time and frequency resource of TBoMS/PUSCH repetition type A transmission occasion is outside UL subband of SBFD configuration. It is necessary to allow UL DMRS is transmitted outside UL subband of SBFD configuration in order to guarantee JCE implementation as shown in Figure 12.

Figure 12. JCE for TBoMS transmission in SBFD symbols and non-SBFD symbols

Proposal 15: For TBoMS/ PUSCH repetition type A transmission in SBFD symbols and non-SBFD symbols, UL DMRS is allowed to be transmitted outside UL subband of SBFD configuration.

3.4.3 SDT transmission in inactive mode

In inactive mode, gNB configures semi-static SBFD resource to the UE through RRC broadcast message/RRC dedicated signaling. If SDT PUSCH transmission resources is allocated within the SBFD resource and the SBFD resource is set to UL, UE can use PUSCH occasion in the SBFD to transmit small data. In addition, gNB dynamically schedules retransmission for SDT PUSCH transmission within the SBFD resource in order to reduce UL transmission delay as shown in Figure 13.

Figure 13. Scheduling UE to transmit SDT PUSCH in SBFD resource in inactive mode

Proposal 16: UE is scheduled to transmit SDT PUSCH in SBFD time-frequency resource in inactive mode.

3.5 PDSCH on SBFD

When there is UL transmission in the UL subband for a UE, the PDSCH of other UE can be scheduled in the DL subband. If the resource in the DL subband is not enough for the PDSCH, we should consider the different options for dynamic, semi-static scheduling. For the dynamic scheduling, three options can be considered:

- Option1: The gNB should avoid schedule PDSCH in the DL subband if the resource in the DL subband is not enough for the current PDSCH.

- Option2: The gNB determines the TBS according to the available resource, then the rest will be transmitted in the coming slot.

- Option3: The gNB performs the RM for the PDSCH, the resource that overlapping with UL suband and the guide band will be considered as unavailable resource.

In the dynamic scheduling, the gNB will determine which option to be used, UE will receive the PDSCH according to the DCI. This also applies to retransmission. For example, the Option2 is used for the UE1 PDSCH reception in Figure 14, where only the resource in DL subband is used.

For the semi-static scheduling, the resource allocation will be delivered to the UE when the configuration is activated. We propose the following option for the PDSCH semi-persistent scheduling:

- Option1: The gNB should avoid schedule PDSCH in the DL subband if the resource in the DL subband is not enough for the current PDSCH, and the UE will not expect to receive the SPS PDSCH.

- Option2: The gNB performs the RM for the PDSCH, the resource that overlapping with UL subband and the guide band will be considered as unavailable resource. The UE only received the SPS PDSCH in the DL subband.

Which option will be used should be informed to the UE. This information can be delivered to the UE in RRC, for example, one parameter in the SPS configuration to indicate which option will be used. Alternatively, this information can also delivered to UE during the SPS activation.

For the slot aggregation or repetition scheduled by dynamic scheduling, the same scheme as the SPS PDSCH should applies to the reTx except the 1st Tx. An indicator together with slot aggregation or repetition configuration can be delivered to UE to indicate which option is used. For the slot aggregation or repetition configured by SPS, the same scheme as the SPS PDSCH should applies to all the Tx, the same indication method can also be used as SPS PDSCH. For example, the Option 2 is used for the UE2 SPS PDSCH reception as shown in Figure 14, where the PDSCH Rep1 is RM around UL subband and guard band.

Proposal 17: The PDSCH transmission scheme in the SBFD symbol should be studied. The PDSCH scheduling schemes should be chosen considering the resource overlapping of the UL subband/guard band and PDSCH.

If PDSCH RM around UL subband and guard band, the UE will not perform the PDSCH reception once any RE of the PDSCH DMRS is collided with UL subband or guard band. Besides, the gNB should consider whether the PDSCH can be detected correctly if PDSCH is RM around UL subband or guard band. In this case, one effective code rate (ECR) can be set, once the effective code rate after RM is larger than a threshold, then the gNB will not performs PDSCH RM. The ECR threshold will be determined by gNB itself.

For PDSCH precoder group (PRG), it can be wideband, 2 or 4PRB. When the UL subband is configured, it may not aligned with PRG. For the wideband PRG, all the PRB used by PDSCH in the DL subband will used the same precoder matrix. For the PRG with the size of 2/4 PRB, partial PRB in one PRG may be occupied by UL subband or guard band. In this case, the PRB number in the PRB will reduce to 1/2/3, so those PRB in the PRG will use the same precoder matrix.

Proposal 18: The RE of PDSCH DMRS should not overlap with UL subband or guard band. The ECR of the PDSCH after the RM should not larger than an ECR threshold. Partial PRB in the PRG that overlapping with UL subband or guard band should use the same precoder matrix.

Figure 14. Scheduling PDSCH in DL Subband resource.

In PDSCH resource allocation type0 (RAT0), the resource allocation granularity is RBG. The location of the UL subband and guard band may not align with RBG boundary. Once the UL subband or guard band occupies only parts of the PRB in one RBG, the PDSCH cannot use this RBG anymore according to the current specification, which will waste the resource. In this case, we can introduce an indicator during the frequency resource allocation to inform UE that the PRB not occupied by UL subband or guard band is allocated to the UE. This will improve the spectrum efficiency in the SBFD mode. For example, this indicator can be an parameter together with frequency domain resource allocation in DCI, this size of the parameter depends on the size of the RBG, i.e., log2(RBG size). It is used to indicate how many contiguous PRBs are used in the RBG partial occupied by UL subband and guard band, and the PRB in this RBG and the PRB in the other RBG not occupied by UL subband or guard band is contiguous. Since the UL subband and guard band may be configured in the middle of the BWP, then two indicators should be introduced.

In case of the PUSCH RA, the same idea can also apply to PUSCH. The indicator will introduced in the DCI used for PUSCH or configured grant type0.

Proposal 19: The PRB not occupied by the UL subband or guard band can be used by the PDSCH, this can be achieved by introducing an indicator in the DCI, the size is log2(RBG size).

3.6 Collision of DL/UL channel for SBFD

In terms of the below agreement of RAN1 #110bis-e meeting, some cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE need identified. If UL subband for SBFD in Dl/flexible symbol is configured, there is collision between CG PUSCH and PDCCH / SPS PDSCH as shown in Figure 16. It is straightforward that collision potentially happens between PUCCH and PDCCH / SPS PDSCH.

Agreement Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE - If there are, whether/how to avoid/handle such collision cases (as second step)

Figure 16. Collision case between CG PUSCH and PDCCH / SPS PDSCH

Proposal 20: At least collision between CG PUSCH/PUCCH and PDCCH / SPS PDSCH should be considered and handled.

- Conclusion

In this contribution, we discuss about sub-band non-overlapping full duplex with the following proposal:

Proposal 1：For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, option1 is used for SBFD aware UEs.

Proposal 2：Support the configurations of a number of dedicated symbols as SBFD symbols, the dedicated SBFD symbols should be consecutive and start from the first symbol in a period of the frame structure.

Proposal 3: For semi-static configuration of subband frequency locations for SBFD operation explicit indication of frequency location of DL subband that a set of contiguous RBs for DL transmission excluding RBs of UL subband is required.

Proposal 4: For dynamic SBFD and SBFD-aware UEs, support allowing DL receptions outside semi-statically configured DL subband(s) in a symbol configured as DL in TDD-UL-DL-ConfigCommon.

Proposal 5: For dynamic SBFD and SBFD-aware UEs, support allowing DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside the semi-statically configured UL subbands in a symbol configured as flexible in TDD-UL-DL-ConfigCommon.

Proposal 6: RO for Type-1 random access procedure is supported to be configured in semi-static SBFD resource.

Proposal 7: MsgA RO and MsgA PUSCH for Type-2 random access procedure is supported to be configured in semi-static SBFD resource.

Proposal 8: The SSB overlapping with SBFD sub-band should carefully studied. The SSB can be configured inside the SBFD sub-band in the legacy UL symbols. The SSB may be configured overlapping with UL SBFD sub-band in legacy Flexible or DL symbols, in this case the SSB has priority.

Proposal 9: The CORESET overlapping with SBFD sub-band should carefully studied. In legacy UL symbol with SBFD configured, only CORESET0 can be configured inside the SBFD sub-band, but has lower priority than RO.

Proposal 10: In legacy F symbol with SBFD configured, when CORESET overkaps with RO, common CORESET has priority, then RO, then UE-specific CORESET. In legacy DL symbol with SBFD configured, CORESET has priority.

Proposal 11: In a symbol configured as DL with SBFD operation in TDD-UL-DL-ConfigCommon, specific CORESETs including two independent CORESETs or one CORESET without overlapping with UL subband/ ULsubband+Guardband are configured for PDCCH reception

Proposal 12: UE is dynamically scheduled to transmit PUSCH w/o repetition in semi-static SBFD time-frequency resource in connection mode.

Proposal 13: UE is dynamically scheduled to transmit PUSCH w/o repetition in dynamic allocated SBFD time-frequency resource in connection mode.

Proposal 14: UE is semi-statically or dynamically scheduled to transmit TBoMS in SBFD symbols and non-SBFD symbols by one unified configuration or separated configuration.

Proposal 15: For TBoMS/ PUSCH repetition type A transmission in SBFD symbols and non-SBFD symbols, UL DMRS is allowed to be transmitted outside UL subband of SBFD configuration.

Proposal 16: UE is scheduled to transmit SDT PUSCH in SBFD time-frequency resource in inactive mode.

Proposal 17: The PDSCH transmission scheme in the SBFD symbol should be studied. The PDSCH scheduling schemes should be chosen considering the resource overlapping of the UL subband/guard band and PDSCH.

Proposal 18: The RE of PDSCH DMRS should not overlap with UL subband or guard band. The ECR of the PDSCH after the RM should not larger than an ECR threshold. Partial PRB in the PRG that overlapping with UL subband or guard band should use the same precoder matrix.

Proposal 19: The PRB not occupied by the UL subband or guard band can be used by the PDSCH, this can be achieved by introducing an indicator in the DCI, the size is log2(RBG size).

Proposal 20: At least collision between CG PUSCH/PUCCH and PDCCH / SPS PDSCH should be considered and handled.

- Reference

- Chairman’s Notes, RAN1#112, final, February27th – March 03rd, 2023