3GPP TSG RAN WG1 #112bis-e                                                                          R1-2302428

e-Meeting, April 17th –April 26th, 2023

Source:	New H3C

Title:	Discussions on increased number of orthogonal DMRS ports

Agenda Item:	9.1.3.1

Document for:	Discussion and Decision

- Introduction

In the RAN1 #112 meeting, increased number of orthogonal DMRS ports in Rel.18 NR MIMO evolution was discussed. Some agreements were made as below [1].

Working Assumption

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs,

- Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0.

Table 4-0: DMRS ports for 2CWs.

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 2 | 0,1,2,3,8 |
| 1 | 2 | 0,1,2,3,8,10 |
| 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0,1,2,3,8,9,10,11 |

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-DCI based M-TRP, support at least the following row(s):

- For one CW, support at least row 30 in the following table.

  - For the above row, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

- FFS: other rows are not precluded

Table 7.3.1.2.2-1A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| … | … | … |
| 30 | 2 | 0,2,3 |

Agreement

For Rel.18 eType1/eType2 DMRS ports for PDSCH/PUSCH, support Alt.1 for PTRS RE mapping.

- Alt 1: Different RE offsets set for different Rel.18 DMRS port indexes as shown in Table 4

Table 4 Different RE offsets set for different Rel.18 DMRS port indexes

| DM-RS antenna port p (p for PUSCH, p+1000 for PDSCH) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DM-RS Configuration type 1 |  |  |  | DM-RS Configuration type 2 |  |  |  |
|  | resourceElementOffset |  |  |  | resourceElementOffset |  |  |  |
|  | offset00 | offset01 | offset10 | offset11 | offset00 | offset01 | offset10 | offset11 |
| 0 | 0 | 2 | 6 | 8 | 0 | 1 | 6 | 7 |
| 1 | 2 | 4 | 8 | 10 | 1 | 6 | 7 | 0 |
| 2 | 1 | 3 | 7 | 9 | 2 | 3 | 8 | 9 |
| 3 | 3 | 5 | 9 | 11 | 3 | 8 | 9 | 2 |
| 4 | - | - | - | - | 4 | 5 | 10 | 11 |
| 5 | - | - | - | - | 5 | 10 | 11 | 4 |
| 8 | 4 | 6 | 10 | 0 | - | - | - | - |
| 9 | 6 | 8 | 0 | 2 | - | - | - | - |
| 10 | 5 | 7 | 11 | 1 | - | - | - | - |
| 11 | 7 | 9 | 1 | 3 | - | - | - | - |
| 12 | - | - | - | - | 6 | 7 | 0 | 1 |
| 13 | - | - | - | - | 7 | 0 | 1 | 6 |
| 14 | - | - | - | - | 8 | 9 | 2 | 3 |
| 15 | - | - | - | - | 9 | 2 | 3 | 8 |
| 16 | - | - | - | - | 10 | 11 | 4 | 5 |
| 17 | - | - | - | - | 11 | 4 | 5 | 10 |

Working Assumption

To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables.

- Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table).

  - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1].

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, following Table 7.3.1.1.2-8-X, Table 7.3.1.1.2-9-X, Table 7.3.1.1.2-10-X, and Table 7.3.1.1.2-11-X are supported.

- FFS: Whether to increase the size of antenna ports field in DCI format 0_1/0_2 or not.

This contribution focuses on further discussion of increased number of orthogonal DMRS ports.

- Discussion

Based on the agreement in last meeting[1], we have below work assumption:

Working Assumption For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs, - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0. Table 4-0: DMRS ports for 2CWs. Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8. 1; 2; 0,1,2,3,8,10. 2; 2; 0,1,2,3,8,9,10. 3; 2; 0,1,2,3,8,9,10,11.

For Rel.18 eType1 DMRS ports with maxLength = 1  and 2CWs for PDSCH,  compared with Alt 3-1, we fail to see other option has better gain and benefit. So we suggest to confirm this work assumption with Alt3-1 without any other alternative.

Proposal 1: For Rel.18 eType1 DMRS ports with maxLength = 1  and 2CWs for PDSCH,  below work assumption with Alt3-1 without any other alternative should be confirmed.

Working Assumption For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs, - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0. Table 4-0: DMRS ports for 2CWs. Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8. 1; 2; 0,1,2,3,8,10. 2; 2; 0,1,2,3,8,9,10. 3; 2; 0,1,2,3,8,9,10,11.

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-DCI based M-TRP, we have below agreement.

Agreement For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-DCI based M-TRP, support at least the following row(s): - For one CW, support at least row 30 in the following table. - For the above row, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). - FFS: other rows are not precluded Table 7.3.1.2.2-1A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1 One Codeword: Codeword 0 enabled, Codeword 1 disabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). …; …; …. 30; 2; 0,2,3.

If row 31 is not supported, it is unnecessary to support any other combinations for DMRS etype1 with single-symbol.

Proposal 2: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-DCI based M-TRP, only row 30  agreed in RAN1#112 meeting is supported and others should be precluded.

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, we have the following agreement on antenna ports.

Agreement For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, following Table 7.3.1.1.2-8-X, Table 7.3.1.1.2-9-X, Table 7.3.1.1.2-10-X, and Table 7.3.1.1.2-11-X are supported. - FFS: Whether to increase the size of antenna ports field in DCI format 0_1/0_2 or not.

Regarding whether to increase the size of antenna ports field in DCI format 0_1/0_2 or not, it is unnecessary to increase the size of antenna ports because Rel.18 eType1 DMRS ports with maxLength = 1 and transform precoder is disabled can reuse 4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2.

Proposal 3: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, the size of antenna ports shouldn’t be increased because Rel.18 eType1 DMRS ports with maxLength = 1 and transform precoder is disabled can reuse 4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2.

- Conclusion

In this contribution, we discussed about increased number of orthogonal DMRS ports. The proposals are as follows:

Proposal 1: For Rel.18 eType1 DMRS ports with maxLength = 1  and 2CWs for PDSCH,  below work assumption with Alt3-1 without any other alternative should be confirmed.

Working Assumption For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs, - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0. Table 4-0: DMRS ports for 2CWs. Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8. 1; 2; 0,1,2,3,8,10. 2; 2; 0,1,2,3,8,9,10. 3; 2; 0,1,2,3,8,9,10,11.

Proposal 2: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-DCI based M-TRP, only row 30  agreed in RAN1#112 meeting is supported and others should be precluded.

Proposal 3: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, the size of antenna ports shouldn’t be increased because Rel.18 eType1 DMRS ports with maxLength = 1 and transform precoder is disabled can reuse 4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2.

- Reference

- Chairman’s Notes, RAN1#112, final, February 27th – March 03rd, 2023

- R1-2212760, FL summary on DMRS#3, Moderator (NTT DOCOMO)