3GPP TSG RAN WG1 #112bis-e                                                                          R1-2302429

e-Meeting, April 17th –April 26th, 2023

Source:	New H3C

Title:	Discussions on XR-specific capacity enhancements

Agenda Item:	9.8.1

Document for:	Discussion and Decision

- Introduction

In the RAN1 #112 meeting, XR-specific capacity enhancements in XR Enhancements for NR was discussed. Some agreements were made as below [1].

Conclusion

For convenience in discussion, the term "multi-PUSCHs CG" refers to " a CG PUSCH configuration with multiple CG PUSCH transmission occasions within a period of the CG PUSCH configuration".

Agreement

- Multi-PUSCHs CG is supported for Type-1 configured grant.

- Multi-PUSCHs CG is supported for Type-2 configured grant.

Agreement

For determination of the time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG, the following alternatives for further study:

- Alt-A: TDRA determination based on repetition framework.

  - Alt-A1: Follow the time domain resource mapping of Type A repetition

    - N configured by higher layers or indicated by activation DCI

    - Single SLIV is determined from TDRA

    - The same SLIV in N PUSCH in consecutive slots per CG period

      - FFS for non-consecutive slots

    - FFS details, including related RRC parameters

  - Alt-A2: Follow the time domain resource mapping of Type B repetition

    - N configured by higher layers or indicated by activation DCI

    - Single SLIV is determined from TDRA

      - The SLIV used for 1st PUSCH per CG period.

    - N consecutive nominal PUSCHs with same duration per CG period

    - Note: N is not necessarily the repetition factor.

FFS details, including related RRC parameters

- Alt-B: TDRA determination based on NR-U framework

    - N and M configured by higher layers

    - Single SLIV is determined from TDRA.

      - The SLIV used for 1st PUSCH per CG period.

    - M consecutive PUSCH TOs with same duration in slot. The M PUSCH TOs are used in N consecutive slots per CG period

    - Note: N and M are configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. M and N configuration is independent from cgRetransmissionTimer configuration.

    - FFS details, including related RRC parameters

- Alt-C: TDRA determination based on single DCI scheduling multiple PUSCHs

  - Alt-C1: Follow Rel-16 single DCI scheduling multiple PUSCHs

    - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with k2-r16

    - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period

      - Note: N PUSCH TOs should be consecutive PUSCH TOs in consecutive slots.

    - FFS details, including related RRC parameters

  - Alt-C2: Follow Rel-17 single DCI scheduling multiple PUSCHs

    - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with extendedK2-r17

    - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period

      - Note: N PUSCH TOs can be non-consecutive PUSCHs and/or in non-consecutive slots.

    - FFS details, including related RRC parameters

Agreement

For the PUSCHs parameters in a multi-PUSCHs CG configuration, the configuration/indication parameters except MCS and FDRA of CG PUSCHs in a multi-PUSCHs CG configuration are the same

- FFS: For MCS and FDRA, study further to decide whether/how to be different.

- FFS: Applicability to type-1 and type-2

- Note: TDRA and HP ID are not in this scope of the above statement.

Conclusion

RAN1 discusses to decide how to determine the HARQ process ID of CG PUSCHs of a multi-PUSCHs CG.

Agreement

For determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH, consider the following alternatives:

- Alt. 1:  The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration.

  - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period.

  - Alt 1-1; X = 1

  - Alt 1-2: X is the number of configured PUSCHs in a period

  - Alt 1-3: X is provided by RRC configuration.

  - FFS details

- Alt. 2: Support that UE can decide, as in NR-U, the HARQ IDs for the multiple CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

  - FFS details

- Alt. 3: The HARQ process ID for the configured PUSCHs in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured.

  - FFS on potential enhancements different from previous alternatives

  - Alt 3-1: Note: Same HP ID would be used for all PUSCHs within a period.

    - FFS details

  - Alt 3-2: Note: Different HP ID could be used for all PUSCHs within a period.

    - FFS details

- Alt. 4:  The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured.

  - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period

  - FFS on potential enhancements different from previous alternatives

- Alt 5: Support that UE can decide, as in NR-U, the HARQ IDs for the first CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

  - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period

  - FFS details

- Alt 6: FFS other solutions

Note: The case of one TB map to multiple PUSCHs is not considered here

Agreement

The physical channel that carries the UCI that provides information about unused CG PUSCH transmission occasions is CG PUSCH.

Agreement

Encoding and multiplexing for “the UCI that provides information about unused CG PUSCH transmission occasions” in a CG PUSCH applies encoding and multiplexing procedures for CG-UCI as baseline.

- FFS on details

Agreement

Consider the following alternatives for “the UCI that provides information about unused CG PUSCH transmission occasions” for down-selection or revision

- Alt. 1: “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI.

  - FFS on details

- Alt. 2: “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI.

  - FFS on details

- Alt. 3: “The UCI that provides information about unused CG PUSCH transmission occasions” replaces/re-purposes some field(s) of the CG-UCI.

  - FFS on details

Agreement

For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, the following options for further down-scoping with possible revision, are considered for the transmission occasion of the UCI:

- Option 1: A transmitted CG PUSCH, includes the UCI.

  - FFS details

- Option 2: A transmitted CG PUSCH includes the UCI, if it is transmitted in an occasion determined by RRC.

  - FFS details

- Option 3: A transmitted CG PUSCH includes the UCI, if it is transmitted in a pre-defined transmission occasion.

  - FFS details

    - Example of a pre-determined occasion: 1st configured PUSCH TO in a CG period or 1st configured PUSCH TO in a multiple CG periods

- Option 4: A transmitted CG PUSCH includes the UCI, if it is transmitted in a transmission occasion determined satisfying given condition(s).

  - FFS details

    - Examples of a condition: A first transmitted PUSCH in a CG period, or a first PUSCH transmission within a multiple of CG periods.

Other options are not precluded. Proponent companies to provide details.

Agreement

For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the following options for further down-scoping, are considered for the information provided by the UCI:

- Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused”

  - Option 1-1: The UCI provides the number of consecutive TO(s) in time domain.

    - Applicable numbers can be determined from information obtained from configuration.

    - FFS details

  - Option 1-2: The UCI provides a time duration/range that includes the consecutive TO(s) in time domain.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

- Option 2: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” (consecutive/non-consecutive TO(s) in time domain)

  - Option 2-1: The UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

  - Option 2-2: The UCI provides a bitmap where a bit corresponds to TOs within a time duration/range. The bit indicates whether all TOs within the time duration/range are “unused”.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

- FFS whether/how the unused TO(s) can be associated to multiple CG configuration.

- Other options are not precluded. Proponent companies to provide details.

This contribution focuses on further discussion of XR-specific capacity enhancements.

- Discussion

Based on the agreement in last meeting[1], we have below agreement on the UCI that provides information about unused CG PUSCH transmission occasions :

Agreement Consider the following alternatives for “the UCI that provides information about unused CG PUSCH transmission occasions” for down-selection or revision - Alt. 1: “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI. - FFS on details - Alt. 2: “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI. - FFS on details - Alt. 3: “The UCI that provides information about unused CG PUSCH transmission occasions” replaces/re-purposes some field(s) of the CG-UCI. - FFS on details

Because the purpose of UCI  for unused CG PUSCH transmission occasions is different with the function of  CG-UCI in NR-U, we prefer option 1 to design new UCI for unused CG PUSCH transmission occasions.

Proposal 1: For the UCI that provides information about unused CG PUSCH transmission occasions, Alt. 1: “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI is supported.

For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, we have below agreement。

Agreement For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, the following options for further down-scoping with possible revision, are considered for the transmission occasion of the UCI: - Option 1: A transmitted CG PUSCH, includes the UCI. - FFS details - Option 2: A transmitted CG PUSCH includes the UCI, if it is transmitted in an occasion determined by RRC. - FFS details - Option 3: A transmitted CG PUSCH includes the UCI, if it is transmitted in a pre-defined transmission occasion. - FFS details - Example of a pre-determined occasion: 1st configured PUSCH TO in a CG period or 1st configured PUSCH TO in a multiple CG periods - Option 4: A transmitted CG PUSCH includes the UCI, if it is transmitted in a transmission occasion determined satisfying given condition(s). - FFS details - Examples of a condition: A first transmitted PUSCH in a CG period, or a first PUSCH transmission within a multiple of CG periods. Other options are not precluded. Proponent companies to provide details.

Compared with Option 1 and Option 3, Option 2 and Option 4 introduce some flexibility of configuration but their benefit aren’t clear.

For option 1, compared with option 3, unnecessary overhead of signaling is introduced. So we support option 3 for dynamic indication of unused CG PUSCH occasion(s) based on a UCI.

Proposal 2: For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, Option 3 on a transmitted CG PUSCH including the UCI, if it is transmitted in a pre-defined transmission occasion is supported.

For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, we have the following agreement on UCI indication.

Agreement For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the following options for further down-scoping, are considered for the information provided by the UCI: - Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused” - Option 1-1: The UCI provides the number of consecutive TO(s) in time domain. - Applicable numbers can be determined from information obtained from configuration. - FFS details - Option 1-2: The UCI provides a time duration/range that includes the consecutive TO(s) in time domain. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” (consecutive/non-consecutive TO(s) in time domain) - Option 2-1: The UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2-2: The UCI provides a bitmap where a bit corresponds to TOs within a time duration/range. The bit indicates whether all TOs within the time duration/range are “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - FFS whether/how the unused TO(s) can be associated to multiple CG configuration. - Other options are not precluded. Proponent companies to provide details. This contribution focuses on further discussion of XR-specific capacity enhancements.

Compared with Option 1, although Option 2 introduces some flexibility, signalling overhead of option 2 is also increased. So we support option 1.

Proposal 3: For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused” is supported.

- Conclusion

In this contribution, we discussed about XR-specific capacity enhancements. The proposals are as follows:

Proposal 1: For the UCI that provides information about unused CG PUSCH transmission occasions, Alt. 1: “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI is supported.

Proposal 2: For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, Option 3 on a transmitted CG PUSCH including the UCI, if it is transmitted in a pre-defined transmission occasion is supported.

Proposal 3: For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused” is supported.

- Reference

- Chairman’s Notes, RAN1#112, final, February 27th – March 03rd, 2023