3GPP TSG RAN WG1 #112bis-e			R1-2302431

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.3

Source: 	Quectel

Title: 	 Discussion on Low Power High Accuracy Positioning

Document for:   Discussion and decision

- Introduction

It is agreed that there should be common SRS parameters across the multiple cells in RAN1 #111 meeting [1]. It is approached that the time and frequency resource configuration can be applied to multiple cells in RAN1 #112 meeting [2]. Based on the before conclusions, it is necessary to study the enhancements on SRS configuration and spatial relation for SRS to meet the target battery life in LPHAP.

- SRS configurations for positioning in multiple cells

In RAN1 #112 meeting, the following agreements were made on the study of SRS configuration enhancements [2]:

Agreement For SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support the following: - An SRS positioning validity area consists of cells configured in the same band and the same carrier, and the following parameters with respect to BWP information of SRS for positioning configuration are commonly applied across cells within the validity area: - BWP parameters - locationAndBandwidth - subcarrierSpacing - cyclicPrefix Agreement For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, at least the following parameters in SRS for positioning configuration are commonly configured across cells within the validity area: - srs-PosConfig - SRS-PosResourceSet - srs-PosResourceSetId - srs-PosResourceIdList - resourceType - SRS-PosResource - srs-PosResourceId - transmissionComb - resourceMapping - freqDomainShift - freqHopping - groupOrSequenceHopping-r16 - resourceType - FFS: whether sequenceId is configured commonly across cells or per cell

Sequence Id in SRS configuration should be configured commonly across cells. SRS configuration for the UE in RRC_INACTIVE state is sent to the UE via RRCRelease message by the serving RAN node. When the UE moves out of the serving RAN node cells, the UE will stop transmitting SRS and release the SRS configuration. Therefore, due to the mobility of the UE, potential cell reselection will occur, which will cause the SRS configuration to no longer be valid and the SRS transmission to be terminated. The UE needs to resume connection to the network for the LMF to re-request configuration of SRS for the UE by the retransmission of the event report. This would cause extra power consumption and signalling overhead for a moving UE that performs cell reselection regularly. For legacy UE, when UE moves from one cell to another cell, there is no RACH procedure. The camped RAN node and neighbour RAN nodes has no information of UE camping. If sequence Id is configured per cell, the camped RAN node and neighbour RAN nodes should detect multiple SRS configurations blindly for every possible sequence. However, RAN2 concluded that the support of SRS positioning validity area should not require gNB to monitor multiple SRS configuration simultaneously in TR 38.859 [3]. So, Sequence Id in SRS configuration should be configured commonly across cells.

Proposal 1:

- Sequence Id in SRS configuration should be configured commonly across cells.

3. Spatial relation information of SRS

Spatial relation information is UE location dependent. As the UE moves within a given cell area, e.g. validity area, supporting configuring separate information per cell for spatial relation information would be very useful. In RAN1 #112 meeting, for spatial relation of SRS configuration in multiple cells, the followings are agreed:

Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Spatial relation information is absent in the configuration - FFS: different approaches for down selection at least include the following: - 1a: UE transmits SRS for positioning resources using different spatial domain transmission filter - 1b: UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter - FFS criterion on UE determination of the fixed spatial domain transmission filter (e.g., up to UE implementation, based on a selected SSB of the camping cell, based on the configured path-loss RS such as SSB, etc.) - Option 2: Spatial relation information is provided in the configuration - FFS details on the configuration and corresponding UE behavior, including whether the information is configured for all or subset of cells - FFS signaling to configure the spatial relation information, e.g., via SRS activation message. - Note: UE power consumption needs to be taken into account FFS validity criteria of spatial relation for the configured RS and UE behavior if it determines that the validity criteria of spatial relation for the configured RS is not met, if any, to avoid frequent RRC connection for SRS (re)configuration.

For SRS Tx beam determination across multiple cells, a simple solution is to use SRS Tx beam sweeping for FR2. Actually, in most cases, the base stations for positioning measurement aren’t around the UE  omni-directionally. Figure 1 gives an illustration case sampled in the lived network, where the SRSs transmitted in beam 1~4 can be not used. The transmission of SRS in these beams will cause power and time-frequency resource wasting. When spatial relation information is absent in the configuration, option 1a and option 1b should be supported. UE can make the decision to use option 1a or 1b by implementation based on UE remaining battery capacity.

Proposal 2：

- When spatial relation information is absent in the configuration, option 1a and option 1b should be supported. UE can make the decision to use option 1a or 1b by implementation based on UE remaining battery capacity.

Figure 1 An illustration for spatial relation between UE and base stations

When the criterion on UE determination of the fixed spatial domain transmission filter is based on a selected SSB of the camping cell and neighbour cells, if without Rx beam sweeping, the UE can only have the information of SSB index, but not the direction of Tx beam. When configuring SRS in multiple cells, network should inform UE the relation between SSB indexes and Tx directions for camping cell and neighbour cells in the validity area. If the max number of SSB indexes are equal and the relation between SSB indexes and Tx directions are the same among the camping cell and neighbour cells, only one set of relation is informed to UE. There are three candidates for the maximum number of SSB indexes, which are 4,8 and 64. If the relation between SSB indexes and Tx directions is the same among the camping cell and neighbour cells for each max number of SSB index, three sets of relation are informed to UE for the max number of SSB indexes L as 4,8 and 64 respectively. The same relation between SSB indexes and Tx directions among the camping cell and neighbour cells for each max number of SSB index can reduce the signalling overhead.

Proposal 3：

- The same relation between SSB indexes and Tx directions among the camping cell and neighbour cells for each max number of SSB index can reduce the signalling overhead.

4. Conclusion

In this contribution, we discussed the methods of positioning for LPHAP UEs to reduce the UE power consumption and improve the network efficiency, and our proposals are summarized below:

Proposal 1:

- Sequence Id in SRS configuration should be configured commonly across cells.

Proposal 2：

- When spatial relation information is absent in the configuration, option 1a and option 1b should be supported. UE can make the decision to use option 1a or 1b by implementation based on UE remaining battery capacity.

Proposal 3：

- The same relation between SSB indexes and Tx directions among the camping cell and neighbour cells for each max number of SSB index can reduce the signalling overhead.

Reference

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 111, Toulouse, November 14th – 18th, 2022

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 112 Athens, Greece, February 27 – March 3, 2023

- 3GPP TS 38.859 V18.0.0, Study on expanded and improved NR positioning