3GPP TSG-RAN WG1 #112bis-e 	R1-2302432

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.2.3.2

Source:	New H3C

Title:	Discussion on other aspects of AI/ML beam management

Document for:	Discussion, Decision

- Introduction

In the RAN1 #111 and #112 sub use cases and potential specification impacts for beam management have been further discussed. In this paper, we follow up on the recently made agreements and the status of the discussion in general as it is captured in the FL summary [1].

- Specification impact

Relationship between Set A and Set B

RAN1#110

Agreement

For the sub use case BM-Case1, support the following alternatives for further study:

- Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

- Alt.2: Set B is a subset of Set A

- Note1: Set A is for DL beam prediction and Set B is for DL beam measurement.

- Note2: The beam patterns of Set A and Set B can be clarified by the companies.

Agreement

For the sub use case BM-Case2, further study the following alternatives:

- Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

- Alt.2: Set B is a subset of Set A (Set A and Set B are not the same)

- Alt.3: Set A and Set B are the same

Note1: The beam pattern of Set A and Set B can be clarified by the companies.

In the RAN1#110 meeting, the agreement was achieved as below:

At the training stage, if Set B is a subset of Set A, we can sample the Set A dataset with different pattern to get Set B. If Set A and Set B are different, for each Set B, we need to get a Set A dataset.

In order to reduce the reporting overhead and improve the training efficiency, we have the following proposals.

Proposal 1: For the sub use case BM-Case1, prioritize the study of Alt.2(Set B is a subset of Set A) for further study.

Proposal 2: For the sub use case BM-Case2, prioritize the study of Alt.2(Set B is a subset of Set A) and Alt.3(Set A and Set B are the same) for further study.

Data collection and training

RAN1#110

Agreement

For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact:

- Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals

- Content/type of the collected data

- Other aspect(s) is not precluded

In previous RAN1 meeting, the agreement(s)/conclusion(s) were made as below:

For the network-side mode, the procedure for data collection may like bellow:

Figure 1 Data collection procedure for beam management AI/ML model training

When the network start collecting training data to feed the AI/ML mode, the gNB sends a RRC message to UEs, then UEs log the measurement results of Set A/Set B and report the results to gNB. As the training data training is not sensitive to latency and the dataset may be large, we think it is more suitable to trigger and report the training data through the high layer signaling, from RAN1 aspect, the format of the training data set should be defined.

Proposal 3: For the data collection with network-side model, leave the study of RRC signaling based mechanism to trigger/stop/report the AI/ML training data collection to RAN2.

Proposal 4: For the data collection with network-side model, study necessity and potential specification impact on the training data format (L1-RSRP, timestamps, RS-indicator, etc.)

Inference

RAN1 112

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements:

- Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable

- Beam indication from network for UE reception

Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused).

For the inference of the UE-side model, the following agreement was achieved:

In our view, for the DL Tx beam prediction, network can inform the total information of the associated Set A, but for the beam pair prediction, network only need to inform the associated Tx beam information the UE, then UE can construct the Set A based on Rx beam information. In this way, we prefer to define a mapping of Tx beams within Set A and Tx beams within Set B. If set B is a subset of Set A, a bitmap can be used to define the relationship of Tx beams in set A and set B. If Set B is different with Set A, Tx beam RS indicator of set B can be mapped with the Tx beam RS indicator of set A.

Table 1 The mapping of beams within Set A and Set B for DL Tx beam prediction

| Set A | Tx beam 1 RS indicator | Tx beam 2 RS indicator | …… | Tx beam N RS indicator |
| --- | --- | --- | --- | --- |
| Set B (bitmap) | 0 | 1 | …… | 1 |

Table 2 The mapping of beams within Set A and Set B for beam pair prediction

| Set A | Set A Tx beam 1 RS indicator | Set A Tx beam 2 RS indicator | …… | Set A Tx beam N RS indicator |
| --- | --- | --- | --- | --- |

| Set B | Set B Tx beam 1 RS indicator | Set B Tx beam 2 RS indicator | …… | Set B Tx beam M RS indicator |
| --- | --- | --- | --- | --- |

Proposal 5: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, associate/map beams within Set A and beams within Set B based on the Tx beam information:

- For DL Tx beam prediction, network can inform UE the beam id of set A together with the Tx beam RS indicator, the mapping of Set B Tx beam with Set A Tx beam can use a bitmap method.

- For beam pair prediction, network can inform UE the beam id of Set A and set B together with the Tx beam RS indicator.

Monitoring

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection:

- Atl1. UE-side Model monitoring

  - UE monitors the performance metric(s)

  - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation

- Atl2. NW-side Model monitoring

  - NW monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

- Alt3. Hybrid model monitoring

  - UE monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

For UE-side models under BM-Case1 and BM-Case2, further considerations are required and in RAN1#110bis-e, the following agreements have been made:

In RAN1 112, different alternatives of performance monitoring have been discussed, in our view, as the model is trained on UE side, UE can monitor the model based on implemented and standard input, the monitor performance could be better than just use standard input. From the spec aspect, UE can monitor the beam prediction performance directly, if NW monitors the performance, UE need to report the L1-RSRP of Set A beams to NW to calculate the performance, which will bring additional reporting overhead. Based on the performance and overhead consideration, we prefer to deprioritize the study of NW-side model monitoring.

Proposal 6: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, deprioritize the study of NW-side model monitoring.

- Conclusion

In this paper, we have discussed some details on BM-Case1 and BM-Case2. We have the following observations and proposals:

Proposal 1: For the sub use case BM-Case1, prioritize the study of Alt.2(Set B is a subset of Set A) for further study.

Proposal 2: For the sub use case BM-Case2, prioritize the study of Alt.2(Set B is a subset of Set A) and Alt.3(Set A and Set B are the same) for further study.

Proposal 3: For the data collection with network-side model, leave the study of RRC signaling based mechanism to trigger/stop/report the AI/ML training data collection to RAN2.

Proposal 4: For the data collection with network-side model, study necessity and potential specification impact on the training data format (L1-RSRP, timestamps, RS-indicator, etc.)

Proposal 5: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, associate/map beams within Set A and beams within Set B based on the Tx beam information:

- For DL Tx beam prediction, network can inform UE the beam id of set A together with the Tx beam RS indicator, the mapping of Set B Tx beam with Set A Tx beam can use a bitmap method.

- For beam pair prediction, network can inform UE the beam id of Set A and set B together with the Tx beam RS indicator.

Proposal 6: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, deprioritize the study of NW-side model monitoring.

- Reference

[1] R1-2301897 Summary#4 for other aspects on AI/ML for beam management