# 3GPP TSG RAN WG1 #112bis-e  	                                        R1-2302436

# e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.2.1

Title:	Discussion on general aspects of common AI PHY framework

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In 3GPP TSG RAN#94e meeting, a new SID was approved to study AI/ML technologies over air interface [1]. Until now, a lot of terminologies related to AI/ML procedures have been comprehensively clarified. In addition, the progress on collaboration levels, common KPIs, and high-level frameworks for LCM-related procedures also help with the overall discussion for AI/ML over air. In addition, RAN1#112 meeting had intensive discussions to clarify the details of model and functionality identification.

In this contribution, we provide our further views on the identified issues that should be discussed for general aspects.

Model Life Cycle Management

Data collection

Conclusion in RAN1#110bis-e: Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

To our understanding, data collection has two different ways:

- Case 1: Data generation entity and data processing entity (e.g., for model training/inference/monitoring) are on the same side.

- Case 2: Data generation entity and data processing entity are on different sides.

For Case 1, there are no dedicated configurations/procedures for measurement report of the data, which may be delivered transparently between 3GPP entity and non-3GPP entity (e.g., from gNB to OAM for UL measurement or from UE to private server for DL measurement). Therefore, enhancements for Case 1 may focus on RS configurations required for data collection. In addition, assistance/association information to data collection is also discussed in previous meetings, which can be used to define applicable scenarios/configurations of AI/ML models or as model input to increase inference accuracy. However, some necessary information, e.g., carrier frequency, bandwidth, RS density, has already been provided along with the RS configuration. It could be the assistance information to help data generation entity to classify the associated scenarios/configurations of the data. Moreover, in RAN1#112 meeting, agenda item 9.2.3.2 concluded that there is no consensus to provide some explicit assistance information from UE to network. The concern is that the assistance data belongs private/proprietary information. This conclusion should be also applicable for assistance information from network side.

Conclusion in 9.2.3.2: Regarding the explicit assistance information from network to UE for UE-side AI/ML model, RAN1 has no consensus to support the following information - NW-side beam shape information - E.g., 3dB beamwidth, beam boresight directions, beam shape, Tx beam angle, etc. - Note: Other information (e.g., relative information) of Tx beam(s) preserving sensitive proprietary information is a separate discussion - e.g., some information following the same principle of Rel-17 positioning agreement Conclusion in 9.2.3.2: Regarding the explicit assistance information from UE to network for NW-side AI/ML model, RAN1 has no consensus to support the following information - UE location - UE moving direction - UE Rx beam shape/direction

Therefore, whether to introduce additional assistance information can be further discussed.

When data generation entity and data processing entity (e.g., for model training/inference/monitoring) are on the same side, the data delivery can be transparent between 3GPP entity and non-3GPP entity so that no dedicated configurations/procedures for reporting collected data are required.

When data generation entity and data processing entity (e.g., for model training/inference/monitoring) are on the same side, the specification enhancements should focus on RS configurations for data collection.

Current RS configurations should be reused to define the associated scenarios/configurations of the collected data.

The sharing of assistance data for data collection should keep private/proprietary information.

For Case 2, when the data generation entity is at UE side and data processing entity is at network side, the UE is required to conduct measurements based on configured reference signals. Then, the measurement results are reported to network side. For supervised learning, reliable ground-truth training data and labels are necessary to guarantee model performance. However, current measurement types defined in specification may not be able to provide the required data quality for model training. Therefore, legacy measurements need to be enhanced to increase reliability on the data collection, e.g., high resolution RSRP measurement and high accuracy channel information based on improved Type II codebook. Meanwhile, new measurement types can also be studied per use case. For example, as discussed in CSI use case, model input/output can either be eigenvectors or raw channels. Obviously, this will lead to a lot of standardization efforts on how to define and report the new measurement types. Meanwhile, it’s inevitable that the overhead of transmitting ground-truth training data and labels will be very high. In that sense, Layer-3 measurement report can be considered since data collection results can be reported at off-peak time with large periodicity. Whether and how to support transmission of training data in Layer 3 can be studied by RAN2.

To define new measurement types for data collection, standardization efforts and report overhead should be considered carefully.

When data generation entity and data processing entity (e.g., for model training/inference/monitoring) are on different sides, further study:

Signaling enhancements on configuration of reference signals and measurement report;

Enhance existing measurements or define new measurement types to be discussed per use case.

In addition, offline field data collected by one side can also be shared with the other side for model training, model monitoring, and model update. As discussed in CSI use case, Type 3 training method for two-sided model needs dataset delivery between UE side and network side. For example, gNB-first training requires network to provide a dataset, including labels and intermediate results, to UE side. Then, the UE side will train a CSI generation part based on the delivered dataset. After that, the UE may inform the available CSI generation part to network via model/functionality identification process. During model/functionality identification, the UE should disclose which dataset has been used for training the CSI generation part to facilitate network to choose corresponding CSI reconstruction part.

For Type 3 training collaboration of a two-sided model, common understanding on the dataset used for model training is necessary, which can facilitate the pairing of CSI generation part and CSI reconstruction part.

In addition, for a specific model/functionality, align the same understanding on the dataset used for model training can be useful for testing/monitoring the model performance (i.e., the dataset implicitly defines the test vector for the model/functionality). This is applicable to both UE-side model and UE-part model of a two-sided model. For network can provide a dataset (with ground-truth label) to verify the performance of a model/functionality. Moreover, companies have strong concerns to disclose proprietary information. Therefore, during the data collection, the dataset ID can avoid sharing the proprietary information explicitly across vendors (e.g., codebook implementation, antenna patterns and Rx beam information). In this way, the applicable conditions of the model/functionality can be implicitly indicated by the dataset used for model training.

Dataset alignment between UE and network can be used for testing/monitoring the model/functionality performance.

Dataset ID can avoid sharing the proprietary information explicitly across vendors during data collection.

In RAN1#111, the description for model identification and functionality identification were agreed as below. In our view, the dataset delivery should also reuse such kind of mechanisms, which is to align common understanding between NW and UE.

| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |
| --- | --- |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Therefore, we propose a similar definition for dataset identification:

- A process/method of identifying a dataset for the common understanding between the NW and the UE

- Note: The process/method of dataset identification may or may not be applicable.

- Note: Information regarding the dataset may be shared during dataset identification.

For the purpose of identifying a dataset between network side and UE side, further define the following terminology:

Dataset identification: A process/method of identifying a dataset for the common understanding between the NW and the UE

Note: The process/method of dataset identification may or may not be applicable.

Note: Information regarding the dataset may be shared during dataset identification.

Model training/update

There were some discussions on whether to prioritize offline learning over online learning. Although we have agreed the definitions for the offline learning and the online training, it’s still not clear what are the major differences from specification perspective. Firstly, for network-side model, online/offline training is up to network implementation. In addition, to our understanding, online training for UE-side/part model includes two types:

- Type 1: The model training is conducted inside the UE and the trained model is used for model inference at the UE. By doing this, the device may require an iterative process involving data collection, model training, model validation, and model deployment. Those procedures are hard to be defined in specifications. Besides, model training and model deployment on the UE requires advanced UE capabilities. Therefore, it can be foreseen it is not supported by UE in the near future.

- Type 2: The model training is conducted outside the UE and the trained model is delivered to UE for model inference. For example, other entity will develop a new model. Then, the model currently used by UE side is continuously updated/replaced by the newly developed model.

In our view, we should further discuss the potential specification impacts based on Type 2. The Type 2 is actually similar as model update. If online training is conducted, the on-going model at UE is updated according to model update procedures. The model update procedures include two cases:

- Case 1: update both model parameters and model structure on a legacy model

- Case 2: only update model parameters on the legacy model

| Terminology | Description |
| --- | --- |
| Model update | Process of updating the model parameters and/or model structure of a model |
| Model parameter update | Process of updating the model parameters of a model |

For Case 1, the procedures of model deployment and model inference operation are different from the legacy model to a great extent. For example, model inference latency and model input/output may also be different to the legacy model. Therefore, a new process for model identification should be triggered. For Case 2, model parameter update doesn’t change model inference latency and model input/output, so it doesn’t require a new model identification process. Thus, for further study on model update, we prefer model parameter update than model structure update. For potential specification impacts on model parameter update, interruption mechanisms to replace the legacy model may need to be studied when deploying the updated model.

For network-side model, online/offline training is up to network implementation.

Online training for UE-side/part model includes two types:

Type 1: The model training is conducted inside the UE and the trained model is used for model inference at the UE;

Type 2: The model training is conducted outside the UE and the trained model is delivered to UE for model inference.

If both model parameters and model structure on a legacy model are updated, model inference latency and model input/output of the new model are different to that of the legacy model.

For further study on model update, model parameter update is preferred than model structure update.

Model/functionality identification

Agreement in RAN1#110bis-e: Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations FFS: Detailed discussion of model ID with associated information and/or model functionality. FFS: usage of model ID with associated information and/or model functionality-based LCM procedure FFS: whether support of model ID FFS: the detailed applicable AI/ML operations Agreement in RAN1#111: For UE-part/UE-side models, study the following mechanisms for LCM procedures: - For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. - FFS: Whether or how to indicate functionality - For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs Working Assumption in RAN1#111: Terminology; Description. Model identification; A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.. Terminology; Description. Functionality identification; A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality. Note: whether and how to indicate Functionality will be discussed separately. Agreement in RAN1#112: - AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Agreement in RAN1#112: - For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature. Agreement in RAN1#112: For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model

It’s agreed by most companies that the model identification is generally for UE-side model or UE-part model of a two-sided model. Whether network-side model should have the model identification among network entities can be discussed by RAN2/3.

From RAN1 perspective, model/functionality identification should focus on UE-side model or UE part model of a two-sided model.

Functionality identification and applicable conditions

In RAN1#112 meeting, we had intensive discussions on how to understand functionality identification and model identification. As quoted in above agreements, functionality identification generally refers to legacy UE capability reporting mechanism. Then, model-level LCM may be transparent to functionality-based LCM. In general, we think that how to understand the functionality is really related to how applicable conditions are associated with a functionality. To our understanding, the following three options capture different levels of network control on a functionality:

- Option 1(UE feature based method): Like legacy UE capability report, UE may report its supported features/components for a given AI/ML-enabled feature, e.g., Set A and Set B configurations for beam prediction; antenna port number and CSI payload size for CSI compression. In this way, the functionality is equal to a ML-enabled feature (e.g., a sub use case). And the supported components are applicable conditions of the functionality. The deployment of UE-side/part model is transparent to NW side. The fixed UE capability report may prevent UE from deploying/reporting multiple functionalities (corresponding to different model deployments) for different configurations. Moreover, without other online/offline coordination between UE and network other than the UE capability report, it’s hard for some cases that the coordination is necessary, e.g., model pairing information, dataset alignment for monitoring/testing and applicable scenarios.

- Option 2(Functionality ID based method): As shown in Figure 1, for a given AI/ML-enable feature (e.g., a sub use case), there are multiple functionalities reported by UE to the AI/ML-enabled feature, e.g., reported by different functionality IDs. Different functionalities may have different applicable conditions, which enables UE to train and deploy various models to be applied to different values of the components (or configurations). Then, functionality-ID based LCM (e.g., functionality selection/activation/deactivation) can be initiated. Similar to Option 1, the functionality ID based method is hard to capture dynamic parts that need some other coordination between UE and network, e.g., model pairing information, dataset alignment for monitoring/testing and applicable scenarios.

Figure 1 Functionality ID based method

- Option 3(Functionality ID based method + Model-level operations within a functionality as shown in Figure 2). The functionality identification in Option 1 and Option 2 don’t require offline coordination beyond the specifications, where the supported components for a functionality are defined in specification statically. Thus, Option 1 and Option 2 cannot adapt to dynamic parts that may be hard to be quantized and specified by some candidate values for static UE features, which at least includes following examples:

- The dataset used for model training. This requires offline/online coordination to have common understanding on the dataset. On the one hand, the dataset can indicate the data type/format used for model training, which may facilitate the model performance test. On the other hand, the dataset can actually define applicable conditions for a model. For example, for Type 3 training of a two-sided model, gNB-first training requires network to provide a dataset, including labels and intermediate results, to UE side. Then, the UE can train a CSI generation part based on the delivered dataset. After that, the UE may inform NW about the available CSI generation part. UE should disclose which dataset has been used for training the CSI generation part, which can facilitate network side to choose corresponding CSI reconstruction part and to know the applicable conditions of the CSI generation part.

- Model pairing information for two-sided models. For example, training collaboration type 1 and Type 2 of CSI compression requires offline coordination between network vendor and UE/chipset vendor to have the common understandings on the pairing information. Once UE reports its model availability, e.g., via a global unique model ID associated with a CSI generation part. Then, network side can know which CSI reconstruction part can be paired with the CSI generation part.

- Applicable scenarios of models. Scenario-specific models may be deployed to ensure high performances and to ease the necessity of deploying high-complexity AI/ML models. The models within the same functionality are associated with the same applicable configurations (i.e., UE features/components for static parts). However, the applicable scenarios are hard to be specified as UE features due to its dynamic natures (e.g., Umi vs. Uma indoor vs. outdoor, and first floor vs. second floor). Thus, it requires some offline coordination on the applicable scenarios for different models. Once UE reports its available models (e.g., via model ID) to network for a specific functionality. Network can indicate model-level operations within the functionality according to the applicable scenarios to the models since network have a better knowledge of the surrounding scenarios/environments of the UE.

Figure 2 Functionality ID based method + Model-level operations within a functionality.

According to above discussions, the following table summarizes the pros and cons of the three options:

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 (UE feature based method) | - Reuse legacy 3GPP framework of Features for UE capabilities | - Prevent UE from deploying multiple models for different configurations of a single AI/ML -enabled feature - Hard to capture dynamic parts that need other coordination between UE and network, e.g., model pairing information, dataset alignment for monitoring/testing and different scenarios. |
| Option 2 (Functionality ID based method) | - Limited specification impacts on top of legacy 3GPP framework of Features for UE capabilities - Flexible to support multiple models for different configurations according to different functionalities | - Hard to capture dynamic parts that need other coordination between UE and network, e.g., model pairing information, dataset alignment for monitoring/testing and different scenarios. |
| Option 3 (Functionality ID based method + Model-level operations within a functionality) | - Flexible to support multiple models for different configurations according to different functionalities - Possible to support multiple models for dynamic parts (e.g., model pairing information, dataset alignment for monitoring/testing) within a functionality | - High specification impacts - Clarifications on the relationship between model identification and functionality identification are necessary |

To understand the functionality identification and its applicable conditions, further study the pros and cons of the following options:

Option 1(UE feature based method): Like legacy UE capability report, UE may report its supported features/components (i.e., applicable conditions) for a given AI/ML-enabled feature (i.e., sub use case).

Option 2(Functionality ID based method): For a given AI/ML-enable feature (e.g., a sub use case), there are multiple functionalities reported by UE to the AI/ML-enabled feature, e.g., reported by different functionality IDs. Different functionalities have different applicable conditions, which enables UE to train and deploy various models.

Option 3(Functionality ID based method + Model-level operations within a functionality): In addition to Option 2, model-level operations within a functionality are enabled to address the dynamic parts (e.g., dataset alignment, model pairing information, applicable scenarios) that are hard to be quantized and specified by static UE features.

Legacy UE capability report for AI/ML-enable feature is hard for UE to deploy multiple functionalities for different applicable configurations and multiple models for addressing dynamic parts beyond the UE features, e.g., model pairing information, dataset alignment for monitoring/testing and different scenarios.

Functionality identification should consider the solutions to incorporate both static parts (e.g., UE features by some candidate values) and dynamic parts (e.g., model pairing information, dataset alignment for monitoring/testing and different scenarios).

Model identification and applicable conditions:

To clarify first, the model identification process is to give an identifier to a model (e.g., a global ‘unique’ ID as assumed by RAN2) so that some LCM procedures (e.g., model capability/availability report and model transfer) can have the same understandings on the model to be reported/transferred. However, for other LCM procedures (e.g., model monitoring, model selection/switching/activation/deactivation), the global ‘unique’ ID is not mandatory in order to reduce signaling overhead. Therefore, a local model ID can be used for indicating the model monitoring, model selection/switching/activation/deactivation.

In addition, the model identification process is a naming process, regardless of which entity will do the model training, model storage and model transfer/delivery. Hence, the model identification should be discussed separately from other LCM procedures.

Model identification process is to give an identifier to a model (e.g., a global ‘unique’ ID as assumed by RAN2) so that some LCM procedures (e.g., model capability/availability report, model transfer) can have the same understandings on the model to be reported/transferred. A local model ID can be used for indicating the model operations other than the model capability/availability report and model transfer.

Model identification should be discussed separately from other LCM procedures.

Regarding the naming process to an AI/ML model, it may include the following types according to the offline email discussion:

- Type A1: Model is identified offline to NW (if applicable) and UE (if applicable)

- As demonstrated in Figure 3, the model is assigned with a global model ID. The model ID can be unique within a cell/cell group/PLMN. Then, the model information is stored in a repository at the NW, which is accessible to both NW side and UE side. The model description information (e.g., applicable conditions of the model) about UE-side/part model being identified is provided during the model identification. With the model ID, UE can report its model capability/availability via the model ID.

- The model ID assignment and the association of model description information to a model need some offline coordination between network side and UE side, which is transparent to RAN1 specification. The model identification type A1 should be further studied by RAN2/SA2(e.g., how to register/de-register the model and how to conduct certification test on the model).

Figure 3 Model identification type A1

- Type A2: Model is identified via signaling from UE to NW

- Unlike the Type A1, the naming procedures of Type A2 are defined clearly in specification. As shown in Figure 4, UE reports its model capability/availability to network side. Meanwhile, the corresponding model description information (e.g., applicable conditions of the model) about the model is provided along with the model capability/availability report to the network side. Finally, network side will assign a global ‘unique’ model ID to UE. The model ID assignment should be further studied by RAN2/SA2(e.g., how to register/de-register the model and how to conduct certification test on the model).

- Companies may argue that model identification Type A2 is the same as the functionality identification Option 2(functionality ID based method). Our understanding is that model capability/availability report and functionality capability/availability report may have similar specification impacts as both procedures need to report the applicable conditions (e.g., defined by UE features) of a model or a functionality. However, the functionality ID is reported by UE, which is a local ID within an RRC configuration. No specification impacts beyond the UE capability report is foreseen. The model ID is assigned by network side, which can be global. The model identification may have RAN2/SA2 impacts. Moreover, depend on whether a full model or a model structure is identified, the model description information may include other information apart from applicable conditions.

Figure 4 Model identification type A2

- Type B: Model is identified via signaling from NW to UE

- In model identification Type B, the model being identified is a new model that the UE has never seen before (or without other model identification Type A1 or Type A2 before Type B). For example, as agreed in RAN1#112 for model transfer options, the Type B is to identify a model during model transfer via z5 (	model transfer in open format of an unknown model structure at UE).

- Companies have concern that whether model parameter update (e.g., transferred from network to UE) should be based on Type A1/A2 or Type B. To our understanding, if a model structure is identified via Type A1/A2. Then, UE can report its capability whether it can support model parameter update based on known model structure that is already identified. Therefore, model parameter update can be indicated by UE capability report after model identification Type A1/A2. Furthermore, multiple models with the same structure but with different parameters may need to use a hierarchical ID to uniquely identify a model. For example, a global ID to identify a model structure + a version/local ID to differentiate models with the same structure. How to design the model ID (whether it is hierarchical or non-hierarchical) may be up to RAN2.

To better understand the model identification process, further study following model identification types:

Type A1: Model is identified offline to NW (if applicable) and UE (if applicable)

Type A2: Model is identified via signaling from UE to NW

Type B: Model is identified via signaling from NW to UE

Model identification Type A1 needs some offline coordination between network side and UE side, which is transparent to RAN1 specification.

Send LS to RAN2/SA2 to study further details of the model identification Type A1, Type A2 and Type B (e.g., how to register/de-register the model and how to conduct certification test on the model).

The model identification Type A2 and the functionality identification Option 2(functionality ID based method) have the following similarities and differences:

Similar specification impacts as both procedures need to report the applicable conditions (e.g., defined by UE features) of a model or a functionality.

The functionality ID is reported by UE, which is a local ID within an RRC configuration. No specification impacts beyond the UE capability report is foreseen. The model ID is assigned by network side, which can be global.

Depend on whether a full model or a model structure is identified, the model description information of Type A2 may include other information apart from applicable conditions.

In model identification Type B, the model being identified is a new model that the UE has never seen before (or without other model identification Type A1 or Type A2 before Type B).

Model parameter update can be indicated by UE capability rather than a separate model identification process.

Model categories:

During the offline email discussion, one issue was brought up that whether a model ID used in signaling between UE and network refers to a physical model or a logical model.

- Physical model: Refer to a model file (or an algorithm) either in proprietary-format or open-format;

- Logical model: Refer to an identifier for a model that may be used for LCM purposes. A logical model may be implemented by one or multiple physical models, e.g. multiple proprietary-format models from the same open-format model that are implemented in different hardware platforms or multiple physical models transparent to NW that are signaled as a single model ID.

It should be clarified that which LCM procedure that a model ID should be associated with a physical model or a logical model. To our understanding, when a model ID is signaled along with a model transfer process, the model ID is to indicate a physical model since there is no model in UE’s memory before the model transfer. While in other cases, it’s hard to prevent UE from implementing multiple physical models behind the same model ID used in signaling. In those cases, if the logical model is identified to NW, it may simply identify the applicable conditions that UE is intended to operate by using AI/ML methods. Even a physical model is identified and stored in NW side, if no model transfer is conducted, the model ID visible in the air interface is a logical model. However, in this case, the applicable conditions of the logical model should be the same as the identified physical model.

The model ID used in signaling between UE and network can refer to a physical model or a logical model:

Physical model: Refer to a model file (or an algorithm) either in proprietary-format or open-format;

Logical model: Refer to an identifier for a model that may be used for LCM purposes. A logical model may be implemented by one or multiple physical models, e.g. multiple proprietary-format models from the same open-format model that are implemented in different hardware platforms or multiple physical models transparent to NW that are signaled as a single model ID.

When a model ID is signaled along with a model transfer process, the model ID is to indicate a physical model.

If a logical model is identified to NW, it may simply identify the applicable conditions that UE is intended to operate by using AI/ML methods.

Even a physical model is identified and stored in NW side, if no model transfer is conducted, the model ID visible in the air interface is a logical model. However, in this case, the applicable conditions of the logical model should be the same as the identified physical model.

On the other hand, whether a model (regardless of a physical model or a logical model) identified to NW should satisfy some predictable performance target? For example, RAN2 assumes that a model assigned with a global ‘unique’ ID may need some certification test. In addition, RAN4 may define some minimum performance requirement for the model. Therefore, the test-ability on multiple physical models behind the same logical model should be discussed. One way to resolve this problem can be that model monitoring is required before model activation for inference. Unless the model satisfies the performance target (e.g., defined by RAN4) during the model monitoring, then the model can be activated, which can be further discussed by RAN4. Moreover, the applicable conditions (e.g., RS configurations, dataset used for model training, model pairing information) of the model actually discloses the necessary information for the model monitoring/testing.

Study whether and how a model (regardless of a physical model or a logical model) identified to NW should satisfy some predictable performance target.

Model transfer/delivery

Working Assumption: Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion, Proprietary-format models; ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format. Open-format models; ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective. From RAN1 discussion viewpoint, RAN1 may assume that: - Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared. - Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared

RAN1#111 gave some concrete descriptions about “proprietary model” and “open-format model”. The differentiation is whether a model can be mutually recognizable across vendors. Also, the pros and cons for “proprietary model” and “open-format model” are discussed as below:

- Concerns with model delivery in an open format:

- Once UE receives the open-format model, UE has to compile the model into an executable format. It may require advanced UE capability for compiling and running the model.

- Proprietary model information is disclosed across vendors. In most cases, network vendor and UE/chipset vendor are from different equipment manufacturers, so the intellectual property should be ensured to protect companies’ innovations. However, when the model is in open format, there is no way to avoid the disclosure of proprietary information.

- Model performance (e.g., intermediate KPIs, end-to-end performance, inference latency) is not guaranteed. Generally, device’s AI/ML processor may not be optimized for a certain type of neural network. If UE receives an AI/ML model that cannot be accelerated, the inference latency may be much larger than optimized model. In addition, when UE compiles the AI/ML model, the model parameters may be quantized/tailored to adapt to UE’s hardware environment. Then, the model inference accuracy will be impacted and unpredictable.

- Large specification impacts. From forward compatibility point of view, open format can embrace the rapid development of machine learning society. However, it may require specification efforts to maintain the standardized model format so that it can adapt to various model designs and various platforms developed by companies. Therefore, it should be better to check with other working groups on the feasibility of supporting open format.

- Benefits of model delivery in an open format:

- Shorter model update timescale compared to proprietary-format models that need offline model re-training, compiling, and testing

- Joint model development across vendors. As discussed in section 2.3, if one vendor provides its supported model structure, another vendor can fine-tune the model.

From RAN1 perspective, model delivery in an open-format has following concerns and benefits:

Concerns with model delivery in an open format:

Require advanced UE capability for compiling and running the model;

Proprietary model information disclosed across vendors;

Model performance (e.g., intermediate KPIs, end-to-end performance, inference latency) is not guaranteed;

Large specification impacts.

Benefits of model delivery in an open format:

Shorter model update timescale compared to proprietary-format models that need offline model re-training, compiling, and testing.

Joint model development across vendors.

RAN1 concludes the pros and cons of supporting “proprietary model” and “open-format model”. Then, RAN1 should send LS to other working groups (e.g., RAN2 and SA2) to check the feasibility of supporting open-format models.

Agreement: To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top; Outside 3gpp Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format; 3GPP Network; NW-side. z3; model transfer in open format; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE; 3GPP Network; NW-side. Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. Note: The Case definition is NOT intended to introduce sub-levels of Level z. Note: Other cases may be included further upon interest from companies. FFS: Z4 and Z5 boundary

Depending on the entities involved to deliver AI/ML models and model formats, there could be different options to do the model transfer delivery as agreed in RAN1#112. As discussed above, the boundary between Z4 and Z5 depend on:

- Whether model structure is identified during model identification: YES-> Z4; NO->Z5

- Whether UE has reported its capability to support an identified model structure before the model transfer/delivery: YES-> Z4; NO->Z5

- Whether UE has reported its capability to perform model parameter update based on the identified model structure before the model transfer/delivery: YES-> Z4; NO->Z5

For model transfer/delivery Case Z4, the following procedures are necessary before the model transfer/delivery:

Model structure is identified during model identification

UE has reported its capability to support an identified model structure

UE has reported its capability to perform model parameter update based on the identified model structure

However, the discussion on model storage location and delivery mechanisms falls outside the RAN1 scope, and RAN2 is expected to evaluate these options with the RAN1 studies/agreements being taken into consideration. In addition, RAN2 already starts their work on this issue. The following solutions are agreed to be further studied by RAN2 in RAN2#121 meeting:

- Agreement in RAN2#121: - Aim to at least analyze the feasibility and benefits of model/transfer solutions based on the following: - Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4: Server (e.g. OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g. transparent to 3GPP). Table: relations between solutions and use cases Solutions; Applicable use cases. Solution 1a, 1b; CSI feedback enhancement Beam management Note: No specific considerations for Positioning accuracy enhancement for Solution 1a and 1b.. Solution 2a, 2b; CSI feedback enhancement Beam management Note: No specific considerations for Positioning accuracy enhancement for Solution 2a and 2b.. Solution 3a, 3b; Positioning accuracy enhancement. Solution 4; CSI feedback enhancement Beam management Positioning accuracy enhancement. - Note: the solutions use case relation is preliminary (work in progress), and the purpose is to have better understanding on what to further analyse

Therefore, the model delivery may be from a 3GPP entity (e.g., gNB, LMF and CN) or from a non-3GPP entity (e.g. OAM, OTT). Different use cases may have different options to do the model delivery. The detailed specification impacts can be further studied by RAN2. And RAN1 can proceed current work assuming model delivery may be from a 3GPP entity or from a non-3GPP entity.

Leave the discussion on model storage and delivery options to RAN2. RAN1 can proceed current work assuming model delivery may be from a 3GPP entity (e.g., gNB, LMF and CN) or from a non-3GPP entity (e.g. OAM, OTT).

Model inference operation

Model inference operation happens after model training, model transfer (if applicable), and model deployment. To our understanding, model inference should focus on data required for model input, report feedback based on the model output, and the inference latency. The model input may have specification impacts on reference signal configuration and assistance information delivery. For model output, its specification impacts mainly include quantization methods, measurement report format/UCI mapping order, and the priority for contents included in the measurement report. For inference latency, it’s related to UE processing capability on the model, which has impact on the determination of CSI reference resource, and time offset between activation command and physical channel with measurement report included.

For model inference operation, further study:

Data required for model input, e.g., reference signal configuration and assistance information delivery

Report feedback based on the model output, e.g., quantization methods, UCI mapping order and priority

Inference latency, e.g., the relationship between inference latency and CSI reference resource

Model monitoring

Agreement in RAN1#110bis-e: Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training). FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) Agreement in RAN1#110bis-e: Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system performance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE Agreement in agenda item 9.2.2: In CSI compression using two-sided model use case, study potential specification impact for performance monitoring including: - NW-side performance monitoring: NW monitors the performance and make decisions of model activation/ deactivation/updating/switching - UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model activation/ deactivation/updating/switching Agreement in agenda item 9.2.3: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

There was some progress on model monitoring in agenda item 9.2.2 and 9.2.3. However, it seems that AI CSI and AI beam have different categorizations on the model monitoring. In addition, both the agreements approved in agenda item 9.2.2 and 9.2.3 mix the model monitoring and model selection/activation/deactivation/switching/fallback operation. To our understanding, it has different procedures for the model monitoring and the model selection/activation/ deactivation/switching/fallback operation. At least for UE-side/part model, the discussion on specification impacts should focus on which side to calculate the model monitoring metrics and whether the model monitoring metrics is reported. Depending on the outcome of model monitoring, UE/network can decide whether and how to do follow-up operations. Therefore, the model selection/activation/deactivation/switching/fallback operation should be discussed separately from the model monitoring.

Model selection/activation/deactivation/switching/fallback operation should be discussed separately from model monitoring.

With above discussions, we propose to have a high-level categorization on the model monitoring in this agenda:

- UE-side model monitoring: model monitoring metrics are calculated by UE, and the model monitoring metrics are not reported to network side.

- Network-side model monitoring: model monitoring metrics are calculated by network (with/without the potential to inform UE about the model monitoring metrics).

- Hybrid model monitoring: monitoring metrics are calculated by UE, and then the model monitoring metrics are reported to network side.

The above options have different specification impacts. For UE-side model monitoring, we should discuss about how to define the monitoring metrics and how to claim an AI/ML model is no longer valid (e.g., below a threshold). For network-based model monitoring, both ground-truth label and AI/ML based measurement results need to be reported by UE so that network can calculate the monitoring metrics. Finally, hybrid model monitoring may require new measurement types and new report procedures for the model monitoring metrics.

For UE-side/part model, depending on which side to calculate the model monitoring metrics and whether the model monitoring metrics should be reported, further study following options:

UE-side model monitoring: model monitoring metrics are calculated by UE, and the model monitoring metrics are not reported to network side.

Network-side model monitoring: model monitoring metrics are calculated by network (with/without the potential to inform UE about the model monitoring metrics).

Hybrid model monitoring: monitoring metrics are calculated by UE, and then the model monitoring metrics are reported to network side.

Model selection, activation, deactivation, switching and fallback

Agreement in RAN1#110bis-e: For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms

- According to above agreement, model selection, activation, deactivation, switching, and fallback can be decided either by network side or UE side. To our understanding, it’s risky to allow UE to decide the model status:

- It’s different from the legacy design principles that network should control UE’s key operations;

- It’s network’s responsibility to ensure the overall performance. It’s easier to guarantee the network performance if the decision is indicated by network;

- It cannot be tracked by network whether UE’s performance degradation is due to model status change or other influencing factors;

- Different models have different model performances (e.g., inference latency) and model inputs/outputs. For example, UE can be configured with different time offsets for CSI processing and contents to be included in a CSI report. This cannot be realized if the UE-autonomous decision is supported.

Network should make final decision on model/functionality selection, activation, deactivation, switching, and fallback for UE side/part model.

In RAN1#111, we had discussions to define model selection. The controversy is whether we need to define model selection that is separate from model activation. To our understanding, there could be different options based on various signaling designs:

- Option 1: Model selection (selection of one AI/ML model for activation among multiple models for the same functionality) + model activation (activate the model selected by model selection);

- Option 2: Model selection (selection of multiple AI/ML models for activation among multiple models for the same functionality) + model activation (activate one of the multiple models selected by model selection);

- Option 3: Model activation (activate one of the models supported by UE) without a separate procedure for model selection.

The above options are analogous to CSI report framework defined in current specification. The CSI report framework has a three-level signaling structure, i.e., RRC + MAC CE + DCI. For example, Option 2 is similar to aperiodic CSI report activated by DCI. Therefore, we think it’s not very important to define model selection as it really depends on how current signaling structure is enhanced to incorporate the AI/ML models.

It’s not important to define model selection as it depends on how current signaling structure is enhanced to incorporate the AI/ML models. For example, there could be different options for model selection and model activation based on various signaling designs:

Option 1: Model selection (selection of one AI/ML model for activation among multiple models for the same functionality) + model activation (activate the model selected by model selection);

Option 2: Model selection (selection of multiple AI/ML models for activation among multiple models for the same functionality) + model activation (activate one of the multiple models selected by model selection);

Option 3: Model activation (activate one of the models supported by UE) without a separate procedure for model selection.

UE capabilities

Compared to conventional fixed UE capability report, AI based method needs to study dynamic UE capability report. Due to high complexity in computation and storage, UE can report its temporary UE capability based on its hardware utilization at specific moment. Furthermore, the rapid development of AI/ML technologies makes it very hard to use a fixed UE capability to adapt to different scenarios.

In addition, possible conflicts between conventional method and AI based method should also be considered. In some cases, the concurrent usage of the two methods should be possible. However, some kind of priority may need to be discussed. In an extreme case, when the AI based method cannot guarantee its performance, a fallback behavior to conventional method is necessary to maintain basic network performance.

Finally, all the LCM-related procedures may have different requirements on UE capabilities. However, given that the framework for each procedure is not clear for now, it’s better to defer the discussion on this aspect. Besides, UE capabilities are highly related to different use cases. Therefore, it is better to further study LCM-related UE capabilities per use case.

Further study UE capability to support AI/ML models, at least consider:

Dynamic UE capability report in addition to conventional fixed UE capability report;

Concurrent UE capability for conventional method and AI based method.

Discussions on LCM-related UE capabilities can be postponed or studied per use case basis.

Conclusion

In this contribution, we provide our further views on the identified issues for general aspects of common AI/ML framework. We have the following observations and proposals:

Observation 1: When data generation entity and data processing entity (e.g., for model training/inference/monitoring) are on the same side, the data delivery can be transparent between 3GPP entity and non-3GPP entity so that no dedicated configurations/procedures for reporting collected data are required.

Observation 2: To define new measurement types for data collection, standardization efforts and report overhead should be considered carefully.

Observation 3: For Type 3 training collaboration of a two-sided model, common understanding on the dataset used for model training is necessary, which can facilitate the pairing of CSI generation part and CSI reconstruction part.

Observation 4: Dataset alignment between UE and network can be used for testing/monitoring the model/functionality performance.

Observation 5: Dataset ID can avoid sharing the proprietary information explicitly across vendors during data collection.

Observation 6: For network-side model, online/offline training is up to network implementation.

Observation 7: Online training for UE-side/part model includes two types:

• Type 1: The model training is conducted inside the UE and the trained model is used for model inference at the UE;

• Type 2: The model training is conducted outside the UE and the trained model is delivered to UE for model inference.

Observation 8: If both model parameters and model structure on a legacy model are updated, model inference latency and model input/output of the new model are different to that of the legacy model.

Observation 9: Legacy UE capability report for AI/ML-enable feature is hard for UE to deploy multiple functionalities for different applicable configurations and multiple models for addressing dynamic parts beyond the UE features, e.g., model pairing information, dataset alignment for monitoring/testing and different scenarios.

Observation 10: Model identification process is to give an identifier to a model (e.g., a global ‘unique’ ID as assumed by RAN2) so that some LCM procedures (e.g., model capability/availability report, model transfer) can have the same understandings on the model to be reported/transferred. A local model ID can be used for indicating the model operations other than the model capability/availability report and model transfer.

Observation 11: Model identification Type A1 needs some offline coordination between network side and UE side, which is transparent to RAN1 specification.

Observation 12: The model identification Type A2 and the functionality identification Option 2(functionality ID based method) have the following similarities and differences:

Observation 13: In model identification Type B, the model being identified is a new model that the UE has never seen before (or without other model identification Type A1 or Type A2 before Type B).

Observation 14: Model parameter update can be indicated by UE capability rather than a separate model identification process.

Observation 15: When a model ID is signaled along with a model transfer process, the model ID is to indicate a physical model.

Observation 16: If a logical model is identified to NW, it may simply identify the applicable conditions that UE is intended to operate by using AI/ML methods.

Observation 17: Even a physical model is identified and stored in NW side, if no model transfer is conducted, the model ID visible in the air interface is a logical model. However, in this case, the applicable conditions of the logical model should be the same as the identified physical model.

Observation 18: From RAN1 perspective, model delivery in an open-format has following concerns and benefits:

• Concerns with model delivery in an open format:

◦ Require advanced UE capability for compiling and running the model;

◦ Proprietary model information disclosed across vendors;

◦ Model performance (e.g., intermediate KPIs, end-to-end performance, inference latency) is not guaranteed;

◦ Large specification impacts.

• Benefits of model delivery in an open format:

◦ Shorter model update timescale compared to proprietary-format models that need offline model re-training, compiling, and testing.

◦ Joint model development across vendors.

Observation 19: For model transfer/delivery Case Z4, the following procedures are necessary before the model transfer/delivery:

• Model structure is identified during model identification

• UE has reported its capability to support an identified model structure

• UE has reported its capability to perform model parameter update based on the identified model structure

Observation 20: It’s not important to define model selection as it depends on how current signaling structure is enhanced to incorporate the AI/ML models. For example, there could be different options for model selection and model activation based on various signaling designs:

• Option 1: Model selection (selection of one AI/ML model for activation among multiple models for the same functionality) + model activation (activate the model selected by model selection);

• Option 2: Model selection (selection of multiple AI/ML models for activation among multiple models for the same functionality) + model activation (activate one of the multiple models selected by model selection);

• Option 3: Model activation (activate one of the models supported by UE) without a separate procedure for model selection.

Proposal 1: When data generation entity and data processing entity (e.g., for model training/inference/monitoring) are on the same side, the specification enhancements should focus on RS configurations for data collection.

Proposal 2: Current RS configurations should be reused to define the associated scenarios/configurations of the collected data.

Proposal 3: The sharing of assistance data for data collection should keep private/proprietary information.

Proposal 4: When data generation entity and data processing entity (e.g., for model training/inference/monitoring) are on different sides, further study:

• Signaling enhancements on configuration of reference signals and measurement report;

• Enhance existing measurements or define new measurement types to be discussed per use case.

Proposal 5: For the purpose of identifying a dataset between network side and UE side, further define the following terminology:

• Dataset identification: A process/method of identifying a dataset for the common understanding between the NW and the UE

• Note: The process/method of dataset identification may or may not be applicable.

• Note: Information regarding the dataset may be shared during dataset identification.

Proposal 6: For further study on model update, model parameter update is preferred than model structure update.

Proposal 7: From RAN1 perspective, model/functionality identification should focus on UE-side model or UE part model of a two-sided model.

Proposal 8: To understand the functionality identification and its applicable conditions, further study the pros and cons of the following options:

• Option 1(UE feature based method): Like legacy UE capability report, UE may report its supported features/components (i.e., applicable conditions) for a given AI/ML-enabled feature (i.e., sub use case).

• Option 2(Functionality ID based method): For a given AI/ML-enable feature (e.g., a sub use case), there are multiple functionalities reported by UE to the AI/ML-enabled feature, e.g., reported by different functionality IDs. Different functionalities have different applicable conditions, which enables UE to train and deploy various models.

• Option 3(Functionality ID based method + Model-level operations within a functionality): In addition to Option 2, model-level operations within a functionality are enabled to address the dynamic parts (e.g., dataset alignment, model pairing information, applicable scenarios) that are hard to be quantized and specified by static UE features.

Proposal 9: Functionality identification should consider the solutions to incorporate both static parts (e.g., UE features by some candidate values) and dynamic parts (e.g., model pairing information, dataset alignment for monitoring/testing and different scenarios).

Proposal 10: Model identification should be discussed separately from other LCM procedures.

Proposal 11: To better understand the model identification process, further study following model identification types:

• Type A1: Model is identified offline to NW (if applicable) and UE (if applicable)

• Type A2: Model is identified via signaling from UE to NW

• Type B: Model is identified via signaling from NW to UE

Proposal 12: Send LS to RAN2/SA2 to study further details of the model identification Type A1, Type A2 and Type B (e.g., how to register/de-register the model and how to conduct certification test on the model).

• Similar specification impacts as both procedures need to report the applicable conditions (e.g., defined by UE features) of a model or a functionality.

• The functionality ID is reported by UE, which is a local ID within an RRC configuration. No specification impacts beyond the UE capability report is foreseen. The model ID is assigned by network side, which can be global.

• Depend on whether a full model or a model structure is identified, the model description information of Type A2 may include other information apart from applicable conditions.

Proposal 13: The model ID used in signaling between UE and network can refer to a physical model or a logical model:

• Physical model: Refer to a model file (or an algorithm) either in proprietary-format or open-format;

• Logical model: Refer to an identifier for a model that may be used for LCM purposes. A logical model may be implemented by one or multiple physical models, e.g. multiple proprietary-format models from the same open-format model that are implemented in different hardware platforms or multiple physical models transparent to NW that are signaled as a single model ID.

Proposal 14: Study whether and how a model (regardless of a physical model or a logical model) identified to NW should satisfy some predictable performance target.

Proposal 15: RAN1 concludes the pros and cons of supporting “proprietary model” and “open-format model”. Then, RAN1 should send LS to other working groups (e.g., RAN2 and SA2) to check the feasibility of supporting open-format models.

Proposal 16: Leave the discussion on model storage and delivery options to RAN2. RAN1 can proceed current work assuming model delivery may be from a 3GPP entity (e.g., gNB, LMF and CN) or from a non-3GPP entity (e.g. OAM, OTT).

Proposal 17: For model inference operation, further study

• Data required for model input, e.g., reference signal configuration and assistance information delivery

• Report feedback based on the model output, e.g., quantization methods, UCI mapping order and priority

• Inference latency, e.g., the relationship between inference latency and CSI reference resource

Proposal 18: Model selection/activation/deactivation/switching/fallback operation should be discussed separately from model monitoring.

Proposal 19: For UE-side/part model, depending on which side to calculate the model monitoring metrics and whether the model monitoring metrics should be reported, further study following options:

• UE-side model monitoring: model monitoring metrics are calculated by UE, and the model monitoring metrics are not reported to network side.

• Network-side model monitoring: model monitoring metrics are calculated by network (with/without the potential to inform UE about the model monitoring metrics).

• Hybrid model monitoring: monitoring metrics are calculated by UE, and then the model monitoring metrics are reported to network side.

Proposal 20: Network should make final decision on model/functionality selection, activation, deactivation, switching, and fallback for UE side/part model.

Proposal 21: Further study UE capability to support AI/ML models, at least consider:

• Dynamic UE capability report in addition to conventional fixed UE capability report;

• Concurrent UE capability for conventional method and AI based method.

Proposal 22: Discussions on LCM-related UE capabilities can be postponed or studied per use case basis.

References

RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

Draft Report of 3GPP TSG RAN WG1 #110bis-e

Draft Report of 3GPP TSG RAN WG1 #111

Draft Report of 3GPP TSG RAN WG1 #112