3GPP TSG RAN WG1 Meeting #112bis-e	R1-2302438

e-Meeting, April 17th – April 26rd, 2023

Source:	ZTE

Title:	Discussion on other aspects for AI CSI feedback enhancement

Agenda Item:	9.2.2.2

Document for:	Discussion and Decision

- Introduction

According to discussions in 3GPP RAN1#112 meeting [1], much progress has been made on other aspects for AI CSI feedback enhancement and some agreements have been reached.

In this contribution, we provide our views on the potential specification impacts for CSI feedback enhancement. In our companion contribution [2], some related evaluation results on AI/ML for CSI feedback are discussed and assessed.

- Spatial-frequency domain CSI compression using two-sided model

  - Training collaboration

This sub use case involves two-sided model inference operations performed at both UE and gNB. Specifically, UE deploys (or be configured with) CSI generation part and gNB deploys corresponding CSI reconstruction part, where the former one is for CSI compression and the latter one is to recover more accurate CSI for better MU operation in massive MIMO scenario. However, how to train and collaborate the two-sided AI model is a key problem, which has impacts on the existing specifications. The following progress about training collaborations has been reached in RAN1#110 meeting [3], RAN1#111 meeting [5] and the following features to compare the training collaboration types are concluded in RAN1#112 meeting [1]:

Agreement: In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, respectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes). - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW - Other collaboration types are not excluded. Conclusion - In CSI compression using two-sided model use case, training collaboration type 2 over the air interface for model training (not including model update) is deprioritized in R18 SI. Conclusion: In CSI compression using two-sided model use case, further discuss the pros/cons of different offline training collaboration types including at least the following aspects: - Whether model can be kept proprietary - Requirements on privacy-sensitive dataset sharing - Flexibility to support cell/site/scenario/configuration specific model - gNB/device specific optimization – i.e., whether hardware-specific optimization of the model is possible, e.g. compilation for the specific hardware - Model update flexibility after deployment - feasibility of allowing UE side and NW side to develop/update models separately - Model performance based on evaluation in 9.2.2.1 - Whether gNB can maintain/store a single/unified model - Whether UE device can maintain/store a single/unified model - Extendability: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use - Whether training data distribution can be matched to the device that will use the model for inference - Whether device capability can be considered for model development - Other aspects are not precluded - Note: training data collection and dataset/model delivery will be discussed separately

Type 1: Joint training of the two-sided model at a single side/entity

For training Type 1, joint training of the two-sided model can be performed either at network side or UE side.

Joint training at NW side

- Advantages on joint training at NW side:

  - Model performance: It can achieve the optimal performance since the CSI generation model and the CSI reconstruction model are jointly designed and trained at network side based on abundant data collected from commercial UEs.

  - Whether gNB can maintain/store a single/unified model: Network can train a unified CSI reconstruction model to adapt to multiple CSI generation models from different UE vendors. It avoids the problem of multi-vendor model pairing that network needs to maintain many pairs of UE-network specific models.

  - Model update flexibility after deployment: Network can be flexible to re-develop/update/switch the models to adapt to channel variations.

  - Network can support cell/site/scenario/configuration specific model flexibility.

- Concerns on joint training at NW side:

  - Device specific optimization: Due to the fact that AI/ML model operation depends on the hardware/software capability, the AI/ML model trained at network side may be not compatible with UE hardware/software platform, which results in the compiling failure of an unseen model delivered from network to UE. In addition, the computation capability/efficiency of network and UE may be mismatched, which results in computation efficiency degradation/inference delay increase or even operation failure. However, if the model structure of UE-part model is known by network, the issue of specific optimization can be alleviated.

  - Model format alignment: UE may not identify the model format and compile the model delivered/transferred from network to UE, since the AI/ML model generation platform at network may be different from UE. Hence, the issue of the model format alignment needs to be solved. There are two separate types of model format are discussed in 9.2.1, which is open-format model and proprietary-format model, respectively. How to define 3GPP-specific model format needs further study, which would bring heavy workload across working groups.

  - Whether model can be kept proprietary: The implementation of AI/ML models is basically a proprietary manner. When the trained model at network side is delivered to UE, some proprietary information has to be inevitably disclosed to UE.

  - Feasibility of allowing UE side and NW side to develop/update models separately: Due to the fact that joint training is performed at NW side, it is not feasible to allow UE to develop/update models on its own since it may incur the mismatch between the pairing of UE-part model and NW-part model.

  - Extendability: The supported model structures of the CSI generation part need to be aligned between the Network vendor and the UE vendor, e.g., in an offline manner. This would lead to large offline co-engineering workload. However, it is possible for UE to develop new models only when UE knows the NW-part model in use.

  - Whether training data distribution can be matched to the device that will use the model for inference: Considering the training dataset may be collected from multiple UE vendors, the training data may follow different data distributions due to the hardware variations and different proprietary data processing operations of UE vendors. Therefore, the training data distribution would mismatch the actual inference data.

  - Whether device capability can be considered for model development: When the supported UE-model structure is known by network side, network can train the UE-specific model which is compatible with the UE capability.

Joint training at UE side

- Advantages on joint training at UE side:

  - Whether UE device can maintain/store a single/unified model: UE can train a unified CSI generation model to adapt to multiple CSI reconstruction model from different NW vendors. It relieves the problem of multi-vendor model pairing that UE needs to store and deploy many pairs CSI generation models.

  - Whether training data distribution can be matched to the device that will use the model for inference: Training data distribution can be matched to the device that will use the model for inference.

  - Whether training data distribution can be matched to the device that will use the model for inference: Since the training dataset is collected from the specific UE vendor and the data processing operation is known by UE vendor, hence training data distribution can match the actual inference data.

- Concerns on joint training at UE side:

  - Model performance: The network performance would be constrained by the models trained at UE side, since the training dataset is UE vendor-specific. In addition, UE vendor-specific models may result in a limited model generalization capability.

  - Whether gNB can maintain/store a single/unified model: The network needs to store or operate many pairs of UE-specific models from multiple UE vendors, which results in huge computation and storage burden of network side.

  - Flexibility to support cell/site/scenario/configuration specific model: For the joint training at Network side, Network vendor can flexibly perform cell/site/scenario specific model training based on specific network planning and site types. However, for the joint training at UE side, dataset collected by UE vendors may not match the specific cell environment of the Network vendor.

  - Model update flexibility after deployment: Model updating at UE side is not flexible since UE cannot train the model at the UE device due to the limitation of the UE capability.

  - Jointly training at UE side also faces the issue of gNB specific optimization, model format alignment, and model proprietary as analyzed in joint training at network side.

  - Feasibility of allowing UE side and NW side to develop/update models separately: Due to the fact that joint training is performed at UE side, it is not feasible to allow NW to develop/update models on its own since it may incur the mismatch between the pairing of NW-part model and UE-part model.

  - Extendability: The supported model structures of the CSI generation part need to be aligned between the Network vendor and the UE vendor, e.g., in an offline manner. This would lead to large offline co-engineering workload. However, it is possible for NW to develop new models only when NW knows the UE-part model in use.

  - Whether device capability can be considered for model development: It is not necessary to consider UE device capability for model development, since it is up to UE vendor implementation.

Based on the above analysis, we propose to prioritize Type 1 joint training at NW side as a starting point for further study. In addition, model transfer/delivery in training Type 1 would bring great specification impact, and this issue can be further discussed in Agenda item 9.2.1. Besides, from procedure and signaling point of view, the two cases should share as much commonality as possible. Therefore, we should further study whether study outcomes of Type 1 joint training at NW side can be also applicable to Type 1 joint training at UE side.

Proposal 1: Prioritize Type 1 joint training at NW side for further study and model transfer/delivery can be further discussed in agenda item 9.2.1.

Type 3: Separate training at NW side and UE side, where the UE-side CSI generation part and the NW-side CSI reconstruction part are trained by UE side and network side, respectively.

For training Type 3, sequential training of the two-sided model can be started either at NW side or UE side.

NW-first training

- Advantages on NW-first training:

  - Model performance: NW-first training achieves the similar performance as training Type 1, where the results are evaluated in our companion contribution [2].

  - Whether gNB can maintain/store a single/unified model: Network can train a unified CSI reconstruction model to adapt to multiple CSI generation models from different UE vendors. The model performance is maintained even in CSI generation models with different model structures, which are well-designed and trained based on the common sharing dataset.

  - Network can train a cell/site/scenario/configuration specific model and deliver the cell/site/scenario/configuration specific dataset to UE for training.

  - Device specific optimization: The development of the CSI generation part and the CSI reconstruction part is performed by the UE vendor and the Network vendor, respectively, thus the gNB/device specific optimization is allowed.

  - Whether model can be kept proprietary: Model proprietary can be guaranteed since the CSI generation model and CSI reconstruction model are trained separately at UE side and network side, hence model delivery/transfer between network and UE is not needed.

  - Feasibility of allowing UE side and NW side to develop/update models separately: The individual engineering of developing/updating can be better ensured since the Network part model and the UE part model are developed and trained based on the corresponding training dataset, respectively.

  - Extendability: The development of the CSI generation part and the CSI reconstruction part is performed by the UE vendor and the Network vendor based on the delivered dataset, respectively. Thus, it is feasible to train a new UE-side model compatible with NW-side model in use only if the new model is trained on the dataset that has the same distribution as the data for model inference.

- Concerns on NW-first training:

  - Model performance: Model performance loss due to the huge differences on model structure between UE-sided CSI generation part and NW-sided CSI generation part. For example, if UE trains a light-weighted CSI generation model to adapt to the NW-sided reconstruction model, the output of CSI generation part doesn’t follow the same data distribution as delivered dataset.

  - Whether training data distribution can be matched to the device that will use the model for inference: The data distribution is the same as the device if the ground-truth CSI is shared from the target UE device. Therefore, the training data distribution can be matched to the model for inference with restriction.

  - Model updating flexibility after deployment: Since the required dataset size of model updating/fine-tuning may be much less than that of model training, dataset sharing for model updating can be also performed via air interface with acceptable overhead. Compared with Type 1 joint training at NW side, there may be less flexibility for model updating, since for Type 1, the delivered model can be directly used, or implemented after compiling; while for Type 3, UE side still needs to perform training in prior.

  - Whether device capability can be considered for model development: It is not necessary to consider UE device capability for model development, since it is up to UE vendor implementation to develop the AI/ML model based on the delivered training dataset.

  - Overhead: NW-first training involves the labels and intermediate results sharing from NW to UE, which needs high resolution of data quantization method to guarantee the dataset accuracy. Hence, the large overhead of sharing dataset is a key issue to be studied.

UE-first training

UE-first training is a symmetric way to the NW-first training, i.e. sequential training started at UE side.

- Advantages on UE-first training:

  - Similar to NW-first training, UE-first training can avoid the model incompatibility issue and model format misalignment issue due to separate training at UE and NW, respectively. In addition, it is feasible for UE and NW to develop/update models separately, and operate the specific optimization method according to its own hardware capability.

  - Whether UE device can maintain/store a single/unified model: UE can train a unified CSI generation model to adapt to multiple CSI reconstruction models from different NW vendors based on the sharing dataset from UE to NW.

  - Whether model can be kept proprietary: Model proprietary can be guaranteed since the CSI generation model and CSI reconstruction model are trained separately at UE side and network side, hence model delivery/transfer between network and UE is not needed.

  - Whether training data distribution can be matched to the device that will use the model for inference: Training data distribution can be matched to the device that will use the model for inference, since the data processing method can be aligned, which is up to UE implementation.

  - Extendability: The development of the CSI generation part and the CSI reconstruction part is performed by the UE vendor and the Network vendor based on the delivered dataset, respectively. Thus it is feasible to train a new NW-side model compatible with UE-side model in use only if the new model is trained on the dataset that has the same distribution as the data for model inference..

- Concerns on UE-first training:

  - Model performance: UE-first training incurs the performance degradation compared with training Type 1 and NW-first Type 3. In our companion contribution [2], the evaluation results show that there is minor margin (~1%) between the performance of the UE-first training and the performance of both training Type 1 and NW-first training. In addition, the system performance suffers from the NW-sided CSI reconstruction model design when the structure of CSI reconstruction part at NW has a big difference from the UE-sided CSI reconstruction part.

  - Whether gNB can maintain/store a single/unified model: The network has to train multiple AI/ML models based on multiple shared dataset from different UE vendors, respectively, which would impose storage burden for NW. Besides, the dataset collected by UE side may not match the channel scenarios at NW side and the quality of the shared dataset from different UE vendors may vary greatly, which may cause certain network performance degradation.

  - Flexibility to support cell/site/scenario/configuration specific model: The dataset collected by UE side may not match the channel characteristics at the Network. Although the Network vendor may want to perform cell/scenario specific model training while the dataset provided by UE vendors may not involve that categorization. Thus, cell/site/scenario/configuration specific model is not flexible to support.

  - Model updating flexibility after deployment: Model update may be not flexible after deployment for the UE-first training, since the training entity of the UE side would be the non-3GPP entity rather than the UE device due to the limitation of the UE capability, while for NW-first training for comparison, the gNB can perform the update of the Network model more flexibly.

  - Whether device capability can be considered for model development: It is not necessary to consider UE device capability for model development, since it is up to UE vendor implementation to develop the AI/ML model based on the delivered training dataset.

  - Overhead: UE-first training involves the labels and intermediate results sharing from UE to NW, which needs high resolution of data quantization method to guarantee the dataset accuracy. Hence, the large overhead of sharing dataset is a key issue to be studied.

Based on the above analysis, we propose to prioritize Type 3 NW-first training as a starting point for further study. To our understanding, dataset used for the model training at the other side/entity may have some specification impact, which needs further discussion in this agenda. In addition, from the view of procedure and signaling, the two cases should share as many commonalities as possible. Therefore, we should further study whether outcomes of NW-first training can be also applicable to UE-first training.

Proposal 2: For training Type 3, prioritize NW-first training as a starting point for further study.

Proposal 3: For training Type 3, further study potential specification impact on the dataset used for the model training at the other side/entity.

Based on the above analysis, the pros and cons of aforementioned two training types are summarized in following Table 1.

Table 1. Brief comparison of the training types for two-sided model

| Training types Characteristics | Type 1 |  | Type 3 |  |
| --- | --- | --- | --- | --- |
|  | NW side | UE side | NW-first | UE-first |
| Whether model can be kept proprietary | No | No | Yes | Yes |
| Requirements on privacy-sensitive dataset sharing | No | No | No | No |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | No | Yes | No |
| gNB/device specific optimization – i.e., whether hardware-specific optimization of the model is possible, e.g. compilation for the specific hardware | Restricted (only when the model structure of UE-part model is known by network) | Restricted (only when the model structure of NW-part model is known by network) | Yes | Yes |
| Model update flexibility after deployment | Flexible | Inflexible | Semi-flexible | Inflexible |
| Feasibility of allowing UE side and NW side to develop/update models separately | Infeasible | Infeasible | Feasible | Feasible |
| Model performance based on evaluation in 9.2.2.1 | Optimal | Optimal | Sub-optimal | Sub-optimal |
| Whether gNB can maintain/store a single/unified model | Yes | No | Yes | No |
| Whether UE device can maintain/store a single/unified model | No | Yes | No | Yes |
| Extendability: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Non-extendable (possible only when UE knows NW-part model in use) | Non-extendable (possible only when NW knows UE-part model in use) | Extendable | Extendable |
| Whether training data distribution can be matched to the device that will use the model for inference | No for UE device | Yes for UE device | Possible for UE device only when ground-truth CSI is shared from target UE device. | Possible for UE device only when ground-truth CSI is shared from target gNB device |
| Whether device capability can be considered for model development | Yes (when the supported UE-part model structure is known by network side) | UE-part model is up to UE implementation | UE-part model is up to UE implementation | UE-part model is up to UE implementation |

Based on the brief comparison of different training collaboration types, it is necessary for companies to identify and analyze the pros and cons of different training collaboration types in SI, since a clear understanding of these training collaborations is beneficial for facilitating the subsequent study of two-sided AI/ML model and corresponding specification impact in WI. We propose to identify and analyze the pros and cons of different training collaborations in SI for facilitating the study of two-sided AI/ML model in WI.

Proposal 4: Conclude the pros and cons of different training collaboration types and prioritize training collaboration types in RAN1#112bis-e meeting.

  - Data collection

As is known to all, the performance of AI/ML model is correlated to the training data and inference data, so data collection is a fundamental process in AI model LCM. For data collection, an agreement was reached in Agenda item 9.2.2.2 in RAN1#112 meeting [1].

Agreement: In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least - Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least: - Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection

For network side data collection, UE is required to conduct measurements based on configured reference signals. Then, the measurements may need to be reported to network side. When model training or monitoring is performed at network side, UE needs to measure the configured CSI-RS and then report the ground-truth CSI to the network. Besides, the overhead of the ground-truth label transmitted over the air-interface is a huge concern if the ground-truth CSI is an ideal CSI (e.g., raw channels, eigenvectors). Therefore, one solution is to enhance legacy CSI to increase its reliability, e.g., higher resolution CSI based on legacy Rel-16 eType II codebook design. In addition, as evaluated in our companion contribution [2], the overhead of enhanced Type II CB (i.e., PC10) for one training sample increases by 50% compared with the maximal payload of Rel-16 TypeII CB (i.e., PC8) but keeps similar model performance as ideal CSI, which can be acceptable to be carried on UCI. In this regard, if the ground-truth CSI is reported per sample, the ground-truth CSI can be reported through PHY signaling, e.g., UCI on PUSCH. On the other hand, if the ground-truth CSI is reported per batch, higher layer signaling may be more appropriate, e.g., RRC signaling.

Observation 1: When model training or monitoring is performed at network side, the overhead of the ground-truth label transmitted over the air-interface from UE to network is a huge concern if the ground-truth CSI is an ideal CSI (e.g., raw channels, eigenvectors).

Observation 2: The overhead of enhanced Type II CB (i.e., PC10) for one training sample increases by 50% compared with the maximal payload of Rel-16 TypeII CB (i.e., PC8) but keeps similar model performance as ideal CSI, which can be acceptable to be carried on UCI.

Proposal 5: For network side data collection, support to further study

- Enhanced Rel-16 TypeII codebook to get high-resolution CSI;

- PHY signaling or RRC signaling to report the high-resolution CSI.

For data collection at NW side, the quality of data reported from different UEs may vary greatly. For example, the data collected by UEs at the edge of a cell may suffer from low signal strength and interference from neighbor cell, resulting in a low quality of collected data. In this case, if the low-quality data is used for model training, it of course incurs model performance degradation. Therefore, there are two potential solutions to tackle this problem. On one hand, UE can report associated information with collected data based on some mechanisms, e.g., SINR, CQI, positioning information, and NW judges the data quality and determines whether/how to apply the collected data. On the other hand, NW can configure a threshold of data quality to UE. The UE judges whether the quality of collected data meets the requirement and only reports the qualified data. Therefore, we propose to further study the potential solutions and specification impacts regarding the data quality during data collection at least for:

- UE reports associated information to NW, e.g., SINR, CQI, positioning information

- NW configures a threshold of data quality to UE and UE only reports the qualified data to NW

Proposal 6: To enable high-quality data collection, at least support:

- UE reports associated information to NW, e.g., SINR, CQI, positioning information

- NW configures a threshold of data quality to UE and UE only reports the qualified data to NW

In addition, offline field data collected by one side can also be shared with the other side for model training, model monitoring, and model update. Type 3 training method for two-sided model needs dataset delivery between UE side and network side. For example, gNB-first training requires network to provide a dataset, including labels and intermediate results, to UE side. Then, the UE side will train a CSI generation part based on the delivered dataset. After that, the UE may inform description of the available CSI generation part to network via model/functionality identification process. During model/functionality identification, the UE should disclose which dataset has been used for training the CSI generation part to facilitate network to choose corresponding CSI reconstruction part.

Observation 3: For Type 3 training collaboration of a two-sided model, common understanding on the dataset used for model training is necessary, which can facilitate the pairing of CSI generation part and CSI reconstruction part.

In addition, for a specific model/functionality, aligning the same understanding on the dataset used for model training can be useful for testing/monitoring the model/functionality performance (i.e., the dataset implicitly defines the test vector for the model/functionality). For network, it can provide a dataset (with ground-truth label) to verify the performance of a model/functionality. Moreover, companies have strong concerns to disclose proprietary information. Therefore, during the data collection, the dataset ID can avoid sharing the proprietary information explicitly across vendors (e.g., codebook implementation, antenna patterns and Rx beam information). In this way, the applicable conditions of the model/functionality can be implicitly indicated by the dataset used for model training. As we discussed in our companion contribution [8], a new terminology for dataset identification may need to be defined to have a common understanding between the NW and the UE on a dataset.

Observation 4: Dataset alignment between UE and network can be used for testing/monitoring the model/functionality performance.

Observation 5: Dataset ID can avoid sharing the proprietary information explicitly across vendors during data collection.

Proposal 7: Support to use dataset ID for collected data and for delivered dataset of Type 3 training collaboration.

  - Model inference operation

After model training, model delivery, and model deployment, model inference operation is an important part in LCM. To our understanding, model inference should focus on data required for model input, report feedback based on the model output and the inference latency. The model input may have specification impacts on reference signal configurations and assistance information delivery. For model output, its specification impacts mainly include quantization methods, measurement report format/UCI mapping order, and the priority for contents included in the measurement report. For inference latency, it’s related to UE processing capability on the model, which may have impact on the determination of CSI reference resource and time offset between activation command and physical channel with measurement report included.

Proposal 8: For model inference operation, further study at least the following aspects:

- Data required for model input, e.g., reference signal configurations and assistance information delivery

- Report feedback based on the model output, e.g., UCI mapping order and priority

- Inference latency, e.g., the relationship between inference latency and CSI reference resource

#### 2.1.3.1 CQI determination

Agreement: In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

In RAN1#112 meeting [1], CQI determination options were discussed and finally agreed. According to the above cases for CQI calculation, the pros and cons of each option are analyzed as below:

- For Option 1a, since UE is not aware of the output CSI recovered by the CSI reconstruction model at Network, a straightforward way is that UE adopts the target CSI with realistic channel measurement for CQI calculation. One issue needs to be considered that AI/ML model may not be able to reconstruct a lossless CSI. Therefore, if CQI is calculated based on the target CSI with realistic channel measurement, UE may over-estimate the channel condition and reconstructed PMI and CQI are not matched. In this way, network may always need to make some adjustment on UE reported CQI according to outer loop control. Therefore, our simulation results [2] show that the system performance loss is obvious if no advanced CQI adjustment algorithm is used.

- Considering the situation that Option 1a would over-estimate the CQI, Option 1b is proposed to introduce some adjustment to calculate a more accurate CQI. In the contribution [6], UE can calculate CQI adjustment value based on the input CSI and the previous output of CSI reconstruction model provided from NW. In this case, the method needs to send back the output of CSI reconstruction part from NW side to UE, which will lead to additional latency. However, the channel condition may already change a lot (e.g., interference) so that PMI and CQI mismatch is unavoidable. In addition, the recovered CSI should be quantized (e.g., by eType II codebook), which will lead to additional quantization loss. Moreover, sending the recovered CSI needs enhanced specification to support it. Therefore, it is not appropriate for this option to determine CQI calculation. To our understanding, a simple way is to allow gNB to provide CQI adjustment strategy to UE. For example, NW can construct a CQI adjustment table according to some channel characteristics based on some priori information at gNB side. Then, UE can calculate the similarity-related metrics between measured channel and the channel characteristics to do corresponding CQI adjustment. Therefore, Option 1b may be a feasible way for CQI determination, however, how to design/calculate the potential CQI adjustment needs to be further studied in 9.2.2.1. Therefore, we propose to further categorize the Option 1b:

  - Option 1b-1: CQI is calculated based on target CSI with realistic channel measurement and adjusted by previous CSI reconstruction output provided by NW.

  - Option 1b-2: CQI is calculated based on target CSI with realistic channel measurement and adjusted by CQI adjustment table provided by NW.

- For Option 1c, UE may not support traditional codebook and AI/ML codebook simultaneously, which will largely increase the UE complexity. Meanwhile, PMI and CQI mismatching is also unavoidable. If traditional codebook can already get accurate PMI, it is not necessary to implement AI/ML models.

- For Option 2a, there are two ways of calculating CQI based on the output of CSI reconstruction part. On one hand, UE can calculate CQI based on the output of CSI reconstruction model if actual CSI reconstruction model is available at the UE and UE can perform reconstruction model inference. In this case, UE may also be not expected to have CSI reconstruction model as it increases UE computation/storage/power consumption burden to a large extent. In addition, the CSI reconstruction model is generally a proprietary design by network side. On the other hand, UE can calculate CQI based on the output of CSI reconstruction part assumed at the UE, which is different from the actual CSI reconstruction part used at the NW. For example, if UE has its own reference model/monitoring model and makes the output of reference model/monitoring model similar to the CSI reconstruction model output. Hence, the assumed reconstructed PMI at UE can be used for CQI calculation, which is shown in Figure 1. In our companion contribution [2], the CQI calculation based on the output CSI assumed at UE can be applicable for CSI compression using two-sided model sub use case and shows that the average system UPT can be achieved almost the same as the case of CQI calculation based on the output of actual CSI reconstruction model (i.e., the performance upper-bound for all options). In addition to the feasibility of this method, specification impact should also be taken into consideration. For example, compared with traditional NW-first Type 3 training, an additional dataset needs to be transferred from NW to UE for model training, since UE side has to train an assumed output to imitate the output of actual CSI generation part at NW side. Therefore, we propose to further study potential specification impact on the CQI determination based on the output of CSI reconstruction part assumed at the UE. Besides, we propose to further categorize the Option 1b:

  - Option 2a-1: CQI is calculated based on CSI reconstruction output, where CSI reconstruction part at the UE is the same as the actual CSI reconstruction part used at the NW.

  - Option 2a-2: CQI is calculated based on CSI reconstruction output assumed at UE side, where CSI reconstruction part at the UE is different from the actual CSI reconstruction part used at the NW.

Figure 1 CQI calculation based on the output of CSI reconstruction part assumed at UE side

- Option 2b needs two-step procedure to finish CQI determination. UE has to receive a CSI-RS and report the PMI compressed by AI/ML model in the first step. Then, UE is to receive a precoded CSI-RS based on the reconstructed PMI at network and report the CQI determined by precoded CSI-RS in the second step. In current specification, this mode is already supported (i.e., when the report quantity is cri-RI-CQI) and less specification impact is foreseen. Besides, the two-step procedure increases the time span of the CQI determination process, which may face the channel variation/aging so that the current CQI cannot match the previous CSI. Therefore, we don’t think additional specification is needed.

Based on the above analysis, we propose to evaluate the performance of different CQI determination options in agenda item 9.2.2.1. In principal, Option 2a can be considered as the performance upper-bound for all options. From our perspective, we propose to conclude the feasibility and necessity for different options according to the evaluation results in 9.2.2.1 and specification impacts.

Observation 6: For CQI calculation based on target CSI with realistic channel measurement, UE may over-estimate the channel condition and reconstructed PMI and CQI are not matched. Our simulation results show that the system performance loss is obvious if no advanced CQI adjustment algorithm is used.

Observation 7: For CQI calculation based on target CSI with realistic channel measurement and adjusted by previous CSI reconstruction output provided by NW, this method needs to send back the output of CSI reconstruction part from NW side to UE, which will lead to additional latency. However, the channel condition may already change a lot (e.g., interference) so that PMI and CQI mismatch is unavoidable. In addition, the recovered CSI should be quantized (e.g., by eType II codebook), which will lead to additional quantization loss. Moreover, sending the recovered CSI needs enhanced specification to support it.

Observation 8: For CQI calculation based on target CSI with realistic channel measurement and adjusted by CQI adjustment table provided by NW, NW can construct a CQI adjustment table according to some channel characteristics based on some priori information at gNB side. Then, UE can calculate the similarity-related metrics between measured channel and the channel characteristics to do corresponding CQI adjustment.

Observation 9: For CQI calculation based on legacy codebook, UE my not support traditional codebook and AI/ML codebook simultaneously, which will largely increase the UE complexity. Meanwhile, PMI and CQI mismatching is also unavoidable. If traditional codebook can already get accurate PMI, it is not necessary to implement AI/ML models.

Observation 10: For CQI calculation based on CSI reconstruction output, where CSI reconstruction part at the UE is the same as the actual CSI reconstruction part used at the NW. UE may also be not expected to have CSI reconstruction model as it increases UE computation/storage/power consumption burden to a large extent. In addition, the CSI reconstruction model is generally a proprietary design by network side.

Observation 11: For CQI calculation based on the output of CSI reconstruction model assumed at UE, this method can be applicable for CSI compression using two-sided model sub use case and shows that the average system UPT can be achieved almost the same as the case of CQI calculation based on the output of actual CSI reconstruction model (i.e., performance upper-bound for all options).

Observation 12: For CQI calculation using two stage approach, it is already supported (i.e., when the report quantity is cri-RI-CQI) and less specification impact is foreseen. Besides, the two-step procedure increases the time span of the CQI determination process, which may face the channel variation/aging so that the current CQI cannot match the previous CSI.

Proposal 9: The performance of different CQI determination options should be evaluated in agenda item 9.2.2.1.

Proposal 10: Further categorize the Option 1b as following:

- Option 1b-1: CQI is calculated based on target CSI with realistic channel measurement and adjusted by previous CSI reconstruction output provided by gNB

- Option 1b-2: CQI is calculated based on target CSI with realistic channel measurement and adjusted by CQI adjustment table provided by gNB.

Proposal 11: Further categorize the Option 2a as following:

- Option 2a-1: CQI is calculated based on CSI reconstruction output, where CSI reconstruction part at the UE is the same as the actual CSI reconstruction part used at the NW.

- Option 2a-2: CQI is calculated based on CSI reconstruction output assumed at UE side, where CSI reconstruction part at the UE is different from the actual CSI reconstruction part used at the NW.

Proposal 12: According to initial evaluations on performance and specification impacts, the following down-selections are proposed:

- Further study the specification impacts (including the feasibility and necessity) on Option 1a, Option 1b-2 and Option 2a-2.

- No further discussion on specification impacts for Option1b-1, Option 1c, Option 2a-1 and Option 2b.

Finally, according to the following descriptions in TS 38.214, LI (Layer Indicator) should also be determined according to PMI. Therefore, we propose to consider both LI and CQI.

5.2.1.4 Reporting configurations The UE shall calculate CSI parameters (if reported) assuming the following dependencies between CSI parameters (if reported) - LI shall be calculated conditioned on the reported CQI, PMI, RI and CRI - CQI shall be calculated conditioned on the reported PMI, RI and CRI - PMI shall be calculated conditioned on the reported RI and CRI - RI shall be calculated conditioned on the reported CRI.

Proposal 13: In CSI compression using two-sided model use case, LI determination should be studied along with CQI determination.

#### 2.1.3.2 RI determination

In addition, RI determination options need further discussion as below:

- Proposal 3-3-3: In CSI compression using two-sided model use case, if RI is configured to be reported, legacy RI determination can be reused as a starting point. - Further enhancements are not precluded

As cited above in TS 38.214, RI determination is nothing to do with PMI. For RI determination, UE can reuse the legacy approach to calculate RI.

Proposal 14: In CSI compression using two-sided model use case, if RI is configured to be reported, legacy RI determination can be reused as a starting point.

In addition, as we stated in another contribution [2], precoding can be refined by using approaches such as SLNR and zero-forcing to reduce inter UE interference in the scenario of MU-MIMO. In such case, MCS estimation based on precoding is quite challenging at gNB side. As discussed in MIMO agenda (section 3.7) [5], more channel information (e.g. wideband Rxx including receiver side information, full rank report including eigenvectors and eigenvalues) can increase system performance significantly. Due to sufficient channel information of UEs at gNB side, precoding for MU-MIMO is more accurate and interference between UEs is controlled effectively. To better analyze the problem, we propose to further study potential specification impact on full rank report based on the AI/ML model.

Proposal 15: Further study potential specification impact on more channel information reported for MU-MIMO scheduling, e.g., full rank report based on the AI/ML model.

#### 2.1.3.3 Quantization

In RAN1#110bis-e meeting [4], quantization method alignment between CSI generation part at UE and CSI reconstruction part at gNB was discussed and reached an agreement, which is shown as below:

- Proposal 3.3.6 In CSI compression using two-sided model use case, further study at least the following potential specification impact on quantization alignment including: - For vector quantization scheme, the format and size of the VQ codebook, the distance metric (or quantization rule), the segmentation approach, and configuration of VQ codebook. - For scalar quantization scheme including uniform and non-uniform quantization

In CSI compression using two-sided model use case, it is crucial that the quantization method between CSI generation part at UE and CSI reconstruction part at gNB should be aligned, since the mismatched quantization method would have incurred the huge model performance degradation. In addition, quantization method alignment is closely related to the training collaboration types. For training Type 1, the training entity/side should indicate the quantization method to the other side accompanied with the AI model. For example, for scalar quantization, the quantization type, e.g. uniform/non-uniform quantization method, quantization level, should be configured by NW. On the other hand, for vector quantization, the codebook-related information, e.g., the length of codeword and the size of codebook, should be indicated to the other side for quantization alignment. For training Type 3, one side performing the first step training can share the dataset of before/after quantization/dequantization to the other side to perform the second step training. For example, for gNB-first Type 3 training, if gNB share the dataset before quantization, the CSI generation part trained with the dataset is not aware of the quantization method, and then gNB needs to indicate the quantization method to UE how to quantize the output of CSI generation part, e.g., scalar quantization/vector quantization. Besides, considering the varying channel scenarios, the quantization codebook may need to be updated to match the new channel scenarios. Therefore, we propose to further study the following quantization alignment options:

- For scalar quantization scheme, alignment on quantization method including quantization type, quantization level, quantization pattern, etc.

- For vector quantization scheme, alignment on quantization codebook including the length of codeword, the size of codebook, etc.

- The configuration/reporting/update of the quantization codebook

Proposal 16: In CSI compression using two-sided model use case, further study the following quantization alignment options:

- For scalar quantization scheme, the quantization dictionary should be aligned including quantization type, quantization level, quantization pattern, etc.

- For vector quantization scheme, the quantization codebook should be aligned including the length of codeword, the size of codebook, etc.

- The configuration/reporting/update of the quantization dictionary/codebook.

  - Model monitoring

In the RAN1#112 meeting [1], the following agreement related to monitoring based on intermediate KPIs had been achieved.

Agreement: In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency.

For training Type 1, though CSI generation model and CSI reconstruction model are deployed at UE side and NW side, respectively, they essentially belong to a unified model. So, if model performance loss happens, it should contribute to the whole two-sided model. Thus, AI/ML model monitoring can be performed at either UE side or NW side to monitor the current performance of two-sided model.

Similar to CQI determination, CSI reconstruction model is not available at UE side, hence it’s hard for UE to monitor the model performance based on the actual CSI reconstruction model output. In addition, UE can monitor the performance based on the output of the actual CSI reconstruction model provided from NW to UE. In this case, the method needs to send back the output of CSI reconstruction part from NW side to UE, which will lead to additional latency. However, the channel condition may already change a lot (e.g., interference) so that PMI and CQI mismatch is unavoidable. In addition, the recovered CSI should be quantized (e.g., by eType II codebook), which will lead to additional quantization loss. Moreover, sending the recovered CSI needs enhanced specification to support it. It is also not appropriate for this method to perform UE-side monitoring. Therefore, we propose two methods to monitor the model performance of a two-sided model at UE side and NW side, respectively:

- UE-side monitoring based on the output of the CSI reconstruction model assumed at the UE side: Intermediate KPIs are calculated by UE based on the output of the CSI reconstruction model assumed at the UE side. As shown in Figure 2, UE has its own reference reconstruction part in CSI generation model which is not the same as the actual CSI reconstruction model used at the NW side. Hence, CSI generation model has two output modules. The first one is feedback part, which is used for model input of CSI reconstruction model. The second output (i.e., assumed output) is for model monitoring. As shown in the Figure 2, the assumed output data is trained to imitate reconstruction model output as much as possible via knowledge distillation technology [7]. By doing this, UE can monitor the loss2 to check the situation of loss1. If the monitoring metrics between input and assumed output cannot meet a target requirement, so as the monitoring metrics between input and actual output. In our companion contribution [2], this method shows good performance monitoring accuracy, which can be adopted as a candidate method for monitoring at UE side.

Figure 2 Intermediate KPIs calculated by UE based on the output of the CSI reconstruction model at the UE side

- NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE side: Intermediate KPIs are calculated by NW based on traditional CSI and CSI reconstruction model output. As shown in Figure 3, due to the fact that network cannot directly obtain the ground-truth label to calculate the monitoring metrics, UE should report ground-truth CSI to for network to calculate the monitoring metrics. In order to improve the performance of network-based model monitoring, a higher resolution ground-truth label needs to be reported by UE, where our companion contribution shows the monitoring results based on high resolution legacy CSI [2]. Similar to data collection, overhead to transmit ground-truth CSI is a big concern. Therefore, an enhanced Type II codebook is a promised solution. In our companion contribution [2], this solution is evaluated that it can provide good performance monitoring accuracy and an enhanced Type II codebook can further reduce the overhead of ground-truth CSI, which can be adopted as a candidate solution for monitoring at NW side.

Figure 3 Intermediate KPIs calculated by NW based on traditional CSI and CSI reconstruction model output

Based on the above analysis, we propose to evaluate the performance of different monitoring cases based on intermediate KPIs and discuss the related evaluation KPIs for companies to calibrate the monitoring performance in agenda item 9.2.2.1. Besides, we propose to further study at least the following two cases for model performance monitoring based on intermediate KPIs in prior,

- Case 1: UE-side monitoring based on the output of the CSI reconstruction model at the UE-side, e.g., Intermediate KPIs are calculated by UE based on the output of the CSI reconstruction model assumed at the UE side, which is not the same as the actual CSI reconstruction model used at the NW side.

- Case 2: NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, e.g., Intermediate KPIs are calculated by NW based on traditional CSI and CSI reconstruction model output.

Proposal 17: The performance of different monitoring cases based on intermediate KPIs and the related evaluation KPIs should be discussed for companies to compare the monitoring performance in agenda item 9.2.2.1.

Proposal 18: Further study the specification impacts on least the following two cases for model performance monitoring,

- Case 1: UE-side monitoring based on the output of the CSI reconstruction model assumed at the UE-sid , e.g., Intermediate KPIs are calculated by UE based on the output of the CSI reconstruction model assumed at the UE side, which is not the same as the actual CSI reconstruction model used at the NW side.

- Case 2: NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, e.g., Intermediate KPIs are calculated by NW based on traditional CSI and CSI reconstruction model output.

Proposal 19: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, further study a high-resolution CSI based on traditional codebook as ground-truth label.

For training type 3, CSI generation model and CSI reconstruction model are actually two separate models. Therefore, if the performance of output CSI is degraded, it cannot be decided whether it’s due to the performance loss of CSI generation model or CSI reconstruction model. Therefore, we should study the mechanisms to monitor model performance of the two models separately (i.e., monitoring the performance of CSI generation part and CSI reconstruction part, respectively). To our understanding, monitoring the performance of CSI reconstruction part can be left up to network implementation, and some assistance information from network is necessary to assist the monitoring of the model trained by UE side. For example, network can send some reference dataset to UE. Then, UE has to feedback the model performance based on the reference dataset. The procedures of dataset sharing can be the same as the procedures of dataset exchange for training type 3. The difference is that the data number for model monitoring can be much smaller than the data number for model training.

Observation 13: For training type 3, CSI generation model and CSI reconstruction model are actually two separate models. Therefore, if the performance of output CSI is degraded, it cannot be decided whether it’s due to the performance loss of CSI generation model or CSI reconstruction model.

Proposal 20: Further study the potential mechanisms and specification impacts on monitoring model performance of the CSI generation model and CSI reconstruction model separately.

Apart from the performance monitoring methods based on intermediate KPIs, eventual KPI-based and input/output-based monitoring methods as below are also agreed in the RAN1#110bis-e meeting [4].

- Agreement In CSI compression using two-sided model use case, further study at least the following options for performance monitoring metrics/methods: - Intermediate KPIs as monitoring metrics (e.g., SGCS) - Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK). - Legacy CSI based monitoring: schemes using additional legacy CSI reporting - Other monitoring solutions, at least including the following option: - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection

For performance monitoring based on eventual KPIs, it is hard to identify whether the eventual performance degrades due to the impacts of other factors other than model performance, e.g., varying channel status, varying scheduling/pairing mechanism, etc. In addition, it is not clear about how to remove the impacts of other factors other than the model performance for the Network side monitoring, hence we propose to deprioritize the model performance monitoring based on eventual KPIs.

Proposal 21: Deprioritize the model performance monitoring based on eventual KPIs.

For input-based monitoring, it is a method to detect the input data distribution drift between training dataset and measured data and then determine the AI/ML monitoring performance. In principal, input-based monitoring solutions are applied for judging the applicable scope of AI/ML models. For example, the AI/ML model is trained based on the dataset for indoor scenarios and the AI/ML model parameters can apply to the distribution features of indoor data, while the outdoor data follows another data distribution, which is out of the applicable scope of AI/ML models and performance degradation occurs. To our understanding, we should firstly identify what features of data distribution can be applied and what metrics can be adopted for evaluating the feature of monitored data. Hence, the way to calculate data distribution drift can be considered. For example, NW can summarize a table of AI/ML recovery performance with multiple levels based on different datasets in advance, e.g., corresponding to different areas or channel scenarios. Then, UE can calculate the similarity-related metrics between input CSI and different dataset, e.g., SGCS, NMSE, etc., to determine the input CSI belongs to which areas/scenarios and then judge the current AI/ML model performance level for model monitoring.

For output-based monitoring, it is a method to detect the output data distribution drift between reference dataset and observed data and then determine the AI/ML monitoring performance. In principal, output-based monitoring solutions are applied for judging whether the current AI/ML model is available for performance monitoring. If large data distribution drift occurs between the output data and the reference dataset, the AI/ML model is not available to the current channel scenarios and the model switching/updating may be needed. Therefore, we propose to further study the feasibility of input/output-based monitoring methods in Agenda item 9.2.2.1.

Proposal 22: Further study the feasibility of input/output-based monitoring methods in Agenda item 9.2.2.1.

  - UE capability

Agreement: In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode. Agreement In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least: - The priority rule regarding CSI collision handling and CSI omission - Codebook subset restriction - CSI processing Unit

AI/ML-based CSI feedback enhancement is a novel operation for current NR system, and diverse UEs have different capabilities of AI/ML processing. Therefore, UE capability for supporting AI/ML-based CSI feedback enhancement in diverse LCM should be studied. For example, different UEs may or may not support the capacity for model training/updating/monitoring and different inference latency for AI/ML inference may occur which may result in different CSI report timelines for AI/ML-based CSI feedback. In addition, due to the fact that different UEs have different UE capabilities, how UE deals with the conflict problem between AI/ML-based CSI feedback and legacy CSI feedback needs further study. In CSI compression using two-sided model use case, further study potential specification impact of the following UE capability options:

- Framework for defining and reporting UE capability for model inference.

- Whether and how LCM-related procedures are captured into UE capability.

Proposal 23: In CSI compression using two-sided model use case, further study potential specification impact of the following UE capability options:

- Framework for defining and reporting UE dynamic capability for model inference.

- Whether and how LCM-related procedures are captured into UE capability.

When uplink transmission resources are not enough to feedback all CSI information, the mapping priority and the omission rules should be defined like legacy Type II codebook. For example, UE can use dynamic quantization resolution for AI/ML model output to reduce payload and to fit in the allocated uplink resource. Then, the quantization type/level/pattern needs to be reported to NW. Besides, AI/ML CSI can be divided into multiple groups based on, e.g. layer, subband, port, different priorities. Then, UE can omit the CSI groups with low priority. In addition, CSI reporting can be separated into multiple reports, which needs to establish the association among the multiple reports. Therefore, we propose to further study the methods and potential specification impact on mapping priority and omission rule for AI/ML CSI report,

- Dynamic quantization resolution to reduce payload

- Divide the CSI into multiple groups with different priority and omit the CSI groups with low priority, e.g., according to layer, subband, port

- CSI reporting is separated into multiple reports, e.g., to establish the association among the multiple reports

Proposal 24: In CSI compression using two-sided model use case, further study the methods and potential specification impact on mapping priority and omission rule for AI/ML CSI report,

- Dynamic quantization resolution to reduce payload

- Divide the CSI into multiple groups with different priority and omit the CSI groups with low priority, e.g., according to layer, subband, port

- CSI reporting is separated into multiple reports, e.g., to establish the association among the multiple reports

- Time domain CSI Prediction

Regarding the sub-use case of time domain CSI prediction, some working assumptions were summarized in 9.2.2.1 in RAN1#111 meeting [3] for further study. As shown in the following working assumptions, there are two baselines for the benchmark of performance comparison in time domain CSI prediction, i.e., the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach. Especially for non-AI/ML based CSI prediction approach, it may be difficult to align the assumptions as companies may use different algorithms (e.g., Wiener filter, Kalman filter, etc.). However, it’s important to avoid over-estimating AI-based time domain prediction.

Especially, time domain CSI prediction using UE sided model is selected as a representative sub-use case. However, according to the agreement, potential specification impact discussion on this sub use case is deferred until the RAN1#112b-e.

Working assumption in 9.2.2.1 in RAN1#111: For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation-based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction Agreement Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Note: Continue evaluation discussion in 9.2.2.1. Note: RAN1 Defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer futher till the end of R18 AI/ML SI. Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases.

According to our preliminary simulation results in [2], AI-based CSI prediction can provide good performance gain when baseline is the nearest historical CSI. However, AI-based CSI prediction almost has similar performance when the baseline is non-AI/ML based CSI prediction. This may be due to the simple channel model used for time prediction so that channel variations only caused by Doppler shift but the large-scale and small-scale parameters are almost static in the measurement window and prediction window. Thus, non-AI/ML based CSI prediction approach can already work well in such assumptions. In addition, some other companies’ contributions of simulation results of time domain CSI prediction are only provided with comparison to the nearest historical CSI, which is not convincing since the performance of AI/ML based CSI prediction is overestimated.

Besides, Rel-18 Type II codebook with Doppler domain can be regarded as another legacy scheme of CSI feedback. This scheme is based on time domain CSI prediction at UE side according to current discussion in Rel-18 MIMO work item. Therefore, it would be fair for comparison between AI/ML-based CSI prediction and Rel-18 Type II codebook with Doppler domain, since they both take the CSI prediction into consideration. However, the design of Rel-18 Type II codebook with Doppler domain has not been completed. In addition, some specification impacts on CSI prediction are being discussed in Rel-18 MIMO WI, which may be overlapped with potential specification impact discussion in this agenda item. Additional workload should not be brought in the end of Rel-18 study item. Considering these factors, we propose to deprioritize the sub-use case of time domain CSI prediction in Rel-18 SI.

Observation 14: Based on current evaluation assumptions in 9.2.2.1, AI-based CSI prediction shows better performance gain when baseline is the nearest historical CSI. However, AI-based CSI prediction almost has similar performance when the baseline is non-AI/ML based CSI prediction.

Proposal 25: Deprioritize the specification impact discussion on the sub-use case of time domain CSI prediction in Rel-18 SI.

- Conclusion

In this contribution, we discuss the sub use case for AI/ML-based CSI feedback enhancement and identify some specification impacts. We have the following observations and proposals:

Proposal 1: Prioritize Type 1 joint training at NW side for further study and model transfer/delivery can be further discussed in agenda item 9.2.1.

Proposal 2: For training Type 3, prioritize NW-first training as a starting point for further study.

Proposal 3: For training Type 3, further study potential specification impact on the dataset used for the model training at the other side/entity.

Proposal 4: Conclude the pros and cons of different training collaboration types and prioritize some training collaboration types in RAN1#112bis-e meeting.

Observation 1: When model training or monitoring is performed at network side, the overhead of the ground-truth label transmitted over the air-interface from UE to network is a huge concern if the ground-truth CSI is an ideal CSI (e.g., raw channels, eigenvectors).

Observation 2: The overhead of enhanced Type II CB (i.e., PC10) for one training sample increases by 50% compared with the maximal payload of Rel-16 TypeII CB (i.e., PC8) but keeps similar model performance as ideal CSI, which can be acceptable to be carried on UCI.

Proposal 5: For network side data collection, support to further study

- Enhanced Rel-16 TypeII codebook to get high-resolution CSI;

- PHY signaling or RRC signaling to report the high-resolution CSI.

Proposal 6: To enable high-quality data collection, at least support:

- UE reports associated information to NW, e.g., SINR, CQI, positioning information

- NW configures a threshold of data quality to UE and UE only reports the qualified data to NW

Observation 3: For Type 3 training collaboration of a two-sided model, common understanding on the dataset used for model training is necessary, which can facilitate the pairing of CSI generation part and CSI reconstruction part.

Observation 4: Dataset alignment between UE and network can be used for testing/monitoring the model/functionality performance.

Observation 5: Dataset ID can avoid sharing the proprietary information explicitly across vendors during data collection.

Proposal 7: Support to use dataset ID for collected data and for delivered dataset of Type 3 training collaboration.

Proposal 8: For model inference operation, further study at least the following aspects:

- Data required for model input, e.g., reference signal configurations and assistance information delivery

- Report feedback based on the model output, e.g., UCI mapping order and priority

- Inference latency, e.g., the relationship between inference latency and CSI reference resource

Observation 6: For CQI calculation based on target CSI with realistic channel measurement, UE may over-estimate the channel condition and reconstructed PMI and CQI are not matched. Our simulation results show that the system performance loss is obvious if no advanced CQI adjustment algorithm is used.

Observation 7: For CQI calculation based on target CSI with realistic channel measurement and adjusted by previous CSI reconstruction output provided by NW, this method needs to send back the output of CSI reconstruction part from NW side to UE, which will lead to additional latency. However, the channel condition may already change a lot (e.g., interference) so that PMI and CQI mismatch is unavoidable. In addition, the recovered CSI should be quantized (e.g., by eType II codebook), which will lead to additional quantization loss. Moreover, sending the recovered CSI needs enhanced specification to support it.

Observation 8: For CQI calculation based on target CSI with realistic channel measurement and adjusted by CQI adjustment table provided by NW, NW can construct a CQI adjustment table according to some channel characteristics based on some priori information at gNB side. Then, UE can calculate the similarity-related metrics between measured channel and the channel characteristics to do corresponding CQI adjustment.

Observation 9: For CQI calculation based on legacy codebook, UE my not support traditional codebook and AI/ML codebook simultaneously, which will largely increase the UE complexity. Meanwhile, PMI and CQI mismatching is also unavoidable. If traditional codebook can already get accurate PMI, it is not necessary to implement AI/ML models.

Observation 10: For CQI calculation based on CSI reconstruction output, where CSI reconstruction part at the UE is the same as the actual CSI reconstruction part used at the NW. UE may also be not expected to have CSI reconstruction model as it increases UE computation/storage/power consumption burden to a large extent. In addition, the CSI reconstruction model is generally a proprietary design by network side.

Observation 11: For CQI calculation based on the output of CSI reconstruction model assumed at UE, this method can be applicable for CSI compression using two-sided model sub use case and shows that the average system UPT can be achieved almost the same as the case of CQI calculation based on the output of actual CSI reconstruction model (i.e., performance upper-bound for all options).

Observation 12: For CQI calculation using two stage approach, it is already supported (i.e., when the report quantity is cri-RI-CQI) and less specification impact is foreseen. Besides, the two-step procedure increases the time span of the CQI determination process, which may face the channel variation/aging so that the current CQI cannot match the previous CSI.

Proposal 9: The performance of different CQI determination options should be evaluated in agenda item 9.2.2.1.

Proposal 10: Further categorize the Option 1b as following:

- Option 1b-1: CQI is calculated based on target CSI with realistic channel measurement and adjusted by previous CSI reconstruction output provided by gNB

- Option 1b-2: CQI is calculated based on target CSI with realistic channel measurement and adjusted by CQI adjustment table provided by gNB.

Proposal 11: Further categorize the Option 2a as following:

- Option 2a-1: CQI is calculated based on CSI reconstruction output, where CSI reconstruction part at the UE is the same as the actual CSI reconstruction part used at the NW.

- Option 2a-2: CQI is calculated based on CSI reconstruction output assumed at UE side, where CSI reconstruction part at the UE is different from the actual CSI reconstruction part used at the NW.

Proposal 12: According to initial evaluations on performance and specification impacts, the following down-selections are proposed:

- Further study the specification impacts (including the feasibility and necessity) on Option 1a, Option 1b-2 and Option 2a-2.

- No further discussion on specification impacts for Option1b-1, Option 1c, Option 2a-1 and Option 2b.

Proposal 13: In CSI compression using two-sided model use case, LI determination should be studied along with CQI determination.

Proposal 14: In CSI compression using two-sided model use case, if RI is configured to be reported, legacy RI determination can be reused as a starting point.

Proposal 15: Further study potential specification impact on more channel information reported for MU-MIMO scheduling, e.g., full rank report based on the AI/ML model.

Proposal 16: In CSI compression using two-sided model use case, further study the following quantization alignment options:

- For scalar quantization scheme, the quantization dictionary should be aligned including quantization type, quantization level, quantization pattern, etc.

- For vector quantization scheme, the quantization codebook should be aligned including the length of codeword, the size of codebook, etc.

- The configuration/reporting/update of the quantization dictionary/codebook

Proposal 17: The performance of different monitoring cases based on intermediate KPIs and the related evaluation KPIs should be discussed for companies to compare the monitoring performance in agenda item 9.2.2.1.

Proposal 18: Further study the specification impacts on least the following two cases for model performance monitoring,

- Case 1: UE-side monitoring based on the output of the CSI reconstruction model assumed at the UE-side, e.g., Intermediate KPIs are calculated by UE based on the output of the CSI reconstruction model assumed at the UE side, which is not the same as the actual CSI reconstruction model used at the NW side.

- Case 2: NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, e.g., Intermediate KPIs are calculated by NW based on traditional CSI and CSI reconstruction model output.

Proposal 19: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, further study a high-resolution CSI based on traditional codebook as ground-truth label.

Observation 13: For training type 3, CSI generation model and CSI reconstruction model are actually two separate models. Therefore, if the performance of output CSI is degraded, it cannot be decided whether it’s due to the performance loss of CSI generation model or CSI reconstruction model.

Proposal 20: Further study the potential mechanisms and specification impacts on monitoring model performance of the CSI generation model and CSI reconstruction model separately.

Proposal 21: Deprioritize the model performance monitoring based on eventual KPIs.

Proposal 22: Further study the feasibility of input/output-based monitoring methods in Agenda item 9.2.2.1.

Proposal 23: In CSI compression using two-sided model use case, further study potential specification impact of the following UE capability options:

- Framework for defining and reporting UE dynamic capability for model inference.

- Whether and how LCM-related procedures are captured into UE capability.

Proposal 24: In CSI compression using two-sided model use case, further study the methods and potential specification impact on mapping priority and omission rule for AI/ML CSI report,

- Dynamic quantization resolution to reduce payload

- Divide the CSI into multiple groups with different priority and omit the CSI groups with low priority, e.g., according to layer, subband, port

- CSI reporting is separated into multiple reports, e.g., to establish the association among the multiple reports

Observation 14: Based on current evaluation assumptions in 9.2.2.1, AI-based CSI prediction shows better performance gain when baseline is the nearest historical CSI. However, AI-based CSI prediction almost has similar performance when the baseline is non-AI/ML based CSI prediction.

Proposal 25: Deprioritize the specification impact discussion on the sub-use case of time domain CSI prediction in Rel-18 SI.

- References

- Chairman’s notes, RAN1#112.

- R1-2302437, Evaluation on AI CSI feedback enhancement, ZTE Corporation.

- Chairman’s notes, RAN1#110.

- Chairman’s notes, RAN1#110bis-e.

- Chairman’s notes, RAN1#111.

- R1-2300109, Discussion on AI/ML for CSI feedback enhancement, Huawei, HiSilicon.

- https://en.wikipedia.org/wiki/Knowledge_distillation

- R1-2302436, Discussion on general aspects of common AI PHY framework, ZTE Corporation.