# 3GPP TSG RAN WG1 Meeting #112bis-e  	                                                             R1-2302439

# e-Meeting, April 17th – April 26th, 2023

# Agenda item:	9.2.3.1

# Title:	Evaluation on AI beam management

# Source:	ZTE Corporation

# Document for:	Discussion and Decision

Introduction

The evaluation methodology and KPIs for AI/ML based beam management were discussed in RAN1#112 and many fruitful consensuses have been achieved as follows. Particularly, the quantization error as well as the measured error were agreed to be further evaluated with the legacy granularity of the quantization error being a baseline. The definition of Top-1 genie-aided Tx beam was clarified for KPI calculation and a new metric for L1-RSRP prediction for the Top-1/K predicted beam was defined. Besides, different options for Rx beam for providing input for AI/ML model were initially discussed and no consensus was achieved yet. In this contribution, we will provide our views and preliminary simulation results of the AI/ML based beam prediction.

Agreement: - Further study the impact of quantization error of inputed L1-RSRP (for training and inference) for AI/ML model for beam management. - Existing quantization granularity of L1-RSRP (i.e., 1dB for the best beam, 2dB for the difference to the best beam) is the starting point for evaluation at least for network-sided model. Agreement: - Further study on whether/how to evaluate the performance impact with L1-RSRP measurement accuracy. Agreement: - For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam is defined as - Option A (baseline): the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B(optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - FFS on specific Rx beam(s) - Note: specific Rx beams are subset of all Rx beams - For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair is defined as - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams - Other options are not precluded and can be reported by companies. - Note: This is only for evaluation discussion Agreement: - For AI/ML models, which provide L1-RSRP as the model output, to evaluate the accuracy of predicted L1-RSRP, companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as: - The difference between the predicted L1-RSRP of Top-1[/K] predicted beam and the ideal L1-RSRP of the same beam. Agreement: - For the evaluation of Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), further study the following options as AI/ML model inputs - Alt 2: Implicit information of Tx beam ID and/or Rx beam ID - E.g., measurements of Set B of beams together with default values (e.g. 0) for the beams not in Set B are used as AI inputs in a certain order/ matrix/ vector. - Detailed assumption can be reported by companies. - Alt 3: Tx beam ID and/or Rx beam ID is used as inputs of AI/ML explicitly - Note: Specification impact can be discussed separately. Agreement: - Additionally study the following option on the selection of Set B of beams (pairs) (for Option 2: Set B is variable) - Opt D: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C - Companies report the number of pre-configured patterns used in the evaluation for Option 2: Set B is variable if applicable (e.g. Opt A and Opt B)

Evaluation for spatial domain beam prediction

General description:

The task of beam selection aims at aligning the best beam at both the transmitter and receiver, which is challenging due to mobility and large attenuation in mmWave communication systems. A typical way for beam selection is to exhaustively sweep all beam pairs to find the best beam pair. However, as the number of beams grows, the beam selection overhead and associated power consumption for measurement would be unacceptably high. Therefore, we utilize AI/ML technologies to assist beam selection and explore the spatial domain beam prediction with low overhead as shown in Figure 2-1 below. The measured RSRPs of partial beams (or beam pairs) are used as the AI input and the predicted RSRPs of all beams (or beam pairs) are used as the AI output, where each beam pair includes a transmit beam at the gNB and a receive beam at the UE. Besides, for the case of spatial domain beam prediction, the beam set for measurement (i.e., set B) is a subset of the whole beam set for prediction (i.e., set A).

Figure 2-1. Illustration of AI/ML based spatial domain beam prediction.

Dataset construction:

To evaluate performance of the AI/ML based method for spatial domain beam prediction, we generate the dataset for model training, model validation and model testing according to agreed parameters in RAN1#109e, which is summarized in Table I in the appendix. Specifically, 10 UEs per sector per drop are randomly generated with a total of 500 simulation drops. 80% of UEs are located in indoor and 20% of UEs are located in outdoor as in TR 38.901. The UE speed is fixed to be 3 km/h. For antenna configuration, the baseline agreed in RAN#110bis-e is adopted directly, where a single panel with (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ and 2 panels with (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) are used by the gNB and UE, respectively. Besides, the gNB and UE are equipped with 64 Tx beams in total and 4 Rx beams per panel, respectively.

AI/ML model and training methodology:

For the spatial domain beam prediction, a fully connected network with 6 layers is used with measured RSRPs of partial beams (or beam pairs) being used as the model input and predicted RSRPs of all beams (or beam pairs) being used as the model output. Both the Tx-Rx beam pair prediction and Tx beam prediction are considered for comparison, which apparently corresponds to the P1 initial beam selection procedure and P2 beam refinement procedure, respectively. For ease of understanding, we use a simple notation <Tx Ry> for AI model input in the following, where x denotes the number of Tx beams picked out of 64 Tx beams and y denotes the number of Rx beams picked out of 8 Rx beams. Particularly, if multiple Tx or Rx beams are used for beam measurement, we assume all measured beams are obtained by sampling the whole beam space <T64 R8> with a pre-determined beam pattern. If only one Rx beam is used for beam measurement (i.e., y=1 in the case of Tx beam prediction), four Rx beam assumptions are considered: 1) assumption 1: the best Rx beam for the best Tx beam within Set B; 2) assumption 2: the best Rx beam searched for one Tx beam within Set B; 3) assumption 3:  the best Rx beam for the best Tx beam within Set A (named by the optimal Rx beam) ; 4) assumption 4: a specific Rx beam (e.g., the first Rx beam in the first UE panel by default unless otherwise mentioned) .

At least for evaluation on the performance of DL Tx beam prediction, consider the following four Rx beam assumptions for providing input for AI/ML model for training and/or inference if applicable

Rx beam assumption 1: the best Rx beam for the best Tx beam within Set B

Rx beam assumption 2: the best Rx beam searched for one Tx beam within Set B

Rx beam assumption 3:  the best Rx beam for the best Tx beam within Set A

Rx beam assumption 4: a specific Rx beam (e.g., the first Rx beam in the first UE panel)

The detailed parameters are captured in Table I for spatial domain beam prediction based on the AI/ML model.

Table I. Description of the AI/ML model for spatial domain beam prediction

| Model architecture | Fully connected network |
| --- | --- |
| Number of layers | 6 |
| Model input | Measured RSRPs of partial beams/beam pairs |
| Model output | Predicted RSRPs of all beams/beam pairs |
| Dataset size | 100,000 |
| Percentage of training/ validity/test samples | 90%, 5%, 5% |
| Model input size | Tx-Rx beam pair prediction: 64×8 DL Tx beam prediction: 64×1 |
| Model output size | Tx-Rx beam pair prediction: 64×8 DL Tx beam prediction: 64×1 |
| Model size | Tx-Rx beam pair prediction: 2.888M DL Tx beam prediction: 0.0456M |

Performance KPIs:

To evaluate the performance of AI/ML in spatial domain beam prediction, we provide our preliminary simulation results in terms of KPIs related to beam predicting accuracy, which include average L1-RSRP difference of Top-1 predicted beam, beam prediction accuracy with 1dB margin for Top-1 beam, beam prediction accuracy for Top-1 /Top-K beams, and predicted L1-RSRP difference. Particularly, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams (unless otherwise mentioned). The predicted L1-RSRP difference is L1-RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of Top-1 predicted beam. The beam prediction accuracy (%) for Top-K beams is defined as the percentage that the Top-1 genie-aided beam is one of the Top-K predicted beams. In this case, a refined second stage beam sweeping over these Top-K predicted beams may be needed to obtain an optimal beam. Nevertheless, whether to conduct the refined beam sweeping over Top-K beams is up to NW.

Comparison of beam pair prediction and Tx beam prediction

In this section, both Tx-Rx beam pair prediction and Tx beam prediction are simulated and compared in term of performance KPIs mentioned above. The sampling rate on the whole beam space in both cases is fixed to be 25%, which means that the associated RS overhead reduction would be 75%. Besides, corresponding non-AI cases are also simulated as baseline performance, where the optimal beam is obtained by measuring the uniformly sampled partial beams and directly choosing the one with the largest measured RSRP. For the performance comparison of Tx-Rx beam pair prediction and Tx beam prediction, three different cases are considered as follows.

- Case 1-1-1: In the <T32 R4> case for Tx-Rx beam pair prediction, 32 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by 4 Rx beams picked out of 8 Rx beams by uniform sampling in the UE. In this case, RSRPs of selected beam pairs (32x4=128 beam pairs) are used as AI model inputs and predicted RSRPs of all beam pairs (64x8=512 beam pairs) are used as AI model outputs.

- Case 1-1-2: In the <T16 R8> case for Tx-Rx beam pair prediction, 16 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by all 8 Rx beams in the UE. In this case, RSRPs of selected beam pairs (16x8=128 beam pairs) are used as AI model inputs and predicted RSRPs of all beam pairs (64x8=512 beam pairs) are used as AI model outputs.

- Case 1-1-3: In the <T16 R1> case for Tx beam prediction, 16 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by the optimal Rx beam (i.e., Rx beam assumption 3) with exhaustive beam sweeping for each model input sample, or by a specific Rx beam (i.e., Rx beam assumption 4) for all model input samples. In this case, RSRPs of selected beams (16x1=16 beams) are used as AI model inputs and predicted RSRPs of all Tx beams (64x1=64 beams) are used as AI model outputs.

For the KPI calculation, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) (i.e., Option B). The simulation results are shown in Table II. The spatial domain beam prediction with the specific or optimal Rx beam achieves a sufficiently high performance with only 25% beam overhead being used. Besides, even with a same sampling rate on the whole beam space, the Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction. As can be concluded from the simulation results, the number of Rx beams is relatively small and any sampling at the Rx beam space may cause serve loss of the spatial channel characteristic. Thus, in the beam pair prediction, it’s better to take measurement results of all Rx beams as the model input instead of performing any sampling at the Rx beam space. Other assistance information such as beam shape or beam usage are not evaluated since they are implementation-related information of the gNB or UE, which may be difficult to be disclosed across vendors.

Table II. Performance comparison of Tx-Rx beam pair prediction and Tx beam prediction

| Cases |  | Specific or optimal Rx beam | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1-1-1 | Non-AI | / | 3.099 | 42.80% | 25.95% | 25.95% | 3.099 |
|  | AI | / | 1.171 | 77.60% | 60.35% | 87.15% | 0.756 |
| Case 1-1-2 | Non-AI | / | 2.604 | 43.25% | 27.55% | 27.55% | 2.604 |
|  | AI | / | 0.357 | 91.10% | 75.55% | 96.55% | 0.352 |
| Case 1-1-3 | Non-AI | Specific | 3.013 | 42.00% | 25.45% | 25.45% | 3.391 |
|  |  | Optimal | 3.391 | 40.45% | 25.95% | 25.95% | 3.013 |
|  | AI | Specific | 0.326 | 92.75% | 77.35% | 97.40% | 0.187 |
|  |  | Optimal | 0.096 | 98.10% | 88.25% | 99.75% | 0.335 |

The spatial domain beam prediction with a fixed beam pattern achieves a sufficiently high performance with only 25% beam overhead being used.

With the same sampling rate on the whole beam space, Case 1-1-2 (without sampling at Rx beam space) obtains a better performance than Case 1-1-1 (with sampling at Rx beam space).

In the beam pair prediction, it’s better to take measurement results of all Rx beams as the model input instead of performing any sampling at the Rx beam space.

Rx beam assumption for DL Tx beam prediction

In RAN1#111, for evaluation of the DL Tx beam prediction and providing input for AI/ML model for training and/or inference, an agreement about Rx beam assumptions was reached and further updated in RAN1#112.

Agreement: At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample - Option 2: Measurements of specific Rx beam(s) - Option 2a: Measurements of specific Rx beam(s) per model input sample - Option 2b: Measurements of specific Rx beam(s) for all model input sample - FFS how to select the specific Rx beam(s) - Option 3: Measurements of random Rx beam(s) per model input sample - Other options are not precluded and can be reported by companies. Proposal: At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable - Case 0: the best Rx beam for the best Tx beam within Set B - Case 1: the best Rx beam searched for one Tx beam within Set B - Case 2: the best Rx beam for each Tx beam within Set B - Case 3: the best Rx beam among specific Rx beams for each Tx beam within Set B - Case 4: the best Rx beam among specific Rx beams for the best Tx beam within Set B - Case 5: the best Rx beam for the best Tx beam with in Set A - Note: The best Rx beam may be based on measurements of always-on SSB, or CSI-RS for Set B

As the AI/ML based beam prediction focuses exclusively on data-driven methods, more input information would generate better beam prediction performance. Therefore, if measurement results of Set B associated with different Rx beams are available, it’s better that all information of measurement results including Rx beam ID are fed to the AI/ML model to facilitate better prediction performance, which falls into the field of beam pair prediction. Therefore, in the case of Tx beam prediction, we prefer that only one Rx beam is used for providing input for AI/ML model for training and/or inference. The four Rx beam assumptions in the above proposal 1 are evaluated and compared in this section.

Performance comparison between Rx beam assumptions 1 and 2

To evaluate the performance of DL Tx beam prediction with different Rx beam assumptions, two sub cases of <T16 R8> are considered for comparison. In particular, 16 Tx beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by 8 Rx beam during the beam measurement procedure. In other words, there are 16 Tx beams and 8 Rx beams in Set B.

- Case 1-2-1: Measurements of the best Rx beam for the best Tx beam within Set B (i.e., Rx beam assumption 1).

- Case 1-2-2: Measurements of the best Rx beam searched for one Tx beam within Set B (i.e., Rx beam assumption 2).

Detailed simulation results of Case 1-2-1 and 1-2-2 are provided in the result collection excel [1]. Some of the simulation results are also provided in Table III for reference. Accordingly, a better performance of DL Tx beam prediction can be achieved by measurements of the best Rx beam for the best Tx beam within Set B (i.e., Case 1-2-1). This observation should be straightforward as better beam prediction performance generally introduceS more RS resource overhead for beam sweeping.

Table III. Evaluation results for BM-Case1 without model generalization for DL Tx beam prediction

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-2-1 | Non-AI | 3.113 40.26 % 25.64 % 25.64 % 2.120 |  |  |  |  |
|  | AI | 1.290 84.58 % 74.24 % 92.02 % 0.490 |  |  |  |  |
| Case 1-2-2 | Non-AI | 5.777 29.72% 24.80 % 24.80 % 5.383 |  |  |  |  |
|  | AI | 4.247 62.16 % 62.16% 75.86 % 3.059 |  |  |  |  |

For DL Tx beam prediction, compared with the Rx beam assumption that the best Rx beam searched for one Tx beam within Set B is used, a better beam prediction performance is achieved if the best Rx beam for the best Tx beam within Set B is used for measurement.

Performance comparison between Rx beam assumptions 3 and 4

In this sub section, four sub cases of <T16 R1> are considered for comparison. In particular, 16 beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by one Rx beam that is optimal (i.e., Rx beam assumption 3) or fixed (i.e., Rx beam assumption 4, a specific Rx beam per model input sample or for all model input sample) during the beam measurement procedure.

- Case 1-2-3: Measurements of the “best” Rx beam (i.e., Rx beam assumption 3) with exhaustive beam sweeping for each model input sample.

- Case 1-2-4: Measurements of a specific Rx beam (i.e., Rx beam assumption 4) per model input sample, i.e., the specific Rx beam is changed from the first Rx beam to the fourth Rx beam on the first UE panel as the input sample changes, and followed by recycling.

- Case 1-2-5: Measurements of a specific Rx beam (i.e., Rx beam assumption 4) per model input sample, i.e., the specific Rx beam is changed from the first Rx beam to the eighth Rx beam on the two UE panels as the input sample changes, and followed by recycling.

- Case 1-2-6: Measurements of a specific Rx beam (i.e., Rx beam assumption 4) for all model input samples, i.e., the first Rx beam on the first UE panel is always used.

For the KPI calculation, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) (i.e., Option B). Detailed simulation results of the above 4 cases are provided in the result collection excel [1]. Some of the simulation results are also provided in Table IV for reference. Accordingly, the best performance of DL Tx beam prediction can be achieved by measurements of the optimal Rx beam (i.e., case 1-2-3) among all four cases. Besides, case 1-2-4, case 1-2-5, and case 1-2-6 have similar performance, which is slightly lower than that of case 1-2-3. As can be concluded from the simulation results, the measured RSRPs corresponding to the optimal Rx beam are usually higher than that of other Rx beams, and thus more features can be extracted by the AI model.

Table IV. Simulation results for DL Tx beam prediction with different Rx beam assumptions

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-2-3 | Non-AI | 2.429 40.11 % 24.53 % 24.53 % 2.417 |  |  |  |  |
|  | AI | 0.047 99.14 % 91.41 % 99.85 % 0.138 |  |  |  |  |
| Case 1-2-4 | Non-AI | 2.118 42.32 % 24.86 % 24.86 % 2.114 |  |  |  |  |
|  | AI | 0.161 96.06 % 82.29 % 98.34 % 0.279 |  |  |  |  |
| Case 1-2-5 | Non-AI | 2.086 43.35 % 25.58 % 25.58 % 2.073 |  |  |  |  |
|  | AI | 0.152 96.19 % 82.32 % 98.55 % 0.269 |  |  |  |  |
| Case 1-2-6 | Non-AI | 2.114 42.99 % 24.72 % 24.72 % 2.116 |  |  |  |  |
|  | AI | 0.129 96.50 % 83.07 % 98.72 % 0.268 |  |  |  |  |

For DL Tx beam prediction, the best performance is achieved by measurements of the “best” Rx beam (i.e., Option 1) among all options of Rx beam assumptions. Besides, Option 2a (i.e., measurements of specific Rx beam(s) per model input sample) and Option 2b (i.e., measurements of specific Rx beam(s) for all model input samples) have similar performances, which is slightly lower than that of Option 1.

Performance comparison of different Set B pattern

In RAN1#112, both options agreed in RAN1#110b-e for the selection of Set B of beams (pairs) were kept open for further study. Besides, option D was added as follows to cover the issue when not all the beams that UE measured is reported, which is valid because now the definition of Set B is the input of the AI/ML model, not the set of beams that UE measures. According to the agreement achieved in RAN1#112, more comprehensive Set B design can be studied and evaluated, including joint designing the number of beams in Set B and their beam shape.

Proposal: - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/ report/measurement during training and/or inference), FFS: - Opt A: Set B is changed following a set of pre-configured/pre-known patterns - Opt B: Set B is changed among pre-configured/ pre-known patterns - Opt C: Set B is changed among Set A beams (pairs) - Opt D: Set B is changed among a set of beams (pairs) for measurement - The number of beams(pairs) in Set B can be fixed or variable - Note: BM-Case1 and BM-Case2 may be considered for different option. - Other options are not precluded. Agreement: - Companies report the pattern of Set B. - Further study the performance with different patterns of set B(s) for fixed Set B (Option 1) and different pre-configured/pre-known patterns of Set B(s) (Option 2A and 2B).

In this section, performance comparison with different patterns of Set B is conducted. In addition, in order to evaluate the performance impact of beam prediction when Set B is changed among a set of beams (pairs) for measurement (i.e., Set C), only RSRPs of Set B, which are subset of Set C, are used as AI model inputs. In other words, UE only reports a subset of Set C (e.g., Top-4, Top-8, or Top-16 of the RSRPs of Set C), which are input to the NW-side model.

Performance comparison with different pattern of set B(s) for fixed Set B

In order to evaluate the performance impact resulted from different Set B pattern, six different Set B patterns are designed as showed in Figure 2-2. The sampling rate of Set B pattern 1/2/3 is 25%, which is two times of that of Set B pattern 4/5/6. Beam pairs prediction and DL Tx beam prediction are conducted by considering the following 12 different simulation cases.

- Case 1-3-1: <T16 R8> case for Tx-Rx beam pair prediction, where 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 are transmitted by the gNB and measured by all 8 receive beams in the UE.

- Case 1-3-2: <T16 R8> case for Tx-Rx beam pair prediction, where 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 2 are transmitted by the gNB and measured by all 8 receive beams in the UE.

- Case 1-3-3: <T16 R8> case for Tx-Rx beam pair prediction, where 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 3 are transmitted by the gNB and measured by all 8 receive beams in the UE.

- Case 1-3-4: <T8 R8> case for Tx-Rx beam pair prediction, where 8 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 4 are transmitted by the gNB and measured by all 8 receive beams in the UE.

- Case 1-3-5: <T8 R8> case for Tx-Rx beam pair prediction, where 8 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 5 are transmitted by the gNB and measured by all 8 receive beams in the UE.

- Case 1-3-6: <T8 R8> case for Tx-Rx beam pair prediction, where 8 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 6 are transmitted by the gNB and measured by all 8 receive beams in the UE.

- Case 1-3-7: <T16 R1> case for DL Tx beam prediction, where 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 are transmitted by the gNB and measured by the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-8: <T16 R1> case for DL Tx beam prediction, where 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 2 are transmitted by the gNB and measured by the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-9: <T16 R1> case for DL Tx beam prediction, where 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 3 are transmitted by the gNB and measured by the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-10: <T8 R1> case for DL Tx beam prediction, where 8 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 4 are transmitted by the gNB and measured by the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-11: <T8 R1> case for DL Tx beam prediction, where 8 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 5 are transmitted by the gNB and measured by the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-12: <T8 R1> case for DL Tx beam prediction, where 8 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 6 are transmitted by the gNB and measured by the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

Figure 2-2. Illustration of different Set B patterns.

Detailed simulation results of the above 12 cases are provided in the result collection excel [1]. Some of the simulation results are also provided in Table V for reference. As observed, for Tx-Rx beam pair prediction and DL Tx beam prediction, similar beam prediction performance can be achieved with different Set B patterns in a same sampling rate. In addition, a sufficient high performance of DL Tx beam prediction can also be achieved with Set B patterns 4/5/6 in a sampling rate of 12.5%, where the beam prediction accuracy for Top-5 beams is above 97%.

Table V. Simulation results for different Set B patterns

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-3-1 | Non-AI | 2.028 44.04 % 25.43 % 25.43 % 2.044 |  |  |  |  |
|  | AI | 0.105 97.09 % 83.83 % 98.72 % 0.261 |  |  |  |  |
| Case 1-3-2 | Non-AI | 0.937 63.43 % 39.09 % 39.09 % 0.970 |  |  |  |  |
|  | AI | 0.131 96.88 % 82.15 % 98.67 % 0.249 |  |  |  |  |
| Case 1-3-3 | Non-AI | 1.737 44.36 % 25.47 % 25.47 % 1.745 |  |  |  |  |
|  | AI | 0.087 97.77 % 85.66 % 99.10 % 0.225 |  |  |  |  |
| Case 1-3-4 | Non-AI | 2.924 32.21 % 18.55 % 18.55 % 2.926 |  |  |  |  |
|  | AI | 0.764 81.77 % 61.49 % 91.45 % 0.700 |  |  |  |  |
| Case 1-3-5 | Non-AI | 2.781 33.60 % 19.62 % 19.62 % 2.770 |  |  |  |  |
|  | AI | 0.795 80.48 % 59.68 % 90.76 % 0.707 |  |  |  |  |
| Case 1-3-6 | Non-AI | 5.384 14.55 % 6.42 % 6.42 % 5.409 |  |  |  |  |
|  | AI | 0.825 79.75 % 60.67 % 90.90 % 0.725 |  |  |  |  |
| Case 1-3-7 | Non-AI | 2.350 41.24 % 25.96 % 25.96 % 2.364 |  |  |  |  |
|  | AI | 0.057 98.93 % 91.66 % 99.81 % 0.149 |  |  |  |  |
| Case 1-3-8 | Non-AI | 0.996 61.70 % 39.96 % 39.96 % 1.014 |  |  |  |  |
|  | AI | 0.071 98.59 % 86.78 % 99.64 % 0.161 |  |  |  |  |
| Case 1-3-9 | Non-AI | 2.052 40.55 % 24.82 % 24.82 % 2.049 |  |  |  |  |
|  | AI | 0.057 98.61 % 89.70 % 99.73 % 0.160 |  |  |  |  |
| Case 1-3-10 | Non-AI | 3.444 29.75 % 19.20 % 19.20 % 3.456 |  |  |  |  |
|  | AI | 0.511 89.90 % 73.79 % 97.47 % 0.411 |  |  |  |  |
| Case 1-3-11 | Non-AI | 3.199 31.56 % 19.81 % 19.81 % 3.169 |  |  |  |  |
|  | AI | 0.492 90.11 % 74.08 % 97.58 % 0.420 |  |  |  |  |
| Case 1-3-12 | Non-AI | 5.988 11.75 % 6.51 % 6.51 % 6.022 |  |  |  |  |
|  | AI | 0.496 87.70 % 72.29 % 98.17 % 0.500 |  |  |  |  |

For Tx-Rx beam pair prediction and DL Tx beam prediction, similar beam prediction performance is achieved with different uniform Set B patterns in the same sampling rate.

For DL Tx beam prediction, a sufficient high performance is achieved with Set B patterns 4/5/6 in a sampling rate of 12.5%, where the beam prediction accuracy for Top-5 beams is above 97%.

Set B is changed among a set of beams (pairs) for measurement (Set C)

To reduce the overhead of UE feeding back beam measurements, evaluation of Set B changed among a set of  beam for measurement (i.e., Set C) is considered in this section. For instance, as shown in Figure 2-3, Set C is used for measurement and Set B is used for model input, which consists of Top-4 beams with highest L1-RSRPs optimally selected from Set C of beams.

To evaluate the performance of Set B changed among Set C, 6 six cases are considered as follows.

- Case 1-3-1: Tx-Rx beam pair prediction, Top 4 L1-RSRPs from 128 beam pair measurements are used as AI model inputs, where the 128 beam pair measurements are results of measurements of 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 and all 8 Rx beams in the UE.

- Case 1-3-2: Tx-Rx beam pair prediction, Top 8 L1-RSRPs from 128 beam pair measurements are used as AI model inputs, where the 128 beam pair measurements are results of measurements of 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 and all 8 Rx beams in the UE.

- Case 1-3-3: Tx-Rx beam pair prediction, Top 16 L1-RSRPs from 128 beam pair measurements are used as AI model inputs, where the 128 beam pair measurements are results of measurements of 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 and all 8 Rx beams in the UE.

- Case 1-3-4: DL Tx beam prediction, Top 4 L1-RSRPs from 16 Tx beam measurements are used as AI model inputs, where the 16 beam measurements are results of measurements of 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 and the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-5: DL Tx beam prediction, Top 8 L1-RSRPs from 16 Tx beam measurements are used as AI model inputs, where the 16 beam measurements are results of measurements of 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 and the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

- Case 1-3-6: DL Tx beam prediction, Top 16 L1-RSRPs from 16 Tx beam measurements are used as AI model inputs, where the 16 beam measurements are results of measurements of 16 fixed beams picked out of 64 Tx beams by uniform sampling with Set B pattern 1 and the optimal Rx beam in the UE (i.e., Rx beam assumption 3).

Figure 2-3. Illustration of selection from Set C pattern to Set B pattern.

Detailed simulation results of the above cases are provided in the result collection excel [1]. Some of the simulation results are also provided in Table VI for reference. For DL Tx beam prediction, marginal performance loss is observed when the AI model inputs are Top 8 L1-RSRPs or Top 4 L1-RSRPs of all 16 Tx beam measurements. For Tx-Rx beam pair prediction, significant performance loss is observed when the AI model inputs are Top 8 L1-RSRPs or Top 4 L1-RSRPs of all 128 beam pair measurements. As can be concluded from the simulation results, to reduce the overhead of UE feeding back beam measurement for DL Tx beam prediction, it is better to only report partial  beams from a set of beams for measurement for a NW-side model.

Table VI. Simulation results for different Top N Set B patterns

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-3-1 | Non-AI | 2.064 42.51 % 24.84 % 24.84 % 2.045 |  |  |  |  |
|  | AI | 0.856 79.85 % 58.90 % 87.77 % 0.682 |  |  |  |  |
| Case 1-3-2 | Non-AI | 1.940 43.66 % 24.70 % 24.70 % 1.965 |  |  |  |  |
|  | AI | 0.496 86.13 % 66.29 % 92.21 % 0.554 |  |  |  |  |
| Case 1-3-3 | Non-AI | 2.062 42.76 % 25.05 % 25.05 % 2.077 |  |  |  |  |
|  | AI | 0.223 93.47 % 76.38 % 96.65 % 0.358 |  |  |  |  |
| Case 1-3-4 | Non-AI | 2.346 40.69 % 24.55 % 25.28 % 2.353 |  |  |  |  |
|  | AI | 0.157 96.15 % 83.18 % 99.03 % 0.228 |  |  |  |  |
| Case 1-3-5 | Non-AI | 2.407 40.34 % 25.28 % 25.28 % 2.423 |  |  |  |  |
|  | AI | 0.072 98.48 % 90.61 % 99.56 % 0.157 |  |  |  |  |
| Case 1-3-6 | Non-AI | 2.381 40.29 % 24.29 % 24.29 % 2.370 |  |  |  |  |
|  | AI | 0.044 99.18 % 91.52 % 99.79 % 0.138 |  |  |  |  |

For DL Tx beam prediction, marginal performance loss is observed when the AI model inputs are Top 8 beams or Top 4 beams of all 16 beam measurements.

For Tx-Rx beam pair prediction, significant performance loss is observed when the AI model inputs are Top 8 beams or Top 4 beams of all 128 beam pair measurements.

To reduce the overhead of UE feeding back beam measurements for DL Tx beam prediction in BM-Case 1, it is better to only report partial beams from a set of beams for measurement for a NW-side model.

Evaluation for model generalization

In RAN1#112, the following working assumption about generalization evaluation was confirmed with fine tuning being optionally considered.

Agreement: The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two - FFS the detailed set of scenarios/configurations - The following case for generalization verification, can be optionally considered by companies: - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance - FFS: Investigate of the feasibility the fine-tuning on the UE/Network side

Besides, a detailed agreement was reached with more examples being provided for generalization performance verification.

Agreement: For generalization performance verification, consider the following - Scenarios - Various deployment scenarios, - e.g., UMa, UMi and others, - e.g., 200m ISD or 500m ISD and others - e.g., same deployment, different cells with different configuration/assumption - e.g., gNB height and UE height - FFS: e.g., Carrier frequencies - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others - Various UE mobility, - e.g., 3km/h, 30km/h, 60km/h and others - Configurations (parameters and settings) - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions) - Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions) - Various Set B of beam (pairs) - T1 for measurement /T2 for prediction for BM-Case2 - Other scenarios/configurations(parameters and settings) are not precluded and can be reported by companies.

In this section, to evaluate the model generalization capability, different number of beams in Set B, different Set B patterns, different Rx beam assumptions, different gNB settings and different number of UE Rx beams are considered as follows.

Generalization on different number of beams in Set B

To evaluate the model generalization capability on different number of beams in Set B, mixed data samples generated under <T8 R1>, <T16 R1>, and <T32 R1> cases are used as AI model inputs to predict quality of all transmit beams. Besides, beam prediction performance of corresponding <T8 R1>, <T16 R1>, and <T32 R1> cases are also provided here for comparison.

- Case 1-4-1-1: Both training and testing with samples of mixed data samples, where the mixed data samples generated under <T8 R1>, <T16 R1>, and <T32 R1> cases are used as AI model inputs to predict RSRPs of all Tx beams. The Rx beam is optimal (i.e., Rx beam assumption 3) during the beam measurement procedure as above.

- Case 1-4-1-2: In the <T8 R1> case for Tx beam prediction, 8 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by one Rx beam that is optimal (i.e., Rx beam assumption 3) during the beam measurement procedure. In this case, RSRPs of selected Tx beams (8x1=8 beams) are used as AI model inputs and predicted RSRPs of all Tx beams (64x1=64 beams) are used as AI model outputs.

- Case 1-4-1-3: In the <T16 R1> case for Tx beam prediction, 16 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by one Rx beam that is optimal (i.e., Rx beam assumption 3) during the beam measurement procedure. In this case, RSRPs of selected beams (16x1=16 beams) are used as AI model inputs and predicted RSRPs of all Tx beams (64x1=64 beams) are used as AI model outputs.

- Case 1-4-1-4: In the <T32 R1> case for Tx beam prediction, 32 fixed beams picked out of 64 Tx beams by uniform sampling are transmitted by the gNB and measured by one Rx beam that is optimal (i.e., Rx beam assumption 3) during the beam measurement procedure. In this case, RSRPs of selected beams (32x1=32 beams) are used as AI model inputs and predicted RSRPs of all Tx beams (64x1=64 beams) are used as AI model outputs.

Detailed simulation results of the above four cases are provided in the result collection excel [2]. Some of the simulation results are also provided in Table VII for reference. It can be observed that for the case of generalization on different number of beams in Set B, AI can achieve a better performance than that of the case of <T8 R1>, but is outperformed by the case of <T32 R1>. Additionally, the case of generalization AI can obtain significant performance gain over the non-AI case. The AI based method with the optimal Rx beam can always achieve a slight better performance than that of the AI based method with the specific Rx beam.

Table VII. Simulation results for generalization performance evaluation on different number of beams in set B

| Cases |  | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams |
| --- | --- | --- | --- | --- | --- |
| Case 1-4-1-1 | Non-AI | 3.405 | 44.17% | 30.03% | 30.03% |
|  | AI | 0.314 | 94.733 | 84.867 | 98.92% |
| Case 1-4-1-2 | Non-AI | 6.9198 | 19.80% | 12.60% | 12.60% |
|  | AI | 0.832 | 86.55% | 72.60% | 96.85% |
| Case 1-4-1-3 | Non-AI | 3.391 | 40.45% | 25.95% | 25.95% |
|  | AI | 0.096 | 98.10% | 88.25% | 99.75% |
| Case 1-4-1-4 | Non-AI | 0.627 | 72.25% | 51.55% | 51.55% |
|  | AI | 0.041 | 99.40% | 92.45% | 100.00% |

The case that model training and testing are performed in mixed datasets of <T8 R1>, <T16 R1>, and <T32 R1> with different beam constructions achieves a better performance than that of the case of <T8 R1>, but is outperformed by the case of <T32 R1>.

The AI based Tx beam prediction with the optimal Rx beam achieves slightly better performance than that of the AI based method with a specific (e.g, first) Rx beam.

The AI model can provide good generalization capability on different number of beams in Set B for Tx beam prediction.

Generalization on different Set B patterns

In order to evaluate the AI model’s generalization capability on different Set B patterns, Set B pattern 1, Set B pattern 2, and Set B pattern 3 showed in Figure 2-2 are used to generate data samples. DL Tx beam prediction based on the optimal Rx beam (i.e., Rx beam assumption 3) are conducted by the following four different variations of <T16 R1>.

- Case 1-4-2-1: The training data samples of the AI model consists of mixed data samples, which are generated under Set B pattern 1, Set B pattern 2, and Set B pattern 3. The testing data samples of the AI model are also the mixed data samples.

- Case 1-4-2-2: The training data samples of the AI model consists of mixed data samples, which are generated under Set B pattern 1, Set B pattern 2 and Set B pattern 3. The testing data samples of the AI model are generated under Set B pattern 1.

- Case 1-4-2-3: The training data samples of the AI model are generated under Set B pattern 1. The testing data samples of the AI model are generated under Set B pattern 2.

- Case 1-4-2-4: The training data samples of the AI model are generated under Set B pattern 1. the testing data samples of the AI model are generated under Set B pattern 1.

Detailed simulation results of the above four cases are provided in the result collection excel [2]. Some of the simulation results are also provided in Table VIII for reference. Accordingly, if the Set B pattern of the testing data samples is different from that of the training data samples, a significant performance loss compared with the non-AI case is observed. However, if the Set B pattern of the testing data samples is included in the beam patterns of the training data samples, or the same Set B pattern is used during the AI model’s training phase and testing phase, the AI method can obtain significant performance gains over the non-AI case.

Table VIII. Simulation results for generalization performance evaluation on different set B patterns

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-4-2-1 | Non-AI | 1.826 46.02 % 28.70 % 28.70 % 1.831 |  |  |  |  |
|  | AI | 0.101 97.75 % 86.74 % 99.56 % 0.222 |  |  |  |  |
| Case 1-4-2-2 | Non-AI | 2.396 39.39 % 24.93 % 24.93 % 2.450 |  |  |  |  |
|  | AI | 0.073 98.29 % 88.10 % 99.71 % 0.225 |  |  |  |  |
| Case 1-4-2-3 | Non-AI | 1.068 58.97 % 38.21 % 38.21 % 10.342 |  |  |  |  |
|  | AI | 11.818 13.33 % 8.70 % 29.52 % 5.322 |  |  |  |  |
| Case 1-4-2-4 | Non-AI | 2.351 40.80 % 24.95 % 24.95 % 2.361 |  |  |  |  |
|  | AI | 0.044 99.12 % 91.33 % 99.79 % 0.144 |  |  |  |  |

For the model generalization on different Set B patterns, if the Set B pattern of the testing data samples is different from that of the training data samples, a significant performance loss compared with the non-AI case is observed.

For the model generalization on different Set B patterns, if the Set B pattern of the testing data samples is included in the beam patterns of the training data samples, or the same Set B pattern is used during the model training phase and testing phase, the AI method obtains significant performance gains over the non-AI case.

The AI model can provide good generalization capability on different Set B patterns for Tx beam prediction.

Generalization on different Rx beam assumptions

To evaluate the AI model’s generalization capability on different Rx beam assumptions, Set B pattern 1 showed in Figure 2-2 is used to generate data samples. With different Rx beam assumptions during AI model’s training phase and testing phase, the DL Tx beam prediction is conducted with the following 8 different variations of <T16 R1> being considered for comparison.

- Case 1-4-3-1: The training data samples of the AI model are mixed data samples, which are generated based on the optimal Rx beam (i.e., Rx beam assumption 3) for each model sample or a fixed Rx beam (i.e., Rx beam assumption 4) for all model samples. The testing data samples of the AI model are also the mixed data samples as mentioned above.

- Case 1-4-3-2: The training data samples of the AI model are the mixed data samples, which are generated based on the optimal Rx beam (i.e., Rx beam assumption 3) for each model sample or a fixed Rx beam (i.e., Rx beam assumption 4) for all model samples. The testing data samples of the AI model are generated based on the optimal Rx beam for each model sample.

- Case 1-4-3-3: The training data samples of the AI model are generated based on the optimal Rx beam (i.e., Rx beam assumption 3) for each model sample. The testing data samples of the AI model are generated based on the first Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all model samples.

- Case 1-4-3-4: The training data samples of the AI model are generated based on optimal Rx beam (i.e., Rx beam assumption 3) for each model sample. The testing data samples of the AI model are generated based on optimal Rx beam (i.e., Rx beam assumption 3) for each model sample.

- Case 1-4-3-5: The training data samples of the AI model are generated based on the first Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all model samples. The testing data samples of the AI model are generated based on the first Rx beam on the second UE panel (i.e., Rx beam assumption 4) for all model samples.

- Case 1-4-3-6: The training data samples of the AI model are generated based on the first Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all model samples. The testing data samples of the AI model are generated based on the fourth Rx beam on the second UE panel (i.e., Rx beam assumption 4) for all model samples.

- Case 1-4-3-7: The training data samples of the AI model are generated based on the first Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all model samples. The testing data samples of the AI model are generated based on the second Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all model samples.

- Case 1-4-3-8: The training data samples of the AI model are generated based on the first Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all model samples. The testing data samples of the AI model are generated based on the fourth Rx beam on the first UE panel (i.e., Rx beam assumption 4) for all model samples.

For the KPI calculation, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) (i.e., Option B). Detailed simulation results of the first four cases above are provided in the result collection excel [2]. Some of the simulation results are also provided in Table IX for reference. Accordingly, compared with the case that the training data samples and testing data samples are generated based on the same Rx beam assumption, only marginal performance loss is observed for the case of different Rx beam assumptions. Therefore, we conclude that the AI model can provide good generalization capability on different Rx beam assumptions.

Table IX.  Simulation results for generalization performance evaluation on different Rx beam assumptions

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-4-3-1 | Non-AI | 2.183 41.43 % 25.05 % 25.05 % 2.181 |  |  |  |  |
|  | AI | 0.107 97.33 % 86.50 % 99.26 % 0.203 |  |  |  |  |
| Case 1-4-3-2 | Non-AI | 2.422 39.92 % 24.91 % 24.91 % 2.435 |  |  |  |  |
|  | AI | 0.054 98.80 % 90.46 % 99.83 % 0.170 |  |  |  |  |
| Case 1-4-3-3 | Non-AI | 2.138 42.74 % 24.11 % 24.11 % 2.161 |  |  |  |  |
|  | AI | 0.230 94.46 % 80.91 % 97.50 % 0.276 |  |  |  |  |
| Case 1-4-3-4 | Non-AI | 2.395 40.46 % 24.90 % 24.90 % 2.383 |  |  |  |  |
|  | AI | 0.053 98.90 % 90.88 % 99.68 % 0.145 |  |  |  |  |
| Case 1-4-3-5 | Non-AI | 2.069 44.27 % 25.89 % 25.89 % 2.073 |  |  |  |  |
|  | AI | 0.133 96.32 % 83.05 % 98.72 % 0.245 |  |  |  |  |
| Case 1-4-3-6 | Non-AI | 2.084 43.58 % 25.01 % 25.01 % 2.080 |  |  |  |  |
|  | AI | 0.123 97.01 % 83.10 % 98.86 % 0.259 |  |  |  |  |
| Case 1-4-3-7 | Non-AI | 2.081 44.32 % 25.50 % 25.50 % 2.087 |  |  |  |  |
|  | AI | 0.179 95.50 % 80.53 % 98.36 % 0.282 |  |  |  |  |
| Case 1-4-3-8 | Non-AI | 2.089 43.31 % 25.20 % 25.20 % 2.088 |  |  |  |  |
|  | AI | 0.150 96.32 % 82.88 % 98.51 % 0.269 |  |  |  |  |

For model generalization on different Rx beam assumptions for Tx beam prediction, compared with the case that the training data samples and testing data samples are generated based on the same Rx beam assumption, only marginal performance loss is observed for the case of different Rx beam assumptions on model training and model testing.

The AI model can provide good generalization capability on different Rx beam assumptions for Tx beam prediction.

Generalization on different gNB settings

To evaluate the AI model’s generalization capability on different gNB settings, two different gNB beam structures are used to generate data samples. For the first gNB beam structure, there are 16 beams in horizontal and 4 beams in vertical, denoted as H16V4 for simplicity. For the second gNB beam structure, there are 8 beams in horizontal and 8 beams in vertical, denoted as H8V8 for simplicity. With different gNB beam structures during the model training phase and testing phase, the DL Tx beam prediction is conducted and the following 4 variations of <T16 R1> design with the optimal Rx beam (i.e., Rx beam assumption 3) are considered.

- Case 1-4-4-1: The training data samples of the AI model are mixed data samples, which are generated based on the two different gNB beam structures of H16V4 and H8V8. The testing data samples of the AI model are also the mixed data samples as mentioned above.

- Case 1-4-4-2: The training data samples of the AI model are mixed data samples, which are generated based on the two different gNB beam structures of H16V4 and H8V8. The testing data samples of the AI model are generated based on the gNB beam structure of H16V4.

- Case 1-4-4-3: The training data samples of the AI model are generated based on the gNB beam structures of H16V4. The testing data samples of the AI model are generated based on the gNB beam structure of H8V8.

- Case 1-4-4-4: The training data samples of the AI model are generated based on the gNB beam structures of H16V4. The testing data samples of the AI model are generated based on the gNB beam structure of H16V4.

Detailed simulation results of the above four cases are provided in the result collection excel [2]. Some of the simulation results are also provided in Table X for reference. Accordingly, a significant performance loss is observed if the training data samples and testing data samples are generated under different gNB beam structures. Compared with the case that the same gNB beam structure is used for the model training and testing, only marginal performance loss is observed if the training data samples of the AI model are mixed data samples.

Table X.  Simulation results for generalization performance evaluation on different gNB setting

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-4-4-1 | Non-AI | 1.994 43.09 % 24.80 % 24.80 % 2.000 |  |  |  |  |
|  | AI | 0.086 97.73 % 87.31 % 99.52 % 0.230 |  |  |  |  |
| Case 1-4-4-2 | Non-AI | 2.380 40.91 % 24.82 % 24.82 % 2.369 |  |  |  |  |
|  | AI | 0.102 97.43 % 86.30 % 99.37 % 0.261 |  |  |  |  |
| Case 1-4-4-3 | Non-AI | 1.567 47.52 % 24.90 % 24.90 % 3.528 |  |  |  |  |
|  | AI | 11.693 34.10 % 24.74 % 72.93 % 1.819 |  |  |  |  |
| Case 1-4-4-4 | Non-AI | 2.390 39.41 % 24.13 % 24.13 % 2.394 |  |  |  |  |
|  | AI | 0.058 98.78 % 90.29 % 99.60 % 0.153 |  |  |  |  |

For model generalization on different gNB settings, a significant performance loss is observed if the training data samples and the testing data samples are generated under different gNB beam structures.

Compared with the case that the same gNB beam structure is used for generating the training data samples and the testing data samples, only marginal performance loss is observed if the training data samples of AI model are mixed data samples.

Generalization on different number of UE Rx beams

To evaluate the AI model's generalization capability on different number of UE Rx beams, two UE antenna configurations with (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) and (M, N, P, Mg, Ng) = (1, 2, 2, 1, 2) are used to generate data samples corresponding to 8 Rx beams and 4 Rx beams, respectively. With different number of UE Rx beams during the model training phase and testing phase, the beam pair prediction is conducted with the following 5 different variations of <T16 R8> or <T16 R4> being evaluated for comparison.

- Case 1-4-5-1: The training data samples of the AI model are mixed data samples, which are generated based on 8 Rx beams and 4 Rx beams. The testing data samples of the AI model are also the mixed data samples as mentioned above.

- Case 1-4-5-2: The training data samples of the AI model are mixed data samples, which are generated based on 8 Rx beams and 4 Rx beams. The testing data samples of the AI model are generated based on 8 Rx beams.

- Case 1-4-5-3: The training data samples of the AI model are generated based on 8 Rx beams. The testing data samples of the AI model are generated based on 4 Rx beams.

- Case 1-4-5-4: The training data samples of the AI model are generated based on 8 Rx beams. The testing data samples of the AI model are generated based on 8 Rx beams.

- Case 1-4-5-5: The training data samples of the AI model are generated based on 4 Rx beams. The testing data samples of the AI model are generated based on 4 Rx beams.

Detailed simulation results of the above four cases are provided in the result collection excel [3]. Some of the simulation results are also provided in Table XI for reference. Accordingly, compared with the case that the training data samples and testing data samples are generated based on the same number of UE Rx beams, only marginal performance loss is observed for the case that different number of UE Rx beams are used for model training and testing. Therefore, we can conclude that the AI model can provide good generalization capability on different number of UE Rx beams for beam pair prediction.

Table XI.  Simulation results for generalization performance evaluation on different number of UE Rx beams

| Cases | Non-AI/AI | Average L1-RSRP difference of Top-1 predicted beam | Beam prediction accuracy with 1dB margin for Top-1 beam | Beam prediction accuracy for Top-1 beam | Beam prediction accuracy for Top-5 beams | Predicted L1-RSRP difference |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1-4-5-1 | Non-AI | 2.018 43.14 % 24.42 % 24.42 % 2.025 |  |  |  |  |
|  | AI | 0.097 97.43 % 64.06 % 97.52 % 0.247 |  |  |  |  |
| Case 1-4-5-2 | Non-AI | 2.021 42.93 % 24.84 % 24.84 % 2.036 |  |  |  |  |
|  | AI | 0.121 96.70 % 82.48 % 98.63 % 0.304 |  |  |  |  |
| Case 1-4-5-3 | Non-AI | 2.060 43.33 % 25.05 % 25.05 % 2.129 |  |  |  |  |
|  | AI | 0.126 94.44 % 44.72 % 95.01 % 0.289 |  |  |  |  |
| Case 1-4-5-4 | Non-AI | 2.043 43.05 % 24.36 % 24.36 % 2.058 |  |  |  |  |
|  | AI | 0.094 97.49 % 84.86 % 98.86 % 0.242 |  |  |  |  |
| Case 1-4-5-5 | Non-AI | 1.974 43.75 % 24.63 % 24.63 % 1.998 |  |  |  |  |
|  | AI | 0.078 98.08 % 43.10 % 96.82 % 0.195 |  |  |  |  |

For model generalization on different number of UE Rx beams for beam pair prediction, compared with the case that the training data samples and testing data samples are generated based on the same number of UE Rx beams, only marginal performance loss is observed for the case that different number of UE Rx beams are used for model training and testing.

The AI model can provide good generalization capability on different number of UE Rx beams for beam pair prediction.

Evaluation for temporal beam prediction

General description:

In high mobility environment, the task of fast beam tracking is more challenging. A typical way for beam tracking is periodic measurement and reporting, which introduces significant overhead that scales with the number of beams. Therefore, we utilize AI/ML technology to realize temporal beam prediction in this section. Specifically, recurrent neutral network is leveraged to implement the temporal beam prediction with assumption of a simple UE mobility pattern with random direction straight-line trajectories. The measured RSRPs of all or partial DL beams (or beam pairs) of some past time instances are used as the AI input and the predicted RSRPs of all DL beams (or beam pairs) of multiple future time instances are used as the AI output. That is, the beam set for measurement and the beam set for prediction are the same, or the beam set for measurement is a subset of the beam set for prediction. Besides, for each time instance within the prediction window, Top-K beams predicted by the adopted AI model may or may not be used for a refined beam sweeping.

Figure 3-1. Illustration of AI/ML based temporal beam prediction.

Dataset construction:

The dataset for model training and evaluation is generated according to the agreed parameters in RAN1#109-e, which is summarized in Table I in the appendix. Specifically, one UE per sector per drop is randomly generated with a total of 20 simulation drops. For antenna configuration, the baseline agreed in RAN#110bis-e is adopted directly, where a single panel with (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ and 2 panels with (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) are used by the gNB and UE, respectively. Besides, the gNB and UE are equipped with 32 Tx beams in total and 4 Rx beams per panel, respectively. In addition, more simulation assumptions for the temporal beam prediction are summarized in Table XII.

For temporal beam prediction, the random direction straight-line trajectories in Option #4 is adopted for modelling UE mobility, which means that multiple UEs are randomly dropped in multiple cells and move in a straight-line trajectory along an initially random direction with fixed speed. More details about Option #4 is provided in the appendix for reference. Compared with other UE trajectory options, Option #4 is simpler and beneficial for model generalization. The modelling of UE orientation and rotation may cause the adopted model to be overly complex with excessive training overhead. Thus, it has not been considered for the time being for simplicity. Additionally, spatial consistency procedure B in TR38.901 is modeled along the mobility trajectory to ensure that propagation parameters maintain continuity across multiple realizations.

Table XII. Simulation assumptions for the temporal beam prediction

| UE distribution | 1 UE per sector per drop with a total of 21 sectors and 20 simulation drops |
| --- | --- |
| TTI number per simulation drop | 150000 |
| BS Antenna Configuration | One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| UE Antenna Configuration | Two panel: (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) |
| Spatial consistency | Procedure B in TR38.901 |
| UE Speed | 30 km/h, 60 km/h, or 90 km/h |
| UE trajectory model | Option #4: Random direction straight-line trajectories |
| UE orientation | Initially random and keeps fixed |
| UE rotation speed | 0 |

AI/ML model and training methodology:

For cross-checking purpose, the adopted AI model for temporal beam prediction is captured in Table XIII. The LSTM network is used, where the model input is measured RSRPs of all or partial DL beams (or beam pairs) of past 5 time instances, and the model output is predicted RSRPs of all DL beams (or beam pairs) of future 5 time instances. The sampling interval between two adjacent time instances is 160ms. For the evaluation of generalization performance, data samples generated by mixed UE speeds of 30km/h and 60km/h are used for AI model training and testing. The corresponding dataset size is 60,000 and 180,000 for the case of UE speed of 30/60km/h and for the case of mixed UE speeds of 30km/h and 60km/h, respectively. Besides, model complexity is also taken into consideration, which includes the model size and the amount of computation (measured in FLOPs). Among them, the former describes the required memory size and the latter describes the required computing complexity, which directly impacts the inference time of the model.

Table XIII. Description of the adopted AI model for temporal beam prediction

| Model architecture | LSTM, FC layer |
| --- | --- |
| Number of layers | 4 |
| Model input | Measured RSRPs of past time instances |
| Model output | Predicted RSRPs of future time instances |
| Observation window length | 5 |
| Prediction window length | 5 |
| Dataset size | For performance comparison of fixed Set B and variable Set B across training and inference: 140000 |
|  | For performance comparison of Tx-Rx beam pair prediction and Tx beam prediction: 140000 |
|  | For evaluation of generalization: 60,000 (for a single UE speed of 30km/h or 60km/h or 90km/h) 180,000 (for mixed UE speeds of 30km/h, 60km/h, and 90km/h) |
| Percentage of training/validity/testing samples | 90%, 5%, 5% |
| Model size | 2.758M |
| FLOPs | 7.250M |

Performance KPIs:

To evaluate the performance of AI/ML in temporal beam prediction, we provide our preliminary simulation results in terms of beam prediction accuracy for Top-K beams. All adopted KPIs are appropriately defined according to the agreement achieved in RAN1#109e. Particularly, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams (unless otherwise mentioned). The beam prediction accuracy (%) for Top-K beams is defined as the percentage that the Top-1 genie-aided beam is one of the Top-K predicted beams. As with the spatial domain beam prediction, a second stage beam sweeping over these Top-K predicted beams is not considered for RS overhead counting. Otherwise, the RS overhead for the second stage beam sweeping would be the product of the number of UEs per sector and the number of Top-K beams. Thus, the RS overhead of the temporal beam prediction method may be even higher than that of a cell-specific exhaustive sweeping over all beams.

Performance comparison of fixed Set B and variable Set B

To study the selection of Set B of beams, performance comparison of fixed Set B and variable Set B during the observation window is conducted, where RSRPs of all measured beams in Set B are used as model inputs. In particular, as with the spatial domain beam prediction, Set C of beams can be measured and Top-4, Top-2, or Top-1 beams of Set C are used as model inputs (i.e., Set B) to reduce the reporting overhead for a NW-side model. More specifically, the following six different cases are considered for comparison.

- Case 2-1-1: Set B is changed following a set of pre-configured patterns (total 4 beam patterns)

- Case 2-1-2: Set B is changed among pre-configured patterns (one beam pattern is randomly selected from 4 beam patterns in each past time instance)

- Case 2-1-3: Set B consists of Top-4 beams of the beam set for measurement in each past time instance, on the basis of Case 2-1-1

- Case 2-1-4: Set B consists of Top-2 beams of the beam set for measurement in each past time instance, on the basis of Case 2-1-1

- Case 2-1-5: Set B consists of Top-1 beams of the beam set for measurement in each past time instance, on the basis of Case 2-1-1

- Case 2-1-6: Set B is fixed during the observation window

Table XIV.  Performance comparison of fixed Set B and variable Set B across observation slots (UE speed: 30km/h)

| Cases |  | Top-K | Top-K Prediction Accuracy (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T+1 | T+2 | T+3 | T+4 | T+5 |
| Case 2-1-1 | Non-AI | 4 | 65.09 | 64.46 | 63.83 | 63.27 | 61.66 |
|  | AI | 4 | 98.05 | 98.27 | 98.10 | 98.03 | 97.53 |
| Case 2-1-2 | Non-AI | 4 | 62.83 | 62.29 | 61.78 | 60.75 | 60.39 |
|  | AI | 4 | 95.49 | 94.95 | 93.92 | 93.24 | 92.37 |
| Case 2-1-3 | Non-AI | 4 | 65.25 | 64.61 | 63.65 | 62.47 | 61.78 |
|  | AI | 4 | 97.87 | 97.97 | 97.79 | 97.39 | 96.82 |
| Case 2-1-4 | Non-AI | 4 | 62.10 | 60.72 | 59.19 | 57.92 | 56.51 |
|  | AI | 4 | 96.78 | 96.22 | 96.29 | 95.34 | 94.51 |
| Case 2-1-5 | Non-AI | 4 | 53.80 | 51.60 | 49.93 | 48.24 | 46.63 |
|  | AI | 4 | 93.06 | 92.18 | 91.38 | 90.76 | 89.47 |
| Case 2-1-6 | Non-AI | 4 | 25.82 | 26.09 | 25.75 | 25.66 | 25.45 |
|  | AI | 4 | 98.02 | 98.07 | 97.69 | 97.73 | 96.67 |

The simulation results are shown in Table XIV. As observed, a significant performance gain over the non-AI method can be achieved by the AI based temporal beam prediction in terms of beam prediction accuracy for Top-4 beams. Additionally, compared with the case that Set B is fixed across training and inference (i.e., Case 2-1-6), a slight performance gain is observed if Set B is changed following a set of pre-configured patterns (i.e., Case 2-1-1).  Compared with the case that the beam pattern of Set B is randomly selected from 4 beam patterns in each past time instance (i.e., Case 2-1-2), a slight performance gain is observed if Set B is changed following a set of pre-configured patterns (i.e., Case 2-1-1). Besides, as can be seen from the simulation results of Case 2-1-3, Case 2-1-4, and Case 2-1-5, only a little performance loss is observed if Top-4 or Top-2 beams of all measured beams are used as model inputs. Therefore, it can be concluded that it is better to take partial beams from the beam set for measurement as the model input to reduce the beam reporting overhead for a NW-side model.

In BM-Case2, compared with the case that Set B is fixed across training and inference, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns.

In BM-Case2, compared with the case that the Set B pattern is randomly selected from a total of 4 beam patterns in each past time instance, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns.

In BM-Case2, if Top-4 or Top-2 beams of all measured beams are used as model inputs in each past time instance, only a little performance loss is observed compared with the case that all measured beams are used as model inputs.

To reduce UE report overhead in BM-Case 2 for a NW-side model, it is better to take partial beams from the beam set for measurement as the model input.

Performance comparison of Tx-Rx beam pair prediction and Tx beam prediction

To compare the performance of Tx-Rx beam pair prediction and Tx beam prediction based on the optimal or fixed Rx beam (i.e., Rx beam assumption 3 or 4), four different cases of AI model inputs for each past time instance and outputs for each future time instance are considered as follows.

- Case 2-2-1: RSRPs of all Tx beams of 5 past time instances are used as the model input and RSRPs of all Tx beams of 5 future time instances are used as the model output, where the optimal Rx beam (i.e., Rx beam assumption 3) is assumed for measurement

- Case 2-2-2: RSRPs of all Tx-Rx beam pairs of 5 past time instances are used as the model input and RSRPs of all Tx-Rx beam pairs of 5 future time instances are used as the model output, where the optimal Rx beam (i.e., Rx beam assumption 3) is assumed for measurement

- Case 2-2-3: RSRPs of all Tx beams of 5 past time instances are used as the model input and RSRPs of all Tx beams of 5 future time instances are used as the model output, where the specific (i.e., first) Rx beam (i.e., Rx beam assumption 4) is assumed for measurement

- Case 2-2-4: RSRPs of all Tx-Rx beam pairs of 5 past time instances are used as model input and RSRPs of all Tx-Rx beam pairs of 5 future time instances are used as model output, where the specific (i.e., first) Rx beam (i.e., Rx beam assumption 4) is assumed for measurement

For the KPI calculation, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) (i.e., Option B). Detailed simulation results of the first two cases above are provided in the result collection excel [4]. Some of the simulation results are also provided in Table XV for reference. As observed, the Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction under the optimal Rx beam or the fixed Rx beam. In addition, compared with the case of the fixed Rx beam, a slight performance gain is observed if the Tx beam prediction or Tx-Rx beam pair prediction is performed under the optimal Rx beam.

Table XV.  Performance comparison of Tx-Rx beam pair prediction and Tx beam prediction (UE speed: 30km/h)

| Cases |  | Top-K | Top-K Prediction Accuracy [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T+1 | T+2 | T+3 | T+4 | T+5 |
| Case 2-2-1 | Non-AI | 4 | 97.06 | 95.48 | 94.05 | 92.19 | 90.37 |
|  | AI | 4 | 99.17 | 98.99 | 98.85 | 98.50 | 98.25 |
| Case 2-2-2 | Non-AI | 4 | 90.35 | 86.07 | 83.22 | 80.67 | 77.77 |
|  | AI | 4 | 96.70 | 96.74 | 96.50 | 96.26 | 94.96 |
| Case 2-2-3 | Non-AI | 4 | 96.61 | 94.98 | 93.13 | 91.29 | 89.35 |
|  | AI | 4 | 98.62 | 98.32 | 98.13 | 98.09 | 97.61 |
| Case 2-2-4 | Non-AI | 4 | 21.42 | 21.05 | 20.54 | 20.13 | 19.85 |
|  | AI | 4 | 95.28 | 95.88 | 95.28 | 94.91 | 93.47 |

In BM-Case2, the Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction with the assumption of the optimal Rx beam or the specific Rx beam being used for measurement.

Evaluation for model generalization

To evaluate the generalization performance of AI model, RSRPs of all measured beam pairs of past time instances are used as model input, and the beam set for measurement and the beam set for prediction are the same. Datasets for training and testing are generated with a UE speed of 30km/h or 60km/h or with mixed UE speeds. According to the type of datasets for training and testing, the following seven different cases are considered for comparison.

- Case 2-3-1: Training with samples of mixed UE speeds, testing with samples of UE speed of 60km/h

- Case 2-3-2: Training with samples of mixed UE speeds, testing with samples of UE speed of 30km/h

- Case 2-3-3: Both training and testing with samples of mixed UE speeds

- Case 2-3-4: Both training and testing with samples of UE speed of 60km/h

- Case 2-3-5: Both training and testing with samples of UE speed of 30km/h

Detailed simulation results for the 30km/h and 60km/h cases above are provided in the result collection excel [5]. Some of the simulation results are also provided in Table XVI for reference. As can be seen from the simulation results in Table XVI, if the AI model is trained with samples of a single UE speed or mixed UE speeds, a little performance loss is observed as the UE speed in testing samples increases from 30km/h to 60km/h. The AI based method in case 2-3-1, case 2-3-2, case 2-3-3, and case 2-3-4 can also achieve a better performance than that of the non-AI method in scenarios with mixed UE speeds. Additionally, compared with the case that the same UE speed is used for generating the training data samples and the testing data samples, average performance is observed if both the training data samples and the testing data samples of AI model are the mixed data samples.

Table XVI.  Evaluation of generalization performance on different UE speeds

| Cases |  | Top-K | Top-K Prediction Accuracy [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T+1 | T+2 | T+3 | T+4 | T+5 |
| Case 2-3-1 | Non-AI | 4 | 93.76 | 88.64 | 83.34 | 78.49 | 74.24 |
|  | AI | 4 | 95.27 | 93.94 | 93.16 | 92.27 | 90.67 |
| Case 2-3-2 | Non-AI | 4 | 95.86 | 92.36 | 89.01 | 86.36 | 83.27 |
|  | AI | 4 | 96.69 | 96.03 | 95.70 | 95.14 | 93.47 |
| Case 2-3-3 | Non-AI | 4 | 93.79 | 88.58 | 83.99 | 79.76 | 75.62 |
|  | AI | 4 | 95.38 | 94.87 | 94.61 | 93.63 | 91.98 |
| Case 2-3-4 | Non-AI | 4 | 93.33 | 88.40 | 83.17 | 79.27 | 74.63 |
|  | AI | 4 | 95.37 | 95.33 | 95.47 | 95.27 | 93.50 |
| Case 2-3-5 | Non-AI | 4 | 96.60 | 93.50 | 90.53 | 86.47 | 83.67 |
|  | AI | 4 | 97.40 | 97.20 | 97.37 | 96.80 | 96.60 |

In BM-Case2, if the AI model is trained with samples of mixed UE speeds, a little performance loss is observed as the UE speed in testing samples increases from 30km/h to 60km/h.

In BM-Case2, compared with the case that the same UE speed is used for generating the training data samples and the testing data samples, average performance is observed if both the training data samples and the testing data samples of the AI model are mixed data samples.

Conclusions

In this contribution, we discuss the potential specification impacts and enhancements for AI/ML based beam management. We have the following observations and proposals.

Observations:

- The spatial domain beam prediction with a fixed beam pattern achieves a sufficiently high performance with only 25% beam overhead being used.

- With the same sampling rate on the whole beam space, Case 1-1-2 (without sampling at Rx beam space) obtains a better performance than Case 1-1-1 (with sampling at Rx beam space).

For DL Tx beam prediction, compared with the Rx beam assumption that the best Rx beam searched for one Tx beam within Set B is used, a better beam prediction performance is achieved if the best Rx beam for the best Tx beam within Set B is used for measurement.

For DL Tx beam prediction, the best performance is achieved by measurements of the “best” Rx beam (i.e., Option 1) among all options of Rx beam assumptions. Besides, Option 2a (i.e., measurements of specific Rx beam(s) per model input sample) and Option 2b (i.e., measurements of specific Rx beam(s) for all model input samples) have similar performances, which is slightly lower than that of Option 1.

For Tx-Rx beam pair prediction and DL Tx beam prediction, similar beam prediction performance is achieved with different uniform Set B patterns in the same sampling rate.

For DL Tx beam prediction, a sufficient high performance is achieved with Set B patterns 4/5/6 in a sampling rate of 12.5%, where the beam prediction accuracy for Top-5 beams is above 97%.

For DL Tx beam prediction, marginal performance loss is observed when the AI model inputs are Top 8 beams or Top 4 beams of all 16 beam measurements.

For Tx-Rx beam pair prediction, significant performance loss is observed when the AI model inputs are Top 8 beams or Top 4 beams of all 128 beam pair measurements.

The case that model training and testing are performed in mixed datasets of <T8 R1>, <T16 R1>, and <T32 R1> with different beam constructions achieves a better performance than that of the case of <T8 R1>, but is outperformed by the case of <T32 R1>.

The AI based Tx beam prediction with the optimal Rx beam achieves slightly better performance than that of the AI based method with a specific (e.g, first) Rx beam.

The AI model can provide good generalization capability on different number of beams in Set B for Tx beam prediction.

For the model generalization on different Set B patterns, if the Set B pattern of the testing data samples is different from that of the training data samples, a significant performance loss compared with the non-AI case is observed.

For the model generalization on different Set B patterns, if the Set B pattern of the testing data samples is included in the beam patterns of the training data samples, or the same Set B pattern is used during the model training phase and testing phase, the AI method obtains significant performance gains over the non-AI case.

The AI model can provide good generalization capability on different Set B patterns for Tx beam prediction.

For model generalization on different Rx beam assumptions for Tx beam prediction, compared with the case that the training data samples and testing data samples are generated based on the same Rx beam assumption, only marginal performance loss is observed for the case of different Rx beam assumptions on model training and model testing.

The AI model can provide good generalization capability on different Rx beam assumptions for Tx beam prediction.

For model generalization on different gNB settings, a significant performance loss is observed if the training data samples and the testing data samples are generated under different gNB beam structures.

Compared with the case that the same gNB beam structure is used for generating the training data samples and the testing data samples, only marginal performance loss is observed if the training data samples of AI model are mixed data samples.

For model generalization on different number of UE Rx beams for beam pair prediction, compared with the case that the training data samples and testing data samples are generated based on the same number of UE Rx beams, only marginal performance loss is observed for the case that different number of UE Rx beams are used for model training and testing.

The AI model can provide good generalization capability on different number of UE Rx beams for beam pair prediction.

In BM-Case2, compared with the case that Set B is fixed across training and inference, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns.

In BM-Case2, compared with the case that the Set B pattern is randomly selected from a total of 4 beam patterns in each past time instance, a slight performance gain is observed if Set B is changed following a set of pre-configured patterns.

In BM-Case2, if Top-4 or Top-2 beams of all measured beams are used as model inputs in each past time instance, only a little performance loss is observed compared with the case that all measured beams are used as model inputs.

In BM-Case2, the Tx beam prediction obtains a better performance than that of the Tx-Rx beam pair prediction with the assumption of the optimal Rx beam or the specific Rx beam being used for measurement.

In BM-Case2, if the AI model is trained with samples of mixed UE speeds, a little performance loss is observed as the UE speed in testing samples increases from 30km/h to 60km/h.

In BM-Case2, compared with the case that the same UE speed is used for generating the training data samples and the testing data samples, average performance is observed if both the training data samples and the testing data samples of the AI model are mixed data samples.

Proposals:

- At least for evaluation on the performance of DL Tx beam prediction, consider the following four Rx beam assumptions for providing input for AI/ML model for training and/or inference if applicable

Rx beam assumption 1: the best Rx beam for the best Tx beam within Set B

Rx beam assumption 2: the best Rx beam searched for one Tx beam within Set B

Rx beam assumption 3:  the best Rx beam for the best Tx beam within Set A

Rx beam assumption 4: a specific Rx beam (e.g., the first Rx beam in the first UE panel)

In the beam pair prediction, it’s better to take measurement results of all Rx beams as the model input instead of performing any sampling at the Rx beam space.

To reduce the overhead of UE feeding back beam measurements for DL Tx beam prediction in BM-Case 1, it is better to only report partial beams from a set of beams for measurement for a NW-side model.

To reduce UE report overhead in BM-Case 2 for a NW-side model, it is better to take partial beams from the beam set for measurement as the model input.

References

BMCase1_v000, simulation result collection excel, 3GPP TSG RAN1#112-bis-e, April 2023.

BMCase1_Txbeam_Generalization_v000, simulation result collection excel, 3GPP TSG RAN1#112-bis-e, April 2023.

BMCase1_BeamPair_Generalization_v000, simulation result collection excel, 3GPP TSG RAN1#112-bis-e, April 2023.

BMCase2_v000, 3GPP TSG RAN1#112-bis-e, April 2023.

BMCase2_Generalization_v000, simulation result collection excel, 3GPP TSG RAN1#112-bis-e, April 2023.

- Appendix

For dataset generation and performance evaluation for AI/ML in beam management, take the parameters (if applicable) in the following table for Dense Urban scenario for SLS.

Table I. Assumptions for Dense Urban scenario for AI/ML in beam management

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz SCS: 120 kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) Other deployment assumption is not precluded |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | For spatial domain beam prediction, 3km/h For time domain beam prediction: 30km/h (baseline), 60km/h (optional) Other values are not precluded |
| UE distribution | FFS UEs per sector/cell for evaluation. More UEs per sector/cell for data generation is not precluded. For spatial domain beam prediction: FFS: Option 1: 80% indoor ,20% outdoor as in TR 38.901 Option 2: 100% outdoor For time domain prediction: 100% outdoor |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | · [One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline] · [Four panels: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ as optional] · Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | [Panel structure: (M, N, P) = (1,4,2)] · 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline · Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | Companies to explain beam correspondence assumptions (in accordance to the two types agreed in RAN4) |
| Link adaptation | Based on CSI-RS |
| Traffic Model | FFS: Option 1: Full buffer Option 2: FTP model Other options are not precluded |
| Inter-panel calibration for UE | Ideal, non-ideal following 38.802 (optional) – Explain any errors |
| Control and RS overhead | Companies report details of the assumptions |
| Control channel decoding | Ideal or Non-ideal (Companies explain how it is modelled) |
| UE receiver type | MMSE-IRC as the baseline, other advanced receiver is not precluded |
| BF scheme | Companies explain what scheme is used |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| Other simulation assumptions | Companies to explain serving TRP selection Companies to explain scheduling algorithm |
| Other potential impairments | Not modelled (assumed ideal). If impairments are included, companies will report the details of the assumed impairments |
| BS Tx Power | [40 dBm] |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |

UE trajectory model Option #4: Random direction straight-line trajectories. - Initial UE location, moving direction and speed: UE is randomly dropped in a cell, and an initial moving direction is randomly selected, with a fixed speed. - The initial UE location should be randomly drop within the following blue area where d1 is the minimum distance that UE should be away from the BS. - Each sector is a cell and that the cell association is geometry based. - During the simulation, inter-cell handover or switching should be disabled. - For training data generation - For each UE moving trajectory: the total length of the UE trajectory can be set as T second if it is in time, of set as D meter if it is in distance. - The value of T (or D) can be further discussed - The trajectory sampling interval granularity depends on UE speed and it can be further discussed. - UE can move straightly along the entire trajectory, or - UE can move straightly during the time interval, where the time interval is provided by using an exponential distribution with average interval length - UE may change the moving direction at the end of the time interval. UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°] - If the UE trajectory hit the cell boundary (the red line), the trajectory should be terminated. - If the trajectory length (in time) is less than the length of observation window + prediction window, the trajectory should be discarded. - At the current stage, the length of observation window + prediction window is not fixed and the companies can report their values.