3GPP TSG-RAN WG1 Meeting #112bis-e		R1-2302445

e-Meeting, April 17th – April 26th, 2023

Title:	[DRAFT] reply LS on Comparison of SL-RSRP and SD-RSRP measurements

Response to:	R1-2302280 (R2-2302234)

Release:	Rel-18

Work Item:	NR_SL_relay_enh

Source:	vivo [RAN1]

To:	RAN2

Cc:	RAN4

Contact person:	Zichao Ji

jizichao@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1 thanks RAN2 for the LS informing about the work on session continuity enhancements and the new measurement report trigger. Regarding the following question on comparisons of RSRP measurements, RAN1 would provide the following answer.

Question:	Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report.

Answer: In RAN1’s view, at least when the following conditions are met:

- SL discovery message and normal SL PSSCH transmission are transmitted in the same pool,

- the (pre-)configured open loop power control parameters are aligned between SL discovery message and normal SL PSSCH transmission,

- SL pathloss based power control is disabled,

In this case, the comparison of SL-RSRP and SD-RSRP measurement is meaningful and can be used to trigger a measurement report. In other cases, it is not guaranteed that the measurement report would work as expected. For example, due to the SL pathloss based power control mechanism for unicast PSCCH transmission, the measurement result of the serving UE’s SL-RSRP may be lower than that of SD-RSRP. Consequently, if SL-RSRP is used for event trigging, the event Z2 may be triggered more frequently based on SL-RSRP than SD-RSRP.

# 2	Actions

To RAN2

RAN1 respectfully asks RAN2 to take the above information into account in related work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG RAN WG1 Meeting #113			May 22th – 26th, 2023				Incheon

TSG RAN WG1 Meeting #114			August 21th – 25th, 2023				Toulouse