3GPP TSG RAN WG1 Meeting #112bis-e		R1-2302448

e-Meeting, April 17th – April 26th, 2023

Title:	                     [Draft] Reply LS on error source distributions

Response to:	LS on error source distributions (R2-2302271)

Release:	Rel-18

Work Item:	        NR_pos_enh2

Source:	vivo[RAN1]

To:	RAN2

Cc:

Contact person:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1	Overall description

RAN1 thanks RAN2 for the LS on the error source distribution.

RAN1 would like to provide the following response for the value range of mean and standard deviation for the agreed error source as the following table, where the deviation is determined according to uncertainty parameter(e.g., nr-TimingQuality, locationUNC)

| Error source | Candidate(s) for distribution for error source | Value range of mean and deviation |
| --- | --- | --- |
| Timing measurement errors (NOTE 1, 2, 3) | - Gaussian distribution | Mean: 0 Deviation: INTEGER (0..31) Resolution:ENUMERATED (0.1m, 1m, 10m, 30m, …) |
| Inter-TRP synchronization errors | - Gaussian distribution | Mean: 0 Deviation: INTEGER (0..31) Resolution:ENUMERATED (0.1m, 1m, 10m, 30m, …) |
| TRP location error (e.g., NR-TRP-LocationInfo in [16]) | - Gaussian distribution | Mean of horizontal: 0 Deviation of horizontal: INTEGER (0..255) Mean of vertical 0 Deviation of vertical: INTEGER (0..255) (and the Deviation value can be mapped to the uncertainty value in TS 23.032) |
| ARP location error (e.g., ARPLocationInformation in [17]) | - Gaussian distribution | Mean of horizontal:0 Deviation of horizontal: INTEGER (0..255) Mean of vertical 0 Deviation of vertical: INTEGER (0..255) (and the Deviation value can be mapped to the uncertainty value in TS 23.032) |
| NOTE 1: Timing measurement errors are applicable to RSTD, RTOA and UE/gNB Rx-Tx time difference measurements. NOTE 2: It is assumed that the timing measurement error is associated with the first path. NOTE 3: It is assumed that the timing measurement error contains TEG related TX/RX timing error if the TEG related information is provided |  |  |

2	Actions

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into consideration in their future work.

3	Dates of next TSG RAN 1 meetings

TSG RAN WG1 Meeting #113			May 22th – 26th, 2023			Incheon

TSG RAN WG1 Meeting #114			August 21th – 25th, 2023			Toulouse