3GPP TSG RAN WG1 #112b-e		           R1- 2302451

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:                     Discussion on impact of SRS antenna switching for TDD-FDD band combinations

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

In [1], RAN4 asks RAN1 to take the observations and identified issues into account and consider possible clarification in specification if needed for better utilizing the feature of SRS antenna switching.

- Discussion

Various SRS antenna switching combinations are supported in NR depending on the UE capability. And, related capability is specified in 38.306, where UE also reports txSwitchImpactToRx and txSwitchWithAnotherBand, however the restriction on simultaneous SRS and PUSCH transmission is not reflected in 38.213. In LTE, following text is captured in the spec 36.213 to address similar issue.

“If a UE is configured with more than one serving cell, and for a group of cells belonging to bands that are signalled to be switched together in txAntennaSwitchUL the UE is not expected to transmit SRS and PUSCH on different antenna ports simultaneously.”

Based on the LS from RAN4[1], we support capturing similar text in 38.213, we are open to discuss whether the corresponding CR is introduced to Rel-15/16/17.

Proposal: for the UEs supporting SRS antenna switching and signaling corresponding txSwitchImpactToRx and txSwitchWithAnotherBand capabilities, introduce a CR to reflect restriction on simultaneous SRS and PUSCH transmission in NR spec 38.213.

- Conclusion

In this contribution, we discuss the RAN4 LS on impact of SRS antenna switching for TDD-FDD band combinations, we agree with the issue raised in the LS and have following proposal.

Proposal: for the UEs supporting SRS antenna switching and signaling corresponding txSwitchImpactToRx and txSwitchWithAnotherBand capabilities, introduce a CR to reflect restriction on simultaneous SRS and PUSCH transmission in NR spec 38.213.

Reference

[1] R1-2302268, “LS on clarification on impact of SRS antenna switching for TDD-FDD band combinations” RAN4, Huawei

[2] TS 36.213