3GPP TSG-RAN WG1 #112bis-e		                                  R1-2302456

e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on mixed R16 SSSG switching and R17 PDCCH monitoring adaptation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2023-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Background: Normally, Rel-16 SSSG switching is used for UE to immediately switching from a dense PDCCH monitoring (SSSG#0) outside the COT to a sparse PDCCH monitoring (SSSG#1) inside the COT. It is practically useful since outside the COT, the gNB need to perform LBT and occupy the channel frequently. Once the channel is occupied, the gNB has to send PDCCH immediately and delivery data to UE, otherwise, the occupancy will be expired. And once the UE enters the COT, for power saving purpose, UE is usually indicated to switch to another SSSG with sparse PDCCH monitoring periodicity. However, Rel-17 PDCCH monitoring adaptation has different purpose. When there is no requirement of traffic transmission, normally gNB will dynamically indicate UE to do PDCCH skipping. Issues: It has been agreed in RAN1 #108e meeting that R17 SSSG and R16 SSSG switching features do not support to be configured simultaneously. So, there will not be the case that R17 SSSG timer expires outside the COT duration. However, the current specification does not restrict the application of R17 PDCCH skipping outside COT. This means that a UE can still perform PDCCH skipping outside COT duration, resulting in unacceptable delay for channel occupancy. Solution: To not delay channel occupation, R17 PDCCH skipping should not be applied outside the COT duration, which means PDCCH skipping should be terminated once out of the COT duration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the current specfication that R17 PDCCH skipping should not be applied outside the COT duration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect UE behaviour if R17 PDCCH monitoring adaptation is applied outside the channel occupancy duration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

10.4	Search space set group switching and skipping of PDCCH monitoring

*** Unchanged text is omitted ***

A UE determines a slot and a symbol in the slot to start or stop PDCCH monitoring according to search space sets for a serving cell that the UE is provided searchSpaceGroupIdList or, if cellGroupsForSwitchList is provided, for a set of serving cells, based on the smallest SCS configuration μ among all configured DL BWPs in the serving cell or in the set of serving cells and, if any, in the serving cell where the UE receives a PDCCH and detects a corresponding DCI format 2_0 triggering the start or stop of PDCCH monitoring according to search space sets.

If a UE is provided with PDCCHSkippingDurationList and searchSpaceGroupIdList, the UE does not apply following procedures related to skipping of PDCCH monitoring outside the channel occupancy duration.

A UE can be provided a set of durations by PDCCHSkippingDurationList for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided searchSpaceGroupIdList-r17 on the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmission, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits.

*** Unchanged text is omitted ***