3GPP TSG-RAN WG1 #112bis-e                                                                   R1-2302457

e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on missing field descriptions of SCI 2-C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, CATT, GOHIGH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2023/4/4 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SCI format 2-C, the ‘Resource combinations’ field contains two FRIVs, and the ‘Lowest subChannel indices’ field contains two lowest subchannel indices of the two FRIVs indicated in the field ‘Resource combinations’. Similarly, the ‘Resource selection window location’ field indicates the starting time location and ending time location of the resource selection window. However, the description of these fields in the PHY layer only defines the size of the fields, while the contents (e.g., the endian, etc.) are unspecified. - Although section 8.1.5A of 38.214 has the following description: “The set of preferred or non-preferred resources r0,r1,r2,…, is indicated by a reference slot tref and M tuples (TRIVm,FRIVm,Prsvp,m), 1≤m≤M indicated by the 'resource combination' field, where for each tuple TRIVm is indicated by the 9 MSBs, followed by FRIVm and Prsvp,m (if present)”, it only defines how each tuple is indicated, but not how these tuples are indicated in one field (i.e., the parsing order of these tuples). - Similarly, the description for the ‘Lowest subChannel indices’ field and the ‘Resource selection window location’ field are respectively as below: - The starting sub-channel nsubCH,0start of the first resource of each tuple is indicated by the 'Lowest subChannel indices' field - The field 'Resource selection window location' is the concatenation of the starting time location and the ending time location of the resource selection window. The starting and ending time locations of the resource selection window are each encoded in the same way as the reference slot as described in clause 8.1.5A. - It can be seen that both descriptions do not define how each tuple is indicated, either. On the other hand, when these fields are carried by MAC CE, the MAC spec already descripes the coding order of these fields in TS 38.321. Consequently, it’s unclear in the SCI format 2-C how the field indicates each tuple, e.g., whether the first tuple is associated with the MSBs, or the LSBs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the first tupleunit in each field is indicated by the MSBs, and the second tuple is indicated by the LSBs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Communication between a pair of UEs having different endian archs may be broken, if one UE decodes the SCI as {MSBs: 1st tuple, LSBs: 2nd tuple}, but the other UE decodes the SCI as {LSBs: 1st tuple, MSBs: 2nd tuple}. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.4.1.3	SCI format 2-C

SCI format 2-C is used for the decoding of PSSCH, and providing inter-UE coordination information or requesting inter-UE coordination information. SCI format 2-C can be used only for unicast.

===omitted===

If the 'Providing/Requesting indicator' field is set to 0, all the remaining fields are set as follows:

-	Resource combinations – 2∙log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+Y bits as defined in Clause 8.1.5A of [6, TS 38.214], where

-	the first tuple is indicated by the log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+Y MSBs, and the second tuple is indicated by the left bits.

===omitted===

-	Lowest subChannel indices – 2∙log2N subChannel SL bits as defined in Clause 8.1.5A of [6, TS 38.214], where the log2N subChannel SL MSBs indicate the starting sub-channel of the first resource in the first tuple, and the starting sub-channel of the first resource in the second tuple is indicated by the log2N subChannel SL LSBs.

===omitted===

-	Resource selection window location – 2∙10+log2(10∙2μ) bits as defined in Clause 8.1.4A of [6, TS 38.214], where μ is defined in Table 4.2-1 of Clause 4.2 of [4, TS 38.211]. The 10+log2(10∙2μ) MSBs indicate the starting time location of the resource selection window and the ending time location is indicated by the 10+log2(10∙2μ) LSBs.