3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2302463

# e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on intra-UE multiplexing for MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | The physical priority can be configured for MBS and non-MBS. For a PUCCH with HARQ-ACK information for MBS overlapping with another PUCCH or PUSCH, it was agreed in RAN1 previous meeting that support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot. For intra-UE mulitplexing with different priorities, multiplexing of PUCCH and PUCCH or PUCCH and PUSCH with different prioirties are supported in URLLC WI. When the UE is not provided uci-MuxWithDiffPrio, UE cacels the PUCCH/PUSCH with lower priority index. It is not capatured in the current spec for the case with HARQ-ACK for MBS. When the UE is provided uci-MuxWithDiffPrio, UE multiplexes non-MBS UL transmisions with different priorities. When HARQ-ACK for MBS is involded, UE behaviour is not clear in this case. Agreement: For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource, - Support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot. - FFS whether it is subject to UE capability. - FFS the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot. - FFS whether sub-slot based PUCCH transmission for HARQ-ACK is supported. - FFS the case of HARQ-ACK feedback for multicast and other UCI for unicast. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of UE behavior for PUCCH with HARQ-ACK information for MBS overlapping with another PUCCH or PUSCH with different priorities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior for PUCCH with HARQ-ACK information for MBS overlapping with another PUCCH or PUSCH with different priorities is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

18	Multicast Broadcast Services

<Unchanged parts are omitted>

If a UE multiplexes in a PUCCH multicast HARQ-ACK information only according to second HARQ-ACK reporting modes and CSI reports, the UE determines a PUCCH resource as described in clause 9.2.5.2 for multiplexing CSI reports with HARQ-ACK information that is in response to PDSCH receptions without corresponding PDCCHs.

If a UE determines that a first PUCCH transmission with multicast HARQ-ACK information that has a priority index overlaps in time domain with a second PUCCH transmission or a PUSCH transmission that have a different priority indexes, including repetitions if any,

-	if the UE is not provided uci-MuxWithDiffPrio, the UE cancels the PUCCH transmission or PUSCH transmission of priority index 0 following the procedures in clause 9.

-	if the UE is provided uci-MuxWithDiffPrio, the UE resolves the overlapping following the procedures in clause 9.

<Unchanged parts are omitted>