3GPP TSG RAN WG1 #112bis-e			R1-2302469

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:	Further discussion on unified TCI framework extension for multi-TRP

Agenda Item:	9.1.1.1

Document for:	Discussion and Decision

Introduction

In previous meetings, progress was made for unified TCI framework extension for MTRP. This contribution provides our views on remaining issues and further discusses other potential topics on unified TCI framework extension for MTRP.

Further discussion on unified TCI framework extension for Rel-16/17 MTRP

Signaling mechanism for Rel16/17 M-DCI based MTRP

TCI state activation and indication

Following TCI state activation and indication mechanism for M-DCI based MTRP has been agreed in RAN1#110bis-e [1].

Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value Agreement On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

One issue related to TCI indication for M-DCI based MTRP is what the UE behavior is when a single DL TCI state or UL TCI state is indicated while both DL and UL TCI states are previously indicated for a cell configured with separate DL/UL TCI mode. Following the legacy behavior as in Rel-17 unified TCI state framework per TRP is preferred, i.e., only DL/UL TCI state is updated while UL/DL TCI state is still in use.

For unified TCI framework extension for M-DCI based MTRP configured with separate DL/UL TCI mode,

When a UE is indicated with one DL TCI state after indicated with both DL TCI state and UL TCI state, the UE maintains the previous UL TCI state for uplink channels/RSs.

When a UE is indicated with one UL TCI state after indicated with both DL TCI state and UL TCI state, the UE maintains the previous DL TCI state for downlink channels/RSs.

TCI state application

In RAN1#110bis-e [1], RAN1#111 [2] and RAN1#112 [3], agreements on the application of indicated TCI states for different channels are shown as follows.

Agreement (PDCCH and PDSCH) On unified TCI framework extension for M-DCI based MTRP: - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value - FFS: Other channel(s)/signal(s) that has explicit or implicit association with a coresetPoolIndex value - FFS: Other channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value Above are applicable to the CORESET(s) that is configured/allowed to follow the indicated joint/DL TCI state FFS: The configuration/rule to configure/allow CORESET(s) to follow the indicated joint/DL TCI state, including the option to reuse the same configuration/rule as in Rel-17 unified TCI framework Agreement (PDCCH and PDSCH) On unified TCI framework extension for M-DCI based MTRP, the same configuration/rule used in Rel-17 unified TCI framework (for determining whether the UE shall apply the indicated joint/DL TCI state to PDCCH on a CORESET and respective PDSCH) is reused to determine whether the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET associated with the same coresetPoolIndex value and PDSCH scheduled/activated by the PDCCH Agreement (PUSCH) On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value Agreement (STxMP PUSCH+PUSCH) - For multi-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission, - Alt1: The first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1 - The PUSCH is associated with SRS resource set with the same value of coresetPoolIndex - FFS: Which is the first SRS resource set, e.g., the set with lower set ID. - Regarding how to interpret the SRI/TPMI field in DCI: - For DG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where scheduling DCI format 0_1 or 0_2 is received - For Type 2 CG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where activation DCI is received. - For Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig and the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set Agreement (PUCCH) On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported

In this subclause, we provide our views on other channels and signal(s) that has explicit or implicit association with a coresetPoolIndex value.

PDCCH for inter-cell MTRP

For inter-cell MTRP when configured to follow the unified TCI states, the CORESET associated with CSS set other than Type3-PDCCH CSS sets, i.e. CORESET B/C/0 defined in Rel-17 for discussion purpose, can only be associated with the coresetPoolIndex value associated with serving cell PCI, and the CSI-RS associated with the indicated TCI state applied to these CORESETs should be QCLed with an SSB associated with serving cell PCI.

PUCCH

As given above, four Options were listed in RAN1#112 meeting, where Opt1 and Opt2 can be categorized into RRC configuration approach with different configuration ways, Opt3 and Opt4 can be seen as implicit approach considering two specific cases, PUCCH for HARQ-ACK and PUCCH for BFR.

For a PUCCH resource scheduled by a PDCCH with the CORESET associated with a coresetPoolIndex value, i.e., Opt3, the indicated joint/UL TCI state corresponding to the same coresetPoolIndex value can be applied to the PUCCH resource. This works well for HARQ-ACK feedback where the PUCCH resource is determined based on the last DCI of the scheduling TRP. Thus, at least for PUCCH resources for HARQ-ACK feedback, configuring an association between the PUCCH resource and a coresetPoolIndex value is unnecessary. The benefit is reduced workload of PUCCH configuration and saved PUCCH resources because a PUCCH resource/group can dynamically apply different joint/UL TCI states corresponding to different TRPs by the scheduling DCI.

Opt4 focuses on the extension of unified TCI state(s) applicable to PUCCH-SR for BFR. As described in subclause 2.5, one or two PUCCH-SR can be configured for UL grant for BFR MAC CE. For the case of one PUCCH-SR configuration, when beam failure event is declared for a TRP, applying the joint TCI state of the non-failed TRP to PUCCH-SR can avoid uplink blockage to the failed TRP at least for joint TCI state mode. Although uplink may not be failure for the downlink failed TRP for separate TCI state mode, above rule can also be used. If the UE is provided both the first and second configurations, the UE uses the first configuration to transmit a PUCCH with LRR associated with BFD-RS set q0,0 and the second configuration to transmit a PUCCH with LRR associated with BFD-RS set q0,1. When beam failure event is declared for a TRP, applying the joint TCI state of the non-failed TRP to the PUCCH-SR transmission associated with the BFD-RS set of the failed TRP is also necessary to avoid uplink blockage for joint TCI state mode. Same rule can also be applied to separate TCI state mode.

For PUCCH resource other than above where the association between the PUCCH resource and the TRP is unclear, either Opt1 or Opt2 can be used.

AP CSI-RS

To save the AP CSI-RS configuration overhead, an AP CSI-RS resource set configured to follow the unified TCI state to be shared between two TRPs needs to be enabled, i.e., either TRP can trigger the same AP CSI-RS but with TRP-specific joint/DL TCI state. Therefore, the indicated joint/DL TCI state corresponding to the coresetPoolIndex value associated with the triggering DCI can be applied.

SRS

For P/SP/AP SRS configured to follow the unified TCI state, the indicated joint/UL TCI state corresponding to a coresetPoolIndex can be applied, where the association between a coresetPoolIndex and an SRS resource set can be configured by RRC for P/SP SRS and can be identified by the triggering DCI for AP SRS. To enable an AP SRS resource set to be shared between two TRPs, i.e., either TRP can trigger the same AP SRS but with TRP-specific joint/UL TCI state, the AP SRS can apply the indicated joint/UL TCI state corresponding to the coresetPoolIndex value associated with the triggering DCI if the coresetPoolIndex value associated with the SRS resource set is not configured.

Type 1 CG PUSCH

To be aligned with the agreement from M-DCI based PUSCH + PUSCH simultaneous transmission where one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig, the same solution can be applied to non-overlapped M-DCI MTRP based Type 1 CG PUSCH transmission. Thus, the joint/UL state corresponding to an RRC-configured coresetPoolIndex value associated with the SRS_resource_set_index value configured in ConfiguredGrantConfig shall be applied.

In summary, we have following proposals on other channels/RSs.

For unified TCI state extension to M-DCI based MTRP, the TCI state application rule is defined per channel/RS,

For PUCCH, both Opt3 and Opt4 are supported. Otherwise, either Opt1 or Opt2 is adopted.

For AP CSI-RS configured to follow the unified TCI state, apply the indicated joint/DL TCI state corresponding to the coresetPoolIndex value associated with the triggering DCI.

For SRS following the unified TCI state, a coresetPoolIndex value can be configured to associate with an SRS resource set

For P/SP/AP SRS following the unified TCI state, apply the indicated joint/UL TCI state corresponding to the coresetPoolIndex value associated with SRS resource set.

If the coresetPoolIndex value associated with the AP SRS resource set is not configured, the AP SRS following the unified TCI state will apply the indicated joint/UL TCI state corresponding to the coresetPoolIndex value associated with the triggering DCI.

For Type 1 CG-PUSCH, apply the joint/UL state corresponding to an RRC-configured coresetPoolIndex value associated with the SRS_resource_set_index value configured in ConfiguredGrantConfig.

Signaling mechanism for Rel-16/17 S-DCI based MTRP

TCI state activation and indication

It has been agreed that the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list. Following agreement has been reached for S-DCI based MTRP in RAN1#110bis-e and RAN1#111.

Agreement On unified TCI framework extension, at least for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, up to 4 TCI states can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation - FFS: The possible combination(s) of joint/DL/UL TCI states that can be indicated to DL receptions and/or UL transmissions in a BWP/CC/TRP - Note: This agreement does not imply that there will be more than 2 DL or UL or joint TCI states indicated in a CC/BWP for the target use cases agreed in RAN1#109-e in AI 9.1.1.1 - Note: The maximum number of TCI states that can be indicated to each of the target use cases agreed in RAN1#109-e in AI 9.1.1.1 is remained the same as in Rel-16/17 Note: The maximum number of TCI states that can be indicated simultaneously to CJT-based PDSCH reception and the required type(s) of TCI states (i.e., DL /UL/joint) are independently discussed in this AI Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field - Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS

As a UE can be configured with either joint TCI mode or separate DL/UL TCI mode in a serving cell, the possible combinations of TCI sates mapped to a codepoint can be listed as follows for Rel-16/17 MTRP schemes according to the offline discussion [4].

2 joint TCI states for joint DL/UL TCI update in the CC/BWP

- 2 pairs of DL and UL TCI states for separate DL/UL TCI update in the CC/BWP

1 pair of DL and UL TCI states + 1 DL TCI state for separate DL/UL TCI update in the CC/BWP

1 pair of DL and UL TCI states + 1 UL TCI state for separate DL/UL TCI update in the CC/BWP

On unified TCI framework extension for S-DCI based MTRP, a TCI codepoint can be mapped to following combinations of joint/DL/UL TCI states in a BWP/CC/TRP:

2 joint TCI states for joint DL/UL TCI mode in the CC/BWP

2 pairs of DL and UL TCI states for separate DL/UL TCI mode in the CC/BWP

1 pair of DL and UL TCI states + 1 DL TCI state for separate DL/UL TCI mode in the CC/BWP

1 pair of DL and UL TCI states + 1 UL TCI state for separate DL/UL TCI mode in the CC/BWP

Another issue is whether the switching between STRP and MTRP for overall operation mode including all DL and UL channels and RSs can depend on the number of TCI states indicated by the TCI field in DCI (e.g., the UE assumes STRP operation when beam indication DCI indicates TCI state(s) only for one TRP, and the UE assumes MTRP operation when beam indication DCI indicates TCI states for both TRPs). In other words, once a UE is indicated with two joint/DL/UL TCI states, is the other joint/DL/UL TCI state previously indicated still valid when it is indicated with one joint/DL/UL TCI state?

In our view, only the indicated joint/DL/UL TCI state should be valid in such a case. Otherwise, there would be more effort to specify, e.g., the condition of switching back to one-unified TCI indication mode as Rel-17, the way to indicate which TCI state is to be updated, etc. Besides, STRP and MTRP switching depending on the number of indicated TCI states can easily achieve dynamic switching between STRP operation and MTRP operation all channels and RSs simultaneously. It seems that updating only one of the two TCI states provides some flexibility, but there is no evidence that current number of TCI codepoints, i.e., up to 8 TCI codepoints, for S-DCI based MTRP as supported in Rel-16 is not sufficient. On top of up to 8 TCI codepoint, Rel-18 has already provided more flexibility than Rel-16/Rel-17 to select one TCI state from the two indicated joint/UL TCI states by a specific indicator field or channel resource associated with a certain TCI state(s). And enhanced group-based beam reporting only supports up to 4 pairs of beam groups, and the gNB is unaware of the Rx/Tx panels for non-group-based beam reporting thus it cannot arbitrarily combine two joint/DL TCI states for simultaneously reception by UE. Thus, the requirement of flexibility is lack of basis.

On unified TCI framework extension for S-DCI based MTRP,

When a UE is indicated with one joint TCI state for joint TCI state mode, the UE switches to STRP operation for both downlink and uplink channels.

When a UE is indicated with one DL TCI state for separate TCI state mode, the UE switches to STRP operation for downlink channels.

When a UE is indicated with one UL TCI state for separate TCI state mode, the UE switches to STRP operation for uplink channels.

On unified TCI framework extension for S-DCI based MTRP, the max number of bits for TCI field is 3.

TCI state application

Up to RAN1#112, following agreements have been made for the TCI state application for channels for S-DCI based MTRP.

Agreement (PDCCH) On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) Agreement (PDSCH) On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. Agreement (PDSCH) On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - FFS: Detail design of the new indicator field Agreement (PDSCH) On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value Agreement (PDSCH) On unified TCI framework extension for S-DCI based MTRP, down-select at least one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]: - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt2: The UE shall apply the first one of two indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt3A: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception - Alt4: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field of the most recently applied beam indication DCI Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold (if the threshold is needed) Agreement (PUCCH) On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design Agreement (PUSCH) On unified TCI framework extension for S-DCI based MTRP, use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 Agreement (PUSCH) On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission

In the following, we further discuss the FFS parts of the above agreements and show our views on TCI state application for other RSs.

PDSCH

A new indicator field, temporally called [TCI selection field] with size of 2 bits in DCI format 1_1/1_2 is introduced to dynamically select one or both TCI states for MTRP PDSCH reception when two joint/DL TCI states are indicated. Codepoints of three cases of TCI selection were agreed and the use of one codepoint “11” is to be determined.

In our views, the mapping order can also be indicated like UL PUSCH repetition to provide more flexibility without consuming extra [TCI selection field] bits and saving the TCI state codepoints in MAC CE. Such flexibility allows the network to indicate proper mapping order by associating a better TCI states to the resources of prioritized spatial/frequency/time domain, which will improve the latency of URLLC services.

On unified TCI framework extension for S-DCI based MTRP, codepoint “10” and “11” of the [TCI selection field] are used to select both indicated joint/DL TCI states for the PDSCH reception scheduled/activated by the DCI format 1_1/1_2

If codepoint “10” is indicated, the 1st and 2nd indicated joint/DL TCI states are mapped to the first and second spatial/frequency/time resources for SDM, FDM, TDM and SFN schemes.

If codepoint “11” is indicated, the 1st and 2nd indicated joint/DL TCI states are mapped to the second and first spatial/frequency/time resources for SDM, FDM, TDM and SFN schemes.

Three cases of default TCI state(s) for PDSCH reception to be determined when two joint/DL TCI states have been indicated are listed as follows:

The default joint/DL TCI state(s) for the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2, if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2

The default joint/DL TCI state(s) for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured without the [TCI selection field]

The default joint/DL TCI state(s) for PDSCH reception scheduled/activated by DCI format 1_0

As a unified solution, the first indicated joint/DL TCI state can be applied to the non-SFN PDSCH reception scheduled or activated by the DCI.

On unified TCI framework extension for S-DCI based MTRP when two joint/DL TCI states are indicated, apply the first indicated joint/DL TCI state for non-SFN PDSCH reception when it is scheduled/activated 1) by DCI format 1_1/1_2 with offset between the reception of the DCI and the PDSCH reception less than a threshold in FR2 if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, 2) by DCI format 1_1/1_2 without [TCI selection field] configured, or 3) by DCI format 1_0.

However, PDSCH-SFN needs careful discussion depending on whether UE supports dynamic switching of SFN with STRP or not and the legacy behavior should be tried to maintain and some implication is desired. Whether PDSCH-SFN applies the first indicated joint/DL TCI state or both indicated joint/DL TCI states also depends on whether UE supports dynamic switching of SFN with STRP for PDSCH or not. When PDSCH-SFN is configured and two joint/DL states are indicated, TCI state application rule for several cases as specified in Rel-17 PDSCH-SFN is given as follows:

If enableTwoDefaultTCI-States is configured and time offset between the reception of the DL DCI format 1_1/1_2 with [TCI selection field] and the PDSCH is less than the threshold timeDurationForQCL, both indicated joint/DL TCI states are used as default TCI states to buffer the signal, then UE will use one or both indicated joint/DL TCI state(s) for PDSCH reception as indicated in [TCI selection field].

For PDSCH reception scheduled by DCI format 1_0, 1_1 and 1_2 without [TCI selection field], if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL, if UE supports dynamic switching between SFN and STRP for PDSCH, UE applies the first indicated joint/DL TCI state(s) for PDSCH reception; otherwise, UE applies both indicated joint/DL TCI state(s) for PDSCH reception.

For a UE supports dynamic switching between SFN and STRP for PDSCH, if enableTwoDefaultTCI-States is not configured and time offset between the reception of the DL DCI format 1_0/1_2 with [TCI selection field] and the PDSCH is less than the threshold timeDurationForQCL, the first indicated joint/DL TCI state is used as default TCI states for signal buffering and for PDSCH reception; for a UE does not support dynamic switching between SFN and STRP for PDSCH, the UE does not expect enableTwoDefaultTCI-States is not configured or time offset between the reception of the scheduling DL DCI and the PDSCH is less than the threshold timeDurationForQCL.

When PDSCH-SFN is configured and two joint/DL states are indicated, TCI state application rule for several cases is given as follows:

If enableTwoDefaultTCI-States is configured and time offset between the reception of the DL DCI format 1_0/1_2 with [TCI selection field] and the PDSCH is less than the threshold timeDurationForQCL, both indicated joint/DL TCI states are used as default TCI states to buffer the signal, then UE will use one or both indicated joint/DL TCI state(s) for PDSCH reception as indicated in [TCI selection field].

For PDSCH reception scheduled by DCI format 1_0, 1_1 and 1_2 without [TCI selection field], if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or larger than the threshold timeDurationForQCL, if UE supports dynamic switching between SFN and STRP for PDSCH, UE applies the first indicated joint/DL TCI state(s) for PDSCH reception; otherwise, UE applies both indicated joint/DL TCI state(s) for PDSCH reception.

For a UE supports dynamic switching between SFN and STRP for PDSCH, if enableTwoDefaultTCI-States is not configured and time offset between the reception of the DL DCI format 1_0/1_2 with [TCI selection field] and the PDSCH is less than the threshold timeDurationForQCL, the first indicated joint/DL TCI state is used as default TCI states for signal buffering and for PDSCH reception; for a UE does not support dynamic switching between SFN and STRP for PDSCH, the UE does not expect enableTwoDefaultTCI-States is not configured or time offset between the reception of the scheduling DL DCI and the PDSCH is less than the threshold timeDurationForQCL.

PUSCH

When two SRS resource sets are configured, it has been agreed that the SRS resource indicator field is used as the indicator field in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH TDM repetition, SDM and SFN-based PUSCH Tx schemes scheduled/activated by the DCI format 0_1/0_2. And four codepoints are used for PUSCH TDM repetition. For SDM and SFN-based PUSCH Tx schemes, since two joint/UL TCI states are simultaneously used and there is a fixed association among one joint/UL TCI state, one SRS resource set and one Tx panel, it doesn’t make sense to switch the association between joint/UL TCI states or SRS resource set and Tx panels, then codepoint “11” can be reserved or have same meaning as codepoint “10”. In other words, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission layer(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission layer(s) associated with the second SRS resource set for CB/NCB.

On unified TCI framework extension for S-DCI based MTRP, for SDM/SFN scheme of STxMP where two SRS resource sets are configured, when two joint/UL TCI states are indicated by TCI field in DCI format 1_1/1_2,

If the DCI format 0_1/0_2 indicates codepoint "10" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission layer(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission layer(s) associated with the second SRS resource set for CB/NCB.

Codepoint “11” is reserved.

For PUSCH scheduled by DCI format 0_0 when two joint/UL TCI states are indicated, a default TCI state, e.g., the first indicated joint/UL TCI state, can be applied to the PUSCH transmission.

On unified TCI framework extension for S-DCI based MTRP, the first indicated joint/UL TCI state is applied for PUSCH scheduled/activated by DCI format 0_0 when two joint/UL TCI states are indicated.

AP CSI-RS

In Rel-17, AP CSI-RS will follow the unified TCI state if QCL-Info is absent in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for an aperiodic CSI-RS resource set configured for CSI/BM. An RRC signaling can be used to inform the AP CSI-RS resource set to apply the first or the second indicated joint/DL TCI state. This can also be used for the enhanced group-based beam reporting where resourcesForChannel and resourcesForChannel2 are configured, each contains one CSI-RS resource set associated with one joint/DL TCI state. But CSI-RS for NCJT CSI reporting, one CSI-RS resource set contains two CMR groups, each associating with a joint/DL TCI state. To achieve the flexibility of the association between CMR groups and joint/DL TCI states, an RRC signaling can be configured per CMR group. Therefore, we have following proposal:

On unified TCI framework extension for S-DCI based MTRP, if QCL-Info is absent in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for an aperiodic CSI-RS resource set configured for CSI/BM,

For aperiodic CSI-RS for NCJT CSI measurement, UE applies the indicated joint/DL TCI states to CMR groups according to the mapping between two CMR groups and two indicated joint/DL TCI states based on RRC configuration provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState;

For enhanced group-based beam reporting, UE applies the indicated joint/DL TCI states to CSI-RS resource sets provided by resourcesForChannel and resourcesForChannel2 according to the mapping between two CSI-RS resource sets and two indicated joint/DL TCI states based on RRC configuration provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState;

Otherwise, an RRC configuration is provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set.

SRS

For P/SP/AP SRS configured to follow the unified TCI state, use RRC configuration to inform that the UE shall apply the first one, the second one, or both indicated joint/UL TCI states to an SRS resource set. Also, one SRS resource set can be shared between two TRPs to save the configuration of SRS resource sets for individual TRPs. If the RRC parameter is not configured, the AP SRS will apply the indicated joint/UL TCI state corresponding to the indicated joint/DL TCI state applied to the CORESET carrying the triggering DCI.

For P/SP/AP SRS configured to follow the unified TCI state

An RRC parameter can be used to inform that the UE shall apply the first one, the second one, or both indicated joint/UL TCI states to an SRS resource set.

If the RRC parameter is not configured, the AP SRS will apply the indicated joint/UL TCI state corresponding to the indicated joint/DL TCI state applied to the CORESET carrying the triggering DCI.

Power control in unified TCI framework extension for MTRP

In Rel-17 unified TCI framework, UL power control mechanism is re-designed and depends on the indicated joint or UL TCI state. The PLRS is included in a joint/UL TCI state, while the spatial relation RS can be used as PLRS for a UE supporting beam alignment only. For each of PUSCH, PUCCH and SRS, one PC setting (including P0, alpha, closed loop index) can be optionally associated with each of the joint/UL TCI state per BWP. When the PC setting is not configured for the indicated TCI state, a default setting independent of TCI state, for each of PUSCH, PUCCH and SRS, can be applied respectively.

In Rel-18 MTRP, the following agreement on UL PC extension was achieved up to RAN1#112 [3].

Agreement On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH /PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH /PUCCH (including P0, alpha for PUSCH , and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion. - FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported - FFS: UL PC enhancement for CB and non-CB SRS in above case FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH Agreement On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS

If the PC parameter settings are not configured for the indicated TCI states, default PC settings should be defined for each channel/RS. For MTRP, it is required that multiple default PC settings for each channel/RS are configured by RRC in BWP, where each default PC setting is for a TRP.

For power control in unified TCI framework extension for MTRP

If the PC parameter settings (including PLRS, P0, alpha, closed loop index) are not associated with the indicated TCI states, use the default PC settings configured by RRC signaling for each channel/RS for each TRP.

In general, UL PC parameter settings required by eMBB and URLLC are different, a TCI state ID is associated with two settings from different lists for eMBB and URLLC, where the additional setting includes only P0 while other parameters are configured in one setting. Based on the value of TCI state ID and the open-loop power control parameter set indication field in DCI, PC parameter settings associated with the TCI state ID can be determined and applied for PUSCH of a specific service. If the PC parameter settings are not associated with the indicated TCI states, P0 or additional P0 in default settings can be determined based on the value of SRI field and OLPC field.

UL PC parameter settings from different setting lists including additional P0 list are associated with an UL or joint TCI state for different services.

TCI state ID and OLPC are used to determine PC parameters.

If the PC parameter settings are not associated with the indicated TCI states, P0 or additional P0 can be determined based on the value of SRI field and OLPC field.

CA case with common TCI state update for Rel-16/17 MTRP

In Rel-17 unified TCI framework, common TCI state activation and update is supported to provide common QCL information and/or common UL Tx spatial filter across a set of configured CCs. The unified TCI state pool can be configured for each BWP/CC or in reference BWP/CC, where the BWPs/CCs without RRC-configured TCI state pool will apply the TCI state pool in a reference BWP of a reference CC. The source RS providing QCL-TypeD information and UL Tx spatial filter can be one source RS across CCs or per CC, where the CC-specific source RSs are further associated with a same QCL-TypeD RS. For the beam application time in CA case, the first slot and BeamAppTime_r17 symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication.

Common TCI state update for M-DCI based MTRP

Multiple CCs supporting M-DCI based MTRP can contain CORESETs associated with same coresetPoolIndex values in Rel-16/17. In Rel-18, common TCI state update for a TRP across multiple CCs can also be supported. For example, a first CC group in a CC list includes the CCs with coresetPoolIndex 0, while a second CC group in the CC list includes the CCs with coresetPoolIndex 1, where some CCs including coresetPoolIndex 0 and coresetPoolIndex 1 belong to two CC groups.

Then it can be clarified whether TCI state pool is configured across TRPs or for a specific TRP in a BWP/CC. If the TCI state pool is absent in a BWP of the CC, the UE can apply the TCI state pool from the reference BWP of a reference CC with same coresetPoolIndex value.

Based on the RRC-configured TCI state pool across all TRPs or per TRP, common TCI state update can be applied for the BWPs/CCs with same coresetPoolIndex value in a CC list. For example, in a CC list, CC1 and CC2 include coresetPoolIndex 0 and coresetPoolIndex 1, while CC3 only include coresetPoolIndex 1. When a common TCI state ID is indicated by a PDCCH associated with coresetPoolIndex 0, the common TCI state ID is applied for TCI state update for CC1 and CC2 but not for CC3.

For CA case in M-DCI based MTRP scenario, common TCI state update is applied for the BWPs/CCs with same coresetPoolIndex in a CC list.

Common TCI state update for S-DCI based MTRP

Common TCI state update can be applied to all CCs in a CC list supporting S-DCI based MTRP. When a TCI codepoint is indicated by TCI field in DCI, where the codepoint is mapped to one or two TCI states, the dynamic switch between single TRP and MTRP operation for all CCs in a CC list can be determined based on the number of the indicated TCI states. If one TCI state is indicated, single-TRP operation is applied for all CCs in a CC list, while if two TCI states are indicated, transmission is switched to MTRP operation.

Dynamic switch between STRP and S-DCI based MTRP operation can be determined based on the number of the indicated TCI states for all CCs in a CC list.

Beam application time for MTRP

Because the common TCI state can be updated for the BWPs/CCs configured with same TRP or for all BWPs/CCs in a CC list, the definition of beam application time in MTRP should be modified based on the different solutions for both S-DCI and M-DCI based MTRP. The first slot and BeamAppTime_r17 symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication, where the carriers are configured with same coresetPoolIndex value, or all carriers in a CC list.

For beam application time in CA case for MTRP, the first slot and BeamAppTime_r17 symbols are both determined on the carrier with the smallest SCS among the carriers applying the beam indication, where the carriers are configured with same coresetPoolIndex value, or all carriers in a CC list.

TRP-specific BFR for unified TCI framework

In Rel-17, TRP-specific BFR procedure and BFR for unified TCI framework are both introduced. When unified TCI is configured for MTRP, the Rel-17 TRP-specific BFR procedure can be directly extended for Rel-18.

For beam failure detection, two BFD-RS sets are configured explicitly by RRC, e.g. each BFD-RS set can be associated with a coresetPoolIndex value or, if applicable, associated with a channel group ID. The TCI state of BFD-RS or BFD-RS resource configured by RRC can be updated with the indicated TCI state associated with this BFD-RS. For implicit BFD-RS determination, the source RSs in the multiple indicated TCI states can be used as the BFD-RSs for MTRP, respectively. For new beam identification, corresponding two NBI-RS sets can be configured similar to Rel-17, where first BFD-RS set is associated with first NBI-RS set, and the second BFD-RS set is associated with second NBI-RS set.

On TRP-specific BFR for unified TCI framework extension

For explicit BFD-RS configuration, each BFD-RS set can be associated with a coresetPoolIndex value or, if applicable, associated with a channel group ID.

For implicit BFD-RS determination, the source RSs in the two indicated TCI states are used as the BFD-RSs for MTRP, respectively.

NBI-RS configuration mechanism in Rel-17 TRP-specific BFR mechanism can be reused.

When beam failure event is declared for a TRP, if the next available UL grant for BFR MAC CE is towards the failed TRP, BFR MAC CE may not reach the gNB successfully due to the unavailable uplink of the failed TRP. At this time, the available UL grant towards a non-failed TRP is preferentially used to ensure the successful BFRQ transmission at least for joint TCI mode. For separate TCI state mode, the available UL grant from any TRP with good uplink can be used to transmit BFR MAC CE. If PUCCH-SR is to be transmitted to require UL grant for BFR MAC CE, the PUCCH-SR resource is transmitted with a proper joint/UL TCI state as discussed in subclause 2.1.2 for M-DCI based MTRP or the configured joint/UL TCI state for S-DCI based MTRP. Thus, the PUCCH-SR resource can assist the gNB in obtaining the failed TRP based on the correspondence between the PUCCH-SR resource and BFD-RS set.

For a failed TRP configured with joint TCI state mode,

The available UL grant of a non-failed TRP can be used preferentially for BFR MAC CE.

For a failed TRP configured with separate TCI state mode,

The available UL grant from any TRP with good uplink is used to transmit BFR MAC CE.

In Rel-18, the BFRR of TRP-specific BFR in Rel-17 can be reused, i.e. a first PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for transmission of the PUSCH carrying BFR MAC CE and having a toggled NDI field value. After X symbols from the BFRR, the TCI state of the PDCCH in all CORESET, PDSCH, aperiodic CSI-RS, PUCCH, PUSCH and SRS should be reset, where the X symbols is determined based on the smallest of the SCS configuration of the active DL BWP for the PDCCH reception and of the active DL BWP(s) of the serving cell which contains the failed TRP indicated by BFR MAC CE.

BFRR mechanism in Rel-17 TRP-specific BFR mechanism can be reused.

Simultaneous multi-panel UL transmission

Much progress has been made in the AI 9.1.4.1, UL precoding indication for multi-panel transmission. Both SDM scheme and SFN scheme from two panels are supported in S-DCI based MTRP, and simultaneous PUSCH + PUSCH transmitted is supported in M-DCI based MTRP.

Enhanced beam report

In Rel-17 MPUE, to facilitate UE-initiated panel activation and selection, the beam report is enhanced to include the index of UE capability value set related to each pair of CRI or SSBRI and L1-RSRP/L1-SINR, where each index only indicates the maximum supported number of SRS ports. Based on the enhanced beam report, UL beam and panel can be indicated by gNB for UL channel/RS transmission. In Rel-18, simultaneous multi-panel UL transmission should be studied and specified. The principle for UL panel selection mechanism of Rel-17 MPUE can be reused, where beam report format is enhanced to facilitate multiple beam indication for simultaneous UL transmission from different UE panels.

In Rel-17 enhanced beam report, each UE capability value set only comprises the supported maximum number of SRS ports and any two capability value sets are different. If multiple UE panels share same capability value set, the number of panels and the exact panel indicated by the index in beam report is transparent to the gNB. As a result, the gNB cannot select two UE panels with same capability value set for simultaneous UL transmission without disclosing UE panels in UE capability report. Considering that two beams can be indicated for DL simultaneous reception by the UE based on the legacy group-based beam report, enhanced group-based beam report in Rel-18 can be introduced for UL beam indication for simultaneous transmission from multiple panels at the UE.

Reuse the principle for UL panel selection mechanism of Rel-17 MPUE, e.g. based on group-based beam report enhancement.

In Rel-15 if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple spatial domain receive filters. In Rel-17 MTRP, if the UE is configured with the higher layer parameter groupBasedBeamReporting-r17, the UE shall report in a single reporting instance nrofReportedRSgroup, if configured, group(s) of two CRIs or SSBRIs selecting one CSI-RS or SSB from each of the two CSI Resource Sets for the report setting, where CSI-RS and/or SSB resources of each group can be received simultaneously by the UE.

For group-based beam report enhancement in Rel-18, one possible solution is to introduce the index indicating UE capability value set for each SSBRI/CRI of every resource group. In a beam reporting instance multiple groups of two SSBRIs/CRIs are reported, where each index is along with the pair of SSBRI/CRI and L1-RSRP/L1-SINR. But a pair of DL beams that can be simultaneously received by different panels may not always be simultaneously transmitted due to factors such as UL power limitations, panel Rx/Tx capabilities, etc.

Another possible solution is to introduce an additional indication bit in each reported resource group to indicate whether the reported beam group can be simultaneously transmitted or not. For example, the contents of enhanced group-based beam report can be included as follows. For each reported resource group, depending on the additional indication bit, e.g. Indicator 1 and Indicator 2, two UE panels indicated by two indexes, i.e. index#X1 and index#Y1 of first resource group, or index#X2 and index#Y2 of second resource group, can be determined to be used for DL-only simultaneous reception or for DL simultaneous reception and UL simultaneous transmission.

{SSBRI/CRI #1, SSBRI/CRI #2} of 1st resource group, {SSBRI/CRI #1, SSBRI/CRI #2} of 2nd resource group, {L1-RSRP/L1-SINR #1, L1-RSRP/L1-SINR #2} of 1st resource group, {L1-RSRP/L1-SINR #1, L1-RSRP/L1-SINR #2} of 2nd resource group, {index#X1, index#Y1} of 1st resource group {index#X2, index#Y2} of 2nd resource group {Indicator 1} of 1st resource group {Indicator 2} of 1st resource group

Introduce an additional indication bit to indicate whether a pair of SSBRIs/CRIs in a resource group is used for DL-only simultaneous reception or for DL simultaneous reception and UL simultaneous transmission for MPUE.

UL TCI state indication

The solutions for indication of multiple DL and UL TCI states focusing on MTRP by using Rel-17 unified TCI framework can be reused. If two joint TCI state or UL TCI states are indicated by gNB based on the enhanced group-based beam report, i.e. two indicated TCI states are determined according to two SSBRIs/CRIs from one resource group in the beam report, two panels can be selected for UL simultaneous transmission.

For M-DCI based MTRP, DL/UL channels can be scheduled by two DCIs corresponding to different coresetPoolIndex values and apply different indicated TCI states, respectively. If the gNB expects to schedule simultaneous multi-panel UL transmission, it needs to ensure that two TCI states indicated by two beam indication DCIs corresponding to different coresetPoolIndex values are from a reported resource group and are applied to the scheduled DL/UL channels.

For S-DCI based MTRP, when the scheme of simultaneous multi-panel UL transmission is configured, if two TCI states corresponding to different UE panels are indicated by a DCI based on the enhanced group-based beam report, the multiple indicated TCI states can be applied to the PUCCH, and the SRS resource set indicator field in UL DCI can indicate two TCI states applied for PUSCH.

Reuse the solutions for indication of multiple TCI states focusing on MTRP use case, where the multiple TCI states are determined according to two SSBRIs/CRIs from one resource group in the beam report.

Power control

In Rel-17 UL S-DCI based MTRP, to achieve TRP-specific open-loop power control, two sets of power control parameters including P0/alpha/PL-RS/closed-loop-index are indicated by two SRI fields for MTRP PUSCH repetition. And, two spatial relation Infos or power control parameter sets containing P0/PL-RS/closed-loop-index are activated for one PUCCH resource. TRP-specific closed-loop power control is also supported for both PUSCH and PUCCH by two TPC fields for two closed-loop indices. To report the power headroom for PUSCH transmission towards each TRP to assist TRP-specific power control, TRP-specific PHR reporting is supported. In the Rel-17 unified TCI framework, power control parameter setting including P0/alpha/closed-loop-index for UL channels or signals are associated with the common TCI state. Common PL-RS for UL transmission is also associated with or derived from the common TCI state.

Based on the TRP-specific power control design and the power control parameter determination in the unified TCI framework, panel-specific power control can be facilitated. From our perspective, the following aspects should be considered for both S-DCI based and M-DCI based STxMP:

Panel-specific open-loop and closed-loop power control

Power limit for multi-panel PUSCH transmission

Panel-specific PHR reporting (discussed in our companion paper [5])

Panel/TRP-specific open-loop and closed-loop power control

Since we focus on multi-panel transmission towards multiple TRPs, two sets of power control parameter settings and two PL-RSs can be acquired by two indicated TCI states as we illustrate in subclause 2.3, and separate open-loop power control can be achieved. Regarding closed loop-power control, two TPC fields are designed for two closed-loop indices respectively in Rel-17 UL MTRP, which can be inherited to STxMP.

Panel-specific open-loop power control can be achieved by two PC settings associated with two indicated TCI states, while panel-specific closed-loop power control can inherit the design in Rel-17.

Panel-specific power limit

Power calculation should be further discussed to make it compatible with STxMP in S-DCI based MTRP operation. Currently power is calculated based on one transmission occasion corresponding to only one PUSCH transmission by using following equation. For transmission occasion i, specific P0/alpha, Pathloss measured by PL-RS, and closed-loop power adjustment are involved to calculate transmission power. According to UE power class, the transmission power is limited by PCMAX, f,c(i)  which is the configured UE maximum output power for carrier f of a serving cell c.

[dBm]

Regarding STxMP on one transmission occasion, panel-specific P0/alpha, Pathloss, and closed-loop power adjustment can be acquired. Then panel-specific transmission power can be calculated by using the above formula respectively to achieve panel-specific power control. To enable this, per-panel power limits should be defined. Considering panel-specific power control should be applicable for both STxMP and single panel transmission because dynamic switch between single panel transmission and STxMP has been agreed, current power limitation should be reused as panel-specific power limits. Panel is identified as one beam or TCI state in spec in order not to disclose implementation. In the same spirit, panel-specific power control can be achieved by defining beam-specific power limits.

From RAN4 LS reply [6], panel-specific power limitation is feasible, and RAN4 confirms that it can define the configured transmitted power requirement per-TCI for STxMP. So, we propose to introduce TCI state-specific power limits to achieve panel-specific power control.

Panel (TCI state)-specific power limits should be introduced for panel-specific power control.

Conclusions

In this contribution, we further discuss the issues on unified TCI framework extension for both S-DCI based MTRP and M-DCI based MTRP, and have the following proposals.

- For unified TCI framework extension for M-DCI based MTRP configured with separate DL/UL TCI mode,

When a UE is indicated with one DL TCI state after indicated with both DL TCI state and UL TCI state, the UE maintains the previous UL TCI state for uplink channels/RSs.

When a UE is indicated with one UL TCI state after indicated with both DL TCI state and UL TCI state, the UE maintains the previous DL TCI state for downlink channels/RSs.

For unified TCI state extension to M-DCI based MTRP, the TCI state application rule is defined per channel/RS,

For PUCCH, both Opt3 and Opt4 are supported. Otherwise, either Opt1 or Opt2 is adopted.

For AP CSI-RS configured to follow the unified TCI state, apply the indicated joint/DL TCI state corresponding to the coresetPoolIndex value associated with the triggering DCI.

For SRS following the unified TCI state, a coresetPoolIndex value can be configured to associate with an SRS resource set

For P/SP/AP SRS following the unified TCI state, apply the indicated joint/UL TCI state corresponding to the coresetPoolIndex value associated with SRS resource set.

If the coresetPoolIndex value associated with the AP SRS resource set is not configured, the AP SRS following the unified TCI state will apply the indicated joint/UL TCI state corresponding to the coresetPoolIndex value associated with the triggering DCI.

For Type 1 CG-PUSCH, apply the joint/UL state corresponding to an RRC-configured coresetPoolIndex value associated with the SRS_resource_set_index value configured in ConfiguredGrantConfig.

On unified TCI framework extension for S-DCI based MTRP, a TCI codepoint can be mapped to following combinations of joint/DL/UL TCI states in a BWP/CC/TRP:

2 joint TCI states for joint DL/UL TCI mode in the CC/BWP

2 pairs of DL and UL TCI states for separate DL/UL TCI mode in the CC/BWP

1 pair of DL and UL TCI states + 1 DL TCI state for separate DL/UL TCI mode in the CC/BWP

1 pair of DL and UL TCI states + 1 UL TCI state for separate DL/UL TCI mode in the CC/BWP

On unified TCI framework extension for S-DCI based MTRP,

When a UE is indicated with one joint TCI state for joint TCI state mode, the UE switches to STRP operation for both downlink and uplink channels.

When a UE is indicated with one DL TCI state for separate TCI state mode, the UE switches to STRP operation for downlink channels.

When a UE is indicated with one UL TCI state for separate TCI state mode, the UE switches to STRP operation for uplink channels.

On unified TCI framework extension for S-DCI based MTRP, the max number of bits for TCI field is 3.

On unified TCI framework extension for S-DCI based MTRP, codepoint “10” and “11” of the [TCI selection field] are used to select both indicated joint/DL TCI states for the PDSCH reception scheduled/activated by the DCI format 1_1/1_2

If codepoint “10” is indicated, the 1st and 2nd indicated joint/DL TCI states are mapped to the first and second spatial/frequency/time resources for SDM, FDM, TDM and SFN schemes.

If codepoint “11” is indicated, the 1st and 2nd indicated joint/DL TCI states are mapped to the second and first spatial/frequency/time resources for SDM, FDM, TDM and SFN schemes.

On unified TCI framework extension for S-DCI based MTRP when two joint/DL TCI states are indicated, apply the first indicated joint/DL TCI state for non-SFN PDSCH reception when it is scheduled/activated 1) by DCI format 1_1/1_2 with offset between the reception of the DCI and the PDSCH reception less than a threshold in FR2 if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, 2) by DCI format 1_1/1_2 without [TCI selection field] configured, or 3) by DCI format 1_0.

When PDSCH-SFN is configured and two joint/DL states are indicated, TCI state application rule for several cases is given as follows:

If enableTwoDefaultTCI-States is configured and time offset between the reception of the DL DCI format 1_0/1_2 with [TCI selection field] and the PDSCH is less than the threshold timeDurationForQCL, both indicated joint/DL TCI states are used as default TCI states to buffer the signal, then UE will use one or both indicated joint/DL TCI state(s) for PDSCH reception as indicated in [TCI selection field].

For PDSCH reception scheduled by DCI format 1_0, 1_1 and 1_2 without [TCI selection field], if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or larger than the threshold timeDurationForQCL, if UE supports dynamic switching between SFN and STRP for PDSCH, UE applies the first indicated joint/DL TCI state(s) for PDSCH reception; otherwise, UE applies both indicated joint/DL TCI state(s) for PDSCH reception.

For a UE supports dynamic switching between SFN and STRP for PDSCH, if enableTwoDefaultTCI-States is not configured and time offset between the reception of the DL DCI format 1_0, 1_1, and 1_2 with [TCI selection field] and the PDSCH is less than the threshold timeDurationForQCL, the first indicated joint/DL TCI state is used as default TCI states for signal buffering and for PDSCH reception; for a UE does not support dynamic switching between SFN and STRP for PDSCH, the UE does not expect enableTwoDefaultTCI-States is not configured or time offset between the reception of the scheduling DL DCI and the PDSCH is less than the threshold timeDurationForQCL.

On unified TCI framework extension for S-DCI based MTRP, for SDM/SFN scheme of STxMP where two SRS resource sets are configured, when two joint/UL TCI states are indicated by TCI field in DCI format 1_1/1_2,

If the DCI format 0_1/0_2 indicates codepoint "10" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission layer(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission layer(s) associated with the second SRS resource set for CB/NCB.

Codepoint “11” is reserved.

On unified TCI framework extension for S-DCI based MTRP, the first indicated joint/UL TCI state is applied for PUSCH scheduled/activated by DCI format 0_0 when two joint/UL TCI states are indicated.

On unified TCI framework extension for S-DCI based MTRP, if QCL-Info is absent in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for an aperiodic CSI-RS resource set configured for CSI/BM,

For aperiodic CSI-RS for NCJT CSI measurement, UE applies the indicated joint/DL TCI states to CMR groups according to the mapping between two CMR groups and two indicated joint/DL TCI states based on RRC configuration provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState;

For enhanced group-based beam reporting, UE applies the indicated joint/DL TCI states to CSI-RS resource sets provided by resourcesForChannel and resourcesForChannel2 according to the mapping between two CSI-RS resource sets and two indicated joint/DL TCI states based on RRC configuration provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState;

Otherwise, an RRC configuration is provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState for the aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set.

For P/SP/AP SRS configured to follow the unified TCI state

An RRC parameter can be used to inform that the UE shall apply the first one, the second one, or both indicated joint/UL TCI states to an SRS resource set.

If the RRC parameter is not configured, the AP SRS will apply the indicated joint/UL TCI state corresponding to the indicated joint/DL TCI state applied to the CORESET carrying the triggering DCI.

For power control in unified TCI framework extension for MTRP

If the PC parameter settings (including PLRS, P0, alpha, closed loop index) are not associated with the indicated TCI states, use the default PC settings configured by RRC signaling for each channel/RS for each TRP.

UL PC parameter settings from different setting lists including additional P0 list are associated with an UL or joint TCI state for different services.

TCI state ID and OLPC are used to determine PC parameters.

If the PC parameter settings are not associated with the indicated TCI states, P0 or additional P0 can be determined based on the value of SRI field and OLPC field.

For CA case in M-DCI based MTRP scenario, common TCI state update is applied for the BWPs/CCs with same coresetPoolIndex in a CC list.

Dynamic switch between STRP and S-DCI based MTRP operation can be determined based on the number of the indicated TCI states for all CCs in a CC list.

For beam application time in CA case for MTRP, the first slot and BeamAppTime_r17 symbols are both determined on the carrier with the smallest SCS among the carriers applying the beam indication, where the carriers are configured with same coresetPoolIndex value, or all carriers in a CC list.

On TRP-specific BFR for unified TCI framework extension

For explicit BFD-RS configuration, each BFD-RS set can be associated with a coresetPoolIndex value or, if applicable, associated with a channel group ID.

For implicit BFD-RS determination, the source RSs in the two indicated TCI states are used as the BFD-RSs for MTRP, respectively.

NBI-RS configuration mechanism in Rel-17 TRP-specific BFR mechanism can be reused.

For a failed TRP configured with joint TCI state mode,

The available UL grant of a non-failed TRP can be used preferentially for BFR MAC CE.

For a failed TRP configured with separate TCI state mode,

The available UL grant from any TRP with good uplink is used to transmit BFR MAC CE.

BFRR mechanism in Rel-17 TRP-specific BFR mechanism can be reused.

Reuse the principle for UL panel selection mechanism of Rel-17 MPUE, e.g. based on group-based beam report enhancement.

Introduce an additional indication bit to indicate whether a pair of SSBRIs/CRIs in a resource group is used for DL-only simultaneous reception or for DL simultaneous reception and UL simultaneous transmission for MPUE.

Reuse the solutions for indication of multiple TCI states focusing on MTRP use case, where the multiple TCI states are determined according to two SSBRIs/CRIs from one resource group in the beam report.

Panel-specific open-loop power control can be achieved by two PC settings associated with two indicated TCI states, while panel-specific closed-loop power control can inherit the design in Rel-17.

Panel (TCI state)-specific power limits should be introduced for panel-specific power control.

References

RAN1 Chair’s Notes, RAN1#110bis-e.

RAN1 Chair’s Notes, RAN1#111.

RAN1 Chair’s Notes, RAN1#112.

R1-2208075, Moderator summary on extension of unified TCI framework (Round 2), Moderator (MediaTek), RAN1#110.

R1-2302474, Further discussion on UL precoding indication for multi-panel transmission, vivo, RAN1#112bis-e.

R4-2303494, Reply LS on UE power limitation for STxMP in FR2, RAN4#106.