3GPP TSG RAN WG1 #112bis-e	                                           R1-2302473

e-Meeting, April 17th – April 26th, 2023

Source: 	vivo

Title:	Further discussion on SRS enhancements

Agenda Item:	9.1.3.2

Document for:	Discussion and Decision

Introduction

In this contribution, remaining issues on SRS enhancements on TDD CJT for cyclic shift/comb offset hopping and capacity enhancement such as power control enhancement are further analyzed. Besides, potential schemes for 8-port SRS for the usage of codebook /antenna switching are discussed, including how to map a 8-port SRS resource on one symbol and how to support 8-port SRS resource mapped on multiple symbols in TDM manner.

SRS enhancement for TDD CJT

Remaining issues on new hopping schemes

It has been agreed that both cyclic shift hopping and comb offset hopping for SRS are supported in Rel-18. The remaining issues of two new hopping schemes should be further discussed, such as time-domain granularity, hopping reinitialization/initialization, co-existing with legacy SRS resources, and combinations of multiple hopping schemes.

The combination of hopping schemes

Agreement For SRS interference randomization, support: - Opt. 3: Both cyclic shift hopping and comb offset hopping. - At least the two features can be separately configured - FFS: Combined cyclic shift hopping and comb offset hopping for a UE - FFS: Separate or combined with SRS sequence group hopping / sequence hopping - FFS: Associated UE capability

In the previous meeting, the issue of the combination of hopping schemes was proposed, such as the combination of cyclic shift hopping and comb offset hopping, the combination of two new hopping schemes and two legacy hopping schemes, i.e., sequence hopping and group hopping. However, whether the additional benefit can be provided by these combinations should be further discussed.

To evaluate the performance of these combinations, LLS simulation is performed for TDD CJT scenario. Assume that there are two TRPs and two UEs, UE1 transmits SRS1 to TRP1 and TPR2, while UE2 transmits SRS2 to TRP1 and TPR2 as shown in Figure 1.

Figure 1. SRS transmission in TDD CJT scenario

Besides, SRS1 and SRS2 are both configured as 4 ports with comb 2 and R=4, where port 1000 and port 1002 are in comb offset 0 with CS=4 and CS=0, and port 1001 and port 1003 are in comb offset 1 with CS=6 and CS=2. The group sequence u=2 and u=7 are used for two SRS resources separately. Moreover, to simulate the large cross-SRS interference between two SRS resources, the SRS transmission power from UE1 to TRP1, UE1 to TRP2, UE2 to TRP1, UE2 to TRP2 is set as 0dB, -3dB, -3dB, 6dB. In the simulation, multiple hopping schemes are evaluated as shown in Figure 2, including sequence hopping, group hopping, comb offset hopping, cyclic shift hopping, and multiple combinations.

Figure 2. The MSE performance of different hopping schemes

In Figure 2, MSE is evaluated to show the performance of SRS channel estimation for UE1 at TRP2. It is observed that all hopping schemes can provide randomization gain for cross-SRS interference. Cyclic shift hopping provides the largest performance gain with 4.5dB due to its effective randomization in the code domain. Regarding the combination schemes, they can only provide limited additional gain than cyclic shift hopping.

The additional benefit of the combination of hopping schemes is limited.

Moreover, from the perspective of complexity at UE/gNB side, the combination of hopping schemes would increase the complexity for UE to transmit SRS, and it would also increase the complexity of SRS channel estimation.

Cyclic shift hopping is not expected to be configured with comb offset hopping in Rel-18.

Both cyclic shift hopping and comb offset hopping are not expected to be configured with sequence hopping and group hopping.

Time-domain granularity

Agreement For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options: - Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions. - Option 2: Within a slot, hopping based on only the symbol index l'. - Option 3: No intra-slot hopping. - FFS: Time domain hopping behavior further depends on system frame number (SFN) nf. - FFS: reinitialization periodicity of N radio frames or reinitialization based on system frame number. - FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately) - FFS: At least support reinitialization at the beginning of each radio frame.

After determining the basic hopping pattern, how to define the hopping granularity in the time domain is another key point for any hopping mechanism. Like in group hopping and sequence hopping for SRS, the hopping granularity is per symbol, while in Rel-17 partial sounding, the hopping granularity is per frequency hopping. For two new hopping schemes, the hopping granularity as per symbol, per R symbols, or per slot should be further discussed.

If only considering the randomization performance, hopping per symbol would exactly provide gratifying performance gain. However, from the perspective of implementation complexity, hopping granularity may affect the complexity of UE and gNB. Smaller hopping granularity requires more UE/gNB complexity when generating SRS and do channel estimation, while providing better randomization performance for SRS. Therefore, the tradeoff between randomization performance and UE/gNB complexity can be considered. Hopping per R symbols is an appropriate granularity between per symbol and per slot .

The tradeoff between randomization performance and UE/gNB complexity should be considered when determining the hopping granularity.

From the perspective of UE/gNB complexity and randomization performance, support both per symbol and per R repetitions as hopping granularity for cyclic shift hopping and comb offset hopping, which can be configured by gNB.

UE-specific hopping initialization

Agreement For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs. - Option 1: Reuse the SRS sequence identity nIDSRS. - Option 2: Introduce new ID(s). - FFS: the value range, one new ID or two separate new IDs, default ID(s)

In the previous meeting, it has been agreed that the hopping pattern is based on the pseudo-random sequence c(i). When cyclic shift or comb offset hopping is configured, if two SRS resources from two UEs have the same initialization of cyclic shifts or comb offset, then they would still collide with each other. Therefore, UE-specific initialization should be introduced to make different hopping patterns for different UEs. One easy way is to reuse SRS sequence ID for UE-specific initialization, which is similar to group hopping where sequence group index u is determined by .

Moreover, some companies proposed to introduce new ID(s) for cyclic shift hopping and comb offset hopping. However, how to randomize the hopping pattern can depend on multiple factors, such as symbol index, slot index, frame index, and so on. In other words, even based on SRS sequence ID for UE-specific initialization, there would be enough randomized hopping patterns for different UEs, considering initialization ID is just one of the parameters in the final randomization formula. Besides, when multiple hopping schemes can’t combine, there would be natural to use SRS sequence ID for UE-specific initialization for new hopping schemes and legacy hopping schemes.

Support sequence ID  as the UE-specific initialization factor for both cyclic shift hopping and comb offset hopping.

Hopping pattern reinitialization

For sequence hopping and group hopping, the hopping pattern would be reinitialized at the beginning of each radio frame. That means sequence hopping and group hopping can only provide randomization in one frame. In the next frame, the hopping pattern would be reused as what in the current frame. One of the advantages of this reinitialization mechanism is to reduce the UE complexity of generating SRS. If there is no reinitialization, UE would generate the hopping pattern with infinite length.

Therefore, for cyclic shift hopping and comb offset hopping, a similar principle should be applied, considering the complexity of generating SRS and randomization gain. Due to the persistent interference between two SRS resources colliding with each other, performing hopping reinitialization per frame would not be enough to handle the cross-SRS interference. Thus, per N frames can be considered as the reinitialization granularity with initialization ID . In this case, UE should only generate the hopping pattern with a length of N frames, and the same hopping pattern can be reused per N frames.

Support to reinitialize the hopping pattern per N frames for both cyclic shift hopping and comb offset hopping with initialization ID .

Co-existing with legacy SRS resources

In the current system, multiple SRS resources from different UEs can be multiplexed on the same time-frequency resource based on orthogonal cyclic shifts. Taking cyclic shift hopping as an example, if the network configures one legacy SRS resource without cyclic shift hopping and Rel-18 SRS resource with cyclic shift hopping on the same time-frequency resource, where their initially configured cyclic shifts are orthogonal. After cyclic shift hopping in the Rel-18 SRS resource, changed cyclic shifts may collide with the cyclic shifts of the legacy SRS resource. That would degrade the channel estimation performance of the legacy SRS resource. Fortunately, the collision caused by cyclic shift hopping is temporary in this case. In other words, even if the collision happens, the cross-SRS interference is randomized. To completely void such cases, the network can also configure the legacy SRS and Rel-18 SRS in the different comb positions by implementation. The principle of comb offset hopping co-existing with legacy SRS resources is similar.

Support to handle the co-existing issue with legacy SRS resources by gNB implementation without specification, when cyclic shift hopping or comb offset hopping is enabled for Rel-18 SRS resources.

Cyclic shift hopping with finer granularity

Since cyclic shift hopping has been agreed in the last meeting, how to design the cyclic shift granularity can be further discussed. For cyclic shift hopping, cyclic shifts could be generated based on a default randomization rule rather than a direct configuration. Therefore, a finer granularity with K oversampling of cyclic shifts can be also considered without additional spec effort.

The LLS simulation result is given in Figure 3. Assume that there are two TRPs and two UEs, UE1 transmits SRS1 to TRP1 and TPR2, while UE2 transmits SRS2 to TRP1 and TPR2. SRS1 and SRS2 are both configured as 4 ports with comb 2 and R=1, where port 1000 and port 1002 are in comb offset 0 with CS=4 and CS=0, and port 1001 and port 1003 are in comb offset 1 with CS=6 and CS=2. The group sequence u=2 and u=7 are used for two SRS resources separately. The SRS transmission power from UE1 to TRP1, UE1 to TRP2, UE2 to TRP1, UE2 to TRP2 is set as 0dB, -3dB, -3dB, 6dB.

Figure 3. The MSE performance of cyclic shift hopping

In Figure, MSE is evaluated to show the performance of SRS channel estimation for UE1 at TRP2 with different values of K oversampling of cyclic shifts. It can be observed that finer granularity can provide additional gain for cyclic shift hopping with the increase of K.

Support cyclic shift hopping with finer cyclic shift granularity in Rel-18.

Enhanced Power control

Agreement For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set: - Option 1: - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha - Each transmission occasion of the SRS resource is towards multiple TRPs - Option 2: - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS - Different transmission occasions of the SRS resource can be towards different TRPs

Power control enhancement techniques were discussed in the previous meeting. However, enhancement on power control seems not helpful for increasing SRS capacity for TDD CJT. If power control is adjusted based on multiple TRPs, though cross-SRS interference can be reduced at some TRPs, the interference at other TRPs would increase.

Assume that TRP 1 is the target TRP and TRP2 is the cooperative TRP for UE1 as shown in Figure 1. Based on the legacy power control mechanism, SRS1 transmitted from UE1 is based on the PL-RS from TRP1, where the transmission power of SRS1 is P1. Thus, the received power of SRS1 at TRP2 is low, since the PL from TRP 2 is larger than from TRP1. In other words, cross-SRS interference on SRS2 transmitted from UE2 at TRP2 is low. However, if the power control of SRS1 is based on two PL-RSs from TRP1 and TRP2, then SRS1 may be transmitted with larger power P2 (P2>P1). In this case, SRS2 would suffer larger cross-SRS interference from SRS1. Therefore, enhancement on power control may be helpful for some UEs in CJT, but would cause more interference on SRS from other UEs.

Enhancement on power control for SRS may be helpful for some UEs in TDD CJT, but would also cause more cross-SRS interference to other UEs.

Don’t support power control enhancement on SRS for TDD CJT.

SRS enhancement for UL 8Tx

Multiplexing principle of legacy SRS

The design for legacy SRS can be classified as a cyclic shift and FDM based multiplexing, since the multiplexing of multiple ports in one SRS resource is achieved by different cyclic shifts and/or frequency positions. The determination of the cyclic shift nSRScs,i for one SRS port can be shown as follows.

where nSRScs is the value of cyclic shift configured by RRC, nSRScs,max is the maximum number of cyclic shifts, pi is the index of one SRS port, NapSRS is the number of ports of the SRS resource. Moreover, the occupying frequency position kTCpi of one SRS port can be presented by the following formula.

where KTC is the comb size of one SRS resource and kTC is the offset in the frequency domain configured by RRC. From the above equations, it can be observed that for different numbers of SRS ports and comb structures, the method used to multiplex SRS ports are different in some cases.

When one SRS resource is configured with 2 ports, no matter whether the comb is equal to 2, 4, or 8, SRS ports are only multiplexed by different cyclic shifts. However, in the case of one SRS resource with 4 ports, due to the different comb structures, there are different ways of multiplexing multiple ports. For instance, regarding comb 4, if SRS is configured with a value of the cyclic shift nSRScs smaller than 6, 4 SRS ports are multiplexed by different cyclic shifts with the same interval. However, if SRS is configured with a value of the cyclic shift nSRScs equal to or larger than 6, the SRS ports 1001 and 1003 are also mapped in different frequency positions additionally, which can be configured for the case with larger delay spread. Regarding comb 8 which is introduced for SRS in Rel-17, 6 cyclic shifts can’t be uniformly distributed with an interval of integer. Therefore, SRS ports 1000 and 1002 (or 1001 and 1003) are multiplexed with two cyclic shifts with the interval as 3 on the same frequency position, and SRS ports 1000 and 1001 (or 1002 and 1003) are mapped in different frequency positions.

8-port SRS resource on single symbol

Agreement For an 8-port SRS resource in a SRS resource set ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with m OFDM symbols (m >= 1), at least support the 8 ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof). - m takes the legacy values, i.e., 1,2,4,8,10,12,14. Agreement For one single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH, when the SRS resource is configured with n ports (n <= 8) and m OFDM symbols (m >= 1), at least support the n ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof). - n can be 8 - m takes the legacy values, i.e., 1,2,4,8,10,12,14. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), at least support: - For comb 2, support 1 and 2 comb offsets - For comb 4, support 2 and [4] comb offset - For comb 8, support 4 comb offsets Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof) - Option 2: For comb 4, do not support 4 comb offsets.

To keep the same multiplexing principle for one SRS resource with 8 ports, a natural way is to reuse a similar multiplexing principle of the legacy SRS resource, i.e., cyclic shift and FDM based multiplexing. However, due to the increased number of SRS ports, the interval of cyclic shifts for each SRS port would be smaller than the legacy SRS resource. Therefore, specific designs consideration might be needed for different comb sizes.

Comb 2

For comb 2, there are up to 8 cyclic shifts that can be used for SRS ports. When the number of ports in one SRS resource equals 8, one most direct way is to map 8 cyclic shifts to 8 ports as given by the following formula, which is the same as the legacy formula for comb 2.

The detailed values of cyclic shifts for comb 2 are shown in Table 1.

Cyclic shift mapping for SRS resource with 8 ports in comb 2

|  | Port 1000 | Port 1001 | Port 1002 | Port 1003 | Port 1004 | Port 1005 | Port 1006 | Port 1007 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| CS 1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 |
| CS 2 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 |
| CS 3 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 |
| CS 4 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 |
| CS 5 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 |
| CS 6 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 |
| CS 7 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 |

Regarding the comb offset mapping, three potential schemes are given as follows.

- Scheme 1: Map the 8 ports on the same frequency position. In this case, the orthogonality between different ports is only guaranteed by associated cyclic shifts.

- Scheme 2: Map port {1000, 1002, 1004, 1006} and port {1001, 1003, 1005,1007} in two different frequency positions.

- Scheme 3: Map two groups in different frequency positions only when SRS resource is associated with a certain value of cyclic shiftby RRC. This scheme is the same as what for 4-port SRS with comb 2.

- Comparing the above three schemes, the main drawback of scheme 1 is that performance degrades significantly when the channel delay is large, due to the small interval between different cyclic shifts. Besides, scheme 3 is a combination of scheme 1 and scheme 2, which would be more flexible to handle cases with different channel delays.

For 8-port SRS with comb 2, support 8 ports mapping on 8 different cyclic shifts separately as the following formula

For 8-port SRS with comb 2, whether 8 ports mapping on 1 or 2 comb offsets based on the value of cyclic shiftconfigured by RRC as the following formula.

Comb 4

For comb 4, there are 12 cyclic shifts available for multiplexing 8-port SRS. However, similar to the case of 4-port SRS in comb 8, 12 cyclic shifts can’t be divided by 8, leading to unequal intervals among 8 ports. To handle this issue, mapping 2 or 4 different cyclic shifts for 8-port SRS can be considered. For instance, when 4 different cyclic shifts are assigned for 8-port SRS, the same solution as 4-port SRS in comb 8 can be reused to map the cyclic shifts to 8-port SRS.

The detailed values of cyclic shifts for comb 4 are shown in Table 2.

Cyclic shift mapping for SRS resource with 8 ports in comb 4

|  | Port 1000 | Port 1001 | Port 1002 | Port 1003 | Port 1004 | Port 1005 | Port 1006 | Port 1007 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS 0 | 0 | 0 | 3 | 3 | 6 | 6 | 9 | 9 |
| CS 1 | 1 | 1 | 4 | 4 | 7 | 7 | 10 | 10 |
| CS 2 | 2 | 2 | 5 | 5 | 8 | 8 | 11 | 11 |
| CS 3 | 3 | 3 | 6 | 6 | 9 | 9 | 0 | 0 |
| CS 4 | 4 | 4 | 7 | 7 | 10 | 10 | 1 | 1 |
| CS 5 | 5 | 5 | 8 | 8 | 11 | 11 | 2 | 2 |
| CS 6 | 6 | 6 | 9 | 9 | 0 | 0 | 3 | 3 |
| CS 7 | 7 | 7 | 10 | 10 | 1 | 1 | 4 | 4 |
| CS 8 | 8 | 8 | 11 | 11 | 2 | 2 | 5 | 5 |
| CS 9 | 9 | 9 | 0 | 0 | 3 | 3 | 6 | 6 |
| CS 10 | 10 | 10 | 1 | 1 | 4 | 4 | 7 | 7 |
| CS 11 | 11 | 11 | 2 | 2 | 5 | 5 | 8 | 8 |

Regarding the comb offset mapping, it has been agreed that 4 comb offsets are supported for 8-port SRS, then the direct approach used for 4-port SRS is to map port {1000, 1002, 1004, 1006} and port {1001, 1003, 1005,1007} in two combs.

For 8-port SRS with comb 4, support 8 ports mapping on 4 cyclic shifts as as the following formula

For 8-port SRS with comb 4, support 8 ports mapping in 2 comb offsets as as the following formula

Come 8

For comb 8, only up to 6 cyclic shifts are available for SRS ports multiplexing. However, it is impossible to assign 6 cyclic shifts to 8 ports with an integer interval. Therefore, using 2 configurable cyclic shifts for 8-port SRS is a potential solution for this case.

The detailed values of cyclic shifts for comb 8 are shown in Table 3.

Cyclic shift mapping for SRS resource with 8 ports in comb 8

|  | Port 1000 | Port 1001 | Port 1002 | Port 1003 | Port 1004 | Port 1005 | Port 1006 | Port 1007 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS 0 | 0 | 0 | 0 | 0 | 3 | 3 | 3 | 3 |
| CS 1 | 1 | 1 | 1 | 1 | 4 | 4 | 4 | 4 |
| CS 2 | 2 | 2 | 2 | 2 | 5 | 5 | 5 | 5 |
| CS 3 | 3 | 3 | 3 | 3 | 0 | 0 | 0 | 0 |
| CS 4 | 4 | 4 | 4 | 4 | 1 | 1 | 1 | 1 |
| CS 5 | 5 | 5 | 5 | 5 | 2 | 2 | 2 | 2 |

Regarding the comb offset mapping,  8 ports mapped in 4 comb offsets is agreed in the last meeting. Therefore, mapping ports {1000, 1004}, ports {1001, 1005}, ports {1002, 1006}, and ports {1003,1007} in 4 comb offsets can be supported.

For 8-port SRS with comb 8, support 8 ports mapping on 2 cyclic shifts as the following formula

For 8-port SRS with comb 8, support 8 pors mapping in 4 comb offsets as as the following formula

8-port SRS resource on multiple symbols

Apart from cyclic shift and FDM based approach, 8-port SRS resource can also be enhanced in the time-domain, i.e., TDM based solutions, which has been agreed in the previous meeting.

Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports. - At least s = 2. - FFS: s = 4, s = 8. - m = 2,4,8,10,12,14, and m is a multiple of s. - Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol. - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports. - FFS: down selection from existing resource mapping designs - FFS: which subset of 8/s ports are mapped onto each OFDM symbol. - FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern: - Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols. - Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

The number of TDMed OFDM symbols

One remaining issue is whether other numbers of OFDM symbols should be supported in addition to the value of 2. From the perspective of SRS overhead in the time domain, 8 ports mapped on more OFDM symbols would affect the transmission of other uplink signals. Besides, how to be compatible with the legacy repetition R should be further considered. From the perspective of the number of ports, when 8 ports of one SRS resource is mapped on 2 OFDM symbol, 4 ports can be mapped in each OFDM symbol like two 4-port SRS resource. This case can provide enough multiplexing room in frequency and code domain for different SRS resources as legacy SRS provided on one OFDM symbol. Thus there is no need to support 4 and 8 OFDM symbols mapped on by one 8-port SRS resource.

Don’t support one 8-port SRS resource mapping on 4 or 8 OFDM symbols in TDM manner.

TDMed time-domain pattern

Additionally, how to map 8 ports in multiple OFDM symbols is another key issue. One potential way with minimal spec effort is to equally divide 8 ports into two parts on two adjacent OFDM symbols or four parts on four adjacent OFDM symbols. In this case, the port multiplexing principle for each comb value can be reused for 8-port SRS as much as possible.

Support to equally divide 8 ports into 2 parts mapping on 2 adjacent OFDM symbols.

Regarding the time-domain mapping with repetition, 8 ports should be adjacent as much as possible. Otherwise, due to the Doppler shifts especially in high speed, it would impact the channel estimation results of the first two ports on the first symbol and the last two ports on the last symbol, which would degrade the determination of TPMI and CQI. Therefore, Option 2-1 is a better mapping pattern, i.e, the s subsets of ports are mapped cyclically as {1, 2, …, s, 1, 2, …, s} on the m OFDM symbols.

For TDM pattern with repetition, support s subsets of 8 ports are mapped cyclically as {1, 2, …, s, 1, 2, …, s} on the m OFDM symbols.

Restrictions of TDMed mapping

After determining the OFDM symbols mapped on by one 8-port SRS resource, some additional restrictions should be considered to simplify the SRS generation and multiplexing. For instance, the comb offset, cyclic shift, transmit power, frequency hopping and so on should be aligned on the multiple OFDM symbols. For convenience, one feasible way is to restrict each part of SRS ports on different OFDM symbols with the same transmission parameters.

The transmission parameters such as comb value, cyclic shifts, transmit power, frequency hopping should be aligned on the multiple OFDM symbols mapped on by one 8-port SRS resource.

Conclusion

In summary, the following proposals are made for Rel-18 SRS enhancement.

- The additional benefit of the combination of hopping schemes is limited.

The tradeoff between randomization performance and UE/gNB complexity should be considered when determining the hopping granularity.

Enhancement on power control for SRS may be helpful for some UEs in TDD CJT, but would also cause more cross-SRS interference to other UEs.

- Cyclic shift hopping is not expected to be configured with comb offset hopping in Rel-18.

Both cyclic shift hopping and comb offset hopping are not expected to be configured with sequence hopping and group hopping.

From the perspective of UE/gNB complexity and randomization performance, support both per symbol and per R repetitions as hopping granularity for cyclic shift hopping and comb offset hopping, which can be configured by gNB.

Support sequence ID  as the UE-specific initialization factor for both cyclic shift hopping and comb offset hopping.

Support to reinitialize the hopping pattern per N frames for both cyclic shift hopping and comb offset hopping with initialization ID .

Support to handle the co-existing issue with legacy SRS resources by gNB implementation without specification, when cyclic shift hopping or comb offset hopping is enabled for Rel-18 SRS resources.

Support cyclic shift hopping with finer cyclic shift granularity in Rel-18.

Don’t support power control enhancement on SRS for TDD CJT.

For 8-port SRS with comb 2, support 8 ports mapping on 8 different cyclic shifts separately as the following formula

For 8-port SRS with comb 2, whether 8 ports mapping on 1 or 2 comb offsets based on the value of cyclic shiftconfigured by RRC as the following formula.

For 8-port SRS with comb 4, support 8 ports mapping on 4 cyclic shifts as as the following formula

For 8-port SRS with comb 4, support 8 ports mapping in 2 comb offsets as as the following formula

For 8-port SRS with comb 8, support 8 ports mapping on 2 cyclic shifts as the following formula

For 8-port SRS with comb 8, support 8 pors mapping in 4 comb offsets as as the following formula

Don’t support one 8-port SRS resource mapping on 4 or 8 OFDM symbols in TDM manner.

Support to equally divide 8 ports into 2 parts mapping on 2 adjacent OFDM symbols.

For TDM pattern with repetition, support s subsets of 8 ports are mapped cyclically as {1, 2, …, s, 1, 2, …, s} on the m OFDM symbols.

The transmission parameters such as comb value, cyclic shifts, transmit power, frequency hopping should be aligned on the multiple OFDM symbols mapped on by one 8-port SRS resource.

Annex

- Simulation assumptions

Link-level simulation assumptions

| Parameter | Value |
| --- | --- |
| Scenario | CJT with N_TRP = 2 |
| Carrier Frequency | 3.5 GHz |
| Subcarrier spacing | 30kHz |
| Channel Model | CDL-B |
| Delay spread | DS=300ns |
| UE velocity | 3km/h |
| Allocation bandwidth | 20MHz |
| BS antenna configuration | 64 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ |
| UE antenna configuration | 4 Rx: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| SRS configuration | 4 ports with comb 2 SRS periodicity = 20ms |
| SRS channel estimation | Realistic channel estimation |