3GPP TSG RAN WG1 #112bis-e                                                         R1-2302476

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:	Discussions on AI/ML framework

Agenda Item:	9.2.1

Document for:	Discussion and Decision

Introduction

At RAN1 #112, some agreements and conclusions have been made as in appendix D [1].

In this contribution, we further discuss the general aspects of AI/ML framework.

Functionality/model/applicability identification

In the last meeting, the following agreement was achieved.

Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model Agreement - AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Agreement - For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

In functionality-based LCM, functionality is the target object for AI/ML operations. Network does not need to touch the actually working AI/ML model, and UE does not need to report the model description to network, such as model structure.

As seen in the above agreement, based on the AI/ML-enabled feature report, AI/ML functionality could be reported from UE to network. Network would need the description of AI/ML functionality and some necessary information, e.g., what is the input needed and what is the output from the AI/ML functionality operation.

Other than functionality-based LCM, model identification also needs the AI/ML-enabled feature report, so that network can obtain the list of UE supported AI/ML-enabled features. Then two sides can identify the needed AI/ML models based on report of supported features.

AI/ML-enabled feature reporting is needed for both functionality identification and model identification and can follow legacy procedure.

Based on agreement from last meeting, the functionality should be static/semi-static features from 3GPP perspective, since functionality identification is based on 3GPP capability framework. Corresponding the functionality to scenario/site information would make the feature non-static since this scenario/site information needs more dynamic adaptation and current capability framework would not work. Thus, functionality identification requires the AI/ML capability to have good generalization performance and whether and how it can be applied to different cases need justification.

Based on evaluation from different use case/sub-use cases, and considering the high requirements on the nearly fixed AI/ML model, it would be hard to overcome the generalization problem, and then it is expected that this is not the typical case for AI/ML operation.

Functionality identification is suitable for cases where “static" AI/ML capability is implemented in the UE.

Functionality identification requires the AI/ML capability to have good generalization performance and whether and how it can be applied to different cases need justification.

In last meeting, there are a lot of discussions on applicable conditions. After several iterations, the following proposal has been proposed by FL.

[FL4] Proposal 5-8i: At least for UE-side models and UE-part of two-sided models, RAN1 to study - How to define and study a (set of) applicable conditions for functionalities/[models]. - Note: Applicable conditions may be used to enable development of scenario/configuration/[site]-specific models [and, if needed, report the models’ applicability to the Network]. - Whether and how to define performance targets (possibly as a part of applicable conditions) for functionality/[models] - Whether and how UE reports a (set of) applicable conditions for supported functionalities (and if needed, for supported models) and/or supported set of functionalities.

Applicable condition may contain the information of the suitable scenario/configuration/site of AI/ML model. For example, when the scenario/configuration/site is not contained in applicable condition of current AI/ML model but is contained in another AI/ML model, UE and network would realize that model switching may be needed when the UE or network enters such scenario/configuration/site.

Thus, alignment of applicable condition is needed for LCM. Similar to functionality identification and model identification, we can have a terminology of applicability identification at least for discussion purpose.

For the purpose of discussion on aligning applicable conditions between entities, define the terminology of “applicability identification” as following.

- Applicability identification: A process/method of identifying an AI/ML applicable condition for the common understanding between the NW and the UE.

  - Note: Information regarding the AI/ML applicable condition may be shared during applicability identification.

In the recent email discussion, “physical model” and “logical model” have been deeply discussed. “Physical model” would be a model that tangibly exists, e.g., model implementation (e.g., binary model file), model during training (e.g., source code model), model description during model transfer, etc. “Logical model” would be a model that is identified and used in signaling. A logical model may be described by an explicit dataset, e.g., dataset used for training. Also, logical model may be described by a condition the model has to satisfy, e.g., described by a functionality/procedure/condition.

The concept of logical model is similar to above applicability identification. LCM procedure based on logical model identification or applicability identification should be studied.

Study “logical model” identification or applicability identification based LCM procedures.

Applicable conditions may change during usage and then need to be updated. The wireless channel feature may change due to various factors, e.g. weather, temperature or new buildings, and then the scenario/configuration/site in the applicable conditions may be not suitable for the AI/ML model after a period of time. UE could monitor the performance of the AI/ML model and find new applicable conditions.

UE side servers or network entities can train new model or new model parameters according to newly collected data, and then send the new model or new model parameters to UE through model delivery/transfer. The applicable conditions of the new model or new model parameters would be different than current applicable conditions. Then UE should report the new applicable conditions to the network.

On the other hand, UE side servers and network entities may provide some assistance information for current UE-sided model, e.g., new applicable conditions for current models. UE side servers and network entities can continuously collect data from plenty of UE devices, time, and areas. Based on collected data, UE or network may suggest new applicable conditions for current models.

In the above case, a new applicability ID may be assigned or the applicable conditions are updated for an existing ID.

Consider applicable condition update for an existing model in “logical model” identification or applicability identification procedure.

Based on the above discussions, adding applicable conditions in functionality identification, a new identification category is generated, which is AI/ML-enabled feature with applicability or logical model identification. Since there is only dataset exchanging and no model delivery/transfer, separate training for two-sided model would be considered as a special identification category. Therefore, the procedures of functionality/model/applicability -based LCM are shown in the below figure. The details of each identification category would be discussed separately in the following.

Figure 2-1. The generally procedure of the model-based LCM.

Functionality identification for UE sided model developed by UE

Functionality-based LCM can be divided into two steps, which is based on UE capability report and typical RRC configuration framework.

- Step 1: UE AI/ML-enabled feature report to network. In functionality-based LCM, functionality is the target object for AI/ML operations. Network does not need to touch the actually working AI/ML model, and UE does not need to report the model description to network, such as model structure. Using AI/ML-enabled feature report, the supported functionality of UE can be reported to network. UE may have one or multiple AI/ML models for one functionality, and then UE selects the suitable AI/ML model according to current wireless conditions transparently from network. Necessary information for UE side operation can be exchanged based on request from both sides.

- Step 2: Based on UE report and continuously monitored wireless conditions, network controls LCM of the AI/ML based functionality. After receiving the UE report, network has the list of the supported AI/ML based functionality of UE. Typical RRC configurations can be used to enable/disable the functionalities.  Since there is no model ID model selection and model switching are not supported in the functionality-based LCM. The AI/ML functionality can be turned on/off based on monitoring procedures.

Functionality identification based LCM contains two steps as legacy UE features, UE capability report and RRC enabling/disabling procedures.

Logical model/Applicability identification for single sided model at UE side and developed by UE

The following is the procedure of AI/ML-enabled feature with applicability identification for UE developed UE-sided model. This procedure provides more flexibility than the functionality-based LCM and is expected to have better generalization performance, but does not touch the concept of “physical model”.

- Step 0: UE collects the data needed for training and UE trains the model. Alignment of necessary information. In this AI/ML-enabled feature with applicability identification, functionality and applicability are the basic unit of AI/ML operations. The “physical model” is not touched here. UE can collect the training data and then train the model. To ensure that the UE-sided model can work well at UE, network and UE or UE side server may need to align some necessary information, e.g., additional information can be provided from network side to facilitate UE training. Online alignment and offline alignment may both be needed.

- Step1: UE AI/ML-enabled feature report to network. AI/ML-enabled feature is reported by UE to network.

- Step2: Alignment of applicable conditions between network and UE. Unlike functionality-based LCM, applicable conditions are used in this procedure to improve the generalization performance. The applicable conditions could be sent from network to UE, during the model parameters delivery. Also, UE could report the applicable conditions of the received model, since UE may do some device specific optimization based on its own hardware environment and collected data.

- Step3: Using applicability ID or logical model ID, network can indicate or assist LCM, including model selection or switching, model activation/deactivation. Instead of functionality ID or model ID, applicability ID can be used for two sides to indicate the right “logical model”. Take performance monitoring as an example. The configuration or indication of performance monitoring measurement resource or report resource would include applicability ID, so that UE can measure and report the performance KPIs of the right “logical model”.

- Step4: Applicable conditions may be updated during usage. Due to the change of wireless channel conditions, the performance monitoring results, the new model or new model parameters and the assistance information from UE server, applicable conditions may be updated to facilitate the LCM of two-sided model.

Study LCM based on AI/ML-enabled feature with applicability identification, in which applicability ID is used for LCM.

Logical/Physical Model identification/Applicability Identification for UE sided model developed by network & Logical Model identification/Applicability Identification for two-sided model

Firstly, the following is the procedure of model/applicability identification for Type 1 training two-sided model and one-sided model transferred from network to UE.

- Step 0: Alignment of model structure. Network trains the model parameters based on collected data. Since UE chipset is designed and optimized for certain model structures, it is important of network to know the supported model structures at UE. Other necessary information may include quantization supported by the UE and inference latency for a specific model etc.  Based on the aligned model structure or other restrictions, the model parameters can be trained by network using the collected data.

- Step1: UE AI/ML-enabled feature report to network. AI/ML-enabled feature is reported by UE to network.

- Step2a: Network delivers the UE-sided model parameters to UE, with other necessary information. After alignment and model training, the UE-sided model can be delivered to UE. Model ID may be also delivered to UE as one part of model description information.

- Step2b: Alignment of applicable conditions between network and UE. Applicable conditions may contain the information of the suitable scenario/configuration/site of AI/ML model, to overcome the generalization problem and facilitate LCM. The applicable conditions could be sent from network to UE, and UE could report the applicable conditions of the received model if adjustment is needed.

- Step3: Using model ID or applicability ID, network controls LCM of the UE sided part of two sided models. One or both of model ID and applicability ID can be used for two sides to indicate the right model. Physical model ID or logical model ID or applicability ID can all be used for LCM.

- Step4: Applicable conditions may be updated during usage. Due to the change of wireless channel feature, the performance monitoring results, the new model or new model parameters and the assistance information from UE server, applicable conditions may be updated to facilitate the LCM of two-sided model.

Then the following is the procedure of model/applicability identification for Type 3 training two-sided model (separate training at two sides).

- Step0: Alignment of model structure, quantization. Network/UE exchange data needed, trains the two-sided model and align on model/applicability ID. As discussed above, to ensure the performance of separate training, network and UE or UE side server would also need to align some necessary model information, including model structure and quantization. After collecting enough training data samples, network/UE can train a two-sided model at one side and this side can generate the needed training data for other side, based on the aligned model structure, quantization or other restrictions. Applicable conditions may be sent along with the needed training data. Then the other side can train its part of the two-sided model. Model ID or applicability ID should be assigned to the exchanged training data, so that two sides can have alignment on the two-sided model.

- Step1: UE AI/ML-enabled feature report to network. AI/ML-enabled feature is reported by UE to network.

- Step2: Alignment of applicable conditions between network and UE. Both UE and network already have dataset ID list based on separating training in Step0. Different dataset IDs refer to different AI/ML models. Applicability ID or logical model ID can be used here for two sides to indicate the same dataset ID, so that the network-sided part and UE-side part could match each other. Moreover, since UE may do some device specific optimization based on its own hardware environment and new collected data, the applicable conditions may be exchanged or updated in this step.

- Step3: Using model ID or applicability ID, network can indicate or assist LCM, including model selection or switching. Model ID or applicability ID can be used for two sides to indicate the same two-sided model, and the LCM of this two-sided model can work correctly.

- Step4: Applicable conditions may be updated during usage. Due to the change of wireless channel, the performance monitoring results, the new model or new model parameters and the assistance information from UE server, applicable conditions may be updated to facilitate the LCM of two-sided model.

Consider to define the procedures as in Figure 2-1 for model/applicability identification.

Consideration on model identification types

As discussed in email discussion, we see the possibility of using all three procedures together to identify a model, e.g., Type A identifies the model structure offline, Type B reports the supported model structure, Type C identifies new model after model transfer.

- Type A: Model is identified offline to network (if applicable) and/or UE (if applicable). In Step0 in Figure 2-1 is belonging to this type. This is potentially air-interface transparent but not necessarily 3GPP transparent.

- Type B: Model is identified via signaling from UE to network. In Step1, UE AI/ML-enabled feature is report from UE to network. In Step2b and Step4, UE could report applicable conditions to network.

- Type C: Model is identified via signaling from network to UE. In Step2a, the model is delivered and identified from network to UE. In Step2b and Step4, applicable conditions could be sent from network to UE.

Consider the following model identification types.

- Type A (Step0 in Figure 2-1): Model is identified offline to network (if applicable) and/or UE (if applicable).

- Type B (Step1, Step2b and Step4 in Figure 2-1): Model is identified via signaling from UE to network.

- Type C (Step2a, Step2b and Step4 in Figure 2-1): Model is identified via signaling from network to UE.

Reference model structure and RAN4 related aspects

The challenges of RAN4 requirement definition and testability procedure definition aspects lie in the following:

- Multi-vendor (network vendor, UE vendor and TE vendor) involvement in two sided cases.  The candidate sub-use case is mainly CSI compression. In RAN1 discussion, there are two training types defined for CSI compression: type 1 training and type 3 training. For type 1 training, the network side trained a model offline and update the model of the other side using model transfer/delivery. For type 3 training, the network side and UE side use separate training to obtain models on each side. With TE vendor involvement, how the three parties are involved together for testing procedure need to be considered carefully.

- Generalization performance definition. Non-AI algorithms are based on communication theories and have physical/interpretable meanings. Their performances are robust and predictable to some extent. AI/ML algorithms are based on machine learning and have weak physical meanings. If the scenarios for test is different from the scenario where the training data is generated, the performance would degrade. The channel conditions of real environment are complex and diversified. It is necessary in RAN4 to discuss whether and how to test the generalization performance of AI/ML algorithms.

- Performance requirement need to be defined to justify AI/ML gains. As a continuously evolving technology and also paving the way for future, performance requirement of the AI/ML based operations need to be defined to justify the efforts from industry.

- Performance of different models would be different, especially considering complicated training types. We show below one case for RAN1 type 3 training. It could be seen that with different model assumption, the performance could be worse than the legacy codebook. Thus, alignment between different parties on the model structure is needed for definition of performance requirement.

- Table 3-1: The performance of separate training with different model assumption.

|  | - Baseline0: Joint training of Encoder0 and Decoder @ NW | - Separate training of perfectly aligned Encoder0 | - Separate training of misaligned CNN Encoder2 | - Legacy codebook |
| --- | --- | --- | --- | --- |
| - Test SGCS | - 0.878 | - 0.870 | - 0.756 | - 0.795 |

The following challenges should be addressed for RAN4 related aspects:

- Multi-vendor (network vendor, UE vendor and TE vendor) involvement in two sided cases.

- Generalization performance definition.

- Performance requirement need to be defined to justify AI/ML gains.

- Performance of different models would be different, especially considering complicated training types.

To address above challenges, one possible way is to define a reference model structure that is widely used in the industry and RAN1 evaluation discussion, e.g., fully connected, CNN and/or transformer.

- Reference model structure can address the concerns due to the following aspects:

  - With reference model structure, multiple vendors could be easily involved for both training type 1 and type 3. With training type1, both network vendor and TE vendor can update the model parameters of UE vendors and test the corresponding performance of paired models. For type 3 training, TE vendors could also take into account of reference model structure of the other side to properly train its own models.

  - With reference model structure, performance requirement can be defined based on the agreed structure. The issues regarding different models having different performance can be resolved. Based on RAN1 evaluations and future RAN4 defined testing conditions, the corresponding performance of the reference model structure can be defined.

  - The generalization performance of reference model structure can also be resolved for two sided case: for type 1 training, the parameters of the reference model can be updated to fit into local environments. For type 3 training, different data sets can be used to separately train different models for use considering the reference model structure. For one sided case, the generalization issue can also be resolved with update of model parameters from the other side. The only exception is UE sided model without parameters from the other side, discussion is needed on whether and how to test generalization for this case.

- Without reference model structure:

  - It would be difficult to define performance requirement which can justify AI/ML gains.

  - It would be difficult for TE vendors to involve in type 1 and type 3 training procedure.

There are some concerns that may also arise for defining reference model structure. We provide our understanding as below:

- Whether the inference performance would be different if the tested parameters are different from those used in field considering e.g., due to different quantization at UE side:

  - We did some experiments as below showing that no matter the quantization level is 32bits/16bits/8bits, the performance difference is very small. Models are trained on FP32 and quantized for inference. Encoder and decoder are both quantized.

Table 3-2: The performance of different quantization levels for CSI compression.

|  | MLP model | CNN model | Transformer model |
| --- | --- | --- | --- |
| SGCS of FP32 quantization inference | 0.9421 | 0.9493 | 0.9581 |
| SGCS of FP16 quantization inference | 0.9421 | 0.9493 | 0.9581 |
| SGCS of int8 quantization inference | 0.9413 | 0.9486 | 0.9573 |

- Whether additional UE compilation is needed with flexible model parameters update:

  - It is widely used already in today’s implementation for UE to do on device parameter update for a specific model structure. Since the model structure is already fully compiled in a way with flexible model update, there is no additional concerns on power consumption related aspects.

  - On device quantization may be needed if the two sides have different understanding. This is also not an issue from UE side due to the following:

    - On the one hand, as shown above, different quantization levels have almost the same inference performance. UE would choose the most feasible way to do the quantization, e.g., by directly quantize the updated 32bits-quantized parameters with fixed 8bits-quantized parameters for inference. This additional effort would still be within feasibility region in typical implementation even with today’s chipset.

    - On the other hand, specification on quantization would be another way to resolve this issue.

- Whether the performance would be different due to device specific implementations for performance requirement definition.

  - We also did some evaluations to see the impact of antenna imbalance and antenna spacing. In Table 2-4, each UE antenna has a different amplitude and phase disturbance, where the power factor of the first antenna is 1 and different random power attenuations are set on other antennas. It could be seen from the performance of different spacings and antenna imbalance is negligible. It is not necessary for the network to maintain different models for different devices.

Table 3-3: The performance of different antenna spacing for CSI compression.

| - Antenna spacing | - 0.3 wave length | - 0.5 wave length | - 0.8 wave length |
| --- | --- | --- | --- |
| - Model performance | - 0.8721 | - 0.8793 | 0.8766 |

Table 3-4: The performance of antenna imbalance for CSI compression.

| Power factor (dB) | Type II | AI |
| --- | --- | --- |
| 0.0 | 0.8319 | 0.8793 |
| 0.5 | 0.8320 | 0.8793 |
| 1.0 | 0.8317 | 0.8791 |
| 2.0 | 0.8312 | 0.8784 |
| 3.0 | 0.8302 | 0.8775 |

- Whether this would expose proprietary processing.

  - From model proprietaries perspective, fully connected or CNN is widely used in the industry for decades. There are no model proprietaries issues for model itself. Other models like Transformer is also widely used and model itself also does not have proprietary issues.

  - On the other hand, it is not prohibitive that UE or network can implement other models for processing outside the minimum requirement perspective. Proprietary processing can anyway be up to vendor’s implementations.

Base on above analysis, we have the following proposal

Consider to define reference model structure in RAN4 or RAN1.

Zone/site specific model v.s. generic model development for two sided cases

As agreed, there are three methods to fight against the AI/ML generalization problem across different scenarios/configurations/sites, i.e., model generalization, model switching and model update. Generic model development is corresponding to model generalization where one model is deployed for different zones/scenarios, while model switching and model update is corresponding to development of zone-specific model with different models or parameters used in different zones.

In generic model development, network or UE collects a large number of training data from various scenarios/configurations/sites. Post-processing of the collected data is needed, such as noise filtering. According to our contribution [3], the ratio of the collected data from different scenarios/configurations/sites has considerable impacts on the AI/ML performance, and then this ratio may be adjusted to match the practical wireless environment. And then the AI/ML model is trained and deployed to network or UE. The generic model would be of high complexity and large storage overhead, to fit the different channel characteristic of various scenarios/configurations/sites. This generic model development can be based on either level y or level z. In level y, joint training or separate training can be used for two-sided model. In joint training or separate training, if there is no data sharing between the network vendor and UE vendor, the AI/ML model is trained by the data collected from one side, and then the AI/ML performance may be decreased. In level z, the trained model can be transferred from network to UE, since network can collect data from various UEs all the time and the AI/ML model is likely to be well trained.

In zone specific model deployment, network or UE collects training data from the target zone. Since the wireless channel condition is much simpler in this case, the number of collected training data would be much smaller and the AI/ML model may be simple and small, which has been verified in the following subsections. Also, the ratio adjustment of the collected data from different scenarios/configurations/sites would be not needed.

Compared with zone/site specific models, generic model applicable across multiple areas would typically have larger computation complexity and storage overhead.

In collaboration level y, UE needs to store a large number of AI/ML models. When the scenario/configuration/sites change, UE can select the proper AI/ML model among a group of models. If there are N1 scenarios, N2 configurations and N3 sites, UE needs to store N1*N2*N3 AI/ML models for just one functionality. The storage overhead may be unaffordable. Model updating is another option to improve the AI/ML generalization performance. If the AI/ML model is not suitable for current scenario/configuration/site, UE can collect training data by itself or network. Using the collected data, UE can update its AI/ML model. It is clear that the data collecting needs extra time and storage. The training procedure needs extra computation resource and time.

In collaboration level z, using model transfer, UE only needs to use the AI/ML model transferred from network. When current AI/ML model is no longer suitable, for both the new zone case and the channel condition changing case, the new model can be transferred to UE.

Collaboration level z can be used to facilitate zone specific model deploy.

Previously it has already been agreed to study zone/site specific model performance. In this section, we had more study on data collected from field and ray tracing to evaluate the performance of zone/site specific model.

Agreement Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models. Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side. Agreement Study various approaches for achieving good performance across different scenarios/configurations/sites, including - Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites - Model switching, i.e., switching among a group of models where each model is for a particular scenario/configuration/site - [Models in a group of models may have varying model structures, share a common model structure, or partially share a common sub-structure. Models in a group of models may have different input/output format and/or different pre-/post-processing.] - Model update, i.e., using one model whose parameters are flexibly updated as the scenario/configuration/site that the device experiences changes over time. Fine-tuning is one example.

Some initial results for field test

We provide some initial results for field test of CSI compression. The data is collected from actual 5G network and the collecting area is about 400m * 350m. About outdoor 50000~100000 samples per area or cell are collected. The detailed parameters are provided in Table 4.1-1.

Table 4.1-1: Parameters of field test of CSI compression.

| Parameters | Value |
| --- | --- |
| Scenario | Actual 5G network, about 400m * 350m collecting area. About outdoor 50000~100000 samples per area or cell. |
| Carrier frequency | 3.45GHz |
| Subcarrier spacing | 30KHz |
| Frequency resources | 52 RBs, 13 subbands for 4RBs per subband |
| Rank | Max rank 1 |
| gNB antenna | 8 antenna ports |
| UE antenna | 4 antenna ports |
| CSI payload | 167/58 bits payload |

Field test result of different areas

There are 3 data collecting areas. Area B is the main road of the industrial park, with many tall trees and cars along the road. Area C is the road behind several buildings. Area D is the indoor scenario in a building. UE in the left part of the industrial park usually accesses to a different cell, compared with the right part of the industrial park. So, we focus on the right part of the industrial park and current areas are chosen.

Figure 4.1-1: The map of data collecting areas.

The SGCS results of eType II codebook and multiple AI/ML models are provide in the following. In Table 4.1-2 and 4.1-3, the AI/ML models are trained by the data in each area separately, and multiple AI/ML models are used. 167 bits overhead is used in Table 4.1-2 and 58 bits overhead is used in Table 4.1-3. In Table 4.1-4 and 4.1-5 only one hidden layer full-connected encoder is used and it is trained by the data of all 4 areas, with 167 bits overhead and 58 bits overhead separately.

It is seen that the performance gaps between different AI/ML models are small. Even one layer MLP encoder can provide good performance, which is very simple and small. With much higher complexity, Transformer encoder has better performance than one layer MLP encoder, but the performance gain is small Area B. Considering the performance and complexity, simple model structure, e.g., one hidden layer full-connected encoder, is good enough for typical per single cell or multiple cell operations.

Table 4.1-2: The SGCS results of multiple AI/ML models trained by the data in each area separately, with 167 bits overhead.

| 167 bits overhead | eTypeII | AI with an area specific model (One layer MLP encoder) ~67kB | AI with an area specific model (small CNN encoder) ~250kB | AI with an area specific model (Transformer encoder) ~3.6MB |
| --- | --- | --- | --- | --- |
| Area B | 0.8429 | 0.9217 | 0.929 | 0.9406 |
| Area C | 0.7871 | 0.898 | 0.9037 | 0.9116 |
| Area D | 0.8489 | 0.9315 | 0.9323 | 0.9423 |

Table 4.1-3: The SGCS results of multiple AI/ML models trained by the data in each area separately, with 58 bits overhead.

| 58 bits overhead | eTypeII | AI with an area specific model (One layer MLP encoder) ~30kB | AI with an area specific model (small CNN encoder) ~213kB | AI with an area specific model (Transformer encoder) ~3.3MB |
| --- | --- | --- | --- | --- |
| Area B | 0.7290 | 0.8573 | 0.8725 | 0.8868 |
| Area C | 0.6438 | 0.8015 | 0.8162 | 0.8389 |
| Area D | 0.6853 | 0.8701 | 0.8814 | 0.8873 |

Table 4.1-4: The SGCS results of one hidden layer full-connected encoder trained by the data of all 4 areas, with 167 bits overhead.

| 167 bits overhead | Training SGCS on data from all areas | Testing SGCS in area B | Testing SGCS in area C | Testing SGCS in area D |
| --- | --- | --- | --- | --- |
| One hidden layer full-connected encoder~67kB | 0.9055 | 0.905 | 0.8799 | 0.8959 |

Table 4.1-5: The SGCS results of one hidden layer full-connected encoder trained by the data of all 4 areas, with 58 bits overhead.

| 58 bits overhead | Training SGCS on data from all areas | Testing SGCS in area B | Testing SGCS in area C | Testing SGCS in area D |
| --- | --- | --- | --- | --- |
| One hidden layer full-connected encoder~30kB | 0.8184 | 0.8201 | 0.7592 | 0.7958 |

From initial results for field test, performance of simple model structure, e.g., one hidden layer full-connected encoder, is good enough for typical zone/site specific optimization.

Field test result of different physical cells

The performance of different physical cells is analyzed in the following. We have tested the coverage of different cells in the industrial park, according to the measured RSRP, RSRQ and SINR. The coverage areas of two typical cells in the industrial park are shown in the below figure.

Figure 4.1-2: The map of data collecting cells.

For cell 1, data samples are collected by different days. The data collection routes in different days have some differences, which results in the different wireless channel features in different days.

The SGCS results of eType II codebook and multiple AI/ML models are provide in the following. In below tables, the AI/ML models are trained by the data in each cell separately, and multiple AI/ML models are used. For cell 1, A large combination of data collected on different days has been used. 167 bits overhead is used. One layer MLP encoder is used in Table 4.1-6, small CNN encoder is used in Table 4.1-7 and Transformer encoder is used Table 4.1-8.

It is seen that using the data collected on various days or routes, the generalization problem of different days or routes could be solved. For example, using Cell 1 data collected on 2.28 as the test data set, the AI model trained using Cell 1 data collected on various day except 2.28 and 2.16, has the nearly the same performance as the AI model trained using Cell 1 data collected on 2.28.

In addition, the AI model trained by Cell 1 data has poor performance on Cell 2 data, which is even worse than eType II.

Table 4.1-6: The SGCS results of multiple AI/ML models trained by the data in each cell separately, with 167 bits overhead and one layer MLP encoder.

|  | eType II | AI model using Cell 1 data, collected on 2.3, 2.10, 2.14, 2.27, 3.2, 3.9 | AI model using Cell 1 data, collected on 2.28 | AI model using Cell 1 data, collected on 2.16 | AI model using Cell 2 data |
| --- | --- | --- | --- | --- | --- |
| Cell 1 data, collected on 2.28 | 0.8137 | 0.8546 | 0.8680 | \ | \ |
| Cell 1 data, collected on 2.16 | 0.8425 | 0.8746 | \ | 0.8952 | \ |
| Cell 2 data | 0.8145 | 0.7832 | \ | \ | 0.8974 |

Table 4.1-7: The SGCS results of multiple AI/ML models trained by the data in each cell separately, with 167 bits overhead and small CNN encoder.

|  | eType II | AI model using Cell 1 data, collected on 2.3, 2.10, 2.14, 2.27, 3.2, 3.9 | AI model using Cell 1 data, collected on 2.28 | AI model using Cell 1 data, collected on 2.16 | AI model using Cell 2 data |
| --- | --- | --- | --- | --- | --- |
| Cell 1 data, collected on 2.28 | 0.8137 | 0.870 | 0.876 | \ | \ |
| Cell 1 data, collected on 2.16 | 0.8425 | 0.888 | \ | 0.907 | \ |
| Cell 2 data | 0.8145 | 0.8099 | \ | \ | 0.9044 |

Table 4.1-8: The SGCS results of multiple AI/ML models trained by the data in each cell separately, with 167 bits overhead and Transformer encoder.

|  | eType II | AI model using Cell 1 data, collected on 2.3, 2.10, 2.14, 2.27, 3.2, 3.9 | AI model using Cell 1 data, collected on 2.28 | AI model using Cell 1 data, collected on 2.16 | AI model using Cell 2 data |
| --- | --- | --- | --- | --- | --- |
| Cell 1 data, collected on 2.28 | 0.8137 | 0.8847 | 0.8934 | \ | \ |
| Cell 1 data, collected on 2.16 | 0.8425 | 0.8998 | \ | 0.9160 | \ |
| Cell 2 data | 0.8145 | 0.8404 | \ | \ | 0.9172 |

From initial results for field test, the model developed for Cell 1 shows robust performance for different moving routes.

Field test shows that model developed for Cell 1 does not perform well for Cell 2.

Field test shows that simple and small models work well for all different cases, at least for typical cell coverage.

Some initial results for spatial consistency data

Here we consider using data where the channel has spatial consistency characteristics. Each UE generates random variables with spatial consistency based on its own geographic location at the T=0, both the cluster specific random variables and the correlation distance for spatial consistency procedure a follow 38.901. The detailed parameters are provided in Table 4.2-1.

Table 4.2-1: Parameters of spatial consistency data of CSI compression.

| Parameters | Value |
| --- | --- |
| Scenario | UMa |
| Channel model | Uma 38.901 with spatial consistency |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 30KHz |
| Frequency resources | 52 RBs, 13 subbands for 4RBs per subband |
| Rank | Max rank 1 |
| gNB antenna | 32 antenna ports [Mg Ng P M N, Mp Np] = [1 1 2 8 8, 2 8] dH,dV= (0.8, 0.5) λ, +45°/-45° polarization |
| UE antenna | 2 antenna ports [Mg Ng P M N, Mp Np] = [1 1 2 1 1, 1 1] dH,dV= (0.8, 0.5) λ, 0°/+90° polarization |
| BS receiver noise figure | 10 |
| UE receiver noise figure | 7 |
| UE distribution | 100% outdoor |
| UE speed | 30km/h |
| Mechanic tilt | 180° in GCS (pointing to the ground) |
| Beam set at TRxP | Azimuth angle φi = [0], Zenith angle θj = [102]. |
| UE beam set | Azimuth angle φi = [0], Zenith angle θj = [90] |

Cell specific model is considered and then different AI/ML models are used for different cells. Simple AI/ML model, which is a one layer MLP encoder, and complex transformer encoder are evaluated in this simulation. It is seen that the performance of simple AI/ML model is similar to that of complex AI/ML model, similar as observed in field test results. Considering the performance and complexity, simple model structure, e.g., one hidden layer full-connected encoder, is good enough for typical per single cell or multiple cell operations.

Table 4.2-2: The SGCS results of multiple AI/ML models trained by the data in each area separately.

|  | AI with a cell specific model (One layer MLP encoder) ~285kB | AI with a cell specific model (Transformer encoder) ~4.08MB |
| --- | --- | --- |
| Cell 0 | 0.8345 | 0.8895 |
| Cell 1 | 0.8815 | 0.9168 |
| Cell 2 | 0.9132 | 0.9412 |
| Cell 3 | 0.9148 | 0.9439 |
| Cell 4 | 0.8718 | 0.9049 |
| Cell 5 | 0.9076 | 0.9380 |
| Cell 6 | 0.8698 | 0.9072 |

From initial results for spatial consistency data, performance of simple model structure, e.g., one hidden layer full-connected encoder, can achieve good performance for zone/site specific operations.

Some initial results for ray tracing data

To better compare the performance of AI/ML model in level y and level z, we utilize a typical ray-tracing channel model [2] in our experiment.  The outdoor scenario map [2] is plotted in Figure 4.3-1.

- Figure 4.3-1: Ray tracing map.

Specifically, we collect the channel from BS3 to UEs in user grid 1 (nearly LoS scenario) and user grid 2 (nearly NLoS scenario) respectively and all channels (~50,000 samples) in one experiment are collected in an area of 100m*35m, which is similar to a cell range. Other parameters with regarding to ray tracing could be referred to the official website [2]. The initial results are presented in Table 4.3-1.

- Table 4.3-1: Results for per-cell (region) model in CSI compression.

|  | SGCS of General model*,** | SGCS of per-cell model with Transformer structure** | SGCS of per-cell model with one-layer fc structure** |
| --- | --- | --- | --- |
| User grid 1 (LoS) | 0.841 | >0.99 | >0.99 |
| User grid 2 (NLoS) | 0.795 | >0.99 | >0.99 |

*General model is trained on channel data (~600,000 samples) collected from 21 cells generated from 38.901 model.

**More simulation parameters: carrier frequency 3.5GHz, subcarrier spacing 15KHz, 13 subbands (10MHz, 4RBs/subband), 32 gNB antenna ([Mg Ng M N P; Mp Np] = [1 1 8 8 2; 2 8]), 4 UE antennas ([Mg Ng M N P; Mp Np] = [1 1 1 2 2; 1 2]), horizontal beam sweeping along x-axis, vertical beam sweeping along z-axis, 180bits payload.

Following observations are drawn on per-cell (region) model:

From initial results for ray tracing based data generated with the map provided in [2], performance of simple model structure, e.g., one hidden layer full-connected encoder, can achieve good performance for zone/site specific operations.

Summary for comparison between zone/site specific models and generic models

In this section, various evaluation results and analysis have been done for zone/site specific models and generic models, including field test, spatial consistency data and ray tracing data. To fight against the AI/ML generalization problem across different scenarios/configurations/sites, generic model applicable across multiple areas would typically have larger computation complexity and storage overhead, while zone/site specific models would need simple model structure and small model size. For example, one hidden layer full-connected encoder is good enough for typical zone/site specific optimization.

Using the model transfer aided model switching, UE can achieve the benefits of zone/site specific models and does not need to store so many AI/ML models. If UE enters into a new scenario/configuration/site and does not have a proper AI/ML model, network can transfer a suitable AI/ML model to UE, which may be small and simple. The cost of storage, computation resource and time would be acceptable.

For separate training based operations, the model is also expected be updated in a long term which have similar shortcomings as mentioned above for generic models. Another issue is related to performance degradation due to single(multiple) reconstruction part paired with multiple(single) construction parts.

To fight against the AI/ML generalization problem, generic model would typically have larger computation complexity and storage overhead, while zone/site specific models would need simple model structure and small model size.

Model transfer considerations and specification impact

In RAN1#110b, the following agreement was achieved.

Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z Clarifying note: Level y includes cases without model delivery.

In this section, we will introduce model transfer considerations and specification impact.

Necessity of model delivery/transfer

In this subsection, the necessity of model deliver/transfer is analyzed.

Flexibility for model update

Model update is needed since AI/ML model would be outdated for various reasons.

- The change of UE hardware. For example, the phone case is widely used and different types of phone case would have different impacts on the UE signal reception. During the usage of UE device, the performance of UE antenna module would also change over time.

- Updated AI/ML model with better performance. Network and UE would collect data samples during daily usage. With the newly collected data samples, the parameters of the AI/ML model would be finetuned to achieve better performance. Moreover, new model structure would be designed to reduce the complexity and improve the performance.

- New AI/ML functionality. The original AI/ML models on the device may not cover all the AI/ML functionalities. New AI/ML functionalities would be needed to improve the system performance, such as channel prediction for higher speed, channel compression for larger PMI overhead, beam prediction for new beam pattern.

- The change of wireless conditions. The AI/ML model is trained based on the data samples from certain areas and certain time. As time goes by, the environment is always changing, such as the increasing or decreasing of the buildings, or the change of weather.

Model delivery/transfer can update the AI/ML model flexibly with small overhead and delay. Model updating or finetuning may be also used, but it would need extra computation resource and large delay due to the training procedure, extra time and storage due to data collecting.

Per cell or area optimization

As shown in Section 3, different cells or areas have different wireless channel characteristics. Per cell or area model is able to achieve good performance, with simple model structures, such as one hidden layer full-connected model or small CNN model. With model delivery/transfer, network can store the AI/ML model optimized for certain cells or areas and deliver it to UE.

Model switching or model updating is hard to achieve the gain of per cell or area optimization. If model switching is used, UE needs to store a large number of AI/ML models. Since there would be large number of cells or areas, the storage overhead may be unaffordable. Then consider model updating. If the AI/ML model is not suitable for current scenario/configuration/site, UE can collect training data by itself or network and then update its AI/ML model. It is clear that the data collecting needs extra time and storage, and the training procedure needs extra computation resource and time.

Without per cell or area optimization, the generic model is needed, which is generalizable to different cells or areas. The generic model would be of high complexity and large storage overhead, to fit the different channel characteristic of various scenarios/configurations/sites.

Model delivery/transfer is necessary due to flexibility for model update and per cell or area optimization.

Comparison between open format and proprietary format

In the following, we share our further views on the comparison between open format and proprietary format.

Proprietary information disclosure across vendors

Little proprietary information is disclosed across vendors when proprietary format is used. The model structure is disclosed in open format, but if widely-known model (e.g., fully connected or CNN) is used, this issue does not exist.

Flexibility for model update and per cell or area optimization

Open format can support flexible model update and optimize the model per cell or area, while proprietary format would need the network to store all the non-recognizable image files at the network side, or UE side would need to store large number of image files.

If open format is used, network can collect the data from nearby cells, optimize its AI/ML model and deliver the optimized AI/ML model to UE. It has been shown in Section 3 that per cell or area model could achieve good performance with simple model structures. It is noted that the same AI/ML model in open format can be optimized by network for multiple UE vendors. Moreover, UE only needs to store small number of models, since the AI/ML model can be easily delivered from network with small deliver delay.

If proprietary format is used, the AI/ML model is non-recognizable image file for network and network could not optimize the AI/ML model for UE. UE would need to store large number of image files to achieve the gain from per cell or area optimization. Moreover, different UE vendors would use different proprietary formats, and then network needs to store all these non-recognizable image files.

Offline co-engineering efforts and feasibility for deployment involving multiple vendors

Delivery model with proprietary format requires offline consensus among multiple vendors on how to use a proprietary file which is non-feasible in most cases. The performance of AI/ML model with proprietary format needs be tested offline across multiple vendors, to ensure that the AI/ML performance meet requirements.

For open format, such offline co-engineering may not be needed since the vendors responsible for the development are well aware of how to use the developed models. As long as the other side can conduct the corresponding operation required by the open format model, the corresponding functionality would be achieved without any offline efforts.

For proprietary format, multiple UE vendors use their own AI/ML models, and network delivers the corresponding AI/ML model to the UE according to its vendor. For open format, multiple UE vendors can use different AI/ML models or same AI/ML model. The storage overhead of open format would be lower than that of proprietary format. Moreover, as discussed above, for cases involving testing equipment vendors, it would not be feasible to design test cases with proprietary format thus non-feasible for deployment involving multiple vendors.

Device capability and delay for compiling, updating and running the model

For parameter-only updating with properly aligned information, no additional compiling is needed. If flexible model structure updating is also involved, it would lead to extra delay or extra device capability.

For executable images in proprietary format, UE does not need to compile the model format. For parameter-only updating, it is also not needed for the device to compile the model with properly aligned information, and then the overhead will be smaller than proprietary format. If the updated model parameters are float-point, UE would need to transform the model parameters into fixed-point. Moreover, UE may compress the AI/ML model into a smaller and simpler model. If network tries its best to optimize the AI/ML model using simple model structures, UE would not need to do the further compression. Model structure updating of open format may need the compiling of the updated AI/ML model, and the overhead will be large than proprietary format and parameter-only updating.

Another aspect is related whether additional offline effort is needed for model updating. For cases where the model is developed at the network side and then deployed at the UE side, the network can deliver the updated model to UE flexibly. For proprietary format, the model delivery would need additional offline effort to change it to executable images.

AI/ML model with proprietary format is usually optimized according to UE hardware implementation. UE hardware would optimize part of AI/ML model structures, such as full-connected layers and convolutional layers. If AI/ML model with open format contains some other AI/ML model structures which are not optimized in UE hardware, the UE hardware efficiency for AI/ML model with open format would be lower than that with proprietary format. However, if AI/ML model with open format only contains simple model structures or with parameter-only update, its hardware efficiency would be the same as to proprietary format.

The model structure is disclosed in open format, but if widely-known model (e.g., fully connected or CNN) is used, this issue does not exist.

One advantage of open format is flexibility for model update and per cell or area optimization. Open format can support flexible model update and optimize the model per cell or area, while proprietary format would need the network to store all the non-recognizable image files at the network side, or UE side would need to store large number of image files.

Delivery model with proprietary format requires offline consensus among multiple vendors on how to use a proprietary file which is non-feasible in most cases. Moreover, for proprietary format, network has to prepare multiple models for different UE vendors.

For parameter-only updating with properly aligned information, no additional compiling is needed. If flexible model structure update is also involved, it would lead to extra delay or extra device capability.

Study open format with widely-used model structures and parameter-only updating, which can obtain the most gain of open format and proprietary format.

- Using widely-used model structures to avoid the concern of disclosing model structure.

- Using parameter-only updating to avoid additional compiling at UE side.

Comparison between Cases y, z1 to z5 for model delivery/transfer

In RAN1#112, the following agreement was achieved.

Agreement To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top; Outside 3gpp Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format; 3GPP Network; NW-side. z3; model transfer in open format; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE; 3GPP Network; NW-side. Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. Note: The Case definition is NOT intended to introduce sub-levels of Level z. Note: Other cases may be included further upon interest from companies. FFS: Z4 and Z5 boundary

The below figure illustrates the procedures of Cases y, z1 to z5 for model delivery/transfer.

Figure 5.3-1: The procedures of Cases y, z1 to z5 for model delivery/transfer.

The pros and cons of all cases are discussed in the following.

- Flexibility for model update and per cell or area optimization: To flexible update the AI/ML model and optimize the AI/ML per cell or area, the AI/ML model should be stored at network side. Then, when UE enters a new cell or area, network can flexibly deliver/transfer the model to UE. Only in Case y, the AI/ML model is stored at UE and can not be flexibly updated.

- Offline co-engineering efforts and feasibility for deployment involving multiple vendors: Delivery model with proprietary format requires offline consensus among multiple vendors on how to use a proprietary file which is non-feasible in most cases. Then Case y, z1 and z2 need large offline co-engineering efforts. For open format, such offline co-engineering may not be needed since the vendors responsible for the development are well aware of how to use the developed models. For Case z4, the model structure would be aligned offline and then medium offline co-engineering efforts are needed. For Case z3 and z5, small offline co-engineering efforts are needed.

- Device capability and delay for compiling, updating and running the model: The model with proprietary format has been compiled before delivery/transfer. For parameter-only updating with known model structure, it is also not needed for the device to compile the model with properly aligned information. Low device capability and small delay are needed for Case y, z1, z2 and z4. For other cases, high device capability and large delay would be needed.

- Participation in model optimization: If the model is trained and compiled in one side with proprietary format, only the one side can participate in model optimization. Otherwise, both two sides have the possibility to optimize the AI/ML model. Since the data samples and AI/ML technology of one side would be limited, it is beneficial to involve both two sides in model optimization.

In sum, the following table shows the pros and cons of Cases y, z1 to z5 for model delivery/transfer.

Table 5.3-1: The pros and cons of Cases y, z1 to z5 for model delivery/transfer.

|  | Flexibility for model update and per cell or area optimization | Offline co-engineering efforts and feasibility for deployment involving multiple vendors | Device capability and delay for compiling, updating and running the model | Participation in model optimization |
| --- | --- | --- | --- | --- |
| Case y | Low | Large offline co-engineering efforts | Low device capability, small delay | Only UE side |
| Case z1 | High | Large offline co-engineering efforts | Low device capability, small delay | Only UE side |
| Case z2 | High | Large offline co-engineering efforts | Low device capability, small delay | Both UE side and network side |
| Case z3 | High | Small offline co-engineering efforts | Dependent on whether the model structure is known at UE | Both UE side and network side |
| Case z4 | High | Small offline co-engineering efforts | Low device capability, small delay | Both UE side and network side |
| Case z5 | High | Small offline co-engineering efforts | High device capability, large delay | Both UE side and network side |

Then we have the following proposal.

Use pros and cons in Table 5.3-1 for the model transfer/delivery discussion.

Model structures in model transfer

It is agreed in last meeting that 3GPP signaling is used for model transfer. There are two options of 3GPP signaling for model transfer. One is control plane based solution, and the other is user plane based solution.  In CP-based solution, model transfer is over SRB, and is point-to-point between UE and RAN node/CN entity. In UP-based solution, model transfer is over DRB, and is point-to-point between UE and server via UPF.

If arbitrary AI/ML is transferred from network to UE, there is possibility that UE can not compile this AI/ML model into executable format, since some complexity AI/ML structures or new AI/ML techniques may be not supported by UE. UE can only compile certain AI/ML structures. It is mentioned above that update of model parameters is already supported by current typical chipset implementations. Model structure can be aligned between companies to update parameters only.

From current simulation results and field test results in Section 3 and our other contributions [3], it is seen that simple model structures, such as full-connected network or CNN, are good enough for typical per single cell or multiple cell operations.

Performance of simple model structures, such as full-connected layers or convolutional layers, are good enough for typical per single cell or multiple cell operations.

Another concern in model transfer is the model proprietorship. There are two key aspects of AI/ML models, where one aspect is model structure and the other aspect is model parameters. If full-connected layers or convolutional layers are used, the privacy of the model structure is not so important since these models have been widely used for a lot of areas. Thus, simple model structures also have low proprietorship risk for model transfer.

The models currently evaluated by companies, such as those with full-connected layers or convolutional layers, are widely used for decades and have low proprietorship risk for model transfer.

Model transfer format for model description

Another important spec impact of model transfer is the model delivery format. There are many options for public format, some of which are listed in the following.

- AI/ML model public format coordinated by the two sides：Current AI/ML frameworks chosen by two sides. Currently, there are many AI/ML frameworks, such as TensorFlow, PyTorch and Caffe. Two sides can choose one of them to describe the AI/ML model. Interoperability token defined by SA2 is a good solution. The interoperability token implicitly maps to an interoperable model information, e.g., file format and platform. The encoding, format and value of the interoperability token is up to vendors’ implementation.

- One public format for model description, such as ONNX (Open Neural Net Exchange, ‘https://onnx.ai/’). ONNX aims to support a common intermediate representation for AI/ML model transmission for both deep learning and machine learning. It is developed as a common data format but not an AI framework or a development tool, so it can be compatible with any AI framework including TensorFlow, PyTorch and so on. With the help of ONNX, all developers can choose their own tools to develop their models and load other models in different framework.

- New format for model description defined by 3GPP. However, 3GPP public format will take great efforts and may not be considered in this release.

It may not be easy for 3GPP to align on a single public format such as ONNX. Due to the great efforts for 3GPP to define new format for model description, this may be not considered in this release. Interoperability token defined by SA2 is simple and effective, and does not have the listed problems. Then interoperability token would be a good solution for two sides to coordinate.

Support to reuse the mechanism defined in SA2 (interoperability token) for aligning model description format for model transfer.

Model transfer requirement

The following potential options of model transfer/delivery were considered in RAN2 post-meeting email discussion.

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4: Server can transfer/delivery AI/ML model(s) to UE (transparent to 3GPP).

When analyzing the pros/cons of the potential solutions, the following evaluation metrics can be considered:

Capability to transfer/delivery models for the following model characteristics (RAN1/RAN2 may discuss it): - AI/ML model size (e.g. individual model size, cumulative model size). It may have some categories, e.g. large size, small size - Model transmission/update frequency. It may have some categories, e.g. frequent/infrequent transmission/update - Latency. It may have some categories, e.g. low-latency/high-latency - Robustness - Signalling overhead - Support of delta configuration - Impacts due to handover - Impacts due to failures (e.g. radio link failure) - Possible specification impacts (e.g. RAN2, SA2, and etc) - Inter-operability impacts

To facilitate the discussion of solutions for the model transfer, further specific requirements on model transfer, such as typical model size, frequency of model transfer/update, latency, ciphering and integrity protection requirements, etc. may be required.

RAN1 concludes typical model size, frequency of model transfer/update and latency requirement and send LS to RAN2 to facilitate the discussion of solutions for the model transfer.

Model transfer related capability

To support collaboration level z, UE should have model transfer capability. Model transfer is one of the key tools to resolve AI/ML generalization problem. Then the AI/ML model, which fits the current wireless environment, can be transferred from the network to UE, to improve the system performance. The model transfer procedure includes the following steps from UE perspective.

- Receive the new AI/ML from the network. Since the signal for transmitting the new AI/ML model is similar to the data signal, UE always has the capability of this step.

- Decode the information of the new AI/ML model. The information of the new AI/ML model has been encoded by the network, to better describe the AI/ML model with lower overhead. Some popular AI/ML frameworks can be used as the encoder and decoder, such as TensorFlow, PyTorch and Caffe. The AI/ML model information decoding can be done in UE baseband chipset or UE AP. Currently, this has been already supported by typical UE AP implementations, for several popular AI/ML frameworks. Other options can also work well. One option is reusing the public format for model description, such as ONNX, and another option is introducing a new format for model description defined by 3GPP.

- Load the new AI/ML model in the chipset. After decoding the AI/ML model information, it is time to load the new AI/ML model and use it in the chipset. Whether the AI/ML model structure is changed or not has a considerable impact on the UE capabilities.

  - The AI/ML model structure is not changed. Only model parameters need to be updated in this situation. The new model parameters are sent to the AI/ML module and then just replace the old model parameters. Recompilation may be not needed here. This has been already supported by nowadays typical chipset implementations. The premise is that UE and network have aligned the AI/ML model structure offline or online before usage.

  - The AI/ML model structure is changed. Dependent on how much the model structure is changed, recompilation may be needed. Also, the changed model structure should be supported by UE. For example, if UE only supports full-connected layers and convolutional layer, the new AI/ML model can only be made of full-connected layers and convolutional layer.

Based on the above analysis, we have the following proposal on model transfer capability.

Model transfer capability may consider the alignment between UE and network on supported structures, quantization and processing.

General AI/ML framework

The General framework for RAN intelligence is illustrated in Figure 6-1.

Figure 6-1: Functional Framework for RAN Intelligence in RAN3.

For the general framework on the network side, model management is mainly an internal implementation, such as model transfer, model performance feedback. In comparison, the AI/ML for air interface involves massive interactions between the UE and the network, which are related to model management, e.g., model monitoring, activation/deactivation. Therefore, reusing the framework for RAN directly is not suitable and it is beneficial to introduce more components in on top of the framework for RAN.

To our understanding, the updated framework is shown in Figure 6-2.

Figure 6-2: Updated general framework of AI for air interface.

Based on the Framework for RAN intelligence, RAN1 to introduce an updated general framework that can reflect the key components of AI for air interface.

Discussion on other aspects of LCM

In this section, other aspects of LCM would be discussed.

Data collection

Some assistance information would be needed for data collection. First, general description of collected data can be reported, such as purpose, size and configuration. Different purposes result in different types and format of collected data. For example, the beam qualities are collected for beam management and the estimation of CSI-RS channels are collected for CSI compression or CSI prediction.

Second, UE hardware information can be reported, such as antenna information, so that network can further split the collected data. For example, the beam qualities of one UE panel containing 16 antenna elements can be separated from data of one UE panel containing 8 antenna elements.

Third, Applicable conditions can be reported, such as cell ID, scenario ID and SNR. UE may report the data collected in a long period, which may contain the data of multiple cells, different scenarios and large range of SNRs. With the cell ID or scenario ID, AI/ML model for a small area or certain scenario can be trained, and then flexible model selection or model switching can be used to improve the generalization performance. SNR information can also be used for model selection or model switching, if AI/ML model per SNR range has been trained. Also, SNR information can be used as assistance information for model inference.

Study the following assistance information for data collection:

- General description of collected data, such as purpose, size and configuration;

- UE hardware information (meta data), such as antenna information;

- Applicable conditions, such as cell ID, scenario ID and SNR.

To report a large number of collected data samples, there are two options. The first option is to report large number of samples in one report with low reporting frequency. UE needs to store a large number of samples on the chipset or memory, which may be not supported by all types of UEs. The second option is to report small number of samples in one report with high reporting frequency. Compared with the first option, the requirement of UE storage capability is lower but the reporting overhead may be higher.

Study the two following reporting formats for a large number of collected data samples:

- Large number of samples in one report with low reporting frequency;

- Small number of samples in one report with high reporting frequency.

In legacy CSI framework, only one sample is reported in one time. A lot of work has been done to compress the information of one sample. However, there may be multiple samples in one report. How to compress the information of multiple samples has not been studied before. It is clear that there is more redundancy in the report containing multiple samples, compared with the report having one sample. Then this topic is worth of studying.

Study the data compression for multiple samples in collected data reporting.

Different UE capabilities would be needed for the expected pre-processing, data storage, feature extraction and report for data collection. Some UEs may have the ability to collect a large amount of data while others would be limited. Depending on the reference point definition, some UEs may leave some interfaces for collection in the corresponding reference point while others may not. These should also be discussed in UE capability.

Study ways for UE to report its capability for data collection regarding expected pre-processing, data storage, feature extraction and report for data collection.

Model inference

In the study of CSI compression, beam and location, assistance information can be used to improve the model inference performance. For example, the network transmit beam pattern can be used to improve the model inference at UE in the beam management. Also, the speed is useful for model inference in both channel prediction and beam prediction in time domain.

Study the assistance information needed for model inference.

Computation power at the UE side is growing fast in recent years. Figure 7.2-1 shows the AI capability of NPU in mobile phones in recent years. The capacity of one typical NPU used in current mobile phone is 22.5T operations (OPs) per second. One OP denotes one addition or one multiplication. From 2017Q1, the capacity of typical NPU in mobile phone is growing very fast year by year. This trend is expected to continue for the coming years.

Figure 7.2-1: The growing capacity of NPU in mobile phone.

Typical physical layer modules have strict requirements for latency. The latency of AI/ML operation should be within several milliseconds, otherwise, the AI model would not be applicable for air interface use cases. Since latency is highly correlated with complexity, they are discussed together in this subsection. For this important issue, we have collected some latency information from the area of image processing. Figure 7.2-2 shows the inference performance of typical AI models for image and video in typical chipsets. The latency of the AI models in Figure 7.2-2 is about 0.9ms~5.1ms. The AI models for air interface would be much simpler than the listed AI models and the latency of AI models for air interface will be much smaller. Then AI models for air interface would be likely to meet the latency requirement.

Figure 7.2-2: The latency of typical AI models for image and video in typical chipsets.

Initial test of typical models for latency on typical chipsets in Figure 7.2-2 shows that the latency for neural network operation latency on UE are within the range of interest for air interface applications.

Study ways for UE to report its capability for latencies with respect to the model inference.

Float point is usually used in study and initial evaluation. Fixed point is usually used in implementation, where the parameters of AI model are transformed to integer value and the complexity could be reduced. Some kinds of hardware only support fixed point.

For float point or fixed point, there are also different levels of bits used for one number. It is clear that the overhead of 32 bits is twice of 16 bits, and 4 times of 8 bits. Considering the overhead reduction of model transfer, low quantization level would be better than high quantization level. The actual complexities of different quantization levels would be slightly different from the overhead, due to the practical hardware design and AI model structure.

Figure 7.2-3: The latency ratio of typical AI models of different quantization levels in one typical chipset, compared to CPU-FP32 of AI model 4.

Figure 7.2-4: The power consumption ratio of typical AI models of different quantization levels in one typical chipset, compared to CPU-FP32 of AI model 4.

Some evaluation results of different quantization levels are shown in Figure 7.2-3 and 7.2-4. INT8 denotes integer value with 8 bits, FP16 denotes float point value with 16 bits. The performance of NPU-INT8 is about 1.8 times of NPU-FP16, 4.8~17 times of GPU-FP32, and 4.5~17 times of CPU-FP32. INT8 is suitable for service with high complexity and power consumption, such as photographing and video. FP16 is suitable for service with high accuracy and low power consumption, such as pixel-wise image processing.

Quantization of the model has impacts on latency performance.

Quantization of the model has impacts on power consumption.

Study UE capability on supported quantization levels.

Model selection/activation/deactivation/switching/fallback

Model selection, activation, deactivation, and switching procedure can be triggered in the following cases: i) any performance degradation observed for the undergoing model during regular performance monitoring; ii) significant changes in UEs’ wireless environment due to mobility, handover, or other reasons; iii) unsatisfying performance of legacy non-AI mechanism over a long time-window. Specifically, it can be a consensus that performance degradation will trigger a model selection procedure to check whether there are better models for the current situation, and it is likely that significant changes in UE’s environments usually indicate a potential performance degradation for the undergoing model. Besides, we believe that an unsatisfying performance of legacy non-AI mechanism can also trigger a model selection procedure to switch from legacy mechanism to AI/ML based solutions to achieve better system performance. Fallback can be triggered when the performance of legacy non-AI mechanism is better than AI mechanism over a long time-window.

Study event triggered model selection, activation, deactivation, switching, and fallback.

As discussed in Section 5, applicable conditions or applicability ID could be used in LCM. Applicable condition may contain the information of the suitable scenario/configuration/site of AI/ML model, which could be used as part of the triggering event. applicability ID can be contained in the triggering event information to reduce the signaling overhead.

Study applicable conditions or applicability ID assisted model selection, activation, deactivation, switching, and fallback.

Network may obtain the AI/ML calculation capability of UE through UE AI/ML capability report. If the AI/ML model structure is known by network, or the AI/ML model complexity has been reported during model registration, network can have an estimation of UE’s free AI/ML calculation resource. Based on this estimation, model selection, activation, deactivation, switching, and fallback can be indicated by network to make sure that UE’s AI/ML calculation resource is enough for all current working AI/ML models.

However, there are gaps (as indicated by previous conclusion) between actual complexity and the complexity evaluated using TOP/FLOP/MACs due to the platform- dependency and implementation (hardware and software) optimization solutions. One possible way is for the UE to dynamically report whether it has enough calculation power to run a specific model indicated by the network side.

Study the mechanism for dynamic UE reporting of whether it has enough resources to run AI/ML models.

Discussion on SA related aspects

In this section, the impacts of the AI/ML aspects on SA will be discussed.

Model transfer/deliver

As discussed in Section 5, Cases y, z1 to z5 for model delivery/transfer have been agreed to facilitating discussion in the last meeting. These cases consider different model delivery/transfer, model storage location and training location, and provide different representative scenarios for detailed study. These different scenarios are highly related to SA2 and corresponding feasibility need to be assessed by SA2 .

To facilitate the study of model transfer/deliver in SA, it is suggested to send the agreed Cases y, z1 to z5 for model delivery/transfer to SA.

Model training

Type 3 training for two sided CSI may need the involvement of SA. As shown in the below figure, SA may need to study Step3, in which the exchange dataset is transmitted between two sides. One possible solution is the UP based dataset exchanging between UE and network. Another possible solution is the dataset exchanging between UE side servers and network entities. It is seen that both possible solutions need to be studied by SA.

Figure 8-1: An illustration of separate training procedure.

Data collection

There are two main solutions of collected data report. One solution is the CP based reporting mainly for small number of data samples. Legacy CSI framework may be largely reused for this solution. The other solution is the UP based reporting mainly for large number of data samples. In the existing framework of the UP based solution, data would be collected from UE Application(s) and Event Exposure (EVEX) mechanism. The detailed UP based solution is worthy of study by SA.

Model/Applicability Identification

For model and applicability identification, there is also work related to SA. The information needed for alignment of model ID and applicability ID would need signaling exchange between network sided servers and UE sided servers.

In sum, we have the following proposal.

Send LS to SA2 and SA4 to study the potential specification impact of at least model transfer/deliver, model training, data collection and model identification.

Conclusions

- AI/ML-enabled feature reporting is needed for both functionality identification and model identification and can follow legacy procedure.

- Functionality identification is suitable for cases where “static" AI/ML capability is implemented in the UE.

Functionality identification requires the AI/ML capability to have good generalization performance and whether and how it can be applied to different cases need justification.

For the purpose of discussion on aligning applicable conditions between entities, define the terminology of “applicability identification” as following.

- Applicability identification: A process/method of identifying an AI/ML applicable condition for the common understanding between the NW and the UE.

  - Note: Information regarding the AI/ML applicable condition may be shared during applicability identification.

Study “logical model” identification or applicability identification based LCM procedures.

Consider applicable condition update for an existing model in “logical model” identification or applicability identification procedure.

Functionality identification based LCM contains two steps as legacy UE features, UE capability report and RRC enabling/disabling procedures.

Study LCM based on AI/ML-enabled feature with applicability identification, in which applicability ID is used for LCM.

Consider to define the procedures as in Figure 2-1 for model/applicability identification.

Consider the following model identification types.

- Type A (Step0 in Figure 2-1): Model is identified offline to network (if applicable) and/or UE (if applicable).

- Type B (Step1, Step2b and Step4 in Figure 2-1): Model is identified via signaling from UE to network.

- Type C (Step2a, Step2b and Step4 in Figure 2-1): Model is identified via signaling from network to UE.

The following challenges should be addressed for RAN4 related aspects:

- Multi-vendor (network vendor, UE vendor and TE vendor) involvement in two sided cases.

- Generalization performance definition.

- Performance requirement need to be defined to justify AI/ML gains.

- Performance of different models would be different, especially considering complicated training types.

Consider to define reference model structure in RAN4 or RAN1.

Compared with zone/site specific models, generic model applicable across multiple areas would typically have larger computation complexity and storage overhead.

Collaboration level z can be used to facilitate zone specific model deploy.

From initial results for field test, performance of simple model structure, e.g., one hidden layer full-connected encoder, is good enough for typical zone/site specific optimization.

From initial results for field test, the model developed for Cell 1 shows robust performance for different moving routes.

Field test shows that model developed for Cell 1 does not perform well for Cell 2.

Field test shows that simple and small models work well for all different cases, at least for typical cell coverage.

From initial results for spatial consistency data, performance of simple model structure, e.g., one hidden layer full-connected encoder, can achieve good performance for zone/site specific operations.

From initial results for ray tracing based data generated with the map provided in [2], performance of simple model structure, e.g., one hidden layer full-connected encoder, can achieve good performance for zone/site specific operations.

To fight against the AI/ML generalization problem, generic model would typically have larger computation complexity and storage overhead, while zone/site specific models would need simple model structure and small model size.

Model delivery/transfer is necessary due to flexibility for model update and per cell or area optimization.

The model structure is disclosed in open format, but if widely-known model (e.g., fully connected or CNN) is used, this issue does not exist.

One advantage of open format is flexibility for model update and per cell or area optimization. Open format can support flexible model update and optimize the model per cell or area, while proprietary format would need the network to store all the non-recognizable image files at the network side, or UE side would need to store large number of image files.

Delivery model with proprietary format requires offline consensus among multiple vendors on how to use a proprietary file which is non-feasible in most cases. Moreover, for proprietary format, network has to prepare multiple models for different UE vendors.

For parameter-only updating with properly aligned information, no additional compiling is needed. If flexible model structure update is also involved, it would lead to extra delay or extra device capability.

Study open format with widely-used model structures and parameter-only updating, which can obtain the most gain of open format and proprietary format.

- Using widely-used model structures to avoid the concern of disclosing model structure.

- Using parameter-only updating to avoid additional compiling at UE side.

Use pros and cons in Table 5.3-1 for the model transfer/delivery discussion.

Performance of simple model structures, such as full-connected layers or convolutional layers, are good enough for typical per single cell or multiple cell operations.

The models currently evaluated by companies, such as those with full-connected layers or convolutional layers, are widely used for decades and have low proprietorship risk for model transfer.

Support to reuse the mechanism defined in SA2 (interoperability token) for aligning model description format for model transfer.

RAN1 concludes typical model size, frequency of model transfer/update and latency requirement and send LS to RAN2 to facilitate the discussion of solutions for the model transfer.

Model transfer capability may consider the alignment between UE and network on supported structures, quantization and processing.

Based on the Framework for RAN intelligence, RAN1 to introduce an updated general framework that can reflect the key components of AI for air interface.

Study the following assistance information for data collection:

- General description of collected data, such as purpose, size and configuration;

- UE hardware information (meta data), such as antenna information;

- Applicable conditions, such as cell ID, scenario ID and SNR.

Study the two following reporting formats for a large number of collected data samples:

- Large number of samples in one report with low reporting frequency;

- Small number of samples in one report with high reporting frequency.

Study the data compression for multiple samples in collected data reporting.

Study ways for UE to report its capability for data collection regarding expected pre-processing, data storage, feature extraction and report for data collection.

Study the assistance information needed for model inference.

Initial test of typical models for latency on typical chipsets in Figure 7.2-2 shows that the latency for neural network operation latency on UE are within the range of interest for air interface applications.

Study ways for UE to report its capability for latencies with respect to the model inference.

Quantization of the model has impacts on latency performance.

Quantization of the model has impacts on power consumption.

Study UE capability on supported quantization levels.

Study event triggered model selection, activation, deactivation, switching, and fallback.

Study applicable conditions or applicability ID assisted model selection, activation, deactivation, switching, and fallback.

Study the mechanism for dynamic UE reporting of whether it has enough resources to run AI/ML models.

Send LS to SA2 and SA4 to study the potential specification impact of at least model transfer/deliver, model training, data collection and model identification.

References

Chair's notes of RAN1#112, February 27th – March 3rd, 2023.

A. Alkhateeb, “DeepMIMO: A Generic Deep Learning Dataset for Millimeter Wave and Massive MIMO Applications,” in Proc. of The Information Theory and Applications Workshop (ITA), San Diego, CA, Feb. 2019. Codes and instructions available at ‘https://deepmimo.net/’.

vivo, R1-2302477, “Evaluation on AI/ML for CSI feedback enhancement”,  RAN1 #112bis-e, April 17th – April 26th, 2023.

vivo, “Dataset for AI CSI feedback”, https://commonbox.vivo.xyz/s/VkhgUFG2hhd.

vivo, “Dataset For AI CSI Prediction”, https://commonbox.vivo.xyz/s/1qv4tjQ5efk.

vivo, “Dataset for beam management”, https://commonbox.vivo.xyz/s/gMEadbdyFtd.

vivo, “Dataset for AI Positioning”, https://commonbox.vivo.xyz/s/UQnWAcqp2DL.

Appendix A: Analysis for zone/site specific model update

One of the key issues for lifecycle management is how often the model needs to be updated. In this section, we have some preliminary analysis on the granularities of model update.

In most cases, the parametric model defines a distribution p(y | x;w) and we simply use the principle of maximum likelihood. This means we use the cross-entropy between the training data and the model’s predictions as the cost function, as described

Jw=-E x,y~pdatalogpmodely | x;w

where y is the input data vector, x is the known data vector (or label), w is the coefficient vector or the weight vector, acquired by the training procedure, all in a given AI neural network.

It is worthwhile noting that, the training set associated with any input pair of (y,x) can be expressed as

(y,x)∈yn,xnn=1N

In such a procedure, accordingly, the AI model can be trained by means of the off-line training manner under the condition of the statistic wireless channel model and can be considered as a universal AI model for any UE or gNB uses.

However, the channel factors influenced by gNB are comparatively stable, while the channel factors influenced by UE are unpredictable, with respect to the antenna direction and location. In addition, the channel model utilized for performance evaluation mainly refers to TR 38.901, where the long-term channel factors such as receive antenna field patterns (i.e., AoA and ZoA), receive antenna location vector, transmit antenna field patterns (i.e., AoD and ZoD), and transmit antenna location vector update statically, while the short-term channel factors such as Doppler frequency update dynamically. Consequently, therefore, a universal AI model purely trained by a statistic wireless channel model may be not feasible in terms of the complexity of neural network and the overall AI-based system performance. Somewhat UE assistance mechanism in addition to cell-based training model may be necessary.

Thanks to the unique wireless channel behaviors, we believe that the training set can be possibly divided into K training subsets relying on the long-term statistic channel parameters. If we assume that the k-th subset is associated with the parameter of zk, the training set can be represented as

yn,xnn=1N=k=1KYk,Xk | zk

where z can be seen as the assisted parameter vector, z=z1,z2,⋯,zK, and the k-th training subset can be expressed as

Yk,Xk=ynk,xnknk=1Nk;			for	  k=1KNk≥N.

If the subset Yk,Xk and subset Yi,Xi are completely independent, i.e., Yk,Xk∩Yi,Xi=∅, for k≠i, and the distribution p(y | x;w) associated with the parameter of zk is approximated as

p(y | x;z;w)≅k=1Kpyk | xk;zk;w

Then, the cross-entropy between the training data and the model’s predictions can be

Jw=-E x,y~pdatak=1Klogpmodelyk | xk;zk;w

If the parameter vector of z is given, the cross-entropy in the training procedure for the parametric model with the pre-known zk can be individually represented as

Jwk|zk=-E xk,yk~pdatalogpmodelyk | xk;zk;wk                                    Eq. 1

where k=1,2,⋯,K, and K is the total number of training models.

It is worthwhile noting that, the AI models can be trained by means of offline manner and utilized by each UE accordingly. This does imply that each AI-model can be seen as a sub model, and the K sub models form a cell-specific AI model which can be operated by all the UEs if connected with the corresponding gNB.

As one specific example, by geographically dividing the network area, AI models associated with different areas can be distinguished and the related tasks associated with AI models would be limited. This results in the improvement of the accuracy and effectiveness of the AI model, and the reduction of the complexity of AI neural networks. To achieve the above purpose, the network may perform regional division of geographic coordinates through a zone identification (i.e., Zone-ID). The network determines the network coverage area related to the maximum communication range Dmax according to the geographic location of the gNB, which is further divided into multi-zones represented by Zone-ID. As illustrated in Figure B-1, the size of each zone with W×L is configurable according to the use-cases and the deployed scenarios, where L is the zone length and W is the zone width. During the AI model training procedure, the training dataset can be distinguished by the Zone-ID in the network coverage area. Therefore, the trained AI model behaves the characteristics of the zone indicated by Zone-ID.

Therefore, the training procedure for the parametric model with the pre-known parameter, zk, can be individually trained as formulated in Eq. 1, where zk is the Zone-ID within the maximum communication range Dmax.

It is worthwhile noting that this type of AI model training process can be completed by either the UE or by the network.

Figure A-1: Schematic diagram of distinguishing AI models based on geographic information

By dividing different orientations of the network, alternatively, AI models associated with different orientations from gNB can be distinguished and the related tasks associated with AI models can be limited. This also results in the improvement of the accuracy and effectiveness of the AI model and the reduction of the complexity of AI neural networks.

As illustrated in Figure B-1, the area covered by the network is divided into K orientations (or azimuths), and each orientation forms a pie-shaped directional sub area, denoted by zn,k, where n is the ID of the gNB and k is the subregion-orientated index. Optionally, the widths of the pie-shaped sub regions formed by the orientation of each sub region could be the same or different and determined by high-level configuration. More specifically, each sub region orientation can be regarded as an orientated beam (i.e., directional beamforming), where the orientated beam width is 2π/K. During the AI model training procedure, if the gNB or UE can roughly acquire the geographic location of the UE or the AoA/DoA associated with the gNB, the AI training dataset can be distinguished by the orientation of each sub region. In such a case, the gNB or UE only uses the data related to the orientation of the sub region to train the AI model, which behaves the orientation features.

Therefore, the training procedure for the parametric model with the pre-known parameter, zn,k, can be individually trained as formulated in Eq. 1, where zn,k is the subregion-orientated index within the maximum communication range Dmax.

It is worthwhile noting that this type of AI model training process can be completed by either the UE or by the network as well.

Figure A-2: Schematic diagram of distinguishing AI models based on direction information

Appendix B: Introduction of ONNX

ONNX (Open Neural Net Exchange, ‘https://onnx.ai/’) aims to support a common intermediate representation for AI model transmission for both deep learning and machine learning. It is developed as a common data format but not an AI framework or a development tool, so it can be compatible with any AI framework including TensorFlow, PyTorch and so on. In the design of ONNX, model structure and weights are sequenced by Protobuf. It defines an extensible computation graph with nodes with operators and handles all weights as inputs or outputs. It also defines the standard data types.

In ONNX, the computation graph is composed of some nodes and each node has several inputs and outputs. All the tensors are identified by its name. The same input name of node A and output name of node B means node A and B are connected. All weights are also identified by their names and corresponding to some nodes as inputs or outputs. Then the computation graph is constructed with the input name and the output name of each node.

With the help of ONNX, all developers can choose their own tools to develop their models and load other models in different framework. And now, ONNX is supported in many frameworks, tools and even some hardwires officially. Since ONNX does not impose restriction on operators, the same construct or function can be transformed to different combinations of nodes for different developers. All developers can have their specific transition code, which means it can be enhanced further to support other destinations like security.

Appendix C: Power consumption

Power consumption is one of the key parameters in current chipsets and much work has been done for power saving. If AI models largely increase power consumption, the commercialization of AI algorithm in air interface would be not a good deal.

The actual power consumptions of typical AI models are listed in Table C-1. From the discussion of power saving, the UE power consumption model for FR1 is shown in Table C-2, in which the basic unit would be assumed as 5 mA. It is seen that the power consumptions of complex AI models are comparable with typical physical layer operations. Power consumption for typical neural network operation on typical chipsets are at the same level of power consumption as for SSB or CSI-RS processing or PDCCH decoding.

Table C-1: The actual power consumptions of typical AI models in typical chipsets.

|  | Electron current (mA) |
| --- | --- |
| AI Model 2 (1.14 GTOPs) | 291 |
| AI Model 4 (11.5 GTOPs) | 420 |

Table C-2: UE power consumption model for FR1 from the discussion of power saving.

| Power state | Relative power (mA) |
| --- | --- |
| Deep sleep | 1*5 (Optional: 0.5) |
| Light sleep | 20*5 |
| Micro sleep | 45*5 |
| PDCCH-only | 100*5 |
| SSB or CSI-RS proc | 100*5 |
| PDCCH+PDSCH | 300*5 |
| UL | 250*5 (0dBm) 700*5 (23dBm) |

Performance of model is not directly related to FLOPs, but specifically tuned for models. Even with the similar FLOPs, the performance may be drastically different. For example, as shown in Figure C-1, the complexity of AI model 1 is 77.2% of AI model 2, and then the expected latency of AI model 1 is 77.2% of AI model 2. However, the actual latency of AI model 1 is 152% of AI model 2 in Chipset 2. For another example, the complexity of AI model 2 is 9.9% of AI model 4, while the power consumption of AI model 2 is 69.3% of AI model 4.

Figure C-1: The complexity and latency comparison between AI models.

Appendix D: Agreement from previous meeting

Some agreements and conclusions have been made in previous meeting.

Agreement To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top; Outside 3gpp Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format; 3GPP Network; NW-side. z3; model transfer in open format; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE; 3GPP Network; NW-side. Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. Note: The Case definition is NOT intended to introduce sub-levels of Level z. Note: Other cases may be included further upon interest from companies. FFS: Z4 and Z5 boundary Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model Agreement - AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Agreement - For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature. Agreement For 3GPP AI/ML for PHY SI discussion, when companies report model complexity, the complexity shall be reported in terms of “number of real-value model parameters” and “number of real-value operations” regardless of underlying model arithmetic.