3GPP TSG RAN WG1 #112bis-e                                                         R1-2302477

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:	Evaluation on AI/ML for CSI feedback enhancement

Agenda Item:	9.2.2.1

Document for:	Discussion and Decision

Introduction

In last meeting, the evaluation performance of CSI compression and CSI prediction are discussed. In this contribution, we discuss the details of remaining evaluation assumptions and provide more evaluation results for the performance gain.

Conclusion For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, except for Method 3 which has been supported, There is no consensus on whether to adopt an additional method. Agreement Confirm the following working assumption of RAN1#110bis-e: Working assumption In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors Note: Eventual KPI can still be used to compare the performance. Conclusion For the intermediate KPI for evaluating the accuracy of the AI/ML output CSI, except for SGCS and NMSE which have been agreed as the baseline metrics, for whether/how to introduce an additional intermediate KPI, NO additional intermediate KPI is adopted as mandatory. - It is up to companies to optionally report other intermediate KPIs, e.g., Relative achievable rate (RAR) Agreement For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on CSI without compression (e.g., eigenvector from measured channel), which is taken as an upper bound for performance comparison Agreement Confirm the following WA on the benchmark for CSI prediction achieved in RAN1#111: Working Assumption For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction. Agreement For the evaluation of training Type 3 under CSI compression, besides the 3 cases considered for multi-vendors, add one new Case (1-on-1 training with joint training) as benchmark/upper bound for performance comparison. • FFS the relationship between the pair(s) of models for Type 3 and the pair(s) of models for new Case Agreement The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE. • For AI/ML based solutions: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank. • For legacy Type II CB: Option 2b is mandatorily reported by companies, while Option 2a can be optionally reported up to companies if partial NZC report is assumed for the legacy Type II CB  Option 2a: The above-mentioned “CSI feedback overhead” is calculated as each CSI reported payload with a given rank  Option 2b: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank Working Assumption For the initial template for AI/ML-based CSI compression without generalization/scalability verification achieved in the working assumption in the RAN1#111 meeting, X, Y and Z are determined as: • X is <=80bits • Y is 100bits-140bits • Z is >=230bits Working Assumption X, Y and Z are applicable for per layer •

CSI compression

In this section, we express our views on the per-area model evaluation and the generalization of CSI compression in input, output scalability and rank number respectively. On top of that, we discuss the performance of CSI compression with different training method and field data.

Evaluation on per-area model

Principle of per-area model

AI/ML is data driven. Thus, it would be natural to use a per-area model for CSI compression: training models based on data collected from a specific area, which will then be used within the corresponding area. By “specific area”, we refer to a relatively smaller region, such as one cell, one sector, or one zone.  One of the most promising advantage of per-area models compared with conventional general models is potentially higher performance gain. As presented in 2.1.2 and 2.1.3, SGCS performance of per-area models is higher than that of general models over 30%~50%. Note that SGCS of general models is only 10% higher than that of legacy Rel-16 Type II codebooks. Therefore, the performance gain achieved by per-area models could reach 40-60% compared with legacy R-16 Type II codebook. The additional performance gain in per-area models comes from the fact that per-area models only need to fit data with less variety of characteristics. It should be clarified that there are no serious overfitting issues for per-area models, as the distribution of testing data (also collected from the same area as training data) is usually similar to that of training data. Our initial evaluation results offered by per-area models in the following subsections also support such observations.

Based on initial field test results, per-cell (region) models can provide more than 30%~50% improvement on SCGS of AI models.

Per-area models could be naturally deployed within each cell, i.e., each cell trains its own model based on data collected within the cell. However, one problem is that as a UE moves from one cell to another, CSI generation part at UE side should also be updated to adapt to the new cell. For training collaboration type 1, such procedure could be done via transferring the updated model to the target UE. For training collaboration type 2, another over-the-air training procedure is needed to update the model. For training collaboration type 3, new model input/output data will be shared from network to UE or vice versa to finish the updating of models. If the model structure of CSI generation part is simple (e.g., one-layer MLP), overhead of the model updating procedure will be very small (probably less than 100kB).

Further study the model update for per-cell (region) models

Training per-area models requires to enhance the data collection mechanism by some assistance information. Cell ID/sector ID/Zone ID or some other information that could represent the collecting area should be assigned to the corresponding data during dataset delivery. However, there could be some concerns on user privacy, UE storage, power consumption or overhead. More studies on data collection for per-area models should be considered in the future meetings.

Further study the data collection for per-cell (region) models.

Some initial results for spatial consistency data

Here we consider using data where the channel has spatial consistency characteristics. Each UE generates random variables with spatial consistency based on its own geographic location at the T=0, both the cluster specific random variables and the correlation distance for spatial consistency procedure follow 38.901. The detailed parameters are provided as follows.

Parameters of spatial consistency data of CSI compression.

| Parameters | Value |
| --- | --- |
| Scenario | Uma |
| Channel model | Uma 38.901 with spatial consistency |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 30KHz |
| Frequency resources | 52 RBs, 13 subbands for 4RBs per subband |
| Rank | Max rank 1 |
| gNB antenna | 32 antenna ports [Mg Ng P M N, Mp Np] = [1 1 2 8 8, 2 8] dH,dV= (0.8, 0.5) λ, +45°/-45° polarization |
| UE antenna | 2 antenna ports [Mg Ng P M N, Mp Np] = [1 1 2 1 1, 1 1] dH,dV= (0.8, 0.5) λ, 0°/+90° polarization |
| BS receiver noise figure | 10 |
| UE receiver noise figure | 7 |
| UE distribution | 100% outdoor |
| UE speed | 30km/h |
| Mechanic tilt | 180° in GCS (pointing to the ground) |
| Beam set at TRxP | Azimuth angle φi = [0], Zenith angle θj = [102]. |
| UE beam set | Azimuth angle φi = [0], Zenith angle θj = [90] |

Cell specific model is considered and then different AI/ML models are used for different cells. Simple AI/ML model, which is one-layer MLP encoder, and complex transformer encoder are evaluated in this simulation. It can be seen that the performance of simple AI/ML model is similar to that of complex AI/ML model. Compared with simple MLP encoder, the SGCS gain of transformer encoder is only about 3%, but the complexity of transformer encoder is about 14 times higher. Considering the performance and complexity, simple model structure, e.g., one hidden layer full-connected encoder, is good enough for typical per single cell or multiple cell operations.

The SGCS results of multiple AI/ML models trained by the data in each area separately.

|  | AI with a cell specific model (One layer MLP encoder) ~285kB | AI with a cell specific model (Transformer encoder) ~4.08MB |
| --- | --- | --- |
| Cell 0 | 0.8345 | 0.8895 |
| Cell 1 | 0.8815 | 0.9168 |
| Cell 2 | 0.9132 | 0.9412 |
| Cell 3 | 0.9148 | 0.9439 |
| Cell 4 | 0.8718 | 0.9049 |
| Cell 5 | 0.9076 | 0.938 |
| Cell 6 | 0.8698 | 0.9072 |

From initial results for spatial consistency data, performance of simple model structure, e.g., one hidden layer full-connected encoder, can achieve good performance when per cell model is used.

Some initial results for field test

We provide some initial results for field test of CSI compression. The data is collected from actual 5G network and the collecting area is about 400m * 350m. About outdoor 50000~100000 samples per area or cell are collected. The detailed parameters are provided in below table.

Parameters of field test of CSI compression.

| Parameters | Value |
| --- | --- |
| Scenario | Actual 5G network, about 400m * 350m collecting area. About outdoor 50000~100000 samples per area or cell. |
| Carrier frequency | 3.45GHz |
| Subcarrier spacing | 30KHz |
| Frequency resources | 52 RBs, 13 subbands for 4RBs per subband |
| Rank | Max rank 1 |
| gNB antenna | 8 antenna ports |
| UE antenna | 4 antenna ports |
| CSI payload | 167/58 bits payload |

Field test result of different areas

There are 3 data collecting areas. Area B is the main road of the industrial park, with many tall trees and cars along the road. Area C is the road behind several buildings. Area D is the indoor scenario in a building. UE in the left part of the industrial park usually accesses to a different cell, compared with the right part of the industrial park. So, we focus on the right part of the industrial park and current areas are chosen.

The map of data collecting areas.

The SGCS results of eType II codebook and multiple AI/ML models are provide in the following. In Table 4 and 5, the AI/ML models are trained by the data in each area separately, and multiple AI/ML models are used. 167 bits overhead is used in Table 4 and 58 bits overhead is used in Table 5. In Table 6 and 7 only one hidden layer full-connected encoder is used and it is trained by the data of all 4 areas, with 167 bits overhead and 58 bits overhead separately.

It is seen that the performance gaps between different AI/ML models are small. Even one layer MLP encoder can provide good performance, which is very simple and small. With much higher complexity, Transformer encoder has better performance than one layer MLP encoder, but the performance gain is small Area B. Considering the performance and complexity, simple model structure, e.g., one hidden layer full-connected encoder, is good enough for typical per single cell or multiple cell operations.

The SGCS results of multiple AI/ML models trained by the data in each area separately, with 167 bits overhead.

| 167 bits overhead | eTypeII | AI with an area specific model (One layer MLP encoder) ~67kB | AI with an area specific model (small CNN encoder) ~250kB | AI with an area specific model (Transformer encoder) ~3.6MB |
| --- | --- | --- | --- | --- |
| Area B | 0.8429 | 0.9217 | 0.929 | 0.9406 |
| Area C | 0.7871 | 0.898 | 0.9037 | 0.9116 |
| Area D | 0.8489 | 0.9315 | 0.9323 | 0.9423 |

The SGCS results of multiple AI/ML models trained by the data in each area separately, with 58 bits overhead.

| 58 bits overhead | eTypeII | AI with an area specific model (One layer MLP encoder) ~30kB | AI with an area specific model (small CNN encoder) ~213kB | AI with an area specific model (Transformer encoder) ~3.3MB |
| --- | --- | --- | --- | --- |
| Area B | 0.7290 | 0.8573 | 0.8725 | 0.8868 |
| Area C | 0.6438 | 0.8015 | 0.8162 | 0.8389 |
| Area D | 0.6853 | 0.8701 | 0.8814 | 0.8873 |

The SGCS results of one hidden layer full-connected encoder trained by the data of all 4 areas, with 167 bits overhead.

| 167 bits overhead | Training SGCS on data from all areas | Testing SGCS in area B | Testing SGCS in area C | Testing SGCS in area D |
| --- | --- | --- | --- | --- |
| One hidden layer full-connected encoder~67kB | 0.9055 | 0.905 | 0.8799 | 0.8959 |

The SGCS results of one hidden layer full-connected encoder trained by the data of all 4 areas, with 58 bits overhead.

| 58 bits overhead | Training SGCS on data from all areas | Testing SGCS in area B | Testing SGCS in area C | Testing SGCS in area D |
| --- | --- | --- | --- | --- |
| One hidden layer full-connected encoder~30kB | 0.8184 | 0.8201 | 0.7592 | 0.7958 |

From initial results for field test, performance of simple model structure, e.g., one hidden layer full-connected encoder, is good enough for typical zone/site specific optimization.

Field test result of different physical cells

The performance of different physical cells is analyzed in the following. We have tested the coverage of different cells in the industrial park, according to the measured RSRP, RSRQ and SINR. The coverage areas of two typical cells in the industrial park are shown in the below figure.

The map of data collecting cells.

For cell 1, data samples are collected by different days. The data collection routes in different days have some differences, which results in the different wireless channel features in different days.

The SGCS results of eType II codebook and multiple AI/ML models are provide in the following. In below tables, the AI/ML models are trained by the data in each cell separately, and multiple AI/ML models are used. For cell 1, A large combination of data collected on different days has been used. 167 bits overhead is used. One layer MLP encoder is used in Table 8, small CNN encoder is used in Table 9 and Transformer encoder is used Table 10.

It is seen that using the data collected on various days or routes, the generalization problem of different days or routes could be solved. For example, using Cell 1 data collected on 2.28 as the test data set, the AI model trained using Cell 1 data collected on various day except 2.28 and 2.16, has the nearly the same performance as the AI model trained using Cell 1 data collected on 2.28.

In addition, the AI model trained by Cell 1 data has poor performance on Cell 2 data, which is even worse than eType II.

The SGCS results of multiple AI/ML models trained by the data in each cell separately, with 167 bits overhead and one layer MLP encoder.

|  | eType II | AI model using Cell 1 data, collected on 2.3, 2.10, 2.14, 2.27, 3.2, 3.9 | AI model using Cell 1 data, collected on 2.28 | AI model using Cell 1 data, collected on 2.16 | AI model using Cell 2 data |
| --- | --- | --- | --- | --- | --- |
| Cell 1 data, collected on 2.28 | 0.8137 | 0.8546 | 0.8680 | \ | \ |
| Cell 1 data, collected on 2.16 | 0.8425 | 0.8746 | \ | 0.8952 | \ |
| Cell 2 data | 0.8145 | 0.7832 | \ | \ | 0.8974 |

The SGCS results of multiple AI/ML models trained by the data in each cell separately, with 167 bits overhead and small CNN encoder.

|  | eType II | AI model using Cell 1 data, collected on 2.3, 2.10, 2.14, 2.27, 3.2, 3.9 | AI model using Cell 1 data, collected on 2.28 | AI model using Cell 1 data, collected on 2.16 | AI model using Cell 2 data |
| --- | --- | --- | --- | --- | --- |
| Cell 1 data, collected on 2.28 | 0.8137 | 0.870 | 0.876 | \ | \ |
| Cell 1 data, collected on 2.16 | 0.8425 | 0.888 | \ | 0.907 | \ |
| Cell 2 data | 0.8145 | 0.8099 | \ | \ | 0.9044 |

The SGCS results of multiple AI/ML models trained by the data in each cell separately, with 167 bits overhead and Transformer encoder.

|  | eType II | AI model using Cell 1 data, collected on 2.3, 2.10, 2.14, 2.27, 3.2, 3.9 | AI model using Cell 1 data, collected on 2.28 | AI model using Cell 1 data, collected on 2.16 | AI model using Cell 2 data |
| --- | --- | --- | --- | --- | --- |
| Cell 1 data, collected on 2.28 | 0.8137 | 0.8847 | 0.8934 | \ | \ |
| Cell 1 data, collected on 2.16 | 0.8425 | 0.8998 | \ | 0.9160 | \ |
| Cell 2 data | 0.8145 | 0.8404 | \ | \ | 0.9172 |

From initial results for field test, the model developed for Cell 1 shows robust performance for different moving routes.

Field test shows that model developed for Cell 1 does not perform well for Cell 2.

Field test shows that simple and small models work well for all different cases, at least for typical cell coverage.

Consider to capture observations from field data test into TR.

Study the performance and overhead of per-cell (region) model transfer in CSI compression.

Generalization of input scalability

The input dimension of AI model is corresponding to the input precoder matrices, i.e., the subband number and port number for each single layer. Different frequency granularity or different ports number can cause different input dimension of AI model. The AI models for different input dimensions need to be trained independently, which may lead to difficulty in generalizing AI models.

In case that the training input dimension of AI model is larger than the inferring input dimension of AI model, the inferring input can expand to the same dimension with zero-padding. On the other side, when inferring input dimension is larger, it can be truncated to the training input dimension.

Also, the input dimension of AI model can be fixed to a given level with pre-processing like angle-delay compression in eType II codebook. With the fixed number of beam and path selected, the dimension of input is certain for different frequency granularity and different ports number. Also, the size of AI model can be reduced because the information to study is decreasing. For AI model, since the compression and quantification are managed together, the restriction of NZC is not needed. So, compared with the eType II codebook, the beta is 1 for AI model and the payload of UCI is influenced by the length of encoder output.

In the simulation, we evaluate the lower boundary with generalization case 2. The AI/ML model is trained based on training dataset from configuration A (13 subbands and 32 ports) only. Then the AI/ML model is tested on a dataset from configuration B with different subband number of port number as below.

Case 1: (baseline) a different drop with 13 subbands and 32 ports

Case 2: (smaller subbands number and the same ports number) a drop with 10 subbands and 32 ports

Case 3: (the same subbands number and smaller ports number) a drop with 13 subbands and 16 ports

Case 4: (smaller subbands number and smaller ports number) a drop with 10 subbands and 16 ports

For each case, we test the normal AI/ML model and the preprocessing AI/ML model. For the normal AI model, the input is 13 subbands and 32 ports and zero-padding is used for less input dimension. For the preprocessing AI/ML model, angle-delay compression is used for preprocessing and 4 top strong beams on each polarization and 4 top strong paths are selected, which means the input dimension is 8 * 4 complex coefficients.

The payload of the normal AI/ML model is fixed to 180 bits and the pre-processing AI/ML model is fixed to 154 bits. With the different payload to report the angle and delay information, the final payload for pre-processing AI/ML model of the four cases are different but all about 180 bits.

The SGCS of AI/ML model with different subband number and port number

|  | Normal AI/ML model | Pre-processing AI/ML model |
| --- | --- | --- |
| Case 1 (13 subbands and 32 ports) | 0.879 | 0.83 |
| Case 2 (10 subbands and 32 ports) | 0.839 | 0.847 |
| Case 3 (13 subbands and 16 ports) | 0.727 | 0.872 |
| Case 4 (10 subbands and 16 ports) | 0.707 | 0.89 |

According to the evaluation results, for normal AI/ML models, the performance declines with the increasing difference between training data set and testing data set. For case 2 (the ports number is the same and the subbands number is different), the SGCS is still in a feasible level while for the case 3 and 4 (the ports number is different) the SGCS is severely influenced. It means that the zero-padding is kind of useful for subband number generalization but useless for port number generalization.

For case 2, zero-padding is feasible for subband number generalization while its performance degrades dramatically in port number generalization.

On contrast, the pre-processing AI/ML model performs even better when the inferring data sets are different. It is because that the dataset in case 2-4 is less complex than case 1. For the pre-processing AI/ML model, even the new data in case 2-4 is unaware, the coefficients projected on some angle-delay pairs are familiar and well trained. Therefore, from case 1 to case 4, the channel matrices are simpler and the subband number is less, which disadvantages in normal AI/ML model due to the unknown of the new data but advantages to pre-processing AI/ML model due to the simplification of channel environment.

For case 2, pre-processing performs well for both subband number generalization and port number generalization.

Also, we evaluate the SE of the four cases and the results are shown below. For the baseline case, there are 2.63% gain loss between pre-processing AI/ML model and normal AI/ML model, which is also seen in SGCS. It is because that some information is lost in the angle-delay compression. So, without zero-padding, the normal AI/ML model performs better when the training data and the inferring data have the same dimensions.

The gain of pre-processing AI/ML model compared with the normal AI/ML model (180bits)

|  | Payload | SE gain (%) |
| --- | --- | --- |
| Case 1 (13 subbands and 32 ports) | 175 | -2.63% |
| Case 2 (10 subbands and 32 ports) | 173 | ~0% |
| Case 3 (13 subbands and 16 ports) | 171 | 21.46% |
| Case 4 (10 subbands and 16 ports) | 169 | 49.45% |

When subband number is different between training and inferring, the SE gains of these two methods are almost the same. The loss from the zero-padding in subband is equal to the loss from angle-delay compression. However, when port number is different between training and inferring, the SE gains is obvious.

In the simulation above, the training data set and inferring data set are independent, which means no information about the inferring data set can be observed in the training stage. It may cause the pre-processing AI/ML model superior because the normal AI/ML model can improve the zero-padding performance with fine-tuning based on data set from case 2-4 or even training with mixed data set. However, for each subband number, or even for each combination of subband number and port number, the corresponding data set is needed in training stage. It is neither effective nor feasible. Instead, the pre-processing AI/ML has no such problem and the performance can be improved further with more angle-delay bases selected.

Besides, some other methods can also be considered like grouping. Zero-padding focuses on the cases where the dimension of training data is larger than the inferring data. In turn, grouping can be used to deal with the cases where the dimension of training data is smaller than the inferring data. For example, an AI/ML model is trained with the data set from 16 ports and in the case of 32 ports, the 32 ports are divided into 2 groups with 16 ports in each group. The data in each group can be compressed independently by a 16-port AI/ML model and report together. In such a case, the performance of the AI/ML model is guaranteed while the overhead may increase. The further study is necessary.

Study the following three methods for generalization of input dimension

- Option 1: use large dimension AI/ML model in small dimension cases: zero-padding

- Option 2: use small dimension AI/ML model in large dimension cases: grouping

- Option 3: use pre-processing to fix the input dimension: angle-delay domain compression

Generalization of output scalability

Similar to input dimension, AI models with different output dimensions also need to be trained independently. To generalize different payload without training a new AI model, we use payload truncation for different payload so the length of encoder output can be fixed. As shown below, the output of the encoder is cut out from the beginning to the specific payload length. After truncation, the truncated output is sent to the decoder.

The schematic of payload truncation.

When the AI model is trained, the loss function is set to include the correlation of all decoder output. We give a weight for each decoder and accumulate the correlation of each decoder output with the weight as a total correlation. The weight is trained with the decoder. We choose four different payload and use the dedicated model for each payload as baseline. We train the joint encoder with different combination of payloads. For each payload combination, only the decoder corresponding to the given payload is used. The SGCS of each joint encoder is shown below.

Baseline: four dedicated models of which the payloads are 223, 199,176 and 132 bits.

Case 1: one joint encoder and two decoders of which the input sizes are 223 and 176 respectively.

Case 2: one joint encoder and three decoders of which the input sizes are 223,199, and 176 respectively.

Case 3: one joint encoder and three decoders of which the input sizes are 223, 176 and 132 respectively.

The SGCS of different payload truncation methods.

|  | 223 | 199 | 176 | 132 |
| --- | --- | --- | --- | --- |
| Baseline | 0.922 | 0.913 | 0.902 | 0.871 |
| Case 1 | 0.915 |  | 0.901 |  |
| Case 2 | 0.911 | 0.908 | 0.9 |  |
| Case 3 | 0.898 |  | 0.887 | 0.867 |

Compared with the case 1 and case 2, for the same span of decoder input size, more decoders may not influence SGCS. Compared with the case 2 and case 3, the span of decoder input size may influence the performance and the SGCS decreases obviously with increasing span. Compared with the baseline and case 1, the performance loss is tiny in reasonable span of decoder input size. Therefore, in a reasonable span of decoder input size, one common encoder can be utilized and corresponds to serval decoders based on payload truncation to save the overhead of AI model transmission and switching complexity.

Payload truncation, as a starting point, performs well in UCI payload size generalization.

In the above evaluation, CSI truncation with fixed bit number is trained. For CSI truncation with more flexible bit number, AI models need to know the bit number in training stage to optimize the performance theoretically. If a new bit number is used in inferring stage (for example, 223, 199 ,176 and 132 are used in training stage and 211 is used in inferring stage), the performance of AI model may decrease and more evaluations are needed to verify the influence.

Study CSI payload truncation for the generalization of UCI payload.

- FFS the flexible truncation strategy and training parameters for more different payload.

- FFS the payload truncation on different layers for rank > 1 cases.

Generalization of rank

Agreement For the evaluation of the AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers. - Option 1-1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference, any specific model operates on multi-layers jointly. - FFS on the reported complexity and storage - FFS: input/output type - Option 1-2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference, the model operates on multi-layers jointly. - FFS: input/output type - Option 2 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference. - FFS on the reported complexity and storage - Note: input/output type is Precoding matrix - Companies to report the setting is - Option 2-1: layer specific and rank common (different models applied for different layers; for a specific layer, the same model is applied for all rank values), or - Option 2-2: layer specific and rank specific (different models applied for different layers; for a specific layer, different models are applied for different rank values) - Option 3 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference. - FFS on the reported complexity and storage - Note: input/output type is Precoding matrix - Companies to report whether the setting is - Option 3-1: layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference), or - Option 3-2: layer common and rank specific (different models applied for different rank values; for a specific rank, the same model is applied for all layers) Other options not precluded.

The input of AI model can be raw channel matrix or eigenvector and the output of AI model can be fixed to eigenvector. If the input is raw channel matrix and the output is eigenvector, the SVD procedure is also completed by AI model. In our opinion, this is much difficult for AI model training. So, we consider the input and the output of AI model are both eigenvectors.

In case that rank number is larger than 1, we evaluate the per-rank model and per-layer model. The per-rank model means, for each rank, an independent AI model is trained. UE can use the corresponding AI model to infer the precoder of a given rank number. The per-layer AI model means an independent AI model is trained for each layer, especially, the AI model for each layer is the same. For each rank, UE can infer each layer with the single AI model, i.e., the generalization of rank number.

Two scheme of high rank AI model, per-ranks and per-layer AI models.

In the simulation, we use the same per-layer model for each layer and train the model with the dataset including all layers and only rank 2 is evaluated. For the per-rank models, one model is trained with rank 1 dataset and the other model is trained with rank 2 dataset. The former is for rank 1 CSI compression and the latter is for rank 2 CSI compression. For the per-layer model, the single model is trained with dataset from all layers. This single model can be applied for different layers and ranks.

The model sizes of the single per-layer model and each of the per-rank models are almost the same, while two models are trained for per-rank models and only one model is trained for per-layer model with the same dataset. So, the per-rank models are double size of the per-layer model.

The SGCS of per-rank models and per-layer model for rank 2.

|  | Layer 0 | Layer 1 | Average number |
| --- | --- | --- | --- |
| Per-rank model | 0.91 | 0.874 | 0.892 |
| Common model across different layers Per-layer model | 0.924 | 0.863 | 0.893 |

According to the evaluation result, per-rank model and per-layer model can achieve similar SGCS, while the total size of per-rank AI model is double as opposed to per-layer model. Also considering the flexibility of layer selection, the per-layer model is better.

Rank generalization with per-layer model can achieve similar SGCS with half model size compared with per-rank model.

To compare the performance between layer common and layer specific models, we use the same model structure for different datasets. We collect 300K samples for each layer and group them into three datasets.

Case 1: Train one common model based on total 600K samples with 300K samples from layer 0 and 300K samples from layer 1.

Case 2: Train one common model based on total 300K samples with 150K samples from layer 0 and 150K samples from layer 1.

Case 3: Train one specific model for layer 0 based on 300K samples from layer 0 and another specific model for layer 1 based on 300K samples from layer 1.

The SGCS of two layers for layer common and layer specific models

|  | Layer 0 | Layer 1 | Average number |
| --- | --- | --- | --- |
| Case 1 (common model with 600K samples) | 0.8698 | 0.7834 | 0.8266 |
| Case 2 (common model with 300K samples) | 0.8569 | 0.764 | 0.8105 |
| Case 3 (specific model with 300K * 2 samples) | 0.8532 | 0.7534 | 0.8033 |

Compared the evaluation results of case 2 and case 3, layer common model performs better than layer specific model with the same training data number for each layer. Compared the evaluation results of case 1 and case 3, the performance gain increases if all the training data for the two layers are used for layer common model training. It means, with the same dataset, layer common model can achieve better performance and the dataset for layer common model is easier to collect. Therefore, not considering other generalizations, layer common model is better than layer specific model from multi layers.

Layer common model can achieve better SGCS with the same dataset.

For rank > 1 cases, study 	Option 3-1: layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference)

- FFS how to choose the layers for training data set

- FFS how to deal with specific payload for each layer

As a consequence, the generalization can be summarized in the table as follows.

| Generalization Parameter | Comments |
| --- | --- |
| Frequency granularity and ports number | Pre-processing with delay domain and spatial domain compression can solve different input dimensions caused by various frequency granularities and port numbers. |
| Payload | Payload truncation can be used to release payload generalization of AI models. |
| Rank | Rank generalization with per-layer model can achieve 12% SE gain compared with Rel-16 Type II codebook. A unified AI/ML model is trained and applied for each layer to perform individual inference. |

Generalization of other scenarios

Carrier frequency

For the carrier frequency, we evaluate 2.2GHz, 3.5GHz, 5.5GHz for rank 1 with entire AI model. In the simulation, the antenna configuration is [8 8 2 1 1] and for each polarization, four adjacent vertical antennas are mapped into one TXRU with fixed 105 degrees DFT beam, i.e., a fixed analogy precoder is used. The total TXRU number is 32 and only rank 1 is considered. The total subband number is 13 with 4 PRB’s per subband. The evaluation results are shown below.

The SGCS for different frequency carrier.

The gain of average SE for different frequency.

According to the evaluation result, the SGCS and spectral efficiency among cases with different carrier frequency are similar to each other. Since the carrier frequency is all below 6GHz and the UE speeds are all 3Km/h, the influence of carrier frequency to channel state is tiny. The AI model perform well in carrier frequency generalization.

AI model performance does not degrade when a generic model (non-optimized for a specific area/cell) trained for a frequency is applied to another frequency.

Scenarios

For generalization across different scenarios, we focus on UMi, UMa and InH. We train AI model with UMi samples and use it in UMi and UMa scenario, respectively. Also, the SGCS of eType2 is calculated for different scenarios. The evaluation results are shown below.

The SGCS in UMi and Uma.

|  | AI model trained based on UMi data | eType II codebook |
| --- | --- | --- |
| UMi | 0.91 | 0.831 |
| UMa | 0.913 | 0.839 |

According to the evaluation result in the table above, the model trained by the UMi-based data set offers a fairly high channel SGCS in both UMi and UMa scenarios.

For a generic model (non-optimized for a specific area/cell), AI model performance does not degrade when generalized from UMi to UMa.

Then, we construct a synthetic dataset with samples from UMi and InH with different ratio including entire UMi dataset and InH dataset. The total number of samples in each dataset is fixed to 300000. The SGCS of each dataset composition is shown in the table below.

The SGCS of AI model with different training dataset composition in InH and UMi.

| Training dataset composition | [300000, 0] | [225000, 75000] | [150000, 150000] | [75000, 225000] | [50000, 250000] | [25000, 275000] | [10000, 29000] | [0, 300000] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| InH | 0.94780 | 0.94907 | 0.94953 | 0.94520 | 0.94660 | 0.93090 | 0.87230 | 0.68597 |
| UMi | 0.74548 | 0.84435 | 0.87930 | 0.90281 | 0.90528 | 0.90778 | 0.90879 | 0.90933 |

According to the evaluation results, the model trained by UMi dataset independently behaves worse in InH scenario and vice versa. The models trained by dataset constructed with mixed InH-based and UMi-based data behave well for both scenarios, even not as good as with the dataset from one entire scenario. It is shown that, the increasing number of correct samples in a mixed dataset can improve the performance and the wrong samples do not influence the performance. So, the AI model can deal with different scenarios by mixing the sample from different scenarios into one dataset.

Also, In comparison between the dataset composition [225000 75000] and [50000 250000], the SGCS for InH is similar. However, the SGCS performed by the former is worse than that by the latter. It is because that the channel state of InH is simple and 50000 samples are enough. The extra InH samples cannot provide more gains. However, the channel state of UMi is much more complicated, reducing the number of UMi samples can lead to severe performance degradation.

For a generic model (non-optimized for a specific area/cell) AI model can generalize across different scenarios with a mixed dataset. A reasonable mixing ratio can provide better performance for each scenario.

Indoor/outdoor

For generalization between indoor users and outdoor users, we use data from different indoor/outdoor ratio to train two AI models. One is for 0.8 indoor and 0.2 outdoor and the other one is 0.2 indoor and 0.8 outdoor. Then we settle them in scenarios with different indoor/outdoor ratio, including 0.8/0.2, 0.5/0.5, and 0.2/0.8. The evaluation results are shown below. For each case, there are two ratios and the former is the indoor ratio of training data and the latter is the indoor ratio of deployment environment.

The SGCS of different indoor/outdoor scenarios

According to the evaluation results, no matter which training data set is used, the SGCS increases with the indoor ratio decreasing from 0.8 to 0.2. Since the floor of indoor user is random, it is more difficult to train the model for indoor user than that for outdoor user. So, the learning results of AI model descend when there are more indoor users. For the same deployment scenario, the AI model trained with 0.8 indoor ratio data performs better than the one trained with 0.2 indoor ratio data. It is because the AI model trained with more indoor users has learnt more complicated channel information, offering a better result.

The gap between the two AI models in case of 0.2 indoor user ratio is about 0.02-0.03 and in case of 0.8 indoor user ratio is about 0.01-0.02. The gap decreases when the deployment scenario becomes more severe i.e., there are more indoor users. It can be seen that, the SGCS calculated in more complicated deployment scenario decreases if the AI model is trained with the data collected in a simpler scenario.

Then, we set the AI model in the SLS system and the evaluation results are shown below. The tendency of each SGCS curve is similar but the gaps among all the curves are different.

The SE gain of different indoor/outdoor scenarios

According to the evaluation results of SE, it can be seen that AI model trained in complicated channel environment (more indoor users) performs better in the simple channel environment (more outdoor users) and vice versa. Nevertheless, even in the simple channel environment, the performance is slightly worse than that of AI model trained in complicated environment.

For a generic model (non-optimized for a specific area/cell), AI model trained in complicated channel environment (more indoor users) has good generalization ability.

The performance of AI model depends on the deployment environment

Antenna spacing

Since different antenna configurations mean different channel state with different beam width, the training data with different antenna configurations can lead to various spatial characters. And, since the encoder and decoder focus on learning the channel state, different antenna configuration can lead to different inference results. We consider the antenna spacing first.

In the simulation above, we use the antenna spacing [0.8 0.5] at gNB side, which means the space between two antenna elements in vertical is 0.8 wave length and in horizontal is 0.5. To verify the generalization of antenna size, two cases are compared with different antenna spaces. We use the training dataset with channel fading matrices based on 0.8 wave length antenna as baseline and compare the training dataset with channel fading matrices based on 0.5 wave length. Both cases are simulated in the environment with [0.8 0.5] antenna spacing. The evaluation results of entire AI model are shown below.

The SGCS of entire AI models based on different training dataset.

The gain of average SE of entire AI models based on different training dataset.

From the evaluation results, we can find that, in case that antenna space is 0.8 wave length, there is almost 4% average SE loss if training dataset is constructed with channel fading matrices based on 0.5 wave length antenna.

For a generic model (non-optimized for a specific area/cell), there is obvious performance loss for antenna spacing mismatch of training data.

Also, we evaluate the influence of the antenna spacing to the AI models with pre-processing, i.e., the small AI models with spatial domain and frequency domain compression as discussed in 2.2.1 and the evaluation results are shown below.

The SGCS of small AI models based on different training dataset.

The gain of average SE of small AI models based on different training dataset.

According to the evaluation results, there are tiny performance loss between two cases. For the small AI model with pre-processing, since the beam and delay are restricted in pre-processing, the performance loss caused by mismatching dataset can be omitted. Therefore, the small AI may have a better generalization performance.

For a generic model (non-optimized for a specific area/cell), the influence of mismatch of training data may be reduced by pre-processing.

Antenna virtualization

Next, we consider the influence of antenna virtualization. In the simulation above, we use antenna configuration [8 8 2] with 4 successive vertical antenna elements mapping to one TXRU with a fixed 105 degrees DFT beam. We draw other two cases of antenna configuration [2 8 2] without antenna virtualization as contrast.

Case 1: AI model is trained with dataset constructed by antenna configuration [8 8 2] and used in the case of antenna configuration [8 8 2].

Case 2: AI model is trained with dataset constructed by antenna configuration [8 8 2] and used in the case of antenna configuration [2 8 2]

Case 3: AI model is trained with dataset constructed by antenna configuration [2 8 2] and used in the case of antenna configuration [2 8 2]

The SGCS’s of the three cases and Rel-16 Type II codebook with antenna configuration [8 8 2] and [2 8 2] are shown below.

The SGCS of three cases and Rel-16 Type II codebook

According to the evaluation results, firstly, the SGCSs of the three AI cases are at least 0.07 higher than that of Rel-16 Type II codebook. From the comparison between the Rel-16 Type II codebook with these two antenna configurations, the channel state is easier to learn for antenna configuration [2 8 2], while more difficult to learn for antenna configuration [8 8 2].

The AI model trained with antenna configuration [8 8 2] has similar SGCS performance in both antenna configurations [8 8 2] and [2 8 2]. It seems that, when antenna configuration changes from [8 8 2] to [2 8 2], the original AI model trained with antenna configuration [8 8 2] can still work properly. However, considering the transmission ability, antenna configuration [8 8 2] can provide more spatial information than antenna configuration [2 8 2]. The SE may decrease if the same AI model is directed used in the case of antenna configuration [2 8 2].

The AI model trained with antenna configuration [2 8 2] performance better in antenna configurations [2 8 2]. So, in the case of antenna configuration [2 8 2], the AI model trained with antenna configuration [2 8 2] may achieve similar SE performance as the AI model trained with antenna configuration [8 8 2] in the case of antenna configuration [8 8 2].

For a generic model (non-optimized for a specific area/cell), SGCS performance of AI model may degrade slightly from 128 antennas with virtualization to 32 antennas without virtualization. While the SE performance may degrade heavily due to the less antennas.

For a generic model (non-optimized for a specific area/cell), in the case of 32 antennas, AI model trained with 32 antennas may have similar SE performance compared with AI model trained with 128 antennas and settled in the case of 128 antennas, which is needed to be further studied.

As a consequence, the generalization performance of various scenarios is shown below.

| Generalization Parameter | Comments |
| --- | --- |
| Carrier Frequency | AI model performance does not degrade when a generic model (non-optimized for a specific area/cell) trained for a frequency applied to another frequency. |
| Channel Scenario | For a generic model (non-optimized for a specific area/cell) AI model can generalize across different scenarios with a mixed dataset. A reasonable mixing ratio can provide better performance for each scenario. |
| Indoor/outdoor | For a generic model (non-optimized for a specific area/cell), AI model trained in complicated channel environment (more indoor users) has good generalization ability of simple channel environment (more outdoor users) |
| Antenna Spacing | For a generic model (non-optimized for a specific area/cell), there is obvious performance loss for antenna spacing mismatch of training data. |
| Antenna virtualization | For a generic model (non-optimized for a specific area/cell), SGCS performance may degrade slightly and SE performance may degrade heavily for large number antennas with virtualization applied to small number antennas with virtualization. |

For a generic model (non-optimized for a specific area/cell), AI models perform well in generalization of carrier frequency, channel scenario, indoor/outdoor ratio.

For a generic model (non-optimized for a specific area/cell), AI models perform bad in antenna spacing and antenna virtualization, which can be further studied.

Evaluations on ground-truth quantization

- Ground-truth CSI reporting is an essential procedure in data collection for CSI compression. In last meeting, FL proposed to study high resolution scalar or codebook quantization methods for ground-truth CSI, and several schemes have been mentioned in the agreement. To this end, we consider CSI quantized via Float32 as our baseline scheme, and train different models based on Float16, high resolution R16 Type-II codebook, and regular resolution R16 Type-II codebook quantized CSI data. All models trained on quantized CSI are tested on Float32 format data to see the performance, and our results towards different methods in table below.

Results of different methods for ground-truth CSI quantization.

|  | Model trained on float32 format quantized data (baseline) | Model trained on float16 format quantized data (baseline) | Model trained on Legacy codebook quantized data (L=12, M=6, beta=1.0) | Model trained on Legacy codebook quantized data (L=4, M=4, beta=0.75) |
| --- | --- | --- | --- | --- |
| SGCS results tested on float32 format data | 0.8710 | 0.8661 | 0.8549 | 0.8192 |

From the table, we could observe that there is only a slight performance loss between ground-truth quantized in float32 and float16, and high-resolution legacy codebook also provide a satisfying performance in quantizing ground-truth CSI. However, when the parameters reduce to a conventional setting (i.e., from L=12, M=6, beta=1.0 to L=4, M=4, beta=0.75), the performance loss is obvious. Considering that the overhead of quantizing ground-truth via high resolution codebook is much lower than that of quantization via float16 (hundreds of bits versus thousands of bits), we believe that high resolution codebook is a promising solution to ground-truth CSI quantization and reporting in CSI compression.

High resolution R16-eType II codebook with large L, M, beta (for example, L=12, M=6, beta = 1.0) performs well for ground-truth CSI quantization.

Evaluation on CSI feedback quantization

There is one agreement in RAN #110b-e to further study quantization of CSI feedback, including different quantization/dequantization methods and the corresponding training approaches. Besides, the FL’s plan for RAN #111 meeting also mentioned some issues on quantization/dequantization for training collaboration type 2/3. Therefore, we would like to discuss the quantization issue for CSI compression systematically in this subsection. Quantization in CSI compression refers to the mapping from float-format CSI generation output to bit-format UCI payload, often placed on the tail of CSI generation part; Dequantization in CSI compression refers to the reverse procedure on the beginning of CSI reconstruction part, i.e., mapping from bit-format UCI payload to float-format decoder input.

There are usually two categories of quantization/dequantization methods, i.e., scalar quantization and vector quantization. In scalar quantization, each number in the float-format sequence will be mapped to several bits. In vector quantization, each sub-sequence of float-format sequence will be mapped to several bits. It could be seen that scalar quantization is a specific case of vector quantization. The averaged quantization bit can be used to describe the quantization effect of a specific quantization method, which is defined as the averaged bit to quantize a float number. For example, if 180bits are used to quantize a sequence of 80 float variables, the averaged quantization bit is 180bit/80float=2.25bits/float.

To define a specific scalar quantization rule, we can directly define the number of bits assigned to each float. For example, we can use a vector [2, 2, …, 2, 3, …, 3, …, 4] to express a scalar quantization method, which assigns 2 bits to the first several float number, 3 bits to the next several float number, and 4 bits to the last several float number. The most trivial scalar quantization method is to uniformly assign K bits for all float numbers in a sequence. The definition of a vector quantization method will be a little more complicated. The whole sequence to be quantized will usually be partitioned into several segments, as it is difficult to directly quantize the whole sequence. Otherwise, there will an extremely large quantization codebook of size. For example, 80 float variables can be partitioned into 16 sub-sequences, each of which is of size 5. Correspondingly, we can set 16 quantization codebooks, each of which will be used to quantize one segment. It is also quite common to assign a uniform codebook for all sub-sequences to save the storage space. Each column in the codebook, i.e., a codeword will be a quantization candidate for the input. The quantization procedure is to select one codeword in the codebook that most represents the input, and the most common criteria is to select the one with the least MSE distance to the input.

For quantization non-aware training, quantization effect will not be considered during training stage, and the float-format variables will be directly passed from CSI generation part to CSI reconstruction part without any loss. After the model is trained, quantization module will be added to quantize and recover the intermediate result (CSI generation output). For quantization-aware training, CSI compression model will be trained under the consideration of the quantization loss of CSI generation output. In addition, the quantization codebook for scalar or vector quantization can be set fixed or optimized during training of CSI compression model. We will compare the performance of current quantization/dequantization methods as well as different training approaches in the following.

Inference performance of quantization non-aware training

In figure above, inference performance of quantization non-aware training is presented, where the length of CSI generation output is set 80. After the model is firstly trained without considering quantization, various amounts of bits are considered to quantize the CSI generation output during inference stage. When the CSI generation output is quantized by 320 bits, the performance is quite close to ideal one, while when the quantization bits decrease to 240, the SGCS result reduces by 5% in absolute value. However, for the case of quantizing by 180bits and 80bits, the model is almost not workable due to the very low SGCS performance.

Quantization non-aware training only achieves good performance when the averaged quantization bit is large (e.g., >= 4bits/float). When the averaged quantization bit is small (e.g., <= 2bits/float), the performance loss is significant.

Comparisons of different quantization methods (all models consider using 180bits to quantize 80 float-format variables).

In figure above we compare different quantization methods, where all models consider using 180bits to quantize 80 float-format variables. We can see that vector quantization with optimized codebook achieves the best SGCS performance among all candidates, while scalar quantization with fixed codebook ranks second with ~0.9% loss in SGCS. Interestingly, vector quantization with random initialized and fixed codebook is slightly inferior to scalar quantization. Last but not least, quantization non-aware training with the same setting demonstrates a much lower performance, which may suggest it is not a good choice.

Vector quantization with optimized codebook can achieve slightly better performance (e.g., by about 0.009 in SGCS in our considered configurations) than scalar quantization with fixed codebook.

Performance of vector quantization with randomly initialization and fixed codebook can be slightly inferior to that of scalar quantization with fixed codebook (e.g., by about 0.0065 in SGCS in our considered configurations).

Performance of quantization non-aware training could be significantly lower than that of quantization aware training (more than 0.1 in SGCS in our considered configurations).

To our understanding, quantization method at UE side and dequantization method at NW side should be aligned anyhow for training collaboration type2 and 3. For training collaboration type2, if quantization/dequantization methods are not aligned at training stage, we find it difficult for the model to learn anything from the data, i.e., the performance stays in a randomly initialized level. Furthermore, if the length for the floating output is not aligned, the gradients cannot properly back propagate to the CSI generation part. For training collaboration type3, we also find that the model cannot even converge to a reasonable performance (loss in SGCS >= 0.1 compared with the case of aligned quantization/dequantization) if quantization/dequantization methods are not aligned. Therefore, we have the following proposal:

Quantization method at UE side and dequantization method at NW side should be aligned for training collaboration type2 and 3 to achieve a satisfying performance.

Evaluations on Type 2: Joint training

The interaction approach of training collaboration type 2 is to exchange necessary training information over the air to enable the training procedure. More precisely, the whole procedure contains three main steps: 1) UE computes the forward-propagation result on CSI generation model based on collected data, and sends the (last layer) forward-propagation result together with the input data to gNB; 2) gNB completes the remaining forward-propagation computation based on the received forward-propagation result, computes loss function based on the received data, and  back propagates through CSI reconstruction part to acquire the gradients on the first layer of CSI reconstruction model. The backward propagation results, i.e., the gradients on the first layer of CSI reconstruction model, will be then sent back to the corresponding UE. UE completes the remaining back propagation procedure for CSI generation part based on the received gradients. 3) UE and gNB update CSI generation/reconstruction part based on the exchanged information. The above procedure will be repeated each batch, until the whole training procedure ends.

In training collaboration type 2, it is not necessary for UE and gNB to fully align their model structure. Namely, it is feasible for the training procedure to converge to a reasonable (usually not optimal) performance with CSI generation part and CSI reconstruction part having totally different model structure, e.g., MLP and CNN for CSI generation part while Transformer for CSI reconstruction part. But from our view, it is necessary to align the quantization method at CSI generation model and dequantization method at CSI reconstruction model. Otherwise, the trained model could not be properly validated, and there would a risk in training failure.

It is proposed to study the combinatorial problem of models in CSI compression in RAN #110, i.e., if dedicated models (CSI generation part and CSI reconstruction part) are made for specific scenarios/configurations, the number of required models for various conditions could be prohibitively high, which makes model management complicated. Basically, Combinatorial problem occurs in all generalization issues. However, given the fact that evaluation methodology for generalization is still under discussion, proposal in last meeting picked out one specific combinatorial issue, i.e., the support of one common CSI reconstruction part to multiple CSI generation parts of different UEs (and vice versa) to study in future meeting. In the following, we would like to report our results towards the above issue.

Training procedure of common CSI reconstruction part to multiple CSI generation parts of different UEs

We start with support of one common CSI reconstruction part to multiple CSI generation parts of different UEs. The basic procedure of the above method is: 1) the involved UEs compute their local forward-propagation results based on their local collected data (there could be some problems here, which will be discussed later.) and report them to gNB. The reported content contains not only the result of forward propagation but also the labels for loss function computation, which is paired with the forward-propagation results in reporting. 2) gNB computes the loss function as well as the gradients for back-propagation for each UE respectively and transmits the gradients to each UE. Note that different UEs’ gradients are computed based on their reported information, which are generally different for different UEs. 3) Each part of models completes the back-propagation procedure and updates the weights according to their gradients. Note that the gradients for CSI reconstruction part would take loss function for all UEs into account, thus achieving a common reconstruction part for multiple generation parts. For two-sided models, the ideal performance of joint training at a single entity and joint training at Network and UE side are almost the same. By “ideal”, we mean that the training data, training policy, training hyperparameters, optimizer, etc. are perfectly aligned for CSI generation and reconstruction part.

When UEs report their forward-propagation result to gNB, it could be challenging to strictly align the data from all UEs, as the amount of training data for different UEs is highly likely to be different. Such issue could bring some inconvenience to the joint training procedure. One way to relieve this problem is to broadcast training data in advance to all UEs, which is illustrated in step0 in Figure 16.

Then we introduce our simulation settings: joint training for one common CSI reconstruction part to two/three CSI generation parts of different UEs is considered, where the basic model structures for CSI generation parts are Transformer, CNN, and MLP, while a Transformer CSI reconstruction part is adopted. Each involved UE generates its forward-propagation results based on datasets with the same configurations (i.e., the same number of subbands, antenna ports, antenna configurations, etc.). We optimize all involved CSI generation parts equally by using the average SGCS as the loss function. However, the optimizer and learning rate scheduling policy for different CSI generation parts are different, as UEs are considered to have their own training implementations. Joint training of one to one CSI reconstruction and generation part serves as the baseline. Results are presented in the following table.

Performance of one common CSI reconstruction part to two/three CSI generation parts of different UEs

|  | Transformer CSI Generation part | CNN CSI Generation part | MLP CSI Generation part |
| --- | --- | --- | --- |
| Baseline of Rel-16 Type II | 0.7950 |  |  |
| Baseline one-to-one model | ~0.87 | / | / |
| Transformer CSI reconstruction part to Transformer and MLP CSI generation part | 0.8601 | / | 0.8137 |
| Transformer CSI reconstruction part to Transformer and CNN CSI generation part | 0.8599 | 0.8485 | / |
| Transformer CSI reconstruction part to Transformer, CNN, and MLP CSI generation part | 0.8475 | 0.8364 | 0.8125 |

From Table 12, it could be observed that there are certain level performance degradations for one common CSI reconstruction part to two/three CSI generation parts of different UEs. As the number of CSI generation parts increases, the performance degradation also enlarges. Considering one common CSI reconstruction part matching three CSI generation parts, SGCS performance gain of AI/ML models over Rel-16 Type II codebook reduce from 0.075 to 0.052, i.e., losing about 30% performance gain. Besides, performance of CSI generation part with MLP structure is lower than those of CSI generation part with CNN and Transformer structure, indicating that certain structures are more suitable for CSI compression.

One common CSI reconstruction part could be trained to match multiple CSI generation parts of different UEs in training collaboration type 2 at the cost of some performance loss.

Considering one common CSI reconstruction part matching three CSI generation parts, SGCS performance gain of AI/ML models over Rel-16 Type II codebook reduces from 0.075 to 0.052, i.e., losing about 30% performance gain.

Training procedure of one common CSI generation part to multiple CSI reconstruction parts of different networks.

Next, we move on to the case of one common CSI generation part to multiple CSI reconstruction parts of different networks. We believe that such case is a little bit simpler than the other one, since the training data is naturally aligned among all involved CSI reconstruction parts. The basic procedure is similar: 1) UE computes the forward-propagation result based on the local data, which will be transmitted to all involved networks. 2) Involved networks compute the loss and the back-propagation results based on the reported information. Back-propagation results are then sent to UE. Note that the sent back-propagation results should be kept in the same order with forward-propagation results. 3) Finally, involved CSI reconstruction/generation parts update their own weights according to the computed gradients. Although technically feasible, we find the joint training of one common CSI generation part to multiple CSI reconstruction parts difficult to be implemented online, as one UE could not connect to multiple networks simultaneously.

We consider one common CSI generation part to two/three CSI reconstruction parts in our experiments. The basic model structures for CSI reconstruction parts are Transformer, CNN, and MLP, while a Transformer CSI generation part is adopted. All involved CSI reconstruction parts equally by using the average SGCS as the loss function. The optimizer and learning rate scheduling policy for different CSI reconstruction parts are also different, as networks are considered to have their own training implementations.

Performance of one common CSI generation part to two/three CSI reconstruction parts of different networks

|  | Transformer CSI reconstruction part | CNN CSI reconstruction part | MLP CSI reconstruction part |
| --- | --- | --- | --- |
| Baseline of Rel-16 Type II | 0.7950 |  |  |
| Baseline one-to-one model | ~0.87 | / | / |
| Transformer CSI generation part to Transformer and MLP CSI reconstruction part | 0.8526 | / | 0.8350 |
| Transformer CSI generation part to Transformer and CNN CSI reconstruction part | 0.8633 | 0.8582 | / |
| Transformer CSI generation part to Transformer, CNN, and MLP CSI reconstruction part | 0.8563 | 0.8525 | 0.8434 |

Results in Table 13 demonstrate that one common CSI generation part to multiple CSI reconstruction parts of different networks also suffer from some performance loss, which enlarges as the number of supported CSI reconstruction parts increases. Considering one common CSI generation part matching three CSI reconstruction parts, SGCS performance gain of AI/ML models over Rel-16 Type II codebook reduce from 0.075 to 0.061, i.e., losing about 19% performance gain. Interestingly, the performance loss in common CSI generation part is generally lower than that in common CSI reconstruction part, which needs further study and verification.

One common CSI generation part could be trained to match multiple CSI reconstruction parts of different networks in training collaboration type 2 at the cost of some performance loss.

Considering one common CSI generation part matching three CSI reconstruction parts, SGCS performance gain of AI/ML models over Rel-16 Type II codebook reduce from 0.075 to 0.061, i.e., losing about 19% performance gain.

One major concern for joint training is the huge overhead of exchanged information. According to the procedure, the forward- and backward- propagation information should be exchanged each batch. Besides, the training data should also be exchanged during training to serve as the labels. The overall overhead could be roughly computed as

Overhead ≈ # of epoch*(forward-propagation information + back-propagation information + input data)

Suppose the size of each forward- and backward-propagation sample is ~1/10 of the input (e.g., 13*32*2 floats are typically compressed into 50 floats without quantization), the total overhead could still be ten times of those for separate training and model transferring depending on the number of epochs. It is worth pointing out that there are still approaches to further reduce the overhead, but it is extremely challenging to reduce the over-the-air overhead to the similar level of separate training or model transfer.

Overhead in information exchange for training collaboration type 2 grows linearly with the number of iterations at training stage.

Evaluations on Type 3: Separate training

During our evaluation of training collaboration type3, we consider sequential training starting with UE side training. Meanwhile, we find the results of sequential training starting with UE side training almost the same with the those of sequential training starting with network side training. Therefore, we believe our observations will hold for both cases. Besides, we think the term “starting with UE side training” or “starting with network side training” are not very suitable to cover all cases for separate training. For example, network and UE could train their model separately in advance, and UE then share the input/output for CSI reconstruction part to network to help to finetune the model trained by network. In such case, it is difficult to say the separate training procedure strictly starting at which side. We propose to define “active” and “passive” side for separate training, where the “active” side actively shares the data for the model at “passive” side to align them. Then we can clearly categorize the interaction approaches for separate training into two types: 1) UE is active side and network is the passive side; 2) network is active side and UE is passive side. Although we propose to modify the terms, we still use “sequential training starting with UE side training” to depict our interaction approach in the remaining part of this subsection.

An illustration of separate training procedure.

Detailed procedure of sequential training starting with UE side training is presented as follows: (also illustrated in Figure 18, where “Encoder” refers to CSI generation part and “Decoder” refers to CSI reconstruction part.)

Step 1: The encoder is trained firstly at UE or a server at UE side using collected dataset0. Specifically, a complete model containing both encoder and decoder is trained and then the encoder is picked out for separate training. The decoder obtained in step 1 is termed as decoder0.

Step 2: UE passes dataset1 into encoder to obtain the encoded feature1, and combines the dataset1 (encoder input) and encoded feature1 (encoder output) into the exchanging dataset, i.e., the encoder output serves as the label of encoder input.

Step 3: UE transmits the exchanging dataset to gNB.

Step 4: gNB utilizes the exchanging dataset to train the decoder via supervised learning. The decoder obtained in this step is termed as decoder1.

Step 5: Test the SGCS of joint inference of encoder and decoder based on dataset2.

Results for separate training in CSI compression.

| Samples in exchanging dataset | Joint training with 300000 samples in step1 | 600000 | 300000 | 100000 | 50000 | 25000 | 10000 | 5000 | 2500 | 1000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test SGCS for setting0 | 0.830 | 0.832 | 0.827 | 0.815 | 0.804 | 0.793 | 0.776 | 0.761 | 0.733 | 0.650 |
| Test SGCS in setting1 | / | / | 0.830 | / | / | / | / | / | / | / |
| Test SGCS in setting2 | / | / | 0.800 | / | / | / | / | / | / | / |
| Test SGCS in setting3 | / | / | 0.712 | / | / | / | / | / | / | / |

The table above presents the results and the details of each setting are given as follows:

- Setting0: Decoder1 and decoder0 share the same model design as well as the hyperparameters in training. The dequantization method in decoder1 also matches the quantization method in encoder. Note that Setting0 is an ideal baseline as gNB could not know any information about the decoder0 which is at UE side.

- Setting1: Decoder1 and decoder0 share the same model backbone structure, but decoder1 has more parameters than decoder0, i.e., decoder1 is an enlarged decoder0. The dequantization method in decoder1 also matches the quantization method in encoder.

- Setting2: Decoder1’s structure is completely different from decoder0. But the dequantization method in decoder1 matches the quantization method in encoder.

- Setting3: Decoder1’s structure is completely different from decoder0 (but the same with the decoder1 in Setting2), and the dequantization method in decoder1 does not match the quantization method in encoder.

Generally, performance of separate training could reach that of joint training if the number of exchanged data samples is large enough, i.e., similar level to the scale of training data, and some key information of encoder and decoder is aligned, such as the quantization and dequantization method. In addition, we find that it is possible to train a pair of encoder and decoder subject to different structures, e.g., an MLP encoder and a Transformer decoder, to a reasonable performance. It is not necessary to fully align the model structure of encoder at UE and decoder at network. Finally, the quantization and dequantization methods play an important role in separate training. Our simulation shows that when the quantization approach at UE and dequantization approach at network do not match, there will be an unacceptable performance loss for the model.

If the model structure is not aligned (e.g., dequantization method at decoder and the quantization method in encoder could not match), there will be an obvious performance loss compared with that in case where the dequantization and quantization method are matching.

Procedure of training one common CSI reconstruction part to multiple CSI generation parts of different UEs for sequential training starting with UE side training.

It is possible for separate training collaborations to develop one common CSI generation/reconstruction part to multiple CSI reconstruction/generation parts. For sequential training starting with UE side training, it is trivial to realize one common CSI generation part to multiple CSI reconstruction parts of different networks, since it is natural for UEs to broadcast the input/output of the same CSI generation part to multiple gNBs. Supporting one common CSI reconstruction part to multiple CSI generation parts of different UEs is also not difficult. One gNB could collect paired input/output data from multiple UEs and then train one CSI reconstruction part based on a mixed dataset of all collected data. The discussion for sequential training starting with gNB side training follows the similar principle and the procedures are almost in reciprocity with those for sequential training starting with UE side training. So the detailed analysis is omitted here for brevity.

However, as the UEs tend to have different model structure for CSI generation part and it is also hard to strictly align the data for exchange among UEs, supporting one common CSI reconstruction part to multiple CSI generation parts of different UEs still takes the risk of obvious performance degradation. To this end, we try to verify the performance of the above case. Consider UE-active separate training with three UEs, each of which uses different backbone structures for their CSI generation part, i.e., Transformer, CNN, and MLP. Each UE reports 10,000, 50,000, or 300,000 data samples for separate training, and the gNB combines all reported data to train the CSI reconstruction model. In this experiment, we consider separate training with one to one CSI generation/reconstruction part (assumed to exchange 300,000 data samples between UR and gNB) to serve as the baseline. Various combinations of amounts of reported samples are simulated, and the results are presented in the table below.

Performance of one common CSI reconstruction part to multiple CSI generation parts of different UEs for UE-active separate training.

|  | Transformer CSI generation part | CNN CSI generation part | MLP CSI generation part |
| --- | --- | --- | --- |
| SGCS for Baseline | 0.8528 | 0.8424 | 0.8025 |
| SGCS for Setting0 | 0.8128 | 0.8021 | 0.7637 |
| SGCS for Setting1 | 0.8358 | 0.8303 | 0.7942 |
| SGCS for Setting2 | 0.8434 | 0.7999 | 0.7631 |
| SGCS for Setting3 | 0.8439 | 0.7957 | 0.6983 |
| SGCS for Setting4 | 0.7313 | 0.8016 | 0.7938 |

|  | Data samples from Transformer CSI generation part | Data samples from CNN CSI generation part | Data samples from MLP CSI generation part |
| --- | --- | --- | --- |
| Setting0 | 50,000 | 50,000 | 50,000 |
| Setting1 | 300,000 | 300,000 | 300,000 |
| Setting2 | 300,000 | 50,000 | 50,000 |
| Setting3 | 300,000 | 50,000 | 10,000 |
| Setting4 | 10,000 | 50,000 | 300,000 |

The table on top side of  Table 15 presents the SGCS results for different settings, where each of the three UE shares different amount of data to gNB for separate training. Specific data amount for each setting is given in the table on bottom side of Table 15. Compared with one-to-one model, one common CSI reconstruction part to multiple CSI generation parts of different UEs demonstrates a degraded performance. Such degradation gets worse as the amount of exchanged data decreases.

One common CSI reconstruction/generation part could be trained to match multiple CSI reconstruction/generation parts of different UEs in training collaboration type 3 at the cost of some performance loss (e.g., considering one common CSI reconstruction part to three CSI generation part and each UE sharing 50,000 samples with NW, the performance loss in SGCS is around 0.04).

Performance of one common CSI generation/reconstruction part to multiple CSI reconstruction/generation parts of different networks/UEs is affected by the amount of exchanged data from each network/UE.

In addition, there was a proposal in last meeting that a more general case of M CSI generation parts to N CSI reconstruction parts could be considered for both collaboration type2 and type3. To our understanding, the implementation of M CSI generation parts to N CSI reconstruction parts can be viewed as the composition of multiple common CSI generation part to multiple CSI reconstruction parts (or multiple CSI generation parts to common CSI reconstruction part). Therefore, it can be foreseen that the performance of M CSI generation parts to N CSI reconstruction parts will further decrease compared with one-to-multiple cases, and following observations still hold for all training collaborations:

Performance loss in supporting common CSI generation/reconstruction part to multiple CSI reconstruction/generation parts gets worse as the number of supported UEs/networks increases.

Evaluation methodology for Performance Monitoring

- Various performance monitoring methods for CSI compression have been proposed by the companies, which are able to offer the different trade-off between monitoring reliability, overhead, complexity and latency. Therefore, it is essential to come up with an evaluation methodology for performance monitoring methods to compare their pros/cons.

Evaluation methodology for performance monitoring

From our understanding, the overhead/complexity/latency of a specific monitoring method can be approximately computed without simulation. Therefore, we mainly discuss the reliability evaluation for a performance monitoring method. In general, if a performance monitoring method involves computing intermediate KPIs for CSI compression, how accurate the intermediate KPIs are computed can reflect its monitoring reliability. For example, by treating the SGCS between reconstructed CSI and ground-truth CSI measurement as the baseline, we can compute the SGCS gap between the monitored SGCS and the baseline SGCS to be the monitoring accuracy measurement. Such a method is easy to be implemented since it does not involve any temporal correlated CSI sequence. However, it may be too stringent to require all monitoring methods to accurately provide the real SGCS. In fact, we monitor CSI compression models mainly to make decisions on model switching or fallback. If a monitoring method could help to make correct and suitable switching/fallback decision (i.e., no false alarm and misdetection), its reliability is also acceptable. Besides, not all monitoring methods make decisions based on intermediate KPIs such as input-based monitoring. It is challenging to link a mismatch in input data distribution to any particular SGCS, which makes it hard to evaluate such kind of monitoring methods.

Another way to evaluate the monitoring reliability is to compare the switching/fallback decisions with baseline. For example, we can construct a scenario involving model switching (e.g., a UE moves across multiple scenarios) and test whether the monitoring method to be evaluated could make the switching decision at correct time. The switching algorithm can be set to monitor a series of candidate models (including the model in use) and directly pick the one with the best performance (e.g., the highest average SGCS over an observing window) as the active model. The ideal switching can be obtained via comparing the accurate SGCS of each model, and the practical switching is guided by the actual monitoring methods. We can thus compute the false alarm rate (where ideal switching does not happen but practical switching happens) and misdetection rate (where ideal switching happens but practical switching does not happen), or demonstrate the overall intermediate KPI after switching to reflect the reliability of particular monitoring methods. In the following part, we give an example of evaluating our proposed proxy model-based monitoring for CSI compression.

The accuracy of a particular monitoring method can be evaluated via testing whether this method could lead to a proper model switching. To measure what is a proper model switching, some metrics can be considered as the starting points:

- Counting the false alarm and misdetection compared with ideal switching

  - Ideal switching refers to the switching decisions guided by accurate performance KPIs of candidate models

  - false alarm refers to the case that ideal switching does not happen, but practical switching happens

  - misdetection refers to the case that switching happens but practical switching does not happen

- Comparing the SGCS after switching for each monitoring method

For the purpose of evaluating performance monitoring methods in CSI compression, the data to model a switching procedure can be collected from moving UEs across multiple scenarios.

Initial results for proxy model based monitoring for CSI compression

In our simulation, we collect the CSI data from a single UE which moves from a LoS-dominant cell to another NLoS-dominant cell. Spatial consistency is considered for data collection. We have two models to be monitored: one is trained for LoS-dominant scenario and the other is for NLoS-dominant scenario. There is a proxy CSI reconstruction model associated with each model at UE side for performance monitoring, respectively. Compared with the complicated transformer CSI reconstruction model used at NW, we consider much simpler proxy models with MLP structure, whose parameter scale and FLOPs are much smaller than those of actual models (as presented in Table 19). We believe that a simpler and smaller structure could facilitate the use of proxy models in performance monitoring. For the principle of proxy model, readers can refer to our companion material for more details [?].

Comparison of actual CSI reconstruction model at NW and proxy CSI reconstruction model at UE .

|  | Model structure | Parameter scale | FLOPs |
| --- | --- | --- | --- |
| Actual CSI reconstruction model at NW | Transformer | 8.4M | 102.9M |
| Proxy CSI reconstruction model at UE | MLP | 0.33M | 0.45M |

Figure 19 and figure 20 illustrate the performance of two proxy model, respectively, where we can find that the SGCS computed based on the proxy model can well approximate that offered by the actual model after shifting a constant bias. Therefore, the UE could switch between the two models based on the estimated SGCS (i.e., shifted SGCS of proxy model output and ground-truth CSI). Figure 21 demonstrates the SGCS after switching for each considered switching approach. Totally 1.4K samples are illustrated, where the former half 700 samples correspond to LoS scenario and the latter half 700 samples correspond to NLoS scenario. Model in use is set to the LoS model at beginning and is expected to switch to the NLoS model after scenario changes. From figure 21, we could see that using proxy model to monitor the performance is able to achieve near-ideal SGCS after switching, while non-switching scheme suffers from an obvious performance loss after the scenario changes. Although it can also be noted that proxy model triggers a false switching in LoS scenario (around 200th samples) which degrades the SGCS, its overall performance is satisfying.

SGCS comparison (left) and SGCS gap distribution (right) for a model trained in LoS dominant scenario

SGCS comparison (left) and SGCS gap distribution (right) for a model trained in NLoS dominant scenario

SGCS comparison after switching considering scenario changes from LoS-dominant case to NLoS-dominant case

Monitoring based on proxy model at UE side in CSI compression can lead to near-ideal model switching when the scenario changes.

CSI prediction

Basic assumptions for CSI prediction

In the AI-based CSI prediction design, the AI model is designed to derive the prediction of CSIs as the output of model when using the historical CSIs as the input. The block diagram of AI-based CSI prediction is illustrated in Figure 22.

The block diagram of AI-based CSI prediction.

For CSI prediction, the data for training is derived from the SLS platform.

Results for CSI prediction

In this subsection, the gain over non-AI scheme, the generalization aspects, the impact of observation window, and the monitoring results of AI-based CSI prediction are discussed.

In RAN1 #111, the following working assumption is achieved [1]:

Working Assumption

For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies.

- Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example

    - It does not imply any restriction on future specification for CSI prediction

FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

At first, in the subsubsection 3.2.1, we show the gain of AI based CSI prediction over the nearest historical CSI w/o prediction and non-AI/ML based CSI prediction approach. Then, in the following subsubsection 3.2.2, we provide the comparison between level y/z AI-based CSI prediction and level x AI-based CSI prediction by discussing the generalization aspects.

For the AI/ML based CSI prediction sub use case, if collaboration level x is reported as the benchmark, the comparison of level y/z AI-based CSI prediction and level x AI-based CSI prediction should be studied in the generalization aspects, e.g., collaboration level x is modeled as generalization Case 2, while collaboration level y/z is modeled as the generalization Case 1 or finetuning.

The gain of AI-based CSI prediction

In this subsection, the UE average spectral efficiency (SE) of scheme with AI-based CSI prediction, nearest historical CSI w/o prediction and AR-based non-AI CSI prediction is evaluated, which is provided in the following Table 20. Simulation parameters are given below:

Umi 38.901; 7 cells, 3 sectors for each cell, 10 user for each sector; carrier frequency 4GHz, subcarrier spacing 30KHz, 13 subbands (10MHz, 4RBs/subband); 32 gNB antenna ( [Mg Ng M N P; Mp Np] = [1 1 2 8 2; 2 8]), 2 UE antenna ([Mg Ng M N P; Mp Np] = [1 1 1 1 2; 1 1]); 100% outdoor UE, 30km/h, Channel type: NLOS; MU max layer number 8, traffic model full buffer; Period of CSI: 5ms; Input of AI model for CSI prediction: 15 raw historic channels in PRB; AI-based CSI compression model: Transformer model with 240 bits payload (CSI feedback payload Z); Spatial consistency is not considered.

It is shown that the AI-based CSI prediction achieves a UE average SE gain of 27.5% and 9.7% over the nearest historical CSI w/o prediction and AR-based non-AI CSI prediction, respectively.

The UE average SE of multiple CSI prediction schemes

| Term | Value |
| --- | --- |
| UE average SE (bps/Hz) of AI-based CSI prediction | 0.394 |
| UE average SE (bps/Hz) of benchmark #1 (nearest historical CSI w/o prediction) | 0.309 |
| UE average SE (bps/Hz) of benchmark #2 (AR-based non-AI CSI prediction) | 0.359 |
| Gain of AI-based CSI prediction over benchmark #1 | 27.5% |
| Gain of AI-based CSI prediction over benchmark #2 | 9.7% |

The AI-based CSI prediction achieves higher spectral efficiency when compared with the nearest historical CSI w/o prediction and AR-based non-AI CSI prediction.

The generalization of AI-based CSI prediction

The generalization describes the adaptability of an AI model to fresh data, which is one of the key capabilities for evaluating the performance of an AI model.

In this subsubsection, the generalization of AI-based CSI prediction over speeds and deployment scenarios (LOS/NLOS, Uma/Umi) are evaluated.

- The generalization of AI-based prediction over speeds

In this subsubsection, the generalization of AI-based CSI prediction over different speeds is evaluated. The corresponding simulation parameters are given below:

Simulation parameters: Uma 38.901 ,carrier frequency 2GHz, subcarrier spacing 15KHz, 32 gNB antenna ( [Mg Ng M N P; Mp Np] = [1 1 2 8 2; 2 8]), 2 UE antenna ([Mg Ng M N P; Mp Np] = [1 1 1 1 2; 1 1]), 100% outdoor UE, Channel type: NLOS, Period of CSI-RS: 4ms; Input of AI model for CSI prediction: 15 raw historic channels in PRB，the spatial consistency procedure A with 50m decorrelation distance is used where the channel updating periodicity is assumed to be 1 ms.

The AI model is trained using the data with one specific UE speed (30, 60, or 120 km/h) or mixed speeds. Then, the trained model is tested on the data with the UE speed of 30 and 60 km/h and 120km/h, respectively, to evaluate the generalization performance.

The generalization performance of AI-based CSI prediction over speeds

| Inference speed(km/h) Scheme | 30 | 60 | 120 |
| --- | --- | --- | --- |
| NMSE (dB) of using model trained at 30 km/h | -19.84 | 7.79 | 8.36 |
| NMSE (dB) of using model trained at 60 km/h | -12.60 | -7.95 | 0.13 |
| NMSE (dB) of using model trained at 120 km/h | -4.85 | -3.14 | -5.25 |
| NMSE (dB) of using model trained at uniformly mixed speed of {30, 60, 120}km/h | -13.45 | -6.16 | -1.81 |

It is shown that the model trained at each speed can only cope with its corresponding speed but performs very poor at other speed. For CSI prediction, as a consequence, with a level y/z collaboration, the speed-specific model can be switched according to the information associated with the UE change speed so as to guarantee the prediction performance for different speed scenarios. However, using a level x AI/ML model, it can be observed that the NMSE increases by a fact of 6.39dB, 1.79 and 3.44dB, respectively for inference speed of 30km/h, 60km/h and 120km/h. Furthermore, the model trained from the mixed-speed data set can improve the generalization performance while there still exist performance gap with speed-specific model. Besides the model switching, finetuning is also a good approach to improve the generalization performance where the model trained by mixed dataset can be a good starting point for finetuning.

The generalization of AI-based CSI prediction over speed is not good if the training set contains only one speed.

The generalization of AI-based CSI prediction over speed can be improved using training set with mixed speed, whose prediction accuracy is still worse than that of speed-specific models.

For AI-based CSI prediction, with a level y/z collaboration, the speed-specific model can be switched according to the information associated with the UE change speed so as to guarantee the prediction performance for different speed. However, using a level x AI/ML model, it is hard to generalize well across different speeds.

Beside the model switching with a level y/z collaboration, we also notice that a preprocessing based model scaling can be utilized to handle the situation of speed changing. For the preprocessing based model scaling, we train a base model using the data with speed  where the CSI-RS periodicity in historical CSI is  and the predicted Future CSI is at . Then, for the scenario with the speed of , we compute the corresponding CSI periodicity in historical CSI () and the predicted Future CSI () using the following rule:

For example, the base model is trained with the UE speed of 30km/h where CSI-RS periodicity in historical CSI is 4ms and the predicted Future CSI is at +4ms. If we want to inference at 15km/h, then the CSI-RS periodicity in historical CSI should turn to 8ms and the predicted Future CSI should be at +8ms; If we want to inference at 60km/h, then the CSI-RS periodicity in historical CSI should turn to 2ms and the predicted Future CSI should be at +2ms (as illustrated in Figure 23 as follows).

The illustration of preprocessing based model scaling for AI-based CSI prediction: from 30km/h to 60km/h

The key issue for preprocessing based model scaling is to derive the input CSI of model with the periodicity of . We consider two options:

1) Reconfigure the CSI-RS periodicity to ;

2) Construct the input CSIs with the periodicity of  from the CSIs with the periodicity of . If , we just need to extract corresponding CSIs; If , CSI interpolation is needed to derive denser CSIs.

The performance of preprocessing based model scaling is provided in the following Table 21. It can be seen that, the prediction accuracy at 60 km/h with 2 ms historical CSI spacing to predict +2 ms and +4 ms are almost the same as that at 30 km/h with 4 ms historical CSI spacing to predict +4 ms and +8 ms. This means that using the preprocessing, the model trained at one speed can scales to other speeds.

The performance of preprocessing based model scaling

| The NMSE (dB) at 30 km/h with 4ms historical CSI spacing | Predict the CSI at +4ms | Predict the CSI at +8ms |
| --- | --- | --- |
|  | -19.84 | -10.65 |
| The NMSE (dB) at 60 km/h with 2ms historical CSI spacing | Predict the CSI at +2ms | Predict the CSI at +4ms |
|  | -19.51 | -10.67 |

Using the preprocessing-based model scaling mechanism, the model trained at one speed can scales to other speeds.

- The generalization of AI-based prediction over deployment scenarios

The generalization over LOS and NLOS channel types:

The LOS and NLOS channel type will lead to different time varying regularity. To this end, we discuss the generalization performance of AI-based CSI prediction over LOS and NLOS channel types. In details, the models are trained by using data set from LOS, NLOS and mixed types respectively and then test these models in LOS and NLOS channel. In this simulation, the period of CSI is 4 ms, and the prediction is with 15 historical CSIs as the input and the future CSI at +4ms as output. The UE is travelling at the speed of 30km/h. The carrier frequency is 2GHz and the subcarrier spacing is 15kHz. The spatial consistency procedure A with 50m decorrelation distance is used where the channel updating periodicity is assumed to be 1 ms. The corresponding performance is provided below.

The generalization performance of AI-based CSI prediction over LOS and NLOS channel types

| Test NMSE(dB) Train | LOS | NLOS |
| --- | --- | --- |
| LOS | -34.20 | -13.87 |
| NLOS | 5.67 | -20.736 |
| Mixed {LOS, NLOS} | -27.96 | -17.67 |

It is shown that the prediction accuracy decreases significantly when the model mismatch is happened. To overcome this problem, assistance information based model switching/selection is a solution. Here, the assistance information is the estimation of LOS and NLOS type of current channel. Once the estimated assistance information changes, the procedure of model monitoring and model switching is triggered and the CSI prediction is switched to the corresponding model by using a level y/z collaboration. Furthermore, as seen from the evaluation result, using the mixed data set of LOS and NLOS can also improve the generalization performance, whose prediction accuracy is still worse than that of scenario-specific models. For the scheme using mixed training set, the data collection principle and procedure should be carefully designed to acquire a good training set.

The generalization over Uma and Umi scenarios:

Similarly, the scenarios of channel such as Uma and Umi also impact the time varying regularity of wireless channel. In details, the models are trained by using data set from Uma, Umi and mixed scenarios, respectively and then test these models in Uma and Umi channel accordingly. In this simulation, the period of CSI is 4 ms, and the prediction is with 15 historical CSIs as the input and the future CSI at +4ms as output. The UE is travelling at the speed of 30km/h. The carrier frequency is 2GHz and the subcarrier spacing is 15kHz. The spatial consistency procedure A with 50m decorrelation distance is used where the channel updating periodicity is assumed to be 1 ms. The corresponding performance is provided in the following Table 23.

The generalization performance of AI-based CSI prediction over Uma and Umi scenarios

| Test NMSE(dB) Train | Uma | Umi |
| --- | --- | --- |
| Uma | -20.736 | -19.89 |
| Umi | -9.53 | -41.48 |
| Mixed {Uma, Umi} | -17.71 | -30.09 |

It can be seen that the prediction performance decreases significantly when the model trained by Uma is tested on the Umi data and the model trained by Umi is tested on the Uma data. This problem can also be solved by a level y/z collaboration based model switching. Furthermore, the model trained by the mixed scenarios can improve the generalization performance while its data collection is needed to be carefully designed.

The generalization over the deployment scenarios, e.g., LOS/NLOS, Uma/Umi, is not good if the training set contains only one scenario.

The generalization over scenarios (e.g., LOS/NLOS, Uma/Umi) and the model adjustment (e.g., model switching/selection, finetuning, deactivation, and fallback) of AI-based CSI prediction should be studied.

The impact of observation window on the AI-based CSI prediction

For the AI-based CSI prediction, time varying characteristic of the CSI required to be extracted from historical CSIs in observation window and utilized to make prediction. Therefore, the construction of the observation window impacts the performance. The observation window can be described by the number of historical CSIs and the spacing of the historical CSIs. The choice of observation window has significant influence on the performance of the CSI prediction. The larger the number and the smaller the spacing of historical CSIs, the better the prediction performance that can be achieved. However, this will in return increase the complexity and the storage (buffer) overhead of the model. Furthermore, for different scenario and different prediction target, the observation window should also be different.

The corresponding simulation parameters are given below and the NMSE of CSI prediction with respect to different observation window is shown in Figure 24.

Simulation parameters: Uma 38.901, carrier frequency 2GHz, subcarrier spacing 15KHz, 32 gNB antenna ( [Mg Ng M N P; Mp Np] = [1 1 2 8 2; 2 8]), 2 UE antenna ([Mg Ng M N P; Mp Np] = [1 1 1 1 2; 1 1]), 100% outdoor UE, Channel type: NLOS. The spatial consistency procedure A with 50m decorrelation distance is used where the channel updating periodicity is assumed to be 1 ms. The future time for prediction is  +4ms.

The NMSE of CSI prediction with respect to different observation windows

It is shown that, the prediction performance can be improved by increasing the number of historical CSIs. However, this improvement is marginal when the number of historical CSIs within the observation window is large enough, e.g., larger than 10 in this case. But the complexity and the storage (buffer) overhead will continuously increases. Therefore, the choice of the number of historical CSIs is important and its tradeoff should be studied. Furthermore, for different speed, the requirement for the spacing of historical CSIs is different. For example, 4ms CSI spacing is enough for 30km/h while 2ms CSI spacing seems to be more suitable for 60 km/h. Therefore, with the change of speed, the observation window should also be changed, perhaps impacting the model switching /selection for AI-based CSI prediction as well.

The observation window can be described by the number of historical CSIs and the spacing of the historical CSIs.

The larger the number and the smaller the spacing of historical CSIs within the observation window, the better the prediction performance that can be achieved. However, this will in return increase the complexity and the storage (buffer) overhead of the model.

For different speeds, the requirement for the observation window is different.

The performance impact of observation window on the AI-based CSI prediction should be studied.

The monitoring results

As shown in the previous subsubsections, the performance of CSI prediction will change with the change of speed, transmission scenario, channel type and also impacted by the observation window. Therefore, the monitoring is needed to be aware of the real time performance of AI-based CSI prediction.

The monitoring is a continuous evaluation of AI model. For the construction of dataset for monitoring, we randomly select N scenarios or configurations (each containing K samples) and concatenate samples of these N scenarios or configurations in the selected order to form a test set. Then, the prediction accuracy is continuously calculated based on this test set to observe the effectiveness of the one scheme in real time and then calculate the average prediction accuracy of one scheme from multiple randomly generated test sets.

Just for an example, we provide the monitoring results when considering multiple speeds including 30, 60, and 120 km/h. For comparison of schemes, we evaluated the scheme using one randomly chosen speed-specific model (i.e., the generalization Case 2) and the scheme with model selection. It should be noted that the delay of monitoring has not been considered in the scheme with model selection. As shown in the following Figure 25, during the monitoring process, the scheme with model selection achieves better prediction accuracy. Meanwhile, the fluctuation of the scheme with model selection is also smaller than the scheme using one randomly chosen speed-specific model (i.e., the generalization Case 2). Furthermore, in the following Table 24, the average prediction accuracy calculated from multiple randomly generated test sets for monitoring is provided. The average prediction accuracy of scheme with model selection is still higher than that without model selection.

(a)                                                                                               (b)

The monitoring results of AI-based CSI prediction

The average prediction accuracy calculated from multiple randomly generated test sets for monitoring

| scheme | Average prediction accuracy described by NMSE (dB) |
| --- | --- |
| using one randomly chosen speed-specific model | 2.31 |
| with model selection | -8.04 |

During the monitoring process, the scheme with model selection achieves better prediction accuracy than the scheme using one randomly chosen speed-specific model (i.e., the generalization Case 2).

During the monitoring process, the fluctuation of the scheme with model selection is smaller than the scheme using one randomly chosen speed-specific model (i.e., the generalization Case 2).

The monitoring of the AI-based CSI prediction should be studied.

The monitoring dataset is formed by randomly selecting N scenarios or configurations (each containing K samples) and concatenating samples of these N scenarios or configurations in the selected order.

Conclusions

- Based on initial field test results, per-cell (region) models can provide more than 30%~50% improvement on SCGS of AI models.

Further study the model update for per-cell (region) models

Further study the data collection for per-cell (region) models.

From initial results for spatial consistency data, performance of simple model structure, e.g., one hidden layer full-connected encoder, can achieve good performance when per cell model is used.

From initial results for field test, performance of simple model structure, e.g., one hidden layer full-connected encoder, is good enough for typical zone/site specific optimization.

From initial results for field test, the model developed for Cell 1 shows robust performance for different moving routes.

Field test shows that model developed for Cell 1 does not perform well for Cell 2.

Field test shows that simple and small models work well for all different cases, at least for typical cell coverage.

For case 2, zero-padding is feasible for subband number generalization while its performance degrades dramatically in port number generalization.

For case 2, pre-processing performs well for both subband number generalization and port number generalization.

Payload truncation, as a starting point, performs well in UCI payload size generalization.

Rank generalization with per-layer model can achieve similar SGCS with half model size compared with per-rank model.

Layer common model can achieve better SGCS with the same dataset.

AI model performance does not degrade when a generic model (non-optimized for a specific area/cell) trained for a frequency is applied to another frequency.

For a generic model (non-optimized for a specific area/cell), AI model performance does not degrade when generalized from UMi to UMa.

For a generic model (non-optimized for a specific area/cell) AI model can generalize across different scenarios with a mixed dataset. A reasonable mixing ratio can provide better performance for each scenario.

For a generic model (non-optimized for a specific area/cell), AI model trained in complicated channel environment (more indoor users) has good generalization ability.

The performance of AI model depends on the deployment environment

For a generic model (non-optimized for a specific area/cell), there is obvious performance loss for antenna spacing mismatch of training data.

For a generic model (non-optimized for a specific area/cell), the influence of mismatch of training data may be reduced by pre-processing.

For a generic model (non-optimized for a specific area/cell), SGCS performance of AI model may degrade slightly from 128 antennas with virtualization to 32 antennas without virtualization. While the SE performance may degrade heavily due to the less antennas.

For a generic model (non-optimized for a specific area/cell), in the case of 32 antennas, AI model trained with 32 antennas may have similar SE performance compared with AI model trained with 128 antennas and settled in the case of 128 antennas, which is needed to be further studied.

High resolution R16-eType II codebook with large L, M, beta (for example, L=12, M=6, beta = 1.0) performs well for ground-truth CSI quantization.

Quantization non-aware training only achieves good performance when the averaged quantization bit is large (e.g., >= 4bits/float). When the averaged quantization bit is small (e.g., <= 2bits/float), the performance loss is significant.

Vector quantization with optimized codebook can achieve slightly better performance (e.g., by about 0.009 in SGCS in our considered configurations) than scalar quantization with fixed codebook.

Performance of vector quantization with randomly initialization and fixed codebook can be slightly inferior to that of scalar quantization with fixed codebook (e.g., by about 0.0065 in SGCS in our considered configurations).

Performance of quantization non-aware training could be significantly lower than that of quantization aware training (more than 0.1 in SGCS in our considered configurations).

Quantization method at UE side and dequantization method at NW side should be aligned for training collaboration type2 and 3 to achieve a satisfying performance.

One common CSI reconstruction part could be trained to match multiple CSI generation parts of different UEs in training collaboration type 2 at the cost of some performance loss.

Considering one common CSI reconstruction part matching three CSI generation parts, SGCS performance gain of AI/ML models over Rel-16 Type II codebook reduces from 0.075 to 0.052, i.e., losing about 30% performance gain.

One common CSI generation part could be trained to match multiple CSI reconstruction parts of different networks in training collaboration type 2 at the cost of some performance loss.

Considering one common CSI generation part matching three CSI reconstruction parts, SGCS performance gain of AI/ML models over Rel-16 Type II codebook reduce from 0.075 to 0.061, i.e., losing about 19% performance gain.

If the model structure is not aligned (e.g., dequantization method at decoder and the quantization method in encoder could not match), there will be an obvious performance loss compared with that in case where the dequantization and quantization method are matching.

One common CSI reconstruction/generation part could be trained to match multiple CSI reconstruction/generation parts of different UEs in training collaboration type 3 at the cost of some performance loss (e.g., considering one common CSI reconstruction part to three CSI generation part and each UE sharing 50,000 samples with NW, the performance loss in SGCS is around 0.04).

Performance of one common CSI generation/reconstruction part to multiple CSI reconstruction/generation parts of different networks/UEs is affected by the amount of exchanged data from each network/UE.

Performance loss in supporting common CSI generation/reconstruction part to multiple CSI reconstruction/generation parts gets worse as the number of supported UEs/networks increases.

Monitoring based on proxy model at UE side in CSI compression can lead to near-ideal model switching when the scenario changes.

The AI-based CSI prediction achieves higher spectral efficiency when compared with the nearest historical CSI w/o prediction and AR-based non-AI CSI prediction.

The generalization of AI-based CSI prediction over speed is not good if the training set contains only one speed.

The generalization of AI-based CSI prediction over speed can be improved using training set with mixed speed, whose prediction accuracy is still worse than that of speed-specific models.

For AI-based CSI prediction, with a level y/z collaboration, the speed-specific model can be switched according to the information associated with the UE change speed so as to guarantee the prediction performance for different speed. However, using a level x AI/ML model, it is hard to generalize well across different speeds.

Using the preprocessing-based model scaling mechanism, the model trained at one speed can scales to other speeds.

The generalization over the deployment scenarios, e.g., LOS/NLOS, Uma/Umi, is not good if the training set contains only one scenario.

The observation window can be described by the number of historical CSIs and the spacing of the historical CSIs.

The larger the number and the smaller the spacing of historical CSIs within the observation window, the better the prediction performance that can be achieved. However, this will in return increase the complexity and the storage (buffer) overhead of the model.

For different speeds, the requirement for the observation window is different.

During the monitoring process, the scheme with model selection achieves better prediction accuracy than the scheme using one randomly chosen speed-specific model (i.e., the generalization Case 2).

During the monitoring process, the fluctuation of the scheme with model selection is smaller than the scheme using one randomly chosen speed-specific model (i.e., the generalization Case 2).

- Consider to capture observations from field data test into TR.

Study the performance and overhead of per-cell (region) model transfer in CSI compression.

Study the following three methods for generalization of input dimension

- Option 1: use large dimension AI/ML model in small dimension cases: zero-padding

- Option 2: use small dimension AI/ML model in large dimension cases: grouping

- Option 3: use pre-processing to fix the input dimension: angle-delay domain compression

Study CSI payload truncation for the generalization of UCI payload.

- FFS the flexible truncation strategy and training parameters for more different payload.

- FFS the payload truncation on different layers for rank > 1 cases.

For rank > 1 cases, study 	Option 3-1: layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference)

- FFS how to choose the layers for training data set

- FFS how to deal with specific payload for each layer

For a generic model (non-optimized for a specific area/cell), AI models perform well in generalization of carrier frequency, channel scenario, indoor/outdoor ratio.

For a generic model (non-optimized for a specific area/cell), AI models perform bad in antenna spacing and antenna virtualization, which can be further studied.

The accuracy of a particular monitoring method can be evaluated via testing whether this method could lead to a proper model switching. To measure what is a proper model switching, some metrics can be considered as the starting points:

- Counting the false alarm and misdetection compared with ideal switching

  - Ideal switching refers to the switching decisions guided by accurate performance KPIs of candidate models

  - false alarm refers to the case that ideal switching does not happen, but practical switching happens

  - misdetection refers to the case that switching happens but practical switching does not happen

- Comparing the SGCS after switching for each monitoring method

For the purpose of evaluating performance monitoring methods in CSI compression, the data to model a switching procedure can be collected from moving UEs across multiple scenarios.

For the AI/ML based CSI prediction sub use case, if collaboration level x is reported as the benchmark, the comparison of level y/z AI-based CSI prediction and level x AI-based CSI prediction should be studied in the generalization aspects, e.g., collaboration level x is modeled as generalization Case 2, while collaboration level y/z is modeled as the generalization Case 1 or finetuning.

The generalization over scenarios (e.g., LOS/NLOS, Uma/Umi) and the model adjustment (e.g., model switching/selection, finetuning, deactivation, and fallback) of AI-based CSI prediction should be studied.

The performance impact of observation window on the AI-based CSI prediction should be studied.

The monitoring of the AI-based CSI prediction should be studied.

The monitoring dataset is formed by randomly selecting N scenarios or configurations (each containing K samples) and concatenating samples of these N scenarios or configurations in the selected order.

References

- R1-2212845, Session notes for 9.2 (Study on AI/ ML for NR air interface), RAN1#111, 2022.11