3GPP TSG RAN WG1 #112bis-e	 	 R1-2302483

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:	Evaluation on NR duplex evolution

Agenda Item:	9.3.1

Document for:	Discussion and Decision

- Introduction

In RAN1#112 meeting, the evaluation methodologies and assumptions for NR full duplex were discussed and agreed, detailed agreements are listed in Annex A [1]. In this contribution, we provide our simulation results and the related assumptions used in our duplex simulations.

- Performance evaluation results

Based on the agreements in last RAN1 meeting, the overall simulation assumptions used in this evaluation are shown in Table B-1 in Annex B. Besides, in the evaluations, following models and assumptions are adopted.

- Channel modeling: In this evaluation, for gNB-to-UE, UE-to-gNB and gNB-to-gNB, both large-scale fading and small-scale fading are modeled. For UE-to-UE, only large-scale fading is modeled according to the agreements in last RAN1 meeting.

- Antenna array: Two options were agreed to be used for evaluation and comparison between SBFD and legacy TDD. We used the option 2 which is the total number of antenna elements for SBFD is two times of that for legacy TDD in our simulation. Moreover, in our simulation, the number of antenna elements used for transmission or reception in a slot will not be changed even if gNB does not perform simultaneous downlink transmission and uplink reception in the slot.

- Packet size: In last RAN1 meeting, some parameter combinations is agreed as high priority for SBFD deployment case 1. For traffic model, the asymmetric packet size with 4Kbytes for DL and 1Kbytes for UL (small packet size) and the asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL (large packet size) are recommended. In our evaluation, both large packet size and small packet size for DL/UL is evaluated. Moreover, UL and DL are simulated simultaneously, and each UE is assigned both UL traffic and DL traffic.

In the following, we evaluate the performances for the SBFD deployment case 1, and dynamic TDD in Indoor Hotspot scenario in FR1. The evaluation results for frame structure#3 (XXXXX) can be found in Section 2.1, the evaluation results for frame structure#2 (XXXXU) can be found in Section 2.2, the evaluation results for dynamic SBFD and dynamic TDD in different adjustment periodicities can be found in Section 2.3.

  - Frame structure#3 (XXXXX)

For SBFD and dynamic TDD with frame structure#3, the following schemes are evaluated:

| Scheme 1-1 (baseline) | Baseline legacy TDD operation with UL/DL configuration of Frame structure #1 DDDSU (S slot: 12D:2S:0U) |
| --- | --- |
| Scheme 1-2 (semi-static SBFD) | Semi-static SBFD operation with 20% UL subband resource in X slot, with Frame structure #3 (XXXXX) |
| Scheme 1-3 (dynamic SBFD) | Dynamic SBFD operation with adaptation among {Full DL, Full UL, SBFD operation with 20% UL subband resource} in X slot, with Frame structure#3 (XXXXX). The X slot is determined based on the required resources for DL/UL traffic transmission per 5 slots. - Note that for Scheme 1-3, the SBFD pattern in frequency domain (SBFD operation with 20% UL subband resource) is semi-statically configured. Scheme 1-3 is to imitate the scheme that dynamic determination on whether to apply the SBFD pattern in the X slot. |
| Scheme 1-4 (dynamic TDD) | Dynamic TDD operation with adaptation among {Full DL, Full UL, S slot (DL: GAP: UL) = (6:2:6)} in F slot, with configuration FFFFF, where F slots are determined based on the required resources for DL/UL traffic transmission per 5 slots |

    - Small packet size

In this section, the asymmetric packet size with 4Kbytes for DL and 1Kbytes for UL is evaluated with different level of RUs. In previous RAN1 meeting, it was agreed that the DL/UL arrival rate for legacy TDD is selected to reach a target DL/UL traffic load. Based on the test results, following traffic models are adopted for all the evaluation schemes. Note the traffic models are also used for the evaluation of frame structure#2(XXXXU) in in Section 2.2 and Section 2.3.

| Low RU (DL/UL type-2 RU < 10%) | Packet size: 4Kbytes for DL and 1Kbytes for UL DL arrival rate: 160 packets/sec, UL arrival rate: 180 packets/sec Note: Under this configuration, the DL type-2 RU is 8.23% and UL type-2 RU is 10.00% in scheme 1-1. |
| --- | --- |
| Medium RU (DL/UL type-2 RU ~ [20%-30%]) | Packet size: 4Kbytes for DL and 1Kbytes for UL DL arrival rate: 500 packets/sec UL arrival rate: 600 packets/sec Note: Under this configuration, the DL type-2 RU is 27.19% and UL type-2 RU is 26.38% in scheme 1-1. |
| High RU (DL/UL type-2 RU >= 50%) | Packet size: 4Kbytes for DL and 1Kbytes for UL DL arrival rate: 1000 packets/sec UL arrival rate: 1400 packets/ Note: Under this configuration, the DL type-2 RU is 69.53% and UL type-2 RU is 55.77% in scheme 1-1. |

The performance gain of DL/UL average-UPT for evaluation schemes compared to baseline scheme in indoor hotspot scenario is shown in Figure 1 to Figure 3. The corresponding DL/UL average-UPT, DL/UL per packet (option 1) and per user (option 2) latency, DL/UL type 1 and type 2 RU are shown in Table C-1 to Table C-21 in the Annex C. Based on the evaluation results, the following can be observed.

For DL, semi-static SBFD can achieve performance gain with low RU, but has performance degradation with medium and high load, the gain of DL average-UPT decreases with the increase of traffic load. Due to the relatively small size of packets, a packet almost only needs one HARQ for transmission. Hence, the latency of packet is mainly composed of two components: the time to compete for resources and the time for the HARQ to be correctly received. Since slot U in legacy TDD is also become to slot X, UE has more transmission opportunities. In addition, due to the UL resource in the X slot, the feedback delay of HARQ-ACK can be reduced, for incorrectly transmitted HARQs, it can be retransmitted faster. However, since DL resources in slot X are smaller than DL resources in slot D, resource competition may result in additional delay. In addition, since the UE only support half-duplex, UL HARQ scheduling can also affect the UE's DL HARQ scheduling, causing additional delay. According to the Table C-3, Table C-10, and Table C-17, compared with legacy TDD, the performance of 95% DL delay in semi-static SBFD is better, but as the load increases, the performance of 95% DL delay in semi-static SBFD become worse and worse. Hence, the DL performance decreases with the increase of traffic load.

For UL, semi-static SBFD can achieve greatly performance with low, medium and high RU. Similar to the DL packet, a UL packet also requires only one HARQ to transmission. The UL resources in the X slot can be used to transmit UL traffic packet, the time to compete for resources of UL packets can be significance reduced. In addition, The UL resources in the X slot also provide faster retransmission opportunities for incorrectly transmitted HARQs. However, with the increase of traffic load, the resource competition becomes more intense due to the limited UL resources in X slot, the gain of UL average-UPT decreases with the increase of traffic load.

For dynamic SBFD and dynamic TDD, since the packet arrival rate is high and the traffic load changes dramatically, the dynamic adjustment algorithm cannot accurately predict the traffic load changes, which makes the resource adjustment and the traffic load not match well, which may bring additional delay. Further shortening the adjustment periodicities or optimizing the dynamic adjustment algorithm, dynamic FD and dynamic TDD may achieve better performance.

As discussed above, the following observations can be obtained.

Observation 1: For FR 1 InH and asymmetric packet size with 4Kbytes for DL and 1Kbytes for UL, compared to legacy TDD with DDDSU (scheme 1-1)

- Semi-static SBFD with XXXXX (Scheme 1-2) achieves 3.01% DL average-UPT gain with low load, but has 4.16% and 16.47% DL average-UPT degradation with medium and high load. The gain of DL average-UPT degradation with the increase of traffic load.

- Semi-static SBFD with XXXXX (Scheme 1-2) achieves 65.28% ,65.79% and 56.44% UL average-UPT gain with low, medium, and high load. The gain of UL average-UPT degradation with the increase of traffic load.

- Dynamic SBFD with XXXXX (Scheme 1-3) achieves 1.62% DL average-UPT gain with low load, but has 3.58% and 12.65% DL average-UPT degradation with medium and high load. The gain of DL average-UPT degradation with the increase of traffic load.

- Dynamic SBFD with XXXXX (Scheme 1-3) achieves 53.80%, 55.13% and 33.01% UL average-UPT gain in low, medium, and high load. The gain of UL average-UPT decreases with the increase of traffic load.

- Dynamic TDD with FFFFF (Scheme 1-4) achieves 18.09%, 17.17% and 11.13% DL average-UPT gain in low, medium and high load. The gain of DL average-UPT decreases with the increase of traffic load.

- Dynamic TDD with FFFFF (Scheme 1-4) achieves 6.59% and 5.21% UL average-UPT gain in low and medium load, but has 3.96% UL average-UPT degradation with high load. The gain of DL average-UPT decreases with the increase of traffic load.

Observation 2: For FR 1 InH and asymmetric packet size with 4Kbytes for DL and 1Kbytes for UL, compared to dynamic TDD (scheme 1-4), dynamic SBFD (scheme 1-3) can obtain a significant UL gain with low, medium and high load.

Figure 1. Low RU for both UL and DL (<10%)

Figure 2. Medium RU for both UL and DL (20%-30%)

Figure 3. High RU for both UL and DL (>=50%)

    - Large packet size

In this section, the asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL is evaluated with different level of RUs. In previous RAN1 meeting, it was agreed that the DL/UL arrival rate for legacy TDD is selected to reach a target DL/UL traffic load. Based on the test results, following traffic models are adopted for all the evaluation schemes. Note the traffic models are also used for the evaluation of frame structure#2(XXXXU) in Section 2.2 and Section 2.3.

| Low RU (DL/UL type-2 RU < 10%) | Packet size: 0.5Mbytes for DL and 0.125Mbytes for UL DL arrival rate: 1.8 packets/sec, UL arrival rate: 2.5 packets/sec Note: Under this configuration, the DL type-2 RU is 9.55% and UL type-2 RU is 10.14% in scheme 1-1. |
| --- | --- |
| Medium RU (DL/UL type-2 RU ~ [20%-30%]) | Packet size: 0.5Mbytes for DL and 0.125Mbytes for UL DL arrival rate: 4.2 packets/sec UL arrival rate: 6.0 packets/sec Note: Under this configuration, the DL type-2 RU is 28.75% and UL type-2 RU is 25.20% in scheme 1-1. |
| High RU (DL/UL type-2 RU >= 50%) | Packet size: 0.5Mbytes for DL and 0.125Mbytes for UL DL arrival rate: 8.0 packets/sec UL arrival rate: 13.0 packets/ Note: Under this configuration, the DL type-2 RU is 67.69% and UL type-2 RU is 57.06% in scheme 1-1. |

The performance gain of DL/UL average-UPT for evaluation schemes compared to baseline in indoor hotspot scenario is shown in Figure 4 to Figure 6. The corresponding DL/UL average-UPT, DL/UL per packet (option 1) and per user (option 2) latency, DL/UL type 1 and type 2 RU are shown in Table C-22 to Table C-42 in the Annex C. Based on the evaluation results, the following can be observed.

For DL, semi-static SBFD can achieve performance gain with low and medium RU, but has performance degradation with high load, the gain of DL average-UPT degradation with the increase of traffic load. Since there are UL resources in the X slot, the reduced feedback delay of HARQ-ACK can be beneficial for the DL performance, but due to the limited DL resources in X slot, a packet may need to be divided into more HARQs. With the increase of traffic load, the resource competition becomes more intense, it takes longer for each HARQ to compete for resources. In SBFD, a packet needs to be divided into more HARQs, resulting in a longer transmission delay.

For UL, semi-static SBFD has performance degradation with low, medium and high RU, and the gain of UL average-UPT degradation with the increase of traffic load. The UL resources in the X slot can be used to transmit UL traffic, the transmission delay of UL packets can be reduced, but due to the limited UL resources in X slot, a packet needs to be divided into more HARQs for transmission. The retransmission of some HARQs will significantly lengthens the transmission delay of some packets. In addition, with the increase of traffic load, the resource competition becomes more intense, and the interference becomes heavy, resulting in more time for a packet to complete.

For dynamic SBFD, it can achieve performance gain with low, medium and high in both DL and UL, especially in UL. In dynamic SBFD, if the traffic load for a certain direction e.g., DL in the cell becomes heavy, the gNB can assign more resources for this direction e.g., disable the UL subband in X slot for subsequent data transmission to ensure better service for users in the cell. Because there are far more DL resources than UL resources, the dynamic resources adjustment is more effective for UL than DL. Like the semi-static SBFD, in dynamic SBFD, UL resources in the X slot is beneficial for the DL performance because of the faster retransmission HARQ feedback.

For dynamic TDD, like the dynamic SBFD, it can also achieve performance gain with low, medium in both DL and UL, especially in UL, but has performance degradation with high load. In dynamic TDD, the phenomenon of fast HARQ feedback retransmission is not obvious. As the traffic load increases, the competition of DL resources becomes more intense, resulting in lower DL gain.

As discussed above, the following observations can be obtained.

Observation 3: For FR 1 InH and asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL, compared to legacy TDD with DDDSU (scheme 1-1)

- Semi-static SBFD with XXXXX (Scheme 1-2) achieves 0.83% and 4.46% DL average-UPT gain with low load, but has 2.33% DL average-UPT degradation with high load.

- Semi-static SBFD with XXXXX (Scheme 1-2) has 12.57% ,13.40% and 20.66% UL average-UPT degradation with low, medium, and high load. The gain of UL average-UPT decreases with the increase of traffic load.

- Dynamic SBFD with XXXXX (Scheme 1-3) achieves 11.23%, 16.76% and 8.57% DL average-UPT gain with low, medium, and high load.

- Dynamic SBFD with XXXXX (Scheme 1-3) achieves 77.71%, 48.24% and 7.61% UL average-UPT gain in low, medium, and high load. The gain of UL average-UPT decreases with the increase of traffic load.

- Dynamic TDD with FFFFF (Scheme 1-4) achieves 1.33%, 7.35% and 8.57% DL average-UPT gain in low, medium and high load. The gain of DL average-UPT increase with the increase of traffic load.

- Dynamic TDD with FFFFF (Scheme 1-4) achieves 87.71% and 49.85% UL average-UPT gain in low and medium load, but has 1.68% UL average-UPT degradation with high load. The gain of UL average-UPT decreases with the increase of traffic load.

Observation 4: For FR 1 InH and asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL, compared to semi-static SBFD (scheme 1-2), dynamic SBFD (scheme 1-3) can achieve higher performance in both DL and UL, especially in UL significant gain can be obtained.

Observation 5: For FR 1 InH and asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL, compared to dynamic TDD (scheme 1-4),

- Dynamic SBFD (scheme 1-3) can obtain a significant DL gain with low, medium and high load

- Dynamic SBFD (scheme 1-3) can obtain similar UL UPT with low, medium load

- Dynamic SBFD (scheme 1-3) can obtain UL gain with high load

Figure 4. Low RU for both UL and DL (<10%)

Figure 5. Medium RU for both UL and DL (20%-30%)

Figure 6. High RU for both UL and DL (>=50%)

  - Frame structure#2 (XXXXU)

For SBFD and dynamic TDD with frame structure#2, the following schemes are evaluated. The baseline scheme (legacy TDD with DDDSU) is scheme 1-1 as in section 2.1.

| Scheme 1-1 (baseline) | Baseline legacy TDD operation with UL/DL configuration of Frame structure #1 DDDSU (S slot: 12D:2S:0U) |
| --- | --- |
| Scheme 2-2 (semi-static SBFD) | Semi-static SBFD operation with 20% UL subband resource in X slot, with Frame structure #3 (XXXXU) |
| Scheme 2-3 (dynamic SBFD) | Dynamic SBFD operation with adaptation among {Full DL, Full UL, SBFD operation with 20% UL subband resource} in X slot, with Frame structure#3 (XXXXU). The X slot is determined based on the required resources for DL/UL traffic transmission per 5 slots. - Note that for Scheme 2-3, the SBFD pattern in frequency domain (SBFD operation with 20% UL subband resource) is semi-statically configured. Scheme 2-3 is to imitate the scheme that dynamic determination on whether to apply the SBFD pattern in the X slot. |
| Scheme 2-4 (dynamic TDD) | Dynamic TDD operation with adaptation among {Full DL, Full UL, S slot (DL: GAP: UL) = (6:2:6)} in F slot, with configuration FFFFU, where F slots are determined based on the required resources for DL/UL traffic transmission per 5 slots |

    - Small packet size

The gain of DL/UL average-UPT between evaluation schemes and baseline for indoor hotspot scenario is shown in Figure 7 to Figure 9. The corresponding DL/UL average-UPT, DL/UL per packet (option 1) and per user (option 2) latency, DL/UL type 1 and type 2 RU is shown in Table C-43 to Table C-63 in the Annex C. Based on the evaluation results, the following can be observed.

For DL, the performance degradation occurred in semi-static SBFD and dynamic SBFD with low/medium/high load. In this frame structure configuration, no SBFD DL subband in the slots that correspond to UL slots in legacy TDD, which cause the different UL/DL resource ratio between legacy TDD and semi-static SBFD. Under this configuration, the DL resources reduction cause the DL performances lost in all traffic load. Especially in high load, the impact of resource reduction is more serve.

Different from DL, the increase in UL resources also brings significant UL gain in all traffic load, with the increase of load, the benefit of increasing UL resources become more obvious.

Meanwhile, the dynamic SBFD and dynamic TDD can reduce the DL performance lost caused by DL resource reduction compared to legacy TDD. It can even achieve performance gain with low, medium and high RU in dynamic TDD, the main reason is the symbol of DL bundle in S slot in dynamic TDD is less than legacy TDD, this can be beneficial for the DL performance in dynamic TDD with small packet. According to the Table C-45, Table C-52, and Table C-59, compared with legacy TDD, the performance of 5% DL delay in dynamic TDD is much better. Moreover, the ratio of UL traffic load to UL resources (UL type 2 RU) is smaller than that of DL traffic load to DL resources (DL type 2 RU) in semi-static SBFD, dynamic adjustment schemes tend to dynamically allocate resources for DL transmissions. In this way, the UL performance of semi-static SBFD is affected.

As discussed above, the following observations can be obtained.

Observation 6: For FR 1 InH and asymmetric packet size with 4Kbytes for DL and 1Kbytes for UL, compared to legacy TDD with DDDSU (scheme 1-1)

- Semi-static SBFD with XXXXU (Scheme 2-2) has 5.31%, 12,21% and 34.80% DL average-UPT degradation with low, medium and high load. The gain of DL average-UPT degradation with the increase of traffic load.

- Semi-static SBFD with XXXXU (Scheme 2-2) achieves 52.86% ,53.55% and 39.62% UL average-UPT gain with low, medium, and high load. The gain of UL average-UPT degradation with the increase of traffic load.

- Dynamic SBFD with XXXXU (Scheme 2-3) has 0.97%, 1.29% and 0.65% DL average-UPT degradation with low load.

- Dynamic SBFD with XXXXU (Scheme 2-3) achieves 18.69%, 10.81% and 4.64% UL average-UPT gain in low, medium, and high load. The gain of UL average-UPT decreases with the increase of traffic load.

- Dynamic TDD with FFFFU (Scheme 2-4) achieves 11.94%, 11.60% and 5.20% DL average-UPT gain in low, medium and high load. The gain of DL average-UPT decreases with the increase of traffic load.

- Dynamic TDD with FFFFU (Scheme 2-4) achieves 18.44%, 20.39% and 19.55% UL average-UPT gain in low, medium and high load.

Figure 7. Low RU for both UL and DL (<10%)

Figure 8. Medium RU for both UL and DL (20%-30%)

Figure 9. High RU for both UL and DL (>=50%)

    - Large packet size

The gain of DL/UL average-UPT between evaluation schemes and baseline for indoor hotspot scenario is shown in Figure 10 to Figure 12. The corresponding DL/UL average-UPT, DL/UL per packet (option 1) and per user (option 2) latency, DL/UL type 1 and type 2 RU is shown in Table C-64 to Table C-84 in the Annex C. Based on the evaluation results, the following can be observed.

For DL, the performance degradation occurred in all the evaluation case with low/medium/high load. In this frame structure configuration, no SBFD DL subband in the slots that correspond to UL slots in legacy TDD, which cause the different UL/DL resource ratio between legacy TDD and semi-static SBFD. Under this configuration, the DL resources reduction cause the DL performances lost in all traffic load. Especially in high load, the impact of resource reduction is more serve.

Different from DL, the increase in UL resources also brings significant UL gain in all traffic load, with the increase of load, the benefit of increasing UL resources become more obvious.

Meanwhile, the dynamic SBFD and dynamic TDD can reduce the DL performance lost caused by DL resource reduction compared to legacy TDD, and further obtain significant performance gain in UL.

As discussed above, the following observations can be obtained.

Observation 7: For FR 1 InH and asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL, compared to legacy TDD with DDDSU (scheme 1-1)

- Semi-static SBFD with XXXXU (Scheme 2-2) has 20.66%, 23.20% and 39.96% DL average-UPT degradation with low, medium and high load. The gain of DL average-UPT decreases with the increase of traffic load.

- Semi-static SBFD with XXXXU (Scheme 2-2) achieves 42.23% ,47.27% and 46.29% UL average-UPT gain with low, medium, and high load.

- Dynamic SBFD with XXXXU (Scheme 2-3) has 4.49%, 6.25% and 15.48% DL average-UPT degradation with low, medium, and high load. The gain of DL average-UPT decreases with the increase of traffic load.

- Dynamic SBFD with XXXXU (Scheme 2-3) achieves 95.02%, 59.65% and 25.69% UL average-UPT gain in low, medium, and high load. The gain of UL average-UPT decreases with the increase of traffic load.

- Dynamic TDD with FFFFU (Scheme 2-4) has 17.97%, 17.46% and 30.20% DL average-UPT gain in low, medium and high load. The gain of DL average-UPT increase with the increase of traffic load.

- Dynamic TDD with FFFFU (Scheme 2-4) achieves 93.32%, 63.03% and 38.13% UL average-UPT gain in low, medium and high load. The gain of UL average-UPT increase with the increase of traffic load.

Observation 8: For FR 1 InH and asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL, compared to semi-static SBFD (scheme 2-2), dynamic SBFD (scheme 2-3) can achieve higher performance in both DL and UL, except UL UPT with high load.

Figure 10. Low RU for both UL and DL (<10%)

Figure 11. Medium RU for both UL and DL (20%-30%)

Figure 12. High RU for both UL and DL (>=50%)

  - Performance comparison on different adjustment periodicities for dynamic SBFD and dynamic TDD

In dynamic SBFD and dynamic TDD, radio resources are dynamically adjusted periodically to match the traffic load of UL and DL. The random arrival of FTP packets may cause large changes in the UL and DL traffic loads. To better match UL and DL traffic loads, we can further shorten the resource adjustment periodicity. The following schemes are evaluated:

| Scheme 3-1 | Dynamic SBFD operation with adaptation among {Full DL, Full UL, SBFD operation with 20% UL subband resource} in X slot, with Frame structure#3 (XXXXU). The X slot is determined based on the required resources for DL/UL traffic transmission per 5 slots. |
| --- | --- |
| Scheme 3-2 | Dynamic SBFD operation with adaptation among {Full DL, Full UL, SBFD operation with 20% UL subband resource} in X slot, with Frame structure#3 (XXXXU). The X slot is determined based on the required resources for DL/UL traffic transmission per slot. |
| Scheme 3-3 | Dynamic TDD operation with adaptation among {Full DL, Full UL, S slot (DL: GAP: UL) = (6:2:6)} in F slot, with configuration FFFFU, where F slots are determined based on the required resources for DL/UL traffic transmission per 5 slots |
| Scheme 3-4 | Dynamic TDD operation with adaptation among {Full DL, Full UL, S slot (DL: GAP: UL) = (6:2:6)} in F slot, with configuration FFFFU, where F slots are determined based on the required resources for DL/UL traffic transmission per slot |

For Scheme 3-2 and Scheme 3-4, an X slot or F slot may be determined as DL slot, SBFD/S slot or UL slot according to the required resources for DL and UL per slot. To enable the scheduling for UL in case of an X slot or F slot which is determined as UL, 1-symbol for PDCCH and 2-symbol for DL/UL gap are reserved at the beginning of the X slot or F slot. This may result in less resources for the UL transmission in the X slot or F slot which is determined as UL compared to Scheme 3-1 and Scheme 3-3.

In this section, the asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL is evaluated with different level of RUs. The DL/UL average-UPT for each evaluation schemes in indoor hotspot scenario with different level of RUs are shown in Figure 13 to Figure 16.

Based on the evaluation results, it can be observed that, as the adjustment periodicity decreases, the DL/UL average-UPT performance further improves for dynamic SBFD and dynamic TDD.

Observation 9: For FR 1 InH and asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL, as the adjustment periodicity decreases, the DL/UL average-UPT performance further improves for dynamic SBFD and dynamic TDD.

Figure 13. Low RU for both UL and DL (<10%)

Figure 14. Medium RU for both UL and DL (20%-30%)

Figure 15. High RU for both UL and DL (>=50%)

- Conclusion

In this contribution, we provide our views on NR duplex evolution, and give our simulation results with the following observations and proposals:

Observation 1: For FR 1 InH and asymmetric packet size with 4Kbytes for DL and 1Kbytes for UL, compared to legacy TDD with DDDSU (scheme 1-1)

- Semi-static SBFD with XXXXX (Scheme 1-2) achieves 3.01% DL average-UPT gain with low load, but has 4.16% and 16.47% DL average-UPT degradation with medium and high load. The gain of DL average-UPT degradation with the increase of traffic load.

- Semi-static SBFD with XXXXX (Scheme 1-2) achieves 65.28% ,65.79% and 56.44% UL average-UPT gain with low, medium, and high load. The gain of UL average-UPT degradation with the increase of traffic load.

- Dynamic SBFD with XXXXX (Scheme 1-3) achieves 1.62% DL average-UPT gain with low load, but has 3.58% and 12.65% DL average-UPT degradation with medium and high load. The gain of DL average-UPT degradation with the increase of traffic load.

- Dynamic SBFD with XXXXX (Scheme 1-3) achieves 53.80%, 55.13% and 33.01% UL average-UPT gain in low, medium, and high load. The gain of UL average-UPT decreases with the increase of traffic load.

- Dynamic TDD with FFFFF (Scheme 1-4) achieves 18.09%, 17.17% and 11.13% DL average-UPT gain in low, medium and high load. The gain of DL average-UPT decreases with the increase of traffic load.

- Dynamic TDD with FFFFF (Scheme 1-4) achieves 6.59% and 5.21% UL average-UPT gain in low and medium load, but has 3.96% UL average-UPT degradation with high load. The gain of DL average-UPT decreases with the increase of traffic load.

Observation 2: For FR 1 InH and asymmetric packet size with 4Kbytes for DL and 1Kbytes for UL, compared to dynamic TDD (scheme 1-4), dynamic SBFD (scheme 1-3) can obtain a significant UL gain with low, medium and high load.

Observation 3: For FR 1 InH and asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL, compared to legacy TDD with DDDSU (scheme 1-1)

- Semi-static SBFD with XXXXX (Scheme 1-2) achieves 0.83% and 4.46% DL average-UPT gain with low load, but has 2.33% DL average-UPT degradation with high load.

- Semi-static SBFD with XXXXX (Scheme 1-2) has 12.57% ,13.40% and 20.66% UL average-UPT degradation with low, medium, and high load. The gain of UL average-UPT decreases with the increase of traffic load.

- Dynamic SBFD with XXXXX (Scheme 1-3) achieves 11.23%, 16.76% and 8.57% DL average-UPT gain with low, medium, and high load.

- Dynamic SBFD with XXXXX (Scheme 1-3) achieves 77.71%, 48.24% and 7.61% UL average-UPT gain in low, medium, and high load. The gain of UL average-UPT decreases with the increase of traffic load.

- Dynamic TDD with FFFFF (Scheme 1-4) achieves 1.33%, 7.35% and 8.57% DL average-UPT gain in low, medium and high load. The gain of DL average-UPT increase with the increase of traffic load.

- Dynamic TDD with FFFFF (Scheme 1-4) achieves 87.71% and 49.85% UL average-UPT gain in low and medium load, but has 1.68% UL average-UPT degradation with high load. The gain of UL average-UPT decreases with the increase of traffic load.

Observation 4: For FR 1 InH and asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL, compared to semi-static SBFD (scheme 1-2), dynamic SBFD (scheme 1-3) can achieve higher performance in both DL and UL, especially in UL significant gain can be obtained.

Observation 5: For FR 1 InH and asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL, compared to dynamic TDD (scheme 1-4),

- Dynamic SBFD (scheme 1-3) can obtain a significant DL gain with low, medium and high load

- Dynamic SBFD (scheme 1-3) can obtain similar UL UPT with low, medium load

- Dynamic SBFD (scheme 1-3) can obtain UL gain with high load

Observation 6: For FR 1 InH and asymmetric packet size with 4Kbytes for DL and 1Kbytes for UL, compared to legacy TDD with DDDSU (scheme 1-1)

- Semi-static SBFD with XXXXU (Scheme 2-2) has 5.31%, 12,21% and 34.80% DL average-UPT degradation with low, medium and high load. The gain of DL average-UPT degradation with the increase of traffic load.

- Semi-static SBFD with XXXXU (Scheme 2-2) achieves 52.86% ,53.55% and 39.62% UL average-UPT gain with low, medium, and high load. The gain of UL average-UPT degradation with the increase of traffic load.

- Dynamic SBFD with XXXXU (Scheme 2-3) has 0.97%, 1.29% and 0.65% DL average-UPT degradation with low load.

- Dynamic SBFD with XXXXU (Scheme 2-3) achieves 18.69%, 10.81% and 4.64% UL average-UPT gain in low, medium, and high load. The gain of UL average-UPT decreases with the increase of traffic load.

- Dynamic TDD with FFFFU (Scheme 2-4) achieves 11.94%, 11.60% and 5.20% DL average-UPT gain in low, medium and high load. The gain of DL average-UPT decreases with the increase of traffic load.

Dynamic TDD with FFFFU (Scheme 2-4) achieves 18.44%, 20.39% and 19.55% UL average-UPT gain in low, medium and high load.

Observation 7: For FR 1 InH and asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL, compared to legacy TDD with DDDSU (scheme 1-1)

- Semi-static SBFD with XXXXU (Scheme 2-2) has 20.66%, 23.20% and 39.96% DL average-UPT degradation with low, medium and high load. The gain of DL average-UPT decreases with the increase of traffic load.

- Semi-static SBFD with XXXXU (Scheme 2-2) achieves 42.23% ,47.27% and 46.29% UL average-UPT gain with low, medium, and high load.

- Dynamic SBFD with XXXXU (Scheme 2-3) has 4.49%, 6.25% and 15.48% DL average-UPT degradation with low, medium, and high load. The gain of DL average-UPT decreases with the increase of traffic load.

- Dynamic SBFD with XXXXU (Scheme 2-3) achieves 95.02%, 59.65% and 25.69% UL average-UPT gain in low, medium, and high load. The gain of UL average-UPT decreases with the increase of traffic load.

- Dynamic TDD with FFFFU (Scheme 2-4) has 17.97%, 17.46% and 30.20% DL average-UPT gain in low, medium and high load. The gain of DL average-UPT increase with the increase of traffic load.

- Dynamic TDD with FFFFU (Scheme 2-4) achieves 93.32%, 63.03% and 38.13% UL average-UPT gain in low, medium and high load. The gain of UL average-UPT increase with the increase of traffic load.

Observation 8: For FR 1 InH and asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL, compared to semi-static SBFD (scheme 2-2), dynamic SBFD (scheme 2-3) can achieve higher performance in both DL and UL, except UL UPT with high load.

Observation 9: For FR 1 InH and asymmetric packet size with 0.5Mbytes for DL and 0.125Mbytes for UL, as the adjustment periodicity decreases, the DL/UL average-UPT performance further improves for dynamic SBFD and dynamic TDD.

# References

- 3GPP RAN1#112 meeting, chairman’s notes.

# Annex A – Agreements in RAN1 #112

Conclusion

Regarding the feasibility analysis of SBFD, RAN1 focus on feasibility analysis from performance perspective, specification perspective and impact on legacy operation perspective. The study on implementation feasibility is up to RAN4.

Agreement

Update the agreement in RAN1#110bis as below:

For SLS evaluation purposes only, Alt 1/2/4 (SBFD UL subband is about 20% of the channel bandwidth) and SBFD Subband configuration#1 with {DUD} pattern, the following is assumed:

- For FR1

  - Baseline: 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <104, 55, 5>

  - Optional: 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <106, 51, 5>

- For FR2

  - Optional Baseline: 100MHz channel bandwidth and 120kHz SCS (66 PRB) < ND, NU, NG > = <25, 14, 1>

  - Baseline Optional: 200MHz channel bandwidth and 120kHz SCS (132 PRB): < ND, NU, NG > = <4752, 3226, 31>

- Other values of < ND, NU, NG > are not precluded and can be reported by companies.

Agreement

Regarding UE Noise Figure for FR2-1, update the previous agreements as follows.

- 10dB (13dB is not considered in SLS)

Agreement

Use the following BS antenna layout for indoor office scenario (referring to Table 1 in RP-180524), wherein,

- X-axis is pointing down to the floor

- The antenna array is mounted in the Y-Z plane with boresight along the X-axis (ceiling mounted with boresight towards the floor)

- The X-axis/Y-axis/Z-axis refer to LCS

Figure X: Top view of the BS antenna layout for indoor office scenario

Agreement

The following macro cell layout and antenna boresight direction are used in SLS for hexagonal grid with 7 macro sites and 3 sectors per site.

Agreement

The topologies used for 0% and 100% grid shift for SBFD deployment case 4 are updated as below.

Agreement

For UE clustering distribution for SBFD Deployment Case 4 with 0% or 100% grid shift, the following is assumed.

- The UE cluster centers of the first operator are the same as that of the second operator.

- For 100% grid shift, the minimum distance requirement between the UE cluster center and macro TRP should be satisfied for both operators.

- Minimum UE-UE 2D distance is 1m regardless the serving operator

- For each operator, the agreement regarding the UE cluster distribution for SBFD deployment Case 1 is reused

Agreement

Agree the following clarification on UE clustering distribution in Dense Urban Macro layer for FR2-1:

- Randomly drop X (X =1 or 2) UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and for X=2, the minimum distance between two UE cluster centers as Dinter-cluster

- Assuming M (M=10 or 20) users per macro TRP per direction, 80% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, 20% users randomly and uniformly dropped in the macro geographical area outside the clusters. All the UEs (including UEs in the clusters and out of the clusters) are outdoor UEs without car penetration loss (3km/h).

  - If each UE is assigned both UL traffic and DL traffic, there are 8 UEs in one UE cluster.

  - If each UE is either assigned UL traffic or DL traffic, there are 8 UEs with DL traffic and 8 UEs with UL traffic in one UE cluster.

- Note that the UE cluster is totally confined within the macro cell geographical area (i.e. a cluster cannot be partially overlap with adjacent cell area).

Agreement

For SLS of SBFD, use the following values for BS ACLR/ACS (ACLRBS and ACSBS).

|  | FR1 | FR2-1 |
| --- | --- | --- |
| BS ACLR | 45 dB | 28 dB |
| BS ACS | 46 dB | 23.5 dB |

Agreement

For SLS of SBFD, use the following values for UE ACLR/ACS (ACLRUE and ACSUE) for UE-UE adjacent-channel CLI modeling.

|  | FR1 | FR2-1 |
| --- | --- | --- |
| UE ACLR | 30 dB | 23 dB |
| UE ACS | 33 dB | 23 dB |

Agreement

For clarification on BS antenna radiation pattern for indoor office scenario, update the previous agreement in RAN1#110 as below:

For evaluation of SBFD and dynamic/flexible TDD, use BS antenna radiation pattern as following:

- InH: reuse the ceiling-mount antenna pattern in Table 10 in Report ITU-R M.2412 for both FR1&FR2-1 (Table A.2.1-7 in TR 38.802)

- Urban Macro/ Dense Urban Macro layer / Dense Urban Micro layer: reuse Table 9 in Report ITU-R M.2412 for both FR1&FR2-1 (same as 3-sector BS antenna radiation model in Table A.2.1-6 in TR 38.802)

  - Companies can also consider evaluation with other realistic BS antenna radiation pattern

Agreement

Take option-2 for UE-UE channel modelling in FR1 as baseline for the SBFD and dynamic/flexible TDD evaluation.

Agreement

For UE-UE path loss computation based on TR 38.901, extend the applicability range of the equations down to 1m (minimum distance between UEs).

PLUMi-LOS=PL11m 10m≤d2D≤dBP'PL2dBP'≤d2D≤5km ,

Agreement

When two UEs are in different clusters in Urban Macro scenario or Dense Urban Macro Layer scenario for FR1, the standard deviation of shadow fading for NLOS in TR38.901 is used.

Agreement

The following criterion is used to determine whether two indoor UEs are in the same building or not for UE-UE penetration loss calculation:

- For indoor office scenario in FR1/FR2-1, all the UEs are in the same building.

- For Urban Macro or Dense Urban Macro layer scenario in FR1,

  - (baseline) if UE clustering distribution is used, two indoor UEs are considered in the same buildings if they are in the same UE cluster; two indoor UEs are considered in different buildings if they are in different UE clusters.

  - (optional) if uniform UE distribution is used, two indoor UEs are considered in the same building if the inter-user 2D distance ≤ 50m; two indoor UEs are considered in different buildings if the inter-user 2D distance > 50m.

- For Dense Urban Macro layer scenario in FR2, baseline is that all the UEs are outdoor UEs.

  - For the optional case that 20% UEs are outdoor UEs and 80% UEs are indoor UEs, two indoor UEs are considered in the same building if the inter-user 2D distance ≤ 50m; two indoor UEs are considered in different buildings if the inter-user 2D distance > 50m.

- For 2-layer Scenario B in FR1, for the baseline case that all the UEs dropped in macro cell outside the Indoor office / Indoor factory are outdoor UEs, all the indoor UEs are in the same building.

  - Do not consider the optional case that both outdoor UEs and indoor UEs are dropped in the macro cell outside the Indoor office / Indoor factory.

- For Dense Urban with 2-layer scenario in FR1, two indoor UEs are considered in the same building if the inter-user 2D distance ≤ 50m; two indoor UEs are considered in different buildings if the inter-user 2D distance > 50m.

- For Dense Urban Micro layer scenario in FR2, consider all the UEs are outdoor UEs.

Agreement

Regarding Clause 7.5 in TR38.901, confirm the following is RAN1’s common understanding:

- For UMa and UMi-Street Canyon, if the UE is assigned as indoor state, d2D-out=d2D-d2D-in, and d2D-out is used for LOS/NLOS probability calculation.

Agreement

For indoor office scenario agreed for SBFD and dynamic/flexible TDD evaluation, the LOS probability of Indoor - Open office in Table 7.4.2-1 of TR38.901 is used.

- d2D-in used for LOS probability in Table 7.4.2-1 in TR 38.901 is the 2D distance between BS and UE (d2D).

Agreement

Regarding UE-UE LOS probability calculation for SBFD and dynamic/flexible TDD evaluation, when channel model of UMi-Street canyon in TR 38.901 is used for UE-to-UE link, d2D-out in LOS probability formula can be interpreted as follow:

- For outdoor UE to outdoor UE, d2D-out=d2D

- For indoor UE to outdoor UE and outdoor UE to indoor UE, d2D-out=d2D-d2D-in

- (Already agreed) For indoor UE to indoor UE in different buildings, it is always NLOS.

- Note: d2D is the UE-UE 2D distance

Agreement

Adopt the high loss and low loss O2I building penetration loss model in Table 7.4.3-2 in TR 38.901 for penetration loss of Macro-gNB-indoor-gNB channel model (for 2-layer Scenario B only) and UE-UE channel model.

- If InF is used as Layer-2 for 2-layer Scenario B

  - 100% high-loss model

- Otherwise

  - 80% low-loss model

  - 20% high-loss model

- For UEs determined in the same building, each UE selects high loss/low loss building type independently.

Agreement

Regarding gNB-gNB channel model with UMa, extend the applicability range of hUT from 13m=<hUT<=23m to 13m=<hUT<=25m in the formula to get C(d2D, hUT) in Table 7.4.1-1 (Pathloss models) in TR38.901.

Agreement

For SBFD evaluation, assume the maximum BS transmit power is proportional to the number of Tx chains used for transmission

- For SBFD antenna configuration Option-1,

  - in DL-only symbols, the maximum BS transmit power for SBFD is the same as that for legacy TDD

  - in SBFD symbols, the maximum BS transmit power for SBFD is half of that for legacy TDD

- For SBFD antenna configuration Option-2, in both DL-only symbols and SBFD symbols, the maximum BS transmit power for SBFD is always the same as that for legacy TDD

- For SBFD antenna configuration Option-3, in both DL-only symbols and SBFD symbols, the maximum BS transmit power for SBFD is always half of that for legacy TDD

Agreement

For 2-layer scenario B in FR1, reuse the BS transmission power of Urban Macro scenario for layer 1, and reuse the BS transmission power of Indoor office scenario for layer 2.

Agreement

- Update the simulation assumptions for SLS calibration as below

|  | Urban Macro (FR1) | Dense Urban Macro Layer (FR2-1) | Indoor office (FR1) | Indoor office (FR2-1) |
| --- | --- | --- | --- | --- |
| UE number per cluster | 8 | 8 | - |  |
| gNB-UE Channel model (large-scale) | Macro-to-UE: UMa in TR 38.901 For FR1, gNB-UE O2I penetration loss: 80% low-loss model, 20% high-loss model |  | TRP-to-UE: InH-Office in TR 38.901 Penetration loss is not modelled. |  |
| UE-UE Channel model (large-scale) | UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m). For FR1, penetration loss between UEs follows Table A.2.1-12 in TR38.802 |  | UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) |  |

Agreement

For clarification on SLS calibration, for UE-to-UE coupling loss statistics, ignoring the UE pairs if the distance between UEs in a UE pair is larger than 50m is applicable for all of Urban Macro, Dense Urban Macro Layer and Indoor office scenarios.

Agreement

For clarification on the coupling loss formula (2) used for SLS calibration

- Not modelling fast fading doesn’t impact the calculation of path loss PL and shadowed fading SF

- The antenna pattern related part (α'0,u,p) is calculated based on the LOS direction between the two nodes, i.e., θLOS,ZOA/ϕLOS,AOA/θLOS,ZOD/ϕLOS,AOD

Agreement

For link level evaluation of coverage performance, focus on Urban Macro scenario for FR1 and Dense Urban Macro Layer scenario for FR2-1.

Agreement

For SBFD deployment case 1, companies are encouraged to provide evaluation results for the following parameter combinations with higher priority.

- Other parameter combinations are not precluded.

- Note: The parameters that have baseline assumptions are not listed here.

| SBFD deployment case 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter sets | Parameters | Indoor office (FR1) | Urban Macro (FR1) | Indoor office (FR2-1) | Dense Urban Macro Layer (FR2-1) |  |
| SBFD subband and slot configurations | SBFD subband and slot configurations | - Alt 2 (TDD{DDDSU}, SBFD{XXXXU}) - Alt 4 (TDD{DDDSU}, SBFD{XXXXX}) |  |  |  |  |
| Traffic model | DL/UL FTP packet size | - Asymmetric packet size with 4Kbytes for DL and 1Kbyte for UL - Asymmetric packet size with 0.5Mbyte for DL and 0.125 Mbytes for UL |  |  |  |  |
|  | DL/UL traffic load | - {DL:UL}={Low, Low} - {DL:UL}={Medium, Medium} - {DL:UL}={High, High} |  |  |  |
| Antenna configuration | BS antenna configuration for SBFD | SBFD antenna configuration Option-2 |  |  |  |
|  | UE antenna configuration | The UE antenna configurations used for SLS calibration |  |  |  |
| Channel model | gNB-gNB co-channel channel model | Both large scale fading and small scale fading are modelled |  |  |  |
|  | UE-UE co-channel channel model | For FR1, at least large scale fading is modelled. For FR2-1, both large scale fading and small scale fading are modelled |  |  |  |

Agreement

For SLS in RAN1, for co-site gNB-gNB adjacent-channel inter-sector co-channel inter-subband CLI modelling, reuse similar method as co-site inter-sector co-channel inter-subband CLI modeling as follows.

Iadj, co-siteper-RB=Iadj, co-site-sector-1per-RB+Iadj, co-site-sector-2per-RB+Iadj, co-site-sector-3per-RB

Iadj,co-site-sector-xper-RB=Ptxadj,sector-x, per-RBαadj,co-site-x*Nused-DL-RBadj, sector-xNDLRBadj

- Ptxadj, sector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxadj,sector-x, per-RB=Ptxadj,sector-x, max/NDLRBadj.

- Ptxadj,sector-x, max is the maximum DL Tx Power of sector x in adjacent channel (in linear scale).

- NDLRBadj is the total number of DL RBs in adjacent channel.

- Nused-DL-RBadj,sector-x is the number of DL RBs allocated for DL transmission of sector x in adjacent channel.

- αadj, co-site-x is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI between the aggressor sector x and the victim sector.

  - 10*log10αadj,co-site-x=spatial isolationdB+10*log1011ACLRBS+1ACSBS+…

  - FFS the concrete value of αadj, co-site-x

  - ACLRBS and ACSBS are in linear scale.

Send an LS to RAN4 to inquire on the value of αadj, co-site-x.

See approved LS in R1-2302087.

Working assumption:

For co-site inter-sector co-channel inter-subband CLI modelling, before receiving RAN4’s reply on the value of αco-site, RAN1 assume the following only for evaluation:

- FR1:

  - 75dB for spatial isolation (RAN4 typical value).

  - 93dB for spatial isolation (RAN4 best value).

  - 100dB for spatial isolation

- FR2:

  - 88dB for spatial isolation (RAN4 typical value).

  - 98dB for spatial isolation (RAN4 best value).

  - 105dB for spatial isolation

- In addition to spatial isolation and frequency isolation, companies can use digital cancelation and report the value, e,g., 10dB. Above does not imply that RAN1 assumes or does not assume digital cancelation is feasible.

- The feasibility of these values is up to RAN4. These values can be revisited based on further RAN4 inputs.

- The 100dB/105dB isolation values for FR1 and FR2 are not from RAN4, but based on RAN4 input that some companies have proposed that isolating material could be added between sectors to increase the isolation. RAN4 has not yet discussed the details whether such approaches can be applied to outdoor sites.

Agreement

For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for UE-UE co-channel channel model, the power of UE-UE co-channel inter-subband CLI experienced by the victim UE on each receiver chain at DL RB n can be modelled as

IUE-UE-CLIB'→B,per-RBn=IUE-UE,IBEn*CLUE B'→UE B+PtxUE B'*CLUE B'→UE B*1ICSUE*1NULRB

where

- IUE-UE-CLIB'→B,per-RBn is the power of UE-UE co-channel inter-subband CLI from aggressor UE B' to victim UE B on each receiver chain at one DL RB n (linear value).

- PtxUE B' is UL transmission power of UE B' across all transmit chains over the allocated UL RBs (linear value)

- CLUE B'→UE B is the coupling loss between UE B' and UE B (linear value), accounting for analog beamforming at the aggressor UE and victim UE

- NULRB is the total number of UL RBs in the UL subband

- ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input.

- 10*log10IUE-UE,IBEn=maxPtxUE B',per-RB-30dB,10*log10IGeneral+IIQfor FR1maxPtxUE B',per-RB-25dB,10*log10IGeneral+IIQfor FR2-1, wherein,

  - For SBFD Subband configuration with {DUD} pattern, IIQ can be ignored

  - 10*log10IGeneral=PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-3-5⋅ΔRB-1LCRB,-57dBm+10log10SCS/15kHz-PtxUE B',per-RBfor FR1PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-5⋅ΔRB-1LCRB,-55.1dBm-PtxUE B',per-RBfor FR2-1

  - PtxUE B',per-RB is UL transmission power of UE B' across all transmit chains per RB (linear value). PtxUE B',per-RB=PtxUE B'/ LCRB, and LCRB is the number of UL RBs allocated for UL transmission of UE B'.

  - NRB is the Transmission Bandwidth Configuration, referring to Table 5.3.2-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1.

    - NRB=273 for FR1 with 100MHz transmission bandwidth and 30kHz SCS

    - NRB=132 for FR2-1 with 200MHz transmission bandwidth and 120kHz SCS

  - ΔRB is the starting frequency offset between the allocated UL RBs and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated UL RBs)

  - EVM is the limit specified in Table 6.4.2.1-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1 for the modulation format used in the allocated RBs.

Send an LS to RAN4 to inform them of the above agreement and to check if the RAN1 agreement is inline with RAN4’s understanding.

See approved LS in R1-2302087.

Working assumption:

For SLS in RAN1, if both large-scale and small-scale fading are modelled for UE-UE co-channel channel model, the UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE can be modeled as:

IInter-UE-CLIn=Ileakagen+Iselectivity, where,

- Ileakagen=HCLIn W yn is the first part of UE-UE co-channel inter-subband CLI across all Rx chains at DL RB n, caused by power leakage at aggressor UE,

  - HCLIn is the NR×NT channel matrix between aggressor UE and victim UE at DL RB n, the beamforming of the aggressor UE and the victim UE can be taken into account by HCLIn

  - NR is the number of Rx chains and NT is the number of Tx chains

  - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1.

  - yn=y0n,y1n,…,yRnT,

    - ykn~N0,IUE-UE,IBEn, k=0,1,…,R-1, is modelled as white Gaussian noise

    - IUE-UE,IBEn has the same meaning as in the agreement for the case only large-scale fading is modelled

- Iselectivity is modelled as frequency flat

Iselectivity=z0n,z1n,…,zNR-1nT

  - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise, σz,n2=PblockerICSUE×NULRB

  - Pblocker=1NR*m∈Used UL RBsHCLImWsm2

  - HCLIm is the NR×NT channel matrix between aggressor UE and victim UE at UL RB m, the analog beams of the aggressor UE and the victim gNB can be taken into account by HCLIm,

  - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1

  - sm is the symbol transmitted at UL RB m at aggressor UE with transmission power for each layer as PtxUE B',per-RB.

    - PtxUE B',per-RB has the same meaning as in the agreement for the case only large-scale fading is modelled

  - NULRB is the total number of UL RBs in the UL subbands,

  - ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input.

Send an LS to RAN4 to inform them of the above agreement and to check if the RAN1 agreement is inline with RAN4’s understanding.

See approved LS in R1-2302087.

Agreement

The following is used to generate  d2D-in for a UE-UE link associated with an indoor UE (the other UE could be an outdoor UE or an indoor UE in a different building) in order to calculate the inside loss component (PLin) of the UE-UE O2I building penetration loss.

- d2D-in =Uniform(0,min⁡{25m,d2D2})

Agreement

For SLS of SBFD and dynamic/flexible TDD (including SLS calibration), distance-based wrap-round is used.

Agreement

For BS transmit power for SBFD, take option 1 as baseline. Option 2 can also be evaluated.

- Option-1: Power boosting is not assumed for SBFD symbols compared to DL-only symbols (as in legacy systems), i.e., BS transmit power spectrum density per Tx chain is kept the same for SBFD symbols and DL-only symbols

- Option-2: Power boosting is assumed for SBFD symbols compared to DL-only symbols, i.e.,

  - DL symbols in SBFD operation have the same PSD as used in TDD DL symbols

  - For SBFD symbols, its PSD is scaled according to the number of RBs in DL subband(s), e.g.,

    - PSDSBFDPer Tx-chain=PSDDL-onlyPer Tx-chain∙BWcarrierBWDL subbands

    - PSDSBFDPer Tx-chain is the BS transmit power spectrum density per Tx chain in SBFD symbols

    - PSDDL-onlyPer Tx-chain is the BS transmit power spectrum density per Tx chain in DL-only symbols

    - BWcarrier is the system bandwidth and BWDL subbands is the total bandwidth of DL subbands

Agreement

RAN1 to update the inter-site gNB-gNB adjacent-channel CLI model as follows

- PtxBS A' is DL transmission power of gNB A' across all transmit chains over all the scheduled DL RBs (linear value).

Agreement

For evaluation of SBFD and dynamic/flexible TDD, the O2I car penetration loss is modelled with μ = 9, and σP = 5.

Agreement

For Deployment case 3-2 (2-layer Scenario B), update Indoor-TRP to outdoor UE channel model as follows:

| Large-scale channel parameters | Indoor TRP to Outdoor UE: - Option 1: - A.2.1.2 in TR36.843 - Penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Option 1: - UMi-Street canyon in TR 38.901 (hBS =3 m) - Option 2: - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - For both options, O2I penetration loss between indoor TRP and outdoor UE follows Table A.2.1-12 in TR38.802 (d2D-in is the distance between the indoor TRP and the building boundary along the direction from Indoor TRP to outdoor UE. The d2D-in may be different for different indoor-TRP-outdoor-UE links associated with the same indoor TRP) |
| --- | --- |
| Fast fading parameters | Indoor TRP to Outdoor UE: - Option 1: - 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 1: - UMi-Street canyon in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA - Option 2: - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 |

Agreement

Regarding the UE distribution of 2-layer Scenario B, for indoor/outdoor UE proportion in Layer 1 (Urban Macro), Option 2 is not considered in SLS.

- Option 2 (optional):

  - 20% outdoor in cars: 30km/h, UE height is 1.5m

  - 80% indoor in houses: 3km/h, UE height is 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8)

Agreement

For Indoor factory of 2-layer Scenario B, the following layout for indoor office scenario is reused, and the other simulation assumptions follow InF-SL in Table 7.8-7 (Simulation assumptions for large scale calibration for the indoor factory scenario) in TR 38.901.

|  | Layout | Inter-BS (2D) distance | Minimum BS-UE (2D) distance | Minimum UE-UE (2D) distance |
| --- | --- | --- | --- | --- |
| Indoor factory | 12BSs per 120m x 50m | 20m | 0m | 1m |

Figure X: Layout for indoor factory (reuse the layout for indoor office)

Agreement

For link level evaluation of coverage performance, focus on the following uplink channels.

- PUSCH with 1Mbps target data rate for FR1

- PUSCH with 5Mbps target data rate for FR2-1

- FFS: PUCCH

- Note: the data rate is based on TR38.830

Agreement

For link level evaluation of coverage performance for SBFD, the following interference components are added per each receive chain to the UL channel at SBFD symbols:

- Self-interference, modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense similar to SLS.

- Co-site inter-sector interference, modelled as additive white gaussian noise with fixed INR = - X dB based on assumption of co-site isolation αco-site

- Inter-site gNB-gNB co-channel inter-subband CLI,

  - Alt-1: the value of interference power is selected according to the INR distribution drawn based on the statistics from SLS.

  - Alt-2: the value of interference power is determined based on the inter-site gNB-gNB co-channel inter-subband CLI model agreed for SLS taking into account the locations of victim gNB and several aggressor gNBs, and the gNB-gNB channel model

- FFS: Receiver blocking model

Agreement

For link level evaluation of coverage performance, use Alt 2 defined in SLS.

- Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD):

  - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U]

  - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about 20% of the channel bandwidth.

Agreement

Regarding the schemes for link level evaluation of PUSCH coverage performance,

- For baseline legacy TDD, consider

  - Single slot PUSCH transmission

- For SBFD, consider the following techniques of coverage enhancement:

  - Case 1: SBFD with single slot PUSCH

  - Case 2: SBFD with PUSCH repetition type A

  - Case 3: SBFD with TBoMS PUSCH

  - Case 4: SBFD with PUSCH repetition type A and joint channel estimation

    - FFS: Joint channel estimation across SBFD and non-SBFD slots

  - Case 5: SBFD with TBoMS PUSCH and joint channel estimation

    - FFS: Joint channel estimation across SBFD and non-SBFD slots

- UL coverage metrics are obtained using link budget template and TDD/SBFD required SINR for target data rate.

Note: Evaluation accounts for different SINR level between SBFD and non-SBFD slots

# Annex B

Table B-1. System-level simulation assumption for NR Full Duplex

| Parameters | values |
| --- | --- |
| Scenario | Indoor hotspot |
| Layout | 12 BSs per 120 m x 50 m |
| Inter-BS distance | 20m |
| Minimum BS-UE (2D) distance | 0m |
| Minimum UE-UE (2D) distance | 1m |
| Carrier frequency | 4GHz |
| Simulation bandwidth | 100MHz |
| Subcarrier spacing | 14 OFDM symbol slot SCS = 30kHz |
| < ND, NU, NG > | DU pattern，<214, 54, 5> |
| gNB-UE Channel model | TRP-to-UE: InH-Office in TR 38.901 |
| gNB-gNB Channel model (large-scale) | TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) |
| gNB-gNB Channel model (small-scale) | TRP-to-TRP: InH-Office in TR 38.901 (hUE=3m), ASA and ZSA statistics updated to be the same as ASD and ZSD |
| UE-UE Channel model (large-scale) | UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) |
| BS Tx power | 24dBm |
| UE number (per direction) (M) | 10 users per TRP |
| UE Tx power | 23dBm |
| UL power control | P0= -60 dBm, alpha = 0.6 |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9 dB |
| UE distribution | 100% indoor |
| BS antenna height | 3m |
| UE antenna height | 1.5m |
| BS antenna element gain + connector loss | 5dBi, Table 10 in Report ITU-R M.2412 |
| BS antenna configurations | For legacy TDD or dynamic TDD: M,N,P,Mg,Ng;Mp,Np = (4,4,2,1,1; 4,4) dH,dV= (0.5, 0.5) λ, +45°/-45° polarization For SBFD, there are two panel groups, for each panel group: M,N,P,Mg,Ng;Mp,Np = (4,4,2,1,1; 4,4) dH,dV= (0.5, 0.5) λ, +45°/-45° polarization |
| Mechanic tilt | 180° in GCS (pointing to the ground) |
| Beam set at TRxP | Azimuth angle φi = [0], Zenith angle θj = [90]. |
| UE antenna gain | 0dBi |
| UE antenna configurations | (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1; 1,2) |
| UE beam set | Azimuth angle φi = [0], Zenith angle θj = [90] |
| Channel estimation | Ideal |
| UE processing capability | UE processing capability 1 |
| receiver | MMSE-IRC as the baseline receiver. |
| Handover margin | 3dB |
| UE attachment | Based on RSRP from port 0 |
| DL/UL Modulation | Up to 256QAM |
| Transmission scheme | SU-MIMO, maximum layers = 2 |
| Scheduling | PF |

# Annex C

## C.1. Frame structure#3 (XXXXX)

For SBFD and dynamic TDD with frame structure#3, the following schemes are evaluated:

| Scheme 1-1 (baseline) | Baseline legacy TDD operation with UL/DL configuration of Frame structure #1 DDDSU (S slot: 12D:2S:0U) |
| --- | --- |
| Scheme 1-2 (semi-static SBFD) | Semi-static SBFD operation with 20% UL subband resource in X slot, with Frame structure #3 (XXXXX) |
| Scheme 1-3 (dynamic SBFD) | Dynamic SBFD operation with adaptation among {Full DL, Full UL, SBFD operation with 20% UL subband resource} in X slot, with Frame structure#3 (XXXXX). The X slot is determined based on the required resources for DL/UL traffic transmission per 5 slots. - Note that for Scheme 1-3, the SBFD pattern in frequency domain (SBFD operation with 20% UL subband resource) is semi-statically configured. Scheme 1-3 is to imitate the scheme that dynamic determination on whether to apply the SBFD pattern in the X slot. |
| Scheme 1-4 (dynamic TDD) | Dynamic TDD operation with adaptation among {Full DL, Full UL, S slot (DL: GAP: UL) = (6:2:6)} in F slot, with configuration FFFFF, where F slots are determined based on the required resources for DL/UL traffic transmission per 5 slots |

- C.1.1. Small packet size

C.1.1.1. Low DL RU and low UL RU

Table C-1. DL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 39.59 | 37.97 | 39.29 | 42.11 |
|  | Scheme 1-2 | 40.78 | 39.29 | 40.53 | 42.80 |
|  | Scheme 1-3 | 40.24 | 38.75 | 40.00 | 42.42 |
|  | Scheme 1-4 | 46.75 | 43.97 | 46.37 | 50.63 |

Table C-2. UL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 3.87 | 3.66 | 3.86 | 4.09 |
|  | Scheme 1-2 | 6.39 | 6.22 | 6.40 | 6.52 |
|  | Scheme 1-3 | 5.95 | 5.75 | 5.96 | 6.10 |
|  | Scheme 1-4 | 4.12 | 3.91 | 4.12 | 4.30 |

Table C-3. DL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 1.02 | 0.52 | 0.82 | 3.91 |
|  | Scheme 1-2 | 0.88 | 0.53 | 0.80 | 2.38 |
|  | Scheme 1-3 | 0.91 | 0.53 | 0.81 | 2.70 |
|  | Scheme 1-4 | 1.04 | 0.35 | 0.73 | 6.31 |

Table C-4. UL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 2.41 | 1.13 | 2.33 | 5.68 |
|  | Scheme 1-2 | 1.28 | 1.03 | 1.26 | 2.33 |
|  | Scheme 1-3 | 1.41 | 1.03 | 1.34 | 2.78 |
|  | Scheme 1-4 | 2.26 | 1.10 | 2.17 | 5.37 |

Table C-5. DL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 1.02 | 0.83 | 1.05 | 1.13 |
|  | Scheme 1-2 | 0.88 | 0.80 | 0.90 | 0.93 |
|  | Scheme 1-3 | 0.91 | 0.81 | 0.92 | 0.97 |
|  | Scheme 1-4 | 1.04 | 0.73 | 1.11 | 1.23 |

Table C-6. UL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 2.41 | 2.22 | 2.40 | 2.67 |
|  | Scheme 1-2 | 1.28 | 1.25 | 1.28 | 1.35 |
|  | Scheme 1-3 | 1.41 | 1.36 | 1.40 | 1.51 |
|  | Scheme 1-4 | 2.26 | 2.12 | 2.25 | 2.45 |

Table C-7. RU for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | DL type-1 RU | UL type-1 RU | DL type-2 RU | UL type-2 RU |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 6.35% | 2.00% | 8.23% | 10.00% |
|  | Scheme 1-2 | 6.55% | 1.98% | 8.36% | 10.00% |
|  | Scheme 1-3 | 6.53% | 2.05% | 8.33% | 10.29% |
|  | Scheme 1-4 | 6.45% | 1.87% | 7.78% | 13.07% |

C.1.1.2. Medium DL RU and medium UL RU

Table C-8. DL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 38.48 | 37.00 | 38.00 | 41.60 |
|  | Scheme 1-2 | 36.88 | 35.68 | 36.50 | 39.36 |
|  | Scheme 1-3 | 37.10 | 35.86 | 36.80 | 39.43 |
|  | Scheme 1-4 | 45.09 | 42.71 | 44.56 | 49.63 |

Table C-9. UL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 3.76 | 3.54 | 3.73 | 4.00 |
|  | Scheme 1-2 | 6.24 | 6.06 | 6.24 | 6.44 |
|  | Scheme 1-3 | 5.84 | 5.63 | 5.82 | 6.05 |
|  | Scheme 1-4 | 3.96 | 3.65 | 3.96 | 4.21 |

Table C-10. DL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 1.10 | 0.52 | 0.83 | 3.94 |
|  | Scheme 1-2 | 1.02 | 0.54 | 0.88 | 2.84 |
|  | Scheme 1-3 | 1.02 | 0.54 | 0.88 | 2.87 |
|  | Scheme 1-4 | 1.17 | 0.36 | 0.75 | 6.41 |

Table C-11. UL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 2.52 | 1.14 | 2.40 | 5.91 |
|  | Scheme 1-2 | 1.33 | 1.03 | 1.28 | 2.45 |
|  | Scheme 1-3 | 1.45 | 1.04 | 1.35 | 2.90 |
|  | Scheme 1-4 | 2.41 | 1.12 | 2.26 | 5.92 |

Table C-12. DL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 1.10 | 0.84 | 1.14 | 1.21 |
|  | Scheme 1-2 | 1.02 | 0.90 | 1.04 | 1.08 |
|  | Scheme 1-3 | 1.02 | 0.89 | 1.03 | 1.08 |
|  | Scheme 1-4 | 1.17 | 0.74 | 1.22 | 1.36 |

Table C-13. UL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 2.52 | 2.26 | 2.56 | 2.82 |
|  | Scheme 1-2 | 1.33 | 1.26 | 1.33 | 1.40 |
|  | Scheme 1-3 | 1.45 | 1.37 | 1.46 | 1.57 |
|  | Scheme 1-4 | 2.41 | 2.18 | 2.38 | 2.79 |

Table C-14. RU for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | DL type-1 RU | UL type-1 RU | DL type-2 RU | UL type-2 RU |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 20.98% | 5.28% | 27.19% | 26.38% |
|  | Scheme 1-2 | 20.61% | 6.81% | 26.29% | 34.42% |
|  | Scheme 1-3 | 20.61% | 6.67% | 25.95% | 35.32% |
|  | Scheme 1-4 | 21.97% | 5.44% | 26.32% | 40.04% |

C.1.1.3. High DL RU and high UL RU

Table C-15. DL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 36.10 | 30.13 | 36.57 | 41.48 |
|  | Scheme 1-2 | 30.16 | 23.93 | 30.88 | 34.48 |
|  | Scheme 1-3 | 31.54 | 26.24 | 32.28 | 35.20 |
|  | Scheme 1-4 | 40.34 | 28.88 | 41.88 | 48.02 |

Table C-16. UL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 3.69 | 3.42 | 3.63 | 3.99 |
|  | Scheme 1-2 | 5.77 | 5.09 | 5.81 | 6.26 |
|  | Scheme 1-3 | 4.91 | 3.58 | 4.95 | 5.88 |
|  | Scheme 1-4 | 3.54 | 2.15 | 3.75 | 4.34 |

Table C-17. DL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 1.32 | 0.52 | 0.87 | 4.87 |
|  | Scheme 1-2 | 1.39 | 0.56 | 1.16 | 4.56 |
|  | Scheme 1-3 | 1.29 | 0.56 | 1.08 | 3.99 |
|  | Scheme 1-4 | 1.88 | 0.41 | 0.83 | 12.84 |

Table C-18. UL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 2.61 | 1.15 | 2.45 | 6.59 |
|  | Scheme 1-2 | 1.50 | 1.04 | 1.35 | 3.46 |
|  | Scheme 1-3 | 2.13 | 1.05 | 1.56 | 10.67 |
|  | Scheme 1-4 | 4.07 | 1.13 | 2.50 | 24.45 |

Table C-19. DL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 1.32 | 0.84 | 1.19 | 1.75 |
|  | Scheme 1-2 | 1.39 | 1.07 | 1.30 | 1.97 |
|  | Scheme 1-3 | 1.29 | 1.04 | 1.22 | 1.62 |
|  | Scheme 1-4 | 1.88 | 0.79 | 1.26 | 3.18 |

Table C-20. UL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 2.61 | 2.27 | 2.68 | 3.20 |
|  | Scheme 1-2 | 1.50 | 1.31 | 1.47 | 2.23 |
|  | Scheme 1-3 | 2.13 | 1.43 | 1.89 | 5.86 |
|  | Scheme 1-4 | 4.08 | 2.11 | 2.76 | 14.63 |

Table C-21. RU for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | DL type-1 RU | UL type-1 RU | DL type-2 RU | UL type-2 RU |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 53.63% | 11.15% | 69.53% | 55.77% |
|  | Scheme 1-2 | 51.74% | 15.07% | 66.01% | 76.21% |
|  | Scheme 1-3 | 52.14% | 14.64% | 66.03% | 75.73% |
|  | Scheme 1-4 | 55.64% | 12.27% | 70.11% | 69.17% |

- C.1.2. Large packet size

C.1.2.1. Low DL RU and low UL RU

Table C-22. DL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 684.54 | 523.64 | 709.43 | 758.93 |
|  | Scheme 1-2 | 690.24 | 620.30 | 698.12 | 739.27 |
|  | Scheme 1-3 | 761.43 | 596.50 | 782.68 | 852.50 |
|  | Scheme 1-4 | 693.63 | 533.57 | 708.81 | 798.69 |

Table C-23. UL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 204.43 | 193.90 | 204.07 | 215.91 |
|  | Scheme 1-2 | 178.73 | 171.62 | 179.00 | 185.03 |
|  | Scheme 1-3 | 363.29 | 333.33 | 363.98 | 391.41 |
|  | Scheme 1-4 | 382.90 | 336.55 | 385.05 | 419.59 |

Table C-24. DL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 6.35 | 4.63 | 5.36 | 15.63 |
|  | Scheme 1-2 | 6.15 | 5.04 | 5.35 | 14.26 |
|  | Scheme 1-3 | 5.72 | 4.11 | 4.81 | 13.46 |
|  | Scheme 1-4 | 6.38 | 4.22 | 5.34 | 15.68 |

Table C-25. UL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 5.24 | 3.64 | 4.91 | 11.28 |
|  | Scheme 1-2 | 5.82 | 5.03 | 5.30 | 12.11 |
|  | Scheme 1-3 | 3.17 | 1.65 | 2.96 | 8.68 |
|  | Scheme 1-4 | 3.01 | 1.66 | 2.63 | 9.40 |

Table C-26. DL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 6.34 | 5.43 | 6.02 | 8.32 |
|  | Scheme 1-2 | 6.15 | 5.50 | 6.04 | 7.10 |
|  | Scheme 1-3 | 5.72 | 4.80 | 5.45 | 7.40 |
|  | Scheme 1-4 | 6.39 | 5.15 | 6.15 | 8.36 |

Table C-27. UL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 5.23 | 4.86 | 5.21 | 5.81 |
|  | Scheme 1-2 | 5.82 | 5.50 | 5.80 | 6.40 |
|  | Scheme 1-3 | 3.17 | 2.85 | 3.15 | 3.72 |
|  | Scheme 1-4 | 3.02 | 2.63 | 2.97 | 3.86 |

Table C-28. RU for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | DL type-1 RU | UL type-1 RU | DL type-2 RU | UL type-2 RU |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 7.37% | 2.03% | 9.55% | 10.14% |
|  | Scheme 1-2 | 6.98% | 2.02% | 8.90% | 10.22% |
|  | Scheme 1-3 | 7.35% | 2.04% | 9.60% | 9.54% |
|  | Scheme 1-4 | 7.57% | 2.02% | 10.66% | 7.75% |

C.1.2.2. Medium DL RU and medium UL RU

Table C-29. DL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 512.53 | 291.85 | 507.99 | 702.78 |
|  | Scheme 1-2 | 535.37 | 349.18 | 543.58 | 671.25 |
|  | Scheme 1-3 | 598.45 | 371.46 | 604.59 | 771.49 |
|  | Scheme 1-4 | 550.21 | 341.48 | 542.88 | 760.11 |

Table C-30. UL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 186.08 | 171.15 | 187.05 | 199.95 |
|  | Scheme 1-2 | 161.15 | 152.95 | 161.36 | 169.02 |
|  | Scheme 1-3 | 275.85 | 248.05 | 277.07 | 298.68 |
|  | Scheme 1-4 | 278.85 | 238.21 | 281.73 | 306.25 |

Table C-31. DL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 10.09 | 4.97 | 7.81 | 37.91 |
|  | Scheme 1-2 | 9.30 | 5.08 | 7.20 | 32.34 |
|  | Scheme 1-3 | 8.14 | 4.28 | 6.65 | 25.99 |
|  | Scheme 1-4 | 9.22 | 4.39 | 7.44 | 31.43 |

Table C-32. UL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 6.20 | 3.68 | 5.24 | 18.13 |
|  | Scheme 1-2 | 6.90 | 5.04 | 5.42 | 18.30 |
|  | Scheme 1-3 | 4.60 | 2.14 | 3.67 | 17.72 |
|  | Scheme 1-4 | 4.65 | 2.18 | 3.47 | 16.46 |

Table C-33. DL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 10.07 | 6.23 | 9.32 | 16.79 |
|  | Scheme 1-2 | 9.31 | 6.54 | 8.84 | 14.03 |
|  | Scheme 1-3 | 8.15 | 5.63 | 7.58 | 12.77 |
|  | Scheme 1-4 | 9.20 | 5.83 | 8.73 | 14.10 |

Table C-34. UL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 6.19 | 5.58 | 6.12 | 7.73 |
|  | Scheme 1-2 | 6.90 | 6.37 | 6.88 | 7.97 |
|  | Scheme 1-3 | 4.60 | 3.95 | 4.54 | 5.87 |
|  | Scheme 1-4 | 4.65 | 3.94 | 4.57 | 5.94 |

Table C-35. RU for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | DL type-1 RU | UL type-1 RU | DL type-2 RU | UL type-2 RU |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 22.18% | 5.04% | 28.75% | 25.20% |
|  | Scheme 1-2 | 20.45% | 5.00% | 26.09% | 25.27% |
|  | Scheme 1-3 | 21.14% | 4.92% | 27.32% | 24.08% |
|  | Scheme 1-4 | 22.55% | 4.99% | 28.34% | 28.43% |

C.1.2.3. High DL RU and high UL RU

Table C-36. DL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 346.01 | 132.71 | 322.75 | 623.75 |
|  | Scheme 1-2 | 337.93 | 146.47 | 316.81 | 592.45 |
|  | Scheme 1-3 | 375.68 | 155.57 | 357.90 | 656.17 |
|  | Scheme 1-4 | 323.93 | 114.67 | 304.70 | 671.64 |

Table C-37. UL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 152.01 | 121.33 | 153.22 | 174.06 |
|  | Scheme 1-2 | 120.61 | 94.20 | 122.72 | 138.55 |
|  | Scheme 1-3 | 163.59 | 110.46 | 163.93 | 208.61 |
|  | Scheme 1-4 | 149.46 | 89.23 | 147.95 | 210.71 |

Table C-38. DL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 23.65 | 5.23 | 13.06 | 156.45 |
|  | Scheme 1-2 | 22.86 | 5.31 | 13.42 | 140.29 |
|  | Scheme 1-3 | 19.26 | 4.94 | 11.73 | 112.22 |
|  | Scheme 1-4 | 26.84 | 5.09 | 14.45 | 177.64 |

Table C-39. UL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 9.72 | 3.76 | 6.21 | 46.04 |
|  | Scheme 1-2 | 11.75 | 5.10 | 8.66 | 49.63 |
|  | Scheme 1-3 | 12.79 | 2.71 | 7.16 | 86.98 |
|  | Scheme 1-4 | 18.18 | 2.81 | 8.74 | 164.58 |

Table C-40. DL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 23.58 | 8.09 | 19.22 | 50.55 |
|  | Scheme 1-2 | 22.80 | 8.29 | 18.90 | 50.36 |
|  | Scheme 1-3 | 19.22 | 7.24 | 15.48 | 39.63 |
|  | Scheme 1-4 | 26.86 | 7.24 | 20.26 | 67.23 |

Table C-41. UL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 9.69 | 7.30 | 9.29 | 15.72 |
|  | Scheme 1-2 | 11.72 | 9.23 | 11.18 | 18.18 |
|  | Scheme 1-3 | 12.76 | 7.31 | 11.22 | 34.62 |
|  | Scheme 1-4 | 18.14 | 7.32 | 14.99 | 53.85 |

Table C-42. RU for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | DL type-1 RU | UL type-1 RU | DL type-2 RU | UL type-2 RU |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 52.22% | 11.41% | 67.69% | 57.06% |
|  | Scheme 1-2 | 52.10% | 11.54% | 66.46% | 58.36% |
|  | Scheme 1-3 | 53.14% | 12.03% | 68.10% | 58.87% |
|  | Scheme 1-4 | 59.69% | 12.59% | 74.82% | 72.52% |

C.2 Frame structure#2 (XXXXU)

For SBFD and dynamic TDD with frame structure#2, the following schemes are evaluated. The baseline scheme (legacy TDD with DDDSU) is scheme 1-1 as in section 2.1.

| Scheme 2-2 (semi-static SBFD) | Semi-static SBFD operation with 20% UL subband resource in X slot, with Frame structure #3 (XXXXU) |
| --- | --- |
| Scheme 2-3 (dynamic SBFD) | Dynamic SBFD operation with adaptation among {Full DL, Full UL, SBFD operation with 20% UL subband resource} in X slot, with Frame structure#3 (XXXXU). The X slot is determined based on the required resources for DL/UL traffic transmission per 5 slots. - Note that for Scheme 2-3, the SBFD pattern in frequency domain (SBFD operation with 20% UL subband resource) is semi-statically configured. Scheme 2-3 is to imitate the scheme that dynamic determination on whether to apply the SBFD pattern in the X slot. |
| Scheme 2-4 (dynamic TDD) | Dynamic TDD operation with adaptation among {Full DL, Full UL, S slot (DL: GAP: UL) = (6:2:6)} in F slot, with configuration FFFFU, where F slots are determined based on the required resources for DL/UL traffic transmission per 5 slots |

- C.2.1. Small packet size

C.2.1.1. Low DL RU and low UL RU

Table C-43. DL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 39.59 | 37.97 | 39.29 | 42.11 |
|  | Scheme 2-2 | 37.49 | 36.17 | 37.25 | 39.49 |
|  | Scheme 2-3 | 39.21 | 37.56 | 38.89 | 41.59 |
|  | Scheme 2-4 | 44.32 | 41.49 | 44.18 | 47.65 |

Table C-44. UL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 3.87 | 3.66 | 3.86 | 4.09 |
|  | Scheme 2-2 | 5.91 | 5.58 | 5.95 | 6.14 |
|  | Scheme 2-3 | 4.59 | 4.38 | 4.59 | 4.79 |
|  | Scheme 2-4 | 4.58 | 4.31 | 4.59 | 4.84 |

Table C-45. DL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 1.02 | 0.52 | 0.82 | 3.91 |
|  | Scheme 2-2 | 0.99 | 0.54 | 0.87 | 2.64 |
|  | Scheme 2-3 | 1.05 | 0.51 | 0.83 | 4.21 |
|  | Scheme 2-4 | 1.01 | 0.35 | 0.78 | 4.36 |

Table C-46. UL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 2.41 | 1.13 | 2.33 | 5.68 |
|  | Scheme 2-2 | 1.43 | 1.03 | 1.34 | 2.85 |
|  | Scheme 2-3 | 2.01 | 1.07 | 1.84 | 4.59 |
|  | Scheme 2-4 | 2.05 | 1.07 | 1.86 | 5.27 |

Table C-47. DL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 1.02 | 0.83 | 1.05 | 1.13 |
|  | Scheme 2-2 | 0.99 | 0.89 | 1.01 | 1.05 |
|  | Scheme 2-3 | 1.05 | 0.85 | 1.08 | 1.15 |
|  | Scheme 2-4 | 1.01 | 0.79 | 1.05 | 1.12 |

Table C-48. UL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 2.41 | 2.22 | 2.40 | 2.67 |
|  | Scheme 2-2 | 1.43 | 1.34 | 1.42 | 1.61 |
|  | Scheme 2-3 | 2.01 | 1.89 | 2.01 | 2.18 |
|  | Scheme 2-4 | 2.05 | 1.85 | 2.03 | 2.33 |

Table C-49. RU for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | DL type-1 RU | UL type-1 RU | DL type-2 RU | UL type-2 RU |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 160 packet/s arrival rate for DL 180 packet/s arrival rate for UL. | Scheme 1-1 | 6.35% | 2.00% | 8.23% | 10.00% |
|  | Scheme 2-2 | 6.47% | 2.22% | 10.32% | 6.19% |
|  | Scheme 2-3 | 6.49% | 2.03% | 8.92% | 8.66% |
|  | Scheme 2-4 | 6.48% | 2.06% | 9.46% | 7.22% |

C.2.1.2. Medium DL RU and medium UL RU

Table C-50. DL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 38.48 | 37.00 | 38.00 | 41.60 |
|  | Scheme 2-2 | 33.78 | 32.02 | 33.61 | 36.44 |
|  | Scheme 2-3 | 37.98 | 36.57 | 37.61 | 40.94 |
|  | Scheme 2-4 | 42.94 | 40.59 | 42.51 | 47.25 |

Table C-51. UL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 3.76 | 3.54 | 3.73 | 4.00 |
|  | Scheme 2-2 | 5.78 | 5.43 | 5.78 | 6.06 |
|  | Scheme 2-3 | 4.17 | 3.86 | 4.15 | 4.53 |
|  | Scheme 2-4 | 4.53 | 4.35 | 4.49 | 4.79 |

Table C-52. DL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 1.10 | 0.52 | 0.83 | 3.94 |
|  | Scheme 2-2 | 1.15 | 0.55 | 0.97 | 3.25 |
|  | Scheme 2-3 | 1.13 | 0.51 | 0.84 | 4.31 |
|  | Scheme 2-4 | 1.08 | 0.36 | 0.80 | 4.38 |

Table C-53. UL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 2.52 | 1.14 | 2.40 | 5.91 |
|  | Scheme 2-2 | 1.48 | 1.04 | 1.36 | 2.95 |
|  | Scheme 2-3 | 2.28 | 1.09 | 2.12 | 5.76 |
|  | Scheme 2-4 | 2.07 | 1.07 | 1.89 | 5.26 |

Table C-54. DL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 1.10 | 0.84 | 1.14 | 1.21 |
|  | Scheme 2-2 | 1.15 | 0.99 | 1.16 | 1.25 |
|  | Scheme 2-3 | 1.13 | 0.88 | 1.17 | 1.23 |
|  | Scheme 2-4 | 1.08 | 0.80 | 1.12 | 1.19 |

Table C-55. UL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 2.52 | 2.26 | 2.56 | 2.82 |
|  | Scheme 2-2 | 1.48 | 1.36 | 1.48 | 1.69 |
|  | Scheme 2-3 | 2.28 | 2.03 | 2.27 | 2.60 |
|  | Scheme 2-4 | 2.07 | 1.87 | 2.12 | 2.25 |

Table C-56. RU for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | DL type-1 RU | UL type-1 RU | DL type-2 RU | UL type-2 RU |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 500 packet/s arrival rate for DL 600 packet/s arrival rate for UL. | Scheme 1-1 | 20.98% | 5.28% | 27.19% | 26.38% |
|  | Scheme 2-2 | 23.50% | 7.13% | 37.47% | 19.89% |
|  | Scheme 2-3 | 21.53% | 5.55% | 29.18% | 25.03% |
|  | Scheme 2-4 | 22.17% | 6.01% | 32.33% | 21.05% |

C.2.1.3. High DL RU and high UL RU

Table C-57. DL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 36.10 | 30.13 | 36.57 | 41.48 |
|  | Scheme 2-2 | 23.54 | 0.41 | 26.71 | 32.46 |
|  | Scheme 2-3 | 35.87 | 30.56 | 36.33 | 39.52 |
|  | Scheme 2-4 | 37.98 | 23.81 | 40.03 | 46.99 |

Table C-58. UL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 3.69 | 3.42 | 3.63 | 3.99 |
|  | Scheme 2-2 | 5.15 | 4.08 | 5.19 | 5.91 |
|  | Scheme 2-3 | 3.86 | 3.50 | 3.80 | 4.36 |
|  | Scheme 2-4 | 4.41 | 4.12 | 4.36 | 4.75 |

Table C-59. DL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 1.32 | 0.52 | 0.87 | 4.87 |
|  | Scheme 2-2 | 12.78 | 0.59 | 1.47 | 302.62 |
|  | Scheme 2-3 | 1.40 | 0.51 | 0.89 | 4.95 |
|  | Scheme 2-4 | 1.59 | 0.41 | 0.90 | 12.51 |

Table C-60. UL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 2.61 | 1.15 | 2.45 | 6.59 |
|  | Scheme 2-2 | 1.78 | 1.05 | 1.49 | 4.93 |
|  | Scheme 2-3 | 2.62 | 1.12 | 2.32 | 7.21 |
|  | Scheme 2-4 | 2.13 | 1.08 | 1.97 | 5.29 |

Table C-61. DL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 1.32 | 0.84 | 1.19 | 1.75 |
|  | Scheme 2-2 | 16.04 | 1.16 | 1.61 | 90.42 |
|  | Scheme 2-3 | 1.40 | 0.93 | 1.21 | 1.77 |
|  | Scheme 2-4 | 1.58 | 0.80 | 1.18 | 3.40 |

Table C-62. UL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 2.61 | 2.27 | 2.68 | 3.20 |
|  | Scheme 2-2 | 1.78 | 1.41 | 1.71 | 2.72 |
|  | Scheme 2-3 | 2.62 | 2.14 | 2.55 | 3.45 |
|  | Scheme 2-4 | 2.13 | 1.88 | 2.19 | 2.36 |

Table C-63. RU for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | DL type-1 RU | UL type-1 RU | DL type-2 RU | UL type-2 RU |
| --- | --- | --- | --- | --- | --- |
| 4Kbytes for DL and 1Kbytes for UL 1000 packet/s arrival rate for DL 1400 packet/s arrival rate for UL. | Scheme 1-1 | 53.63% | 11.15% | 69.53% | 55.77% |
|  | Scheme 2-2 | 56.00% | 14.52% | 89.30% | 40.52% |
|  | Scheme 2-3 | 51.71% | 11.19% | 69.80% | 51.58% |
|  | Scheme 2-4 | 54.83% | 12.52% | 79.94% | 43.84% |

- C.2.2. Large packet size

C.2.2.1. Low DL RU and low UL RU

Table C-64. DL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 684.54 | 523.64 | 709.43 | 758.93 |
|  | Scheme 2-2 | 543.09 | 459.14 | 551.30 | 594.86 |
|  | Scheme 2-3 | 653.77 | 488.59 | 683.77 | 749.59 |
|  | Scheme 2-4 | 561.52 | 424.75 | 581.43 | 640.79 |

Table C-65. UL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 204.43 | 193.90 | 204.07 | 215.91 |
|  | Scheme 2-2 | 290.76 | 280.32 | 291.05 | 299.56 |
|  | Scheme 2-3 | 398.68 | 372.23 | 398.18 | 424.56 |
|  | Scheme 2-4 | 395.19 | 356.59 | 396.81 | 424.85 |

Table C-66. DL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 6.35 | 4.63 | 5.36 | 15.63 |
|  | Scheme 2-2 | 7.98 | 6.07 | 6.73 | 19.57 |
|  | Scheme 2-3 | 6.75 | 4.62 | 5.58 | 17.51 |
|  | Scheme 2-4 | 7.89 | 5.58 | 6.64 | 20.66 |

Table C-67. UL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 5.24 | 3.64 | 4.91 | 11.28 |
|  | Scheme 2-2 | 3.52 | 3.03 | 3.34 | 6.04 |
|  | Scheme 2-3 | 2.75 | 1.64 | 2.50 | 5.95 |
|  | Scheme 2-4 | 2.74 | 1.66 | 2.59 | 5.42 |

Table C-68. DL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 6.34 | 5.43 | 6.02 | 8.32 |
|  | Scheme 2-2 | 7.97 | 6.89 | 7.84 | 9.73 |
|  | Scheme 2-3 | 6.75 | 5.49 | 6.33 | 9.07 |
|  | Scheme 2-4 | 7.87 | 6.40 | 7.49 | 10.63 |

Table C-69. UL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 5.23 | 4.86 | 5.21 | 5.81 |
|  | Scheme 2-2 | 3.52 | 3.37 | 3.50 | 3.80 |
|  | Scheme 2-3 | 2.75 | 2.53 | 2.74 | 3.07 |
|  | Scheme 2-4 | 2.74 | 2.54 | 2.73 | 3.12 |

Table C-70. RU for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | DL type-1 RU | UL type-1 RU | DL type-2 RU | UL type-2 RU |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 1.8 packet/s arrival rate for DL 2.5 packet/s arrival rate for UL. | Scheme 1-1 | 7.37% | 2.03% | 9.55% | 10.14% |
|  | Scheme 2-2 | 7.35% | 1.99% | 11.72% | 5.56% |
|  | Scheme 2-3 | 7.69% | 2.03% | 10.84% | 8.59% |
|  | Scheme 2-4 | 7.73% | 2.03% | 11.74% | 6.46% |

C.2.2.2. Medium DL RU and medium UL RU

Table C-71. DL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 512.53 | 291.85 | 507.99 | 702.78 |
|  | Scheme 2-2 | 393.63 | 237.44 | 389.26 | 531.05 |
|  | Scheme 2-3 | 480.49 | 281.11 | 467.93 | 683.28 |
|  | Scheme 2-4 | 423.05 | 240.06 | 419.84 | 575.71 |

Table C-72. UL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 186.08 | 171.15 | 187.05 | 199.95 |
|  | Scheme 2-2 | 274.05 | 258.96 | 275.05 | 283.99 |
|  | Scheme 2-3 | 297.09 | 263.40 | 300.04 | 319.76 |
|  | Scheme 2-4 | 303.37 | 276.39 | 304.70 | 323.97 |

Table C-73. DL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 10.09 | 4.97 | 7.81 | 37.91 |
|  | Scheme 2-2 | 13.42 | 6.21 | 10.17 | 49.89 |
|  | Scheme 2-3 | 10.89 | 5.01 | 8.50 | 40.54 |
|  | Scheme 2-4 | 12.59 | 5.71 | 9.48 | 48.50 |

Table C-74. UL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 6.20 | 3.68 | 5.24 | 18.13 |
|  | Scheme 2-2 | 3.87 | 3.04 | 3.42 | 9.31 |
|  | Scheme 2-3 | 3.81 | 2.15 | 3.48 | 10.14 |
|  | Scheme 2-4 | 3.61 | 2.16 | 3.43 | 7.82 |

Table C-75. DL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 10.07 | 6.23 | 9.32 | 16.79 |
|  | Scheme 2-2 | 13.45 | 8.48 | 12.85 | 21.95 |
|  | Scheme 2-3 | 10.87 | 6.57 | 10.22 | 17.63 |
|  | Scheme 2-4 | 12.58 | 7.78 | 11.64 | 21.02 |

Table C-76. UL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 6.19 | 5.58 | 6.12 | 7.73 |
|  | Scheme 2-2 | 3.87 | 3.67 | 3.84 | 4.39 |
|  | Scheme 2-3 | 3.81 | 3.46 | 3.77 | 4.64 |
|  | Scheme 2-4 | 3.61 | 3.35 | 3.59 | 4.05 |

Table C-77. RU for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | DL type-1 RU | UL type-1 RU | DL type-2 RU | UL type-2 RU |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 4.2 packet/s arrival rate for DL 6.0 packet/s arrival rate for UL. | Scheme 1-1 | 22.18% | 5.04% | 28.75% | 25.20% |
|  | Scheme 2-2 | 22.20% | 4.91% | 35.41% | 13.71% |
|  | Scheme 2-3 | 22.77% | 4.89% | 32.49% | 19.67% |
|  | Scheme 2-4 | 23.22% | 4.89% | 35.38% | 15.52% |

C.2.2.3. High DL RU and high UL RU

Table C-78. DL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 346.01 | 132.71 | 322.75 | 623.75 |
|  | Scheme 2-2 | 207.73 | 20.18 | 182.59 | 471.13 |
|  | Scheme 2-3 | 292.44 | 91.36 | 265.28 | 595.99 |
|  | Scheme 2-4 | 241.53 | 49.25 | 218.48 | 516.07 |

Table C-79. UL Average-UPT for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (Mbps) | 5%(Mbps) | 50%(Mbps) | 95%(Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 152.01 | 121.33 | 153.22 | 174.06 |
|  | Scheme 2-2 | 222.37 | 156.00 | 230.10 | 257.74 |
|  | Scheme 2-3 | 191.07 | 153.16 | 190.95 | 225.04 |
|  | Scheme 2-4 | 209.97 | 186.30 | 207.73 | 239.14 |

Table C-80. DL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 23.65 | 5.23 | 13.06 | 156.45 |
|  | Scheme 2-2 | 150.97 | 6.75 | 26.25 | 2564.37 |
|  | Scheme 2-3 | 34.36 | 5.51 | 16.42 | 261.44 |
|  | Scheme 2-4 | 74.60 | 6.25 | 21.18 | 941.26 |

Table C-81. UL latency (per packet: option 1) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 9.72 | 3.76 | 6.21 | 46.04 |
|  | Scheme 2-2 | 5.48 | 3.08 | 4.12 | 17.94 |
|  | Scheme 2-3 | 7.43 | 2.97 | 5.23 | 33.94 |
|  | Scheme 2-4 | 5.57 | 3.15 | 4.73 | 16.73 |

Table C-82. DL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 23.58 | 8.09 | 19.22 | 50.55 |
|  | Scheme 2-2 | 183.54 | 11.15 | 46.54 | 722.06 |
|  | Scheme 2-3 | 34.27 | 8.78 | 23.94 | 90.56 |
|  | Scheme 2-4 | 77.85 | 9.83 | 33.58 | 180.43 |

Table C-83. UL latency (per user: option 2) for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | Mean (ms) | 5%(ms) | 50%(ms) | 95%(ms) |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 9.69 | 7.30 | 9.29 | 15.72 |
|  | Scheme 2-2 | 5.48 | 4.27 | 5.17 | 8.70 |
|  | Scheme 2-3 | 7.42 | 5.56 | 7.07 | 13.23 |
|  | Scheme 2-4 | 5.56 | 4.79 | 5.53 | 6.89 |

Table C-84. RU for FR1 in Indoor Hotspot scenario

| Configuration | Scheme | DL type-1 RU | UL type-1 RU | DL type-2 RU | UL type-2 RU |
| --- | --- | --- | --- | --- | --- |
| 0.5Mbytes for DL and 0.125Mbytes for UL 8.0 packet/s arrival rate for DL 13.0 packet/s arrival rate for UL. | Scheme 1-1 | 52.22% | 11.41% | 67.69% | 57.06% |
|  | Scheme 2-2 | 56.30% | 10.78% | 89.77% | 30.08% |
|  | Scheme 2-3 | 56.38% | 11.11% | 78.30% | 48.44% |
|  | Scheme 2-4 | 58.61% | 10.94% | 86.47% | 37.25% |