3GPP TSG RAN WG1 #112bis-e		R1-2302485

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:	Potential enhancements on dynamic/flexible TDD

Agenda Item:	9.3.3

Document for:	Discussion and Decision

- Introduction

In RAN1#112 meeting, the following agreements were achieved [1].

Agreement

For the study of gNB-to-gNB co-channel interference measurement, it is assumed that both CD-SSB and NCD-SSB can be used for gNB-to-gNB CLI measurement.

Agreement

Study the benefit of knowledge among gNBs of configurations such as

- SBFD time/frequency configuration

Agreement

For spatial domain enhancement of gNB-to-gNB co-channel CLI handling, DL Tx beam information of the gNB can be exchanged between gNBs. Reference signal resource ID (e.g., NZP-CSI-RS resource ID, SSB index) can be used as beam information exchange between gNBs.

Agreement

For spatial domain enhancement of gNB-to-gNB CLI handling, study the benefit and the procedure of the information exchange of at least the preferred/non-preferred DL beams of the aggressor gNBs, based on the beam information exchanged between gNBs.

Agreement

For L1/L2 based UE-to-UE CLI measurement, SRS-RSRP and CLI-RSSI are to be further studied as baseline metrics.

Agreement

For the study of L1/L2 based UE-to-UE co-channel CLI measurement, measurement resource for CLI-RSSI measurement as defined in Rel-16 and SRS resource for SRS-RSRP measurement as defined in Rel-16 can be considered. Enhancement of measurement resource can be studied.

Agreement

For L1/L2 based UE-to-UE co-channel CLI measurement and reporting mechanism, study the following measurement and report framework.

- Use existing CSI framework as the baseline.

- Others are not precluded.

In this contribution, we share our views on potential enhancements for dynamic/flexible TDD.

- Discussion

For dynamic/flexible TDD scheme, gNB can dynamically allocate UL and DL resource in the time-domain to match traffic condition, which can significantly improve resource utilization efficiency and reduce the packet latency. In NR Rel-16, flexible resource adaptation for unpaired spectrum based on Cross Link Interference (CLI) handling has been studied and evaluated, mechanisms such as cross-link interference measurements and reporting at UE side and exchange of intended DL/UL configuration among gNBs at network side are specified. In the following, we provide our views on potential enhancements for better use of dynamic/flexible TDD.

  - Interference for dynamic TDD

In existing specification, dynamic/flexible TDD has already been supported, i.e., slot format can be dynamically indicated by gNB via SFI. However, this feature may not be well deployed in practice. One of the reasons is lack of efficient solutions to address inter-gNB (and inter-operator) cross-link interference compared to TDD systems with semi-static and same UL/DL configuration.

For example, at gNB side, gNB A suffers co/adjacent-channel CLI from gNB B in slot m and from gNB C in slot n, as shown in red dashed line in Figure 1. For different time, the dominant CLI may come from different gNBs.

Similar situation may also occur at UE side. Specifically, a UE receiving DL signal may suffer co/adjacent-channel CLI from UEs of surrounding cells with different TDD configurations. It can be observed that, dominant CLI at gNB and UE side may come from surrounding gNB(s) or UE(s) with different TDD configurations and varies from time to time.

Figure 1 CLI for dynamic TDD configuration

Observation 1: Dominant CLI at gNB and UE side may come from surrounding gNB(s) or UE(s) with different TDD configurations and varies from time to time.

For Rel-18 SubBand Full Duplex (SBFD) operation, CLI also exists including both intra-subband CLI and inter-subband CLI. When the subband granularity in frequency domain covers the full bandwidth, it becomes dynamic TDD. So, dynamic TDD can be viewed as a subset of SBFD. From CLI handling perspective, the schemes introduced for dynamic TDD should also be reused for SBFD as much as possible. In the following sections, we discuss potential enhancements for UE-to-UE and gNB-to-gNB CLI handling.

Proposal 1: Considering CLI issues existing in both dynamic TDD and SBFD operations, unified solutions for mitigating CLI should be strived for both SBFD and dynamic TDD.

  - UE-to-UE CLI handling

2.2.1 CLI measurement and reporting

For UE-to-UE CLI handling, SRS-RSRP and CLI-RSSI measurement and reporting at UE side were specified as main solutions in Rel-16. However, for SRS-RSRP measurements ad reporting, the mechanism may not work well in practice since exchange of SRS configurations among gNBs is not discussed or specified in Rel-16. For efficient/accurate UE-to-UE CLI measurement and reporting, gNBs should exchange their cell’s or UE’s SRS configurations over the Xn/F1 interface. In addition, gNBs can also exchange the victim UE’s CLI measurement results and associated CLI-RS resources in case the victim UE suffers stronger CLI so that the surrounding gNB(s) can avoid scheduling the aggressor UE in the slot where strong CLI is expected, or schedule the aggressor UE with low transmission power.

Proposal 2: For efficient UE-to-UE CLI measurement and reporting as well as coordinated scheduling, the following enhancements for Rel-16 CLI should be considered.

- gNBs should exchange their cell or UE’s SRS configurations over the Xn/F1 interface.

- gNBs should exchange the victim UE’s CLI measurement results and associated CLI-RS resources in case the victim UE suffers stronger CLI.

To reduce the reporting latency of CLI measurement to timely update/reflect the interference status, it has been agreed to study L1/L2 based UE-to-UE CLI measurement and reporting. For L1/L2 based UE-to-UE CLI measurement, it has been agreed that SRS-RSRP and CLI-RSSI are to be further studied as baseline metrics, and measurement resource for CLI-RSSI measurement as defined in Rel-16 and SRS resource for SRS-RSRP measurement as defined in Rel-16 can be considered, and existing CSI framework is used as the baseline for L1/L2 based UE-to-UE co-channel CLI measurement and reporting. With this baseline, it is easy to configure beam information for each CLI measurement resources which is important especially for FR2. For example, for an aggressor UE, spatialRelationInfo can be configured per CLI SRS resource. For a victim UE, TCI-state can be configured per CLI measurement resource.

Proposal 3: For UE-to-UE CLI measurement and reporting, the following enhancements can be considered.

- The beam information can be configured per CLI measurement resource.

2.3.2 Transmission and reception timing adjustment at UE side

As shown in Figure 1, UL transmission of UE 1 from cell 2 may bring strong interference to DL reception of UE 3 from cell 1 due to close distance between these two UEs. Accurate channel and interference estimation are beneficial to suppress the interference from an aggressor UE.

In Rel-16, for CLI measurement, when a UE measures SRS-RSRP or CLI-RSSI, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation. Furthermore, if timing is aligned between UL transmission of the aggressor UE and DL reception of the victim UE, interference covariance matrix information can be estimated based on CLI RS at victim UE side to improve DL reception performance.

However, UL transmission of the aggressor UE is expected to apply timing advance (TA) which targets to its own serving cell. The propagation delay from the aggressor UE to its serving cell may be very different from that of the victim UE in a neighbouring cell. Hence, the CLI may not be fully confined within the receiving window of the victim UE which follows DL timing of its serving cell. Especially, when the victim UE suffers CLI from multiple aggressor UEs with different TAs, CLI may exceed the CP duration of the victim UE which would bring severe interference on DL reception.

Observation 2: When the victim UE suffers CLI from multiple aggressor UEs with different TAs, CLI may exceed the CP duration of the victim UE which would bring severe interference on DL reception.

  - gNB-to-gNB CLI handling

2.3.1 CLI measurement and reporting

To further mitigate gNB-to-gNB CLI, some coordination among gNBs can be considered. For example, gNBs can exchange assistance information including measurement reports of RSRP/RSSI, scheduling information and so on. Specifically, for measurement reports of RSRP/RSSI, the best or worst RSRP/RSSI based on certain CLI measurement resources can be included in the measurement reports.

For scheduling information, it can include time/frequency resources where the victim gNB is intended to perform UL scheduling or the aggressor gNB is intended to perform DL scheduling. With the assistance information provided by surrounding gNBs, a gNB can perform coordinated scheduling or resource configuration, thus mitigate the CLI impact.

Proposal 4: Assistance information exchange among gNBs can be considered for gNB-to-gNB CLI handling, including measurement reports of RSRP/RSSI, scheduling information.

2.3.2 Spatial domain coordination

For spatial domain coordination, the exchange of beam related information among gNB(s) is beneficial for CLI management among gNBs.

To identify the preferred/non-preferred DL beams of aggressor gNB, aggressor gNB can indicate SSB index or CSI-RS resource IDs associated with the intended transmitted DL beam to victim gNB. Victim gNB can measure CLI per beam resource or panel, similar to beam sweeping between aggressor gNB and victim gNB. According to the beam sweeping procedure, victim gNB can measure the highest and lowest CLI and identify the preferred/non-preferred DL beams of aggressor gNB based on measurement results. Then, the information can be fed back to aggressor gNB so that aggressor gNB can avoid scheduling the beam with the stronger CLI to victim gNB or schedule DL transmission with the less power.

The beam sweeping procedure at victim gNB side to identify the preferred/non-preferred DL beams to aggressor gNB can base on implementation.

Furthermore, Aggressor gNBs can also indicate the time/frequency resources association with the beam for more accurate coordination. Victim gNB can apply the information for subsequent scheduling and resource management. The related scheduling resources associated with the beam information can also be exchanged among gNBs.

Proposal 5: For spatial domain coordination, beam sweeping procedure to identify preferred/non-preferred DL beams of aggressor gNB can bese on implementation.

2.3.3 Transmission and reception timing adjustment at gNB side

As discussed in section 2.1, cross link interference from the neighboring gNBs with the opposite transmission direction becomes a main challenge, which may decrease the performance of victim cell even lead to unfeasible network deployment. Interference mitigation mechanism is required in such case to suppress interference and improve the resource utilization efficiency.

In order to suppress the interference at victim gNB, accurate channel estimation for the wanted signal and interference estimation from aggressor gNB are required. If RS for channel estimation from victim gNB and RS for interference estimation from aggressor gNB are aligned, accurate channel and interference estimation can be enabled at victim gNB and MMSE-IRC receiver can be adopted to suppress the CLI from aggressor gNB. CLI can be effectively suppressed by employing interference covariance matrix information, which would bring the significant performance improvement, such as the previous investigation [2].

At present, UL transmission timing of a UE is adjusted based on TA to ensure the UL signal from the UE arrives at the time aligned with DL timing of serving gNB. For dynamic/flexible TDD configuration as described in Figure , slot n and slot n+1 may suffer CLI from DL signal of neighboring gNB. For victim gNB, RS for channel estimation may be interfered by DL transmission of aggressor gNB, which incur the inaccurate channel estimation. Victim gNB cannot identify the dominant interference from aggressor gNBs and would deem all interference as the ‘other cell’ interference. Therefore, the performance loss would be observed.

Figure 3 aggressor gNB DL transmission leads to interference for UL reception of victim gNB

A candidate enhancement is depicted in Figure , victim gNB can adjust transmission timing of the serving UEs to align with arrival timing of DL transmission from aggressor gNB, e.g., a timing offset for the UL transmissions of the UEs in victim gNB can be adopted. In this case, victim gNB can accurately estimate interference covariance matrix from aggressor gNB to suppress CLI. For example, victim gNB can mute some REs on PUSCH to measure the interference channel from aggressor gNB. Especially, if aggressor gNB transmits a specific RS on these REs, e.g., CSI-RS or DMRS, the measurement on interference covariance matrix can base on these RSs and the higher channel estimation performance can be achieved as shown in Figure. On the other hand, aggressor gNB can mute some DL resources corresponding to DMRS RE position of PUSCH scheduled by victim gNB to improve UL channel estimation accuracy of victim gNB.

Figure 4 UL reception timing of victim gNB aligned with DL transmission timing of aggressor gNB

Victim gNB's UL reception timing can be adjusted to align with the arrival timing for DL signal from dominant aggressor gNB, for example, a negative TA can be configured for UEs served by victim gNB. The timing adjustment is slot specific, which is only applied for slot with CLI. For slot without CLI, normal TA is used.

Figure 5 DL slot of aggressor gNB is aligned with UL slot of victim gNB

Proposal 6: Transmission and reception timing adjustment can be supported in Rel-18 dynamic/flexible TDD to accurately estimate interference channel and effectively suppress CLI from aggressor gNB.

Proposal 7: For transmission and reception timing adjustment, victim gNB should adjust transmission timing of the served UEs to align with DL transmission signal arrival of aggressor gNB. A negative TA can be configured for UEs served by victim gNB. The timing adjustment is slot specific.

  - Potential enhancement for UL power control

To counter gNB-to-gNB or UE-to-UE CLI, power control-based solution can be considered.

As shown in Figure 1, in slot m UL reception of gNB A will suffer from the interference from DL transmission of gNB B. To mitigate the impact of CLI and guarantee UL reception performance of gNB A, one direction is to enhance the UL power control. For example, boosting power for UL transmission of UE1 and UE2 can be considered when gNB A is suffering strong CLI in UL reception. This can be realized in a similar way as that specified in Rel-16 URLLC WI for services with different priorities. That is, different power control parameters e.g. P0 and different power loops can be used depending on the flexible TDD resource allocation or the existence/strength of the CLI. However, there is no solution available for CG-PUSCH power boosting in Rel-16, which may be studied further in Rel-18. Slot-specific, i.e., slot with/without CLI, power control parameter can be considered for CG-PUSCH power boosting in Rel-18.

For SBFD, UL reception in victim gNB A will suffer from the inter-subband CLI interference from DL transmission of gNB B and self-interference from DL of gNB A. Currently, UL power control for UE does not take into account the self-interference nor inter-subband CLI interference. Enhanced UL power control to mitigate the impact of CLI can be considered. For SBFD UL transmission, the power control parameters or TPC commands can be indicated by DCI or determined depending on the SBFD resource allocation or the existence/strength of the CLI.

Furthermore, the above enhanced UL power control can also be used to decrease UE-to-UE CLI. Taking Figure 1 as an example, UE 3 of cell 1 suffers the CLI of UE 1 from cell 2. In this case, gNB A can reduce UL transmitting power of UE1 to alleviate the interference to UE 3 of adjacent cell. As mentioned above, gNB B can inform its serving UE being interfered by UE from gNB A. Therefore, coordination on CLI measurement and reporting among gNBs may be needed in this case.

Proposal 8: For dynamic TDD/SBFD CLI handling, enhanced UL power control can be considered, e.g., different power control parameters can be used depending on resource allocation or the existence/strength of the CLI.

- Conclusion

In this contribution, we make discussions on potential enhancement on dynamic/flexible TDD and have the following proposals.

Observation 1: Dominant CLI at gNB and UE side may come from surrounding gNB(s) or UE(s) with different TDD configurations and varies from time to time.

Observation 2: When the victim UE suffers CLI from multiple aggressor UEs with different TAs, CLI may exceed the CP duration of the victim UE which would bring severe interference on DL reception.

Proposal 1: Considering CLI issues existing in both dynamic TDD and SBFD operations, unified solutions for mitigating CLI should be strived for both SBFD and dynamic TDD.

Proposal 2: For efficient UE-to-UE CLI measurement and reporting as well as coordinated scheduling, the following enhancements for Rel-16 CLI should be considered.

- gNBs should exchange their cell or UE’s SRS configurations over the Xn/F1 interface.

- gNBs should exchange the victim UE’s CLI measurement results and associated CLI-RS resources in case the victim UE suffers stronger CLI.

Proposal 3: For UE-to-UE CLI measurement and reporting, the following enhancements can be considered.

- The beam information can be configured per CLI measurement resource.

Proposal 4: Assistance information exchange among gNBs can be considered for gNB-to-gNB CLI handling, including measurement reports of RSRP/RSSI, scheduling information.

Proposal 5: For spatial domain coordination, beam sweeping procedure to identify preferred/non-preferred DL beams of aggressor gNB can bese on implementation.

Proposal 6: Transmission and reception timing adjustment can be supported in Rel-18 dynamic/flexible TDD to accurately estimate interference channel and effectively suppress CLI from aggressor gNB.

Proposal 7: For transmission and reception timing adjustment, victim gNB should adjust transmission timing of the served UEs to align with DL transmission signal arrival of aggressor gNB. A negative TA can be configured for UEs served by victim gNB. The timing adjustment is slot specific.

Proposal 8: For dynamic TDD/SBFD CLI handling, enhanced UL power control can be considered, e.g., different power control parameters can be used depending on resource allocation or the existence/strength of the CLI.

References

- Chair‘s notes RAN1#112 meeting.

- 3GPP TR 36.866,Study on Network-Assisted Interference Cancellation and Suppression (NAIC) for LTE