3GPP TSG RAN WG1 #112bis-e		R1- 2302490

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:	Discussion on SL positioning reference signal

Agenda Item:	9.5.1.1

Document for:	Discussion and Decision

Introduction

In RAN #98e e-meeting, specify SL PRS and specify procedures for transmit power control for SL PRS transmission for support of sidelink positioning have been captured in the WID [1]. In this contribution, we present our views on the potential design and power control procedure for SL PRS.

- Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1].

SL PRS design

It has been agreed that SL PRS should be specified for supporting SL positioning. In this section, we present our views on SL PRS design for SL positioning.

SL-PRS sequence initialization

Agreement SL PRS sequence is generated based on Gold sequence: rn=121-2⋅c2n+j121-2⋅c2n+1, where c(i) is a pseudo-random sequence as defined in Clause 5.2.1 of TS 38.211. Agreement - For SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID,seqSL-PRS. - The pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS Agreement For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option: - Option 1: nID,seqSL-PRS is a higher layer configured parameter - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined. Agreement Range of the parameter nID,seqSL-PRS is: nID,seqSL-PRS∈0,1,…,4095

It has been agreed that the pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS. The remaining issue for SL-PRS sequence is to define the parameter nID,seqPRS. In DL-PRS, the nID,seqPRS is configured by a higher layer and can be used to identify the PRS sequence (e.g., to identify the  PRS from different TRPs). In the NR SL, the NID in SL RS sequence generation is 10 bits or 16 bits of CRC of corresponding PSCCH. We prefer to take the following options into consideration to define the parameter nID,seqPRS.

- Option 1: nID,seqSL-PRS is a higher layer configured parameter

- Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission

The simplest way was to reuse the  NID of SL RS for SL PRS. However, there are some privacy concerns about directly reusing that for SL PRS, for example

- If the current SL parameter NID is used to SL PRS for positioning, any UE can get  NID based on the CRC of the PSCCH, and any UE could measure the positioning signal. It would expose some information about the UE transmitting the SL PRS.

- For RTT-based positioning, some relationships between SL-PRS1 from UE1 and SL-PRS2 from UE2 may be defined. Therefore, UE1 would know that SL-PRS2 is a feedback signal.

Therefore, in our view, the nID,seqPRS of SL-PRS can be associated with some UE information configured by higher layer. In this case, if UEs cannot obtain that information in advance, those UEs would not measure the corresponding SL-PRS successfully. And, if the higher layer configured parameter is used for nID,seqPRS, the other issue is whether the  nID,seqPRS is the same or not for the different positioning sessions. For us, to avoid a sequence being traced, different nID,seqPRS for different positioning sessions is preferred.

- The nID,seqPRS of SL PRS is a higher layer configured parameter (i.e., Option 1 should be supported).

Time domain and frequency domain pattern for SL-PRS

Agreement For RE-offset sequence for SL PRS, the RE-offset sequences specified for DL PRS are considered as a starting point. - FFS: Exact RE-offset sequences Agreement For SL PRS in shared or dedicated resource pools, - at least comb sizes (N) 2, 4 are supported. - Comb size 6 is supported at least in dedicated resource pool - FFS: comb size 6 in shared resource pool - Comb size 1 is supported at least in shared resource pool - FFS: comb size 1 in dedicated resource pool - comb sizes (N) > 12 are not supported. - FFS: support of comb sizes (N) of 8, 12. Agreement For SL PRS in shared and dedicated resource pools, - SL PRS patterns with full staggering are supported. - FFS: whether (M,N)=(6,6) is supported - SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs: - (M, 2) with M = {1} - (M, 4) with M = {2} - FFS: constraints on maximum effective comb size - FFS: support of partial staggering for other comb sizes - FFS: Support of SL PRS patterns with M > N at least with full staggering.

It has supported comb sizes N = 2, 4, 6 for SL-PRS in the dedicated resource pool, and comb sizes N= 1, 2, 4 for SL-PRS in the shared resource pool. The remaining issue is whether to support comb size 6 in the shared resource pool and whether to support comb size 1 in the dedicated resource pool.

In the shared resource pool, considering the NR SL structure, 2-3 symbols can be used for PSCCH, 2 symbols for AGC and GP, and 1-4 symbols can be used for PSSCH DMRS up to higher layer configuration and PSCCH indication. If SL-PRS is introduced without comb-based and TDM-based multiplexing, no additional AGC is required for SL-PRS in the slot. And there would be 5-9 symbols remaining in the slot that could be used for SL-PRS and PSSCH. However, if only consecutive symbols for a SL-PRS are supported in the shared resource pool, at most 4 consecutive symbols for SL-PRS can be used considering the DMRS pattern defined in NR SL. Therefore, We think comb size 6 can be achieved in the shared resource pool considering partial staggering. Besides, whether configured comb size 6 or not can be up to the higher layer configuration in the resource pool.

In our opinion, if fully staggering is configured, the value of N = 12 is not supported. In this case, the M is equal to or larger than 12, there is no additional symbol(s) for PSCCH which is used for SL-PRS scheduling. If partial staggering is configured, the value of N =12 can be supported that the value of M can be smaller than 12 which means that symbol(s) may be available for PSCCH transmission.

Therefore, we propose

- For the comb size of SL-PRS in the shared or dedicated resource pool

  - Support comb size 6 in the shared resource pool with partial staggering;

  - Comb size 12 can be supported with partial staggering.

The RE-offset sequence for DL PRS can be reused for comb size N= 2, 4, 6. And for comb size 1, it means that no comb structure is used for SL-PRS. The following RE patterns for SL-PRS can be supported.

- Comb-2: Symbols {0, 1} have relative RE offsets {0, 1}

- Comb-4: Symbols {0, 1, 2, 3} have relative RE offsets {0, 2, 1, 3}

- Comb-6: Symbols {0, 1, 2, 3, 4, 5} have relative RE offsets {0, 3, 1, 4, 2, 5}

Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs

In the previous meeting, the comb-based multiplexing of SL-PRS has been supported in the dedicated resource and supports a single (M, N) value in a resource pool. Whether to support multiple (M, N) values in the dedicated resource pool should be further studied. To avoid resource fragmentation or complexity resource multiplexing and sensing, only a comb size value and an SL-PRS symbol value is allowed to be configured in a resource pool. In the same slots and same PRBs, different SL-PRS only have different RE offsets or different symbol offsets.

- Only a comb size and an SL-PRS symbol value is allowed to be configured per resource pool to avoid complexity resource multiplexing and sensing.

It has discussed whether comb-based multiplexing can be supported in the shared resource pool, and needs further study. In our view, the comb-based multiplexing in the shared resource pool would increase the specification complexity to guarantee backward compatibility and some details can be found in our companion contribution [2]. If the requirement of SL-PRS is critical, the UE can be (pre-)configured to perform SL-PRS transmission in the dedicated resource pool. Therefore, we prefer not to support comb-based multiplexing in the shared resource pool.

Observation 1:

- The backward compatibility cannot be guaranteed if comb-based multiplexing for SL-PRS is supported in the shared resource pool.

- Not support comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools.

Agreement TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability

In the shared resource pool, if the TDM-based multiplexing of SL PRS is supported, the Rel-18 positioning UE needs to perform sensing and resource selection in granularity of symbols. It would make the resource selection more complex. Therefore, we prefer the TDM-based multiplexing of SL PRS from different UEs in a slot that is not supported.

- Not support TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pool.

For the TDM-based multiplexing in a slot for dedicated resource pool, the candidate can be supported in the following two options.

- Option 1. Candidate resources of PSCCH used to schedule SL-PRS from different UEs are located in starting symbols of the slot. (As illustrated in the left figure of Figure 2)

- Option 2. Candidate resources PSCCH used to schedule SL-PRS are located in front of the associated SL-PRS. It means that mini-slot based SL-PRS are supported, and transmission and reception switching is supported in the slot. (As illustrated in the right figure of Figure 2)

For option 2, the transmission and reception of PSCCH associated with SL-PRS switching in the slot is beneficial for decreasing the probability of half-duplex of SL-PRS. However, it may need UE to support monitoring additional PSCCH candidates in the middle of the slot which is not supported in the NR SL. And it may be required additional UE capability.

- The following options for candidate PSCCH can be taken into consideration in TDM-based multiplexing of SL-PRS in the dedicated resource pool

  - Option 1. Candidate resource of PSCCH is located in the starting symbols of a slot.

  - Option 2. Candidate resources of PSCCH are located in the front of the associated SL-PRS, and can be in the middle symbol of a slot.

AGC and Rx-Tx Turnaround

Agreement For a potential new SL PRS, study further the following - Number of symbol(s) for AGC and/or Rx-Tx turnaround time - Conditions under which AGC training and/or Rx-Tx turnaround time are needed Agreement TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability

It has been agreed that AGC and/or Rx-Tx turnaround time (i.e., GP symbol) should be studied. For the AGC symbol, the receiver collects the energy around and adjusts the gain of the power amplifier accordingly to make the power amplifier operate in a linear operating range, which keeps the signal from being distorted. For Rx-Tx turnaround time, the UE can switch between transmission and reception during the symbol. An SL UE is not expected to receive any signal during the Rx-Tx turnaround time symbol.

- Slot based SL-PRS

In Rel-16 SL, one symbol AGC is assumed in FR1, and it can be assumed for SL-PRS. Considering Rx-Tx turnaround time, according to RAN4 evaluation for Rel-16 SL, 13 us is required for FR1, and 7 us is required for FR2. Therefore, one symbol can be assumed for Rx-Tx turnaround time for SL-PRS.

- Mini-slot based SL-PRS

In the previous meeting, the TDM-based multiplexing of SL PRS in a slot has been supported. Therefore, multiple AGC symbols or multiple GP symbols are required. The following option can be taken into consideration.

Option 1. Multiple AGC symbols and one GP symbol are required in a slot;

Option 2. Multiple AGC symbols and multiple GP symbols are required in a slot.

For option 1, the UE transmitting SL PRS only can turn to the reception in the next slot, which may lead to more half-duplex problems than option 2. However, the overhead of GP symbols is less than option 2. It may increase the performance of throughput. For option 2, mini-slot based SL-PRS is supported and UE can operate TX-RX or RX-TX turnaround in a slot which is beneficial to low latency scheduling. In our opinion, option 2 is preferred. Figure 4 is an example of multiple SL-PRS multiplexed in a slot for option 1 and option 2.

| Option 1. Multiple AGC+ one GP | Option 2. Multiple AGC+ Multiple GP |
| --- | --- |

Figure 2 An example of mini-slot based SL-PRS

- With regards to AGC training

  - One symbol preceding a SL-PRS resource can be used

- With regards to Rx/Tx turnaround

  - At least, one GP symbol is required at the end of slot

  - FFS one GP symbol after an SL-PRS resource

Bandwidth of SL PRS

Agreement At least for a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives: - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool - Note: Companies are encouraged to provide their analysis and views on the above alternatives - FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported)

In NR SL, only one SL BWP is supported on a carrier. And resource pool is configured in the BWP. In our opinion, the SL PRS is transmitted in the BWP and should be restricted in the bandwidth of a resource pool.

Compared to Alt.1 and Alt.2, there is no doubt that Alt. 1 has better flexibility and allows different UEs and services to use different bandwidths. However, opponents argue it would result in resource fragmentation, so they propose configuring multiple dedicated resource pools to satisfy different positioning requirements. Considering the resource pool configuration is a static or area-specific configuration that cannot be adjusted by channel state, more resources would be wasted by inappropriate resource pool allocation (similar to the following figure). If the resource pool is configured as a broadband resource, the dynamic resource indication can be achieved if the bandwidth of SL-PRS can be smaller than that of the resource pool.

Figure 3 An example of multiple bandwidth resource pool configuration

In this case, the motivation for restricting UE transmission with different bandwidths in different resource pools instead of letting UE sense resources based on channel conditions in a resource pool is unclear. So, we still prefer option 1.

In the shared resource pool, the bandwidth of SL-PRS should be the same as the indicated bandwidth of SL PSSCH which is also restricted in the bandwidth of the resource pool to guarantee backward compatibility for the legacy SL communication transmission.

- For a dedicated resource pool, the bandwidth of SL-PRS can be the same or smaller than that of the resource pool

- For a shared resource pool, the bandwidth of SL-PRS should be the same as the indicated bandwidth of SL PSSCH.

Configuration and reservation for SL-PRS

Agreement With regards to SL-PRS, study the following options: - Option 1: High-layer-only signaling involvement in the SL-PRS configuration - No Lower layer involvement, e.g., SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS. - Based on the study, this option may correspond to - A SL-PRS configuration that is a single-shot or multiple shots - A high-layer configuration that may be received from an LMF, a gNB, or a UE - Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS. - Option 3: Only lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed. Agreement - With regards to the configuration/activation/deactivation/triggering of SL-PRS, Option 3 from the previous corresponding RAN1 #109 agreement will not be considered further. - With regards to reservation of SL-PRS, it can be considered based on the Option 1 or Option 2 from the previous corresponding RAN1 #109 agreement. Agreement With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration. - FFS: Whether SCI is single stage SCI or two stage SCI - FFS: SL-MAC-CE or other higher-layer signaling reservation/indication

In RAN1#110 meeting, it has been agreed that Option 1 or Option 2 can be used for the configuration/activation/deactivation/triggering of SL-PRS.

For the configuration of SL-PRS, option 2 is preferred. we think the high-layer signaling is involved to configure some SL-PRS parameters (such as comb N, number of symbols M, configuration of the resource pool and configuration of resource, etc.). The candidate high-layer signaling can be

Opt a: RRC or SLPP between network and UE

Opt b: PC5-SLPP between UEs

Opt c: pre-configured RRC

In our opinion, the Rel-16/Rel-17 resource pool (i.e., shared resource pool in the positioning)is (pre-)configured by RRC. Therefore. in scheme 1 resource allocation for SL-PRS in the shared resource pool, the configuration can be only (pre-)configured by  RRC since it has been agreed that scheme 1 resource allocation is scheduled by gNB and not LMF. Besides, in SL positioning, it prefers that the configuration of shared and that of dedicated resource pool (e.g., comb N, number of symbols M, configuration of the resource pool and configuration of resource, etc) can be the same to reduce the complexity of specification.   It means that the configuration of shared and dedicated resource pool and should be (pre-)configured by RRC. Apart from the resource pool level configuration, some UE-specific parameters (e.g., sequence ID, priority) if supported can be configured by SLPP.

In addition, we also think the low layer is involved in some SL-PRS parameters(e.g., PRS timing/ frequency resource, period indication, priority, and RE offset.).

Furthermore, according to the previous meeting, the SCI is supported to be used for SL-PRS resource reservation. Considering the content of SCI, the following information should be carried in the SCI at least: SL PRS timing/ frequency resource, destination ID, source ID,  period indication, priority, RE offset, and symbol offset.

- For the configuration of SL-PRS, option 2 is preferred

  - Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration.

    - Resource pool configuration parameters (e.g., comb N, number of symbols M, configuration of the resource pool and configuration of resource, etc) should be (pre-)configured by RRC

    - UE-specific parameters (e.g., sequence ID, priority) if supported can be configured by SLPP

- For the SCI used for the reservation/indication of SL-PRS, the following information should be carried in the SCI.

  - SL PRS timing/frequency resource, destination ID, source ID,  period indication, priority, and RE offset, and symbol offset.

Power control

Power control of SL-PRS for positioning

Agreement The OLPC framework defined for PSSCH/PSCCH is considered as a starting point for OLPC for SL PRS.

According to the previous meeting, the OLPC framework defined for PSSCH/PSCCH is considered as a starting point for SL PRS. In NR sidelink, the open loop power control of SL is based on DL pathloss and/or SL pathloss (only for unicast). The DL pathloss is taken into account to avoid serious interference to the UL transmission performance. And if distance between TX UE and RX UE is small, the power control based on SL pathloss can be considered. It would decrease the power of TX UE which satisfies the performance of the SL transmission and can reduce the interference to other UEs. Therefore, SL pathloss and DL pathloss can be considered in SL-PRS power control. E.g., as in Rel-16 SL, the minimum pathloss of DL and SL can be used.

Figure 4 Open-loop power control for SL PRS

- The minimum of SL pathloss and the DL pathloss should be applied in the power control of the SL-PRS.

Power control of PSCCH for positioning

As discussed in the slot structure in our companion contribution [2], the PSCCH and the associated scheduled SL PRS should be TDMed in a slot. To avoid the switching gap between the PSCCH and the scheduled SL PRS, the power of the PSCCH should be equal to the power of the SL PRS.

- The power of PSCCH should be equal to the power of the SL PRS when PSCCH and SL PRS are TDMed in a slot.

Conclusion

In this contribution, we discuss potential positioning enhancements with the following observations and proposals.

Observation 1:

- The backward compatibility cannot be guaranteed if comb-based multiplexing for SL-PRS is supported in the shared resource pool.

- The nID,seqPRS of SL PRS is a higher layer configured parameter (i.e., Option 1 should be supported).

- For the comb size of SL-PRS in the shared or dedicated resource pool

  - Support comb size 6 in the shared resource pool with partial staggering;

  - Comb size 12 can be supported with partial staggering.

- Only a comb size and an SL-PRS symbol value is allowed to be configured per resource pool to avoid complexity resource multiplexing and sensing.

- Not support comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools.

- Not support TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pool.

- The following options for candidate PSCCH can be taken into consideration in TDM-based multiplexing of SL-PRS in the dedicated resource pool

  - Option 1. Candidate resource of PSCCH is located in the starting symbols of a slot.

  - Option 2. Candidate resources of PSCCH are located in the front of the associated SL-PRS, and can be in the middle symbol of a slot.

- With regards to AGC training

  - One symbol preceding a SL-PRS resource can be used

- With regards to Rx/Tx turnaround

  - At least, one GP symbol is required at the end of slot

  - FFS one GP symbol after an SL-PRS resource

- For a dedicated resource pool, the bandwidth of SL-PRS can be the same or smaller than that of the resource pool

- For a shared resource pool, the bandwidth of SL-PRS should be the same as the indicated bandwidth of SL PSSCH.

- For the configuration of SL-PRS, option 2 is preferred

  - Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration.

    - Resource pool configuration parameters (e.g., comb N, number of symbols M, configuration of the resource pool and configuration of resource, etc) should be (pre-)configured by RRC

    - UE-specific parameters (e.g., sequence ID, priority) if supported can be configured by SLPP

- For the SCI used for the reservation/indication of SL-PRS, the following information should be carried in the SCI.

  - SL PRS timing/frequency resource, destination ID, source ID,  period indication, priority, and RE offset, and symbol offset.

- The minimum of SL pathloss and the DL pathloss should be applied in the power control of the SL-PRS.

- The power of PSCCH should be equal to the power of the SL PRS when PSCCH and SL PRS are TDMed in a slot.

- References

- RP-223549 New WID on Expanded and Improved NR Positioning, Dec. 12 – 16, 2022

- R1-2302492  Discussion on resource allocation for SL positioning reference signal.