3GPP TSG RAN WG1 #112bis-e			                   R1-2302491

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:	Discussion on measurements and reporting for SL positioning

Agenda Item:	9.5.1.2

Document for:	Discussion and Decision

Introduction

In RAN #98 e-meeting, the measurements and reporting for SL positioning have been captured in NR_pos_enh2 [1]. In this contribution, we present our views on the issue.

- Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].

Positioning measurement

Agreement With regards to the Positioning methods supported using SL-PRS measurements - at least the following measurements should be introduced: - SL-PRS based Rx-Tx measurement - SL-PRS based RSTD measurement - SL-PRS based RSRP measurement - SL-PRS based RSRPP measurement - SL-PRS based RTOA measurement - SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement

In the RAN1#112 meeting, some measurement definitions are agreed upon.  In this section, we present our views on the remaining issue for SL measurement.

The measurement for RTT

SL-PRS based Rx-Tx measurement

Based on the following agreement, the following alternative is considered

Agreement For definition of SL-PRS based Rx-Tx measurement, downselect one of the following alternatives in RAN1# 112b to minimize the impact of UE reference timing offset and mobility - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt2: SL-PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu.

But in Rel-16 and Rel-17 positioning, only Alt 1 and Alt 3 are involved. In addition, the symbol level definition is unclear for us. In this case, we propose to downselect from Alt 1 and Alt 3 for Rx-Tx timing difference. So the definition can still be TUE-RX – TUE-TX

Where:

- TUE-RX is the UE received timing of sidelink subframe #i from another UE that is transmitted the SL-PRS, defined by the first detected path in time.

- TUE-TX  can be downselected from Alt 1 and Alt 3

  - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement

  - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu.

Figure 1 the example of Rx-Tx timing difference

Actually, considering the above figure, the two alternatives are complementary and the other can be achieved by either (e.g., Alt 1=3ms- Alt 3) if there is no timing reference offset.

In the last meeting, we agree to consider the definition to minimize the impact of UE reference timing offset and mobility. That is if the timing reference is changed due to UE movement as in the following Figure 2, the sidelink transmission timing may be changed between subframe#i and subframe#j, and then an additional error will be introduced in RTT positioning

- 12.2.5 SyncRef UE as synchronization reference source The requirements in this subclause are applicable when the reference timing used for deriving sidelink transmission is from SyncRef UE transmitting sidelink synchronization signals. The sidelink transmissions takes place before the reception of the first detected path (in time) of the corresponding timing reference frame from the SyncRef UE, where = 0 and=0. The transmission timing error for sidelink transmissions shall be less than or equal to Te where the timing error limit value Te is defined in Table 12.2.5-1. Table 12.2.5-1: Te Timing Error Limit Frequency Range of sidelink; SCS of sidelink signals (kHz); Te. FR1; 15; 12*64*Tc. 30; 8*64*Tc. 60; 5*64*Tc. Note 1: Tc is the basic timing unit defined in TS 38.211 [6]..

For example, for the SyncRef UE case, the UE 2 sidelink transmission timing equals to the reception timing of the corresponding timing reference frame from the UE1. That is, UE2 Rx-Tx timing difference can be

Alt 1: (j-i) *1ms+T1, T1 is timing adjustment due to the UE movement

Alt 3:  0, if SL-PRS reception time is equal to the #i subframe transmission time

Figure 2 the change in subframe transmission time

The actual difference between Alt 1 and Alt 3 is whether UE2 considers the timing offset (e.g., T1 in Figure 2) between subframe#i and subframe#j of UE2. However, whether T1 is considered for the UE2 side,  both Alt 1 and Alt 3 UE 1 Rx-Tx timing difference measurement has been impacted by T1 of the UE2 side since the actual reception time on UE1 side will be impacted by the actual transmission time of UE2. In this case, if only one reception side considers the timing offset(ie., T1), the additional error will be introduced in RTT positioning.

Alt 1:RTT=UE 1Rx-Tx timing difference -UE 2Rx-Tx timing difference  2≈j-i*1ms+2ρ+T1-（j-i*1ms+T1) 2≈ρ

Alt 3:RTT=UE 1Rx-Tx timing difference -UE 2Rx-Tx timing difference  2≈2ρ+T1-0 2≈ρ+T1/2

Where ρ is the propagation delay between UE 1 and UE2.

Based on the above equation, T1/2 additional error will be introduced for Alt 3. So, we propose:

- For RTT,  Rx – Tx time difference is defined as TUE-RX – TUE-TX , where

  - TUE-RX  is the UE received timing of sidelink subframe #i from another UE, defined by the first detected path in time.

  - TUE-TX is the UE transmit timing of sidelink subframe #j considering the actual SL-PRS transmission time.

If Alt 1 is agreed upon, the remaining issue is whether the integer subframe part needs to be included in Rx – Tx time difference. Where Alt1-1 reports the integer subframe part, and Alt1-2 reports the mod part of Alt1-1 to remove the integer part.  For us, the following two methods are equivalent since the time stamp can be reported with Rx-Tx timing difference so that the integer subframe part can be inferred by two time stamps of DL reception and UL transmission.

Alt 1-1:RTT=UE 1Rx-Tx timing difference -UE 2Rx-Tx timing difference  2≈j-i*1ms+2ρ+T1-（j-i*1ms+T1) 2≈ρ

Alt 1-2: RTT=UE 1Rx-Tx timing difference -UE 2Rx-Tx timing difference  2≈modj-i*1ms+2ρ+T1,1ms-modj-i*1ms+T1,1ms) 2≈2ρ+T1-T12≈ρ

So, we propose to further consider the following alternatives for Alt 1

Alt 1-1: report the Rx – Tx time difference as: Nms+X×2u×TC  with DL reception time stamp

Alt 1-2: report the Rx – Tx time difference as:  X×2u×TC with DL reception time stamp and UL transmission time stamp

- For RTT,  if Alt 1 is adopted, downselect UE Rx-Tx time difference measurement as the following

  - Alt 1-1: report the Rx – Tx time difference as: Nms+X×2u×TC with DL reception time stamp

  - Alt 1-2: report the Rx – Tx time difference as: X×2u×TC with DL reception time stamp and UL transmission time stamp

single-sided and double-sided RTT

Based on the SI agreement, both single-sided and double-sided RTT are agreed upon, in this section, we would like to discuss the measurement for single-sided and double-sided RTT

- both single-sided and double-sided RTT methods, striving to minimize the changes needed on top of the specification support for single-sided RTT, if any, for the introduction of double-sided RTT

single-sided RTT                                                               double-sided RTT

Figure3 single-sided and double-sided RTT

According to Figure 3, regardless of Alt1 and Alt 3 as discussed above, there is no difference in the Rx-Tx timing difference definition.

In addition, even for measurement reports, except to report multiple Rx-Tx measurements, there is no difference between single-sided RTT and double-sided RTT for Alt 3.

For Alt1, the two received SL-PRS should be associated with the same transmit SL-PRS for double-sided RTT calculation.

So, we can find that

- There is no difference in the Rx – Tx time difference definition for single-sided RTT and double-sided RTT

The measurement for SL-TDOA

Agreement For SL-TDOA, DL-TDOA-like operation and UL-TDOA-like operation should be introduced. - A UE is not required to support both DL-TDOA-like operation and UL-TDOA-like operation Agreement With regards to SL-TDOA positioning method for SL-only positioning, - It corresponds to a method where SL-PRS is being transmitted from multiple anchor UEs to a target UE (i.e., DL-TDOA-like operation), and/or from a target UE to multiple anchor UEs (i.e. UL-TDOA-like operation) at least for the purpose of absolute positioning estimation of the target UE - Study the detailed procedure, necessary signalling for SL-TDOA, method(s) to mitigate impact of synchronization error between multiple anchor UEs including whether such method(s) are needed. Proposal Study whether and how to mitigate impact of synchronization errors between anchor UEs for TDoA measurement, considering the following options, and decide by RAN1#112bis whether solutions specifically addressing the impact of synchronization errors between anchor UEs for TDoA measurement will be considered in Rel-18: - Opt1: indication/reporting of synchronization related information, e.g., synchronization source, synchronization source type, synchronization error or quality of synchronization information, TA offset - Opt2: SL-PRS transmission between anchor UEs. No synchronization procedure is expected to be defined based on SL-PRS transmission. - Opt3: (pre-)configuration and capability report of the clock source’s accuracy requirement/threshold. If a UE does not satisfy the requirement, it does not participate in the positioning. - Others options are not precluded Proposed conclusion Enhancements in procedures (e.g. synchronization procedures) specifically addressing the impact of synchronization errors between anchor UEs for TDoA measurement will not be considered by RAN1 in Rel-18.

For TDOA, studying the method to mitigate the impact of synchronization error is a part of the work. And some options are proposed based on the above proposals. For us, only some synchronization information exchange can be supported. That means no additional procedure is expected to be introduced in Rel-18.

For the detailed synchronization information, in NR positioning, the SFN Initialisation time and RTD information can be exchanged and used to align the reference time between multiple TRPs or mitigate the synchronization error between TRPs. For DL-TDOA-like operation and UL-TDOA-like operation, we think it can also reuse the corresponding information(i.e., SFN Initialisation time and RTD information between anchor UEs ) to mitigate the synchronization error or align the reference timing. In addition, considering the multiple synchronization sources that are supported in sidelink (e.g., GNSS,  gNB, and UE),  the synchronization source in use can also be provided.

- To mitigate the impact of synchronization errors between anchor UEs for TDoA measurement, no physical layer synchronization procedures are introduced in Rel-18.

- To mitigate the impact of synchronization errors between anchor UEs for TDoA measurement, the following information enhancement can be supported

  - Exchange of SFN/DFN Initialisation Time of anchor UEs between anchor UE and target/server UE/ LMF.

  - Exchange of synchronization source of anchor UEs between anchor UE and target/server UE/ LMF

  - Exchange of synchronization error (e.g., RTD) of anchor UEs

SL-PRS based RTOA

Agreement SL-PRS based RTOA TSL-RTOA is defined as the beginning time of SL subframe #i containing SL-PRS received from a UE, relative to the RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where - T0 is the nominal beginning time of SFN 0 or DFN0. - FFS on how to select between SFN 0 or DFN 0 for determination of T0. - FFS: the source for the reference timing - tSL-PRS = (10nf + nsf) x 10-3, where nf and nsf are the SFN or DFN and the subframe number of the SL-PRS, respectively - FFS on how to select between SFN or DFN

Firstly, for NR, the T0 is the nominal beginning time of SFN 0 provided by SFN Initialization Time. It is more like a common beginning time between multiple devices as following figure. Similarly, in sidelink, T0 can also be provided by SFN Initialization Time or DFN Initialization Time.

Figure4 RTOA measurement of multiple anchor UEs

For the FFS issue, how to select between SFN 0 or DFN 0 for determination of T0.  In our view, T0 corresponds to SFN 0 only when serving cell timing reference is in use, otherwise, DFN0 is used.

So, we propose

- T0 is the nominal beginning time of SFN 0 provided by SFN Initialization Time if serving cell timing reference is in use, or DFN0 provided by DFN Initialization Time otherwise.

SL-PRS based RSTD

| Definition | DL reference signal time difference (DL RSTD) is the DL relative timing difference between the Transmission Point (TP) [18] j and the reference TP i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from TP j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from TP i that is closest in time to the subframe received from TP j. Multiple DL PRS resources can be used to determine the start of one subframe from a TP. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

In Rel-16, the RSTD measurement in TDOA is the relative timing difference between different TRPs. For SL, the RSTD measurement is the relative timing difference between different anchor UEs.

But, in the last meeting, some companies thought the performance can also be impacted by UE reference timing offset and mobility. In this case, we propose to consider the issue after the RTT solution is clear.

For us, the impact is slightly less than RTT since only target UE moves in TDOA positioning. And anchor UE resource allocation for SL-PRS transmission can be negotiated with server UE or LMF  to reduce the time gap between multiple transmissions, and then the impact of UE reference timing offset and mobility can be further reduced.

So, we propose

- Defer the RSTD definition discussion after the RTT definition is clear.

  - Further consider the impact  of UE reference timing offset and mobility on RSTD measurement

The measurement for SL-AoA

Agreement Support both GCS and LCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement. - FFS on the applicable scenario/service for AoA/ZoA relative to LCS without translation of the LCS to GCS

Firstly, for the FFS issue, if AoA/ZoA is measured by the target UE, and the measurement is used for ranging service (e.g., determining the relative direction relative to target UE). Logically, the target UE can use the AoA/ZoA relative to LCS directly without translation of the LCS to GCS.

In this case, the remaining problem is which scenario/service need to report the AoA/ZoA relative to LCS directly without translation of the LCS to GCS. For us, at least, the scenario/service can be Ranging/SL Positioning service exposure through 5GC network. For example, for MO-LR ranging service requests through 5GC network, the target UE can report the AoA/ZoA relative to LCS directly without translation of the LCS to GCS to LMF

- The target UE can report the AoA/ZoA relative to LCS directly without translation of the LCS to GCS to LMF for Ranging/relative positioning service exposure through 5GC network

In addition, considering the GCS and LCS are supported for AoA/ZoA measurement. The definition can be

- SL Angle of Arrival (SL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined:

-	In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction

-	In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901 [15].

- The SL AoA is determined at measurement UE antenna for a SL channel corresponding to this UE.

Lastly, in NR, the maximum number of UL-AOAs values (pair of AOA & ZOA values) to be reported per SRS resource for the first arrival path corresponding to the same timestamp is 8, which can also be reused for SL-AOA.

For SL, we propose a similar value can be reused.

- The maximum number of SL-AOAs values (pair of AOA & ZOA values) to be reported per SL-PRS for the first arrival path corresponding to the same timestamp can be 8.

SL-PRS RSRP and SL-PRS RSRPP

Agreement SL PRS reference signal received power (SL PRS-RSRP) - is defined as the linear average over the power contributions (in W) of the resource elements that carry SL PRS reference signals configured for RSRP measurements within the considered measurement frequency bandwidth With regard to the reference point - For frequency range 1, the reference point for the SL PRS-RSRP shall be the antenna connector of the UE. - For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches. Agreement SL PRS reference signal received path power (SL PRS-RSRPP), - is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry SL PRS signal configured for the measurement, where SL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. With regard to the reference point - For frequency range 1, the reference point for the SL PRS-RSRPP shall be the antenna connector of the UE. - For frequency range 1, if receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches.

In RAN 1#112 meeting, the definition of SL-PRS RSRP and SL-PRS RSRPP has been agreed upon as above. The remaining issue is whether to introduce the reference point of frequency range 2 in the definition. We are okay with introducing the FR2 reference point and not pursuing the other FR2-specific feature since other SL measurements support the FR2 reference point.

So, we propose

- With regard to the reference point for frequency range 2 of SL-PRS RSRP

  - For frequency range 2, SL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.

  - For frequency range 2, if receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches.

- With regard to the reference point for frequency range 2 of SL-PRS RSRPP

  - For frequency range 2, SL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.

  - For frequency range 2, if receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches.

Measurement report

Agreement For the content of the sidelink positioning measurement report, potential elements may include at least the following: - One or more sidelink positioning measurement(s) - Timestamp(s) associated with a sidelink positioning measurement - Quality metric(s) associated with a sidelink positioning measurement - Identification Information for a sidelink positioning measurement FFS any detail for the above

Separate method VS unified method

- Note: The above categorization does not necessarily mean that there will be separate SL positioning methods specified, or whether there will be a unified SL Positioning method.

In the SID, whether there will be separate SL positioning methods or unified SL positioning methods specified is discussed. For us, unified SL positioning methods are preferred since the combination of multiple measurement results on the UE side is suggested under different positioning methods in Rel-17, and choosing one or more measurement results reporting which is not tied to the positioning method based on measurement request is supported in the TRP side.

So, for sidelink positioning, we propose to refer to the UL positioning solution on the TRP side to choose one or more measurement results reporting for SL positioning based on measurement request and independent of the positioning method.

- A unified positioning method is supported, and to support one or more measurement reporting associated with the same identification can be reported together.

Timestamp

In the SI phase, the timestamp has been agreed to associate with measurement. In this section, we discuss the detailed information on the timestamp in MR. In NR, the timestamp related descriptions are as follows

For the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, and UE Rx-Tx time difference measurements the UE reports an associated higher layer parameter nr-TimeStamp. The nr-TimeStamp can include the dl-PRS-ID, the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by nr-DL-PRS-ReferenceInfo.

For SL, the slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell if the serving cell timing reference is in use, or DFN 0 otherwise. So, an additional indication can be included in the SL timestamp to indicate the timestamp is relative to SFN 0 of the serving cell or DFN 0.

To accompany the SFN and the slot number, nr-TimeStamp can also indicate the reference of the value through nr-DL-PRS-ReferenceInfo in NR positioning. Similarly, for the SL time stamp, if the timing is related to SFN 0 of the serving cell, the cell index of the serving cell can be included. If the timing is related to DFN 0, the synchronization reference source (e.g., GNSS or UE) can be reported.

So, the SL TimeStamp can be

- SFN and the slot number relative to SFN 0 of the serving cell

  - And the nr-PhysCellID, nr-ARFCN, nr-CellGlobalID can be included to identify the serving cell (NR).

- DFN and the slot number relative to DFN 0

  - The synchronization reference source indication (e.g., GNSS or UE), the DFN time (e.g., Tref, offset DFN) can be included

- The SL TimeStamp can be :

  - the SFN and the slot number relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell if the serving cell timing reference is in use

    - And the nr-PhysCellID, nr-ARFCN, nr-CellGlobalID can be included to identify the serving cell

  - or the DFN and the slot number, otherwise

    - And the synchronization reference source indication (e.g., GNSS or UE), the DFN time (e.g., Tref, offset DFN) can be included

Quality

In NR, only timing quality has been defined on the UE side.

TS 38.214 The UE may be configured to report quality metrics NR-TimingQuality corresponding to the DL RSTD and UE Rx-Tx time difference measurements which include the following fields: - timingQualityValue which provides the best estimate of the uncertainty of the measurement - timingQualityResolution which specifies the resolution levels used in the timingQualityValue field.

For the TRP side, timing and angle quality are defined as follows.

Ta 38.455 9.2.43 Measurement quality IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. CHOICE Measurement Quality; M. >Timing Measurement Quality. >>Measurement Quality; M; INTEGER(0..31); TS 37.355 [14]. >>Resolution; M; ENUMERATED(0.1m, 1m, 10m, 30m, …); TS 37.355 [14]. >Angle Measurement Quality. >>Azimuth Quality; M; INTEGER(0..255). >>Zenith Quality; O; INTEGER(0..255). >>Resolution; M; ENUMERATED (0.1deg, …).

Considering the support of SL AoA, we propose both timing quality corresponding to the UE Rx-Tx time difference, RSTD, and RTOA and angle quality corresponding to the AoA are supported.

- Both timing quality corresponding to the timing related measurements and angle quality corresponding to the AoA measurement are supported.

Identification Information for a sidelink positioning measurement

Based on the following specification, TRP ID, nr-DL-PRS-ResourceSetID and nr-DL-PRS-ResourceID-r16 are used to uniquely identify a DL PRS resource in NR positioning.

- 5.1.6.5 PRS reception procedure The UE expects that one of these dl-PRS-ID along with a nr-DL-PRS-ResourceSetID and a nr-DL-PRS-ResourceID-r16 can be used to uniquely identify a DL PRS resource

Naturally, for the SL MR, the UE ID is used to identify the SL-PRS measurement for which UE

Agreement Study the following candidates for identification information in sidelink positioning report, considering different measurements and different reporting targets (LMF and UE): - SL-PRS resource ID/SL-PRS resource set ID if multiple resources/resource sets are configured to a UE - FFS: whether SL-PRS resource set is supported - Source ID and/or destination ID - Other identification information not precluded

In the RAN1#112 meeting, the above informations are studied as candidates for identification information. In addition, SA2 agrees to report the identity of the Located UE(s) with the Ranging measurement data or estimation result. In this case, the unique UE identity will be FFS by SA2.

- Either the Target UE or LMF determines if network assisted SL positioning will be applied. When LMF determines that network assisted SL positioning is used, LMF may trigger the Target UE to perform the discovery of Located UE(s). - The Target UE discovers Located UE(s) for network assisted SL positioning. Optionally, to assist the Target UE the LMF may provide to the Target UE a list of candidate Located UE(s). In this case the LMF maintains the candidate list of Located UE(s) including e.g. the capability and location information of the Located UE(s). - The Target UE and Located UE(s) perform ranging/SL positioning. The Target UE includes the UE identity of the Located UE(s) to the LMF together with the Ranging measurement data or estimation result. The LMF may interact with GMLC to get the location of Located UE. Editor's note: It is FFS what UE identity of the Located UE that the Target UE provides to the LMF. Depending on which UE identity, the LMF or the GMLC may need to resolve this by query another NF. The security impact will be coordinated with SA3.

The remaining issue is whether source ID/ destination ID are needed to be reported in the measurement report. For us, if the source ID/ destination ID of the SL-PRS measurement is different from the source ID/ destination ID of the measurement report, the source ID/ destination ID of the SL-PRS measurement needs to be reported in a measurement report. That is if the measurement report is reported to LMF/server UE as the following figure, the source ID/ destination ID of SL-PRS measurement should be included in SLPP.

SL MR1

SL MR2

SL MR

SL PRS

SL PRS

SL MR1

Target UE

Anchor UE 1

Target UE

Anchor UE 1

Case A                                                                  Case B

Figure5 The alternative architecture for measurement report

Besides, in NR positioning enhancement, ARP and TEG ID have been introduced for multi-panels to achieve high-accuracy positioning considering the current antenna configuration in TR37.885 for V2X is placed as option 2 shown below which is a distributed antenna system and the distributed antennas on the vehicle may affect the positioning performance and reduce the required number of gNB/RSUs to achieve the target positioning accuracy based on the 5GAA LS.

We also think the panel information is helpful for determining the location (e.g., front or rear of a car) of the collision.

Figure6 The ambiguity of the direction of the coming vehicle

Therefore, the ARP ID or panel ID can be one of the identification information to associate measurement information. So, we propose

- For identification information for a sidelink positioning measurement, the following options can be supported.

  - Source ID and/or destination ID of corresponding SL-PRS measurement.

  - ARP ID or panel Id if multiple panels are used.

  - SL-PRS resource ID if multiple resources are agreed to configure for a UE

- Defer the discussion of unique UE identity in RAN1, and may reconsider the issue after SA2 is clear.

The reference identification for RSTD

Based on the NR positioning, and section 2.2.2, the RSTD is defined as the timing difference between the target and reference.

Besides, the NR reference includes TRP ID, resource set ID, and resource ID.

DL-PRS-ID-Info The IE DL-PRS-ID-Info provides the IDs of the reference TRPs' DL-PRS Resources. -- ASN1START DL-PRS-ID-Info-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-DL-PRS-ResourceID-List-r16 SEQUENCE (SIZE (1..nrMaxResourceIDs-r16)) OF NR-DL-PRS-ResourceID-r16 OPTIONAL, -- Need ON nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL -- Need ON }

So, for SL TDOA, the UE ID and SL-PRS resource ID (if more than one resource is introduced) can be used to indicate the reference ID.

- The UE ID and SL-PRS resource ID (if more than one resource is introduced) can be used to indicate the reference of SL-TDOA.

The detailed measurement report for different SL measurement

The detailed measurement report for SL RTT

Reporting to LMF

SL MR2

SL MR1

SL MR

SL PRS

SL PRS

SL MR1

Target UE

Anchor UE 1

Anchor UE 1

Target UE

Case A                                                                  Case B

Figure7 The alternative architecture for measurement report to LMF

For reporting to LMF, there are two cases as above figure

- Case A, the Target UE can report the target UE and anchor UE measurement to LMF after reception anchor UE measurement results.

- Case B, the Target UE and anchor UE separately report their measurement to LMF

Where, for case A, the measurement report can include the Rx-Tx measurement, or ranging information calculated by the target UE if the target UE can perform the ranging calculation.

So, we propose

- The measurement result can be one of the following options based on SL-PRS based Rx-Tx measurement

  - Rx-Tx timing difference measurement

  - Ranging of distance

In addition, we would like to note that the Rx-Tx measurement can be measured by target UE and anchor UE,  how to distinguish whether the Rx-Tx measurement is measured from target UE or anchor UE is a question at least for case A.

In our view, the Rx-Tx measurement from target UE and anchor UE should be associated with different identities to distinguish. For example, to distinguish measurement from which UE, the Rx-Tx measurement of anchor UE can always be associated with source ID and destination ID of anchor UE and for the Rx-Tx measurement of target UE, the destination ID of anchor UE ID can be omitted. Such as, if the measurement is performed by target UE, only the source ID of anchor UE1 can be associated with Rx-Tx measurement. And if the measurement is performed by anchor UE1, the Source ID and destination ID of anchor UE1 can be associated with Rx-Tx measurement.

- To distinguish the SL-PRS based Rx-Tx measurement is measured from target UE or anchor UE on the LMF side for case A (the Target UE reports the target UE and anchor UE measurement to LMF after reception anchor UE measurement results)

  - SL-PRS based Rx-Tx measurement from target UE and anchor UE should be associated with different identities, for example

    - if the measurement is performed by target UE, the Source ID of anchor UE1 can be associated with Rx-Tx measurement

    - if the measurement is performed by anchor UE, the Source ID and destination ID of anchor UE can be associated with Rx-Tx measurement.

For some cases, if  the target UE can perform the ranging calculation and the ranging information needs to exposure to LMF,  the ranging can be reported to LMF, and the Ranging of distance should be associated with anchor UE’s source ID and destination ID to illustrate the distance is between the anchor UE and target UE

- For reporting measurement to LMF, the Ranging of distance should be associated with anchor UE’s source ID and destination ID.

In the following table, take the target UE report to LMF as an example, we provide detailed information for case A and case B architecture.

Table 1 measurement report content for RTT

|  | Reporting to LMF by case A from Target UE | Reporting to LMF by case B from Target UE |
| --- | --- | --- |
| SL-PRS based Rx-Tx measurement | Anchor UE1 identity information（ie., Source ID and destination ID of anchor UE1） ARP/Panel ID if multiple ARP/Panel are used for Target UE Resource/set ID if multiple resources/sets are used Rx-Tx timing difference measured by target UE Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used Quality Time stamp LoS indicator Anchor UE1 identity information（ie., Source ID only of anchor UE1） ARP/Panel ID if multiple ARP/Panel are used for anchor UE 1 Resource/set ID if multiple resources/sets are used Rx-Tx timing difference measured by anchor UE 1 Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used Quality Time stamp LoS indicator | Anchor UE1 identity information（ie., Source ID and destination ID of anchor UE1 for SL-PRS transmission） ARP/Panel ID if multiple ARP/Panel are used for Target UE Resource/set ID if multiple resources/sets are used Rx-Tx timing difference measured by target UE Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used Quality Time stamp LoS indicator |
| Target UE-based Ranging information based on SL-PRS based Rx-Tx measurement | Anchor UE1 identity information（ie., Source ID and destination ID of anchor UE1 for SL-PRS transmission） ARP/Panel ID if multiple ARPs/Panels are used for Target UE Resource/set ID if multiple resources/sets are used Distance of Anchor UE1 and target UE | / |

Based on the above table, we can find the difference between case A and case B is case A can include multiple measurement results from different UEs in a measurement report. So, case A can be seen as multiple case B. In this case, we merge case A and case B as the following table for SL-PRS based Rx-Tx measurement

Table 2 measurement report content for RTT

|  | Reporting to LMF |
| --- | --- |
| SL-PRS based Rx-Tx measurement | One or multiple measurement elements can be included, for each measurement element can include - Anchor UE identity information（ie., Source ID and/or destination ID of anchor UE ） - ARP/Panel ID if multiple ARP/Panel are used for reception UE - Resource/set ID if multiple resources/sets are used - Rx-Tx timing difference which is measured by reception UE - Other measurement results(e.g RSRP, RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |
| Target UE-based Ranging information based on SL-PRS based Rx-Tx measurement | - Anchor UE1’s identity information（ie., Source ID and destination ID of anchor UE1） - ARP/Panel ID if multiple ARPs/Panels are used for Target UE - Resource/set ID if multiple resources/sets are used - Distance of Anchor UE1 and target UE |

Therefore, we propose,

- For the reporting including SL-PRS based Rx-Tx measurement, one or multiple measurement elements can be included in a measurement reporting, for each measurement element can include

  - Anchor UE  identity information（ie., Source ID and/or destination ID of Anchor UE）

  - ARP/Panel ID if multiple ARP/Panel are used for reception UE

  - Resource/set ID if multiple resources/sets are used

  - Rx-Tx timing difference which is measured by reception UE

  - Other measurement results(e.g RSRP, RSRPP, AoA) if multiple measurements are used

  - Quality

  - Time stamp

  - LoS indicator

Reporting to UE

SL PRS

SL MR

Target UE

Anchor UE 1

Figure8 The architecture for measurement report between anchor UE1 and target UE

The big difference between measurement reporting between anchor UE1 and target UE and measurement reporting to LMF is the source ID/destination ID has been transmitted in SCI and MAC CE so that the UE identity information can be omitted in SLPP. Other part elements same as the content to report to LMF

Table 3 measurement report content for RTT to UE

|  | Reporting to Anchor UE1 or Target UE1 |
| --- | --- |
| SL-PRS based Rx-Tx measurement | For each measurement element can include - (Option) UE identity information（ie., Source ID and destination ID of transmission UE for SL-PRS transmission） - ARP/Panel ID if multiple ARP/Panel are used for reception UE - Resource/set ID if multiple resources/sets are used - Rx-Tx timing difference which is measured by reception UE - Other measurement results(e.g RSRP, RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |

- For measurement reporting to target UE or anchor UE for SL-PRS based Rx-Tx measurement,  UE’s source ID and destination ID can be omitted since it has been transmitted by low layer.

- The measurement report content for SL-PRS based Rx-Tx measurement  to target UE or anchor UE same as the content to report to LMF except UE’s source ID and destination ID can be omitted.

The detailed measurement report for DL-like TDOA

SL MR

SL PRS

SL PRS

Anchor UE 2

Target UE

Anchor UE 1

Figure9 The architecture for measurement report for DL-like TDOA

Based on the analysis, the DL-like TDOA architecture same as DL TDOA as above figure. The target UE needs to measure all the SL-PRS from anchor UEs and report the SL-PRS based RSTD measurement to LMF or server UE. Based on the unified architecture, we think there is no difference in measurement report content to LMF and server UE.

- Unified architecture and measurement report content will be used to report to LMF and server UE for SL-PRS based RSTD measurement

In addition, according to TS 23.586, target UE needs to perform a discovery procedure to discover anchor UE (s) with UE identity and report to LMF. And then LMF maintains the candidate list of anchor UE(s). In this case, if LMF maintains the relationship of unique UE identity and source/ destination ID. For measurement reports, only reporting source/ destination ID of anchor UE is enough to identify anchor UE. Meanwhile, the report procedure to server UE is also the same as the report to LMF based on proposal 21

- For reporting measurement to LMF and server UE, SL-PRS based RSTD measurement should be associated with anchor UE identity (ie., anchor UE source ID and destination ID) that is used for discovery and positioning.

For DL-like TDOA, The measurement reporting can also include multiple measurement elements and reference anchor UE information, each element corresponding to different anchor UE(s)

- For the reporting including SL-PRS based RSTD measurement, multiple measurement elements and reference anchor UE information can be included in a measurement report

  - each measurement element corresponding to an anchor UE and reference anchor UE measurement difference

So, the measurement content can be

|  | Reporting to LMF from target UE | Reporting to anchor UE from target UE |
| --- | --- | --- |
| SL-PRS based RSTD measurement | Reference anchor UE information includes: - Reference Anchor UE identity information（ie., Source ID and destination ID of anchor UE1 for SL-PRS transmission） One or multiple measurement elements can be included, for each measurement element can include - Anchor UE identity information（ie., Source ID and destination ID of anchor UE for SL-PRS transmission） - ARP/Panel ID if multiple ARP/Panel are used for Target UE - Resource/set ID if multiple resources/sets are used - RSTD measurement - Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |  |

The detailed measurement report for UL-like TDOA

Report to LMF

SL MR2

SL MR1

SL PRS

SL PRS

SL MR

SL PRS

SL PRS

Anchor UE 2

Target UE

Anchor UE 1

SL MR1

Anchor UE 2

SL MR2

Target UE

Anchor UE 1

Case A                                                                                           Case B

Figure10 The alternative architecture for RTOA measurement report to LMF

Based on the above architecture, we can find that UL-like TDOA is more like SL-RTT. Where

- Case A, the Target UE can report the multiple anchor UE measurements to LMF after reception anchor UE measurement results.

- Case B, the anchor UEs  separately report their measurement to LMF

So, Case A can be seen as multiple case B.

But, for RTOA, only anchor UE can perform the measurement. And different anchor UEs can measure the same SL-PRS from Target UE. That means the SL-PRS may be associated with the same source ID and destination ID. So, similar to RTT and DL-like TDOA, using anchor UE identity to identify the measurement from which anchor UE is more logical for SL-PRS basesd RTOA measurement.

In this case, we propose

- For the reporting including SL-PRS based RTOA measurement, one or multiple measurement elements can be included in a measurement report

  - each measurement element corresponding to an anchor UE measurement

- For reporting measurement to LMF, SL-PRS based RTOA measurement should be associated with anchor UE identity (ie., anchor UE source ID and destination ID).

Lastly, the measurement content is identified in the following table

|  | Reporting to LMF |
| --- | --- |
| SL-PRS based RTOA measurement | One or multiple measurement elements can be included, for each measurement element can include - Anchor UE identity information（ie., Source ID and destination ID of anchor UE for SL-PRS transmission） - ARP/Panel ID if multiple ARP/Panel are used for anchor UE - Resource/set ID if multiple resources/sets are used - RTOA measurement - Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |

Report to UE

In our view, for SL-PRS based RTOA measurement, reporting to UE is almost the same as reporting to LMF except for anchor UE report to target UE case. For example, the blue color MR in case A of Figure 10. In this case, the anchor UE identity can be omitted in SLPP since it is covered in the low layer.

- For measurement reporting to target UE from anchor UE for SL-PRS based RTOA measurement, anchor UE’s  source ID and destination ID can be omitted since it has been transmitted by low layer.

- The measurement report content to target UE from anchor UE for SL-PRS based RTOA measurement, same as the content to report to LMF except anchor UE’s source ID and destination ID can be omitted.

- The measurement report content to server UE from anchor UE for SL-PRS based RTOA measurement, same as the content to report to LMF except anchor UE’s source ID and destination ID can be omitted.

Lastly, the measurement content is identified in the following table

|  | Reporting to UE |
| --- | --- |
| SL-PRS based RTOA measurement | One or multiple measurement elements can be included, for each measurement element can include - (Optional)Anchor UE identity information（ie., Source ID and destination ID of anchor UE for SL-PRS transmission） - ARP/Panel ID if multiple ARP/Panel are used for anchor UE - Resource/set ID if multiple resources/sets are used - RTOA measurement - Other measurement results(e.g RSRP, RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |

The detailed measurement report for AoA

Similar to SL-RTT, Figure 7 can be the architecture to report AoA to LMF and server UE, and Figure 8 can be the architecture between UEs, and the AoA can also be measured by anchor UE and target UE. So, the AoA measurement also needs to be associated with different identities to distinguish the AoA measurement is from target UE or anchor UE

- To distinguish the SL-PRS based SL-PRS based AoA measurement is measured from target UE or anchor UE on the LMF side

  - SL-PRS based AoA measurement from target UE and anchor UE should be associated with different identities, for example

    - if the measurement is performed by target UE, the Source ID of anchor UE1 can be associated with AoA measurement

    - if the measurement is performed by anchor UE, the Source ID and destination ID of anchor UE1 can be associated with AoA measurement.

In addition, based on the timing measurement analysis, reporting to UE is almost the same as reporting to LMF except for anchor UE report to target UE case that UE identity can be omitted.

- For measurement reporting to target UE from anchor UE for SL-PRS based AoA measurement, anchor UE’s  source ID and destination ID can be omitted since it has been transmitted by low layer.

- The measurement report content to UE for SL-PRS based AoA measurement same as the content to report to LMF except anchor UE’s source ID and destination ID can be omitted.

Therefore, the measurement content is identified as following table

|  | Reporting to LMF | Reporting to UE |
| --- | --- | --- |
| SL-PRS based AoA measurement | One or multiple measurement elements can be included, for each measurement element can include - (Optional)Anchor UE identity information（ie., Source ID and destination ID of anchor UE for SL-PRS transmission） - ARP/Panel ID if multiple ARP/Panel are used for Target UE - Resource/set ID if multiple resources/sets are used - AoA measurement - Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |  |

The detailed measurement report  for RSRP and RSRPP measurement

RSRP and RSRPP measurement can only be reported along with other methods. In this case, the information should be reported together with other SL measurements as optional part

- RSRP and RSRPP measurements are reported together with other SL measurements as an optional part.

Summary

Based on the analysis in sections 3.5.1~3.5.5, we can summarize the measurement report content in the following table

|  | Reporting to LMF | Reporting to UE |
| --- | --- | --- |
| SL-PRS based Rx-Tx measurement | One or multiple measurement elements can be included, for each measurement element can include - （Optional）Anchor UE identity information（ie., Source ID and/or destination ID of anchor UE ） - ARP/Panel ID if multiple ARP/Panel are used for reception UE - Resource/set ID if multiple resources/sets are used - Rx-Tx timing difference which is measured by Rx UE - Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |  |
| SL-PRS based RSTD measurement | Reference anchor UE information includes: - Reference Anchor UE identity information（ie., Source ID and destination ID of anchor UE1 for SL-PRS transmission） One or multiple measurement elements can be included, for each measurement element can include - Anchor UE identity information（ie., Source ID and destination ID of anchor UE for SL-PRS transmission） - ARP/Panel ID if multiple ARP/Panel are used for Target UE - Resource/set ID if multiple resources/sets are used - RSTD measurement - Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |  |
| SL-PRS based RTOA measurement | One or multiple measurement elements can be included, for each measurement element can include - (Optional)Anchor UE identity information（ie., Source ID and destination ID of anchor UE for SL-PRS transmission） - ARP/Panel ID if multiple ARP/Panel are used for anchor UE - Resource/set ID if multiple resources/sets are used - RTOA measurement - Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |  |
| SL-PRS based AoA measurement | One or multiple measurement elements can be included, for each measurement element can include - (Optional)Anchor UE identity information（ie., Source ID and destination ID of anchor UE for SL-PRS transmission） - ARP/Panel ID if multiple ARP/Panel are used for Target UE - Resource/set ID if multiple resources/sets are used - AoA measurement - Other measurement results(e.g RSRP,RSRPP, AoA) if multiple measurements are used - Quality - Time stamp - LoS indicator |  |
| SL-PRS based RSRP/RSRPP | Needs to be reported with other measurements |  |

According to the above table, we can find other elements are the same except for the yellow highlighted part. In this case, the measurement report can be further summarized as follows

|  | Reporting to LMF | Reporting to anchor UE |
| --- | --- | --- |
| SL-PRS based measurement | One or multiple measurement elements can be included, for each measurement element can include - （Optional）Anchor UE identity information（ie., Source ID and/or destination ID of anchor UE ） - ARP/Panel ID if multiple ARP/Panel are used for reception UE - Resource/set ID if multiple resources/sets are used - Measurement results - （Optional）Rx-Tx timing difference - （Optional）RSTD measurement - （Optional）RTOA measurement - (Optional) AoA measurement - (optional ) RSRP,RSRPP measurement - Quality - Time stamp - LoS indicator |  |

- Considering the majority of parameters can be reused for a measurement element,  the unified measurement content can be summarized as follows for all SL measurement

  - （Optional）Anchor UE identity information（ie., Source ID and/or destination ID of anchor UE ）

  - ARP/Panel ID if multiple ARP/Panel are used for reception UE

  - Resource/set ID if multiple resources/sets are used

  - Measurement results

    - （Optional）Rx-Tx timing difference which is measured by Rx UE

    - （Optional）RSTD measurement

    - （Optional）RTOA measurement

    - (Optional)   AoA measurement

    - (optional ) RSRP, RSRPP measurement

  - Quality

  - Time stamp

  - LoS indicator

Location information reporting

Agreement Study the necessity and scenarios of including location information and quality information of the location of a UE in sidelink positioning measurement report, considering different measurements and different reporting targets (LMF and UE).

In NR, UE-based positioning is supported, the UE location can be reported to LMF. For SL ranging service and SL positioning service, RAN2 agrees on the following procedure including location calculation and providing location information.

Agreement:

With respect to the overall signaling procedure for PC5-only positioning (including at least IC and OOC; FFS if there are differences for PC), it is proposed to agree that the sidelink positioning procedure comprises the following series of steps as a baseline, between the LMF/positioning server UE/NG-RAN/candidate Anchor UE(s) and Target UE(s):

- Triggering event

- Sidelink positioning capability exchange

3.	Sidelink positioning assistance data transfer

4.	SL Positioning Request Location Information

5.	Measurement of SL-PRS

6.	Location calculation

7.	SL Positioning Provide Location Information

Some steps may have dependencies on SA2 and can be revisited in this light.  The order is subject to further discussion.  FFS if discovery and selection of anchor UEs and/or server UE are part of the positioning layer in RAN2 scope.

LS to SA2 to ask for confirmation and guidance on the SA2 aspects.

Therefore, we propose, the following information can be reported

- For Ranging/SL positioning service, the following information can be reported

  - Ranging

  - UE location

  - UE relative location.

Additional path measurement

In SI, the SL-RSRPP is agreed upon, in this case, there are some issues that need to be further discussed.

Issue 1:  the number of additional paths at least for SL-RSRPP for an SL-PRS

Issue 2:  whether the timing and angle measurement can be supported per the first and additional path or not.

For us, considering NR positioning, reporting the first path and additional path measurement for timing and angle measurement has been supported. And the number of additional paths for different methods are summarized as the following

| Method | RSTD | Rx-Tx | RTOA | AoA | RSRPP |
| --- | --- | --- | --- | --- | --- |
| Path number | {2, 4, 8} | {2, 4, 8} | {2, 4, 6, 8} | {1, 2, 4, 8} | Support for timing-based positioning |

So we propose

- The first path and additional path can be supported for SL-based RSTD, Rx-Tx time difference, RToA, AoA measurement.

- The additional path can be [2, 4, 8] for a SL-PRS.

Signaling for measurement or measurement report

Signaling for measurement reports

##### [MEDIUM] Feature Lead Proposal 5.2-v0 With regards to the sidelink positioning measurement report, - RAN1 assumes as a starting point that higher layer signaling for the positioning measurement report shall be used. - FFS: Whether other signaling (e.g. low-layer signaling) could also be applicable.

Considering the payload and flexibility transmission of the measurement report, the MAC CE or high-layer signaling (e.g., SLPP) can be used to carry SL measurement report for SL-PRS. In addition, considering the SLPP has been agreed to support, we prefer to carry SL-PRS measurement reports by SLPP.

- SLPP is used to carry the SL-PRS measurement report.

Signaling for measurement report requests

In addition, in NR, the measurement report can be triggered by the LMF or UE itself by LPP as follow.

LMF-initiated                                                    UE-initiated

Figure8  LMF-initiated/UE-initiated Location Information Transfer Procedure

For SL, the measurement report also can be triggered by the LMF or UE itself. In addition, the measurement report can also be triggered by another UE. The remaining issue is which signaling is used to trigger the MR. Considering CSI reports in NR SL can be triggered by SCI or MAC CE, and it has agreed to support SLPP in SL positioning, we prefer that SL-PRS measurement report can be triggered by SLPP at least, and FFS triggered by lower layer.

Anchor UE 1

Target UE

SL report request

SL PRS

measurements

MR

Figure9  A typical Location Information Transfer Procedure between UEs

- SL-PRS measurement reports can be triggered by SLPP

  - FFS whether SCI can be used to trigger feedback SL-PRS or measurement report.

In addition, considering the multiple measurements (e.g., RSTD, Rx-Tx difference, RSRP) is supported for SL. The measurement report request can include the type of measurement.

Latency condition

For SL CSI reporting, the MAC entity maintains an sl-CSI-ReportTimer for each pair of the Source Layer-2 ID and the Destination Layer-2 ID. Sl-CSI-ReportTimer is used for an SL-CSI reporting UE to follow the latency requirement signalled from a CSI triggering UE. The value of sl-CSI-ReportTimer is the same as the‎ latency requirement of the SL-CSI reporting in sl-LatencyBoundCSI-Report configured by RRC.

Similar to sl-CSI-ReportTimer, a latency boundary used for positioning measurement reporting can be introduced.

- Positioning measurement reporting latency boundary can be considered to be introduced.

Conclusion

In this contribution, we discuss potential positioning enhancements with the following observation and proposals.

- There is no difference in the Rx – Tx time difference definition for single-sided RTT and double-sided RTT

- For RTT,  Rx – Tx time difference is defined as TUE-RX – TUE-TX , where

  - TUE-RX  is the UE received timing of sidelink subframe #i from another UE, defined by the first detected path in time.

  - TUE-TX is the UE transmit timing of sidelink subframe #j considering the actual SL-PRS transmission time.

- For RTT,  if Alt 1 is adopted, downselect UE Rx-Tx time difference measurement as the following

  - Alt 1-1: report the Rx – Tx time difference as: Nms+X×2u×TC with DL reception time stamp

  - Alt 1-2: report the Rx – Tx time difference as: X×2u×TC with DL reception time stamp and UL transmission time stamp

- To mitigate the impact of synchronization errors between anchor UEs for TDoA measurement, no physical layer synchronization procedures are introduced in Rel-18.

- To mitigate the impact of synchronization errors between anchor UEs for TDoA measurement, the following information enhancement can be supported

  - Exchange of SFN/DFN Initialisation Time of anchor UEs between anchor UE and target/server UE/ LMF.

  - Exchange of synchronization source of anchor UEs between anchor UE and target/server UE/ LMF

  - Exchange of synchronization error (e.g., RTD) of anchor UEs

- T0 is the nominal beginning time of SFN 0 provided by SFN Initialization Time if serving cell timing reference is in use, or DFN0 provided by DFN Initialization Time otherwise.

- Defer the RSTD definition discussion after the RTT definition is clear.

  - Further consider the impact  of UE reference timing offset and mobility on RSTD measurement

- The target UE can report the AoA/ZoA relative to LCS directly without translation of the LCS to GCS to LMF for Ranging/relative positioning service exposure through 5GC network

- SL Angle of Arrival (SL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined:

-	In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction

-	In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relative to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901 [15].

- The SL AoA is determined at measurement UE antenna for a SL channel corresponding to this UE.

- The maximum number of SL-AOAs values (pair of AOA & ZOA values) to be reported per SL-PRS for the first arrival path corresponding to the same timestamp can be 8.

- With regard to the reference point for frequency range 2 of SL-PRS RSRP

  - For frequency range 2, SL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.

  - For frequency range 2, if receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches.

- With regard to the reference point for frequency range 2 of SL-PRS RSRPP

  - For frequency range 2, SL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.

  - For frequency range 2, if receiver diversity is in use by the UE, the reported SL PRS-RSRPP value shall not be lower than the corresponding SL PRS-RSRPP of any of the individual receiver branches.

- A unified positioning method is supported, and to support one or more measurement reporting associated with the same identification can be reported together

- The SL TimeStamp can be :

  - the SFN and the slot number relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell if the serving cell timing reference is in use

    - And the nr-PhysCellID, nr-ARFCN, nr-CellGlobalID can be included to identify the serving cell

  - or the DFN and the slot number, otherwise

    - And the synchronization reference source indication (e.g., GNSS or UE), the DFN time (e.g., Tref, offset DFN) can be included

- Both timing quality corresponding to the timing related measurements and angle quality corresponding to the AoA measurement are supported.

- For identification information for a sidelink positioning measurement, the following options can be supported.

  - Source ID and/or destination ID of corresponding SL-PRS measurement.

  - ARP ID or panel Id if multiple panels are used.

  - SL-PRS resource ID if multiple resources are agreed to configure for a UE

- Defer the discussion of unique UE identity in RAN1, and may reconsider the issue after SA2 is clear.

- The UE ID and SL-PRS resource ID (if more than one resource is introduced) can be used to indicate the reference of SL-TDOA.

- The measurement result can be one of the following options based on SL-PRS based Rx-Tx measurement

  - Rx-Tx timing difference measurement

  - Ranging of distance

- To distinguish the SL-PRS based Rx-Tx measurement is measured from target UE or anchor UE on the LMF side for case A (the Target UE reports the target UE and anchor UE measurement to LMF after reception anchor UE measurement results)

  - SL-PRS based Rx-Tx measurement from target UE and anchor UE should be associated with different identities, for example

    - if the measurement is performed by target UE, the Source ID of anchor UE1 can be associated with Rx-Tx measurement

    - if the measurement is performed by anchor UE, the Source ID and destination ID of anchor UE can be associated with Rx-Tx measurement.

- For reporting measurement to LMF, the Ranging of distance should be associated with anchor UE’s source ID and destination ID.

- For the reporting including SL-PRS based Rx-Tx measurement, one or multiple measurement elements can be included in a measurement reporting, for each measurement element can include

  - Anchor UE  identity information（ie., Source ID and/or destination ID of Anchor UE）

  - ARP/Panel ID if multiple ARP/Panel are used for reception UE

  - Resource/set ID if multiple resources/sets are used

  - Rx-Tx timing difference which is measured by reception UE

  - Other measurement results(e.g RSRP, RSRPP, AoA) if multiple measurements are used

  - Quality

  - Time stamp

  - LoS indicator

- For measurement reporting to target UE or anchor UE for SL-PRS based Rx-Tx measurement,  UE’s source ID and destination ID can be omitted since it has been transmitted by low layer.

- The measurement report content for SL-PRS based Rx-Tx measurement  to target UE or anchor UE same as the content to report to LMF except UE’s source ID and destination ID can be omitted.

- Unified architecture and measurement report content will be used to report to LMF and server UE for SL-PRS based RSTD measurement

- For reporting measurement to LMF and server UE, SL-PRS based RSTD measurement should be associated with anchor UE identity (ie., anchor UE source ID and destination ID) that is used for discovery and positioning

- For the reporting including SL-PRS based RSTD measurement, multiple measurement elements and reference anchor UE information can be included in a measurement report

  - each measurement element corresponding to an anchor UE and reference anchor UE measurement difference

- For the reporting including SL-PRS based RTOA measurement, one or multiple measurement elements can be included in a measurement report

  - each measurement element corresponding to an anchor UE measurement

- For reporting measurement to LMF, SL-PRS based RTOA measurement should be associated with anchor UE identity (ie., anchor UE source ID and destination ID).

- For measurement reporting to target UE from anchor UE for SL-PRS based RTOA measurement, anchor UE’s  source ID and destination ID can be omitted since it has been transmitted by low layer.

- The measurement report content to target UE from anchor UE for SL-PRS based RTOA measurement, same as the content to report to LMF except anchor UE’s source ID and destination ID can be omitted.

- The measurement report content to server UE from anchor UE for SL-PRS based RTOA measurement, same as the content to report to LMF except anchor UE’s source ID and destination ID can be omitted.

- To distinguish the SL-PRS based SL-PRS based AoA measurement is measured from target UE or anchor UE on the LMF side

  - SL-PRS based AoA measurement from target UE and anchor UE should be associated with different identities, for example

    - if the measurement is performed by target UE, the Source ID of anchor UE1 can be associated with AoA measurement

    - if the measurement is performed by anchor UE, the Source ID and destination ID of anchor UE1 can be associated with AoA measurement.

- For measurement reporting to target UE from anchor UE for SL-PRS based AoA measurement, anchor UE’s  source ID and destination ID can be omitted since it has been transmitted by low layer.

- The measurement report content to UE for SL-PRS based AoA measurement same as the content to report to LMF except anchor UE’s source ID and destination ID can be omitted.

- RSRP and RSRPP measurements are reported together with other SL measurements as an optional part.

- Considering the majority of parameters can be reused for a measurement element,  the unified measurement content can be summarized as follows for all SL measurement

  - （Optional）Anchor UE identity information（ie., Source ID and/or destination ID of anchor UE ）

  - ARP/Panel ID if multiple ARP/Panel are used for reception UE

  - Resource/set ID if multiple resources/sets are used

  - Measurement results

    - （Optional）Rx-Tx timing difference which is measured by Rx UE

    - （Optional）RSTD measurement

    - （Optional）RTOA measurement

    - (Optional)   AoA measurement

    - (optional ) RSRP, RSRPP measurement

  - Quality

  - Time stamp

  - LoS indicator

- For Ranging/SL positioning service, the following information can be reported

  - Ranging

  - UE location

  - UE relative location.

- The first path and additional path can be supported for SL-based RSTD, Rx-Tx time difference, RToA, AoA measurement.

- The additional path can be [2, 4, 8] for a SL-PRS.

- SLPP is used to carry the SL-PRS measurement report.

- SL-PRS measurement reports can be triggered by SLPP

  - FFS whether SCI can be used to trigger feedback SL-PRS or measurement report.

- Positioning measurement reporting latency boundary can be considered to be introduced.

- References

- RP-223549 New WID on Expanded and Improved NR Positioning. Dec.12-16, 2022

- TS 38.215 Physical layer measurements, v17.2.0

- R1-2211011 Evaluation of sidelink positioning performance, vivo.