3GPP TSG RAN WG1 #112bis-e			                               R1-2302495

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:	Discussion on bandwidth aggregation for positioning measurements

Agenda Item:	9.5.4

Document for:	Discussion and Decision

Introduction

In RAN #98 e-meeting, bandwidth aggregation has been captured in NR_pos_enh2 [1]. In this contribution, we present our views on the issue.

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

In the SI phase, RAN4 has studied PRS/SRS bandwidth aggregation and reached a bunch of agreements. In this contribution, we discuss the issue from the RAN1 perspective.

The condition for bandwidth aggregation

Agreement To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs: - In the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna), this implies - FFS: The same gNB Tx TEG and the same UE Rx TEG, the maximum TX timing error margin - The same QCL - The same number of symbols, symbol location within one slot, repetition factor, - FFS: the same periodicity and slot offset - FFS muting pattern - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of PRS resource sets and resources for a TRP - The same power per subcarrier - FFS: the same NR-DL-PRS-SFN0-Offset - Aggregated PFLs are configured on the same aligned numerology grid - FFS: How to maintain contiguous PRS pattern across aggregated bandwidths even in the presence of guard tones (e.g, PFLs with different RE-offset configurations, PFLs with different point A) - Phase continuity between aggregated PFLs Agreement To enable SRS bandwidth aggregation between SRS in two or three carriers, the following conditions should be satisfied for the aggregated SRS resources across the aggregated carriers - In the same slot, in same symbols, from the same antenna, this implies - FFS: The same gNB Rx TEG and the same UE Tx TEG - The same spatial relation - The same startPosition, nrofSymbols - FFS: periodicityAndOffset, and slotOffset - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of SRS resource sets and resources - The same Tx PSD (power per subcarrier) - FFS whether to need the same pathloss RS, Po and alpha - Note: the Tx PSD is not captured in RAN1 specifications - FFS: SRS with RE-offset configuration which maintains contiguous SRS pattern across aggregated bandwidths even in the presence of guard tones - Phase continuity between aggregated SRS in different carriers

In RAN1#112 meeting, some conditions are agreed upon as above, but there are some FFSs for the condition of PRS bandwidth aggregation, including

- TEG issue

Agreement: - No need to discuss TAE for single RF chain architecture - TAE refers the TAE requirements across carriers in BS specification. Agreement: - Agree on Option 1 - single chain Tx architecture means the single RF chain with single antenn

Based on the RAN4 agreement, the single RF chain means the same TEG at least. Besides, due to TEG being per band capability, the same TEG means the same TEG ID. For the ideal situation, the same RF chain and antenna may mean the timing error difference between different carriers is tiny. But considering the legacy TAE requirement will be reused based on RAN4 agreement, we are not sure if any RAN1 condition for the timing error difference is needed. So, from the RAN1 perspective, the first FFS can be the same TEG ID

- time domain issue

In the last meeting, some companies propose that partial periodicity can be aggregated and partial can not. The enhancement condition may lead introduce the different UE behaviors in different periodicities and complex RAN4 latency requirements for CA. So, for simplicity, we prefer the periodicity and offset to be the same.

- The number of resource/set issue

In our view, the resource number is defined according to the beam configuration, as for intra-band and single RF chain case, the resource numbers in a set that can be aggregated should be the same for different carriers. Regareds the condition for the same set number, it is only needed for the case that all PRS set per TRP are linked to being aggregated. So, we prefer the same number of PRS resources for an aggregated set of a TRP at least

(4) Guard band issue

The same aligned numerology grid condition for aggregated PFLs has been agreed upon. It asks the aggregated PFLs to use the same raster. The channel spacing between adjacent carriers will be multiple of 0.3MHz  based on the definition as following RAN4 definition and may not be multiple of 180kHz(1PRB) for SRS/PRS. So it is difficult to guarantee the continuous SRS/PRS patterns with the same RE offset for different carriers. The potential solutions can be

- PFLs with different RE-offset configurations

- PFLs with the common point A for intra-band CA and different start PRB.

For intra-band contiguous carrier aggregation with two or more component carriers, the nominal channel spacing between two adjacent NR component carriers is defined as the following unless stated otherwise: For NR operating bands with a 100 kHz channel raster: while for NR operating bands without a 100 kHz channel raster:

From the transmission or reception side, common point A is more simple to generate a larger signal in aggregation bandwidth.  But anyway the condition is to maintain the contiguous PRS pattern, how to maintain contiguous PRS pattern can be up to the implementation. Therefore, we propose

- To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following additional conditions should be satisfied for the aggregated PRS resources from a TRP

  - The same periodicity and offset, and slot offset

  - The same number of PRS resource

  - The frequency gap of PRS RE between PFLs is the multiple of comb size and SCS

    - How to maintain contiguous PRS pattern can be up to the implementation

- Phase/timing continuity between PFLs can be up to RAN4

Similar to PRS, the SRS is proposed as follows

- To enable SRS bandwidth aggregation between SRS in two or three carriers, the following additional conditions should be satisfied for the aggregated SRS resources

  - The same periodicity and offset, and slot offset

  - The same number of SRS resource

  - The frequency gap of SRS RE between carriers is the multiple of comb size and SCS

    - How to maintain contiguous SRS pattern can be up to the implementation

- Phase/timing continuity between carriers can be up to RAN4

The enhancement for DL bandwidth aggregation

  - 3.1 The configuration and reporting information enhancement

3.1.1 The configuration for PRS bandwidth aggregation

Agreement For PRS bandwidth aggregation across PFLs, support enhancement of PRS configuration to inform UE by LMF (or inform LMF by NG-RAN) PRS resources from which two or three PFLs are linked. - FFS whether the link is for all TRPs or per TRP basis - FFS whether the link is per PRS resource set basis or per PRS resource basis.

The remaining two FFS issues are about the link level. We prefer per resource set per TRP level since only intra-band CA is supported. This is due to the fact the same beams are expected to be configured (i.e., same resource number is expected) for a TRP in intra-bands. Besides, regarding the link is per PRS resource set basis or per PRS resource basis, only part resources support CA is weird for a PRS resource set considering the PRS is a cell-specific signal and UE can be in any direction. So, we propose

- For PRS bandwidth aggregation across PFLs, the link is per resource set per TRP

Besides, based on the above agreement, enhancement of PRS configuration is supported to inform UE by LMF (or inform LMF by NG-RAN) PRS resources from which two or three PFLs are linked. The remaining issue is how to inform UE that two or three PFLs can be linked. Due to per resource set link is proposed, we suggest adding the following information under the PRS resource set configuration

- linked PFL information(e.g.,PFL group information, up two PFL indices, up two sets of frequency information (e.g.(dl-PRS-PointA, StartPRB, Bandwidth))) to indicate which positioning frequency layers can be linked with the PRS resource set

- linked PRS resource set ID under linked PFL.

- Adding linked  PFLs and PRS resource set information under PRS resource set configuration for set level PRS bandwidth aggregation across PFLs

  - linked PFL information(e.g ., PFL group information, or up two PFL indices, up two sets of frequency information (e.g.(dl-PRS-PointA, StartPRB, Bandwidth))

  - linked PRS resource set ID under linked PFL

3.1.2 The measurement reporting

Agreement Support joint measurement and report for the PRS resources aggregated across the PFLs for DL-TDOA and multi-RTT positioning methods - In a measurement report element, single RSTD or single UE Rx-Tx time difference is reported for the PRS resources across aggregated PFLs - FFS: RSRP, RSRPP - FFS: In a measurement report, PFL aggregation indication is supported to indicate whether/which PFLs are aggregated for the PRS measurement - FFS whether to use PRS assistance data or use location information request message to indicate UE to perform joint measurement across aggregated PFLs - FFS RSTD reference configuration or report should be enhanced

For the FFS about PFL aggregation indication is supported to indicate whether/which PFLs are aggregated for the PRS measurement, in our view, the PFL aggregation indication anyway is needed. The problem is detailed signaling and content of the PFL aggregation. Based on the analysis, the PFL aggregation can be included in the three pieces of information.

- Assistance data

The assistance data includes PFL aggregation indication including detailed PFL information (e.g., which PFLs can be aggregated). But the assistance date can be broadcast, it not means all the UE should be aggregated PFLs if the assistance data includes PFL aggregation indication. So, it may be combined location request information to enable PFL aggregation measurement.

- Location request information

The location request information can include PFL aggregation indication(e.g., whether PFLs) are aggregated for the PRS measurement, and then UE can combine the location request information and assistance date to determine whether and which PFLs can be aggregated

- Location reporting information

The PFL aggregation indication can also associate with the joint RSTD or single UE Rx-Tx time difference measurement to indicate which PFLs are aggregated for joint measurements if multiple PFLs can be aggregated. E.g, 3 PFLs can be aggregated from TRP and assistance data level, but UE can only aggregate two PFLs. In this case, which PFLs can be aggregated may be needed to inform LMF.

Therefore, we propose

- For PFL aggregation for the measurement reporting and location request information, support the following.

  - The location request information can include PFL aggregation request to indicate whether PFLs are requested to be aggregated for the PRS measurement

  - The location reporting information can include PFL aggregation indication to indicate which PFLs are aggregated for the PRS measurement

And then the remaining problem is what is the PFL aggregation information. The candidate PFL aggregation indication in a measurement report are listed as follows

Option 1: association with up three PFL information(e.g., up three PFL indices, up three sets of frequency information (e.g.(dl-PRS-PointA, StartPRB, Bandwidth)

Option 2: association with a PFL group indication.

Therefore, we propose.

- PFL aggregation indication can be one of the following options in a measurement report

  - Option 1: association with up three PFL information(e.g., up three PFL indices, up three sets of frequency information (e.g.(dl-PRS-PointA, StartPRB, Bandwidth)

  - Option 2: association with a PFL group indication.

Agreement: - From RRM perspective, FFT/IFFT size is up to UE implementation. PRS/SRS bandwidth aggregation should allow UE implementation flexibility i.e., single FFT/IFFT or multiple FFTs/IFFTs (i.e. FFT/IFFT per carrier) implementations. - PRS/SRS bandwidth aggregation may be supported in RRC_CONNECTED and RRC_INACTIVE subject to UE capability. - PRS/SRS bandwidth aggregation across Positioning Frequency Layers (PFLs) for positioning measurements is concluded as feasible from RRM perspective.

Based on the previous agreement, the FFT size is up to UE implementation. So to distinguish the measurement behavior (i.e., single FFT/IFFT or multiple FFTs/IFFTs (i.e., FFT/IFFT per carrier)), the FFT type indicator can be introduced.

- Introduce an indicator to distinguish single FFT/IFFT or multiple FFT/IFFT operation for bandwidth aggregation measurement.

  - 3.2 The enhancement for on-demand PRS

Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details

3.2.1 On-demand PRS request

Agreement - From RAN1 perspective, for LMF-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signaled - per resource set per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - LMF recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - LMF requests to provide the QCL information in the assistance data in NRPPa - per FR - Number of DL PRS frequency layers - either per resource set per positioning frequency layer or per UE - Start/end time of DL PRS transmission - either per resource, or per resource set, or per UE - ON/OFF indicator (for LMF initiated request only) Agreement - From RAN1 perspective, for UE-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signalled - per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN - per FR - Number of DL PRS frequency layers - per UE - Start/end time of DL PRS transmission Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - UE recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - UE requests to provide the QCL information in the assistance data

To support LMF-initiated and UE-initiated on-demand PRS requests for PRS bandwidth aggregation, there are two options

Option 1: adding PFL aggregation indication under existing request and restricting some request parameters (e.g., periodicity, combsize N, symbol number, QCL) of multiple PFLs are the same if PFL aggregation indication is enable

Option 2: per aggregated PFLs on demand parameters are provided by the following red IE.

NR-On-Demand-DL-PRS-Request-r17 ::= SEQUENCE { dl-prs-StartTime-and-Duration-r17 DL-PRS-StartTime-and-Duration-r17 OPTIONAL, nr-on-demand-DL-PRS-Information-r17 NR-On-Demand-DL-PRS-Information-r17 OPTIONAL, nr-on-demand-DL-PRS-Information-CA-r18 NR-On-Demand-DL-PRS-Information-CA-r18 OPTIONAL, dl-prs-configuration-id-PrefList-r17 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF DL-PRS-Configuration-ID-r17 OPTIONAL, ... }

For us, option 2 is preferred. Therefore, we propose

- From RAN1 perspective, for LMF-initiated request of on-demand DL PRS,  the following group of on-demand DL PRS parameters is defined and signaled for bandwidth aggregation case

  - per set of positioning frequency layer per FR

      - Number of DL PRS frequency layers

      - DL PRS Periodicity

      - DL PRS Resource Bandwidth

      - DL PRS Resource Repetition Factor

      - Number of DL PRS Resource Symbols per DL PRS Resource

      - DL-PRS CombSizeN

  - per set of positioning frequency layer per UE

- Start/end time of DL PRS transmission

Two options for indication of DL PRS QCL-Info, either

  - Option 1: per resource set per set of positioning frequency layer per FR

      - UE recommends a list of QCL sources

  - Option 2: per resource set per set of positioning frequency layer per FR

      - UE requests to provide the QCL information in the assistance data

- From RAN1 perspective, for UE-initiated request of on-demand DL PRS,  the following group of on-demand DL PRS parameters is defined and signalled for band aggregation case

  - per set of positioning frequency laye rper FR

      - Number of DL PRS frequency layers

      - DL PRS Periodicity

      - DL PRS Resource Bandwidth

      - DL PRS Resource Repetition Factor

      - Number of DL PRS Resource Symbols per DL PRS Resource

      - DL-PRS CombSizeN

Two options for indication of DL PRS QCL-Info, either

  - Option 1: per resource set per set of positioning frequency layer per FR

      - UE recommends a list of QCL sources

  - Option 2: per resource set pper set of positioning frequency layer per FR

      - UE requests to provide the QCL information in the assistance data

  - either per resource set per set of positioning frequency layer or per UE

      - Start/end time of DL PRS transmission

  - either per resource, or per resource set, or per UE

      - ON/OFF indicator (for LMF initiated request only)

3.2.2 Pre-configured on-demand PRS

Pre-configured on-demand PRS is also supported, and the information also includes the PFL information(i.e., nr-DL-PRS-PositioningFrequencyLayer).

#### – NR-On-Demand-DL-PRS-Configurations The IE NR-On-Demand-DL-PRS-Configurations provides a set of possible DL-PRS configurations which can be requested by the target device on-demand. -- ASN1START NR-On-Demand-DL-PRS-Configurations-r17 ::= SEQUENCE { on-demand-dl-prs-configuration-list-r17 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF On-Demand-DL-PRS-Configuration-r17, ... } On-Demand-DL-PRS-Configuration-r17 ::= SEQUENCE { dl-prs-configuration-id-r17 DL-PRS-Configuration-ID-r17, nr-DL-PRS-PositioningFrequencyLayer-r17 NR-DL-PRS-PositioningFrequencyLayer-r16, nr-DL-PRS-Info-r17 NR-DL-PRS-Info-r16, ... } DL-PRS-Configuration-ID-r17 ::= SEQUENCE { nr-dl-prs-configuration-id-r17 INTEGER (1..maxOD-DL-PRS-Configs-r17), ... } -- ASN1STOP NR-DL-PRS-On-Demand-Configurations field descriptions. dl-prs-configuration-id This field provides an identity for the On-Demand-DL-PRS-Configuration.. nr-DL-PRS-PositioningFrequencyLayer This field, together with nr-DL-PRS-Info, provides the On-demand DL-PRS Configuration information. Only the following fields in IE NR-DL-PRS-PositioningFrequencyLayer are applicable: dl-PRS-ResourceBandwidth, dl-PRS-CombSizeN. The target device shall ignore the remaining fields in IE NR-DL-PRS-PositioningFrequencyLayer.. nr-DL-PRS-Info This field, together with nr-DL-PRS-PositioningFrequencyLayer, provides the On-demand DL-PRS Configuration information. Only the following fields in IE NR-DL-PRS-Info are applicable: DL-PRS periodicity in dl-PRS-Periodicity-and-ResourceSetSlotOffset, dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols, comb-size in dl-PRS-CombSizeN-AndReOffset, dl-PRS-QCL-Info. The target device shall ignore the remaining fields in IE NR-DL-PRS-Info..

The max value of dl-PRS-ResourceBandwidth is 273 PRBs. In this case, reusing the existing NR-On-Demand-DL-PRS-Configurations cannot request bandwidth aggregation with bandwidth larger than 272 PRB.

To support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations, we propose

- From RAN1 perspective, preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations through

  - Extent nr-dl-prs-configuration-id to associate a set of possible DL-PRS configurations for bandwidth aggregation PFLs may include

    - Larger Bandwidth per aggregation PFLs

    - Combsize N per aggregation PFLs

    - Periodicity, RepetitionFactor, NumSymbols, ReOffset, QCL-Info Per set

  - 3.3 MG VS PPW

Firstly, considering only MG-based requirements are specified based on WID, MG-based bandwidth aggregation should be supported first.

- MG-based bandwidth aggregation measurement is supported.

For PPW, the difference between PPW and MG is the UE only can measure the signal within BWP in PPW. Considering the following figure and WID, anyway, Tx and Rx UE have to use one RF chain and antenna to transmit or receive the signal. In this case, even in the PPW, Rx UE has received a wide-bandwidth signal since it should guarantee the received signal across PFLs with the same RF chain and antenna. But if we limited UE only receive the signal in the BWP, the received PRS signal outside BWP should be discarded. We think it an inefficient behavior to discard the useful signal, and cannot understand the motivation and benefit of PRS bandwidth aggregation measurement within PPW.

Figure 1:example figure about the PRS reception within BWP

But based on some offline discussions, some companies think the PPW can be used in large BWP cases for two carriers. For us, even in this case, it may be related to some relationship between communication and positioning. For example,

- Whether the multiple carriers are activated

- Whether the single RF chain is used for multiple carrier data reception, especially within PPW type 2

Considering the limited timeline and RAN4 scope, we prefer the joint measurement within PPW can be dropped in Rel-18.

- PPW-based bandwidth aggregation measurement can be dropped in Rel-18

The enhancement for UL bandwidth aggregation

  - 4.1 Configuration

TS 38.214 For intra-band and inter-band CA operations, a UE can simultaneously transmit more than one SRS resource configured by SRS-PosResource on different CCs, subject to UE's capability For intra-band and inter-band CA operations, a UE can simultaneously transmit more than one SRS resource configured by SRS-PosResource and SRS-Resource on different CCs, subject to UE's capability. Agreement For SRS bandwidth aggregation across two or three carriers, support enhancement of SRS configuration to indicate the SRS resources from which two or three carriers are linked - SRS resources are per BWP per carrier configuration - FFS whether the link is per SRS resource set basis or per SRS resource basis.

Firstly, since per BWP per carrier SRS configuration is supported, the existing SRS configuration can be reused for bandwidth aggregation. For the FFS issue, we share the same view as DL PRS, per set aggregation is preferred, otherwise, it may lead some gNB to measure the non-aggregated SRS, and some gNB measure the aggregated SRS if the link is resource basis.

- For SRS bandwidth aggregation across two or three carriers, the link is per resource set basis

And then, to realize SRS bandwidth aggregation, SRS resource set can be configured to associate the multiple cells or an indication of co-scheduling cells reusing the agreement in the agenda of multi-cell PUSCH/PDSCH scheduling.

Agreement For a set of cells which is configured for multi-cell scheduling using DCI format 0_X and DCI format 1_X, support the following: - If table defining combinations of co-scheduled cells for the set of cells is configured, - an indicator in the DCI is included and points to one row of the table. - The table is configured by RRC signaling for the set of cells. - Separate tables are configured for downlink scheduling and uplink scheduling - The size of the indicator is equal to ceil(log2(N)), where N is the number of rows in the table. - The max number of rows in the table is 16 - The size of the per-cell Type 2 fields for each co-scheduled cell does not change according to the indicated co-scheduled cell combination - The payload size of DCI format 1_X is derived by UE based on RRC configuration of the active BWP(s) of co-scheduled cell combinations within the set of cells. - The payload size of DCI format 1_X is the same for the active BWP(s) of all the co-scheduled cell combinations and equal to the largest payload size among the active BWP(s) of all the co-scheduled cell combinations determined by the co-scheduled cell combination table. - The payload size of DCI format 0_X is derived by UE based on RRC configuration of the active BWP(s) of co-scheduled cell combinations within the set of cells. - The payload size of DCI format 0_X is the same for the active BWP(s) of all the co-scheduled cell combinations and equal to the largest payload size among the active BWP(s) of all the co-scheduled cell combinations determined by the co-scheduled cell combination table. - Otherwise, - The UE determines the actually scheduled cell(s) based on the FDRA field of each cell of the set of cells. - For Type 0 FDRA, all 0s indicates the cell is not scheduled. - For Type 1 FDRA, all 1s indicates the cell is not scheduled. - The size of the Type 2 fields for each cell does not change according to actually co-scheduled cells. - The payload size of DCI format 0_X is derived by UE based on RRC configuration of the active BWP(s) of all cells within the set of cells. - The payload size of DCI format 1_X is derived by UE based on RRC configuration of the active BWP(s) of all cells within the set of cells.

- The Positioning SRS resource set configuration included

  - Cell group information(e.g., indication of co-scheduling cells reusing the agreement in the agenda of multi-cell PUSCH/PDSCH scheduling)

  - Multiple serving cell indices

    - The positioning SRS resource set ID under linked cells if the SRS resource set ID for aggregation is different.

Besides, due to multiple carriers (e.g., 4) can be configured in a band, whether multiple links can be configured for a carrier of SRS, and whether the linked carriers for aggregation SRS transmission can be updated based on the current activated carrier and active BWP in multiple carriers are issues to be solved. For example, band 1 includes 4 carriers {carrier 1 2 3 4}, the bandwidth aggregation of carriers can be {1,2},{2,3},{3,4}. So, whether UE can transmit SRS resource set 1 in carrier {1,2} at time 1 if carrier 1 and carrier 2 are activated or carrier 1 and carrier 2 are more suitable for positioning requirement, and transmit SRS resource set 1 in carrier {2,3} at time 2 if carrier 2 and carrier 3 are activated or carrier 2 and carrier 3 are more suitable for positioning requirement may be further discussed.

If multiple cells for bandwidth aggregation can be updated, the MAC CE can also be used to update or configure the linked carriers for periodic SRS.

- .

- To discuss whether the multiple cells for bandwidth aggregation can be updated for periodic SRS

  - If it can, the MAC CE can be used to update the association cells for periodic SRS transmission

  - 4.2 SRS activation

Agreement At least support periodic positioning SRS and semi-persistent positioning SRS for bandwidth aggregation - Support single MAC CE activating positioning SRS resource sets across the linked carriers - FFS whether support aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state. Study a single DCI scheduling positioning SRS across the linked carriers, and check whether the conclusion/agreements in agenda of multi-cell PUSCH/PDSCH scheduling with a single DCI can be reused - FFS MIMO SRS can be supported for bandwidth aggregation, e.g. with UE transparent way

Since the single MAC CE can be used for SRS resource sets across the linked carriers, the MAC CE can include Cell group information or multiple cells information, SRS resource set ID-related information, and Spatial relation for SRS resources. In addition, we would like to note that the spatial relation for SRS resources across carries should be the same and can be updated at the same time.

Figure 2: SP Positioning SRS Activation/Deactivation MAC CE

- For single MAC CE activate Positioning SRS resource set across carriers, the MAC CE includes

  - Cell group information or multiple cells information

  - SRS resource set ID

  - Spatial relation for SRS resource across multiple cells or cell group

In addition, PDSCH/PUSCH in multi-cells with a single DCI is supported in the CA agenda with a new DCI format.  The DCI can include the indication of co-scheduling cells or FDRA to indicate the co-scheduling cells. For us, it can schedule the SRS from multiple cells, but cannot guarantee only the SRS across cells is transmitted by a single RF. So, we would like to study the potential enhancement for the aperiodic positioning SRS first.

- Study the potential enhancement for the aperiodic positioning SRS before supporting aperiodic Positioning SRS across carriers

  - 4.3 Power control for SRS bandwidth aggregation transmission

Agreement Study potential power control enhancement of simultaneous transmission of SRS for SRS bandwidth aggregation especially in the case when the total uplink transmission power across multiple carriers exceeds P_c,max

For the power control, based on the current specification, the power will be adjusted based on the priority if the power across multiple carriers exceeds P_c,max. But based on the condition in the agreement of the last meeting(ie., the same Tx PSD (power per subcarrier)), we propose

- To enable SRS bandwidth aggregation in two or three carriers, the power allocation of different carriers should be adjusted based on the occupied PRB of SRS transmission if the power across multiple carriers exceeds P_c,max.

## 4.4 The drop rule for SRS bandwidth aggregation transmission

Similar to power control, if one carrier’s SRS is dropped, whether other carrier’s SRS can be transmitted is an issue. In Rel-17 intra CA and inter CA, whether the SRS transmission or not is only based on the current cell scheduling.

But for SRS bandwidth aggregation transmission, it may be related to the aggregating reception and searching. In this case, if single IFFT and SRS reception searching is used, dropping the SRS in all the carriers is more straightforward.

So, we propose

- For SRS bandwidth aggregation transmission, to discuss whether other cell’s  SRS  can be transmitted if SRS of one cell is dropped

  - 4.5 The relationship between UL communication CA and SRS bandwidth aggregation

Agreement Study the relationship between UL communication CA and SRS bandwidth aggregation, including - Whether to support the decoupling of the SRS bandwidth aggregation and the communication carrier aggregation for UE capabilities - Whether to support the configuration of SRS BW aggregation not limited by the allowed configuration of communication CA, i.e. SRS outside BWP and across carriers

In Rel-16, it restricts only configured SRS in carriers with PUSCH/PUCCH schedules. And if the SCell is deactivated, the SRS cannot be transmitted.

TS 38.214 The UE does not expect to be configured with SRS-PosResource on a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission. TS 38.321 1> if the SCell is deactivated: 2> not transmit SRS on the SCell; 2> not report CSI for the SCell; 2> not transmit on UL-SCH on the SCell; 2> not transmit on RACH on the SCell; 2> not monitor the PDCCH on the SCell; 2> not monitor the PDCCH for the SCell; 2> not transmit PUCCH on the SCell.

Figure 3:example figure about the activated and deactivated carrier

In this case, if carrier 3 is deactivated, whether UE can transmit SRS in carrier 1, carrier 2, and carrier 3 simultaneously? For us, we are not sure we can reserve resources of the non-activated cell for UE. In Rel-18, we prefer to restrict the transmission within the active BWP and activated cell.

But there is still a problem, if the positioning CA requirement is different from communication CA, whether only restrict SRS transmission within communication CA, or activate SCell first for SRS transmission. Both solutions have their pros and cons, which can be further discussed.

- The SRS transmission is within the active BWP and activated cell.

Conclusion

In this contribution, we discuss potential enhancement for bandwidth aggregation with the following proposals.

- To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following additional conditions should be satisfied for the aggregated PRS resources from a TRP

  - The same periodicity and offset, and slot offset

  - The same number of PRS resource

  - The frequency gap of PRS RE between PFLs is the multiple of comb size and SCS

    - How to maintain contiguous PRS pattern can be up to the implementation

- Phase/timing continuity between PFLs can be up to RAN4

- To enable SRS bandwidth aggregation between SRS in two or three carriers, the following additional conditions should be satisfied for the aggregated SRS resources

  - The same periodicity and offset, and slot offset

  - The same number of SRS resource

  - The frequency gap of SRS RE between carriers is the multiple of comb size and SCS

    - How to maintain contiguous SRS pattern can be up to the implementation

- Phase/timing continuity between carriers can be up to RAN4

- For PRS bandwidth aggregation across PFLs, the link is per resource set per TRP

- Adding linked  PFLs and PRS resource set information under PRS resource set configuration for set level PRS bandwidth aggregation across PFLs

  - linked PFL information(e.g ., PFL group information, or up two PFL indices, up two sets of frequency information (e.g.(dl-PRS-PointA, StartPRB, Bandwidth))

  - linked PRS resource set ID under linked PFL

- For PFL aggregation for the measurement reporting and location request information, support the following.

  - The location request information can include PFL aggregation request to indicate whether PFLs are requested to be aggregated for the PRS measurement

  - The location reporting information can include PFL aggregation indication to indicate which PFLs are aggregated for the PRS measurement

- PFL aggregation indication can be one of the following options in a measurement report

  - Option 1: association with up three PFL information(e.g., up three PFL indices, up three sets of frequency information (e.g.(dl-PRS-PointA, StartPRB, Bandwidth)

  - Option 2: association with a PFL group indication.

- Introduce an indicator to distinguish single FFT/IFFT or multiple FFT/IFFT operation for bandwidth aggregation measurement.

- From RAN1 perspective, for LMF-initiated request of on-demand DL PRS,  the following group of on-demand DL PRS parameters is defined and signaled for bandwidth aggregation case

  - per set of positioning frequency layer per FR

      - Number of DL PRS frequency layers

      - DL PRS Periodicity

      - DL PRS Resource Bandwidth

      - DL PRS Resource Repetition Factor

      - Number of DL PRS Resource Symbols per DL PRS Resource

      - DL-PRS CombSizeN

  - per set of positioning frequency layer per UE

- Start/end time of DL PRS transmission

Two options for indication of DL PRS QCL-Info, either

  - Option 1: per resource set per set of positioning frequency layer per FR

      - UE recommends a list of QCL sources

  - Option 2: per resource set per set of positioning frequency layer per FR

      - UE requests to provide the QCL information in the assistance data

- From RAN1 perspective, for UE-initiated request of on-demand DL PRS,  the following group of on-demand DL PRS parameters is defined and signalled for band aggregation case

  - per set of positioning frequency laye rper FR

      - Number of DL PRS frequency layers

      - DL PRS Periodicity

      - DL PRS Resource Bandwidth

      - DL PRS Resource Repetition Factor

      - Number of DL PRS Resource Symbols per DL PRS Resource

      - DL-PRS CombSizeN

Two options for indication of DL PRS QCL-Info, either

  - Option 1: per resource set per set of positioning frequency layer per FR

      - UE recommends a list of QCL sources

  - Option 2: per resource set pper set of positioning frequency layer per FR

      - UE requests to provide the QCL information in the assistance data

  - either per resource set per set of positioning frequency layer or per UE

      - Start/end time of DL PRS transmission

  - either per resource, or per resource set, or per UE

      - ON/OFF indicator (for LMF initiated request only)

- From RAN1 perspective, preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations through

  - Extent nr-dl-prs-configuration-id to associate a set of possible DL-PRS configurations for bandwidth aggregation PFLs may include

    - Larger Bandwidth per aggregation PFLs

    - Combsize N per aggregation PFLs

    - Periodicity, RepetitionFactor, NumSymbols, ReOffset, QCL-Info Per set

- MG-based bandwidth aggregation measurement is supported.

- PPW-based bandwidth aggregation measurement can be dropped in Rel-18

- For SRS bandwidth aggregation across two or three carriers, the link is per resource set basis

- The Positioning SRS resource set configuration included

  - Cell group information(e.g., indication of co-scheduling cells reusing the agreement in the agenda of multi-cell PUSCH/PDSCH scheduling)

  - Multiple serving cell indices

    - The positioning SRS resource set ID under linked cells if the SRS resource set ID for aggregation is different.

- .

- To discuss whether the multiple cells for bandwidth aggregation can be updated for periodic SRS

  - If it can, the MAC CE can be used to update the association cells for periodic SRS transmission

- For single MAC CE activate Positioning SRS resource set across carriers, the MAC CE includes

  - Cell group information or multiple cells information

  - SRS resource set ID

  - Spatial relation for SRS resource across multiple cells or cell group

- Study the potential enhancement for the aperiodic positioning SRS before supporting aperiodic Positioning SRS across carriers

- To enable SRS bandwidth aggregation in two or three carriers, the power allocation of different carriers should be adjusted based on the occupied PRB of SRS transmission if the power across multiple carriers exceeds P_c,max.

- For SRS bandwidth aggregation transmission, to discuss whether other cell’s  SRS  can be transmitted if SRS of one cell is dropped

- The SRS transmission is within the active BWP and activated cell.

# References

- RP-223549 New WID on Expanded and Improved NR Positioning. Dec.12-16, 2022

- 3GPP TS 38.321, “NR; Medium Access Control (MAC) protocol specification”.