3GPP TSG RAN WG1 #112bis-e                                                                                        R1-2302503

e-Meeting, April 17th - April 26th, 2023

Source:	vivo

Title:	Discussions on remaining issues of UE location verification in NR NTN

Agenda Item:	9.9.2

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the remaining issues related to the UE location verification in NTN.

# Discussions

In RAN #98-e meeting, objectives for network verified UE location have been discussed and updated as summarized in [1] according to the study in RAN1[2].

4.1.3 Network verified UE location Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied. Note 1: Enhancements assume reuse of the RAT dependent positioning framework Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity) Note 4 : Multiple satellite in view by the UE may be considered if time allows Note 5 : The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved Note 6 : The enhancements should take into account the mirror-image ambiguity Note 7 : Network verified UE location is an optional UE feature

In this contribution, we provide our views on RAT dependent positioning in NTN, including following aspects:

- Methods to be specified for RAT dependent positioning in NTN,

- Potential enhancements needed to support Multi-RTT method in NTN.

## 2.1	NTN positioning methods

In the evaluations of RAT positioning methods performed by companies in RAN1, only single satellite based positioning was assumed, as illustrated in Figure 1. Therefore, the normative work on positioning in NTN in Rel-18 should be based on single satellite.

Figure 1. RAT dependent positioning based on single satellite in NTN

There’re 2 methods, i.e. DL-TDOA and Multi-RTT, which are identified as candidates to be specified. RAN1 should discuss and determine whether only Multi-RTT method should be specified or both methods should be specified.

For DL-TDOA method, to support it with single satellite in NTN, there’s one concern from companies on the clock drift issue which is noted in the WID. As is already specified in section 10.1.23.2 of 38.133 v18.0.0, 32Tc (around16ns) is the maximum error for the measurement gap no larger than 160ms. And for measurement gap larger than 160ms, UE will be assumed to be able to adjust the timing since the SSB period is no larger than 160ms. Therefore, there’s no issue of clock drift for DL-TDoA method.

Technically, both methods can work with single satellite in view by UE. However, considering there’s limited TU and limited number of meetings left in RAN1 for Rel-18, it is preferred to have only one method specified. Given the majority would like to treat Multi-RTT method with higher priority as is also agreed in RAN plenary meeting, we would propose to only specify Multi-RTT method for RAT dependent positioning in NTN assuming single satellite in view by UE in NR Rel-18.

Proposal 1:

- Only specify Multi-RTT method for RAT dependent positioning in NTN assuming single satellite in view by UE in NR Rel-18.

## 2.2	Potential enhancements of Multi-RTT method in NTN

### Definition of Rx-Tx time difference

As is known, to calculate the RTT in TN for Multi-RTT method, both UE Rx-Tx time difference and gNB Rx-Tx time difference should be measured and reported to LMF. These 2 measurements are independent from each other as the UE Rx-Tx time difference definition is decoupled from SRS transmission and gNB Rx-Tx time difference definition is decoupled from PRS transmission. The value range of gNB/UE Rx-Tx time difference is [-0.5ms, 0.5ms] since a time unit of one subframe, i.e. 1ms, is applied for determining the TX time as is shown in Figure 2, and thereby the RTT calculated in TN is no larger than 1ms.

Figure 2. UE Rx-Tx time difference calculation in TN

For NTN, this mechanism can be reused in principle. However, RTT of NTN is much larger than TN. For example, as provided in table below excerpted from 3GPP TR 38.821, RTT of GEO transparent payload can be up to 541.46ms.

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35786 km |  | 600 km |  |  |  |
| Relative speed of Satellite with respect to earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 10° for feeder link |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 3500 km |  | 50 km / 1000 km |  |  |  |
| Maximum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 541.46 ms (Worst case) | 270.73 ms | 25.77 ms |  | 12.89 ms |  |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 477.48 ms | 238.74 ms | 8 ms |  | 4 ms |  |

Therefore, the range of Rx-Tx time difference is determined by the maximum RTT depending on the orbit and payload mode in NTN.

Observation 1:

- For supporting Multi-RTT method in NTN, the maximum RTT of NTN determines the range of UE Rx-Tx time difference.

In RAN1#112 meeting, following agreement has been made on which reference point to assume for calculating gNB Rx-Tx time difference, which should be discussed first in order to determine the definition of gNB Rx-Tx time difference.

Agreement - In NTN, for the position of the reference point for definition of gNB Rx – Tx time difference measurement, consider the following options: - Option 1: Onboard the satellite - Option 2: The uplink time synchronization reference point - Option 3: on the gNB

As is known, in TN, the reference point is on the gNB where uplink and downlink timing are aligned. In NTN, the uplink time synchronization reference point may be not in gNB. Therefore option 2 is a bit preferred. With option 2, the final distance between satellite and UE can be derived by RTT and the common TA offset which should be reported to LMF as well.

Proposal 2:

- Take uplink time synchronization reference point as the position of the reference point for definition of gNB Rx-Tx time difference.

In RAN1 #112 meeting, following agreement has been made for definition of UE Rx-Tx time difference, wherein the agreed options are expected to be down-selected.

Agreement Select one (or more) of the following options for enhancing UE Rx-Tx time difference in NTN - Option 1: The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX - Where: - UE Rx-Tx time difference is defined with respect to the Rx and Tx subframe timing associated with the TRP. - For a Transmission Point - TUE-RX is the UE received timing of downlink subframe #i from this Transmission Point (TP), defined by the first detected path in time. - TUE-TX is the UE transmit timing of the uplink subframe corresponding to subframe #i received from the TP - One or multiple DL RS for positioning, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the TP. - FFS: For a Transmission Point different from the serving cell (e.g. a DL-PRS-only TP) - Option 2: - For RTT measurement in NTN, support UE report that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS. - FFS: details of report and the definition of UE Rx-Tx time difference - Option 3: The legacy R17 definition of UE Rx-Tx time difference is adopted for NTN with an offset that is determined based on one of the following options: - Option 3-1: This offset is reported as the nearest integer value in the unit of milliseconds by rounding the time difference of transmit timing of uplink subframe #i and receive timing of downlink subframe#i - Option 3-2: UE report the index of the subframe j that is closest in time to the subframe #i received from the TP and LMF can derive the offset - Option 3-3: TA report which corresponds to the time difference of received timing of downlink subframe #i and transmit timing of uplink subframe#i rounding up to slot granularity. - Option 4: - UE Rx – Tx time difference TUE-RX – TUE-TX can be directly derived from timing advance TTA - FFS: the granularity and the reporting range of TA. - Note: This implies that the existing framework for Multi-RTT positioning report can be used without need to specify a new TA report. - Note: The impact of UE autonomous adjustment of TA (when applied) should be taken into account

Among these options, option 3-3 and option 4 depend on the UE TA value which may be not reliable as was discussed earlier, therefore these options should be precluded before companies can reach a common understanding on the reliability of TA report.

For option 3-2, it only forces UE to report TUE-TX defined in 38.215 and does not help LMF to derive the offset.

For option 2, it is simpler though it couples UL and DL which is different from the UE Rx-Tx time difference definition in TN.

For option 1 and option 3-1, they are actually both trying to derive a large time difference based on UL timing when PRS is received and decouples UL and DL. Option1 would provide a UE Rx-Tx time difference that is always positive since Tx timing is always earlier than Rx timing, and option 3-1 would provide a range of UE Rx-Tx time difference from X-0.5ms to X+0.5ms, where X is a positive integer value reflecting the integer number of milli-seconds of the TA when PRS is received. As is known, for TN, the range UE Rx-Tx time difference is within [-RTTmax2, RTTmax2] where RTTmax is equal to 1ms. Similarly, for NTN, the value range of UE Rx-Tx time difference may also be within a range from [-RTTmax2, RTTmax2] except that the RTTmax is the maximum RTT of NTN which is much larger than TN.

According to above, RAN1 should first discuss whether UL and DL should be decoupled for UE Rx-Tx time difference definition. If it is agreed to have DL and UL decoupled, RAN1 should discuss the allowed range of UE Rx-Tx time difference following same rule applied for TN before discussing how to define the UE Rx-Tx time difference.

Proposal 3:

- For definition of UE Rx-Tx time difference in NTN, RAN1 should first discuss following 2 aspects following same rules applied in TN:

  - whether DL and UL should be decoupled,

  - allowed range of UE Rx-Tx time difference.

Similar to discussions on UE Rx-Tx time difference definition, in RAN1 #112 meeting, following agreement has been made for definition of gNB Rx-Tx time difference.

Agreement Select one (or more) of the following options for the enhancement of gNB Rx-Tx time difference in NTN - Option 1: The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: For a Transmission Point - TgNB-RX is the Transmission and Reception Point (TRP) received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. - TgNB-TX is the TRP transmit timing of the downlink subframe corresponding to uplink subframe #i received from the UE - Multiple SRS resources can be used to determine the start of one subframe containing SRS. FFS: For a Transmission Point different from the serving cell (e.g. a DL-PRS-only TP) - Option 2: - For RTT measurement in NTN, support gNB report of gNB Rx-Tx time as defined in 38.215 with the following change: - Only the SRS resource starting within a subframe can be used to determine the start of the subframe. - Option 3: - Keep the current gNB Rx-Tx definition, and report an offset which can covers the time duration corresponds to kmac if needed. - Option 4: - For RTT measurement in NTN, support gNB report that indicates the time difference between the transmit time of a DL RS for positioning and the arrival time of an SRS. - FFS: details of report. Note: The impact of UE autonomous adjustment of TA (when applied) should be taken into account

First of all, when discussing options for defining gNB Rx-Tx time difference, each of the option should be associated to one option of defining UE Rx-Tx time difference since the final RTT calculation is the time difference between gNB Rx-Tx time difference and UE Rx-Tx time difference which should be derived with same mechanism.

Option 1 of gNB Rx-Tx time difference is associated to option 1 of UE Rx-Tx time difference, where the derived time difference values are actually the TA values on gNB side (at the time when SRS is received) and UE side (at the time when PRS is received) respectively. When the position of the reference point for definition of gNB Rx-Tx time difference measurement is the reference point where UL and DL are expected to be aligned, gNB Rx-Tx time difference should be zero and the UE Rx-Tx time difference should be RTT unless there’s some timing errors that can be corrected by close loop TA adjustment via TA commands transferred from gNB to UE.

Option 4 of gNB Rx-Tx time difference is associated to option 2 of UE Rx-Tx time difference, where the absolute Rx and Tx time are considered for time difference measurement.

Option 2 of gNB Rx-Tx time difference is not clear to us as its only change is “Only the SRS resource starting within a subframe can be used to determine the start of the subframe” which seems not able to derive a large RTT in NTN.

Option 3 of gNB Rx-Tx time difference is associated to option 3-1 of UE Rx-Tx time difference. If the position of the reference point for definition of gNB Rx-Tx time difference measurement is the reference point where UL and DL are aligned, the range of gNB Rx-Tx time difference should be close to zero given there could be some timing error and therefore the range of gNB Rx-Tx time difference could be the same as legacy.

According to above, following combinations can be further discussed in RAN1 on how to define Rx-Tx time difference on both gNB and UE side:

- {Option 1 of gNB Rx-Tx time difference, option 1 of UE Rx-Tx time difference},

- {Option 4 of gNB Rx-Tx time difference, option 2 of UE Rx-Tx time difference},

- {Option 3 of gNB Rx-Tx time difference, option 3-1 of UE Rx-Tx time difference}.

Proposal 4:

- RAN1 should further discuss following combinations of {gNB Rx-Tx time difference, UE Rx-Tx time difference} definition:

  - {Option 1 of gNB Rx-Tx time difference, option 1 of UE Rx-Tx time difference},

  - {Option 4 of gNB Rx-Tx time difference, option 2 of UE Rx-Tx time difference},

  - {Option 3 of gNB Rx-Tx time difference, option 3-1 of UE Rx-Tx time difference}.

### Triggering of UE location verification and Rx-Tx time difference report

In TN Multi-RTT method, UE can be triggered to report the measurements with following 3 options when a location request message is received.

- option 1: conditionally triggered (cell change),

- option 2: periodically triggered (multiple reports with an interval configured),

- option 3: based on the response time configured in the location request message.

For option 1, to support it in NTN, it can be up to LMF to decide when to send a location request message to trigger the measurement report based on the location of the single satellite in view of UE. However, there’s only one report triggered by cell change and only one report can be requested in one cell. Therefore, to support single satellite based positioning, multiple reports and the corresponding reporting intervals should be configured. In addition, the reporting condition may not have to be cell change for UE location verification. Generally, only when UE switches to the cells across borders of countries, the UE location verification is needed.

For option 2, each request message would trigger multiple reports and multiple reports are apart from each other with a configured reporting interval. This can be reused directly by single satellite based positioning. However, the reporting interval in TN (up to 64s) may have to be extended up to e.g. 1024s considering the potential maximum measurement interval required so that the single satellite is located far enough in different positions.

For option 3, the request message would trigger one report within the response time. Either multiple reports should be configured for NTN for each request message or multiple request messages can be assumed so that multiple reports can be derived when the single satellite is located in different positions. The response time configured for QoS may have to be increased considering the large RTT in NTN.

According to above, all the TN triggering methods can be applied for supporting Multi-RTT method in NTN in principle but some updates are needed considering the single satellite based positioning requirements and the large RTT in NTN. Therefore, we have following observation and proposal.

Observation 2:

- All the TN triggering methods can be applied for supporting Multi-RTT method in NTN in principle but some updates are needed considering the single satellite based positioning requirements and the large RTT in NTN.

Proposal 5:

- For supporting Multi-RTT method in NTN, RAN1 to discuss the necessary updates to each measurement report triggering methods in TN.

For Multi-RTT in TN, the UE may be configured to report quality metrics NR-TimingQuality corresponding to the UE Rx-Tx time difference measurements which include the following fields:

- timingQualityValue which provides the best estimate of the uncertainty of the measurement

- timingQualityResolution which specifies the resolution levels used in the timingQualityValue field.

This report can be reused by NTN. However, as agreed in RAN1, the accuracy of UE location verification in NTN is up to 5 ~ 10km. This means the timing quality resolution should be further extended to meet the larger range. The current timing quality value is within [0, 31] and resolution is within [1, 30] meters, which means it supports a timing quality in maximum corresponds to 31x30 meters, i.e., less than 1km. Therefore, the timing quality resolution can be extended to hundreds of meters so that up to 10km quality report can be supported.

Proposal 6:

- For supporting Multi-RTT method in NTN, extend the timing quality resolution up to hundreds of meters so that up to 10km quality can be supported.

### Uplink timing error mitigation for Multi-RTT positioning

In NTN, more factors would affect the timing measurement and thereby the accuracy of the positioning finally calculated. For example, as specified in 38.133 v18.0.0, for each PRS transmission for UE Rx-Tx time difference calculation, the corresponding SRS transmissions that can be used for gNB Rx-Tx time difference for final RTT calculation should be completed within a time window of [-160, 160] ms.

[38.133] The requirements in clause 9.9.4 apply for periodic and triggered UE Rx-Tx time difference measurements, provided: - UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25 are met for a corresponding band. - SRS is configured on at least one of the PCell, PSCell and SCell. - The UE transmits SRS within [-160, 160] msec of at least one DL PRS resource of each of the TRPs in the assistance data.

During this time window in NTN, other than the TAC received from gNB which may also happen in TN, there could be satellite ephemeris updates and thereby common or UE specific TA offset updates meaning that it’s more probable that different TA values may be applied for each SRS transmissions.

In another aspect, in NTN same or different polarizations may be applied on the transmitter and receiver side for transmitting and receiving positioning reference signals. The signal quality may be different and thereby the timing quality may also be different.

Therefore, discussions on whether and how to mitigate the timing error may be more necessary in RAT dependent positioning in NTN compared to TN.

Proposal 7:

- For supporting Multi-RTT method in NTN, RAN1 to discuss whether and how to mitigate the uplink timing error.

### Mirror position ambiguity for Multi-RTT positioning

Regarding the mirror position ambiguity for Multi-RTT positioning in NTN, following agreement has been made. In our understanding, the mirror issue can be solved by gNB implementation e.g. via beam management to roughly estimate the raw UE location that can preclude the mirror position. Therefore, option 1 is preferred and no spec. impact is expected.

Agreement Study the following options to resolve the mirror positions ambiguity for multi-RTT positioning: - Option 1: gNB or LMF implementation to solve the mirror error issue. - FFS: whether there is spec impact - Option 2: Reuse existing ECID method (e.g. combine UE neighbor measurements to solve the ambiguity between mirror positions), with potential enhancements - Option 3: NR NTN UE should report the Doppler calculated on the service link - Option 4: a VSAT UE should report its beam pointing in respect to satellite beam line of sight - Option 5: Reporting of cell coverage information (e.g. cell footprint and reference point, or antenna pattern) to the LMF - Option 6: Support and potentially enhance the optional Rel-17 UL-AoA measurements defined for multi-RTT positioningOther solutions are not precluded

Proposal 8:

- Mirror position ambiguity issue in NTN UE location verification can be solved by gNB implementation, and no spec. impact is expected.

# Conclusion

In this contribution, we provide our views on the potential enhancements needed to support Multi-RTT positioning for UE location verification in NTN. According to the discussions, we have following observations and proposals:

Observations 1 to 2:

- For supporting Multi-RTT method in NTN, the maximum RTT of NTN determines the range of UE Rx-Tx time difference.

- All the TN triggering methods can be applied for supporting Multi-RTT method in NTN in principle but some updates are needed considering the single satellite based positioning requirements and the large RTT in NTN.

Proposals 1 to 8:

- Only specify Multi-RTT method for RAT dependent positioning in NTN assuming single satellite in view by UE in NR Rel-18.

- Take uplink time synchronization reference point as the position of the reference point for definition of gNB Rx-Tx time difference.

- For definition of UE Rx-Tx time difference in NTN, RAN1 should first discuss following 2 aspects following same rules applied in TN:

  - whether DL and UL should be decoupled,

  - allowed range of UE Rx-Tx time difference.

- RAN1 should further discuss following combinations of {gNB Rx-Tx time difference, UE Rx-Tx time difference} definition:

  - {Option 1 of gNB Rx-Tx time difference, option 1 of UE Rx-Tx time difference},

  - {Option 4 of gNB Rx-Tx time difference, option 2 of UE Rx-Tx time difference},

  - {Option 3 of gNB Rx-Tx time difference, option 3-1 of UE Rx-Tx time difference}.

- For supporting Multi-RTT method in NTN, RAN1 to discuss the necessary updates to each measurement report triggering methods in TN.

- For supporting Multi-RTT method in NTN, extend the timing quality resolution up to hundreds of meters so that up to 10km quality can be supported.

- For supporting Multi-RTT method in NTN, RAN1 to discuss whether and how to mitigate the uplink timing error.

- Mirror position ambiguity issue in NTN UE location verification can be solved by gNB implementation, and no spec. impact is expected.

# References

- RP-223534, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, 3GPP TSG RAN #98-e meeting, December 12-16, 2022.

- RP-221820, New SID: Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR, 3GPP TSG RAN #96-e meeting, June 6-9, 2022.