3GPP TSG RAN WG1 #112bis-e		R1-2302508

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:	Discussion on physical signal and procedure for low power WUS

Agenda Item:	9.11.3

Document for:	Discussion and Decision

- Introduction

In RAN1#112 meeting, physical signal design and procedure for low power WUS was discussed, and some agreements were made [1]. In this contribution, we will provide our further considerations on the physical channel design, and potential functionalities and procedures for LP-WUS.

- Physical signal Design

  - Waveform/Modulation

    - OOK waveform

Agreement For MC-ASK waveform generation, where K is size of iFFT of CP-OFDMA, N is number of SCs used by LP-WUS including potential guard-bands, study further - Option OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are - OOK=1 means all SCs are modulated - OOK=0 means all SCs are zero power (from base-band point of view) - Option OOK-2: Parallel M-bit OOK in frequency domain, - N SCs of LP-WUS is further separated into M segments (M=2 in Figure) possibly with guard-bands in-between and/or around - OOK=1 means all SCs in segment are modulated - OOK=0 means all SCs in segment are zero power (from base-band point of view) - FFS architecture. - Option OOK-3: Multi-tone single-bit OOK - N SCs of LP-WUS is separated into L segments (L=2 on Figure) without guard-bands in-between segment, but possibly around - OOK=1 means 1 sub-carrier (known by UE) of each segment is modulated, rest of SC is zero power (from base-band point of view) - OOK=0 means all SCs in all segments are zero power (from base-band point of view) - FFS architecture - Option OOK-4: Transform M-bit OOK in time domain - N SCs of OOK-1 are generated by a transformation (DFT/Least square) - N’ samples are generated from M-bits - signal modification may or may NOT be used - truncation or other additional modification may or may NOT be used, if not used, N is the same as N’ - N’ can be the same as K - FFS modulated SCs are e.g. QAM symbols, sequences or other signals - Companies to report their assumptions - potential guard-band SCs are zero power (from base-band point of view) - [optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted] Other options are not precluded (e.g. OOK-1 with multiple bits in one OFDM symbol)

- Option OOK-1: Single-bit in 1 OFDM symbol

For option OOK-1, one on/off chip is mapped to one OFDM symbol. A predefined sequence, e.g., ZC sequence, can be mapped to REs to modulate ‘on’ chip in time domain, and zeros are mapped to the REs to modulate ‘off’ chip. The SCS of an OFDM symbol used for LP-WUS generation can be the same as the SCS used for other NR transmissions in OFDM symbol overlapping in time. Hence, LP-WUS can well co-exist with legacy NR DL channels, and the impact on existing NR OFDM generator is marginal. However, the chip rate of the OOK signal in option OOK-1, highly depends on the SCS of an OFDM symbol used for LP-WUS generation. For example, if the SCS is 30kHz, the chip rate of the OOK signal is less than 28k chips/sec. The data rate of LP-WUS is even lower if additional overhead (e.g., Manchester code, CRC bits, and etc.), is considered.

Observation 1: For option OOK-1, the data rate/chip rate highly depends on the SCS of an OFDM symbol used for LP-WUS generation.

In 802.11ba [4], the OOK signal is generated by mapping sequences on a set of REs in comb pattern in frequency domain. The time domain signal is repeated within an OFDM symbol. Multiple on/off chips can be generated by setting part of the time domain repetitions to zero. In 802.11ba, the wake-up signal is not designed to be FDMed with other DL signals, erasing part of the time domain signal would not lead to any co-existence issues. However, multiplexing of LP-WUS and legacy NR DL signals in FDM manner should be supported for better resource efficiency in cellular system. Erasing the time domain signal is not feasible.

Alternatively, to increase the data/chip rate of OOK, higher SCS can be used to generated shorter on/off chips. As shown in Figure 1, additional IFFT is introduced to generate the OOK waveform, and the time domain signal of the OOK signal is added with that for the legacy DL channels. Thus, the LP-WUS can be FDMed multiplexing with NR signals. However, Additional IFFT is required.

Figure 1. Generation of MC-OOK waveform for high chip rate using additional IFFT module

Observation 2: For OOK-1, new IFFT branch corresponding to high SCS is needed to achieve higher data/chip rate.

For option OOK-1, the spectrum can be uniformly distributed in the allocate resources, e.g., when ZC sequence is mapped in frequency domain to create on pulse.

Observation 3: For OOK-1, uniformly distributed frequency spectrum density can be achieved.

- Option OOK-2: Parallel M-bit OOK in frequency domain

For option OOK-2, OOK waveform are generated by multiple parallel generation branches. In each branch of OOK-2, the OOK waveform can be either generated by OOK-1 and OOK-4. Hence, OOK-2 provide additional degrees of freedom to exploit more frequency resources in our understanding. The multiple branches can be either used to achieve high data rate, or to achieve frequency diversity. To achieve the benefits from OOK-2, multiple parallel receiver branches is needed to detect the parallel waveforms simultaneously.

Observation 4: OOK-2 provides additional degrees of freedom to exploit more frequency resources, the parallel transmissions can be used to achieve either high data rate or frequency diversity.

- Option OOK-3: Multi-tone single-bit OOK

For option OOK-3, single sub-carrier of each segment is used for one UE, providing potential frequency diversity and enabling multiple LP-WUS multiplexed among UEs. However, at receiver side, it requires to subtract the signal carried by single sub-carrier of each segment, which requires signal processing in frequency domain. When there exists frequency error, i.e., causing a frequency shift larger than the spacing between two adjacent sub-carriers in one segment, it will cause detection error by subtracting another sub-carrier for detection mistakenly.

Observation 5: Option OOK-3 detection is susceptible to frequency error.

Further, as single sub-carrier of each segment is used, the total transmission power allocated for transmission maybe limited. Hence, the reliability of option OOK-3 should be carefully considered. Besides, the waveform generated by single sub-carrier transmission within certain frequency range may violate the RAN4 requirements for RE dynamic range as given in Table 1. When some of the subcarriers are left empty, the empty RE power to average RE power will exceed the lower bound of dynamic range. Hence, it is not clear how to maintain such requirement in option OOK-3.

Observation 6: It is not clear how to fulfill RAN4 requirement on RE dynamic range at gNB side in option OOK-3.

Table 1 RE power control dynamic range (in TS38.104, Table 6.3.2.2-1)

| Modulation scheme used | RE power control dynamic range (dB) |  |
| --- | --- | --- |
| on the RE | (down) | (up) |
| QPSK (PDCCH) | -6 | +4 |
| QPSK (PDSCH) | -6 | +3 |
| 16QAM (PDSCH) | -3 | +3 |
| 64QAM (PDSCH) | 0 | 0 |
| 256QAM (PDSCH) | 0 | 0 |
| NOTE: The output power per carrier shall always be less or equal to the maximum output power of the base station. |  |  |

- Option OOK-4: Transform M-bit OOK in time domain

Regarding the block ‘signal generation and modification’ in option OOK-4, signal modification can be applied for the purposes below

- Reduce spectral line effect and make PSD of LP-WUS flatter

Due to spectral line effect, the power spectrum density of LP-WUS in frequency domain is centralized in a super narrower bandwidth as shown in Figure 2, which is quite sensitive to the frequency selectivity. In order to solve this problem, one solution is to scramble the phase of the time domain signal before DFT. As shown in Figure 3, the signal power is more evenly distributed within the allocated bandwidth after phase scrambling, and the RE dynamic range is between {-3.42, 1.41} dB, which fulfills emission requirements for existing NR DL channels.

- Inter-cell interference mitigation

For OOK detection, signal spreading and de-spreading can be applied for inter-cell interference rejection. A spreading code can be multiplied to the LP-WUS in time domain, and the spreading code with a frequency shift is also transmitted together for reference, i.e., the input signal of the subsequent block ‘DFT/least square’ is an addition of a spreading code with a frequency shift and LP-WUS multiplied with the same spreading code. At the receiver side, LP-WUS is equivalently de-spread after envelop detector by having the received reference spreading code multiplied to the received spread LP-WUS, while the inter-cell interference is mitigated via spread by the received reference spreading code [8].

Observation 7: Regarding the block ‘signal generation and modification’ in option OOK-4, signal modification can be used for making PSD of LP-WUS flatter and mitigating inter-cell interference.

Figure 2. PSD and time domain waveform of MC-OOK signal generated through opt OOK-4

Figure 3. PSD and waveform of MC-OOK signal generated through OOK-4

    - LP-WUS waveform impacted by frequency error

In order to investigate the impact on performance due to frequency error, option OOK-4 waveform with envelop detector and sequence-based modulation waveform with OFDM receiver are simulated with the evaluation assumptions listed in Appendix A Table 1 with config1 and Appendix A Table 2, respectively. The performance curves are depicted in Figure 4(a) and Figure 4(b).

(a) option OOK-4 waveform               (b) sequence-based modulation waveform

Figure 4. Performance comparison between option OOK-4 and sequence-based modulation

According to the simulation results, it is observed that performance degradation of sequence-based modulation waveform is significant even with the 5ppm frequency error, which is much lower than 50ppm and 200ppm assumed in option OOK-4 waveform simulation. Therefore, sequence-based modulation with OFDM receiver is more sensitive to frequency error, than option OOK-4 waveform with envelop detector.

Observation 8: For sequence-based OFDM signal, e.g., ZC sequence, the detection performance is more sensitive to frequency error compared with OOK waveform.

  - LP-WUS bandwidth

Following agreements were made on LP-WUS bandwidth.

Agreement For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further - whether BW of LP-WUS is configurable (implicitly or explicitly) - size of guard band [FFS: within or outside of BW X], if any - whether there is different X for Idle, Connected, Inactive modes FFS: Whether FR2 is included in the scope of LP-WUS SI

- Whether BW of LP-WUS is configurable

In last meeting, the question whether the bandwidth of LP-WUS is configurable was raised. According to discussion in AI 9.13.2, BPF is needed in homodyne/zero-IF architecture with baseband envelope detection and heterodyne architecture with IF envelope detection, to exclude the interference from adjacent NR signals. In our understanding, different bandwidth may imply different components in WUR to adapt each bandwidth. If the BW of LP-WUS is configurable, BPFs with different bandwidth should be implemented to better reject the interference, but resulting in higher receiver complexity.

Observation 9: If the BW of LP-WUS is configurable, BPFs with different bandwidth should be implemented to better reject the interference, causing higher receiver complexity.

- Size of guard gap

When LP-WUS is deployed in a band same as the UE’s NR band, it may suffer from three types of interferences, interference from adjacent channels, interference from adjacent subcarriers occupied by legacy NR signals or other LP-WUS, as well as inter-cell interference. For interference from adjacent subcarriers, it comes out as the orthogonality among subcarriers may not be guaranteed due to non-coherent detection of LP-WUR, i.e., for OOK and FSK detection. For the interference from adjacent channel or adjacent subcarriers, filters as well as a guard gap between LP-WUS signal and adjacent subcarriers/channels can be applied for interference rejection. Considering the guard gap is mainly adopted for LP-WUS for interference rejection, it is better to include the guard gap within the BW of LP-WUS.

Furthermore, depending on the receiver requirements under discussion in RAN 4 for LP-WUR in terms of interference rejection from adjacent channels and adjacent subcarriers, proper filter designs and the guard gap can be determined by considering of both the implementation complexity and the resource efficiency. Therefore, it is better not to determine the size of guard gap in RAN1 and leave it to RAN 4 as it highly depends on receiver requirements on interference rejection and filter characteristics.

Proposal 1: Guard gap is included within LP-WUS bandwidth.

Proposal 2: The size of guard gap is not determined in RAN1.

- Whether different LP-WUS bandwidth for different RRC states

LP-WUS can be used for different RRC states to achieve benefits for under different traffic. As discussed previously, supporting configurable(variable) channel bandwidth may imply higher receiver complexity, that is BPFs with different bandwidth should be implemented to better reject the interference. Hence, different bandwidth configuration for different RRC states is not preferred.

Observation 10: If the BW of LP-WUS can be different for each RRC states, BPFs with different bandwidth should be implemented to better reject the interference, causing higher WUR complexity.

Proposal 3: The same BW of LP-WUS should be designed for different RRC states for simple implementation

  - Signal/channel structure for LP-WUS and LP-SS

LP-WUS is expected to be able to wake up an individual UE, a subgroup of UE or all the UEs depending on wake-up purpose. Thus, the wake-up indication, i.e, the identification information of the target UE(s) should be carried. Besides, some information, other than wake-up indication, can also be carried in LP-WUS, e.g., necessary information to be handled in main radio but carried by LP-WUS and agnostic to LP-WUR, which facilitate main radio’s reception, e.g., system information modification. Such payload information can be carried by sequence(s) selection or encoded bits with CRC or both of them. The suitable channel structure of LP-WUS may vary when different number of payload information bits are delivered. Further, the channel structure of LP-WUS also depends on the synchronization requirements. If LP-WUR takes a low accuracy clock to keep lower power consumption, it may not keep synchronization to the TX side very precisely, and thus, at least a preamble should be included in LP-WUS channel structure, which can facilitate LP-WUR to reliably determine the start timing for the payload part. On the other hand, if LP-WUR can keep synchronization to the TX side very well, a preamble may not be needed. Therefore, the following structure of LP-WUS can be studied as starting point:

- Alt 1: WUS payload only

- Alt 1a: WUS payload information is carried by sequence(s) selection

- Alt 1b: WUS payload information is carried by encoded bits (with CRC)

- Alt 2: a preamble (for sync) followed by WUS payload

- Alt 2a: WUS payload information is carried by sequence(s) selection

- Alt 2b: WUS payload information is carried by encoded bits (with CRC)

Proposal 4: Study LP-WUS signal structure including:

- Alt 1: WUS payload only

- Alt 1a: WUS payload information is carried by sequence(s) selection

- Alt 1b: WUS payload information is carried by encoded bits (with CRC)

- Alt 2: a preamble (for sync) followed by WUS payload

- Alt 2a: WUS payload information is carried by sequence(s) selection

- Alt 2b: WUS payload information is carried by encoded bits (with CRC)

LP-WUS is transmitted on demand for waking up purpose. When a UE doesn’t detect a LP-WUS successfully for a period, it may not determine whether WUS is not transmitted, or the channel quality of low power link is poor for reception. Therefore, a reference signal transmitted periodically can be considered for RRM purpose. Furthermore, such signal can be also used to provide timing for LP-WUS reception. Therefore, a periodic signal at least for RRM and sync purpose can be also studied, e.g., low-power synchronization signal (LP-SS). The following structure of LP-SS can be studied as starting point:

- Alt 1: sequence(s)

- Alt 2: sequence(s) followed by additional message

- Alt 2a: additional message is carried by sequence(s) selection

- Alt 2b: additional message is carried by encoded bits (with CRC)

Proposal 5: Study LP-SS signal structure including:

- Alt 1: sequence(s)

- Alt 2: sequence(s) followed by additional message

- Alt 2a: additional message is carried by sequence(s) selection

- Alt 2b: additional message is carried by encoded bits (with CRC)

  - Sequence Length for LP-WUS structure with preamble

For sequence detection, e.g., the preamble sequence of the LP-WUS for the purpose of sync or identification, it is always beneficial to have a longer OOK sequence with the overhead increasing. With the support of better detection accuracy, e.g., multiple-level ADC, it is expected that the OOK sequence can suffer from the interference.

(a)  Config1-56kbps chiprate        		 (b) 	Config1-112kbps chiprate

Figure 5. Performance of LP-WUS with different preamble length

Observation 11: For LP-WUS structure with preamble using OOK waveform, the preamble with at least 16 chips can achieve reliable synchronization performance.

The preamble sequence may be transmitted across multiple OFDM symbols, with CP part every M chips, where M is the number of segments within one OFDM symbol. To avoid impacted by the CP parts when performing synchronization at WUR, the local sequence used for correlation with the received signal, can be padded with a number of zeros samples corresponding to the CP length. Thus, the impact of CP part in timing search can be eliminated.

  - LLS simulation methodologies and details

- Whether to align parameter for different schemes

The followings are agreed and some are for further discussion.

Agreement - When evaluating and/or comparing link performance of MC-ASK, MC-FSK, and CP-OFDMA waveforms of LP-WUS at least - raw information bit-size - [time/frequency resources (including any guard bands), if applicable] - [total energy of LP-WUS across the time/frequency resources] - FFS: false alarm probability/rate - FFS: misdetection probability/rate - are kept [comparable or fixed]. - Study at least - impact of timing error - impact of frequency error - impact of phase noise and I/Q imbalance, if applicable - impact of ADC resolution and sampling rate - impact of interference - impact of delay spread - impact of doppler spread - Companies to report - how they modelled SINR - time/frequency resources (including any guard bands) for the scheme - false alarm probability/rate and misdetection probability/rate - power consumption of the MR if false alarm probability/rate not fixed across MC-ASK, MC-FSK, and CP-OFDMA waveforms - When comparing waveforms of LP-WUS, consider the impact to gNB for each of the waveform generation schemes. Consider whether there is impact to PAPR and a need for additional hardware for WUS.

For different waveform types, it is not possible to assume the same time-frequency resources considering their designs can be quite different. Keeping T-F resources as the same for different designs within one waveform type is reasonable. Companies can report the T-F resources they assumed to be used.

Meanwhile, considering their T-F resources can be different, the total energy can be different for different waveform types. Instead, keeping the EPRE as the same for different waveforms is more reasonable.

Proposal 6:

  - When evaluating and/or comparing link performance of MC-ASK, MC-FSK, and CP-OFDMA waveforms of LP-WUS at least

    - raw information bit-size

    - [time/frequency resources (including any guard bands), if applicable]

    - [total energyEPRE of LP-WUS across the time/frequency resources]

    - FFS: false alarm probability/rate

    - FFS: misdetection probability/rate

    - are kept [comparable or fixed].

Companies to report the time/frequency resources (including any guard bands), if applicable

- How to model SINR

In LLS for NR signal/channels, the SINR is defined per RE level, i.e., per RE signal power divided by per RE interference and noise power. Similarly, for LP-WUS reusing OFDM generator, the SINR can be defined per RE level.

For OOK-1, each OFDM symbol is used to create one on/off chip, assuming on and off chips are equally distributed, e.g., when Manchester code is used, the power allocated to on pulse can be doubled on the average RE power to achieve certain average SINR.

For OOK-4, multiple on/off chips are transmitted within each OFDM symbol, and different combinations of on/off chips can be generated by using the same RE resources. If on/off chips are equally distributed within one OFDM symbol, e.g., when Manchester code is used, the amplitude/power for on chips is consistent across OFDM symbols, the SINR in each OFDM symbol is equivalent to average SINR.

However, if the number of on chip and that of off chip is not equal in each OFDM symbol for option OOK-4, e.g., 3 on chips with 1 off chip, or 1 on chip with 3 off chips within one OFDM symbol, the amplitude/power for on chip is not consistent across OFDM symbols if the transmission power for each OFDM symbol is consistent, additional scaling on each OFDM symbol is needed to align the amplitude/power for on chips. Hence, for option OOK-4, in which multiple on/off chips are transmitted within one OFDM symbol, equally distributed on/off chips within one OFDM symbol is preferred.

Observation 12: For LP-WUS generated by reusing OFDM generator, the SINR can be defined per RE level, as that for OFDM based signal/channels.

Observation 13: For option OOK-1, assuming ‘on’ and ‘off’ chips are equally distributed, the power allocated to ‘on’ chip can be doubled on the average RE power to achieve certain average SINR.

Observation 14: For option OOK-4, if ‘on’ and ‘off’ chips are equally distributed within one OFDM symbol, the amplitude/power for ‘on’ chip is consistent across OFDM symbols, the SINR in each OFDM symbol is equivalent to average SINR.

Observation 15: For option OOK-4, if ‘on’ and ‘off’ chips are not equally distributed within one OFDM symbol, the amplitude/power for ‘on’ chip is not consistent across OFDM symbols, additional scaling on each OFDM symbol is needed to align the amplitude/power for ‘on’ chip.

  - Adaptive scalable sampling rate

The increased LP-WUS sampling rate could lead to higher power consumption to ADC, digital processing and etc. These aspects contribute as one of the main power consuming components. Hence, it is always beneficial to lower down the sampling rate. However, without enough feedback to the gNB, it is not easy to let the gNB indicate UE the received signal sampling rate. A more sophisticated way is to let the LP-WUS receiver decide which sampling rate is enough to decode. This can be based on the aforementioned ‘LP-SS’ signal to provide the ability to let the LP-WUS receiver autonomously detect the appropriate sampling rate, which can be then used for detecting LP-WUS preamble and message(s) part.

For example, the LP-WUS receiver can under sampling the ‘LP-SS’ sequence to save power. However, such operation requires the sequence for the ‘LP-SS’ signal having special characteristics, i.e., for different under sampling rate, the sequences of length s for which the autocorrelations are collectively as small as possible.

Proposal 7: Study LP-WUS receiver autonomously determine the sampling rate, which can be beneficial from reducing power consumption perspective.

- Physical functionalities and procedures

  - DRX reception of LP-WUS for RRC idle/inactive mode

Agreement Study further pros and cons of the following monitoring behaviors of LP-WUR - Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states - Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time

Monitoring behaviours of LP-WUR can be duty cycle or continuous monitoring. For duty cycle monitoring , LP-WUR switches between ON/OFF states, and the LP-WUS or LP-SS can only be received when LP-WUR is ON state. In this case, the false alarm rate and power consumption will decrease as the number of monitoring occasions of LP-WUS/LP-SS decreases. The power saving gain and latency varies with various duty cycle ratio. There is a trade-off between power saving gain and latency. For continuous monitoring, there is more flexibility to send the LP-WUS or LP-SS at any moment. The false alarm rate and the power consumption will increase as the number of monitoring occasions of LP-WUS/LP-SS increases, but latency will decrease. To fulfil FAR target with reliable MDR performance, the more resource for LP-WUS is required, e.g., longer sequences or CRC attachments. The detailed discussion can be found in our companion paper [3].

Observation 16: For duty cycle LP-WUS monitoring

- Pros: better MDR performance under certain FAR and higher power saving gain

- Cons: less NW flexibility and larger latency

Observation 17: Following pros and cons are observed for continuous LP-WUS monitoring

- Pros: higher NW flexibility and lower latency

- Cons: degraded MDR performance under certain FAR, which relies on signal/channel design, especially for long DRX cycle and non-ideal timing.

  - Measurements/synchronizations on WUR for RRC idle/inactive mode

In previous meeting, the following agreements are made on RRM measurements.

Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighboring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE) Agreement Study potential measurement metric used for RRM measurements performed by LP-WUR. - examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal - companies to report assumption of signal used for measurements

In this section, we will further discuss the details on RRM measurements performed by LP-WUR.

    - RRM mechanisms to reduce power consumption

In previous meeting, reducing power consumption in RRM measurements was discussed, since measurement on SSBs should be performed by main radio, and it becomes the bottleneck in power saving after LP-WUR is introduced.

Some enhancements on RRM measurements are proposed to reduce the power consumption on RRM measurements, including:

- Alt 1: LP-WUR performs LP-RRM,

  - Alt 1a: MR performs relaxed RRM for serving and possibly neighboring cells

  - Alt 1b: MR does not perform RRM

- Alt 2: LP-WUR does not perform measurement,

  - MR performs relaxed RRM for serving and neighboring cells

If the RRM measurements are totally offloaded to LP-WUR, the most power saving gain can be achieved. When the link quality of LP-SS gets worse, the MR can be used to perform RRM measurements. The RRM measurements on the main radio performed in relaxed periodicity, together with measurements in WUR, to assist the channel quality determination. According to our evaluation [3], 88%~93% additional power saving gain can be achieved when RRM measurement is totally offloaded to WUR, compared to RRM performed in main radio but in a relaxed manner, together with WUR RRM measurements.

Proposal 8: It is beneficial to support RRM measurements based on LP-WUR, and the following enhancements can be considered.

- RRM measurements are totally offloaded to LP-WUR.

- LP-WUR performs RRM measurements, and MR performs relaxed RRM measurements.

    - Unified LP-SS for synchronization and measurement

In the last meeting, it is agreed that to study a synchronization signal used by LP-WUR.

Agreement Study synchronization signal used by LP-WUR, if needed, based on - Option 1: aperiodic signal transmitted as part of LP-WUS - FFS: Whether the signal can additionally be transmitted separately from LP-WUS - Option 2: periodic signal transmitted separately from LP-WUS - Option 3: Option1 + Option2

At the same, it is agreed to study measurement metrics performed by LP-WUR such as signal quality, signal power, detection rate of LP-WUS/synch signal. It can be achieved by a low power synchronization signal (LP-SS).

For the above two purpose (i.e., synchronization and measurement), a unified low power synchronization signal (LP-SS) performed by LP-WUR can be considered to achieve this. It has the following benefits,

- One low power synchronization signal (LP-SS) for both synchronization and measurement can save network overhead.

- Less power consumption for MR measurements

- For IDLE/INACTIVE, it can even measure more frequently than i-DRX measurement, so the reachability/mobility can be assured. However, the measurement accuracy needs to be FFS.

At least for envelope detector, the low power synchronization signal(LP-SS) is quite different from the legacy signals/channels.

Observation 18: Design for a unified LP-SS performed by LP-WUR for both synchronization and measurement is beneficial.

Proposal 9: Study suitable transmission periodicity of the LP-SS performed by LP-WUR for the following cases,

- LP-SS case 1: LP-SS is used for synchronization and measurement purpose

- LP-SS case 2: LP-SS is used for measurement purpose

    - Measurement quantity for RRM measure on WUR

For non-coherent detection via envelop detector, the signal power and the interference & noise power can be estimated, according to [7]. Various SNR estimation techniques have been discussed in the literature and it is observed that the bit error rate (BER) performance of those estimation methods is well aligned with the analytically evaluated BER. For illustration, a simplified OOK receiver model with SNR estimation is used as shown in Figure 6.

Figure 6. OOK receiver model with SNR estimation

Generally, SNR estimation techniques can be classified into data aided (DA) and non-data aided (NDA) schemes [7]. In this contribution, we mainly discuss DA estimation based on M1V estimator. Take the signal structure shown in Figure 7 as an example for illustration, the total packet length is N bits, where the first K bits are the preamble and the next N - K bits are the payload. Let CN=c0⋯cN-1T represent the transmitted signal, and CK=c0⋯cK-1T represents the transmitted binary sequence corresponding to the preamble and CK⊆CN. Let RN=r0⋯rN-1T represent the received baseband signal. RK=r0⋯rK-1T represents the received baseband signal sequence corresponding to the preamble and RK⊆RN.

Figure 7. The data packet structure with K preamble bits and N-K payload bits

- DA estimation based on M1V estimator

The DA estimation is carried out in the case of known preamble structure CK. M1V estimator is based on the first moment (M1) and the first central moment (V) of RK. For a known symbol sequence CK=1⋯1T, the pdf of rk follows non-central Chi-square distributed function and thus, the joint pdf of RK is obtained as,

pRk=j=0K-112σ2e-rj+A22σ2I0rjAσ2                           (eq.1)

M1 and V for the non central Chi-square distributed random variable rk is known as,

M1=2σ2+A2                                   (eq.2)

V=4σ4+4σ2A2                                 (eq.3)

Where A is the amplitude of the signal and σ2 is the noise variance.

Accordingly, M1V estimator contains 2 steps.

- Step1: obtain the estimated values of  M1 and V of RK, where the estimated values of  M1 and V are average value and variance of RK, respectively.

- Step2: calculate for A and σ2 according to eq.2 and eq.3.

Finally, the SNR is obtained from amplitude of the signal and noise variance.

Additionally, such estimation can be extended to estimate the interference power. Consequently, the desired signal power as well as the noise and interference power can be derived, the RSRP, RSRQ and SINR can be derived for WUR.

Also, a simple way to evaluate the link quality can be detection error rate. UE can calculate the detection error rate in certain duration. If the error rate is higher than certain threshold, it means channel quality is getting worse. Thus, measurements for signal power or noise/interference power may not needed. Additional mechanisms based on this new metric can be further studied.

Proposal 10: Following measurement metrics can be considered for measurement using WUR

- LP-RSRP;

- LP-RSRQ;

- LP-SINR;

- Detection error rate.

  - Coexistence for RRC idle/inactive and connected modes

One objective for low-power wake-up signal is to study the system impact on the coexistence with non-low-power-WUR UEs. More specifically, for LP-WUS in band transmission, how to multiplex between LP-WUS and the legacy signals/channels. As agreed in the RAN1#112 meeting that both TDMed and FDMed multiplexing with other NR transmissions should be studied for MC-ASK or MC-FSK waveform generation. TDMed multiplexing can be used for a carrier with smaller bandwidth, and FDMed multiplexing between LP-WUS and legacy signals/channels is beneficial for a carrier with large bandwidth.

For FDMed multiplexing, the legacy DL signals/channels maybe located on both sides of the LP-WUS, given the RF filter for LP-WUR is not ideal, to reduce the interference from the legacy signals/channels, some resources need to be reserved as guard band for LP-WUS. The amount of guard band highly depends on the receiver requirements on adjacent channel and adjacent subcarrier interference rejection and filter characteristics and should be decided by RAN4.

For coexistence between the LP-WUS and NR legacy signals/channels, it can always rely on gNB implementation to void configuring or scheduling the overlapped resource in both frequency and time-domain between the LP-WUS and legacy signals/channels. To improve the resource multiplexing efficiency, for coexistence between the LP-WUS and the PDSCH, current mechanism of semi-static and dynamic RB-symbol-level rate-matching pattern(s) that fit with the resources occupied by LP-WUS can be used. Note that the function of semi-static rate-matching resource set is UE mandatory capability and the function of dynamic rate-matching resource set is UE optional capability since Rel-15, it works well for legacy UEs with different capabilities. In addition, in case the LP-WUS is not transmitted, the LP-WUS resources should be reusable by the network to transmit legacy signals/channels.

Observation 19: For coexistence between legacy PDSCH and LP-WUS,

- Semi-static resource sharing by configuring RB-symbol-level or RE-level rate-matching patterns covering LP-WUS related signals can be used to improve the spectral efficiency.

- Dynamic resource sharing can be used if PDSCH is scheduled by DCI format 1_1; If PDSCH is not scheduled by DCI format 1_1, it is up to gNB implementation whether and how LP-WUS related signal is transmitted in PDSCH resource.

Proposal 11: Study both TDM and FDM multiplexing for co-existence between LP-WUS and legacy signals/channels, with both semi-static and dynamic manner.

Proposal 12: Reusing the LP-WUS resources for other NR signal/channel transmissions should be allowed.

  - Procedures for MR upon wake-up from ultra-deep sleep for RRC idle/inactive mode

During the discussion in the last meeting, there are different candidate procedures after UE is woken up by LP-WUS. From our point of view, the basic functionality of LP-WUS is to wake up UE for PO monitoring. We can further consider the need of grouping information to be carried in LP-WUS so that the false alarm rate of LP-WUS can be reduced.

| Procedure after MR woken up by LP-WUS | Pros | Cons |
| --- | --- | --- |
| Perform PO monitoring | The basic function which is expected for the design of LP-WUS. Required LP-WUS content [Note 1]: grouping indication. Medium latency: wake-up latency [ Note 2] + paging latency |  |
| Perform PEI monitoring |  | largest latency: wake-up latency + PEI monitoring latency + paging latency Once grouping indication is carried by LP-WUS, UE does not need to monitoring PEI. |
| Transmit PRACH for initial access | Lowest latency: wake-up latency | Plenty of spec impact and workload. Required LP-WUS contents: all paging message carried in paging DCI and paging PDSCH. SI update info etc. |
| Notes: 1, larger LP-WUS content, larger false detection rate of LP-WUS. 2, wake-up latency includes 1) the delay caused by LP-WUS duty cycle 2) MR transition time (ramp up time and sync/re-sync time). |  |  |

Proposal 13: UE performs PO monitoring after main radio is waken up from ultra-deep sleep.

  - Procedures of entering or exiting LP-WUS monitoring for RRC idle/inactive mode

A UE can start LP-WUS monitoring according to an explicit indication from gNB e.g., SIB or dedicated RRC signaling. For example, LP-WUS monitoring is activated right after gNB configure LP-WUS parameters. And gNB can also decide to deactivate LP-WUS monitoring by RRC reconfiguration signaling etc.

Besides, another way to enter or exit LP-WUS monitoring can be automatically determined by UE based on pre-configured conditions. For example, a UE can decide to start LP-WUS monitoring when its current channel quality is higher than a pre-defined threshold. Otherwise, it exits LP-WUS monitoring. Upon this method, whether UE needs to inform gNB its LP-WUS monitoring state i.e., enter or exit LP-WUS monitoring should be further discussed.

Proposal 14: For RRC idle/inactive mode, UE entering or exiting LP-WUS monitoring can be triggered:

- Alt1: via explicit network indication;

- Alt2: based on pre-configured condition(s). And a UE does not need to send notifications to gNB about the updates on LP-WUS monitoring behavior.

  - Procedures of LP-WUS monitoring in RRC connected mode

The design target of LP-WUS monitoring for RRC connected mode is to reduce the unnecessary PDCCH monitoring for power saving and meanwhile resume the PDCCH monitoring once traffic arrives for latency reduction. One use case for applying LP-WUS in RRC connected mode is XR, which is sensitive for both latency and power consumption. For RRC connected mode, during the LP-WUS monitoring, main radio of a UE can enter the three legacy sleep state i.e., micro, light or deep sleep instead of ultra-deep sleep to avoid long wake up delay. Regarding the RLM/BFD/CSI measurements, there seems to be no need to apply LP-WUR to do assisted measurement, so it is better to be performed by the main radio of UE to ensure the link quality with the serving cell and follow the legacy procedures for simplicity and effectiveness.

Proposal 15: For LP-WUS monitoring in RRC connected mode, RLM/BFD/CSI measurements are performed by UE Main Radio (MR) and the legacy procedures is reused.

The following are some considerations on the procedures of LP-WUS in RRC connected mode. The network configures the necessary LP-WUS monitoring parameters such as time-/freq. resources, SCS etc. Once LP-WUS monitoring is configured, when the UE starts/stops LP-WUS monitoring (or call it activation/deactivation of LP-WUS monitoring) can be studied with the interaction with C-DRX and other UE power saving features like Rel-16 DCP, R17 PDCCH skipping and SSSG switching. Taking XR use case as one example, when C-DRX with DRX on duration covering the jitter range is configured, from the beginning of the DRX on duration, the UE starts monitoring LP-WUS instead of PDCCH to reduce unnecessary PDCCH monitoring during jitter range. Once LP-WUS is detected, the UE resumes PDCCH monitoring from micro or light sleep of MR. Besides, the LP-WUS can also be monitored during the PDCCH skipping period and/or outside DRX active time. For example, the UE starts monitoring LP-WUS when it applies the PDCCH skipping and/or enter DRX off. During the PDCCH skipping period or DRX off, when LP-WUS is detected, the UE resumes PDCCH monitoring until the next PDCCH skipping command is received or the DRX timer expires. Accordingly, for eMBB use case e.g., FTP3, the long latency caused by DRX off applied can be reduced by LP-WUS monitoring. Details and some evaluation results for applying LP-WUS in RRC connected mode can be found in our companion contribution [3].

Proposal 16: Study the procedures of LP-WUS monitoring interacting with the following for RRC connected mode:

- R16 DCP, R17 PDCCH skipping and R17 SSSG switching

- C-DRX

Proposal 17: For RRC connected mode, once LP-WUS is detected by LP-WUR, UE resumes PDCCH monitoring from micro/light/deep sleep of MR.

- During LP-WUS monitoring, UE MR is not expected to enter the ultra-deep sleep to avoid long wake up delay.

- Conclusion

In this contribution, we provide our views on LP-WUS physical signal design, related functionalities and procedures. The observations and proposals are provided as follows.

Observation 1: For option OOK-1, the data rate/chip rate highly depends on the SCS of an OFDM symbol used for LP-WUS generation.

Observation 2: For OOK-1, new IFFT branch corresponding to high SCS is needed to achieve higher data/chip rate.

Observation 3: For OOK-1, uniformly distributed frequency spectrum density can be achieved.

Observation 4: OOK-2 provides additional degrees of freedom to exploit more frequency resources, the parallel transmissions can be used to achieve either high data rate or frequency diversity.

Observation 5: Option OOK-3 detection is susceptible to frequency error.

Observation 6: It is not clear how to fulfill RAN4 requirement on RE dynamic range at gNB side in option OOK-3.

Observation 7: Regarding the block ‘signal generation and modification’ in option OOK-4, signal modification can be used for making PSD of LP-WUS flatter and mitigating inter-cell interference.

Observation 8: For sequence-based OFDM signal, e.g., ZC sequence, the detection performance is more sensitive to frequency error compared with OOK waveform.

Observation 9: If the BW of LP-WUS is configurable, BPFs with different bandwidth should be implemented to better reject the interference, causing higher receiver complexity.

Proposal 1: Guard gap is included within LP-WUS bandwidth.

Proposal 2: The size of guard gap is not determined in RAN1.

Observation 10: If the BW of LP-WUS can be different for each RRC states, BPFs with different bandwidth should be implemented to better reject the interference, causing higher WUR complexity.

Proposal 3: The same BW of LP-WUS should be designed for different RRC states for simple implementation

Proposal 4: Study LP-WUS signal structure including:

- Alt 1: WUS payload only

- Alt 1a: WUS payload information is carried by sequence(s) selection

- Alt 1b: WUS payload information is carried by encoded bits (with CRC)

- Alt 2: a preamble (for sync) followed by WUS payload

- Alt 2a: WUS payload information is carried by sequence(s) selection

- Alt 2b: WUS payload information is carried by encoded bits (with CRC)

Proposal 5: Study LP-SS signal structure including:

- Alt 1: sequence(s)

- Alt 2: sequence(s) followed by additional message

- Alt 2a: additional message is carried by sequence(s) selection

- Alt 2b: additional message is carried by encoded bits (with CRC)

Observation 11: For LP-WUS structure with preamble using OOK waveform, the preamble with at least 16 chips can achieve reliable synchronization performance.

Proposal 6:

  - When evaluating and/or comparing link performance of MC-ASK, MC-FSK, and CP-OFDMA waveforms of LP-WUS at least

    - raw information bit-size

    - [time/frequency resources (including any guard bands), if applicable]

    - [total energyEPRE of LP-WUS across the time/frequency resources]

    - FFS: false alarm probability/rate

    - FFS: misdetection probability/rate

    - are kept [comparable or fixed].

Companies to report the time/frequency resources (including any guard bands), if applicable

Observation 12: For LP-WUS generated by reusing OFDM generator, the SINR can be defined per RE level, as that for OFDM based signal/channels.

Observation 13: For option OOK-1, assuming ‘on’ and ‘off’ chips are equally distributed, the power allocated to ‘on’ chip can be doubled on the average RE power to achieve certain average SINR.

Observation 14: For option OOK-4, if ‘on’ and ‘off’ chips are equally distributed within one OFDM symbol, the amplitude/power for ‘on’ chip is consistent across OFDM symbols, the SINR in each OFDM symbol is equivalent to average SINR.

Observation 15: For option OOK-4, if ‘on’ and ‘off’ chips are not equally distributed within one OFDM symbol, the amplitude/power for ‘on’ chip is not consistent across OFDM symbols, additional scaling on each OFDM symbol is needed to align the amplitude/power for ‘on’ chip.

Proposal 7: Study LP-WUS receiver autonomously determine the sampling rate, which can be beneficial from reducing power consumption perspective.

Observation 16: For duty cycle LP-WUS monitoring

- Pros: better MDR performance under certain FAR and higher power saving gain

- Cons: less NW flexibility and larger latency

Observation 17: Following pros and cons are observed for continuous LP-WUS monitoring

- Pros: higher NW flexibility and lower latency

- Cons: degraded MDR performance under certain FAR, which relies on signal/channel design, especially for long DRX cycle and non-ideal timing.

Proposal 8: It is beneficial to support RRM measurements based on LP-WUR, and the following enhancements can be considered.

- RRM measurements are totally offloaded to LP-WUR.

- LP-WUR performs RRM measurements, and MR performs relaxed RRM measurements.

Observation 18: Design for a unified LP-SS performed by LP-WUR for both synchronization and measurement is beneficial.

Proposal 9: Study suitable transmission periodicity of the LP-SS performed by LP-WUR for the following cases,

- LP-SS case 1: LP-SS is used for synchronization and measurement purpose

- LP-SS case 2: LP-SS is used for measurement purpose

Proposal 10: Following measurement metrics can be considered for measurement using WUR

- LP-RSRP;

- LP-RSRQ;

- LP-SINR;

- Detection error rate.

Observation 19: For coexistence between legacy PDSCH and LP-WUS,

- Semi-static resource sharing by configuring RB-symbol-level or RE-level rate-matching patterns covering LP-WUS related signals can be used to improve the spectral efficiency.

- Dynamic resource sharing can be used if PDSCH is scheduled by DCI format 1_1; If PDSCH is not scheduled by DCI format 1_1, it is up to gNB implementation whether and how LP-WUS related signal is transmitted in PDSCH resource.

Proposal 11: Study both TDM and FDM multiplexing for co-existence between LP-WUS and legacy signals/channels, with both semi-static and dynamic manner.

Proposal 12: Reusing the LP-WUS resources for other NR signal/channel transmissions should be allowed.

Proposal 13: UE performs PO monitoring after main radio is waken up from ultra-deep sleep.

Proposal 14: For RRC idle/inactive mode, UE entering or exiting LP-WUS monitoring can be triggered:

- Alt1: via explicit network indication;

- Alt2: based on pre-configured condition(s). And a UE does not need to send notifications to gNB about the updates on LP-WUS monitoring behavior.

Proposal 15: For LP-WUS monitoring in RRC connected mode, RLM/BFD/CSI measurements are performed by UE Main Radio (MR) and the legacy procedures is reused.

Proposal 16: Study the procedures of LP-WUS monitoring interacting with the following for RRC connected mode:

- R16 DCP, R17 PDCCH skipping and R17 SSSG switching

- C-DRX

Proposal 17: For RRC connected mode, once LP-WUS is detected by LP-WUR, UE resumes PDCCH monitoring from micro/light/deep sleep of MR.

- During LP-WUS monitoring, UE MR is not expected to enter the ultra-deep sleep to avoid long wake up delay.

# References

- Chairman’s Notes of RAN1 #112.

- IEEE P802.11ba, Part 11: Wireless LAN Medium Access Control  (MAC) and Physical Layer (PHY) Specifications, Amendment 3: Wake-Up Radio Operation.

- R1-2302506, Evaluation methodologies for R18 LP-WUS/WUR, RAN1#112e-bis, vivo.

- Venkata Subramanian Sridharan (2012). Interference cancellation techniques for low power wireless sensors. Delft University of Technology

- D. Ye, R. van der Zee and B. Nauta, "A 915 MHz 175 μW Receiver Using Transmitted-Reference and Shifted Limiters for 50 dB In-Band Interference Tolerance," in IEEE Journal of Solid-State Circuits, vol. 51, no. 12, pp. 3114-3124, Dec. 2016, doi: 10.1109/JSSC.2016.2602943.

# Appendix A –Simulation assumptions

Table 1. Assumptions for link level simulation for LP-WUS

| Attributes | Assumptions |  |
| --- | --- | --- |
| Carrier Frequency | 2.6GHz |  |
| Case name | Config1-56kbps | Config1-112kbps |
| Channel structure | sync: 16/8 chips data: 12 bits (24 chips) CRC: 8 bits (16 chips) | sync: 16/8 chips data: 40 bits (80 chips) CRC: 8 bits (16 chips) |
| Chip rate | 56kbps (1 OFDM symbol 2 chip) | 112kbps (1 OFDM symbol 4chip) |
| WUS duration | 2 slots |  |
| Waveform | OOK -4 |  |
| Coding | 1/2 rate Manchester coding (For information bits and CRC bits) |  |
| Impairment modelling | {200 ppm,0.1ppm/s} , in saturated region |  |
| Beacon periodicity | 1.28 sec Note: beacon periodicity is used to calculate the time drift for WUS monitoring |  |
| SCS | 30kHz |  |
| gNB Channel BW | 20MHz (50 RB) |  |
| WUS BW | 12RB ~4.32MHz |  |
| Guard band | 1RB on each side of LP-WUS bandwidth |  |
| Filter | 5th Order Butterworth with 4.32MHz bandwidth |  |
| ASCI | PDSCH mapped on RBs not used for LP-WUS and guard band; EPRE of LP-WUS vs EPRE of PDSCH = 1:1. |  |
| Sampling Rate | 3.84 MHz |  |
| ADC bit width | 4 bits ADC |  |
| Channel Model | TDL-C 300 |  |
| Performance metric | {FAR, MDR}: {0.1%, 1%} |  |

Table 2. Assumptions for link level simulation for sequence-based modulation waveform

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz |
| Channel structure | data: 40 bits + CRC: 8 bits |
| Chip rate | 56kbps (1 OFDM symbol 2 chip) |
| WUS duration | 2 slots |
| frequency error | 5 ppm |
| SCS | 30kHz |
| gNB Channel BW | 20MHz (50 RB) |
| WUS BW | 10RBs, 4RBs |
| Guard band | 1RB on each side of LP-WUS bandwidth |
| Filter | 5th Order Butterworth with 4.32MHz bandwidth |
| Sequence | ZC sequence, with different root - 10RBs with sequence length 119 - 4RBs with sequence length 47 |
| ADC bit width | ideal ADC |
| Channel Model | TDL-C 300 |
| FAR | 0.1% |