3GPP TSG RAN WG1 #112bis-e	                                              	                 R1-2302511

e-Meeting, April 17th - April 26th, 2023

Source:	vivo

Title:	Discussions on remaining issues of dynamic waveform switching

Agenda Item:	9.12.3

Document for:	Discussion and Decision

- Introduction

As is approved in the latest WID [1] for Rel-18 coverage enhancement work item, following objective has been defined for supporting dynamic waveform switching between DFT-s-OFDM and CP-OFDM waveform:

- Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

In this contribution, we discuss remaining issues on how to support dynamic waveform switching (DWS) between DFT-s-OFDM and CP-OFDM, including following aspects:

- Signaling of supporting DWS of PUSCH,

- UE assistance information needed for better PUSCH scheduling when DWS is enabled,

- Other relevant issues.

- Discussions

2.1 Signaling of supporting DWS of PUSCH

In this section, we discuss how the DWS can be indicated by the network.

In RAN1#111 meeting and RAN1#112 meeting, following agreements and working assumption have been made on the signaling of dynamic waveform indication. The working assumption is not confirmed and instead the single bit signaling is only agreed for indicating DWS of PUSCH transmissions with single TB.

Agreement For DCI based solution, - For supported dynamically scheduled PUSCH, support dynamic waveform switching indication from UL scheduling DCI Note: “Supported dynamically scheduled PUSCH” is to be confirmed in further discussion Note: It does not imply that the waveform switching indication applies to other transmission or not - Indication from non-UL scheduling DCI is not supported. Note: the working assumption made in RAN1#110b-e for “Support at least one of the following options for the dynamic waveform indication in R18” does not need to be confirmed Working Assumption Support new 1-bit field for dynamic waveform indication from UL scheduling DCI - Note: no change of the current size alignment procedure between UL DCI and DL DCI Agreement - For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI. - Note: no change of the current size alignment procedure between UL DCI and DL DCI.

One remaining issue is about whether PUSCH transmissions with different TBs should be indicated with DWS indepently.

In our view, for the case when multiple PUSCH transmissions are scheduled on different cells by single DCI, there’s no need to restrict the PUSCH transmissions to have same waveform in different cells as the legacy waveforms for those PUSCH transmissions are already independently configured by RRC. If there’s concern on overheaed of DCI due to one additional bit is required for each serving cell, at least for inter-band serving cells, independent DWS signalings are needed.

In addition, for PUSCH transmissions scheduled by single DCI for operation with shared spectrum channel access, multiple independent DWS indications should be applied as well. This means that multiple bits are needed for PUSCH transmissions when different TBs are used.

Therefore, we have following proposal.

Proposal 1:

- Multiple bits are used to indicate DWS for following cases

  - PUSCH transmissions on different cells (at least for inter-band serving cells) scheduled by single DCI,

  - PUSCH transmissions with different TBs scheduled by single DCI for operation with shared spectrum channel access.

In RAN1#112 meeting, DCI size alignment when DWS indication is configured has been extensively discussed. Following options has been proposed by feature leader for further discussion in this meeting.

FL proposal 2-4r2: RAN1 to select one of the following Options for DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2: Option 1: Per-format alignment - Step 1: Determine field sizes of DCI addressed to C-RNTI based on the waveform indicated by DWS field. - Step 2: If DCI is addressed to CS-RNTI, align field sizes to that of DCI addressed to C-RNTI based on the waveform indicated by DWS field using the existing C-RNTI/CS-RNTI alignment clause. - Select between: - Alt. A: - By specification, at least one field preceding NDI has size that depends on waveform (e.g. FDRA). - Note: This implies DWS field must be decoded before finding position of NDI field. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Impact on procedure for PDCCH validation of CG type 2: if DWS field value does not match waveform RRC-configured in ConfiguredGrantConfig for the CG, and NDI=0, it is an error case. - Alt. B: - By specification, no field preceding NDI has size that depends on waveform. - Note: It seems that it is currently the case for DCI format 0_1/0_2 in R17 (may need further checking) - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Note: This means DWS field can be positioned after NDI, but before the first field that depends on waveform. - No impact on procedure for PDCCH validation of CG type 2. UE can ignore value of DWS field in the validation. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). Option 2: Per-field alignment - Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Step 2: If DCI is addressed to CS-RNTI, align field sizes to those of C-RNTI according to existing alignment clause between CS-RNTI and C-RNTI. - Note: DWS field does not need to be decoded before finding position of NDI field or any other field. - Note: No constraint on position of DWS field in DCI (but positioning before any field that has waveform-dependent interpretation may help implementation). Option 3: Per-field alignment only for fields preceding NDI field - Step 1: For fields preceding NDI, apply per-field alignment as per Steps 1 and 2 of Option 2. - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Step 2: For fields following NDI, determine field sizes based on DWS as per Steps 1 and 2 of Option 1. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Note: Option 3 has same features as Option1/Alt-B with respect to impact on PDCCH validation and positioning of DWS field. In case all fields preceding NDI are guaranteed to have size independent of waveform by specification, Option 3 may not be relevant. Option 4: Per-field alignment only for DCI addressed to CS-RNTI - For DCI addressed to C-RNTI: - Step 1: Determine size of fields based on DWS indication. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Step 2: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - For DCI addressed to CS-RNTI: - Apply padding to each field, if necessary, to match the size of the corresponding field for the DCI addressed to C-RNTI and (DWS=CP-OFDM). - Note: This means DWS field does not need to be decoded before finding position of NDI field in the case of CS-RNTI. - Note: Field introduced in future specs needs to have size for CP-OFDM at least equal to size for DFT-S-OFDM, or otherwise cannot be used for CS-RNTI and NDI=1.

Among all the options, option 2 has no restrictions on the either position of DWS field and nor the field size before or after NDI field. Therefore option 2 is a bit preferred from our side.

Proposal 2:

- Support option 2 for DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2.

2.3 UE assistance information

In RAN1 #111 meeting, following agreement has been made with respect to the enhancements for assisting the network to schedule proper resources for PUSCH transmissions from UEs enabled with dynamic waveform switching.

Agreement Study the necessity of the following potential enhancements to assist the scheduler in determining waveform switching: - Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform - Target waveform can be same or different from waveform of an actual PUSCH transmission - FFS target RB allocation and/or target modulation order can be same or different from respective properties of an actual PUSCH transmission - FFS determination of target waveform, target RB allocation, target modulation order - FFS details, e.g. report PCMAX,f,c or Type 1 power headroom for a waveform, or difference thereof between waveforms - PHR triggering enhancements, e.g. - Network-triggered PHR - PH becomes lower (higher) than a threshold - PHR triggered by waveform switching - Reporting of recommended waveform or request to switch waveform - Other solutions not precluded

As is known, in current NR spec., PHR procedure is used to provide the serving gNB with a Type 1 power headroom, i.e. the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH transmission per activated Serving Cell.

According to section 7.7.1 of TS 38.213 v17.4.0, this Type 1 power headroom can be based on actual PUSCH transmission, which is calculated according to following formula,

[dB],

where PCMAX,f,c(i), PO_PUSCH,b,f,c(j), MRB,b,f,cPUSCH(i), αb,f,cj, PLb,f,c(qd), ∆TF,b,f,ci and fb,f,ci,l are defined in clause 7.1.1 of TS 38.213 v17.4.0.

Type 1 power headroom report can also be based on a reference format. In this case the PHR is virtual PHR, and for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, it is computed as

[dB]

where PCMAX,f,c(i) is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. ΔTC = 0 dB. MPR, A-MPR, P-MPR and ΔTC are defined in TS 38.101-1, TS 38.101-2 and TS 38.101-3.

As can be seen, for actual PHR, it’s based on actual waveform since the PCMAX,f,c(i) is derived according to the actual waveform dependent power reduction values defined in 38.101. While for virtual PHR, it does not depend on waveform since power reduction related parameters are assumed to be 0 dB for determination of PCMAX,f,c(i) in the formula. Therefore, when a PHR reported is virtual PHR, there’s no need to report UE assistance information for scheduling PUSCH transmissions using different waveforms.

Proposal 3:

- When a Type 1 PHR reported is virtual PHR, there’s no need to report UE assistance information for scheduling PUSCH transmissions using different waveforms.

One issue is about what kind or UE assistance information UE should report. Since this assistance information is mainly used for the case when dynamic waveform switching is enabled, one simple way is to define it as another new actual power headroom information which is also based on the actual PUSCH transmission but with the waveform different from the waveform assumed for the calculation of legacy actual Type 1 PHR.

Proposal 4:

- UE assistance information is defined as real power headroom information which is also based on the actual PUSCH transmission but with different waveform from the waveform assumed for the calculation of legacy actual Type 1 PHR.

Another issue is about the triggering of the assistance information report. As is known, Type 1 power headroom can be triggered based on periodic events and a list of conditions specified in section 5.4.6 of 38.321 v17.3.0. In our view, reusing the same triggering mechanism based on Type 1 PHR should be pursued for triggering UE assistance information report to reduce the workload and spec. impacts of this topic.

Some additional conditions are needed for reporting UE assistance information as it is only necessarily to be supported when dynamic waveform switching is enabled. In addition, considering UE may not update the waveform so frequently in some period, UE assistance information doesn’t have to be always reported in each PHR reporting period. Therefore, UE power assistance information reporting is only necessary when waveform is changed.

As an example, UE assistance information can be simply included in the Type 1 PHR MAC-CE for serving cells in which UE reports actual PHR as long as the actual PHR based on actual PUSCH transmission is triggered on condition that UE is indicated to change the waveform for current PUSCH transmission compared to last PUSCH transmission when dynamic waveform switching is enabled.  As is shown in Figure 1, one UE is served by 3 active serving cells. Assuming cell 2 is the PHR reporting CC, when PHR is triggered and waveform of a DG PUSCH in cell 2 is changed as indicated by DCI with UL grant #2, UE power assistance information (e.g. MPR calculated corresponding to the changed waveform) can be transmitted together with legacy PHR. However, for cell 1 and cell 3, UE power assistance information is not reported since the corresponding PHR is virtual due to the UL grant #1 and #3 are later than UL grant #2 in cell 2 according to legacy rules of determining whether a power headroom report for an activated serving cell is based on an actual transmission (real PH) or a reference format (virtual PH). Note that the UE power assistance information reported is not intended for improving resource configuration of currently scheduled PUSCH (transmitting PHR MAC-CE), it’s mainly intended for scheduling of future PUSCH transmissions that would probably use same waveform as the waveform of current PUSCH.

Figure 1. UE power assistance information reporting with legacy PHR trigging mechanism when waveform is changed.

According to above and the observation 1, we have following proposal.

Proposal 5:

- UE assistance information can be included in the entries of MAC-CE for reporting PHR based on actual PUSCH transmissions assuming same triggering mechanism as is used for triggering the actual PHR, on condition that the waveform is changed for current PUSCH transmission compared to last PUSCH transmission when dynamic waveform switching is enabled.

2.4 Other issues

In NR up to Rel-17, some features can be supported only for CP-OFDM or only for DFT-s-OFDM, including

- Rank>1 which is only for CP-OFDM,

- Pi/2 BPSK modulation which is only for DFT-s-OFDM,

- Frequency domain resource allocation Type 0 which is only for CP-OFDM.

Therefore, it is possible that the target waveform of a PUSCH can be indicated as a waveform that is not expected to work together with other features when dynamic waveform switching is applied.

Observation 2:

- It is possible that the target waveform of a PUSCH can be indicated as a waveform that is not expected to work together with other features when dynamic waveform switching is applied.

For example, to support DWS indicated by DCI scheduling a PUSCH with rank>1, some rules may be needed to make sure the target waveform is only CP-OFDM. Similarly, rules may be needed to make sure pi/2 BPSK and frequency domain resource allocation type 0 are not indicated while the target waveforms are CP-OFDM and DFT-s-OFDM for these 2 cases respectively.

Proposal 6:

- RAN1 should discuss rules to avoid indicating a target waveform that is not supposed to be supported together with existing features like rank>1, pi/2 BPSK, FDRA type 0.

As is known, waveform is configured in PUSCH-Config which is used to configure the UE specific PUSCH parameters applicable to a particular BWP. When BWP change is indicated by DCI, the waveform changing may be not necessary since it’s a first transmission in the new BWP. Therefore, we propose to not have waveform changed when BWP is changed.

Proposal 7:

- RRC configured waveform of target BWP should be applied for PUSCH transmission scheduled by DCI indicating BWP change, even if dynamic waveform switching is supported in the target BWP.

- Conclusion

In this contribution, we discuss issues of supporting dynamic waveform switching between DFT-s-OFDM and CP-OFDM waveform, and have following  observations and proposals:

Observations 1 to 2:

- When a Type 1 PHR reported is virtual PHR, there’s no need to report UE assistance information for scheduling PUSCH transmissions using different waveforms.

- It is possible that the target waveform of a PUSCH can be indicated as a waveform that is not expected to work together with other features when dynamic waveform switching is applied.

Proposals 1 to 7:

- Multiple bits are used to indicate DWS for following cases

  - PUSCH transmissions on different cells (at least for inter-band serving cells) scheduled by single DCI,

  - PUSCH transmissions with different TBs scheduled by single DCI for operation with shared spectrum channel access.

- Support option 2 for DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2.

- UE assistance information is defined as real power headroom information which is also based on the actual PUSCH transmission but with different waveform from the waveform assumed for the calculation of legacy actual Type 1 PHR.

- UE assistance information can be included in the entries of MAC-CE for reporting PHR based on actual PUSCH transmissions assuming same triggering mechanism as is used for triggering the actual PHR, on condition that the waveform is changed for current PUSCH transmission compared to last PUSCH transmission when dynamic waveform switching is enabled.

- RAN1 should discuss rules to avoid indicating a target waveform that is not supposed to be supported together with existing features like rank>1, pi/2 BPSK, FDRA type 0.

- RRC configured waveform of target BWP should be applied for PUSCH transmission scheduled by DCI indicating BWP change, even if dynamic waveform switching is supported in the target BWP.

References

- RP-221858, Revised WID on Further NR coverage enhancements, 3GPP TSG RAN Meeting #96, Budapest, Hungary, June 6-9, 2022.