3GPP TSG RAN WG1 #112bis-e		R1-2302514

e-Meeting, April 17th – April 26th, 2023

Source:	vivo

Title:	Discussion on UE features for NCR

Agenda Item:	9.17.4

Document for:	Discussion and Decision

- Introduction

The WI on network-controlled repeaters (NCR) has been completed from RAN1 perspective. In this contribution, we provide our view on the UE features for the NCR.

- Discussion

NCR is comprised of NCR-Fwd and NCR-MT, for definition of UE features, the features for both NCR-Fwd and NCR-MT should be defined.

NCR-Fwd features

In RAN1, several features were discussed and supported for NCR-Fwd, including beam control, ON-OFF control, TDD, timing, C-link and backhaul link multiplexing and so on. The features should be further categorized and defined as optional or mandatory feature to easy the implementation.

Beam control

For the beam control, both backhaul link (BH) beam control and access link (AL) beam control are supported.

For BH beam control, either fixed beam or adaptive beam can be used, where the adaptive BH beam is determined based on either explicit indication or implicit rule. From implementation perspective, only one of the fixed beam and the adaptive beam may be implemented by vendor, thus it is beneficial to support an optional feature for adaptive BH beam determination.

Proposal 1: Support an optional feature for adaptive BH beam determination, including explicit BH beam indication and implicit BH beam determination.

For the AL beam indication, it is straightforward to define a feature group which includes periodic indication, semi-persistent indication and aperiodic indication. Since the AL beam indication controls the ON operation of the NCR (without AL beam indication the NCR is always OFF by default), it is reasonable to define this AL beam indication feature as mandatory feature.

Proposal 2: Support a mandatory feature group for access beam indication, including periodic indication, semi-persistent indication and aperiodic indication.

ON-OFF control

For the ON-OFF control, since no dedicated signaling or procedure is specified for the ON state indication, a feature for the OFF state determination is sufficient and the feature group includes the following functions.

- The NCR is off on the time resource where AL beam indication is not applied.

- If BF is detected by C-link, the NCR is off before BF is covered.

- For the symbols semi-statically configured as flexible, the NCR is off on these symbols.

Since the OFF state determination is the basic operation of NCR, it is preferred to define a mandatory feature for the OFF state determination. Considering ON-OFF state as tightly correlated and the ON state determination is based on AL beam indication, it is preferred to define a unified feature for AL beam indication and ON-OFF state determination.

Proposal 3: Support a mandatory feature for OFF state determination, and the feature can be merged to AL beam indication feature group as a unified feature group.

TDD

The determination of TDD configuration can be an optional feature for the NCR-Fwd. The reasons are listed as following

- TDD UL-DL configuration is required for the operation on TDD band, while for the operation on FDD band, NCR does not need to know any TDD configuration.

- Even for TDD band, NCR can still acquire TDD configureation via OAM.

Proposal 4: Support an optional feature for TDD determination of NCR-Fwd.

Timing

The determination of timing is basic function for NCR, thus a mandatory feature should be defined.

Proposal 5: Support a mandatory feature for NCR-Fwd timing determination.

C-link and BH link multiplexing

110b Agreement

The following aspects should be NCR capability:

- Simultaneous UL transmission of C-link and backhaul link

- Adaptive beam for C-link/backhaul-link

- Note-1: Fixed beam for C-link/backhaul link is default capability

- Note-2: TDMed UL transmission of C-link and backhaul link is default capability.

- FFS: How to define the capability for adaptive beam for C-link/backhaul-link

In RAN1#110bis meeting, it has been agreed that TDMed UL transmission of C-link and backhaul link is default capability and optionally NCR capability can support simultaneous UL transmission of C-link and backhaul link. To reflect such NCR capability, it is preferred to define an optional feature for this.

Proposal 6: Support an optional feature for simultaneous UL transmission of C-link and backhaul link.

  - NCR-MT features

Besides the features defined for NCR-Fwd, the features for NCR-MT should be defined. Regarding the NCR-MT features, the NCR-MT should inherit the UE features from a normal UE, and RAN1 should focus on the discussion on which mandatory UE features of a normal UE can be set as optional feature for NCR-MT considering that NCR-MT can be a simplified UE.

Proposal 7: RAN1 should focus on the discussion on which mandatory UE features of a normal UE can be set as optional feature for NCR-MT

Similar as the discussion for IAB-MT features, the Rel-15 mandatory UE feature [1] without capability signaling or mandatory UE feature with capability signaling set as “1” can be the baseline for further discussion to set NCR-MT mandatory feature.

FG 0: Waveform and modulation

For FG 0-1/0-2/0-3/0-4, since these waveform and modulation features are always needed, thus NCR-MT should support these feature as mandatory as well.

FG 1: Initial access and mobility

For FG 1-1/1-3, considering the NCR-MT also supports initial access, FG 1-1 should be mandatory; and considering the NCR-MT also performs radio link monitoring, especially when NCR is deployed with lower elevation and the channel condition varies, FG 1-3 should also be mandatory for NCR-MT.

FG 2: MIMO

For FG 2-1, the feature ensure basic PDSCH reception, which should be mandatory for NCR-MT. For FG 2-5/2-6, since the NCR-MT DL does not requires complicated scheduling scheme considering normal data transmission is not expected in the C-link, only one of the FG 2-5/2-6 is sufficient for DL DMRS. For FG 2-12/FG 2-16/2-16a, the feature ensure basic PUSCH reception which should be mandatory for NCR-MT considering that at least initial access procedure requires PUSCH transmission. Moreover, only one of the FG 2-16/2-16a is sufficient for UL DMRS mapping. For FG 2-32, CSI feedback feature is included in this FG, this FG seems not necessary for NCR-MT, since only control information is transmitted between gNB and NCR-MT and lower MCS can be always used for control signaling transmission. For the same reason as for FG 2-32, FG 2-52 for basic SRS can also be set as optional feature for NCR-MT. For FG 2-50, since TRS is beneficial to improve receiver decoding performance, basic component of 2-50 can be mandatory for NCR-MT, e.g., component 1 of FG 2-50.

Proposal 8: RAN1 should discuss whether CSI feedback and SRS are mandatory or optional for NCR-MT.

FG 3: DL control channel and procedure

For the FG 3-1, the FG enables basic DL control channel reception, which should be supported as mandatory feature. Moreover, DCI format 5-0 is defined to carry side control information, monitoring of DCI format 5-0 should also be mandatory for NCR-MT.

FG 4: UL control channel and procedure

For FG 4-1, the FG enables basic UL control channel reception. Since CSI feedback may be not mandatory for NCR-MT as discussed above, it is reasonable to at least define a subset of components in FG 4-1 as mandatory. In our opinion, at least component 1-6 can be baseline for further discussion to support basic HARQ-ACK feedback and SR functionality. For FG 4-10, RAN1 should further discuss whether codebook based HARQ-ACK feedback is mandatory for NCR-MT or not, since only some control information is transmitted via C-link, such feature may not be popular for NCR-MT.

FG 5: Scheduling and HARQ

For FG 5-1, the FG describes basic scheduling/HARQ operation, considering complicated scheduling scheme may be not required for NCR-MT since only control information is to be transmitted via C-link, only a set of components need to be set as mandatory, e.g., components 1/2/3/4/5/6/7/9/10/12 as for IAB-MT.

Other FG

For FG 6-1, Basic BWP operation should be mandatory for NCR-MT. For FG 7-1, channel coding should be mandatory for NCR-MT. For FG 8-3, basic power control operation should be mandatory for NCR-MT.

Proposal 9: RAN1 at least considers the following FGs for normal UE as NCR-MT mandatory features.

- FG 0-1, FG 0-2, FG 0-3, FG 0-4

- FG 1-1, FG 1-3

- FG 2-1, one of the FG 2-5/2-6, FG 2-12, one of FG 2-16/2-16a, FG 2-50 (FFS which components are mandatory)

- FG 3-1 including reception of DCI format 5-0

- FG 4-1 (FFS which components are mandatory)

- FG 5-1 (FFS which components are mandatory)

- FG 6-1

- FG 7-1

- FG 8-3

- Conclusion

In this contribution, we have the following proposals.

Proposal 1: Support an optional feature for adaptive BH beam determination, including explicit BH beam indication and implicit BH beam determination.

Proposal 2: Support a mandatory feature group for access beam indication, including periodic indication, semi-persistent indication and aperiodic indication.

Proposal 3: Support a mandatory feature for OFF state determination, and the feature can be merged to AL beam indication feature as a unified feature group.

Proposal 4: Support an optional feature for TDD determination of NCR-Fwd.

Proposal 5: Support a mandatory feature for NCR-Fwd timing determination.

Proposal 6: Support an optional feature for simultaneous UL transmission of C-link and backhaul link.

Proposal 7: RAN1 should focus on the discussion on which mandatory UE features of a normal UE can be set as optional feature for NCR-MT

Proposal 8: RAN1 should discuss whether CSI feedback and SRS are mandatory or optional for NCR-MT.

Proposal 9: RAN1 at least considers the following FGs for normal UE as NCR-MT mandatory features.

- FG 0-1, FG 0-2, FG 0-3, FG 0-4

- FG 1-1, FG 1-3

- FG 2-1, one of the FG 2-5/2-6, FG 2-12, one of FG 2-16/2-16a, FG 2-50 (FFS which components are mandatory)

- FG 3-1 including reception of DCI format 5-0

- FG 4-1 (FFS which components are mandatory)

- FG 5-1 (FFS which components are mandatory)

- FG 6-1

- FG 7-1

- FG 8-3

- Reference

[1] 3GPP TS 38.822, User Equipment (UE) feature list.