3GPP TSG RAN WG1 #112-bis-e		                                                       R1-2302517

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.17.5

Source: 	vivo

Title: 	Discussion on UE features for BWP without restriction

Document for:	Discussion and Decision

- Introduction

In RAN#99, a new WI of “Complete the specification support for BandWidth Part operation without restriction in NR” was approved in [1]. This contribution presents our view on RAN1 UE features for achieving following objectives for BWP Without Restriction for Rel-18 NR.

### The work item includes following objectives: - For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to IoDT availability). - The interruption related requirements will be decided and specified in RAN4. The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

- Discussion

  - Option B-1-1

For Option B-1-1, an FG X-1 of “Support RLM/BM/BFD measurements based on SSB outside active BWP without interruptions” needs to be defined. Following componnets can be included for the FG X-1:

- Componnets:

  - BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell

  - UE performs RLM/BM/BFD measurements based on SSB without interruptions, where the SSB is outside active DL BWP but is in the bandwidth of the corresponding carrier(s) to be measured

  - The UE is not expected to receive PDSCH, PDCCH, or CSI-RS outside an active DL BWP.

Given FG X-1 and FG X-3 are to replace legacy FG 6-1a, the component of legacy 6-1a should be included as one component for FG X-1 and FG X-3.

In addition, component is also added for FG X-1 and FG X-3 to make clear that for Option B-1-1 and Option B-1-2, UE is allowed to only measure SSB outside the active DL BWP, but it does not mean UE needs to receive other DL signals/channels outside the active DL BWP.

Proposal 1: Introduce FGX-1 of “Support RLM/BM/BFD measurements based on SSB outside active BWP without interruptions” for Option B-1-1, with following comopnents:

- BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell

- UE performs RLM/BM/BFD measurements based on SSB without interruptions, where the SSB is outside active DL BWP but is in the bandwidth of the corresponding carrier(s) to be measured

- The UE is not expected to receive PDSCH, PDCCH, or CSI-RS outside an active DL BWP.

About Prerequisite feature groups for FG X-1, we do not FG 6-1a should be the prerequisite FG for Option B-1-1 and Option B-1-2, due to following reasons:

- This approved WI indicates that the specification support for FG 6-1a is not yet fully complete; and

- The component of FG 6-1a is now added as one component for FG X-1 and X-3..

Proposal 2: FG6-1a should not be the prerequisite FG for the FGs introduced for Option B-1-1 and Option B-1-2.

About the Type, same as the Type defined for FG 6-1a “bwp-WithoutRestriction”, FG X-1 (Option B-1-1) should be defined per band and no need of FDD/TDD and FR1/FR2 differentiation.

Proposal 3: For the FGs introduced for Option B-1-1, Option C and Option B-1-2, the Type should be per band and no FDD/TDD and FR1/FR2 differentiation.

In addition, FG X-1 (Option B-1-1) should have a note of “FGX-1 is applicable to FG 6-1, 6-2, 6-3 or 6-4”, which is the same as that for FG6-1a.

Proposal 4:  Add the note that the FGX-1 is applicable to FG 6-1, 6-2, 6-3 or 6-4.

  - Option C

For Option C, an FG X-2 of “Support RLM/BM/BFD measurements based on NCD-SSB within active BWP” needs to be defined. The component that UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP can be included for the FG X-2.

Proposal 5: Introduce FGX-2 of “Support RLM/BM/BFD measurements based on NCD-SSB within active BWP” for Option C, with following comopnent:

- UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP

About the Prerequisite feature groups for FG X-2, it seems the current feature that active BWP includes NCD-SSB contradicts the legacy FG6-1a since in FG 6-1a, the “SSB” includes both CD- and NCD-SSB. Hence the FG 6-1a should not be the Prerequisite feature group for FG X-2.

Proposal 6: FG6-1a should not be the prerequisite FG for the FG introduced for Option C.

Besides the note that FGX-2 is applicable to FG 6-1, 6-2, 6-3 or 6-4”, it should be clear that FGX-2 is only applicable to non-RedCap UEs.

Proposal 7: Following should be included in the “Note” colomn for FGX-2 (Option C).

- X-2 is applicable to 6-1, 6-2, 6-3, or 6-4.

- X-2 is only applicable to non-RedCap UEs.

  - Option B-1-2

For Option B-1-2, an FG X-3 of “Support RLM/BM/BFD measurements based on SSB outside active BWP with interruptions” needs to be defined. Following componnets can be included for the FG X-3:

- Componnets:

- BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell

- The UE is allowed to perform RLM/BM/BFD measurements based on SSB outside the active BWP with interruptions only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active DL BWP of the corresponding carrier(s) to be measured.

- The UE is not expected to receive PDSCH, PDCCH, or CSI-RS outside an active DL BWP.

Proposal 8: Introduce FGX-3 of “Support RLM/BM/BFD measurements based on SSB outside active BWP with interruptions” for Option B-1-2, with following comopnents:

- BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell

- The UE is allowed to perform RLM/BM/BFD measurements based on SSB outside the active BWP with interruptions only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active DL BWP of the corresponding carrier(s) to be measured.

- The UE is not expected to receive PDSCH, PDCCH, or CSI-RS outside an active DL BWP.

About Prerequisite feature groups for FG X-3, based on WID that

- The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and

- UE shall support option (C) NCD-SSB (subject to IoDT availability).

Hence, FG 1-7, 2-24, 2-31, X-2 should be the prerequisite FGs for Option B-1-2.

Proposal 9: FG 1-7, 2-24, 2-31, X-2 should be the prerequisite FGs for FG X-3 (Option B-1-2).

Besides the note that FGX-3 is applicable to FG 6-1, 6-2, 6-3 or 6-4”, following notes should added for FG X-3.

- This feature only applies if there is no CSI-RS, no NCD- SSB, and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured

- The interruption time and corresponding UE behevaior is left to RAN4 to decide

Proposal 10:  Following should be included in the “Note” colomn for FGX-3 (Option B-1-2).

- X-3 is applicable to 6-1, 6-2, 6-3, or 6-4.

- This feature only applies if there is no CSI-RS, no NCD- SSB, and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured

- The interruption time and corresponding UE behevaior is left to RAN4 to decide

- Conclusion

This contribution provides our views for UE features required for BWP without restriction. Following is the proposal summary followed by the proposaed Feature Table “X. NR_BWP_wor”.

Proposal 1: Introduce FGX-1 of “Support RLM/BM/BFD measurements based on SSB outside active BWP without interruptions” for Option B-1-1, with following comopnents:

- BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell

- UE performs RLM/BM/BFD measurements based on SSB without interruptions, where the SSB is outside active DL BWP but is in the bandwidth of the corresponding carrier(s) to be measured

- The UE is not expected to receive PDSCH, PDCCH, or CSI-RS outside an active DL BWP.

Proposal 2: FG6-1a should not be the prerequisite FG for the FGs introduced for Option B-1-1 and Option B-1-2.

Proposal 3: For the FGs introduced for Option B-1-1, Option C and Option B-1-2, the Type should be per band and no FDD/TDD and FR1/FR2 differentiation.

Proposal 4:  Add the note that the FGX-1 is applicable to FG 6-1, 6-2, 6-3 or 6-4.

Proposal 5: Introduce FGX-2 of “Support RLM/BM/BFD measurements based on NCD-SSB within active BWP” for Option C, with following comopnent:

- UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP

Proposal 6: FG6-1a should not be the prerequisite FG for the FG introduced for Option C.

Proposal 7: Following should be included in the “Note” colomn for FGX-2 (Option C).

- X-2 is applicable to 6-1, 6-2, 6-3, or 6-4.

- X-2 is only applicable to non-RedCap UEs.

Proposal 8: Introduce FGX-3 of “Support RLM/BM/BFD measurements based on SSB outside active BWP with interruptions” for Option B-1-2, with following comopnents:

  - BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell

  - The UE is allowed to perform RLM/BM/BFD measurements based on SSB outside the active BWP with interruptions only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active DL BWP of the corresponding carrier(s) to be measured.

  - The UE is not expected to receive PDSCH, PDCCH, or CSI-RS outside an active DL BWP.

Proposal 9: FG 1-7, 2-24, 2-31, X-2 should be the prerequisite FGs for FG X-3 (Option B-1-2).

Proposal 10:  Following should be included in the “Note” colomn for FGX-3 (Option B-1-2).

- X-3 is applicable to 6-1, 6-2, 6-3, or 6-4.

- This feature only applies if there is no CSI-RS, no NCD- SSB, and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured

- The interruption time and corresponding UE behevaior is left to RAN4 to decide

- NR_BWP_wor

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X. NR_BWP_wor | X-1 | Support RLM/BM/BFD measurements based on SSB outside active BWP without interruptions | - BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell - UE performs RLM/BM/BFD measurements based on SSB without interruptions, where the SSB is outside active DL BWP but is in the bandwidth of the corresponding carrier(s) to be measured - The UE is not expected to receive PDSCH, PDCCH, or CSI-RS outside an active DL BWP. |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on SSB outside active BWP without interruptions | Per Band | No | No | n/a | X-1 is applicable to 6-1, 6-2, 6-3, or 6-4. | Optional with capability signalling |
| X. NR_BWP_wor | X-2 | Support RLM/BM/BFD measurements based on NCD-SSB within active BWP | - UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on NCD-SSB within active BWP | Per Band | No | No | n/a | - X-2 is applicable to 6-1, 6-2, 6-3, or 6-4. - X-2 is only applicable to non-RedCap UEs. | Optional with capability signalling |
| X. NR_BWP_wor | X-3 | Support RLM/BM/BFD measurements based on SSB outside active BWP with interruptions | - BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell - The UE is allowed to perform RLM/BM/BFD measurements based on SSB outside the active BWP with interruptions only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active DL BWP of the corresponding carrier(s) to be measured. - The UE is not expected to receive PDSCH, PDCCH, or CSI-RS outside an active DL BWP. | 1-7, 2-24, 2-31, X-2 | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on SSB outside active BWP with interruptions | Per Band | No | No | n/a | - X-3 is applicable to 6-1, 6-2, 6-3, or 6-4. - This feature only applies if there is no CSI-RS, no NCD- SSB, and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured - The interruption time and corresponding UE behevaior is left to RAN4 to decide | Optional with capability signalling |

- Reference

- RP-230805, New WID: Complete the specification support for BandWidth Part operation without restriction in NR, Moderator (Vodafone), vivo